3GPP TSG RAN WG1 #118		                                              R1-2407219

Maastricht, NL, August 19th – 23rd, 2024

Source:	Moderator (CATT)

Title:	Summary #2 of 	SBFD TX/RX/measurement procedures

Agenda Item:	9.3.1

Document for:	Discussion and Decision

- Introduction

NR duplex evolution was studied in Rel-18 with outcome captured in TR 38.858 [1].

Rel-19 WI on evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved in RP-234035 [2] and updated in RP-241614 [4] with the following objectives.

This document summarizes the inputs and the discussions in RAN1#118 on Rel-19 SBFD TX/RX/measurement procedures corresponding to objectives highlighted in cyan below.

Note that it was agreed in RAN1#116 that CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3 and it was agreed in RAN1#116bis that support of UL Tx power control enhancements can be discussed in AI 9.3.1 and 9.3.2 (for PRACH only).

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for co-channel CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

- Proposals for online sessions

  - Tuesday (Aug 20th)

- Proposal 1-1

Proposed Agreement:

For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are configured.

- A starting slot index

- A starting symbol index within the starting slot

- An ending slot index

- An ending symbol index within the ending slot

- Proposal 1-2a

Proposed Agreement:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275

- One or two SBFD DL subbands can be configured

- Proposal 2-1a

Proposed Conclusion:

If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs across two DL subbands in SBFD symbols.

- Proposal 2-2a

Proposed Agreement:

Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2.

- Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols.

Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH.

- Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols.

  - Support separate configurations for DCI format 0_2 and other DCI formats as legacy

  - The number of bits used to indicate FH offset in DCI is determined as legacy

UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions.

FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

- Proposal 2-3

Proposed Agreement:

Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols in PUCCH-Resource.

- Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols.

- FFS whether to support separate configurations of intraSlotFrequencyHopping

- Proposal 2-4a

Proposed Agreement:

Support separate configurations of SRS resource sets for SBFD symbols and non-SBFD symbols.

- Introduce new RRC parameters in SRS-Config to configure SRS resource sets for SBFD symbols.

- FFS applicable SRS usages

- Proposal 3-2a

Proposed Agreement:

For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition.

  - Tuesday (Aug 21st)

- Proposal 2-2b

Proposed Agreement:

Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2.

- Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols.

Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH.

- Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols.

  - Support separate configurations for DCI format 0_2 and other DCI formats as legacy

  - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy

UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions.

FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

- Proposal 2-3a

Proposed Agreement:

Support separate configurations for SBFD symbols and non-SBFD symbols in PUCCH-Resource.

- pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols

- Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols

  - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols

- FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2

- FFS whether to support separate configurations of format for Configuration 1 or for both Configuration 1 and 2

FFS UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

- Proposal 2-5b

Proposed Agreement:

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), down-select from the following options.

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration to determine frequency resource for the other symbol type

- Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid. For PDSCH repetitions and multi-PDSCH scheduled by a single DCI, a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is dropped.

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

  - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only

  - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition.

  - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

- Proposal 2-7b

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), down-select from the following options.

- Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration determination to determine frequency resource for SBFD symbols

- Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid. A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is dropped or postponed.

  - For PUSCH repetition type-A with available slot counting and TBoMS, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is postponed.

  - For PUSCH repetition type-A without available slot counting and multi-PUSCH scheduled by a single DCI, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is dropped.

- Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid

  - For CG PUSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within UL usable PRBs only.

  - For PUSCH repetitions and TBoMS, FFS whether the number of PRBs for TBS determination is based on assigned PRBs PRBs, or based on assigned PRBs within UL usable PRBs only, or based on assigned PRBs within UL usable PRBs of the first repetition/PUSCH of TBoMS in the first slot.

  - For multi-PUSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within UL usable PRBs only.

- Proposal 2-9a

Proposed Agreement:

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following.

- For semi-statically configured transmissions/receptions, the valid symbol type is explicitly configured by RRC.

- For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception.

- For SP-CSI on PUCCH or PUSCH, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in CSI-ReportConfig.

  - Option 2: The valid symbol type is determined based on the symbol type of the first PUSCH/ PUCCH after activation.

- For type 2 CG PUSCH and SPS PDSCH, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in ConfiguredGrantConfig/SPS-Config.

  - Option 2: The valid symbol type is determined based on the symbol type of the first CG PUSCH/SPS PDSCH after activation.

- For SPS HARQ-ACK, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in SPS-Config.

  - Option 2: The valid symbol type is determined based on the symbol type of the first PUCCH carrying SPS HARQ-ACK after activation.

- For Semi-persistent SRS, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in SRS-Config.

  - Option 2: The valid symbol type is determined based on the symbol type of the first SRS after activation.

- Summary of input contributions

The inputs from companies’ contributions are summarized below according to moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or are missed, in which case please feel free to correct/update the summary with revision marks.

  - SBFD subband indication

    - Signalling design details

      - Time location indication

In RAN1#116, it was agreed that SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period and a slot can consist of SBFD symbols and non-SBFD symbols.

Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details

Various signaling designs are provided by companies to configure the consecutive SBFD symbols within a TDD-UL-DL pattern period.

Opt 1: Separate indication of starting SBFD symbol and ending SBFD symbol

    - Support: MediaTek, Spreadtrum, LGE, vivo, OPPO, CATT, Fujitsu, Kookmin University, CATT, Ericsson, Nokia

      - Starting slot index, starting symbol index with a slot, ending slot index, ending symbol index within a slot: MediaTek, Ericsson, CATT

      - Starting slot index, starting symbol index with a slot, number of slots, ending symbol index within a slot: OPPO, Fujitsu, Kookmin University

      - Reference symbol for the ending SBFD symbol is the last symbol among the DL or Flexible symbols configured by TDD-UL-DL-ConfigCommon: LGE

Opt 2: SLIV of SBFD starting symbol and duration

    - Support:ZTE, CMCC,vivo, xiaomi, CATT, Samsung, Sharp, DOCOMO, QC

Opt 3: Separate indications of number of slots/symbols of non-SBFD symbols at the beginning and at the end of a TDD-UL-DL pattern period

    - Support: xiaomi, CATT, Samsung

Separate indication of starting SBFD slot/symbol and number of SBFD slot/symbols

    - Support: Spreadtrum, ITRI, CEWiT

SBFD slots + the last X symbols in the first SBFD slot as SBFD symbols and/or the first Y symbols in the last SBFD slot as SBFD symbols

    - Support: Huawei

      - Frequency location indication

For semi-static frequency indication of SBFD subbands, the following agreements were made in RAN1#116 and RAN1#116bis.

Agreement (RAN1#116) The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS reference starting RB and reference SCS Agreement (RAN1#116bis) For cell-specific configuration of frequency locations of SBFD subbands, - Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

Based on the above agreements, in order to configure the frequency locations of SBFD subbands, the following approaches are proposed by companies and illustrated in Figure 1.

Approach a: RIV for each subband, i.e. 3 RIVs

    - Support:Spreadtrum, ZTE, vivo, xiaomi, CATT, Sharp, DOCOMO, QC

Approach b: 1 RIV for UL subband + 2 subband sizes for DL subbands (or 1 ending RB index of lower DL subband + 1 starting RB index of upper DL subband)

    - Support: OPPO, Samsung, Nokia

Approach c: starting RB index and subband size for UL subband + 2 subband sizes for DL subbands, where starting RB index for UL subband can be omitted if UL subband is located on one side of the carrier

    - Support: LGE

Figure 1: SBFD subband frequency location indication schemes

The signalling overhead of the approaches are provided in the table below based on moderator’s understanding.

|  | Approach a | Approach b | Approach c |
| --- | --- | --- | --- |
| Overhead (bits) | 16+16+16 = 48 | 16+9+9 = 34 | 9+9+9+9 = 36 |
| - For each RIV, log2(NBWPsize(NBWPsize+1)/2)=16 bits are required - For number of RBs, ending RB index or starting RB index, log2NBWPsize=9 bits are required |  |  |  |

Lenovo proposed that the number of RBs is indicated as a percentage value of a total number of RBs.

In addition, Google proposed to consider the following methods to indicate UL/DL subband frequency resource.

- Option 2: Indicate SB boundary set by using RIV-based method with NBWPsize=275, and a range-reduced LRBs (e.g., 0 to 15 [or 28]) to indicate UL-to-DL SB boundaries offset.

- Option 3: Indicate SB boundary set, where the starting CRB (e.g., 0 to 274) and offset (e.g., 0 to 15) are indicated separately.

Figure 2: Example of SBFD resource configuration [40]

    - UE-specific indication of SBFD frequency locations

For semi-static indication of frequency locations of SBFD subbands, the following agreements were made in RAN1#116.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

Huawei, ZTE, CMCC, vivo, Fujitsu, Samsung proposed to confirm the above working assumption.

Companies’ views on support of UE-specific configuration on frequency locations of SBFD subbands are summarized below.

- UE-specific configuration on frequency location of SBFD subbands

  - Support: LGE, CMCC, ETRI, QC

  - Not support: CT, Ericsson, Samsung, Spreadtrum, vivo (no Rx filter at UE side assumed), Kookmin University, Fujitsu, HONOR, Transsion

  - Send LS to RAN4: Huawei, Nokia

    - Guard periods

Some companies discussed whether to explicitly configure guard periods between SBFD and non-SBFD symbols.

- Guard periods between SBFD and non-SBFD symbols

  - Explicitly configured

    - Support: CMCC, CATT, Apple

    - Interaction with legacy TDD slot configuration indications

It was agreed in RAN1#116 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0, where DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol.

It was agreed to support cell-specific configuration on time location of SBFD subbands and any UE-specific or group-common configuration cannot revert a SBFD/non-SBFD symbol to a non-SBFD/SBFD symbol.

Agreement (RAN1#116) For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands Agreement (RAN1#116bis) A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

One related discussion point is that whether explicit link direction is supported via TDD-UL-DL-ConfigDedicated and/or SFI in DCI format 2_0, which is summarized in section 3.2.4.1.

Companies’ views on interation with TDD-UL-DL-ConfigDedicated and SFI if link direction indication is not supported nor provided are summarized below.

TDD-UL-DL-ConfigDedicated

For a serving cell configured with SBFD subband time and frequency location,

- An SBFD aware UE does not expect to be provided with TDD-UL-DL-ConfigDedicated for the serving cell

  - Support:

  - Not support:

- An SBFD aware UE does not expect to be provided with TDD-UL-DL-ConfigDedicated for SBFD symbols

  - Support: Ericsson, Samsung, Nokia, QC

  - Not support:

- TDD-UL-DL-ConfigDedicated is not applicable to SBFD symbols. An SBFD aware UE interprets TDD-UL-DL-ConfigDedicated as legacy for non-SBFD symbols.

  - Support: Huawei, ZTE, LGE, Ericsson, Nokia, QC

- TDD-UL-DL-ConfigDedicated is applicable to both non-SBFD and SBFD symbols.

  - Support: Spreadtrum, CEWiT

- TDD-UL-DL-ConfigDedicated is applicable to both non-SBFD and SBFD symbols in flexible symbols.

  - Support: Sony

SFI in DCI format 2_0

For a serving cell configured with SBFD subband time and frequency location,

- An SBFD aware UE does not expect to be configured to monitor DCI format 2_0 for the serving cell.

  - Support: Samsung, Nokia, DOCOMO

  - Not support:

- An SBFD aware UE does not expect to receive SFI for SBFD symbols.

  - Support: QC

  - Not support:

- An SBFD aware UE ignores SFI in DCI format 2_0 for the serving cell, if provided.

  - Support: DOCOMO

- SFI in DCI format 2_0 is not applicable to SBFD symbols. An SBFD aware UE interprets SFI in DCI format 2_0 as legacy for non-SBFD symbols.

  - Support: Huawei, ZTE, LGE, Ericsson, Nokia, QC, DOCOMO,

- SFI in DCI format 2_0 is applicable to both non-SBFD and SBFD symbols.

  - Support: Spreadtrum, CEWiT

- SFI in DCI format 2_0 is applicable to both non-SBFD and SBFD symbols in flexible symbols.

  - Support: Sony

  - TX/RX/measurement procedures

    - TX/RX/measurement behaviors

The following agreement was made in RAN1#116.

Agreement For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL usable PRBs are allowed - FFS SSB symbols - DL receptions within DL usable PRBs are allowed - UL transmissions outside UL usable PRBs are not allowed - DL receptions outside DL usable PRBs are not allowed - This restriction is not applicable for CLI measurement CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3. RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0 - DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

Determination of UL/DL usable PRBs

The following agreement was made in RAN1#116.

Agreement For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs. For determining UL/DL usable PRBs, consider the following options. - Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols. - Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

Companies’ views on the two options are summarized below.

- Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

  - Support: Huawei, Spreadtrum, ZTE, LGE, CT, vivo, CMCC, Samsung, Fujitsu, Ericsson, OPPO, xiaomi, Kookmin University, Nokia, ETRI (w/o cell-specific), DOCOMO, QC (if UL/DL usable PRBs are not explicitly configured within active UL/DL BWP), WILUS

- Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

  - Support: TCL, Sony, NEC, QC

Tx/Rx occasion mapped to SBFD and non-SBFD symbols within a slot

For a physical channel/signal occasion, except for a RO, mapped to SBFD and non-SBFD symbols within a slot if any,

Option 1: UE does not transmit or receive the physical channel/signal within the slot.

  - Support: Huawei (except for PUSCH repetition type B), Spreadtrum (except for PUSCH repetition type B), Tejas (for PUSCH repetition type B), LGE (except for PUSCH repetition type B), Ericsson (actual repetition involving different symbol types are invalidated), ZTE (except for PUSCH repetition type B), OPPO (error case for DG), NEC, QC (error case for DG), DOCOMO (except for PUSCH repetition type B)

Option 2: UE can transmit or receive the physical channel/signal within the slot only under certain conditions.

  - Support: TCL, vivo (if consistent configuration), IDC, Nokia, Sony

    - FDRA enhancements in SBFD symbols

Accroding to WID, enhancement on resource allocation in frequency domain in SBFD symbols include:

- resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols

- handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

FDRA RA type 1

The following agreement was made in RAN1#117 on potential enhancements on RA type 1 for PDSCH in a single slot scheduled at least by DCI format in USS.

Agreement For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4}, - Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only - FFS: DMRS sequence mapping - FFS: when PRG is determined as wideband - FFS: partial PRG

DMRS sequence mapping

Regarding the FFS DMRS sequence mapping, companies’ views are quite convergent.

- Legacy DMRS sequence mapping is reused. DMRS is mapped to assigned PRBs within DL usable PRBs only.

- Support: Huawei, Spreadtrum, Tejas, ZTE, CT, Ericsson, vivo, xiaomi, CATT, Samsung, QC, Nokia

According to current spec, a UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

Clause 5.1.4 (TS 38.214) A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

Qualcomm proposed that above restriction should be relaxed for SBFD-aware UEs. The invalid assigned PRBs for PDSCH should be considered not available for both PDSCH and DMRS. However, this relaxation should be valid only for narrow-band PRG. When PRG is wideband, the scheduled PRBs should be contiguous. Then, the UE is not expected to have invalid assigned PRB for PDSCH when PRG is wideband. In other words, the PDSCH allocation should be within one of the DL subband.

Spreadtrum proposed to make clarification for assigned PRBs for PDSCH outside of DL usable PRBs/UL usable PRBs are not assigned PRBs or “not available for PDSCH” and the above restriction is still kept.

PDSCH scheduled by DCI format in CSS

Sharp proposed that enhancement to RA type 1 for DCI format 1_0 scheduled in CSS is not pursued in Rel-19.

LGE proposed that the previous agreement for frequency domain resource allocation Type1 for PDSCH scheduled by DCI format in USS is applied to PUSCH RA type 1 scheduled by DCI format in USS and/or PDSCH/PUSCH RA type 1 scheduled by DCI format in CSS.

ZTE has the following proposal for PDSCH scheduled by DCI format in CSS.

For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled by DCI format in CSS, support the following:

- Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid.

  - Existing RB indexing and VRB-to-PRB mapping are reused.

  - Down-select one of the following options.

    - Option 1: The number of PRBs for TBS determination is based on the assigned PRBs as legacy. The invalid PRBs are punctured

    - Option 2: The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only. VRB-to-PRB mapping is always disabled and a UE does not expect that the PDSCH is scheduled out of the DL usable PRBs

- No impact on DMRS sequence generation.

- DMRS sequence mapped to the RBs outside the DL usable PRBs are punctured.

- When an assigned PRG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception.

RA type 1 with RBG granularity

Spreadtrum discussed RA type 1 with RBG granularity and proposed that for frequency resource allocation Type 1 with RBG for PDSCH or PUSCH in a single slot by DCI format 0_2/0_3/1_2/1_3 based scheduling (without repetition or TBoMS),

- When an allocated RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be assigned for PDSCH reception and only the PRBs within UL usable PRBs are considered to be assigned for PUSCH transmission.

- Allocated RBGs that fall outside DL usable PRBs are considered to be not assigned and should not be used for PDSCH resource mapping

ZTE had the following proposal for frequency resource allocation Type 1 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS),

- When an assigned RB group overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with an RB group for PDSCH which is fully outside DL usable PRBs or an RB group for PUSCH which is fully outside UL usable PRBs.

- The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

Note: the RB group is the indication granularity K or P of RA type 1 for PDSCH or PUSCH in TS 38.214.

FDRA RA type 0

The following agreements were made in RAN1#116bis and RAN1#117 meeting.

Agreement For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs. Agreement For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively

Introduction of additional partial RBG

To support partial RBG at the edge of the DL/UL usable PRBs within the UE DL/UL BWP, companies’ views on whether to introduce additional partial RBGs are summarized below.

- Introduce additional partial RBGs at the edge of the DL/UL usable PRBs within the UE DL/UL BWP

- Support: Huawei, QC

- Not support:

RBG size

RBG size for RA type 0 in SBFD symbols:

- Option 1: RBG size is determined based on the size of DL/UL BWP

  - Support: Huawei, Spreadtrum, ZTE, CT, Samsung, CMCC

- Option 2: RBG size is determined based on the size of DL/UL usable PRBs

  - Support: CMCC

PRG

Wideband PRG

For wideband PRG, the following agreements were made in Rel-18 SI.

Agreement If PRG is determined as wideband, study the following two options: - Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated - FFS: Precoding assumption within and across the two DL subbands - Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated The study should include the impact on UE complexity Agreement The following conclusion is to be captured in the TR If PRG is determined as wideband, better scheduling flexibility and higher DL data rate can be achieved if non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated. Compared to the case that PRG is determined as wideband and only contiguous frequency resources can be allocated, non-contiguous frequency resources across two DL subbands requires UE to handle two non- contiguous segments of contiguous RBs that may increase UE complexity for channel estimation.

The proposal to allow non-contiguous frequency resources across two DL subbands if PRG is determined as wideband (i.e. Option 1) was discussed in RAN1#116 without conclusion.

Companies’ views in the contributions submitted to this meeting are summarized below.

If PRG is determined as wideband,

- Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated

  - Support: Huawei, Spreadtrum, LGE, Samsung (required processing time can be reported as UE capability), vivo, CMCC, CATT, OPPO, CT (subject to UE capability), Sony, Transsion, NEC, Panasonic, Ericsson, Google, DOCOMO (prefer)

  - Precoding assumptions in two DL subbands

    - Can be different: Spreadtrum, vivo, Ericsson

    - Same: LGE, NEC, Google

- Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated

  - Support: ZTE, QC, DOCOMO (acceptable if strong concern on UE complexity increase)

- Ensure that the PRG is completely within one DL sub band. If not, disable VRB-PRB interleaving and continue mapping

  - Support: Tejas

For dynamic bundling indication when gNB configures two sets of bundles and first set is configured with two values ‘n2-wideband’ or ‘n4-wideband’, the condition for determining wideband or narrowband depends on number scheduled RBs with respect to half of the size of the BWP NBWPsize/2. Spreadtrum, OPPO and QC proposed to further study the conditions for wideband precoder determination when precoding bundling is determined dynamically in SBFD symbols. ZTE and xiaomi proposed that legacy rules on determination of PRG size, which is based on BWP size, are reused.

Partial PRG

For partial PRG, the following agreement was made in Rel-18 SI.

Conclusion For a PRG that overlaps with subband boundary, if the part of DL PRG inside the DL subband can be used, better scheduling flexibility and resource utilization can be achieved, however degraded channel estimation quality in the partial PRG is expected compared to a PRG due to limited RBs in the partial PRG. - Note: UE complexity could increase if this feature is supported

The proposal to allow using the part of the DL PRG inside the DL subband was discussed in RAN1#116 without conclusion.

Companies’ views in contributions submitted in this meeting are summarized below.

For a PRG that overlaps the subband boundary,

- The part of the DL PRG inside the DL subband can be used

  - Support: MediaTek, Spreadtrum, Huawei, ZTE, vivo (both), CMCC, OPPO, xiaomi, Ericsson, Nokia, DOCOMO, CT, CATT (in case the number of partial RBGs in SBFD symbols does not exceed two), Lenovo

- Avoid by gNB implementation (UE doesn’t expect partial PRG(s) other than the first and last PRG within the UE DL BWP)

  - Support: vivo (both), ASUSTeK, QC

- Do not perform VRB-PRB interleaving

  - Support: Tejas

CSI-RS resource

CSI-RS resource allocation across two DL subbands

The following agreement was made in RAN1#117.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

Confirm WA

- Confirm the working assumption

  - Support: Spreadtrum, ZTE, New H3C, CMCC, LGE (for CSI-RS used for NZP-CSI, CSI-IM, CSI-RS for tracking, FFS for CSI-RS for mobility), vivo, OPPO, Ericsson, DOCOMO, RUIJIE, NEC, Nokia

CSI processing timeline to process the CSI-RS across the two DL subbands

- No impact on CSI processing time

  - Support: Spreadtrum, Huawei, CMCC, Tejas, OPPO, Ericsson, Samsung, DOCOMO

- Extend existing CSI processing time

  - Support: MediaTek, Samsung, QC

    - Extend by X symbols: MediaTek

    - Scale the CSI processing timeline Z/Z’ by factor of 2: QC

- Increases the number of occupied CPU

  - Support: MediaTek, Samsung, ETRI

    - The CPUs count for computing the CSI report(s) is twice: MediaTek

CSI reporting subband

The following agreement was made in RAN1#117 for partial CSI reporting subband.

Agreement For a CSI reporting subband which overlaps with at least one PRB within DL usable PRBs and one or more PRBs outside DL usable PRBs, the CSI reporting subband includes PRB(s) within DL usable PRBs only.

OPPO proposed to further study whether a CSI reporting subband which fully overlaps with UL subband and/or guard band can associate to a subband CSI report.

DOCOMO, QC proposed that UE doesn’t expect to be configured for CSI reporting subband(s) that is completely outside the DL usable PRBs.

CEWit proposed CSI-Reportband skipping status of CSI reporting subband which does not overlap with DL usable PRBs is supported.

CSI reporting subband size

CSI reporting subband size in SBFD symbols:

- Option 1: CSI reporting subband size is determined based on the size of DL BWP

  - Support: Huawei, Spreadtrum, Samsung, CMCC

- Option 2: CSI reporting subband size is determined based on the size of DL usable PRBs.

  - Support: CT, Samsung, CMCC, CEWiT

    - Physical channels/signals across SBFD and non-SBFD symbols

Accroding to WID, enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols include:

- resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots

- CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

TX/RX across different slots framework

The following agreement was made in RAN1#117 meeting.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability

Configuration provision

- Method to provide configuration(s)

  - RRC

    - Support: Huawei, Spreadtrum, ZTE, LGE, OPPO, Fujitsu, Panasonic, Ericsson, Samsung, RUIJIE,

  - RRC + DCI

    - Support: CMCC, NEC?, QC (based on SRS resource sets indicator)?

- Granularity of configuration

    - Per UE:

      - Support: Spreadtrum, ZTE, LGE, OPPO, xiaomi, Panasonic, RUIJIE,

    - Per channel/signal:

      - Support: Huawei, Spreadtrum, vivo, HONOR, Samsung, Kookmin University, CEWiT

    - Per cell:

      - Support: DOCOMO

    - Per time domain type of across SBFD symbols and non-SBFD symbols:

      - Support: Spreadtrum

    - Per link direction:

      - Support: Panasonic

UE capability

- Different UE capabilities for support of Configuration 1 and Configuration 2

  - Support: Huawei

- Support of configuration 2 is subject to UE capability

  - Support: Spreadtrum, ZTE, Tejas, LGE, vivo, CATT, Ericsson, HONOR, Kookmin University, RUIJIE,

- Configuration 1 is the default configuration

  - Support: Ericsson, Samsung

Ericsson proposed that SBFD-aware UE operates by default in Configuration 1 during initial access and in RRC connected state, if the UE is not explicitly provided semi-statically.

Tx/Rx for Configuration 1

Determination of symbol type for configuration 1

For configuration 1, the transmissions/receptions are restricted to one symbol type. The following approaches were considered by companies to determine the applicable symbol type.

- Explicitly configured by RRC

  - Support: Spreadtrum, Samsung (for CG/SPS), CATT (for transmissions/receptions w/o scheduling or activation DCI), DOCOMO (For higher-layer configured transmission/reception), Sharp (for SPS PDSCH, type 1 CG)

- Explicitly indicated by DCI

  - Support: Samsung (for DG PUSCH repetition type A/PDSCH repetition and TBoMS),

- Implicitly determined by the symbol type of the first Tx/Rx occasion

  - Support: Samsung (for DG PUSCH repetition type A/PDSCH repetition and TBoMS), DOCOMO, CATT (for transmissions/receptions with scheduling or activation DCI)

- Up to gNB scheduling to ensure that the transmissions/receptions are within a single symbol type

  - Support: Samsung (for multi-PUSCH/multi-PDSCH scheduling)

Handling of Tx/Rx in the invalid symbol type for configuration 1

- SPS PDSCH

  - The SPS PDSCH reception occasions are invalid

    - Support: CATT, QC

- CG PUSCH

  - The CG PUSCH transmission occasions are invalid

    - Support: CATT, QC

- PUSCH available slot counting

  - The PUSCH transmissions in the invalid symbol type is postponed

    - Support: CATT, DOCOMO

- PUCCH repetition

  - The PUCCH transmissions in the invalid symbol type is postponed

    - Support: CATT, DOCOMO

- TBoMS

  - The PUSCH transmissions in the invalid symbol type is postponed

    - Support: CATT

CATT proposed to FFS SPS HARQ-ACK with defer in the invalid symbol type.

Tx/Rx for Configuration 2

The following agreements were made for PUSCH transmissions in SBFD symbols and non-SBFD symbols in RAN1#116bis meeting.

Agreement [...] For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols - FFS type 2 CG PUSCH - FFS other separate configurations for SBFD symbols and non-SBFD symbols - Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid - Option 4: Only CG PUSCH transmission occasion in one symbol type is valid and CG PUSCH transmission occasion in the other symbol type is invalid - Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid for CG PUSCH - Other options are not precluded Agreement For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols, and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols - Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed - Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid - Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid - Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs - Other options are not precluded - FFS: Applicable conditions

Companies’ views on the above options are summarized below.

CG PUSCH

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols

  - Support: TCL, ZTE (for type 1 CG PUSCH),  CT (for type 1 CG PUSCH),  New H3C, Sony, NEC (down-select from Option 1 and 2), OPPO (for type 1 CG PUSCH), Lenovo, Nokia (for type 1 CG), CEWiT

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

  - Support: MediaTek (for CG PUSCH w/ repetition), TCL, ZTE (for type 2 CG PUSCH), CT (for type 2 CG PUSCH), OPPO (for type 2 CG PUSCH), Ericsson, vivo, IDC,  Sony, NEC (down-select from Option 1 and 2)

- Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid

  - Support: Spreadtrum, LGE (down-selection from Option 3/5), Samsung (if FH is not enabled), vivo, xiaomi, CATT, NEC, Nokia (default if no other option configured), WILUS

- Option 4: Only CG PUSCH transmission occasion in one symbol type is valid and CG PUSCH transmission occasion in the other symbol type is invalid

- Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid for CG PUSCH

  - Support: LGE (down-selection from Option 3/5), OPPO (for type 2 CG PUSCH),  New H3C, vivo, Kookmin University, CATT, IDC, DOCOMO,

PUSCH repetition type-A

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

  - Support: New H3C, TCL, Sony, NEC, Lenovo, Nokia (CG)

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

  - Support: MediaTek, TCL, ZTE, CT, Ericsson, vivo, Sony, NEC, OPPO, HONOR, Nokia, IDC, QC

- Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed

  - Support: Huawei (Option 3 or 5 depending on the number of assigned RBs outside usable PRBs and consider other options combined with these two options),  Spreadtrum, Samsung (if FH is not enabled), vivo, CATT, Panasonic, xiaomi, Nokia (default if no other option configured), WILUS

- Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid

- Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid

  - Support: Huawei (Option 3 or 5 depending on the number of assigned RBs outside usable PRBs and consider other options combined with these two options), LGE, CT, New H3C, vivo, OPPO, Kookmin University, CATT, DOCOMO, Nokia, IDC, CEWiT

- Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs

  - Support: LGE

  - Not support:

Multi-PUSCH scheduled by a single DCI

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

  - Support: New H3C, Sony

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

  - Support: ZTE, vivo, OPPO, CT, Ericsson,  IDC, Sony, Nokia, QC

- Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed

  - Support: Huawei (Option 3 or 5 depending on the number of assigned RBs outside usable PRBs and consider other options combined with these two options), Spreadtrum, vivo, Samsung (if FH is not enabled), xiaomi, CATT, Nokia (default if no other option configured, , WILUS

- Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid

- Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid

  - Support: Huawei (Option 3 or 5 depending on the number of assigned RBs outside usable PRBs and consider other options combined with these two options), Spreadtrum, New H3C, vivo, OPPO, CT, CATT, Kookmin University, IDC, DOCOMO, Nokia, CEWiT

- Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs

  - Support: LGE, QC

TBoMS

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

  - Support: Lenovo, New H3C,

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

  - Support: ZTE,  CT, vivo, Nokia, IDC, QC

- Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed

  - Support: vivo, Spreadtrum, Samsung (if FH is not enabled), xiaomi, CATT, Nokia (default if no other option configured), WILUS

- Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid

- Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid

  - Support: New H3C, CT, LGE, vivo, DOCOMO, Nokia, IDC

- Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs

  - Support:

  - Not support:

The following agreements were made for PDSCH receptions across SBFD and non-SBFD symbols in RAN1#116bis meeting.

Agreement For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate resource allocations for SBFD symbols and non-SBFD symbols - FFS other separate configurations for SBFD symbols and non-SBFD symbols - Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid - Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid for SPS PDSCH - Other options are not precluded [...] Agreement For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols - Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid, e.g. the PDSCH in the slot is dropped - Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid - Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid - Option 6: gNB does not schedule any PDSCH in SBFD symbols in a slot to be overlapping with PRBs outside DL usable PRBs - Other options are not precluded - FFS: Applicable conditions

Companies’ views on the above options are summarized below.

SPS PDSCH

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate resource allocations for SBFD symbols and non-SBFD symbols

  - Support: New H3C, TCL, Sony, Kookmin University, Nokia, CEWiT

  - Not support:

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

  - Support: TCL, ZTE, CT, IDC, Sony

  - Not support:

- Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid

  - Support: Spreadtrum, LGE (down-selection from Option 3/5), CATT, Samsung, vivo, xiaomi, Nokia (default if no other option configured), WILUS

- Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid for SPS PDSCH

  - Support: New H3C, LGE (down-selection from Option 3/5), Ericsson, CATT, vivo, DOCOMO, OPPO, Kookmin University, IDC

PDSCH repetitions

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

  - Support: New H3C, TCL, Sony, NEC

  - Not support:

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

  - Support: TCL, ZTE, CMCC, Nokia, IDC, Sony, NEC

  - Not support:

- Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid, e.g. the PDSCH in the slot is dropped

  - Support: MediaTek, Huawei (Option 3 or 5 depending on the number of assigned RBs outside usable PRBs and consider other options combined with these two options), Spreadtrum, CATT, Samsung, vivo, Panasonic, xiaomi, Nokia (default if no other option configured), WILUS

- Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid

- Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid

  - Support: Huawei (Option 3 or 5 depending on the number of assigned RBs outside usable PRBs and consider other options combined with these two options), LGE, New H3C, vivo, CATT, OPPO, Kookmin University, Nokia, DOCOMO, Nokia, IDC, Lenovo, QC, CEWiT

- Option 6: gNB does not schedule any PDSCH in SBFD symbols in a slot to be overlapping with PRBs outside DL usable PRBs

  - Support: QC, LGE

  - Not support:

Multi-PDSCH scheduled by a single DCI

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

  - Support: New H3C, Sony

  - Not support:

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

  - Support: ZTE, Nokia, IDC, Sony

  - Not support:

- Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid, e.g. the PDSCH in the slot is dropped

  - Support: Huawei (Option 3 or 5 depending on the number of assigned RBs outside usable PRBs and consider other options combined with these two options), Spreadtrum, Samsung, vivo, CATT, Nokia (default if no other option configured), WILUS

  - Not Support:

- Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid

- Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid

  - Support: Huawei (Option 3 or 5 depending on the number of assigned RBs outside usable PRBs and consider other options combined with these two options), Spreadtrum, New H3C, vivo, Kookmin University, Nokia,  CATT, DOCOMO, IDC, Lenovo, QC (for configuration #2), CEWiT

- Option 6: gNB does not schedule any PDSCH in SBFD symbols in a slot to be overlapping with PRBs outside DL usable PRBs

  - Support: LGE, QC (for configuration #1)

  - Not support:

Separate UL power control

The following agreement was made in RAN1#117 meeting.

Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework.

Framework

Companies’ views on the framework for separate UL power control for FR2 are summarized below.

- UL spatial relation Info framework only

  - Support:

- Unified TCI state framework only

  - Support: Spreadtrum, ZTE, CMCC, TCL, LGE, CATT, Samsung (prioritize), Nokia, IDC, DOCOMO (prioritize), RUIJIE, QC

- Both UL spatial relation Info framework and unified TCI state framework

  - Support: Huawei, vivo, Fujitsu

Separate configuration of power control parameters

- Separate configuration of power control parameters for SBFD and non-SBFD symbols

  - Support: TCL, Fujitsu, CATT, Panasonic, Lenovo, Samsung (Separate DL pathloss RS configuration, separate open-loop power control parameters (alpha, P0), and associated PHR reporting), QC

- Same power control parameters for SBFD and non-SBFD symbols with a power offset for transmissions in SBFD symbols

  - Support: Ericsson

To support separate PC parameters for SBFD and non-SBFD symbols, the following options are considered by companies.

- Option 1: One TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively

  - Support: CATT

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - Support: CATT, DOCOMO, QC

| Option 1 | Option 2 |
| --- | --- |

Figure 3: Separate UL power control parameters based on unified TCI framework

Based on unified TCI framework, ul-powerControl is configured in TCI-State or TCI-UL-State, which configures a Uplink-powerControlId. In this way, P0 and alpha are configured for PUSCH, PUCCH and SRS respectively.

- Uplink-PowerControl The IE Uplink-PowerControl is used to configure UE specific power control parameter for PUSCH, PUCCH and SRS. Uplink-PowerControl information element -- ASN1START -- TAG-UPLINK-POWERCONTROL-START Uplink-powerControl-r17 ::= SEQUENCE { ul-powercontrolId-r17 Uplink-powerControlId-r17, p0AlphaSetforPUSCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforPUCCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforSRS-r17 P0AlphaSet-r17 OPTIONAL -- Need R } P0AlphaSet-r17 ::= SEQUENCE { p0-r17 INTEGER (-16..15) OPTIONAL, -- Need R alpha-r17 Alpha OPTIONAL, -- Need S closedLoopIndex-r17 ENUMERATED { i0, i1 } } Uplink-powerControlId-r17 ::= INTEGER(1.. maxUL-TCI-r17) -- TAG-UPLINK-POWERCONTROL-STOP -- ASN1STOP

Close loop power control

- Separate TPC commands for uplink transmission

  - Support: Fujitsu, QC

- Separate closed loop accumulation

  - Support: Fujitsu, IDC, QC

- Same closed loop power control process for transmissions across SBFD symbols and non-SBFD symbols

  - Support: Ericsson

Samsung proposed that TPC field in a DCI scheduling PUSCH is applied to the corresponding symbol type.

Multi-TRP

Joint operation of SBFD and mTRP

- Support: Nokia

- Deprioritize: Spreadtrum

- FFS: ZTE

Separate TCI states

Separate beam/spatial relations

Separate spatial relations for SBFD and non-SBFD symbols for single-TRP scenario

- Support: Spreadtrum, Fujitsu (UL and DL), Samsung, Sony, QC

- Not support: Ericsson

PUSCH Frequency Hopping

The following agreement was made in RAN1#117 meeting.

Agreement Support separate FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols respectively. - FFS: How to indicate/determine the FH offsets for PUSCH transmissions in SBFD and non-SBFD symbols, respectively. - FFS: Whether/how to update FH calculation to only consider the UL usable PRBs

Separate configuration of FH offsets

- Separate configurations of FH offsets for SBFD symbols and non-SBFD symbols

- Support: Huawei, ZTE, Tejas, CMCC, LGE, vivo, OPPO, CATT, Ericsson, Samsung, Sharp, DOCOMO, QC, WILUS

- Not support:

HONOR proposed to support indicating by one FH offset and one offset ∆ in DCI, which means the second FH offset is obtain by the first FH offset plus ∆.

Bit width of frequency offset indicator

- Maximum number of values in two frequencyHoppingOffsetLists

- Support: CMCC

- The number of FH offsets for SBFD symbols does not exceed the number of FH offsets for non-SBFD symbols

- Support: CATT

- UE expects that the number of FH offsets for PUSCH transmission in SBFD symbol is the same as the number of FH offset for PUSCH transmission in non-SBFD symbol

- Support: Samsung

FH calculation

- Update PUSCH frequency hopping pattern/formula by replacing UL BWP size by the number of UL usable PRBs

- Support: MediaTek, Huawei, Spreadtrum, ZTE, Tejas, CMCC, vivo, CATT, Ericsson, Samsung, DOCOMO, QC, WILUS

- Not support:

- Modify the RBstart calculation based on NUL.,usablePRBsize rather than NBWPsize

- Support: Tejas, NEC

Enable/disable FH in SBFD symbols

Tejas proposed to consider the following options for frequency hopping in SBFD slots

- Option 1:  Frequency hopping not allowed

- Option 2:  Allow wrapping of the resource allocation around NUL.,usablePRBsize

- Option 3: Based on a rule enable /disable frequency hopping,

- Possible Rule: If the ratio of  LRB to NUL.,usablePRBsize is more than 0.5, then disable frequency hopping.

CATT proposed that in case in case FH offsets for PUSCH transmissions in SBFD symbols are not configured, FH for PUSCH transmissions in SBFD symbols is disabled.

PUCCH

Separate configurations for SBFD and non-SBFD symbols

In general, companies support separate configurations of PUCCH resources for SBFD and non-SBFD symbols. Companies’ views on how to achieve separate configurations are summarized below.

- Option 1: Support separate configurations of PUCCH-Config for SBFD symbols and non-SBFD symbols.

Support: CMCC, vivo, Kookmin University

- Option 2: Under PUCCH-Config, support separate PUCCH resource sets for SBFD symbols and non-SBFD symbols.

Support: Huawei, QC (for configuration #1)

- Option 3: For a certain PUCCH resource set, support separate PUCCH resources for SBFD symbols and non-SBFD symbols.

Support:

- Option 4: For a certain PUCCH resource, support separate configurations of parameters including at least startingPRB and secondHopPRB.

Support: ZTE, Ericsson, LGE, vivo, CATT, Samsung, QC (for configuration #1), WILUS

- Option 5: No separate configurations of PUCCH resource sets for SBFD symbols and non-SBFD symbols. PUCCH resource in SBFD symbols are derived based on RB offset(s) configuration/indication/determination.

Support: QC (for configuration #2)

SRS

The following agreement was made in RAN1#116bis meeting.

Agreement Study the feasibility and enhancements to support separate power control and/or spatial relation for SRS, PUCCH and PUSCH in SBFD and non-SBFD symbols in different slots, including repetition and non-repetition, by considering existing schemes, e.g. multi-TRP PUCCH/PUSCH repetition schemes.

Separate SRS resource set

In current specification, a UE can be configured with a list of SRS resource sets.

Each SRS resource set configuration includes:

- One or multiple SRS resources

Time/frequency resource and spatial relation are separately configured for each SRS resource.

- usage (‘beamManagement’, ‘codebook’, ‘nonCodebook’, ‘antennaSwitching’)

- power control parameters

In Rel-17, multi-TRP PUSCH repetition is supported with up to two SRS resource sets with usage set to ‘codebook’ or ‘nonCodebook’.

Companies’ views on support of two SRS resource set configurations for SBFD symbols and non-SBFD symbols are summarized below.

- Separate configurations of SRS resource sets for SBFD symbols and non-SBFD symbols respectively

- Support: Huawei, Tejas, New H3C, CATT, Transsion, QC(with usage set to ‘codebook’ or ‘nonCodebook’)

- Not support:

- Separate SRS-config for SBFD symbols and non-SBFD symbols respectively

- Support: CMCC

Qualcomm proposed that PUSCH transmission in SBFD or non-SBFD symbols should be associated with the most recent SRS transmission of same duplex type.

A-SRS available slot counting

Qualcomm proposed to further discuss the available slot counting for AP-SRS transmission with SBFD operation given that the couting mechanism excludes DL symbols and the availability for UL/FL slots is determined only based on time availability.

Figure 5: Available slot for AP-SRS set [41]

CSI measurements and reporting

The following agreement was agreed in RAN1#117.

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

Companies’ views on the options are summarized below.

- Option A

  - Support: MediaTek, Huawei, Spreadtrum, ZTE, Tejas, New H3C, vivo, CATT, Ericsson, LGE, Samsung, Sony, Lenovo, ETRI, DOCOMO, NEC, Apple, Nokia, QC, WILUS

  - Not support: xiaomi

- Option B

  - Support:  Spreadtrum, CMCC, LGE, vivo, Fujitsu, CATT, Samsung, Sony, NEC

    - less signalling configuration overhead and the port indication framework is also useful to adapt the TX/RX antenna architecture [CMCC]

  - Not support: Huawei, xiaomi

    - Larger impact on existing CSI reporting framework [Huawei]

    - Impact on UE implementation [Huawei]

    - Not clear how to support NES and SBFD operations simultaneously [Huawei]

ETRI proposed to clarify that for a CSI report, a UE shall only consider the CMR and/or IMR, which shares the same duplex mode with the CSI reference resource.

- For a CSI report with CSI reference resource in SBFD symbols, UE does not take the CMR and/or IMR into account for CSI derivation, if the CMR and/or IMR is on the non-SBFD symbols

- For a CSI report with CSI reference resource in non-SBFD symbols, UE does not take the CMR and/or IMR into account for CSI derivation, if the CMR and/or IMR is on the SBFD symbols

  - Collision handling

    - Link direction determination

In RAN1#116 meeting, two options were agreed to be considered to determine link direction.

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded.

Companies’ views on support of explicit link direction indication are summarized below.

- Support of explicit link direction indication

- Support: MediaTek, Spreadtrum, CMCC, vivo, Fujitsu (at least for Case3), CATT, Sony, Nokia, ITRI

  - Semi-static indication

    - Support: MediaTek, CMCC, Sony, Nokia (for semi-static UL vs. semi-static DL),

    - Not support:

  - Dynamic indication

    - Support: CMCC, Nokia (link direction for upcoming slots)

    - Not support:

- Not support: OPPO, CT, Ericsson, Samsung, WILUS

    - Collision handling Cases

The following agreement was made in RAN1#116 meeting.

Agreement For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

The following agreement was made in RAN1#116bis meeting.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met. - The above does not imply link direction indication is supported - FFS on dynamically scheduled DL reception with repetition

For dynamically scheduled DL reception with repetition vs. semi-staically configured UL transmission, companies’ views are summarized below.

Same handling rule as dynamic DL w/o repetition vs. semi-static UL i.e. UL transmission is cancelled if cancellation timeline is met.

  - Support: Huawei, CT, Spreadtrum, ZTE, Ericsson, CATT, Samsung, Kookmin University DOCOMO, Nokia, QC, WILUS

Support UL CG override DL DG reception in some conditions: CMCC, NEC

    - Option 1: gNB configures UE for which CG configuration, the UL CG can override DL DG with repetition: CMCC

    - Option 2: UL CG with higher priority can override DL DG with repetition with lower priority: CMCC, NEC

Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

The following agreement was made in RAN1#116bis meeting.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

For semi-staically configured DL reception with repetition vs. dynamically scheduled UL transmission:

Same handling rule as semi-static DL vs. dynamic UL w/o repetition

  - Support: Huawei, CT, Spreadtrum, ZTE, CATT, Ericsson, Samsung, Kookmin University, DOCOMO, QC, WILUS

Support DL SPS override UL DG repetition in some conditions: CMCC

    - Option 1: gNB configures UE for which SPS configuration, the DL SPS can override UL DG with repetition.

    - Option 2: DL SPS with higher priority can override UL DG with repetition with lower priority.

Prioritize DL transmission over UL repetition when UE decides to skip the given UL repetition occasion

  - Support: NEC

The gNB may indicate one or more slots per TDD UL-DL pattern that shall not be considered available for the UE for UL transmission

  - Support: Nokia

Dynamic signaling in DCI can indicate whether DL reception should be prioritized over a repetition of a UL transmission in upcoming slots

  - Support: Nokia

Collision handling rule can be defined on some slots during the UL repetition which should be used for DL PDCCH reception for the scheduling of UL in UL/SBFD slots after the ongoing UL repetitions

  - Support: Nokia

Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

The following agreement was made in RAN1#117 meeting.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases

There is an FFS in the above agreement on additional cases.

CATT identified that Type-0B CSS set is missing, which was introduced in Rel-17 for MBS. It is included for Redcap HD-UE in TS38.213 Clause 17.2 as highlighted in pink.

A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE does not expect to receive both a Type-0/0A/0B/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except a Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring configured-grant based PUSCH transmission as described in clause 19.1 in the set of symbols for which case the UE follows the procedure as in clause 5.1B.2.6 in [10, TS 38.133]. The UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission as described in clause 19.1 per SI modification period.

ETRI proposed to add Type-0B/1A/2A/3 to the above agreements.

Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

The following agreement was made in RAN1#117 meeting.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 4 (dynamically scheduled DL reception vs. dynamically scheduled UL transmission) in a SBFD symbol for SBFD-aware UEs, it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions. - FFS DL reception with repetitions and/or UL transmission with repetitions

For DL reception with repetitions and/or UL transmission with repetitions, companies’ views are summarized below.

Same handling rule as dynamic DL w/o repetition vs. dynamic UL w/o repetition

  - Support: Huawei, Spreadtrum, ZTE, Tejas, vivo, CT, Ericsson, Kookmin University, CATT, DOCOMO, WILUS

Dynamically scheduled DL reception (or UL transmission) without repetition always override dynamically scheduled UL transmission (or DL reception) with repetition

  - Support: CMCC

Dynamically scheduled DL reception (or UL transmission) without repetition with higher priority can override dynamically scheduled UL transmission (or DL reception) with repetition with lower priority

  - Support: CMCC

Leave to UE implementation

  - Support: HONOR

Case 5: SSB vs. dynamically scheduled or configured UL transmission

The following agreement was made in RAN1#117 meeting.

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

SSB for RRM

Ericsson proposed to clarify that the above agreement only covers the case of SSBs transmitted by the serving cell and leave the discussion on scheduling availability due to RRM measurements to RAN4.

Nokia proposed that for the collision of SSB vs configured or dynamically scheduled UL transmission, any UL restriction should take into account RAN4 RRM requirements for SSB reception: e.g. for intra-frequency measurements, UL transmissions not expected on SSB symbols as well as 1 symbol before and after SSB symbols.

Whether/how SSB symbols fall back to full DL symbols

Companies’ views on the FFS are summarized below.

          - Option 1: A slot consisting of SSB symbols is considered as a full DL slot

  - Support: MediaTek, ZTE, CATT, Samsung (not applied if the number of transition points with a TDD UL/DL pattern period gets larger than two), Nokia (based on gNB configuration)

    - Maximum two transition points with a TDD UL-DL pattern period: MTK, SS

          - Option 2: UL subband in SSB symbols are reserved

  - Support: Spreadtrum, New H3C, Tejas, CMCC, vivo, OPPO, CATT, Ericsson, HONOR

          - Option 1 + Option 2:

  - Support: Huawei (if the number of SSB symbols within a slot exceeds a threshold, Option 1 is enabled, otherwise, Option 2-2 is enabled), CT (Consider the existence and length of the guard period between SBFD and non-SBFD symbols), LGE (depending on SBFD symbol location)

    - Option 2-2: SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. UL transmissions are allowed within the UL usable PRBs not overlapping with SSB symbols.

    - When the SSB is located in an SBFD symbol within a slot that includes an SBFD symbol following a non-SBFD symbol, a slot consisting of SSB symbols is considered as a full DL slot; When the SSB is located within an SBFD symbol among a sequence of SBFD symbols, SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs: LGE

          - Option 3: The SSB symbol is considered as full DL symbol

  - Support: DOCOMO, Sharp

MediaTek proposed that within a half radio frame containing SSBs, all SBFD slots up to a “reference slot” are considered as full downlink slots. The reference slot overlaps with the slot containing the last SSB within the half frame.

Figure 6: Convert SBFD slots before reference slot to full DL slots [8]

Case 6: Dynamic or semi-static DL vs. valid RO

ZTE proposed to reuse the existing collision handling principles and timeline that are defined for operation in flexible symbols on a single carrier in unpaired spectrum. However, UE behaviour is not clear as discussed before in RAN1#104b-e [6].

For RedCap HD-FDD UE, it was agreed that PRACH triggered by PDCCH order is considered a dynamically scheduled UL transmission. And it was agreed to leave it to UE implementation for collision between DL receptions and PRACH transmission triggered by higher layer.

Agreements: [38.213] - For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreements: [38.213] - For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), leave it to UE implementation whether to receive the DL or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreements: [38.213] - For Case 8 of valid RO overlapping with dynamically scheduled DL reception, leave it to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH

The related UE behaviour defined in TS38.213 Clause 17.2 is copied below.

A HD-UE does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols. …… If a HD-UE is configured by higher layers to receive a PDCCH, or PDSCH, or CSI-RS, or DL PRS in a set of symbols, the HD-UE receives the PDCCH, or PDSCH, or CSI-RS, or DL PRS if the HD-UE does not detect a DCI format that indicates to the HD-UE to transmit a PUSCH, or PUCCH, or PRACH, or SRS in at least one symbol of the set of symbols; otherwise, the HD-UE does not receive the PDCCH, or PDSCH, or CSI-RS, or DL PRS in the set of symbols. …… If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks within the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the PL RS, or the PDCCH, or the SS/PBCH blocks.

For collision Case 6, companies’ views are summarized below.

Re-use the existing collision handling principles for HD-FDD RedCap UE

  - Support: CMCC, Kookmin University, CATT, Panasonic

UE prioritizes valid ROs when it would transmit PRACH, otherwise it receives the DL

  - Support: Ericsson, NEC, DOCOMO, WILUS

- Reuse the existing collision handling principles and timeline that are defined for operation in flexible symbols on a single carrier in unpaired spectrum

  - Support: ZTE

Up to UE implementation

  - Support: Spreadtrum, Samsung

UE does not receive DL transmission which overlaps with valid RO in the SBFD symbol

  - Support:OPPO

Drop Case 6, PRACH is included in other cases

  - Support: QC

Hold on discussion till RO validity rule is defined in AI 9.3.2

  - Support: vivo

Others

ZTE proposed to discuss collision between RSSI/RRM measurement resource and UL transmission.

Collision handling order

CATT proposed to study the order of collision handling of the following collision types.

- Type 1: Collision between transmissions/receptions and unusable time/frequency resources;

- Type 2: Collision between transmissions with same transmission direction;

- Type 3: Collision between transmissions and receptions;

- Type 4: Collision between transmissions/receptions and the link direction indicated by gNB (if supported).

CMCC proposed to consider the following collision handling order as a starting point.

- Step 1: drop transmissions/receptions without available resources to be allocated.

- Step 2: if multiple UL transmissions are overlapped in time domain, then follow the legacy rules to solve intra-UL-direction collision.

- Step 3: drop transmissions/receptions which contradicts to symbol direction based on legacy TDD configuration and UE-specific link direction of the SBFD symbol.

- Step 4: if there are remaining collision issues after Step 3, e.g., some transmissions/receptions in flexible symbols are overlapped in time domain, drop transmissions or receptions if they are overlapped in time domain based on certain rules.

OPPO proposed that case 5/6 is handled first and then collision 1-4 is handled.

- Discussions during the meeting

  - [Closed] 1st round discussion

    - SBFD subband indication

Proposal 1-1

Proposed Agreement:

For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are configured.

- A starting slot index

- A starting symbol index within the starting slot

- An ending slot index

- An ending symbol index within the ending slot

|  | Company |
| --- | --- |
| Support | New H3C, Spreadtrum, Xiaomi, CEWiT, Tejas, Sony, IDC, Samsung, Nokia, NSB (with update), MediaTek, CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Separate indication of starting SBFD symbol and ending SBFD symbol is supported by the most companies. For maximum 320 slots within a TDD-UL-DL pattern period, it requires 26 bits (log2320+log214+log2320+log214). The overhead can be reduced by 2 bits via joint indication of starting symbol and length via SLIV-based indication (log2320×14×(320×14+1)2). However, it is moderator’s understanding that SLIV in TS38.214 is defined assuming L=14. For SLIV-based indication with L other than 14, impact on TS 38.214 is expected. Therefore, it is proposed to separately configure the slot and symbol indexes for starting and ending SBFD symbols respectively as proposed above. |
| ZTE | Prefer SLIV based indication for less overhead. Could FL clarify more for your understanding about ‘For SLIV-based indication with L other than 14, impact on TS 38.214 is expected.’ In our view, 214 spec may only need to refer to the RRC parameters in 331 for time domain location of SBFD symbols, regardless of how to define the RRC parameters. |
| Moderator | Thanks ZTE for the question. The SLIV defined in current 38.214 is as follows which assumes L=14. If we use SLIV-based indication to indication the starting symbol and length of SBFD symbols within a TDD-UL-DL pattern period, L should be 320*14, which requires new definition of SLIV in 38.214. Companies are welcome to check whether it is correct understanding. if then else where, and. |
| Sharp | We also prefer SLIV-based (or RIV-based). The SLIV coding can be updated by replacing ‘7’ to Nsym/2 and ‘14’ to Nsym where Nsym represents the number of OFDM symbols of the TDD-UL-DL pattern period. In another method, we can reuse RIV by replacing LRBs to L, NBWPsize to Nsym, RBstart to S, and LRBs to L. if then else where 1 and shall not exceed .. |
| Tejas | Support the proposal in general. As we have already agreed that there will be 2 switching points in a TDD configuration, can the end symbols and end slots can be optional? |
| QC | There are two approaches - Alt 1) Start slot/symbols + Last slot/symbol - Alt 2) Start slot/symbols + Lengths in slots + symbol for last slot or SLIV-like We can list the two alternatives and it is up to RAN2 to select which options. |
| ITRI | We also prefer SLIV-based |
| IDC | Support in principle based on the suggested parameters for explicitly indicating slot/symbol indexes. |
| Google | Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. In RAN1 #117 we have an agreement above, and we think the proposal is not aligned with the agreement with 2 TDD patterns. |
| LGE | Borrowing from the configuration of TDD-UL-DL-Pattern in TDD-UL-DL-ConfigCommon, we prefer to support separate indication of starting/ending slot index and starting/ending symbol index as the proposed agreement. More specifically, we propose to configure slot/symbol offset instead of slot/symbol index. The reference position of the starting slot offset can be the starting symbol of the starting symbol of the TDD-UL-DL pattern period and the reference position of the ending slot offset can be the last symbol of the TDD-UL-DL pattern period |
| Samsung | We are fine with the proposal. In addtion, we are also fine with the SLIV-basesd indication. Our understanding is in the SLIV equation, 14 and 7 can be replaced with 320*14 and 320*14/2, respectively. |
| OPPO | Similar view with QC. |
| DOCOMO | We are fine with the proposal. But we feel the required bits for the indication of “starting symbol index + duration” is same as the method in the proposal: Number of bits for “starting symbol index” is log2(320*14)=log2320+log214 bits. Number of bits for “duration” is log2(320*14)=log2320+log214 bits. The sum is equal. |
| Nokia, NSB | Starting slot and starting symbol will always needed. Then for the ending slot/symbol, there may be cases to be considered. If SBFD symbol only last for a short duration, and there are DL symbol/slots after the SBFD symbol ends, then there need an indication on when SBFD symbol/slots will end, which can be indication of SBFD symbol length or ending of SBFD symbol. If SBFD symbol last until the end of DL in the TDD pattern, there will be no need to indicate any of SBFD symbol length or ending of SBFD symbol. Thus, we think it should be updated as For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are configured. - A starting slot index - A starting symbol index within the starting slot - Optional: An ending slot index - Optional: An ending symbol index within the ending slot |

Proposal 1-2

Proposed Agreement:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList, down-select from the following options:

- Option 1: starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275

  - One or two SBFD DL subbands can be configured

- Option 2: an ending RB index of the lower DL subband and a starting RB index of the upper DL subband can be configured assuming NBWPsize=275

  - The starting RB of the lower DL subband is the first RB of the carrier

  - The ending RB of the upper DL subband is the last RB of the carrier

  - If ending RB index is not configured, no lower DL subband is configured

  - If starting RB index is not configured, no upper DL subband is configured

|  | Company |
| --- | --- |
| Support | ZTE，New H3C, Xiaomi, CEWiT, Tejas, Sony, QC, ITRI, IDC, DOCOMO (clarification on SCS for UL, and SCS for DL), ZTE, Nokia, NSB (need update), CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | The proposal corresponds to Approaches a and b illustrated in Figure 1. Compared with approach b, the advantage of approach c is not clear. For the proposal to indicate the number of RBs as a percentage value of total number of RBs, it is not a typical signalling design and requires additional spec efforts. For the proposal from Google, it is more like indicating guardband, which was not agreed. |
| ZTE | Prefer Option 1. As analyzed in our contribution, one way that NW may choose for multiplexing of legacy UEs and new UEs is via FDMed manner. That is, configuring SBFD only in part of carrier BW and legacy UEs can be scheduled in the rest part of the carrier BW. This is a cleaner multiplexing way at least for UEs in RRC connected modes. The restriction in Option 2 is not necessary. |
| Spreadtrum | We support the sub bullet in Option 2. It can save signaling overhead. But for main bullet in Option 2, we prefer to configure subband sizes for two DL subbands. It has same signaling overhead compared to ending RB index or starting RB index, as such the following: Option 2: an ending RB index of the lower DL subband and a starting RB index of the upper DL subband, or one/two subband sizes for DL subband can be configured assuming NBWPsize=275. For ZTE’s comments, we share a different view that the size of DL subband and UL subband are for gNB, not necessary from UE’s perspective. One SBFD aware UE and another SBFD aware UE/ legacy UE can be scheduled by FDM manner. |
| Xiaomi | Slightly prefer option 2 for less overhead. For option 2, the mechanism of how to indicate the ending PRB index for lower DL subband and starting PRB index of upper DL subband need to further clarified. From our understanding, there are at least two solutions: - Indicate the absolute PRB index within carrier directly - Indicate the relevant PRB index with taking the first PRB or last PRB of UL subband as reference, i.e., indicate an offset to UL subband boundary. |
| Sharp | For subbands with subcarrier spacing different than the initial BWP, cell-specific signaling is not necessary. Therefore, SIB1 overhead for configuring multiple subbands in multiple numerologies are not an issue. Therefore, Option 1. |
| CEWiT | Support option 1, as it is simple and straight forward for different scenarios. |
| Tejas | We prefer either option 1 or Approach C. Approach C is a traditional approach of representing uplink and downlink sub bands. Representing all sub bands using RIV is also OK as decoding logic for RIV is same across sub bands. |
| QC | Option 2 is restrictive and not future-compatible for partial/fully overlapping SBFD operation. |
| IDC | Support such explicit indication approach based on Option 2 which is clear and sufficient. |
| Google | For this proposal, we should check companies’ understanding on whether overhead reduction is necessary or not. If the common understanding that there is no need for overhead reduction, we can support Option 1 for simplicity, otherwise we prefer our approach. @Moderator: We don’t agree your conclusion regarding our proposal. The proposal itself is nothing to do with the guard band, and it does not preclude the case where the base station can have additional guard band configurations. If overhead reduction is relevant to this feature, our proposal needs only 26 bits (smaller than all approaches listed by Moderator) to indicate 3 subbands, thereby it should be took into consideration. |
| LGE | If RIV is applied to configuration of frequency location of SBFD UL subband, we prefer Option 1 for configuration of frequency location of SBFD DL subband(s) for signaling commonality between UL and DL subbands. |
| Samsung | We support Option 2. We think UE-specific BWP can provide FDMed multiplexing. |
| OPPO | Fine with the proposal and for the clarification mentioned by xiaomi, we prefer to indicate the absolute PRB index within DL carrier directly since UL subband is configured within UL SCS-SpecificCarrierList, DL subband is configured within DL SCS-SpecificCarrierList and in such a case, the SCS of UL carrier and DL carrier may be not the same. |
| DOCOMO | We are fine with the indication methods for UL and DL subbands. But we prefer to clarify the “each SCS configuration in SCS-SpecificCarrierList”. As there are SCS-SpecificCarrierList for DL and UL respectively, it is preferred to clarify that SBFD UL subband is configured for each SCS configuration in SCS-SpecificCarrierList for UL, and DL subband(s) is/are configured for each SCS configuration in SCS-SpecificCarrierList for DL. Following updates are suggested. Proposed Agreement: For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, down-select from the following options: - Option 1: starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275 - One or two SBFD DL subbands can be configured - Option 2: an ending RB index of the lower DL subband and a starting RB index of the upper DL subband can be configured assuming NBWPsize=275 - The starting RB of the lower DL subband is the first RB of the carrier - The ending RB of the upper DL subband is the last RB of the carrier - If ending RB index is not configured, no lower DL subband is configured - If starting RB index is not configured, no upper DL subband is configured |
| Nokia, NSB | As the carrier starting position and length is clear, when one subband is configured, e.g. UL subband is configured, then there may not need all the other information. If cell-specific guard band is configured, and if it is before the UL subband, then only the duration of the guard band is needed as the end of the guard band is the beginning position of the UL subband. Similar for case if guard band is after the UL subband. For DL subbband, UL subband and guard band (if any) will be a reference point for DL subband. Thus, configuration of UL subband is needed while whether other configuration is needed depends on cases. Thus we proposed to update as “For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. FFS: whether configuration of DL subband and guard band if any is configured or not should be discussed.” |

    - TX/RX/measurement procedures

Proposal 2-1

Proposed Agreement:

If PRG is determined as wideband, non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated.

- UE assumes same precoding is applied to allocated RBs within a DL subband

- UE does not assume same makes no assumptions on precodings is applied to allocated RBs across two DL subbands

- For conditions for wideband precoder determination when precoding bundling is determined dynamically in SBFD symbols, down-select from the following options:

  - Option A: based on size of DL BWP as legacy

  - Option B: based on size of DL usable PRBs

    - FFS details

- FFS whether to introduce UE capability

|  | Company |
| --- | --- |
| Support | Spreadtrum, Sharp, Tejas, Sony, Panasonic, IDC, Samsung, Nokia, NSB |
| Not support | ZTE, Xiaomi, QC, LGE |

| Company | Comments |
| --- | --- |
| ZTE | The proposal is against with the definition of ‘wideband precoding’ that a UE is not expected to be scheduled with non-contiguous PRBs. Instead of complicating the UE behavior which may result in a feature that is challenging to be implemented in reality, we prefer not to do any enhancement to wideband precoding. In addition, the following bullet is not clear to us. - UE makes no assumptions on precodings applied to allocated RBs across two DL subbands |
| Moderator | Regarding the second bullet, the intention is to say that the precodings in two DL subbands can be different. The proposal is updated in red to make it clearer. |
| Spreadtrum | We think it is still worth to make wideband PRG valid in SBFD operation, considering its significant gain comparing 2/4 PRG size. If PRG is determined as wideband, better scheduling flexibility and higher DL data rate can be achieved if non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated. |
| Xiaomi | We tend to agree with ZTE. Besides, allowing to determine wideband PRG per DL subband introduce more issues, i.e., the conditions on how to determine wideband PRG, which is not expected. |
| Tejas | We support the proposal and can have further discussion. |
| QC | We have strong concerns on the proposals. We don’t see a need or a motivation to relax current relaxation of contiguous PRBs when PRG is determined is wideband. |
| IDC | Support in principle the proposal, which offers benefits by using wideband precoding across DL subbands based on such UE assumptions that simplify the operation. |
| LGE | We have a concern on second bullet regarding non-contiguous PRBs. We are also prefer not to enhance on the wideband precoding. |
| Samsung | We support the proposal. PDSCH processing timeline relexation can be considered to support two contiguous frequency resource. |
| OPPO | One clarification question, for the option B under the third sub-bullet, it means conditions for wideband precoder determination is based on the comparison of the total scheduled PRBs across to DL subbands and the sum of size of two DL usable PRBs, OR it leaves the door open to determine wideband precoder for each DL subband independently? |
| DOCOMO | We are fine with the main bullet and the first and second sub-bullet. For the third bullet, we think the wideband PRG determination and PRG size determination should be discussed together for precoding bundling. In our understanding, it is straightforward to apply the legacy determination rule. The necessity/benefit of determination based on DL usable PRBs is not clear to us. Moreover, it may cause misalignment issue when PDSCH repetitions across SBFD symbols and non-SBFD symbols if different number of RBs are applied (e.g. option 5). For the last bullet, we don’t see the necessity of separate UE capabilities and too many UE capabilities is not preferred from NW operator perspective. |

Proposal 2-2

Proposed Agreement:

Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH.

- Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols.

Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH.

- Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols.

  - Support separate configurations for DCI format 0_2 and other DCI formats as leagcy

  - The number of bits used to indicate FH offset in DCI is determined as legacy

    - The number of FH offsets configured for SBFD symbol is the same or less than the number of FH offsets configured for non-SBFD symbol

FFS whether/how to enable FH in non-SBFD symbols and disable FH in SBFD symbols

|  | Company |
| --- | --- |
| Support | Sharp, CEWiT, Tejas, QC, LGE, DOCOMO, CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | It seems that all the companies support separate configurations of FH offsets/FH offset lists for SBFD and non-SBFD symbols. Moderator also tried to provide some details which are expected to be straightforward. For the last FFS, the intention is to disable FH in SBFD symbols in some cases, e.g. the UL subband size is small. |
| ZTE | Suggest deleting the following two bullets. The FH offsets are separately configured for different symbol types. No need to add any additional artificial restrictions, no matter on the number of FH offsets or on enabling of FH in different symbol types. - The number of FH offsets configured for SBFD symbol is the same or less than the number of FH offsets configured for non-SBFD symbol FFS whether/how to enable FH in non-SBFD symbols and disable FH in SBFD symbols |
| Moderator | Moderator’s understanding is that the sub-bullet proposed to be deleted above by ZTE is the condition to determine the number of bits used to indicate FH offset in DCI is the same as legacy. For the FFS bullet, as clarified above, the intention is to allow the possibility to disable FH in SBFD symbols in some cases. |
| Spreadtrum | If the number of FH offsets configured for SBFD symbol is less than the number of FH offsets configured for non-SBFD symbol, how to use legacy bits to indicate FH offset for SBFD symbol should be clarified. For the FFS bullet, same view as ZTE. |
| Xiaomi | We think all configuration related issues should be up to RAN2. Hence, all the sub-bullets can be removed. |
| Tejas | Support the proposal of separate FH offsets for SBFD. For the FFS, also consider the proposal of wrapping of the RBs around the SBFD sub bands. This will be useful when SBFD sub bands are small. |
| Sony | Similar views with ZTE. We can discuss number of bits for DCI when it is clearer what the separate FH configuration looks like (e.g. does it consists of a MOD function to wrap around the hop as proposed by some companies). |
| QC | Support. Further clarification whether this agreement should be applicable to PUSCH with intra-slot freq. hopping only or additionally PUSCH repetition with inter-slot frequency hopping. The RRC parameter of FH offsets is applicable to both hopping scenarios, however, the equations for determining the start-RB are different. |
| Samsung | We prefer to avoid mis-configuration by gNB. UE expects the numbers of configured hopping offsets for non-SBFD symbol and SBFD symbols are same. Also, enabling/disabling is the same as the legacy, i.e., indicated by the legacy 1-bit DCI field |
| OPPO | For the number of FH offsets configured for SBFD symbol, in the existing specification, it relates to the size of UL BWP as following: - When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant. - When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant. It should be clarified whether the number of FH offsets configured for SBFD symbol is also based on size of UL BWP or based on size of UL usable PRB. We are fine with either way and just for clarification. For the last FFS, same view with ZTE |
| Nokia, NSB | We share similar view as ZTE that we just need an additional set of offsets for SBFD symbols. We don’t need other conditions. The FFS is also unnecessary. |

Proposal 2-3

Proposed Agreement:

Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols in PUCCH-Resource.

- Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols.

- FFS whether to support separate configurations of intraSlotFrequencyHopping

|  | Company |
| --- | --- |
| Support | Sharp, CEWiT, Tejas, QC, LGE, Samsung, Nokia, NSB, CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Option 4 has the most supporting companies and thus is proposed. Companies who want to support Options 1-3, please elaborate which additional parameters need separate configurations and the reasons. |
| ZTE | Support in general. More clarification on the FFS point is needed. |
| Moderator | The following parameters are included in PUCCH-Resource. Except startingPRB and secondHopPRB, pucch-ResourceId and format are expected to be the same for a PUCCH resource for different symbol types. For intraSlotFrequencyHopping, it is not clear whether different configurations are necessary for different symbol types and proposed to be FFS. PUCCH-Resource ::= SEQUENCE { pucch-ResourceId PUCCH-ResourceId, startingPRB PRB-Id, intraSlotFrequencyHopping ENUMERATED { enabled } OPTIONAL, -- Need R secondHopPRB PRB-Id OPTIONAL, -- Need R format CHOICE { format0 PUCCH-format0, format1 PUCCH-format1, format2 PUCCH-format2, format3 PUCCH-format3, format4 PUCCH-format4 } }. |
| Spreadtrum | Clarification for the proposal: Is it for PUCCH without repetition only, or also including PUCCH with repetition, period/semi-static PUCCH? |
| Xiaomi | It is to rush to boil down to option 4. From our understanding, option 1 is the simplest solution which causes least standardization effort. More discussion is needed. |
| Tejas | Support the proposal |
| QC | Few clarifications are needed on the proposal: - In our understanding, this applies to configuration 2 when same PUCCH resource is used in SBFD and non-SBFD symbols. - Whether applicable intra-slot and/or inter-slot frequency hopping. |
| OPPO | Similar view with xiaomi, we prefer to avoid parameter-wise discussion which would be complicated and difficult to achieve consensus and go with option 1 or option 2 which is simple and flexible. |
| DOCOMO | Clarification on understanding of the proposal. In our understanding, the proposal implies the PUCCH resource is shared for SBFD symbols and non-SBFD symbols. |
| WILUS | Support this proposal |

Proposal 2-4

Proposed Agreement:

Support separate configurations of SRS resource sets for SBFD symbols and non-SBFD symbols.

- Introduce new RRC parameters in SRS-Config to configure SRS resource sets for SBFD symbols.

|  | Company |
| --- | --- |
| Support | ZTE，New H3C, Spreadtrum, Sharp, CEWiT, Tejas, Sony, QC, ITRI, LGE, Samsung, OPPO, Nokia, NSB, CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| xiaomi | We don’t think the sub-bullet is needed. It is RAN2’s responsibility to determine what the RRC parameter looks like. |
| Tejas | Support the proposal of introducing new RRC parameters for SBFD symbols |
| QC | Clarify whether applicable to ‘CB’, ‘NCB’ or all SRS usages. In our views, this is needed only for CB and NCB SRS trnamission. |
| DOCOMO | Fine with the main bullet. The meaning of the sub-bullet is not so clear to us. Is the intention that a new RRC parameter in SRS-Config indicating “SBFD symbol type” for all SRS resource sets in the SRS-Config? And it implies the legacy SRS-Config without the new parameter is for “non-SBFD” symbol type? We have similar view as Xiaomi to let RAN2 to decide the parameter looks like. We only need to identify usage of the parameter, e.g. indicating symbol type. |

Proposal 2-5

Proposed Agreement:

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), down-select from the following options.

- Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid for SPS PDSCH

  - FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only

|  | Company |
| --- | --- |
| Support | New H3C, Spreadtrum, Xiaomi, Sharp, CEWiT, Tejas, QC, ITRI, LGE, DOCOMO, CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | For SPS PDSCH with Configuration 2, moderator thinks that Option 3 and 5 are more reasonable among Option 1/2/3/5. Option 1 can be achieved by Configuration 1. For Option 2, it is not clear to moderator how to derive the DL resources in SBFD symbols especially considering DUD pattern. |
| ZTE | Our first preference is to support Option 2. For Option 2, the RB offset is used to determine the frequency resources in SBFD symbols, and this is not relevant to whether the pattern is DU or DUD. Option 3/5 would degrade the performance. In addition, our understanding of the intention of Configuration 2 is to allow the transmission/reception in both symbol types valid as much as possible, otherwise it would be no different compared to Configuration 1. In this sense, Option 3 should be precluded for Configuration 2. One middle ground is to support both Option 2 and Option 5. That is, an RB offset is introduced while the SPS PDSCH may still overlap the subband boundary, e.g., one RBG may overlap with the subband boundary. If that happens, Option 5 can be applied. |
| Spreadtrum | Option 3 is simpler than Option 5. We support Option 3. From our perspective, we prefer a unified and simple solution except for multi PDSCH/PUSCH scheduled by single DCI. |
| Sharp | We thought that SPS PDSCH can be used for periodic DL data transmission with the same upper layer data size. Among the options, Option 3 ensures the same TBS and simple implementation. |
| CEWiT | We are not supportive of option 3 as it leads to wastage of resources, as the resources in DL-SB are also dropped. Also, it will also impact the latency. Our preference is UE performing the operation using separate resources, which are explicitly configured or implicitly derived. |
| Tejas | Support the proposal Option 5. |
| Sony | We have preference for Option 1 and Option 2. It isn’t clear how Configuration 1 would requires two separate SPS configurations to achieve Option 1 and hence used up the available number of SPS configurations. |
| QC | Support unified solution for all downlink scenarios (SPS-PDSCH, PDSCH repetition and multiple PDSCHs). Support option 5 over option 3. Option 3 has the following limitation and is not a complete solution. - With option 3, SPS occasions in SBFD symbols are valid only when the SPS-PDSCH freq. resource are withion DL usable PRBs - Otherwise, they are invalid and PDSCH reception will happen only in non-SBFD (DL/FL) symbols as ones in SBFD symbols are dropped. This is against the objectives of having DL reception in both SBFD and non-SBFD symbols. |
| IDC | Similar view as ZTE. Option 2 (with RB offset) is the most clear indication way which can avoids the worst situation like Option 3 (invalid for the occasion). Option 5 can also help avoiding the situation of Option 3. Therefore, we can support both Option 2 (with RB offset) and Option 5 for the compromised way forward, but we have concerns on Option 3. |
| LGE | We support the proposed agreement in general. For the FFS point in Option 5, we support to determine TBS based on the assigned PRBs within DL usable PRBs only. In the last meeting, it was agreed that the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only for PDSCH in a single slot by DCI based scheduling. The same principle can be applied for this case. |
| Samsung | We support option 3 as baseline. By activating multiple SPS configurations, a UE can receive SPS PDSCHs with different RB allocations. This approach is much simpler and beneficial, since the required MCS could be different in non-SBFD symbol and SBFD symbol due to UE-UE CLI. |
| OPPO | Generally fine with the proposal, For the FFS bullet under option 5, it would be better to clarify whether it means to separately determine TBS for each SPS PDSCH occasion? |
| DOCOMO | Prefer Option 5, for a unified solution for DG PDSCH with RA type 0/1 without repetition. |
| Nokia, NSB | Option 1 or Option 2 can be supported for SPS PDSCH. For Option 1, separate resource allocation can be provided via separate activation DCIs in each slot type. Option 3/5 can be applied as default option if Option 1 or Option 2 are not configured. |
| WILUS | Prefer Option 3 |

Proposal 2-6

Proposed Agreement:

For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), down-select from the following options.

- Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is dropped.

- Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid.

  - For PDSCH repetitions, the number of PRBs for TBS determination is based on assigned PRBs.

  - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

|  | Company |
| --- | --- |
| Support | New H3C, Xiaomi, Sharp, CEWiT (Option 5), Tejas, ITRI, Panasonic, Samsung, DOCOMO, CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Similar as for SPS PDSCH, moderator thinks that Option 3 and 5 are more reasonable. |
| ZTE | Similar comment as above. |
| Spreadtrum | We prefer to discuss the cases of PDSCH repetitions and multi-PDSCH scheduled by a single DCI separately. For PDSCH repetition, they are a bundling across different slots and same frequency and time resource is allocated for different slots. So the number of PRBs for TBS determination is same across different slots. Option 3 is more suitable for PDSCH repetition. It does not break the characteristics of repetition transmission. And it is simpler than Option 5. If Option 5 is supported, the number of PRBs for TBS determination needs further discussion. For multi-PDSCH scheduled by a single DCI, the PDSCHs in different slots are separate TB scheduling. They are independent from each other. Separate SLIV but same FDRA can be indicated, so each TBS is calculated according to its own frequency and time resource. It is more like single slot scheduling by single DCI, assigned PRBs can be within or outside DL usable PRBs. Thus, the Option 5 can be supported for multi-PDSCH scheduling. The number of PRBs for TBS determination is based on assigned PRBs with-in DL usable PRBs only. |
| CEWiT | Not supportive of option 3 as it leads to unnecessary reduction of resources for performing the operation. Also, the resources overlapping with DL-SB will not be utilized, leading to wastage of resources. However, option 5 efficiently utilizes the configuration of DL/UL SB to perform the operation and minimizes the wastage of resources. |
| Tejas | Support the proposal, prefer Option 5. |
| Sony | Same comment as Proposal 2-5. |
| QC | Similar comment as FL proposal 2-5. |
| Panasonic | We prefer option 3 in order to reduce the complexity. |
| IDC | Similar comments as above. Option 2 (with RB offset) is the most clear indication way which can avoids the worst situation like Option 3 (invalid for the occasion). Option 5 can also help avoiding the situation of Option 3. Therefore, we can support both Option 2 (with RB offset) and Option 5 for the compromised way forward, but we have concerns on Option 3. |
| LGE | We are ok to down-select from Option 3 and 5. However, regarding TBS determination in Option 5, we believe that TBS for PDSCH repetition should be determined based on assigned PRBs within DL usable PRBs only. If TBS determination is based on assigned PRBs, since the number of coded bits transmitted in a SBFD slot is reduced, some coded bits may not be transmitted even with repetitions. In particular, if the first slot is an SBFD slot, some systematic bits may not be transmitted and the performance can be significantly reduced. |
| Samsung | We prefer option 3 as baseline. |
| OPPO | For the two bullets under option 5 (the TBS determination part), we see other candidate solutions for TBS determination besides the listed one, it can be also based on the available PRB number for the first repetition, which is the similar with TBS determination for PUSCH repetition type B. So at this stage we prefer to leave this part to FFS like following: - Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid. - FFS TBS determination for PDSCH repetitions and multi-PDSCH scheduled by a single DCI - For PDSCH repetitions, the number of PRBs for TBS determination is based on assigned PRBs. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only. |
| DOCOMO | Prefer Option 5, for a unified solution for DG PDSCH with RA type 0/1 without repetition. |
| Nokia, NSB | Option 2 may also be supported on top of option 3 and 5. |
| WILUS | Prefer Option 3 |

Proposal 2-7

Proposed Agreement:

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), down-select from the following options.

- Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) determination to determine resource for SBFD symbols

- Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid

- Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid for CG PUSCH

  - FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within UL usable PRBs only

|  | Company |
| --- | --- |
| Support | ZTE，New H3C, Spreadtrum, Xiaomi, CEWiT, Tejas, Sony, ITRI, IDC, Samsung, DOCOMO, CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | Compared with DL, Option 2 is added for UL transmission. Furthermore, RB offset configuration/indication is removed. However, moderator still doubts the necessity and benefit of Option 2. If the RBs with offset are avaialbe in SBFD symbols, why does not gNB configure/allocate those RBs? If the intention is to achieve different FDRA in SBFD and non-SBFD symbols, it can be achieved by Option 1. Companies are encouraged to share your views. |
| ZTE | For CG PUSCH, it’s better to discuss CG PUSCH type 1 and CG PUSCH type 2 separately. For CG PUSCH type 2, it can be treated similar as DG PUSCH, and we are fine with the proposal. Similar as above comment, we prefer Option 2 and also ok with additionally support Option 5. For the FFS point of Option 5, we support TBS determination is based on assigned PRBs. For CG PUSCH type 1, we support Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols. |
| Spreadtrum | Similar comments as Proposal 2-5. |
| Sharp | We share the same understanding as ZTE. Two CG types should be discussed differently. For CG PUSCH type 1, we support Option 1. For CG PUSCH type 2, we think the proposal 2-13 should be discussed first. If the proposal 2-13 is agreed, Option 3 is preferred. If the proposal 2-13 is not agreed, Option 2 is preferred. The reason behind is that if the proposal 2-13 is not agreed, for the configuration 2, the network needs to assign the RBs for the first hop within the UL subband. Otherwise, any transmission occasions in SBFD symbols will be dropped because the assigned PRBs are outside of the UL subband. |
| CEWiT | We are not supportive of option 3 as it leads to wastage of resources, as the resources in DL-SB are also dropped. Also, it will also impact the latency. Regarding option 2, we share similar view with moderator, the use/benefit of additional offset is not clear. Our preference is UE performing the operation using separate resources, which are explicitly configured or implicitly derived. |
| Tejas | Support the proposal, prefer Option 5. |
| QC | We do have concerns on option 5 as it creates new UE procedure for UL transmission based on rate-matching. In addition, we believe only two options can be listed for further down selection, similar to DL reception acoss SBFD and non-SBFD. So, we suggest dropping option 5 and RAN1 to downselect between two options (2,3). Also, it is preferred to have a unified across all UL tranmissions (CG-PUSCH, PUSCH repetions and multiple PUSCHSs) |
| IDC | Similar view as ZTE. Option 2 (with RB offset) is the most clear indication way which can avoids the worst situation like Option 3 (invalid for the occasion). Option 5 can also help avoiding the situation of Option 3. Therefore, we can support both Option 2 (with RB offset) and Option 5 for the compromised way forward, but we have concerns on Option 3. |
| LGE | We prefer to leave Option 3 and 5. For Option 5, we support to determine TBS based on the assigned PRBs within UL usable PRBs only. |
| Samsung | We prefer option 3 as baseline. |
| OPPO | Similar comments as proposal 2-5 |
| DOCOMO | Prefer Option 5, for a unified solution for PDSCH and PUSCH. |
| Nokia, NSB | We share similar views as ZTE. We are fine if this proposal is for CG PUSCH type 2. For type 1, we prefer Option 1 and Option 3 (if Option 1 is not configured), as mentioned above for SPS PDSCH. |
| WILUS | Prefer Option 3 |

Proposal 2-8

Proposed Agreement:

For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), down-select from the following options.

- Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) determination to determine resource for SBFD symbols

- Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is dropped or postponed

  - For PUSCH repetition type-A with available slot counting and TBoMS, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is postponed.

  - For PUSCH repetition type-A without available slot counting and multi-PUSCH scheduled by a single DCI, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is dropped.

- Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid

  - For PUSCH repetitions and TBoMS, the number of PRBs for TBS determination is based on assigned PRBs.

  - For multi-PUSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within UL usable PRBs only.

|  | Company |
| --- | --- |
| Support | ZTE，New H3C, Xiaomi, CEWiT, Tejas, Sony, ITRI, Panasonic, IDC, Samsung, DOCOMO, Nokia, NSB, CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Fine with the proposal. Similar as above, we prefer Option 2, and also ok to support more than one option. |
| Spreadtrum | We prefer to discuss the cases of PUSCH repetition type-A/TBoMS and multi-PUSCH scheduled by a single DCI separately. Similar comment as Proposal 2-6. For PUSCH repetition type-A and TBoMS, Option 3 is simpler and preferred. For multi-PUSCH scheduled by a single DCI, we support Option 5. |
| Sharp | We think the proposal 2-13 should be discussed first. If the proposal 2-13 is agreed, Option 3 is preferred. If the proposal 2-13 is not agreed, Option 2 is preferred. The reason behind is that if the proposal 2-13 is not agreed, for the configuration 2, the network needs to assign the RBs for the first hop within the UL subband. Otherwise, any transmission occasions in SBFD symbols will be dropped because the assigned PRBs are outside of the UL subband. |
| CEWiT | Support option 5, as it efficiently utilizes the configuration of DL/UL SB to perform the operation and minimizes the wastage of resources. |
| Tejas | We prefer option 5. Option 2 doesn’t guarantee that assigned PRBs are within the UL usable PRBs. We can have more than one option, to enable efficient use of UL usable PRBs. |
| QC | Similar comment as FL 2-7. |
| Panasonic | We prefer option 3 in order to reduce the complexity. |
| IDC | Similar comments as above. Option 2 (with RB offset) is the most clear indication way which can avoids the worst situation like Option 3 (invalid for the occasion). Option 5 can also help avoiding the situation of Option 3. Therefore, we can support both Option 2 (with RB offset) and Option 5 for the compromised way forward, but we have concerns on Option 3. |
| LGE | We prefer to leave Option 3 and 5. For Option 5, we have the similar comment with Proposal 2-6. Regarding TBS determination in Option 5, we believe that TBS for PUSCH repetition should be determined based on assigned PRBs within UL usable PRBs only. If TBS determination is based on assigned PRBs, since the number of coded bits transmitted in a SBFD slot is reduced, some coded bits may not be transmitted even with repetitions. In particular, if the first slot is an SBFD slot, some systematic bits may not be transmitted and the performance can be significantly reduced. |
| Samsung | We prefer option 3 as baseline. |
| OPPO | Similar comment as proposal 2-6 |
| DOCOMO | Prefer Option 5, for a unified solution for PDSCH and PUSCH. |
| WILUS | Prefer option 3 |

Proposal 2-9

Proposed Agreement:

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following.

- For semi-statically configured transmissions/receptions, the valid symbol type is explicitly configured by RRC.

- For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception.

- For SP-CSI on PUCCH or PUSCH, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in CSI-ReportConfig.

  - Option 2: The valid symbol type is determined based on the symbol type of the first PUSCH/ PUCCH after activation.

- For type 2 CG PUSCH and SPS PDSCH, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in ConfiguredGrantConfig/SPS-Config.

  - Option 2: The valid symbol type is determined based on the symbol type of the first CG PUSCH/SPS PDSCH after activation.

|  | Company |
| --- | --- |
| Support | ZTE，New H3C, Spreadtrum, Sharp, CEWiT, Tejas, Sony, ITRI, LGE, Samsung, DOCOMO, Nokia, NSB, CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Support. For both the last two subbullets, we prefer Option 2. |
| Xiaomi | Our understanding on configuration 1 is that gNB indicate which symbol type can be used for reception and transmission directly. With this understanding, not matter semi-statically configured transmissions/receptions or dynamically scheduled transmissions/receptions, UE should follow gNB’s indication which symbol type is available. Hence, we feel that we need to confirm whether companies are on the same page for configuration#1 before we dive into detail solutions. |
| Tejas | We support the proposal. We prefer option 2 as it gives more flexibility. |
| QC | First, RAN1 should agree that configuration is per signal/channel as this agreement implicitly supports. |
| LGE | For SP-CSI and type 2 CG PUSCH, we prefer Option 1. |
| OPPO | Generally fine with the proposal but it would better to clarify which category does SPS HARQ belongs to? |

Proposal 2-10

Proposed Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1,

- For PUSCH repetitions with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type.

- For CG PUSCH and SPS PDSCH, the transmission/reception occasions in the invalid symbol type are invalid for transmissions/receptions.

- For the other transmissions/receptions except for SPS HARQ-ACK deferral, transmissions/receptions in the invalid symbol type are dropped.

- FFS SPS HARQ-ACK deferral in the invalid symbol type

|  | Company |
| --- | --- |
| Support | New H3C, Sharp, CEWiT, Tejas, Panasonic, IDC, LGE, Samsung, OPPO, DOCOMO, Nokia, NSB (with small remark), CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Fine with the proposal. |
| Spreadtrum | We think the second bullet and the third bullet is duplicated. We prefer to delete the second bullet and revise the third bullet as: - For the other transmissions/receptions except for SPS HARQ-ACK deferral, transmission/reception occasions in the invalid symbol type are dropped. |
| Xiaomi | Fine with the proposal. |
| CEWiT | Support the proposal in general. Request moderator to clarify the difference between second and third bullet. |
| Tejas | Support the proposal in general. |
| QC | - Few comments: - It is good to clarify what is meant by ‘For the other transmissions/receptions” in the third subbullet, e.g. P/SP SRS, P/SP CSI-RS, P/SP PUCCH, etc. - For PUSCH repetition w/o available slot count, it is expected that PUSCH transmission is dropped and counted. |
| Nokia, NSB | It would be first good to understand the motivation for having the SPS deferral case as FFS. |

Proposal 2-11

Proposed Agreement:

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters.

- Option 1: One TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

- Note: The maximum number of closed power control loops is not increased, i.e. up to 2 closed loops as legacy.

|  | Company |
| --- | --- |
| Support | ZTE，New H3C Xiaomi, CEWiT, TCL, QC, ITRI, Panasonic, LGE, Samsung, Nokia, NSB, CATT, WILUS |
| Not support | DOCOMO |

| Company | Comments |
| --- | --- |
| ZTE | Prefer Option 1. |
| Spreadtrum | Clarification: For SRS, since we have Proposal 2-4, each SRS resource set only have one power control parameter set, so it can be separately discussed. For Option 1/2, it depends on the discussion of separate spatial domain is supported or not for SBFD/non-SBFD. So this proposal can be postponed. |
| Xiaomi | Prefer option 1. |
| CEWiT | Prefer option 1 |
| TCL | Prefer option 2. |
| QC | Support and prefer option1. Regarding option-1, the TCI codepoint could be mapped upto 4 TCI staes for separate unfied UL/DL states in SBFD and non-SBFD |
| IDC | We have strong concern on Option 1 because it may have conflict with the current MIMO-related use cases where one TCI-codepoint can be associated with more than one TCI-states (for specific multi-TRP feature). So, Option 2 is the clear and sufficient option for supporting SBFD which can avoid any potential collision with already supported MIMO-features. The FFS part can be removed (as this is just a straightforward second-level detail) or changed with “ul-powerControl-r17” and “pathlossReferenceRS-Id” In our contribution, we explained this based on the following: TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ..., [[ additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI pathlossReferenceRS-NEW-Id PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-NEW Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI ]], [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] }. |
| LGE | Prefer option 1. |
| Samsung | We prefer option 1. |
| DOCOMO | We have concern on the proposal to include two options based on different assumptions. It is not good to “down-select” from two options. In our understanding, option 1 is based on the assumption of separate UL beams for SBFD and non-SBFD. Separate UL power control can be realized when separate UL beams are applied. But it should be discussed in the issue of “separate UL beams for SBFD and non-SBFD”. Option 2 is based on the assumption of common UL beam for SBFD and non-SBFD. Currently, only separate UL power control is agreed, and separate UL beams is not agreed yet. Moreover, even if separate UL beams are also agreed, we think the enabling of separate UL power control should be independent from the enabling of separate UL beams. At least NW should have the freedom to determine whether both or only separate UL power control to be applied. Therefore, at least option 2 should be supported, and it can be clarified that option 2 is for the case when “no separate UL beams are applied for SBFD and non-SBFD”. And option 1 should be discussed separately in the issue of “separate UL beams for SBFD and non-SBFD”. |
| WILUS | Prefer option 1 |

Proposal 2-12

Proposed Agreement:

For CSI report associated with periodic/semi-persistent CSI-RS, support Option A.

- Introduce a new RRC parameter in CSI-ReportConfig to indicate whether CSI-RS(s) are restricted to SBFD symbols only or non-SBFD symbols only.

|  | Company |
| --- | --- |
| Support | ZTE, Sharp, Tejas, QC, LGE, Samsung, DOCOMO, Nokia, NSB, CATT, WILUS |
| Not support | Xiaomi |

| Company | Comments |
| --- | --- |
| Spreadtrum | Basically, we support the Proposal. But we think the Option B proposed in last meeting also can be supported. For Option B, a new type of sub-configuration needs to be defined, but it has little impact on specification. So we suggest to support both Option and Option A/B can be depend on UE capability. |
| Xiaomi | As we discussed in the previous meeting, it is clear that both option A and option B are potential optimization compared to legacy mechanisms, i.e., option 1-1 and option 2-2. The common understanding is that option 1-1 and option 2-2 are already supported by current specification and can be applied to SBFD operation directly. For option A and option B, the specification impact is not trivial. We don’t agree both of them before see sufficient validation. |
| CEWIT | Request to clarify the meaning of “CSI-RS(s) are restricted”. Is it like the CSI-RS resources under a CSI-report configuration is restricted to SBFD/non-SBFD or restricting the UE to measure and report only in SBFD/non-SBFD In our understanding, there is no need to restrict the CSI-RS to either SBFD or non-SBFD symbols. The report configuration can configure the UE whether to report CSI parameters for SBFD symbols, non-SBFD symbols or both. Since the UE knows the parameters of SBFD, it can segregate the CSI-RS resources in SBFD symbols and non-SBFD symbols and can measure the CSI parameters separately. Also, in case of SP reporting the trigger can indicate the UE whether to report parameters corresponding to SBFD/non-SBFD/both. |
| Sony | We think both can be supported but OK if either one of them is specified only. |
| Samsung | We are supportive of option A. Also, we have similar view that Option B can also be considered which is a efficient manner to save the number of CSI report configuration (which is very limited due to UE capability cap). An FFS point of option B can be added for further decision. |
| OPPO | The motivation for option A is not strong since the periodicity of CSI-RS is expected to be not that short, it is not difficult for gNB to configure CSI-RS resource well match the TDD and SBFD time pattern by configuration of appropriate periodicity and offset. |

Proposal 2-13

For PUSCH FH in SBFD symbols, update the PUSCH frequency hopping formula according to one of the following options.

- Option 1: RBUL SB start+(RBstart+RBoffset)modNBWPsizeNUL SBsize

- Option 2: RBUL SB start+(RBstart-RBUL SB start+RBoffset)modNBWPsizeNUL SBsize

Where RBUL SB start is the starting PRB index of the UL usable PRB and NUL SBsize is the size of UL usable PRB

|  | Company |
| --- | --- |
| Support | Sharp, CEWiT, Tejas, QC, LGE, DOCOMO, CATT, WILUS |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | For a PUSCH scheduled ONLY in SBFD symbols, it seems sufficient to only change BWP size to UL subband size. For a PUSCH transmission across different symbol types in different slots, it may depend on the proposals above for the frequency resource determination in SBFD symbols. If Option 1/2 is supported, RB_start is the start RB after the frequency resource determination in SBFD symbols. The only change needed is also to change BWP size to UL subband size. |
| Moderator | Even for PUSCH in SBFD symbols only, changing BWP size to UL subband size only is not sufficient according to moderator’s understanding. Because otherwise the RB index would be in range of [0, UL usable PRB size-1] and the RB index is with reference to the starting PRB of UL BWP, which is not the intended the RB location. |
| Spreadtrum | More clarification is needed for first red part in Options. RBstart still takes the starting position of BWP as the reference point in our understanding. |
| Xiaomi | We are not clear about the functionality/necessity of RBUL SB start. At this stage, a high level proposal like below is sufficient. ‘PUSCH frequency hopping pattern/formula is updated by replacing UL BWP size by the number of UL usable PRBs for SBFD symbols. FFS details’. |
| Tejas | We support option 2. |
| QC | Option 1 |
| LGE | Option 1 |
| Samsung | We think the starting PRB index without FH and the starting PRB index with FH and RBoffset=0 should be same. With this understanding, option 2 is preferred. |
| OPPO | We are not clear for the motivation of the red part. |
| DOCOMO | We support the proposal. The output value range of both option 1 and option 2 are [RBUL SB start, RBUL SB start+NUL SBsize], which is within the UL usable PRBs. If simply replacing the modulation part (i.e. without “RBUL SB start+”, the output value range is [0, NUL SBsize], which still may be out of the UL usable PRBs. We prefer Option 2. In our understanding, the RBstart in the legacy formula represents the offset relative to the UL BWP. Therefore, we prefer RBstart-RBUL SB start representing the offset relative to the UL subband in the updated formula. |
| Nokia, NSB | Since FH in SBFD symbols was not there in the specs, we can avoid saying “updating the formula”. We can discuss these details when we agree on the proposals above, or simply leave it to editors. |
| WILUS | We prefer Option 2. |

    - Collision handling

Proposal 3-1

Proposed Agreement:

For SBFD-aware UE transmission and reception in an SBFD symbol, support Option 2 to determine whether to transmit or to receive in the SBFD symbol.

- Option 2: link direction is indicated by gNB explicitly.

  - Link direction is indicated by dedicated RRC signalling.

  - Link direction indication by DCI is not supported.

|  | Company |
| --- | --- |
| Support | Spreadtrum, CEWiT, Tejas, Sony, ITRI, LGE， CMCC, CATT |
| Not support | ZTE, QC, Panasonic, Samsung, OPPO, DOCOMO, Nokia, NSB, WILUS |

| Company | Comments |
| --- | --- |
| ZTE | On top of Option 1, it is unclear to us why we need to additionally support Option 2. |
| Tejas | Support in general. Link direction indication via dedicated RRC signaling will address semi static collision cases. But how will the given option address dynamic collision cases? |
| Sony | There needs a mechanism to implicitly overwrite the semi-statically link direction especially for cases where there is no collision. |
| Panasonic | We share ZTE's view that the necessity to support option 2 would be unclear. |
| DOCOMO | Similar view as ZTE. |
| Nokia, NSB | Before agreeing to this, we first need to discuss more details on the expected UE behaviour for Option 2. We disagree with Option 2 if the intention is to assign a pre-defined and strict link direction to a set of SBFD symbols as this would diminish most of the benefits of SBFD. Instead, we think there are benefits of indicating to the UE a “preferred” link direction which is only applicable e.g. in case of collisions between UL and DL, while UEs can still be scheduled in either direction in SBFD slots as long as there is no collision. |
| CMCC | We support explicit link direction indication. From UE perspective, SBFD symbol is similar to the legacy flexible symbol in R15, and link direction indication is similar to the legacy TDD slot configuration indications (e.g., Tdd-UL-DL-ConfigurationDedicated and SFI in DCI format 2_0), i.e., which is potential to easy gNB and UE implementation. Furthermore, if only scheduling signalling based solution is supported, UE needs to monitor DCI on each PDCCH monitoring occasion which is not beneficial to UE power saving. In addition, there may be some UL/DL collision cases in the same SBFD symbol, e.g., higher layer configured DL reception and UL transmission, and the scheduling DCI based solution cannot solve this issue. |

Proposal 3-2

Proposed Agreement:

For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied.

|  | Company |
| --- | --- |
| Support | ZTE，New H3C, Spreadtrum, Sharp, CEWiT, Tejas, Sony, QC, Panasonic, LGE, Samsung, OPPO, DOCOMO, CATT, WILUS |
| Not support | Nokia, NSB (to be updated) |

| Company | Comments |
| --- | --- |
| Sony | Just to clarify, the collision rule is applied per repetition basis, i.e.: For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied per repetition. |
| Nokia, NSB | We are OK for collision case 1/4, but not ok for reuse legacy collision handling for case 2. We think at least for Case 2, as there may be both PDCCH and ongoing UL repetition only in UL subband. If always keep the UL repetition in the consecutive SBFD slot, then there will be no chance to receive PDCCH for following UL slot, then the traffic in UL slot may be not possible to be scheduled in some cases, which is not the motivation of SBFD to increase UL resource to reduce latency and increase UL resource. Thus we think at least for Case 2, RAN1 should discuss rules to provide chance for UE to receive PDCCH that will schedule the UL transmission for following UL or SBFD UL subband in following SBFD slots. Or there will be case that SBFD UE utilize the SBFD UL subband but following UL slot can not be scheduled for UL transmission as no PDCCH can be received to scheduling the following UL slot, which is opposite to the motivation of SBFD. Thus, to be updated as “For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied.” |

Proposal 3-3

Proposed Agreement:

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 6 (Dynamic or semi-static DL vs. valid RO), reuse the existing collision handling rules for HD-FDD RedCap UEs.

- UE does not expect collision between PRACH triggered by PDCCH order and dynamically scheduled DL reception.

- For collision between PRACH triggered by PDCCH order and semi-statically configured DL reception, UE does not receive DL channel/signal.

- For collision between PRACH triggered by higher layer and DL channel/signal, leave it to UE implementation whether to receive the DL or transmit PRACH.

|  | Company |
| --- | --- |
| Support | Spreadtrum, Sharp, CEWiT, Tejas, ITRI, Panasonic, LGE, Samsung, DOCOMO, CATT, WILUS |
| Not support | ZTE |

| Company | Comments |
| --- | --- |
| ZTE | Thanks FL for clarifying previous CR discussion. However, we are still confused why we cannot reuse existing UE behavior defined for unpaired spectrum. UE behavior in the current spec for unpaired spectrum: According to the discussion of CR R1-21-3809 in RAN1#104b-e, the UE behavior may not be clear ONLY on the following two cases. - Case 1: CSI-RS/PDSCH scheduled by a DCI format overlaps with a valid RO - Case 2: A downlink/flexible symbol indicated by DCI format 2_0 overlaps with a valid RO But, UE behaviour in the current spec is clear on the following case. For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.. It is noted that, there might be other cases and the corresponding UE behaviour is expected to be clear. UE behavior in the current spec for HD FDD UE: As summarized by FL, HD-UE behavior related to RO is specified ONLY for the following case (PDCCH order triggered RO). For other cases, they are left to UE implementation. The UE behavior is the same as normal TDD UE. If a HD-UE is configured by higher layers to receive a PDCCH, or PDSCH, or CSI-RS, or DL PRS in a set of symbols, the HD-UE receives the PDCCH, or PDSCH, or CSI-RS, or DL PRS if the HD-UE does not detect a DCI format that indicates to the HD-UE to transmit a PUSCH, or PUCCH, or PRACH, or SRS in at least one symbol of the set of symbols; otherwise, the HD-UE does not receive the PDCCH, or PDSCH, or CSI-RS, or DL PRS in the set of symbols.. In summary, our understanding is that the UE behavior defined for HD UE is also clearly defined for unpaired spectrum. Then, why we would reuse HD FDD UE behavior to TDD spectrum? In addition, if we reuse the existing UE behavior in unpaired spectrum, no spec impact is expected. But if we reuse the HD UE behavior, it has spec impact. |
| Moderator | The following email discussion outcome is summarized in R1-2103809. It is clear that the spec is not clear and companies have different understandings on some PRACH collision cases. Based on the discussions, different companies have different interpretations for the collision cases discussed in this document. Therefore, following was proposed to reflect the situation for Rel-15 considering the potential different UE implementations and leave some room to have an aligned UE behaviour for Rel-16: Proposed conclusion for Rel-15 to include the following interpretations: For Rel-15 PRACH collision Case#1 mentioned in the R1-2102935, there are 3 different interpretations. - Interpretation 1: When the cancellation timeline is satisfied, the UE neither performs transmission nor receives any DL signal/channels on the symbols overlapping with PRACH occasion. - Interpretation 2: When the cancellation timeline is satisfied, the UE cancels the PRACH transmission and receives the DL signal/channels on the symbols overlapping with PRACH occasion. - Interpretation 3: the UE performs PRACH transmission and does not perform the DL receptions. For Rel-15 PRACH collision Case#2 mentioned in the R1-2102935, there are 2 different interpretations. - Interpretation 1: - a DCI format 2_0 indicating flexible can cancel the PRACH transmission if the cancellation timeline is satisfied; - the UE does not expect a DCI format 2_0 indicating the symbols that overlapping with the valid RO is downlink. - Interpretation 2: the same handling as for Case#1 for a DCI format 2_0 indicating the symbols that overlapping with the valid RO is downlink or flexible.. |
| Tejas | Support the proposal |
| Sony | Should this be handled in Agenda 9.3.2? |
| QC | We are okay with first two subbullets as they follow same handling rules for case 3 (first subbulet) and case 2 (2nd subbelt). Then, there are two remaining scenios: - PRACH triggered by higher layer versus dynamic DL: follow case 1 where UE cancel UL (within cancellation timeline) and receive the DL. - PRACH triggered by higher layer versus higher-layer DL: follow case 4 (error case: UE doesn’t expect …) |

Proposal 3-4

Proposed Agreement:

Update the following agreement made in RAN1#117 meeting:

If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case.

- An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol

- An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol

  - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/0B/1/2 CSS set

    - FFS: Additional cases

|  | Company |
| --- | --- |
| Support | Spreadtrum, Sharp, CEWiT, Tejas, QC, LGE, Samsung, OPPO, Nokia, NSB, CATT, WILUS |
| Not support | ZTE |

| Company | Comments |
| --- | --- |
| Moderator | CATT proposed to add Type-0B CSS as for RedCap HD-FDD UEs. ETRI in addition proposed to add Type-1A/2A/3. Type-1A and Type-2A CSS are for SDT and PEI respectively, which are not applicable to RRC_CONNECTED. Therefore, moderator’s understanding is that there cannot be collision between dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and Type-1A and Type-2A CSS. For Type-3 CSS, it is configured via dedicated higher layer parameters. Therefore, it is already included in the first bullet of previous agreement. Therefore, it is proposed to add Type-0B only, which is also aligned with current specification descriptions for RedCap HD-FDD UEs. A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE does not expect to receive both a Type-0/0A/0B/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except a Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring configured-grant based PUSCH transmission as described in clause 19.1 in the set of symbols for which case the UE follows the procedure as in clause 5.1B.2.6 in [10, TS 38.133]. The UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission as described in clause 19.1 per SI modification period.. |
| ZTE | As FL noted, Type-0B CSS is for RedCap HD-FDD UEs. So, why it is relevant here for SBFD UEs operating in a TDD carrier. More clarification is needed. |
| CATT | Type-0B CSS was introduced for MBS, not for RedCap. Maybe ZTE can clarify why it is not relevant for SBFD operation. |
| Spreadtrum | For questions raised by ZTE, Type0B-PDCCH CSS set is for MBS, SBFD aware UE can also support it, same as Redcap HD UE. - a Type0B-PDCCH CSS set configured by - searchSpaceMCCH and searchSpaceMTCH for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI for broadcast, on the primary cell of the MCG - searchspaceMulticastMCCH for a DCI format 4_0 with CRC scrambled by a Multicast MCCH-RNTI, or by searchSpaceMulticastMTCH for a DCI format 4_1 with CRC scrambled by a G-RNTI for PDCCH receptions in RRC_INACTIVE state. |

  - [Closed] 2nd round discussion

    - TX/RX/measurement procedures

Proposal 2-2a

Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2.

- Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols.

Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH.

- Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols.

  - Support separate configurations for DCI format 0_2 and other DCI formats as legacy

  - The number of bits used to indicate FH offset in DCI is determined as legacy

UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions.

FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

|  | Company |
| --- | --- |
| Support | Sony, Nokia, NSB,New H3C, China Telecom, Ericsson, Spreadtrum, CEWiT, Tejas, WILUS, Transsion |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia, NSB | It’s fair enough to clarify UE behaviour when the FH offset/FH offset list is not provided. However, it’s rather straightforward to us that, if the FH offset/FH offset list is not provided, then FH is disabled for PUSCH transmissions in SBFD symbols. |
| Tejas | Support the proposal of new RRC parameter for SBFD. But when SBFD sub bands are small, FH offset will not be useful. Can we also consider the proposal of wrapping of the RBs around the SBFD sub bands when the allocated RBs cross over sub-band as shown in the figure. |
| Samsung | Two points for further clarifications 1) Regarding how to determine the number of bits, as we commented, there might be a case where is no non-SBFD symbols. In this case, our understanding is to determine the number of bits based on the # of FH offsets for SBFD. 2) We are open to the FFS. Basically, the FFS can be easily avoided by gNB’s configuration, i.e., the same number of FH offsets are configured for non-SBFD symbol and SBFD symbol. |

Proposal 2-3

Proposed Agreement:

Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols in PUCCH-Resource.

- Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols.

- FFS whether to support separate configurations of intraSlotFrequencyHopping

|  | Company |
| --- | --- |
| Support | Sony, Nokia, NSB,New H3C, Ericsson, Spreadtrum, CEWiT, Tejas, Samsung, WILUS, Transsion |
| Not support |  |

| Company | Comments |
| --- | --- |
| OPPO | As commented in the 1st round, we prefer to avoid parameter-wise discussion which would be complicated. From our understanding, there are two motivations of separate parameter configuration for SBFD symbols and non-SBFD symbols, one is to accommodate different frequency resources in different symbol types to avoid resource fragmentation, another is to achieve similar detection performance in different symbol types. Based on that, it would be- hard to say that only startingPRB/secondHopPRB and power control parameters are related to the above motivations, and other parameters, like number of symbols, maxcoderate, are not related to that. So, it is preferred to go with option 1 or option 2 which is simple and flexible. |

Proposal 2-4a

Proposed Agreement:

Support separate configurations of SRS resource sets for SBFD symbols and non-SBFD symbols.

- Introduce new RRC parameters in SRS-Config to configure SRS resource sets for SBFD symbols.

- FFS applicable SRS usages

|  | Company |
| --- | --- |
| Support | Sony, Nokia, NSB,New H3C, China Telecom, Spreadtrum(partially), CEWiT, Tejas, Samsung, WILUS, Transsion, ITRI |
| Not support | Ericsson |

| Company | Comments |
| --- | --- |
| Nokia, NSB | The FFS is a bit unclear to us, but we are open to discuss more. |
| Ericsson | It appears that the proposal intends to duplicate SRS resource sets with all the parameters in addition to the new parameters specific to SBFD symbol. We are not sure whether it is the right direction to consider given that larger specification impact and higher signaling overhead due to this approach. Behavior of periodic SRS is unclear if the SBFD-aware UE operates in Configuration 2, e.g., SRS resource corresponding to one type of symbol (e.g., SBFD) encounters different type of symbol (i.e., non-SBFD symbol). There exist limits on the maximum number of SRS resource sets for certain usage (e.g., codebook, non-codebook, etc) in Release-15/16. If one utilizes the multi-TRP based framework for this to expand the number of SRS resource sets for this purpose, then it prohibits to combine operations of SBFD and multi-TRP later. Alternately, one can consider introducing new relevant RRC parameters within the SRS resource (e.g., as followed in Proposal 2-3 for PUCCH). This allows to reduce the signaling overhead as well as not necessary to expand the number of SRS resource sets for a specific usage. Therefore, we would at least to propose to consider both alternatives and assess before agreeing to go in the direction mentioned in the proposal. |
| Spreadtrum | Support the intention, and also share same view with DCM’s comments in the first round. The “new RRC parameters in SRS-Config” may cause confusion. So “in SRS-Config” can be removed. The following is clear: Introduce new RRC parameters in SRS-Config to configure SRS resource sets for SBFD symbols. |

Proposal 2-5a

Proposed Agreement:

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), down-select from the following options.

- Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid. For PDSCH repetitions and multi-PDSCH scheduled by a single DCI, a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is dropped.

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

  - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only

  - For PDSCH repetitions, the number of PRBs for TBS determination is based on assigned PRBs.

  - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

|  | Company |
| --- | --- |
| Support | New H3C, CEWiT (Option 5), Tejas, Samsung, WILUS, Transsion |
| Not support | Nokia, NSB |

| Company | Comments |
| --- | --- |
| Moderator | The intention is to have unified solution for DL. So Proposal 2-5 and 2-6 are merged. |
| Sony | We have preference for Option 2, which is also considered for PUSCH. Option 5 is too crude in dropping packets which may lead to excessive loss of PDSCH. |
| Nokia, NSB | We understand the intention from the FL. A unified solution can be applied for all dynamic PDSCH. However, SPS and dynamic PDSCH have different characteristics in scheduling. For that reason, we don’t need to find a unified solution for SPS and dynamic PDSCH. We think the ideal middle ground is to apply - Option 1 for SPS PDSCH - Option 2 for dynamic PDSCH - Option 3 or Option 5 as default if Option 1 is not configured for SPS PDSCH (or Option 2 is not configured for dynamic PDSCH). |
| OPPO | We are fine to narrow down the candidate options to option 3 and option 5. However, for the TBS determination part under option 5, we see other candidate solutions besides the listed one, for example, for PDSCH repetition, it can be also based on the available PRB number for the first repetition. So at this stage we prefer to leave this part to FFS. |
| China Telecom | We prefer option 2 is supported using the RB offset to flexibly utilize the different available frequency resource in SBFD and non-SBFD symbols. It is the understanding of the spirit of Configuration 2 to use the available resource in different symbol types as much as possible. Option 5 can be combined with option 2, as when the shifted allocated resource by the RB offset still overlaps with RBs outside usable PRBs in SBFD symbols, only the shifted allocated resource within usable PRBs are valid. No difference between DUD and other patterns, as well as DL and UL, the allocated resource is shifted before determining the validity. |
| Ericsson | We believe that Option 3 is too restrictive and can lead to inefficient usage of DL radio resources, especially if the UE operates in Configuration 2. And Option 3 can rather be considered in the context of Configuration 1. Option 5 is natural choice for Configuration 2 to have a unified solution as it is also agreed to support only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH in other cases. In this context, we also note that Option 4 is more natural option for Configuration 1 and shall be considered in the proposal related to Configuration 1, if any. Under Option 5, second sub-bullet, for PDSCH repetitions, we also prefer to have - For PDSCH repetitions, FFS the number of PRBs for TBS determination is based on assigned PRBs. as in some cases it is desirable to choose the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs only. In that case, it may be good to replace three bullets with one with FFS the number of PRBs for TBS determination. |
| Spreadtrum | We prefer to discuss SPS PDSCH/PDSCH repetition and multi-PDSCH scheduled by a single DCI separately. For SPS PDSCH/ PDSCH repetition, Option 3 is simpler and preferred. For multi-PDSCH scheduled by a single DCI, the PDSCHs in different slots are separate TB scheduling and is more like single slot scheduling by single DCI. So it can use same solution as DG PDSCH with RA type 0/1 without repetition. We support Option 5 for multi-PUSCH scheduled by a single DCI. |
| CEWiT | We are not supportive of option 3 as it is too restrictive and leads to wastage of resources, as the resources in DL-SB are also dropped. Also, it will also impact the latency. Our preference is UE performing the operation using separate resources, which are explicitly configured or implicitly derived. |
| Tejas | We understand the intention from the FL. We prefer option 5. |
| Samsung | We are fine to further consider option 3 and option 5. Under option 5, we are unclear if different TBS can be supported for SPS PDSCHs associated to the same SPS configuration. |

Proposal 2-7a

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), down-select from the following options.

- Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) determination to determine resource for SBFD symbols

- Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid. A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is dropped or postponed.

  - For PUSCH repetition type-A with available slot counting and TBoMS, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is postponed.

  - For PUSCH repetition type-A without available slot counting and multi-PUSCH scheduled by a single DCI, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is dropped.

- Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid

  - For CG PUSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within UL usable PRBs only.

  - For PUSCH repetitions and TBoMS, the number of PRBs for TBS determination is based on assigned PRBs.

  - For multi-PUSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within UL usable PRBs only.

|  | Company |
| --- | --- |
| Support | Sony,New H3C,Tejas, Samsung, WILUS, Transsion |
| Not support | Nokia, NSB |

| Company | Comments |
| --- | --- |
| Moderator | The intention is to have unified solution for UL. So Proposal 2-7 and 2-8 are merged. |
| Sony | It will be good to have the same options for PDSCH as well. |
| Nokia, NSB | Similar view as for 2-5a, we think the ideal middle ground is to apply - Option 1 for Type 1 CG PUSCH - Option 2 for dynamic PUSCH and Type 2 CG PUSCH - Option 3 or Option 5 as default if Option 1 is not configured for Type 1 CG PUSCH (or Option 2 is not configured for dynamic PUSCH/Type 2 CG PUSCH). |
| OPPO | See our comments above. |
| China Telecom | Prefer CG PUSCH type 1 is discussed separately. For type 1 CG PUSCH configuration, option 1 is preferred. For option 2, as it is still not clear how to determine RB offset for RB offset(s) determination, prefer not to remove RB offset configuration/indication. One RB offset can be adequate, “(s)” can be removed for RB offset(s). |
| Ericsson | Similar comments as in Proposal 2-5a with respect to TBS determination. |
| Spreadtrum | Similar comments as Proposal 2-5a. For CG PUSCH / PUSCH repetition type-A/ TBoMS, Option 3 is simpler and preferred. For multi-PUSCH scheduled by a single DCI, we support Option 5. |
| CEWiT | Similar view as 2-5a |
| Tejas | We prefer option 5. Option 2 doesn’t guarantee that assigned PRBs are within the UL usable PRBs. We can have more than one option, to enable efficient use of UL usable PRBs. |
| Samsung | We are fine to further consider option 3 and option 5. Under option 5, we are unclear if different TBS can be supported for CG PUSCHs associated to the same CG configuration. |

Proposal 2-9a

Proposed Agreement:

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following.

- For semi-statically configured transmissions/receptions, the valid symbol type is explicitly configured by RRC.

- For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception.

- For SP-CSI on PUCCH or PUSCH, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in CSI-ReportConfig.

  - Option 2: The valid symbol type is determined based on the symbol type of the first PUSCH/ PUCCH after activation.

- For type 2 CG PUSCH and SPS PDSCH, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in ConfiguredGrantConfig/SPS-Config.

  - Option 2: The valid symbol type is determined based on the symbol type of the first CG PUSCH/SPS PDSCH after activation.

- For SPS HARQ-ACK, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in SPS-Config.

  - Option 2: The valid symbol type is determined based on the symbol type of the first PUCCH carrying SPS HARQ-ACK after activation.

- For Semi-persistent SRS, down-select from the following options:

  - Option 1: The valid symbol type is explicitly configured in SRS-Config.

  - Option 2: The valid symbol type is determined based on the symbol type of the first SRS after activation.

|  | Company |
| --- | --- |
| Support | Sony,New H3C, China Telecom, Spreadtrum, Tejas, Transsion, ITRI |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia, NSB | The determination for CSI-RS seems to be missing. In addition, please find our further comments on SRS and CSI-RS in the next proposal. |
| OPPO | To clarify, if we go with option 1 for both SPS PDSCH and SPS HARQ-ACK, then the explicit indication for SPS PDSCH and SPS HARQ-ACK is independent, right understanding? |
| Ericsson | We have a comment with respect to “For semi-statically configured transmissions/receptions, the valid symbol type is explicitly configured by RRC.” This is similar to having different configurations for different CG PUSCH/PUCCH, each for one symbol type. One could use similar rule to dynamic scheduled transmissions: valid symbol type is based on symbol type of the first transmission/reception, e.g., CG configure resource in both symbol types, valid symbol type is based on which symbol type UE starts the transmission. |
| CEWiT | Question for clarification in case of CSI-RS, whether valid symbol type is applicable for measurement on CSI-RS resources as well? E.g., if valid symbol type is configured or determined as SBFD then whether the intention is to restrict UE measurement in CSI-RS resources in SBFD symbol only and report (or) CSI reporting will happen only in SBFD symbols whereas the report can contain CSI for SBFD and non-SBFD |
| Tejas | We support the proposal. We prefer option 2 as it gives more flexibility. |
| Samsung | Prefer to make a unified rule, i.e., support either option 1 or 2 for all cases except DG PDSCH/PUSCH |

Proposal 2-10a

Proposed Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1,

- For PUSCH repetitions with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type.

- For CG PUSCH and SPS PDSCH, the transmission/reception occasions in the invalid symbol type are invalid for transmissions/receptions.

- For P/SP SRS, P/SP CSI-RS, P/SP PUCCH, PUSCH repetitions without available slot countingthe other transmissions/receptions except for SPS HARQ-ACK deferral, transmissions/receptions in the invalid symbol type are dropped.

- FFS SPS HARQ-ACK deferral in the invalid symbol type

|  | Company |
| --- | --- |
| Support | Sony,New H3C, China Telecom, Tejas, Samsung, Transsion |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia, NSB | We would like to understand further the FFS before agreeing to it. In addition, ignoring SRS and CSI-RS may have a significant impact, which may make Configuration 1 being a less interesting set-up. Therefore, we are not ready to agree to the proposal. |
| Ericsson | With respect to the first bullet, it is also possible that UE drops transmissions in the invalid symbol type for TBoMS, which is preferrable in our view. In the second bullet, it is good to clarify the UE behaviour, e.g., whether UE drops transmission/reception. |
| Spreadtrum | We think CG PUSCH and SPS PDSCH can be included in the third bullet. We prefer to combine the second bullet and third bullet as: For CG PUSCH, SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, PUSCH repetitions without available slot countingthe other transmissions/receptions except for SPS HARQ-ACK deferral, transmission/reception occasions in the invalid symbol type are dropped. |
| CEWiT | Similar comment as proposal 2-9a. In our understanding, the UE should measure CSI separately for SBFD and non-SBFD symbols and can report both or either one based on configuration. |

Proposal 2-11a

Proposed Agreement:

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters.

- Option 1: One TCI codepoint is mapped to two separate TCI states for SBFD symbols and non-SBFD symbols respectively

  - No RRC impact

  - New MAC CE introduced for activation of separate TCI states for SBFD and non-SBFD symbols

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

  - No impact on MAC CE

- Note: The maximum number of closed power control loops is not increased, i.e. up to 2 closed loops as legacy.

- FFS whether the maximum number of closed power control loops is increased to be more than 2.

- Note: If separate TCI states for SBFD and non-SBFD symbols are agreed to be supported,

  - Option 1 can support separate TCI states for SBFD and non-SBFD symbols without additional work.

  - For Option 2, FFS whether/how separate TCI states can be supported for SBFD and non-SBFD symbols

|  | Company |
| --- | --- |
| Support | New H3C, CEWiT (Option 1), Tejas, ITRI |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support in general. It’s unclear to us the motivation to increase the number of closed power control loops. |
| Ericsson | We would also like to propose third alternative due to its advantage in reduced overhead, i.e., - Option 3: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - A power offset with respect to open-loop power control is introduced for transmission in SBFD symbols to compensate for higher interference level for PUSCH, PUCCH and SRS. |
| Spreadtrum | From our perspective, whether to support separate spatial relation for SBFD/non-SBFD or not should be discussed first. If separate spatial relation for SBFD and non-SBFD is supported, the Option 1 can be supported naturally and Option 2 is not needed. Otherwise, the Option 2 should be supported. |
| Tejas | Support. We prefer option 2. |

Proposal 2-13a

For PUSCH FH in SBFD symbols, update the PUSCH frequency hopping formula according to one of the following options.

- Option 1: RBUL SB start+(RBstart+RBoffset)modNBWPsizeNUL SBsize

- Option 2: RBUL SB start+(RBstart-RBUL SB start+RBoffset)modNBWPsizeNUL SBsize

- Option 3: (RBstart+RBoffset)modNBWPsizeNUL SBsize

  - For Option 3, the above result provides the RB offset with reference to the start of UL usable PRB

Where RBUL SB start is the starting PRB index of the UL usable PRB within UL BWP and NUL SBsize is the size of UL usable PRB

- Applicable at least to intra-slot FH and inter-slot FH

  - FFS inter-repetition FH

|  | Company |
| --- | --- |
| Support | Nokia, NSB,New H3C, CEWiT, Tejas, Samsung, Transsion |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia, NSB | It’s good to clarify the refence of the RBstart in each Option. |
| China Telecom | For the reference point of RBstart, is it the starting PRB of UL BWP in each Option? |
| Ericsson | We would like to have the following clarifications: - How the PUSCH frequency hopping formula translates to odd and even SBFD slots? - Whether same or different slot counters for SBFD and non-SBFD slots for repetitions Is RBstart corresponding to the starting RB non-SBFD symbol indicated according to the legacy FDRA? |
| Spreadtrum | For Option 3, we think it should be revised as - Option 3: (RBstart-RBUL SB start+RBoffset)modNBWPsizeNUL SBsize - For Option 3, the above result provides the RB offset with reference to the start of UL usable PRB RBstart-RBUL SB start represents offset between PUSCH starting RB of the first hop relative to the start of UL usable PRB, which is equivalent to use start of UL usable PRB as reference point for the PUSCH starting RB of the first hop. In this way, revised Option 3 can provide the RB offset with reference to the start of UL usable PRB. |
| Tejas | Option 3 is preferred with the updated definition of RBstart and RBoffset. |
| Samsung | As we mentioned in the first round, we think the starting PRB index without FH and the starting PRB index with FH and RBoffset=0 should be same. With this understanding, option 2 is preferred. |

    - Collision handling

Proposal 3-1

Proposed Agreement:

For SBFD-aware UE transmission and reception in an SBFD symbol, support Option 2 to determine whether to transmit or to receive in the SBFD symbol.

- Option 2: link direction is indicated by gNB explicitly.

  - Link direction is indicated by dedicated RRC signalling.

  - Link direction indication by DCI is not supported.

|  | Company |
| --- | --- |
| Support | Spreadtrum, CEWiT, Tejas, Sony, ITRI, LGE， CMCC, CATT |
| Not support | ZTE, QC, Panasonic, Samsung, OPPO, DOCOMO, Nokia, NSB, Ericsson, Transsion |

| Company | Comments |
| --- | --- |
| ZTE | On top of Option 1, it is unclear to us why we need to additionally support Option 2. |
| Tejas | Support in general. Link direction indication via dedicated RRC signaling will address semi static collision cases. But how will the given option address dynamic collision cases? |
| Sony | There needs a mechanism to implicitly overwrite the semi-statically link direction especially for cases where there is no collision. |
| Panasonic | We share ZTE's view that the necessity to support option 2 would be unclear. |
| DOCOMO | Similar view as ZTE. |
| Nokia, NSB | Before agreeing to this, we first need to discuss more details on the expected UE behaviour for Option 2. We disagree with Option 2 if the intention is to assign a pre-defined and strict link direction to a set of SBFD symbols as this would diminish most of the benefits of SBFD. Instead, we think there are benefits of indicating to the UE a “preferred” link direction which is only applicable e.g. in case of collisions between UL and DL, while UEs can still be scheduled in either direction in SBFD slots as long as there is no collision. |
| CMCC | We support explicit link direction indication. From UE perspective, SBFD symbol is similar to the legacy flexible symbol in R15, and link direction indication is similar to the legacy TDD slot configuration indications (e.g., Tdd-UL-DL-ConfigurationDedicated and SFI in DCI format 2_0), i.e., which is potential to easy gNB and UE implementation. Furthermore, if only scheduling signalling based solution is supported, UE needs to monitor DCI on each PDCCH monitoring occasion which is not beneficial to UE power saving. In addition, there may be some UL/DL collision cases in the same SBFD symbol, e.g., higher layer configured DL reception and UL transmission, and the scheduling DCI based solution cannot solve this issue. |
| Moderator | Inputs from Round 1 are copied above. Please provide additional inputs, if any. |
| OPPO | We do not understand why link direction can simply UE implementation since even if link direction is supported, gNB may not provide link direction to UE, so UE anyway needs to implement collision handling based on gNB scheduling and configurations. |
| China Telecom | Semi-static link direction indication restricts the UE tx/rx behaviour even there is no collision, the benefit is unclear to us. |
| Ericsson | This is an unnecessary proposal/agreement and not in line with companies’ majority view. The specification already includes methods for direction resolution, hence this proposal is not needed. |
| CEWiT | Explicit indication of link direction is inline with the existing NR procedure (UE first receive DL/UL configuration with TDD configuration/SFI and then scheduling). Also, it provides enough time for the UE to adjust the hardware and RF components for performing transmission/reception. Further, it simplifies the collision handling process. |

Proposal 3-3a

Proposed Agreement:

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 6 (Dynamic or semi-static DL vs. valid RO), reuse the existing collision handling rules for HD-FDD RedCap UEs.

- UE does not expect collision between PRACH triggered by PDCCH order and dynamically scheduled DL reception.

- For collision between PRACH triggered by PDCCH order and semi-statically configured DL reception, UE does not receive DL channel/signal.

- For collision between PRACH triggered by higher layer and DL channel/signal, dynamically scheduled DL reception, down-select from the following options:

  - Option 1: leave it to UE implementation whether to receive the DL or transmit PRACH.

  - Option 2: PRACH transmission is cancelled subject to existing timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum

- For collision between PRACH triggered by higher layer and semi-statically configured DL reception, down-select from the following options:

  - Option 1: leave it to UE implementation whether to receive the DL or transmit PRACH.

  - Option 2: UE does not expect to be configured to receive DL overlapping with valid RO in time domain.

|  | Company |
| --- | --- |
| Support | Spreadtrum (first two bullets), CEWiT, Tejas, ITRI |
| Not support |  |

| Company | Comments |
| --- | --- |
| Spreadtrum | For the new changes, we prefer the original proposal, left to UE implementation, which is also aligned with HD FDD. If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks within the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the PL RS, or the PDCCH, or the SS/PBCH blocks.. |

Proposal 3-4a

Proposed Agreement:

Update the following agreement made in RAN1#117 meeting:

If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case.

- An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol

- An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol

  - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/0B/1/2 CSS set

    - FFS: Additional cases

    - No additional spec impact is expected to support joint operation of MBS and SBFD.

|  | Company |
| --- | --- |
| Support | Spreadtrum |
| Not support |  |

| Company | Comments |
| --- | --- |
| Sony | Do we expect no additional specs impact for MBS and SBFD for collision cases or for all MBS aspects? |

- Contact person

Please provide/update the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| Apple | Ali Fakoorian | sfakoorian@apple.com |
| CATT | Yanping Xing | xingyanping@catt.cn |
| CEWiT | Deepak P M | deepakpm@cewit.org.in |
| CMCC | Tuo Yang Fei Wang | yangtuo@chinamobile.com wangfei@chinamobile.com |
| CT | Nanxi Li | linanxi@chinatelecom.cn |
| DOCOMO | Qiping Pi Hiroki Harada | piqp@docomolabs-beijing.com.cn hiroki.harada.sv@nttdocomo.com |
| Ericsson | Magnus Åström Narendar Madhavan Ratheesh Kumar Mungara | magnus.astrom@ericsson.com narendar.madhavan@ericsson.com ratheesh.kumar.mungara@ericsson.com |
| ETRI | Hoondong Noh Cheolsoon Kim Junhyeong Kim | hoondong.noh@etri.re.kr cs.kim@etri.re.kr jhkim41jf@etri.re.kr |
| Fujitsu | Qinyan Jiang | jiangqinyan@fujitsu.com |
| Google | Abdellatif Salah Kaopeng Chou | asalah@google.com nevillechou@google.com |
| Huawei | Xinghua Song Zhiheng Guo | songxinghua@huawei.com guozhiheng@huawei.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| ITRI | Jen-Hsien Chen | itriA40175@itri.mail.org.tw |
| Langbo | Liu Zheng | liuzheng@langbomobile.com |
| LG | Hyunsoo Ko Hyangsun You | hyunsoo.ko@lge.com sssun.you@lge.com |
| MediaTek | Mohammed Al-Imari | Mohammed.Al-Imari@mediatek.com |
| NEC | Pravjyot Singh Deogun Frank Zhang | pravjyot.deogun@emea.nec.com zhang_bohang@nec.cn |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Nokia, NSB | Jingyuan Sun Quang Nhan | jingyuan.sun@nokia-sbell.com nhat-quang.nhan@nokia.com |
| OPPO | Yi Zhang Wenfeng Zhang | zhangy@oppo.com zhangwenfeng@oppo.com |
| Panasonic | Tomoya Nunome Hidetoshi Suzuki | nunome.tomoya@jp.panasonic.com suzuki.hidetoshi@jp.panasonic.com |
| Qualcomm | Muhammad Abdelghaffar | mabdelgh@qti.qualcomm.com |
| Samsung | Marian Rudolf Kyungjun Choi Sa Zhang Zhe Chen | m.rudolf@samsung.com kyungj.choi@samsung.com sa.zhang@samsung.com tom.chenzhe@samsung.com |
| Spreadtrum | Zhongdan Zhang Huan Zhou | Zhongdan.Zhang@unisoc.com Huan.Zhou@unisoc.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| TCL | Shahid Jan | shahid.jan@tcl.com |
| Tejas | Abhijith Barangi Gopalakrishna | abhijithb@tejasnetworks.com |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| vivo | Na Li Xiaohang Chen | lina5g@vivo.com chenxiaohang@vivo.com |
| Xiaomi | Lei Wang | wanglei25@xiaomi.com |
| ZTE | Xianghui Han | han.xianghui@zte.com.cn |

- Reference

- TR 38.858		Study on Evolution of NR Duplex Operation		Rapporteur (CMCC)	V18.0.0

- RP-234035		New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)	CMCC (Moderator, RAN1 VC)

- RP-240789		WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)	Huawei

- RP-241614		Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)		Huawei

- R4-2406107		Way Forward for [110bis][313] NR_duplex_evo	Samsung	RAN4#110bis

- RP-220802		WI summary for WI Core part: Further enhancements on MIMO for NR	Samsung	RAN#95e

- R1-2103809		Summary for [104b-e-NR-7.1CRs-03] Discussion on collision handling for PRACH transmission		Moderator (vivo)		RAN1#104b-e

- R1-2405815		Discussion on SBFD TX/RX/measurement procedures	MediaTek Inc.

- R1-2405831		Discussion for SBFD TX RX measurement procedures	New H3C Technologies Co., Ltd.

- R1-2405847		On subband full duplex design	Huawei, HiSilicon

- R1-2405907		Discussion on SBFD TX/RX/measurement procedures	Spreadtrum Communications

- R1-2405933		Discussion on transmission, reception and measurement procedures for SBFD operation	ZTE Corporation, Sanechips

- R1-2405940		Discussion for SBFD TX/RX/ measurement procedures	Tejas Networks Limited

- R1-2405984		Discussion on SBFD TX/RX/measurement procedures	CMCC

- R1-2406059		Discussion on SBFD TX/RX/measurement procedures	LG Electronics

- R1-2406087		Discussion on SBFD TX/RX/measurement procedures	China Telecom

- R1-2406102		Discussion on SBFD TX/RX/measurement procedures	TCL

- R1-2406134		SBFD TX/RX/measurement procedures	Ericsson

- R1-2406181		Discussion on Rel-19 SBFD operation	vivo

- R1-2406209		Discussion on SBFD TX/RX/measurement procedures	Kookmin University

- R1-2406237		Discussion on SBFD Tx/Rx/measurement procedures		OPPO

- R1-2406283		Discussion on reception and transmission procedure for SBFD operation	Xiaomi

- R1-2406314		Discussion on Tx/Rx/measurement procedures for SBFD operation	Fujitsu

- R1-2406367		Discussion on SBFD TX/RX/measurement procedures	CATT

- R1-2406470		SBFD Operations		Sony

- R1-2406568		Discussion on SBFD TX/RX/measurement procedures	Panasonic

- R1-2406578		Discussion on SBFD TX/RX/measurement procedures	HONOR

- R1-2406596		Discussions on SBFD TX/RX/measurement procedures	Ruijie Networks Co. Ltd

- R1-2406598		Views on SBFD TX/RX Measurement Procedures	Lenovo

- R1-2406649		SBFD operation and procedures	Samsung

- R1-2406684		On SBFD TX/RX/measurement procedures	Nokia, Nokia Shanghai Bell

- R1-2406691		Discussion on subband non-overlapping full duplex Tx-Rx and measurement operations	NEC

- R1-2406702		Discussion on SBFD operation	Transsion Holdings

- R1-2406707		PRG allocation for SBFD	ASUS

- R1-2406725		Discussion on SBFD TX/RX/measurement procedures	ETRI

- R1-2406749		Discussion on TX RX and measurement procedures for SBFD operation	InterDigital, Inc.

- R1-2406800		SBFD Tx/Rx/measurement aspects		Sharp

- R1-2406835		Views on SBFD TX/RX/measurement procedures	Apple

- R1-2406929		Discussion on SBFD TX/RX/measurement procedures	NTT DOCOMO, INC.

- R1-2406963		Discussion on SBFD TX/RX/measurement procedures	Google Ireland Limited

- R1-2407028		SBFD Transmission, Reception and Measurement Procedures	   Qualcomm Incorporated

- R1-2407067		Discussion on SBFD TX/RX/measurement procedures	ITRI

- R1-2407084		Discussion on SBFD TX/RX/measurement procedures	CEWiT

- R1-2407158		Discussion on SBFD TX/RX/measurement procedures	WILUS Inc.

Appendix A: Rel-19 WI agreements

RAN1#116

Agreement

For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier.

- FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

Agreement:

For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options.

- Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- FFS: Further details

FFS: Details when two TDD-UL-DL patterns are configured

Agreement

A slot can consist of SBFD symbols and non-SBFD symbols.

Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.

The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

For semi-static indication of SBFD subband frequency location, down-select from the following options.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Agreement

A slot can consist of SBFD symbols and non-SBFD symbols.

For semi-static indication of SBFD subband time location,

- When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period.

  - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon

  - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot.

  - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS.

  - FFS details

Agreement

The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

  - Subject to RAN4 guidance on the size of subband/guardband, if any

  - FFS reference starting RB and reference SCS

Agreement

For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs.

For determining UL/DL usable PRBs, consider the following options.

- Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

- Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

Agreement

For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL usable PRBs are allowed

  - FFS SSB symbols

- DL receptions within DL usable PRBs are allowed

- UL transmissions outside UL usable PRBs are not allowed

- DL receptions outside DL usable PRBs are not allowed

  - This restriction is not applicable for CLI measurement

CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3.

RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0

- DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

Agreement

For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Other options are not precluded.

Agreement:

For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary:

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 6: Dynamic or semi-static DL vs. valid RO

Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

RAN1#116bis

Agreement

A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling.

A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

Agreement

For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

Agreement

Study the feasibility and enhancements to support separate power control and/or spatial relation for SRS, PUCCH and PUSCH in SBFD and non-SBFD symbols in different slots, including repetition and non-repetition, by considering existing schemes, e.g. multi-TRP PUCCH/PUSCH repetition schemes.

Agreement

For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS, discuss and decide whether/which of the following options is supported.

- Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only

  - FFS: DMRS sequence mapping

- Option 1-2: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs as legacy

  - FFS: DMRS sequence mapping

- Option 2: Introduce new RB indexing/PRB bundle indexing to ensure VRBs are mapped to DL usable PRBs only.

  - Existing VRB-to-PRB mapping is reused

  - Legacy TBS determination method is used

  - FFS: DMRS sequence mapping

- Option 3: Modify VRB-to-PRB mapping interleaver to ensure VRBs are mapped to DL usable PRBs only.

  - Existing RB indexing/PRB bundle indexing is reused

  - If the interleaver is not enabled, Option 1-1 or Option 1-2 is used

  - Legacy TBS determination method is used

  - FFS: DMRS sequence mapping

Agreement

For cell-specific configuration of frequency locations of SBFD subbands,

- Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList.

  - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

Agreement

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate resource allocations for SBFD symbols and non-SBFD symbols

  - FFS other separate configurations for SBFD symbols and non-SBFD symbols

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid

- Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid for SPS PDSCH

- Other options are not precluded

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols

  - FFS type 2 CG PUSCH

  - FFS other separate configurations for SBFD symbols and non-SBFD symbols

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid

- Option 4: Only CG PUSCH transmission occasion in one symbol type is valid and CG PUSCH transmission occasion in the other symbol type is invalid

- Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid for CG PUSCH

- Other options are not precluded

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal.

- The above does not imply link direction indication is supported

- FFS on dynamically scheduled UL transmission with repetition

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met.

- The above does not imply link direction indication is supported

- FFS on dynamically scheduled DL reception with repetition

Agreement

For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid, e.g. the PDSCH in the slot is dropped

- Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid

- Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid

- Option 6: gNB does not schedule any PDSCH in SBFD symbols in a slot to be overlapping with PRBs outside DL usable PRBs

- Other options are not precluded

- FFS: Applicable conditions

Agreement

For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols, and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed

- Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid

- Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid

- Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs

- Other options are not precluded

- FFS: Applicable conditions

RAN1#117

Agreement

For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options.

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

  - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only.

    - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

Working Assumption

For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs.

  - No impact on CSI-RS sequence generation

  - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

  - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

Agreement

For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols

- Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework.

Agreement

For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware.

  - No impact on CSI-RS sequence generation

  - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Agreement

For a CSI reporting subband which overlaps with at least one PRB within DL usable PRBs and one or more PRBs outside DL usable PRBs, the CSI reporting subband includes PRB(s) within DL usable PRBs only.

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 4 (dynamically scheduled DL reception vs. dynamically scheduled UL transmission) in a SBFD symbol for SBFD-aware UEs, it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions.

- FFS DL reception with repetitions and/or UL transmission with repetitions

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case.

- An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol

- An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol

  - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set

    - FFS: Additional cases

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations.

  - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

  - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

  - FFS granularity of the configuration, e.g. per UE, per channel/signal etc.

  - FFS whether support of configuration 2 is subject to UE capability

Agreement

For RRC connected mode UEs, SBFD subband time period:

- When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

Agreement

For cell-specific indication of SBFD subband frequency location, adopt the following option.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

Agreement

For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4},

- Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only

  - FFS: DMRS sequence mapping

- FFS: when PRG is determined as wideband

- FFS: partial PRG

Agreement

Support separate FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols respectively.

- FFS: How to indicate/determine the FH offsets for PUSCH transmissions in SBFD and non-SBFD symbols, respectively.

- FFS: Whether/how to update FH calculation to only consider the UL usable PRBs

Agreement

For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

Agreement

Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission

- FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.