3GPP TSG RAN WG1 Meeting #118-bis	R1-2407616

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.1.1

Source:	Futurewei

Title:	Discussion on specification support for AI/ML-based beam management

Document for:	Discussion and decision

# Introduction

In RAN1#118 meeting, the Rel-19 NR_AIML_Air WID [1] was discussed. The following was agreed [2] on the topic of specification support for AI/ML-based beam management.

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) Agreement For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted.

In this contribution, we present our views on specification support for AI/ML-based beam management and proposals for moving forward.

# Specification Support for AI/ML-based BM

In technical report TR 38.843 [3], the physical layer aspects of the potential specification changes/impact were categorized into four areas: performance monitoring related, model inference related, data collection related, and assistance information related. In this section, we discuss the potential specification impact in each of the four areas.

## Performance Monitoring Related

In RAN1 #117 meeting, it was agreed to support Type 1 performance monitoring, including both Option 1 (NW-side performance monitoring) and Option 2 (UE-assisted performance monitoring), for BM-Case1 and BM-Case2 with a UE-side AI/ML model.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In our view, one design principle of specification support for Rel-19 AI/ML-based BM is that it should reuse the existing CSI framework as much as possible to reduce the required specification effort.  To that end, Type 1 Option 1 is preferred as it can mostly reuse the existing CSI framework.

In RAN1 #118 meeting, the following was further agreed regarding performance monitoring for Type 1 Option 2:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

As indicated in the agreement above, to complete the design of Type 1 Option 2, one of the four alternatives is to be selected as performance metric for Type 1 Option 2.  In our views, Alt 1 is the best among the four alternatives.  First of all, Alt 1 can be applicable to models with or without RSRP prediction capability, whereas Alt 3 can only be applicable to models that can predict RSRP.  Regarding Alt 2, it is to compare measured L1-RSRP of Top K predicted beam(s) and measured L1-RSRP of a resource set (or resources) for monitoring.  However, the Top K predicted beam(s) could be different from the monitored resource set/resources.  It is unclear how well the difference between measured L1-RSRP of different beams can indicate AI/ML model performance.  Regarding Alt 4, it alone cannot serve as performance metric since it only involves the probability information of the predicted beam(s) to be the Top 1 or Top K beam without a comparison to actual measurement.  Furthermore, it is only applicable to models that can generate probability information.  Therefore, Alt 1 is better than the other three alternatives and should be supported for Type 1 Option 2.

Regarding how to configure the resource set/resources for monitoring, in Alt 1, in order to obtain the true Top 1 or Top K beams based on the measurements, the full set of Set A should be configured for measurement, preferably with a much longer period than Set B to reduce overhead.

Based on the above analysis, we propose the following:

Proposal 1: For Rel-19 AI/ML-based BM, for BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), support

- Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring

- The full set of Set A is configured as the resource set/resources for monitoring

## Model Inference Related

Network-sided Model

For network-sided model, model training, inference, and performance monitoring are all conducted at the network side.  The NW can configure the Set A and Set B beams using the existing CSI framework, e.g., through CSI-ResourceConfig, CSI-ReportConfig, and CSI-MeasConfig.  How the Set A and Set B beams are configured, e.g., which resource set is for Set A and which is for Set B beams and how they are associated, are transparent to the UE.  The UE just follows the CSI configurations to report the L1-RSRP(s) and the corresponding CRI/SSBRI(s).  The NW can then use the received L1-RSRP reports to perform model training, inference, and/or performance monitoring based on its implementation.

In RAN1 #117 meeting, the following was agreed regarding beam report for network-sided model:

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

Regarding the first FFS, it requires that the reported beam(s) should have L1-RSRPs within X dB gap to the largest measured value of L1-RSRP, therefore it allows the UE to not report beams with low L1-RSRP.  In our view, this can reduce reporting overhead and therefore should be supported.  Since the number of reported beams is variable, it can be indicated as part (e.g., a field) of the beam report.

Regarding the maximum value of M, in our view, increasing the maximum value from the existing “4” to “8” could be a good starting point.

As indicated in the agreement above, the beam information is also FFS.  In our view, the existing CSI framework should be reused as much as possible, and in this case, the beam information should be CRI/SSBRI.

Based on the above analysis, we propose the following:

Proposal 2: For Rel-19 AI/ML-based BM, at least for inference for network-sided AI/ML model of BM-Case1, for the content in a beam report in L1 signaling,

- Support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and the number of reported beams is indicated in the beam report.

- The beam information is CRI/SSBRI.

- Increase the maximum value M from the existing “4” to “8” as a starting point.

UE-sided Model

For UE-sided model, model inference is conducted at the UE side.  UE needs to be aware of the configuration of Set A and Set B beams and the association between them in order to perform model inference and report the predicted Top K beam(s) from Set A.  The indication of the association/mapping of beams within Set A and beams within Set B is also helpful to ensure consistency between training and inference at the UE.

In RAN1 #116-bis meeting, the following was agreed regarding configuration of Set A and Set B for interference results reporting for UE-sided model:

Agreement For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded.

Regarding Alt 1, since there is no explicit configuration for Set A, information about Set A needs to be derived from Set B.  To our understanding, in Alt 1, Set B has a set of wide beams (e.g., SSB resources), and Set A is determined as the set of CSI-RS (e.g., with narrow beams) that is QCLed with the SSBs in Set B with respect to QCL-TypeD.  However, in our opinion, Alt 1 has too stringent restriction on the configuration of Set A and Set B, and it is unclear to us how it works for the case where Set B is a subset of Set A.

Regarding Alt 2, with one CSI-ResourceConfigId configured for both Set A and Set B, there are a few options to configure Set A and Set B in CSI-ResourceConfig.

The first option is to configure two resource sets in one CSI-ResourceConfig, one for Set A and one for Set B, respectively.  For example, the first resource set configured in CSI-ResourceConfig is for Set B and the second one configured in the same CSI-ResourceConfig is for Set A.  The association of Set A and Set B is implicitly indicated by the fact that these two resource sets are configured in the same CSI-ResourceConfig.

The second option can be applied when Set B is a subset of Set A.  In this option, one resource set, which is for Set A, is configured in one CSI-ResourceConfig.  The indication of Set B and its association with Set A can be simplified by taking advantage of the fact that Set B is a subset of Set A.  For example, Set B can be indicated by a bitmap, where each bit in the bitmap is associated with each beam (e.g., CSI-RS/SSB resource) in Set A, respectively.  The value of “1” or “0” of each bit in the bitmap indicates whether the associated beam in Set A is for Set B or not.  This bitmap can be included in the CSI-ResourceConfig that configures the resource set for Set A to indicate the association/mapping of Set A and Set B.  In another example, if Set B is obtained by sampling Set A with a regular interval, Set B can be indicated by the starting offset of the sampling within Set A and the sampling interval.  For example, let’s assume there are a total of 32 beams in Set A and they are labeled as Beams [#0, #1, … #31].  Furthermore, there are 8 beams in Set B, and they are obtained by sampling the beams in Set A with an interval of 4.  A Set B that includes Beams [#1, #5, #9, …, #29] can then be indicated by the starting offset Beam #1 together with the sampling interval of 4.  Similarly, the starting offset and sampling interval can be included in the CSI-ResourceConfig that configures the resource set for Set A to indicate the association/mapping of Set A and Set B.

Comparing the first option and the second option, the second option can reduce the signaling overhead in the case where Set B is a subset of Set A.

From the above, Alt 2 can work for both cases in BM-Case1 and BM-Case2 that either Set A and Set B are different, or Set B is a subset of Set A.

Regarding Alt 3, with two CSI-ResourceConfigIds configured for Set A and Set B separately, to establish the association between Set A and Set B, some sort of explicit indication is needed.  For example, the CSI-ResourceConfigId of Set A can be indicated in the same CSI-ReportConfig for Set B.

Regarding Alt 4, one CSI-ResourceConfigId is configured for Set B, and Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId.  In our view, Alt 4 is unnecessarily complicated as it requires developing new scheme for resource set configuration while the existing CSI resource set configuration mechanism can still be used.

Based on the above analysis, we propose the following:

Proposal 3: For Rel-19 AI/ML-based BM, for UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting.  On the details in the CSI-ReportConfig, further consider Alt 2 and Alt 3:

- Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B.

- Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately.

In RAN1 #116 and #116-bis meeting, the following was agreed regarding content in the report of inference results for UE-sided model:

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction. Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded.

As indicated in the first agreement above, Opt 1 and Opt 2 are supported while Opt 3 and Opt 4 are still FFS.  Both Opt 3 and Opt 4 require defining new metrics in the spec, e.g., probability information of predicted Top K beam(s) and confidence information for the RSRP for Opt 3 and Opt 4, respectively.  In our views, it is difficult to define and test these new metrics and it will take significant effort and time of the group to agree on these new metrics while the benefit of reporting these new metrics is unclear.

Based on the above analysis, we propose the following:

Proposal 4: For Rel-19 AI/ML-based BM, for UE-sided model, at least for BM-Case1, for content in the report of inference results, do NOT support Opt 3 and Opt 4.

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

- Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

Regarding the RSRP of predicted Top K beam(s) in the report of inference results, Option A in the second agreement above is to always report predicted RSRP while Option B is to report measured L1-RSRP when it is available and predicted RSRP otherwise.  In our view, measured L1-RSRP is in general more accurate than the predicted RSRP and therefore should be reported when available.

Based on the above analysis, we propose the following:

Proposal 5: For Rel-19 AI/ML-based BM, for report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, support Option B

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.

## Data Collection Related

It was mentioned in [3] that for UE-sided AI/ML model, UE should report to NW its supported/preferred configurations of DL RS transmission.  While we support this new signaling, we prefer a simple mechanism to reduce the complexity related to an otherwise large number of possible DL RS configurations and the related signaling overhead.  For example, NW can provide/signal multiple possible configurations of DL RS transmission to the UE and the UE can then report its supported/preferred one(s) out of these configurations.  A few bits should be sufficient for the related signaling overhead from the UE reporting.

Based on the above analysis, we propose the following:

Proposal 6: For Rel-19 AI/ML-based BM, for data collection for UE-sided AI/ML model of BM-Case1 and BM-Case2, support that NW provides/signals multiple possible configurations of DL RS transmission to the UE and the UE reports its supported/preferred one(s) out of the multiple configurations.

Regarding data collection related signaling/configuration/measurement/report, there has been a discussion in Rel-18 SI FS_NR_AIML_Air on defining new assistant information, e.g., TX beam ID, for data collection.  In our view, this new TX beam ID is not necessary and instead, the RS ID should be used as implicit indication of the beam ID.  In general, we prefer reusing the legacy L1-RSRP reporting including CRI and/or SSBRI as much as possible.

Based on the above analysis, we propose the following:

Proposal 7: For Rel-19 AI/ML-based BM, for data collection for AI/ML model of BM-Case1 and BM-Case2, support using RS ID as implicit indication of beam ID and reusing L1-RSRP reporting as much as possible.

## Assistance Information Related

Regarding assistance information, it was mentioned in [3] that RAN1 has no consensus to support information such as UE location, UE moving direction, UE Rx beam shape/direction, NW-side beam shape information, etc.  On the other hand, it was mentioned that for BM-Case1 and BM-Case2 with a UE-sided AI/ML model, consistency / association of Set B beams and Set A beams across training and inference is beneficial from performance perspective.

In RAN1 #118 meeting, the following was agreed regarding consistency for additional condition for UE-sided model:

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

As indicated in the agreement above, associated ID is supported. However, it is still a working assumption that the associated ID at least can be configured within CSI framework.  In our views, since the associated ID is to make sure consistency of NW-side additional condition of Set A beams and Set B beams across training and inference, which involves CSI measurement of Set A and Set B beams, it is nature that the associated ID can be configured within CSI framework, e.g., in CSI-ReportConfig which configures the measurement of Set A and Set B.  Therefore, we support confirming the above working assumption.

Based on the above analysis, we propose the following:

Proposal 8: For UE sided model in beam management, confirm the following working assumption

[Working Assumption]

- The associated ID at least can be configured within CSI framework

- FFS on details

- FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

# Conclusions

In this contribution, we present our views on specification support for AI/ML-based beam management.  Based on the discussions in the previous sections we propose the following:

Proposal 1: For Rel-19 AI/ML-based BM, for BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), support

- Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring

- The full set of Set A is configured as the resource set/resources for monitoring

Proposal 2: For Rel-19 AI/ML-based BM, at least for inference for network-sided AI/ML model of BM-Case1, for the content in a beam report in L1 signaling,

- Support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and the number of reported beams is indicated in the beam report.

- The beam information is CRI/SSBRI.

- Increase the maximum value M from the existing “4” to “8” as a starting point.

Proposal 3: For Rel-19 AI/ML-based BM, for UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting.  On the details in the CSI-ReportConfig, further consider Alt 2 and Alt 3:

- Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B.

- Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately.

Proposal 4: For Rel-19 AI/ML-based BM, for UE-sided model, at least for BM-Case1, for content in the report of inference results, do NOT support Opt 3 and Opt 4.

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

- Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

Proposal 5: For Rel-19 AI/ML-based BM, for report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, support Option B

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.

Proposal 6: For Rel-19 AI/ML-based BM, for data collection for UE-sided AI/ML model of BM-Case1 and BM-Case2, support that NW provides/signals multiple possible configurations of DL RS transmission to the UE and the UE reports its supported/preferred one(s) out of the multiple configurations.

Proposal 7: For Rel-19 AI/ML-based BM, for data collection for AI/ML model of BM-Case1 and BM-Case2, support using RS ID as implicit indication of beam ID and reusing L1-RSRP reporting as much as possible.

Proposal 8: For UE sided model in beam management, confirm the following working assumption

[Working Assumption]

- The associated ID at least can be configured within CSI framework

- FFS on details

- FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

# References

RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103.

3GPP RAN1, RAN1#118, Chairman Notes.

TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”.