3GPP TSG-RAN WG1 Meeting #118bis	R1-2407641

Hefei, China, 14th – 18th October 2024

Agenda Item:	9.4.2.4

Source:	Ericsson

Title:	Waveform characteristics of carrier wave provided externally to the Ambient IoT device

Document for:	Discussion, Decision

# 1	Introduction

The “Study on solutions for Ambient IoT (Internet of Things) in NR” [1][2][3] targets a further assessment at RAN WG level of Ambient IoT (A-IoT), a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very low-end IoT applications. The study follows an initial study captured in TR 38.848 [4].

In this contribution, we present our views on performance of single-tone CW with frequency hopping, D2R spectrum efficiency, and CW transmission cases with large frequency shift.

# 2	Characteristics of CW waveforms

## 2.1	Reception performance of single-tone with frequency hopping

In RAN1#118, an observation regarding D2R performance comparison between single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform was made. Due to limited time, the performance of single-tone unmodulated sinusoid waveform with frequency hopping has not been discussed yet. Here, we provide our simulation results based on FSK backscatter modulation.

The evaluation assumptions are summarized in Table 1.

Table 1: The evaluation assumptions and corresponding values

| Parameter | Value |
| --- | --- |
| CW type | - Single-tone sine wave with frequency hopping - Single-tone sine wave without frequency hopping - Unmodulated 2-tone CW |
| Baseband modulation frequency (Backscatter Link Frequency) | 15 kHz for “0” and 30 kHz for “1” (FSK) |
| A-IoT device modulation method | FSK |
| Receiver | FFT-based Non-coherent detector for FSK |
| Interference | 0 dB, i.e., Interference signal power is the same as the received backscattered signal power. |
| Channel type | TDL-A, delay spread of 150 ns |
| Bandwidth of a single-tone CW fSCS | 15 kHz |
| Separation between adjacent tones Δf | frequency hopping offset for single-tone CW: 1.65MHz 2-tone CW: 1.65 MHz |

Figure 1: Detection performance of backscattered signals with FSK

Figure 1 shows the performances of the different CW waveforms assuming a non-coherent detector which detects a bit by comparing the power of the received shifted tones corresponding to zeros and those corresponding to ones.

- Option 1, Single-tone sine wave without frequency hopping

- Option 2, Single-tone sine wave with frequency hopping

- Option 3, Unmodulated 2-tone CW

It is shown that Option 1 has the worst performance in a fading channel. Frequency hopping with an offset of 1.65 MHz in Option 2 can improve the performance by 1dB. Option 3 which backscatters a two-tone CW with the same frequency offset in between outperforms Option 1 by 5dB.

We also simulated the three options together with bit-level repetition with repetition factor of 2. For both Option 1 and Option 3, the repetitions can lead to a 2-3dB improvement, and this improvement is mainly contributed by power aggregation. For Option 2 without repetition, each information bit is transmitted in one of the two hops, illustrated in Figure 2, which maps down to each bit being transmitted on a single tone with single frequency, and thus, not exploiting frequency diversity. With repetition, the two repetitions are distributed on two hops, meaning that the transmission of each bit makes use of frequency diversity. This repetitions in Option 2 can lead to nearly 6dB improvement.

|  | Option 1 |  |  |  | Option 2 |  |  |  | Option 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| hop2 |  |  |  |  |  | i2 |  | i1 |  | i2 |
| hop1 | i1 | i2 |  | i1 |  |  |  | i1 |  | i2 |

- Without bit-level repetition

|  | Option 1 |  |  |  |  |  |  | Option 2 |  |  |  |  |  |  | Option 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| hop2 |  |  |  |  |  |  |  |  | i1 |  | i2 |  |  | i1 |  | i1 | i2 | i2 |
| hop1 | i1 | i1 | i2 | i2 |  |  | i1 |  |  | i2 |  |  |  | i1 |  | i1 | i2 | i2 |

- With bit-level repetition

Figure 2: Depicting options 1, 2 and 3 with and without bit-level repetition.

- In fading channels with FSK, two-tone CW signal outperforms a single-tone CW signal for the backscattered signal.

- In fading channels with FSK, for backscattering based on 2-hop single-tone signal, to exploit the benefits of frequency diversity, repetition on bit-level or packet-level is required.

- In fading channels with FSK and bit-level repetition, single-tone CW with 2 frequency hops achieves similar detection performance as 2-tone CW, better than single-tone CW without frequency hopping.

## 2.2	D2R spectrum efficiency

During the discussion of the number of frequency domain resources required for D2R transmission in RAN1#118, the issue of frequency resources reserved as guard band between FDMed D2R transmissions was brought up and agreed to be treated in the discussion of spectrum efficiency. We discuss this issue and the impact on harmonics on D2R spectrum efficiency in this section.

There are different kinds of guard band. The first one is Bguard,D2R in the following agreement, the gap between D2R transmission bandwidth and D2R occupied bandwidth. The second one is unoccupied bandwidth between occupied bandwidth of two FDMed devices, illustrated as inter-device GB in Figure 3 [5]. For the sake of simplicity, in Figure 3 we show only one side band of FDMed D2R transmissions.

Agreement The following bandwidths for D2R are defined for the purpose of the study: - Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device. - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined - Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R - Note: this guard band is not for coexistence with NR/LTE - If/how to define guard band for coexistence between A-IoT D2R and NR/LTE is up to RAN4. - Bocc,D2R >= Btx,D2R - Possible values of each bandwidth are FFS Proposal 3.8.1b(I): For the study of FDMA, Bocc,D2R of the D2R transmission associated with one/each single-tone of a carrier wave: - Bocc,D2R includes ≥99% power of the D2R transmission, with harmonics being taken into account - The guard band Bguard,D2R would be necessary due to SFO - The guard band Bguard,D2R would be necessary due to CFO for Device 2b - The guard band Bguard,D2R is around the main lobes on the two sides of one/each single-tone of a carrier-wave for DSB modulation - Bocc,D2R does not count the unoccupied bandwidth between the two main lobes - FFS the SSB modulation case

Figure 3: Inter-device guard band and Bguard,D2R

The power of a D2R transmission outside its transmission bandwidth is a potential interference to FDMed devices, which spans in Bguard,D2R and the frequency range outside Bocc,D2R. We think the power leakage in Bguard,D2R should be more significant.

Now we consider the effect of passband and stopband of reader’s filter. As illustrated in Figure 4(a) and (b), there is no overlapping between the passband of reader’s filter, which is for the reception of yellow D2R transmission bandwidth, and the occupied bandwidth of the blue D2R transmission. The adjacent-channel interference is no more than 1% of the power of interfering signal, if Bocc,D2R includes ≥99% power of the D2R transmission. Less interference is passed by the filter in Figure 4(a) compared with Figure 4(b). In Figure 4(c), the passband of the filter is larger and passes some power in occupied bandwidth of the interfering signal with the absence of inter-device guard band. In this case, a guard band may be needed to make sure the passband of a filter would not overlap with occupied bandwidth of another device’s main lobe.

In addition to the passband bandwidth of a reader, another reason for inter-device guard band is the device clock inaccuracies. With SFO and CFO, it cannot be always guaranteed that the main lobe of D2R signal from one device falls in the stopband of the BPF filter corresponding to the reception from another device. However, adjacent-channel interference due to FDMed D2R transmission is not included in RAN4 co-existence study, and there is no quantitative study on the size of inter-device guard band. It should be noted that, if inter-device guard band is used for FDMed D2R transmissions, 2-tone carrier wave requires two times of guard band compared with single-tone CW.

- Inter-device guard band may be needed due to CFO and SFO of devices and passband bandwidth of a reader’s filter, especially if it is larger than the occupied bandwidth of a main lobe of a D2R transmission.

- Adjacent-channel interference due to FDMed D2R transmission is not included in RAN4 co-existence study, and there is no quantitative study on the size of inter-device guard band.

- If inter-device guard band is used for FDMed D2R transmissions, 2-tone carrier wave requires two times of guard band compared with single-tone CW.

Figure 4: Passband of a reader’s filter vs. D2R bandwidth

Below, we consider the impact of harmonics on spectrum utilization from two different perspectives.

1. Spectrum utilization from the perspective of power distribution across frequency bands

Consider the power distribution of backscattered transmissions from a single tone versus multi-tone signal. A backscattered transmission from a single-tone signal has all the signal energy concentrated at a single frequency resulting in sidelobes extending wider.

On the other hand, in a multi-tone signal, the signal consists of multiple tones or frequencies simultaneously. Each tone contributes its own frequency component to the overall signal. Since the energy is distributed across multiple frequencies, each individual tone typically has less spectral energy compared to the single tone of equivalent power. Consequently, the backscatter transmissions corresponding to each tone of a multi-tone signal tends to have reduced sidelobe levels compared to a single-tone signal with the same total bandwidth.

- A multi-tone carrier wave signal distributes energy across multiple tones with each tone having less spectral energy compared to a single-tone carrier wave. Backscattered transmission corresponding to each tone of a multi-tone signal has reduced sidelobe levels.

These observations are further verified by backscattering DSB-OOK signal with and without frequency shifts over a single tone as well as a dual-tone as shown in Figures 5 and 6.

The frequency spectrum for DSB-OOK for a single tone at 900MHz if a frequency shift is implemented shows two main lobes after backscattering as shown in Figure 5. This is compared with those produced by backscattering over a dual-tone carrier, where one of the two tones is at the same frequency as the single tone carrier, and another tone separated at 1.65MHz, which results in 4 frequency peaks (2 corresponding to each tone). If a frequency shift is not implemented as in Figure 6, then a single tone carrier produces only one peak of backscattered transmission at 900MHz and dual tone produces two peaks, one corresponding to each tone at 900MHz and 901.65MHz.

The observation from the power spectral density depicted in the two figures reveal that even at sufficiently large frequency spacing, side lobes produced by backscatter transmissions from the dual-tone carrier contributes to less power individually compared to those produced by backscatter transmissions from a single carrier. In fact, for DSB-OOK, the overall power/frequency contributed by the single tone carrier wave and dual-tone carrier wave was almost equal.

Figure 5: Spectral utilization of single-tone vs 2-tone for a DSB-OOK modulation with frequency shift

Figure 6: Power per frequency of single-tone vs 2-tone for a DSB-OOK modulated transmission without frequency shift.

Therefore, we conclude the following,

- Overall power per frequency contribution from the backscattered transmission of a dual-tone carrier wave might be almost similar to that of single-tone carrier wave.

2. Spectrum utilization from the perspective of system capabilities

The number of frequency resources used for backscattering transmission is dependent on the number of tones of carrier wave. With many FDMed devices, A-IoT system bandwidth, provided by the operator, may be insufficient, and single-tone CW may be beneficial. On the other hand, in the cases below, backscattering a single-tone CW may lead to a large portion of the system bandwidth remaining underutilized, and backscattering a multi-tone CW can achieve a better spectrum utilization, namely fewer unused resources.

1. Without FDMA, transmission bandwidth of a device << allocated system bandwidth.

2. If FDMA transmissions are implemented, there are a small number of devices accessing the system simultaneously.

- Using a multi-tone CW can improve spectrum utilization compared with a single-tone, when the latter leads to a large portion of the spectrum remaining underutilized for the cases of 1) transmission bandwidth of a device << allocated system bandwidth and 2) a small number of FDMed devices.

## 2.3	CW interference suppression at D2R receiver

Value range of CW interference cancellation capability was not discussed in RAN1#118. Nevertheless, RAN4 agreed that companies can report the capability used for co-existence evaluation. RAN guidance is that RAN4 document the agreements of the study. Therefore, the discussion of value range of CW cancellation capability and how it would be captured in TR would be done in RAN4.

RAN4 agreement Issue 3-3-2: CW cancellation capability Agreements during ad-hoc - For the CW cancellation capability, companies to report the CW cancellation capability used for co-existence evaluation, CW cancellation capability for inside topology and outside topology can be different. - total interference cancellation capability = spatial isolation + RF cancellation + digital cancellation - CW transmission impact on the received SINR, e.g. degradation RAN#105 Conclusion 6 RAN guidance to RAN4: - RAN confirms that RAN4 studies and documents the methods and feasible values of CW interference cancellation according to their agreements.

- RAN4 to discuss the value range of CW interference cancellation capability and how to capture it in TR.

# 3	CW transmission cases with large frequency shift

Observations were made in RAN1#118 regarding the six cases of CW transmission cases where D2R backscattering is transmitted in the same carrier as CW. There has been no discussion for cases with larger frequency shift due to limited time. One reason is that large frequency shift is still FFS in 9.4.1.2 and TR 38.769. If it is supported, corresponding cases should be discussed, and observations captured in TR.

- If large frequency shift is supported, corresponding cases should be discussed, and observations captured in TR 38.769.

For Topology 1, a case is worth studying, where CW is transmitted in a DL band, and backscattered transmission in an UL band. This allows gNB reader to receive backscattered transmissions in an UL band. BS is not required to support full-duplex. The case can be supported for scenario A1, A2, or B. Such case with scenario B is sketched in Figure 7.

Figure 7: for Topology 1 a case with a large frequency shift, where CW is transmitted in DL spectrum

- If large frequency shift is supported, RAN1 can study a case for Topology 1, where CW is transmitted in a DL band, and backscattered transmission in an UL band.

A counterpart case for Topology 2 is illustrated in Figure 8, where CW transmission is in a DL band and D2R is in UL band.

Figure 8: An additional case for Topology 2, with a large frequency shift between UL and DL FDD bands, where CW is transmitted outside the topology, transmitted in DL spectrum.

Therefore, we propose the following.

- If large frequency shift is supported, RAN1 can study a case for Topology 2, where CW is transmitted in a DL band, and backscattered transmission in an UL band.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	In fading channels with FSK, two-tone CW signal outperforms a single-tone CW signal for the backscattered signal.

Observation 2	In fading channels with FSK, for backscattering based on 2-hop single-tone signal, to exploit the benefits of frequency diversity, repetition on bit-level or packet-level is required.

Observation 3	In fading channels with FSK and bit-level repetition, single-tone CW with 2 frequency hops achieves similar detection performance as 2-tone CW, better than single-tone CW without frequency hopping.

Observation 4	Inter-device guard band may be needed due to CFO and SFO of devices and passband bandwidth of a reader’s filter, especially if it is larger than the occupied bandwidth of a main lobe of a D2R transmission.

Observation 5	Adjacent-channel interference due to FDMed D2R transmission is not included in RAN4 co-existence study, and there is no quantitative study on the size of inter-device guard band.

Observation 6	If inter-device guard band is used for FDMed D2R transmissions, 2-tone carrier wave requires two times of guard band compared with single-tone CW.

Observation 7	A multi-tone carrier wave signal distributes energy across multiple tones with each tone having less spectral energy compared to a single-tone carrier wave. Backscattered transmission corresponding to each tone of a multi-tone signal has reduced sidelobe levels.

Observation 8	Overall power per frequency contribution from the backscattered transmission of a dual-tone carrier wave might be almost similar to that of single-tone carrier wave.

Observation 9	Using a multi-tone CW can improve spectrum utilization compared with a single-tone, when the latter leads to a large portion of the spectrum remaining underutilized for the cases of 1) transmission bandwidth of a device << allocated system bandwidth and 2) a small number of FDMed devices.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to discuss the value range of CW interference cancellation capability and how to capture it in TR.

Proposal 2	If large frequency shift is supported, corresponding cases should be discussed, and observations captured in TR 38.769.

Proposal 3	If large frequency shift is supported, RAN1 can study a case for Topology 1, where CW is transmitted in a DL band, and backscattered transmission in an UL band.

Proposal 4	If large frequency shift is supported, RAN1 can study a case for Topology 2, where CW is transmitted in a DL band, and backscattered transmission in an UL band.

# References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, RAN#103, March 2024.

R1-2400328, “Ambient IoT Study Item work plan”, CMCC, Huawei, T-Mobile USA, RAN1#116, February 2024.

R1-2401795, “Skeleton for TR 38.769 “Study on Solutions for Ambient IoT (Internet of Things)” v0.0.1”, Huawei (TR editor), RAN1#116, February 2024.

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN (Release 18)”, 3GPP, September 2023.

R1-2407638, “General aspects of physical layer design for Ambient IoT”, Ericsson, RAN1#118b, October 2024.