3GPP TSG-RAN WG1 Meeting #118bis	R1-2407672

Hefei, China, October 14-18, 2024

Agenda Item:	9.4.2.4

Source:	Huawei, HiSilicon

Title:	On external carrier wave for backscattering based Ambient IoT device

Document for:	Discussion and Decision

# Introduction

In the RAN#105 meeting, it is concluded that the study of CW waveform characteristics will continue in RAN1.

(Copied from RP-242360 [1]) Conclusion: keep the CW waveform characteristics on the RAN1 agenda in 2024/Q4.

In the RAN1#117 meeting, a table with the technical aspects for the observations and comparisons between the unmodulated single-tone waveform without frequency hopping and two unmodulated single-tone waveform is agreed. Based on the discussions during the RAN1#117 and RAN1#118 meeting, the corresponding study has been completed.

In the RAN1#118 meeting, observations were made on the CW transmission of the cases that D2R backscattering is transmitted in the same carrier as CW. The CW characteristics needing control of the CW node(s) were also identified.

This contribution firstly discusses about CW transmission cases for Device 2a with large frequency shifter, where D2R backscattering is transmitted in different carrier from CW. After that, observations are provided on the unmodulated single-tone waveform with frequency hopping, referring to the technical aspects considered for the two waveforms been studied. Considering this potential new waveform, the CW characteristics needing control of the CW node(s) are further discussed.

# On the CW characteristics needing control of the CW node(s)

In the RAN1#118 meeting, three characteristics have been identified as needing control of the CW node(s).

(Copied from R1-2407479 [4]) Agreement Observation With respect to the following RAN guidance: - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - From RAN1 perspective, the following CW characteristics are identified: - When CW is transmitted or not transmitted - Transmission Power - Frequency resources - FFS other characteristics

In our understanding, whether there are other characteristics to be identified depends on whether the “waveform” of CW can be included in the item of “frequency resources”. To control the CW node(s), the exact frequency of the one or two single-tones has to be chosen. For the unmodulated single-tone waveform without frequency hopping, one CW frequency is assigned to the CW node(s). For the two unmodulated single-tone waveform, two independent CW frequencies with non-fixed interval between them have to be assigned, so as to support flexible usage in various deployment scenarios. From the above, whether the waveform is unmodulated single-tone or two unmodulated single-tone can be distinguished by the frequency tone(s) chosen for the CW transmission. There is no need to also control the ‘name’ of the waveform. Based on the above, it is observed that no other characteristic needs control of the CW node(s).

Any other discussions of CW waveform characteristics can be left to a WI phase, when more is known about the exact characteristics that are appropriate for D2R transmission.

Proposal 1: For the observation of the CW characteristics needing control of the CW node(s), remove the sub-bullet of “FFS other characteristics”, i.e.

With respect to the following RAN guidance:

- Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation.

    - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

- From RAN1 perspective, the following CW characteristics are identified:

    - When CW is transmitted or not transmitted

    - Transmission Power

    - Frequency resources

    - FFS other characteristics

# On the unmodulated single-tone waveform with frequency hopping

To obtain frequency diversity for backscattering based D2R transmission, another theoretical option, besides the multiple unmodulated single-tone waveform, is to apply frequency hopping to the unmodulated single-tone waveform. As discussed in our previous paper on device architecture [3], a buffer exceeding a certain size (e.g., around or larger than 100 bits) is too complicated and power consuming for at least Device 1. Consequently, interleaving is assumed not to be supported by Ambient IoT devices. Without interleaving, frequency hopping has to be used together with repetition to obtain frequency diversity, e.g. by carrying the different repetitions of each bit or block on different carrier frequency. It means the waveform is only applicable for the devices in the worst coverage.

Observation 1: To obtain frequency diversity, the unmodulated single-tone waveform with frequency hopping has to be used together with repetition, which makes the waveform only useful for the devices in bad coverage.

The following two aspects need to be considered to justify the applicability and benefit of the unmodulated single-tone waveform with frequency hopping.

- The frequency of the external carrier-wave is transparent to devices, as a device only needs to control the impedance switching circuit according to the D2R baseband signal. In other words, the devices are unaware of the frequency hopping of the external carrier-wave, which means devices cannot align the baseband signal with the boundary of each hop.

- Considering the inaccurate frequency of the extreme-low power clock in Ambient IoT devices, the timing is not aligned between the reader and the device. The overall timing unalignment includes both the unpredictable processing latency of each device and the accumulated timing offset due to clock frequency offset during the D2R transmission. It results in the CW node not being able to align the boundary of each hop with a corresponding segment of the baseband signal.

According to the above two aspects, it is too difficult to align the boundary of each hop of the external carrier-wave with a corresponding bit or block repetition of the D2R transmission.

Observation 2: The unaligned timing between reader and devices makes it hard to align the boundary of each hop of the external carrier-wave with a corresponding bit or block repetition of the D2R transmission.

From the perspective of D2R receiver, the RF interference cancellation usually requires a processing latency of a few or up to ten milli-seconds to detect the amplitude and phase of the received carrier-wave interference. When the frequency of the external carrier-wave changes, the amplitude and phase of the received carrier-wave interference would probably change as well due to e.g. the variation of the experienced multi-path channel. A D2R receiver has to complete the detection for CW interference cancellation before appropriately receiving the D2R data. It is even not sure the fast detection can be supported by the intermediate UE in D2T2, especially for the case of CW node out the topology. The issue can obviously impact the D2R link performance.

Observation 3: For the RF interference cancellation at the D2R receiver, the frequency hopping of external carrier-wave requires fast re-detection of the parameters of the received carrier-wave interference, which significantly increase the implementation complexity of D2R receiver and may not be supported by the intermediate UE in D2T2.

Based on the above issues, it is recommended that the unmodulated single-tone waveform with frequency hopping is not studied in Rel-19.

Proposal 2: The unmodulated single-tone waveform with frequency hopping is not studied in Rel-19.

- If it is studied, the following observations are captured:

  - To obtain frequency diversity, the unmodulated single-tone waveform with frequency hopping has to be used together with repetition, which makes the waveform only useful for the devices in bad coverage.

  - The unaligned timing between reader and devices makes it hard to align the boundary of each hop of the external carrier-wave with a corresponding bit or block repetition of the D2R transmission.

  - For the RF interference cancellation at the D2R receiver, the frequency hopping of external carrier-wave requires fast re-detection of the parameters of the received carrier-wave interference, which significantly increase the implementation complexity of D2R receiver and may not be supported by the intermediate UE in D2T2.

# CW transmission cases for Device 2a with large frequency shifter

Whether to include large frequency shifter cases for device 2a is debatable, since according to [2], there are few cases where it is currently likely to be applicable. We think it is at least not necessary to construct numerous ‘transmission cases’ as was done for the small frequency-shift cases, since few of the large-frequency shift cases have reasonable use cases, if the hardware to perform it even exists in reality.

## Topology 1

In Topology 1, carrier-wave can be transmitted by the BS in FDD downlink spectrum, while the corresponding backscattering is shifted to FDD uplink spectrum. The spectrum usage is similar to conventional 3GPP FDD systems, except that the external carrier-wave has to be provided to devices during D2R transmissions. As the carrier-wave and backscattering are not in the same carrier, the benefit is no need of carrier-wave interference suppression at the D2R receiver. As the carrier-wave is transmitted in FDD downlink spectrum, it is natural to use basestation for the transmission, which is inside the topology. It also has the added advantage that higher transmission power can be used by the BS to transmit the CW in the downlink spectrum, when compared to CW transmission in the uplink spectrum. This would avoid the issue of an increased gNB density being required to provide the CW to all the required devices.

Proposal 3: If the case that D2R backscattering is transmitted in different carrier from CW for D2R backscattering (e.g., in UL and DL spectrum, respectively) is to be included in the TR, for Topology 1:

- The following observations are captured:

  - There is no need of CW interference suppression at the D2R receiver if different carriers for CW and D2R transmissions are used.

  - BS inside the topology can transmit the CW using higher power in the DL spectrum as compared to the UL spectrum.

## Topology 2

For Topology 2, the external carrier-wave is recommended to be transmitted in downlink spectrum as well, with the backscattering being shifted to the uplink spectrum. In this way, the same device implementation can be reused between Topology 1 and 2, which is important considering the relatively high power consumption and complexity of large frequency shifter. Meanwhile, the intermediate UE receives D2R transmission in FDD uplink spectrum, which is similar to Case 2-2 and 2-4 when the D2R backscattering is transmitted in the same carrier as the carrier-wave. It also benefits the receiver implementation of intermediate UE. Considering the potential impact on regulation, CW node(s) outside the topology is preferred to transmit CW in FDD downlink spectrum.

The reason the CW being inside the topology is not considered is due to the fact that the intermediate UE would have to transmit the CW in the uplink spectrum, using a lower power as compared to a CW node outside the topology that is able to transmit using a higher power on the downlink spectrum.

Proposal 4: If the case that D2R backscattering is transmitted in different carrier from CW for D2R backscattering (e.g., in UL and DL spectrum, respectively) is to be included in the TR, for Topology 2,

- The following observations are captured:

  - Same device implementation is used between D1T1 and D2T2 if the device performs D2R backscattering in the UL spectrum.

  - Intermediate UE receives the D2R transmission in the UL spectrum, similar to case 2-2 and 2-4, benefitting the receiver implementation of the intermediate UE.

  - CW node outside the topology can transmit the CW using higher power in the DL spectrum as compared to the UL spectrum.

As discussed in [2], though the low-power FDD frequency shifter has been seen in theoretical studies, there is still concern on its necessity and practical applicability. It is recommended to de-prioritize the study on the cases that D2R backscattering is transmitted in a different band as the carrier-wave for D2R backscattering.

Proposal 5: The study de-prioritizes further effort on the cases that D2R backscattering is transmitted in a different carrier from the carrier used for CW transmissions.

# Conclusions

In this contribution, the suitable spectrum and waveform characteristics for the external carrier-wave are discussed. It is recommended to capture the following observations, together with the proposals, in the TR.

Observation 1: To obtain frequency diversity, the unmodulated single-tone waveform with frequency hopping has to be used together with repetition, which makes the waveform only useful for the devices in bad coverage.

Observation 2: The unaligned timing between reader and devices makes it hard to align the boundary of each hop of the external carrier-wave with a corresponding bit or block repetition of the D2R transmission.

Observation 3: For the RF interference cancellation at the D2R receiver, the frequency hopping of external carrier-wave requires fast re-detection of the parameters of the received carrier-wave interference, which significantly increase the implementation complexity of D2R receiver and may not be supported by the intermediate UE in D2T2.

Proposal 1: For the observation of the CW characteristics needing control of the CW node(s), remove the sub-bullet of “FFS other characteristics”, i.e.

With respect to the following RAN guidance:

- Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation.

    - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

- From RAN1 perspective, the following CW characteristics are identified:

    - When CW is transmitted or not transmitted

    - Transmission Power

    - Frequency resources

    - FFS other characteristics

Proposal 2: The unmodulated single-tone waveform with frequency hopping is not studied in Rel-19.

- If it is studied, the following observations are captured:

  - To obtain frequency diversity, the unmodulated single-tone waveform with frequency hopping has to be used together with repetition, which makes the waveform only useful for the devices in bad coverage.

  - The unaligned timing between reader and devices makes it hard to align the boundary of each hop of the external carrier-wave with a corresponding bit or block repetition of the D2R transmission.

  - For the RF interference cancellation at the D2R receiver, the frequency hopping of external carrier-wave requires fast re-detection of the parameters of the received carrier-wave interference, which significantly increase the implementation complexity of D2R receiver and may not be supported by the intermediate UE in D2T2.

Proposal 3: If the case that D2R backscattering is transmitted in different carrier from CW for D2R backscattering (e.g., in UL and DL spectrum, respectively) is to be included in the TR, for Topology 1,

- The following observations are captured:

  - There is no need of CW interference suppression at the D2R receiver if different carriers for CW and D2R transmissions are used.

  - BS inside the topology can transmit the CW using higher power in the DL spectrum as compared to the UL spectrum.

Proposal 4: If the case that D2R backscattering is transmitted in different carrier from CW for D2R backscattering (e.g., in UL and DL spectrum, respectively) is to be included in the TR, for Topology 2,

- The following observations are captured:

  - Same device implementation is used between D1T1 and D2T2 if the device performs D2R backscattering in the UL spectrum.

  - Intermediate UE receives the D2R transmission in the UL spectrum, similar to case 2-2 and 2-4, benefitting the receiver implementation of the intermediate UE.

  - CW node outside the topology can transmit the CW using higher power in the DL spectrum as compared to the UL spectrum.

Proposal 5: The study de-prioritizes further effort on the cases that D2R backscattering is transmitted in a different carrier from the carrier used for CW transmissions.

# References

- RP-242360, “Moderator's summary for offline discussion on Ambient IoT”, RAN1 Vice-Chair (Huawei).

- R1-2407668, “Ultra low power device architectures for Ambient IoT”, RAN1#118bis, Oct. 2024, Huawei, HiSilicon

- R1-2405851, “Ultra low power device architectures for Ambient IoT”, RAN1#118, Aug. 2024, Huawei, HiSilicon

- R1-2407479, “Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things) in NR”, Ad-Hoc Chair (Huawei).