3GPP TSG RAN WG1 #118bis                                                     R1-2407712

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.5.2

Source:	Spreadtrum Communications

Title: 	Discussion on on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

# Introduction

It was agreed in WID [1] to specify on-demand SIB1 for case 2 for UEs in idle/inactive state.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

According to SI stage, it is common understanding that on-demand SIB1 can enable a cell switching between on and off, which provides energy saving gain.

# UL WUS design

In RAN1#116 [2], it was agreed that PRACH is starting point for UL WUS design.

Agreement - For study of UL WUS design, consider at least PRACH as a starting point - FFS: Whether there is dedicated PRACH resource for SIB1 request - Other option(s) not precluded

Since preambles and PRACH occasions (time/frequency resource) are flexible enough, a dedicated part of preambles and PRACH occasions (ROs in short) can be used for UL WUS.

In RAN1#116bis [3], it was agreed that PRACH resource (including time/frequency/preamble resource) dedicated for requesting SIB1 can be configured.

Agreement For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1 - FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS - FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

In RAN1#118 [4], there were two options for dedicated PRACH resource, i.e. shared RO or separate RO.

Agreement For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options: - Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages. - Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages.

When UE transmits PRACH in NES cell, UE knows the cell is in status of NES, i.e. there is no other UEs camp on the cell. Therefore, there is no PRACH contention issue. It means dedicated PRACH resource for on-demand SIB1 request can be separated RO.

Proposal 1: Dedicated PRACH resource for on-demand SIB1 request can be separated RO, i.e. Option 2.

# UE identification of status of NES cell

In RAN1#116bis, it was agreed that UE identification of NES cell with/without on-demand SIB1 has two options.

Agreement RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options: - Option 1: By WUS configuration - Option 2: By PBCH payload of NES cell

For Option 1, if WUS configuration is from cell A, the status of NES cell (with or without on-demand SIB1) should be transferred from NES cell to cell A via backhaul.

For Option 2, if legacy UE can camp on NES cell and PBCH payload supports UE to access NES cell, the available bits for PBCH payload of NES cell is very limited. However, 1 bit is enough for UE identification, so we think it is feasible.

Proposal 2: If WUS configuration is from cell A, UE identification of NES cell with/without on-demand SIB1 is based on PBCH payload of NES cell.

# Feedback from gNB

In RAN1#116, it was agreed that feedback from gNB in response to the SIB1 request (UL WUS) could be studied.

Agreement For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

In our view, if UL WUS is PRACH to NES cell, the feedback could be Msg2 (RAR), and NES cell can inform UE about the estimated TA.

In RAN2#125bis [5] agreement, RAR based feedback was agreed, since RACH procedure is reused for UE to request on-demand SIB1.

Agreements on on-demand SIB1 1. At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded. 2. RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1. 3. UL WUS configuration includes at least RACH configuration. 4. A UE needs to know a UL WUS configuration to request SIB1 of which cell. 5. Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

In RAN1#118, feedback from gNB through RAR for on-demand SIB1 feedback was confirmed.

Agreement RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Anyway, it can be up to RAN2 discussion for details of feedback from gNB.

Proposal 3: It can be up to RAN2 discussion for details of feedback from gNB.

# Duration of Type0-PDCCH monitoring

After receiving RAR as feedback from gNB, UE will monitor Type0-PDCCH for a duration to get SIB1 scheduling, at least for Case-2.

In RAN1#117 [6], it was agreed that starting point and duration of Type0-PDCCH monitoring has two options.

Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration

In our view, the starting point and duration of Type0-PDCCH monitoring is almost constant, which does not change dynamically. Therefore, we support they can be indicated in the UL WUS configuration.

Proposal 4: Support starting time and duration are indicated in the UL WUS configuration, i.e. Option 2.

The definition of starting time needs the reference time point. In RAN1#118, there were three options for the reference time point.

Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

In our view, the reference time point may be related to RAR reception. Therefore, we support Option 1.

Proposal 5: The reference time point is defined based on the RAR reception time of the UL-WUS transmission.

# Configuration of searchSpaceZero

UE should have configuration of seachSpaceZero (i.e. 4bits in legacy initial cell selection)  to monitor Type0-PDCCH.

In RAN1#117, it was agreed that searchSpaceZero for Type0-PDCCH monitoring has three options.

Agreement For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded.

In our view, it natural that searchSpaceZero is provided from the UL WUS configuration.

Proposal 6: Support searchSpaceZero for on-demand SIB1 is provided from the UL WUS configuration, i.e. Option 2.

# SSB periodicity for NES Cell

In RAN1#117, it was agreed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

Agreement For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

In our understanding, the NES cell with on-demand SIB1 can be detected and measured by UE based on always-on SSB. The always-on SSB may have long periodicity. Hence, whether the periodicity of the always-on SSB can be updated after on-demand SIB1 transmission can be discussed, i.e. SSB adaptation.

In our simulation, SSB adaptation (change from 20ms periodicity to 160ms periodicity) is more important for additional energy saving gain, when NES cell offloads all its data to cell A, since SIB1 repetition period can be 160ms, which is up to gNB implementation.

Observation 1: In our evaluation, SSB adaptation (e.g. change from 20ms periodicity to 160ms periodicity) is more important for additional energy saving gain.

In our view, NES cell can send SSB with periodicity larger than 20ms, otherwise NES gain of on-demand SIB1 is not promising. However, in current spec, before SIB1 transmission, UE may only perform measurement for SSB, which can be configured as SSB with long periodicity, e.g. through SMTC, and after SIB transmission, UE needs to perform sync and data transfer, which can be configured as SSB with default 20ms periodicity. Therefore, it is up to gNB configuration.

Observation 2: Adaptation of SSB periodicity for NES cell may be up to gNB configuration.

# Conclusion

We have the following proposals.

UL WUS design

Proposal 1: Dedicated PRACH resource for on-demand SIB1 request can be separated RO, i.e. Option 2.

UE identification of status of NES cell

Proposal 2: If WUS configuration is from cell A, UE identification of NES cell with/without on-demand SIB1 is based on PBCH payload of NES cell.

Feedback from gNB

Proposal 3: It can be up to RAN2 discussion for details of feedback from gNB.

Duration of Type0-PDCCH monitoring

Proposal 4: Support starting time and duration are indicated in the UL WUS configuration, i.e. Option 2.

Proposal 5: The reference time point is defined based on the RAR reception time of the UL-WUS transmission.

Configuration of searchSpaceZero

Proposal 6: Support searchSpaceZero for on-demand SIB1 is provided from the UL WUS configuration, i.e. Option 2.

# Reference

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, RAN#105, Sep 9-12, 2024.

- 3GPP RAN1#116 Chair’s Notes, Feb. 26th – Mar. 1st, 2024.

- 3GPP RAN1#116bis Chair’s Notes, Apr. 15th – 19th, 2024.

- 3GPP RAN1#118 Chair’s Notes, Aug 19th – 23th, 2024.

- 3GPP RAN2#125bis Chair’s Notes, Apr. 15th – 19th, 2024.

- 3GPP RAN1#117 Chair’s Notes, May 20th – 24th, 2024.