3GPP TSG RAN WG1 #118bis                                                     R1-2407713

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.5.3

Source:	Spreadtrum Communications

Title: 	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and decision

# Introduction

It was agreed in WID [1] to specify adaptation of common signal/channel transmission.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

According to SI stage, it is common understanding that adaptation of common signal/channel transmission can enable a cell switching between on and off, which provides energy saving gain.

# Adaptation of SSB in time domain

In RAN1#116bis [2], it was agreed to support DL indication for SSB adaptation at least.

Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

In RAN1#116bis, three scenarios were agreed to be supported or studied.

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

In RAN1#116bis, RAN1 also agreed to study further applicable scenarios and associated legacy UE impact/handling.

Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/o SCell(s)

In RAN1#118, further agreements on PCell/SCell (for connected UEs) were made.

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

It can be observed that:

- For Rel-19 NES-capable UE’s PCell, adaptation of CD-SSB on sync raster is NOT supported.

- For Rel-19 NES-capable UE’s SCell, the following cases are supported

  - Adaptation of non CD-SSB on sync raster (non CD-SSB on sync raster has supported in R15 which is mentioned in 38.213 as special k_SSB values)

  - Adaptation of non CD-SSB not on sync raster (non CD-SSB not on sync raster has supported in R15 which can be SSB for measurement which is not be blindly detected)

In our view, non CD-SSB on sync raster and not on sync rater for PCell are both supported in R15.

- Non CD-SSB on sync raster has supported in R15 which is mentioned in 38.213 as special k_SSB values

- Non CD-SSB not on sync raster has supported in R15 which can be SSB for measurement which is not be blindly detected

However, adaptation of non CD-SSB for PCell for connected UEs can be replaced by “TRS adaptation”. We should not forget TRS is more fundamental for connected UEs in terms of DL sync. Considering TRS, we think that necessity of adaptation of non CD-SSB for PCell for connected UEs is not justified.

Proposal 1: Necessity of adaptation of non CD-SSB for PCell for connected UEs should be discussed further.

In RAN1#118 [3], there were 3 options listed about cell DTX extension to SSB. Option 3 means no extension.

Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

To be honest, in R18, we had no consensus on TRS impacted by cell DTX, so we may not have the consensus on SSB impacted by cell DTX. TRS is essentially more fundamental and flexible than SSB for connected UEs in terms of DL sync.

Proposal 2: Support cell DTX does not impact UE assumption on SSB transmission (no spec impact), i.e. option 3.

## SSB periodicity

NES gain of SSB adaptation in time domain

No matter for which scenario listed above (e.g. PCell, SCell, serving cell for idle/inactive UE, the first issue to be resolved is whether UE can live with SSB with periodicity longer than 20ms. For legacy UE, SSB periodicity should not be larger than 20ms in most companies’ view, since the default SSB periodicity is 20ms for cell search and 20ms is usually the SSB periodicity for mobility, e.g. measurement, during our discussion. For R19 NES capable UE, whether SSB periodicity can be larger than 20ms should be discussed. If it cannot, we do not think NES gain of SSB adaptation in time domain is not justified.

To be honest, periodicities of SSB larger than 20ms has been specified in spec since R15, but now companies argued that these periodicities are not applicable in real deployment. It is really confusing to us.

Observation 1: If SSB periodicity cannot be larger than 20ms, NES gain of SSB adaptation in time domain is not justified.

Zero and low load

In [4], we have expressed view that SSB periodicity larger than 20ms should be supported by at least R19 eNES capable UEs.

Actually, according to evaluation results in SI shown in Appendix-1, the most cases of SSB periodicity adaptation with promising NES gain have assumptions of zero (empty) or low load. It hints that SSB periodicity larger than 20ms is meaningful for zero or low load in companies’ evaluation assumption.

Observation 2: In companies’ evaluation assumption during SI for SSB periodicity adaptation, zero or low load is assumed.

In zero load case, i.e. for idle/inactive UE, in our view, at least R19 NES capable UE can work with SSB with periodicity larger than 20ms, since performance requirement for idle/inactive UE is not so high. For cell search, we do not think UE cannot detect SSB with periodicity larger than 20ms. The default periodicity means typical case, and UE try cell search starting from hypothesis of 20ms SSB periodicity.

Observation 3: SSB periodicity larger than 20ms is feasible for idle/inactive UEs.

In low load case, i.e. for connected UE, with long SSB periodicity, UE may consider large CFO/TO in CQI calculation, and gNB may configure low MCS to UE. It seem workable. However, performance of mobility may be affected by long SSB periodicity.

Fortunately, TRS can be used by connected UE. Specifically, SSB periodicity can be relaxed, e.g. 80ms, and connected UE can use both SSB and TRS with short periodicity, e.g. 20ms, for data transfer. Nevertheless, in this case NES gain is reduced, since gNB has to transmit TRS timely.

Observation 4: SSB periodicity larger than 20ms with assistance of TRS is feasible for connected UEs, but NES gain is reduced.

According to above discussion, SSB periodicity larger than 20ms is feasible at least for R19 NES capable UEs.

Observation 5: SSB periodicity larger than 20ms is feasible at least for R19 NES capable UEs.

In our view, zero load case is more important than low load case, from NES gain perspective, and CD-SSB adaptation is more straightforward than NCD-SSB (e.g. SSB off sync-raster). CD-SSB adaptation is effective for idle/inactive UEs and connected UEs. Therefore, we can focus on CD-SSB adaptation for idle/inactive UE.

Proposal 3: Focus on CD-SSB adaptation for idle/inactive UEs and connected UEs.

# Adaptation of PRACH in time domain

For UEs in idle/inactive state, PRACH is used for gNB to calculate TA (assist UE to achieve UL sync) and detect UE initiating random access.

If gNB realizes more UEs are initiating random access, gNB can use short PRACH periodicity (more RACH occasions) to balance load in each RACH occasion, and vice versa. Meanwhile, gNB should inform UE that PRACH periodicity is updated.

For UE in connected state, PRACH is used for gNB to re-calculate TA (assist UE to re-achieve UL sync), when UE is RLF/BF.

Similar to idle/inactive state, gNB can change PRACH periodicity for load balancing. Meanwhile, gNB should inform UE that PRACH periodicity is updated.

Observation 6: PRACH periodicity can be updated adaptively.

In RAN1#116 [5], it was agreed to introduce additional PRACH resource, e.g. adaptation of PRACH periodicity/RO, adaptation of PRACH association period/association pattern period, extending cell DRX.

Agreement For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study - Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For the additional PRACH resources, - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH - Concentrating ROs in time domain - Other options are not precluded

In RAN1#116, it was agreed for DL indication for PRACH adaptation.

Agreement For the adaptation mechanisms of PRACH in time-domain - Support at least PRACH adaptation provided by gNB without UE trigger - FFS: PRACH adaptation with UE trigger - Note: UE trigger means UE requests adaptation of PRACH - Study at least the following, - Dynamic signaling and/or semi-static signaling of PRACH adaptation - Adaptation of PRACH transmission according to certain condition - Applicability to idle/inactive and/or connected mode UEs - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

In RAN1#116bis, additional PRACH resource was elaborate, e.g. semi-static signaling based configuration for additional PRACH resource, separate SSB-RO-mapping for additional PRACH resource.

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources

In RAN1#116bis, it was agreed that PRACH adaption is applicable for idle/inactive/connected UEs.

Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

## Signaling

In RAN1#117 [6], it was agreed that signaling can be higher layer signaling (MAC CE or RRC), L1 signaling, or other signaling.

Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

In our view, L1 signaling is feasible for UE, since miss detection of the signaling will only cause latency of PRACH transmission at UE. L1 signaling is also beneficial for UE, which can provide opportunity of fast PRACH transmission.

Proposal 4: Support L1 signaling for triggering additional PRACH resource.

CBRA is used by idle/inactive UEs and connected UEs.

CFRA is used by connected UEs. It can be regarded as a method of PRACH adaptation for connected UEs.

Proposal 5: CFRA is not considered in PRACH adaptation in time domain.

In RAN1#118, DCI format is narrowed down, i.e. DCI format 1_0/2_7/2_9.

Agreement For DCI-based adaptation for additional PRACH resources, - Select from the following DCI format(s) to carry the adaptation indication. - DCI format 1_0 - DCI format 2_7 - DCI format 2_9 - FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

DCI format 2_7 is PEI, which has specific function. DCI format is activation/deactivation of cell DTX/DRX, but we have no consensus on supporting PRACH adaptation through cell DRX extension. Therefore, we support DCI format 1_0.

Proposal 6: For DCI format for PRACH adaptation, DCI format 1_0 is supported.

# Conclusion

We have the following proposals.

Adaptation of SSB in time domain

Proposal 1: Necessity of adaptation of non CD-SSB for PCell for connected UEs should be discussed further.

Proposal 2: Support cell DTX does not impact UE assumption on SSB transmission (no spec impact), i.e. option 3.

Proposal 3: Focus on CD-SSB adaptation for idle/inactive UEs and connected UEs.

Adaptation of PRACH in time domain

Proposal 4: Support L1 signaling for triggering additional PRACH resource.

Proposal 5: CFRA is not considered in PRACH adaptation in time domain.

Proposal 6: For DCI format for PRACH adaptation, DCI format 1_0 is supported.

# Reference

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson, RAN#102, Dec.  11th - 15th, 2023.

- 3GPP RAN1#116bis Chair’s Notes, Apr. 15th – 19th, 2024.

- 3GPP RAN1#118 Chair’s Notes, Aug 19th – 23th, 2024.

- RP-241132, “Considerations on enhancements of Network energy savings for NR”, Spreadtrum, RAN#104, Jun.  17th - 20th, 2023.

- 3GPP RAN1#116 Chair’s Notes, Feb. 26th – Mar. 1st, 2024.

- 3GPP RAN1#117 Chair’s Notes, May 20th – 24th, 2024.

# Appendix-1: Evaluations results for adaptation of SSB periodicity

Evaluation results for technique A-1-3 are shown as follow. It can be observed that most cases with promising NES gain have zero (empty) or low load.

Table 6.1.1.2-3: BS energy savings by adapting SSB/SIB1 periodicities

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/Access delay/latency/UE power consumption, etc. (loss w.r.t. baseline) | Baseline configuration/assumption/Other notable setting |
| --- | --- | --- | --- | --- | --- | --- |
| CMCC [23] | SSB periodicity 20ms, SIB repetition period 40ms. | cat.2 | Zero | 13.7% |  | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |
|  | SSB and SIB1 repetition period 40ms. |  |  | 17.6% |  |  |
|  | SSB periodicity 20ms, SIB repetition period 40ms. | cat.1 | Zero | 25.7% |  |  |
|  | SSB and SIB1 repetition period 40ms. |  |  | 28.7% |  |  |
| vivo [10] [20] | Period adaptation of common signals and channels (ES scheme: 160ms SSB and SIB1, 160ms RACH listening) | Cat1 | Zero | 78.8% | UE power consumption: 0% | Baseline scheme: 20ms SSB and SIB1, 20ms RACH listening |
|  |  | Cat2 |  | 16.6% | UE power consumption: 0% |  |
| NOKIA/NSB [12] | SSB/SIB1/RO monitoring period= 160ms | Cat 2 | Zero, Low, Light, Medium | 48.4%, 44.3%, 43.7%, 39.9% | UPT: 0 Mbps, 83 Mbps, 70 Mbps, 55 Mbps | SSB/SIB1/random-access occasion (RO) monitoring periodicity @ 20ms UEs are initially in RRC_idle state |
|  | SSB/SIB1/RO monitoring period= 640ms |  | Zero, Low, Light, Medium | 53.6%, 49.0%, 48.8%, 46.1% | UPT: 0 Mbps, 29 Mbps, 27 Mbps, 25 Mbps |  |
|  | SSB/SIB1/RO monitoring period= 1280ms |  | Zero, Low, Light, Medium | 83.6%, 51.3%, 51.7%, 50.6% | UPT: 0 Mbps, 11.2 Mbps, 11 Mbps, 10.5 Mbps |  |
| Spreadtrum [13] | Prolonging the periodicity of SSB/SIB1/paging: 1) SSB burst periodicity is 160ms, and SIB1 repetition periodicity is 160ms. 2) PF periodicity at gNB side is 160ms (T=1280ms, N=8). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. | Cat 1 | Zero | Set 1- Set 3: 23.8%, 19.6%, 16.3% |  | 1) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. 2) PF periodicity at gNB side is 20ms (T=1280ms, N=64). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. |
|  |  | Cat 2 |  | Set 1- Set 3: 9.3%, 8.3%, 9.4% |  |  |
|  | Transmission window of SSB/SIB1/paging: 1) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. 2) PF periodicity at gNB side is 20ms (T=1280ms, N=64). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. | Cat 1 |  | Set 1- Set 3: 23.8%, 19.6%, 16.3% |  | 1) The transmission window periodicity is 1280ms, and the transmission window duration is 160ms. 2) SSB burst periodicity is 20ms within the transmission window, and SIB1 repetition periodicity is 20ms within the transmission window. 3) PF periodicity at gNB side is 160ms (T=1280ms, N=8) within the transmission window. 4) gNB can enter light sleep for Cat 1, and can enter both light sleep and micro sleep for Cat 2 (at the tail of the transmission window). |
|  |  | Cat 2 |  | Set 1- Set 3: 51.5%, 47.3%, 20.6% |  |  |
| Intel [22] | Increasing the common channel/signal periodicity | Cat 1 | Low | 40.1% | UPT: 819.66 Mbps Avg EE* (baseline): 5.10 Avg EE (ES scheme): 9.17 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity. EE* is defined as cell throughput (in Mbps) / average power consumption (in relative power), and averaged from all BS. |
|  |  |  |  | 45.0% | UPT: 819.66 Mbps Avg EE (baseline): 5.10 Avg. EE (ES scheme): 10.60 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  |  | Light | 14.6% | UPT: 611.45Mbps Avg EE (baseline): 2.66 Avg. EE (ES scheme): 3.31 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 16.8% | UPT: 611.45Mbps Avg EE (baseline): 2.66 Avg. EE (ES scheme): 3.46 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  |  | Medium | 6.2% | UPT: 457.92Mbps Avg EE (baseline): 1.50 Avg. EE (ES scheme): 1.63 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 7.1% | UPT: 457.92Mbps Avg EE (baseline): 1.50 Avg. EE (ES scheme): 1.65 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  | Cat2 | Low | 8.2% | UPT: 819.66Mbps Avg EE (baseline): 35.82 Avg. EE (ES scheme): 39.23 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 10.9% | UPT: 819.66Mbps Avg EE (baseline): 35.82 Avg. EE (ES scheme): 40.09 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
|  |  |  | Light | 5.1% | UPT: 611.45Mbps Avg EE (baseline): 20.75 Avg. EE (ES scheme): 22.00 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 5.8% | UPT: 611.45Mbps Avg EE (baseline): 20.75 Avg. EE (ES scheme): 22.19 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
|  |  |  | Medium | 3.0% | UPT: 457.92Mbps Avg EE (baseline): 12.44 Avg. EE (ES scheme): 12.89 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 3.4% | UPT: 457.92Mbps Avg EE (baseline): 12.44 Avg. EE (ES scheme): 12.96 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms. ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
| CATT [25] | Adaptation of common signals and channels | Cat 1 | Zero load | 10.2%, 72.7%, 84.8% |  | Baseline: 20ms SSB; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Low load | 3.4%, 18.8%, 19.7% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms; CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Light load | 1.9%, 5.2%, 5.6% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms; CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Medium load | 1.3%, 2.2%, 2.6% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms; CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
| Fujitsu [11] | SSB/SIB1 period= 40ms | Cat2 | Zero, low, light, medium | 17.9%, 13.7%, 11.1%, 8.6% |  | Baseline scheme: 20 ms SSB/SIB1 period |
|  | SSB/SIB1 period= 80ms |  | Zero, low, light, medium | 26.8%, 20.6%, 16.7%, 12.8% |  |  |
|  | SSB/SIB1 period= 160ms |  | Zero, low, light, medium | 31.4%, 24.1%, 19.4%, 15.0% |  |  |
|  | SSB/SIB1 period= 40ms |  | Zero, low, light, medium | 18.3%, 12.6%, 9.4%, 6.9% |  |  |
|  | SSB/SIB1 period= 80ms |  | Zero, low, light, medium | 27.4%, 18.8%, 14.1%, 10.4% |  |  |
|  | SSB/SIB1 period= 160ms |  | Zero, low, light, medium | 32.0%, 22.0%, 16.5%, 12.1% |  |  |
| Ericsson [18] | 40ms SSB+SIB1 | Cat1 | Zero | 0.9% |  | Baseline scheme: 20ms SSB + 160ms SIB1 ES: one SSB. Energy calculation: per symbol energy consumption is modeled. |
|  | 80ms SSB+SIB1 |  |  | 48.5% |  |  |
|  | 160ms SSB+SIB1 |  |  | 72.6% |  |  |
|  | 40ms SSB+SIB1 |  |  | -6.2% |  | Baseline scheme: 20ms SSB + 160ms SIB1 ES: Four SSBs. Energy calculation: per symbol energy consumption is modeled. |
|  | 80ms SSB+SIB1 |  |  | 43.8% |  |  |
|  | 160ms SSB+SIB1 |  |  | 70.5% |  |  |
| Qualcomm [17] | Adaptation of Common Signals and Channels | Category 1 | No Load | 13.9% | Access delay/latency: additional 20 ms; UE power consumption increment: 99% | Note: "SSB period of 40 ms" without any network traffic either in DL or UL. Therefore, there are no statistics for UPT, latency, etc.. |