3GPP TSG RAN WG1 #118bis		R1-2407727

Hefei, China, October 14th – 18th, 2024

Source:	China Telecom

Title:	Discussion on RAN2 LS on data block sizes for Ambient IoT

Agenda Item:	5

Document for:	Discussion

Introduction

In last meeting, RAN2 sends an LS to RAN1 and SA2 asking questions about the data block sizes for Ambient IoT [1].

- 1 Overall description RAN2 have discussed the role of the MAC in handling upper-layer data blocks (MAC SDUs) and processing them into transport blocks (MAC PDUs) and the need of segmentation in RAN2. RAN2 expect to follow RAN1 conclusion and define MAC PDU sizes to align with the capacity of the physical layer/TB sizes. To take an informed decision about the need for the segmentation, RAN2 would like to know the TB sizes (in both D2R and R2D directions) RAN1 intend to specify/use. TR 38.848 and TS 22.369 indicate that a maximum message size of “approximately” 1000 bits is expected. RAN2 would like to understand what maximum size of application data could be expected in reality and if upper layers need to support segmentation to adapt their data to the maximum and typical TB sizes (both D2R and R2D directions). - 2 Actions To RAN1 RAN2 respectfully ask RAN1 to indicate what maximum and minimum TB sizes are expected to be specified in PHY, in both D2R and R2D directions and the limitations/conditions under which TBs of different sizes can be transmitted.

In this contribution, we provide our views on the raised questions.

Discussion

Based on the maximum application layer packet size, TR38.848 indicates that the design target for the maximum message size to be received by or transmitted from the Ambient IoT device is approximately 1000 bits [2]. In RAN1#117 meeting, message sizes of {20 bits, 96 bits, 400 bits} were considered for link-level simulations in coverage evaluations [3]. Hence, from RAN1 perspective, we suggest the minimum TB size is 20 bits, and the maximum TB size is 1000 bits.

RAN1#117 meeting Agreement For the link level simulation in coverage evaluation, {20 bits, 96 bits, 400 bits} are considered for message size. - Note: companies to report the M value and chip length used for each message size

Proposal 1: In both D2R and R2D directions, from RAN1 perspective, the minimum TB size can be 20 bits, and the maximum TB size can be 1000 bits.

Conclusion

In this document, we provide our views on the data block sizes for Ambient IoT. The proposal is summarized below:

Proposal 1: In both D2R and R2D directions, from RAN1 perspective, the minimum TB size can be 20 bits, and the maximum TB size can be 1000 bits.

Reference

- R2-2407831, “LS on data block sizes for Ambient IoT”, 3GPP TSG-RAN WG2#127, Maastricht, Netherlands, August 19th – 23rd, 2024.

- TR 38.848 v18.0.0, Study on Ambient IoT (Internet of Things) in RAN, September 2023.

- 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.