3GPP TSG RAN WG1 #118bis			R1-2407735

Hefei, China, October 14th – October 18th, 2024

Agenda item:		9.4.2.2

Source:	China Telecom

Title:	Discussion on frame structure and timing aspects for Ambient IoT

Document for:		Discussion

Introduction

In the RAN1 #118 meeting, issues on timing aspects are discussed [1], and the related agreements are listed as follows:

Agreement When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices Agreement Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following - How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 - How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1 Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection. For Direction 1, following can be the template to capture the solution details proposed by companies, if any. Source; Details. Source [x]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. Source [y]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. For Direction 2, following can be the template to capture the solution details proposed by companies, if any. Source; Details. Source [x]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. Source [y]; Solution description Observations or Analysis or Evaluations Specification impacts, if any.

According to the achieved agreements, several aspects need further discussion, including frame structure, synchronization design, access procedure, and schedule mechanism. This contribution will provide our views on the details of those functionalities.

Potential frame structure and timing design

Ambient IoT is targeted for ultra-low power consumption and complexity devices. Based on the achieved agreement in the SID [2], three types of devices are introduced for ambient IoT devices. Device 1 is a passive device with no proactive transmitting capability and only 1µW peak power consumption allowed. Such strict requirements cause almost all current NR mechanisms to be inapplicable. For device 2a, since some extra components are introduced to guarantee a better transmission performance, the power consumption is relaxed to a few hundred µW, but only backscatter transmission is supported as well, hence, a similar design may be reused for device 1. For device 2b, internal transmission is supported, which means the possibility of reusing the current NR procedure. But considering a harmonized design with minimized differences for those devices is recommended in the scope, at least the same design shall be utilized for device 2b.

Boundary alignment

In the NR system, timing units are uniformly defined as frame, slot, or symbol between the gNB and UE so that a harmonized transmission/reception and scheduling process can be performed. However, a similar definition is hard to utilize in IoT devices due to its high initial SFO, making it difficult to align the boundaries of those timing units. Hence, the necessity and the feasibility of timing unit boundary alignment should be further confirmed.

Boundary alignment for R2D transmission

Based on the agreement in the previous meeting, the start of the NR OFDM symbol boundary is assumed to be aligned, and the necessity of the end of the OFDM symbol boundary alignment is left for the next meeting. In the current R2D transmission, the number of bit values mapped in one OFDM symbol will depend on its target data rate. For example, in a typical NR system with 15KHz sub-carrier space without the consideration of Manchester coding, one-bit mapping in one OFDM symbol will achieve the data rate of 14kbps, and one-bit mapping in two OFDM symbol will achieve the data rate of 7kbps. Considering the value of bits mapped in one OFDM symbol is changed accordingly with the target data rate changes and the division of the number of OFDM symbols per milliseconds and the target data rate may not form an integer, the possible mapping scenario in a R2D transmission can be divided into three types:

- Scenario 1: The division is an integer, e.g., the target data rate can be configured as 14kbps or 28kbps.

- Scenario 2: The division is not an integer but is divisible, e.g., the target data rate can be configured as 7kbps or 11.2kbps.

- Scenario 3: The division is neither an integer nor can be divisible, e.g., the target data rate can be configured as 9kbps or 1.7kbps.

In scenario 1, since the number of bits within one OFDM symbol can only be integers, the end of the symbol boundary is naturally aligned and does not need to be further analyzed. For scenario 2, since the division is not an integer, the end of this chip mapped by the corresponding information bit may not be aligned with the OFDM symbols in the middle of the R2D transmission. Assuming that one chip needs to be mapped in one and a half OFDM symbols, it is obvious that two symbols are enough to carry 3 information bits, and 3 OFDM symbol allocations are not necessary for the pursuit of the alignment at the end of every symbol. In this case, the end occasion of the first OFDM symbol will be not aligned with the end occasion of the first chip, but the end occasion of the third transmission chip will be aligned with the end occasion of the second OFDM symbol, and no extra implement complexity will be introduced. Hence, the end alignment in the middle of the OFDM symbols occupied by the whole R2D transmission is not necessary.

Observation 1: The end of the NR OFDM symbol boundary is naturally aligned if the division is an integer.

Observation 2: During R2D transmission, the end alignment of the NR OFDM symbol is not necessary.

However, if the end occasion of the last R2D transmission chip is not aligned with the end of an OFDM symbol, several problems will be caused. For the waveform generation aspect, considering the IFFT point is based on a whole sampling size of one OFDM symbol, the OFDM modulation process will be regarded as the sum of the R2D transmission chips and the padding zeros, which will introduce extra information and have impacts on the detection performance. Seems this problem can be solved by scheduling, but it is impossible to schedule a half OFDM symbol or three-quarters of four OFDM symbols in the current NR system for a transmission unless we want to define them only for R2D transmission. Hence, the end occasion of the last R2D transmission chip shall be aligned with an OFDM symbol.

Observation 3: If the end occasion of the last R2D transmission chip is not aligned with an OFDM symbol, additional impacts on the gNB implementation will be caused.

Proposal 1: For the end of R2D transmission, the end occasion of the last R2D transmission chip shall be aligned with an OFDM symbol.

For scenario 3, considering the division is not divisible, the end alignment in the last OFDM symbol is impossible, which may cause trouble in the scheduling and waveform generation. Meanwhile, considering rounding is unavoidable for the bit numbers per OFDM symbol due to the inexhaustible division, CP handling will be a tricky problem since the CP distribution is not uniform and their locations are hard to seek after down-sampling at IoT devices. Hence, if a data rate under scenario 3 is provided, the data rate value is better avoided, and an approximate data rate value based on scenario 1 or scenario 2 can be considered to replace the scenario 3 data rate.

Proposal 2: The data rate value that is not divisible by the number of OFDM symbols per millisecond is better replaced by an approximate value that can be divided by the number of OFDM symbols per millisecond.

Boundary alignment for D2R transmission

Based on the progress in the last meeting, the necessity of NR slot/symbol boundary alignment is still being discussed. As described in the current SID, the SFO value is up to 105, which makes it hard to align each transmission to NR slot or symbol boundaries. Meanwhile, considering the D2R transmission will arrive at the gNB side at any time, there is still no need, and not realistic to ask for an alignment between the slot/symbol boundary and the D2R transmission. Hence, time alignment between NR slot/symbol and IoT devices during the D2R transmission is not necessary.

Proposal 3: The alignment for the NR slot/symbol boundary alignment is not necessary for the D2R transmission.

In that case, the first and the last OFDM symbol will be utilized only part of them, thus both of them should be considered as the target reception time unit. For example, if a D2R transmission is just received in the middle of the first or the last symbol, as described in Figure 1, the total time unit size required for the NR system is 5 symbols even if the total length of the D2R transmission is around 4 symbols. Then, gNB starts demodulation work at the first sampling point where the preamble is received to enable detection of the whole D2R transmission.

Figure 1 Illustration of time resource allocation on the gNB side

Proposal 4: Both the first and last symbol occupied by D2R transmission should be regarded as the allocated reception time unit of gNB.

Considering that D2R transmission will last for an indeterminate time interval, a minimum or maximum protection time gap may need to be defined so that D2R transmission can be detected without additional interference. Besides, the possibility that several D2R transmission derived by multiple devices arrives at gNB side still exists, thus the used time domain resource configured by gNB can be larger than only considering the first and last symbol occupied by D2R transmission as illustrated in Fig. 2, and each detection time duration of the multiple devices can have a time interval to guarantee no cross-detection interference will be caused.

Figure 2 Illustration of time resource allocation on the gNB side with multiple D2R transmissions

Proposal 5: The time domain resource allocation of multiple D2R transmission detection shall be studied.

Synchronization

In the current NR mechanism, a whole network-wide synchronization is adopted to assist the coherent OFDM demodulation and schedule the UL/DL transmissions to avoid interference. Considering the initial SFO of the clock component is up to 105 ppm for both device 1 and device 2, the timing effect will be extremely poor compared with the current 0.1 ppm NR system. Hence, the whole network-wide synchronization scheme does not apply to Ambient IoT devices.

RFID provides a possible synchronization transmission method [3], a predefined preamble or frame-sync is added ahead of the RFID command or UE reply for R=>T or T=>R communication, and a dummy 1 symbol is also carried at the end of the signaling in the T=>R communication. Considering the device complexity should be comparable with RFID, such designs can be regarded as a reference.

Figure 3 frame-sync and preamble for R=>T communication

Figure 4 frame-sync and preamble for T=>R communication

R2D synchronization procedure

- Postamble

Based on agreements in the last meeting, 2 options are considered to determine or derive the end of PRDCH transmission. In option 1, a postamble is utilized to sign the end of an R2D transmission, and such design may be not necessary if the length or payload size is indicated by R2D control information, which is regarded as option 2. Option 1 is a more general method, IoT devices can realize the end of R2D transmission as long as postamble is detected, and no need to recognize the length or payload size information included in the R2D control information. Therefore, the message size for R2D transmission can be configured more flexibly, and the potential payload overhead indicated for the end occasion of the PRDCH transmission will be saved. Besides, considering the postamble can be designed differently from the other aspects, the miss-detection issue may be not so critical. If a longer high-level voltage or longer chip length is utilized in the postamble part to distinguish from data parts, the potential miss-detection or data confusion issue can be avoided. Based on the above analysis, the utilization of postamble is more suitable to indicate the end occasion of PRDCH transmission.

However, the PRDCH channel structure is not defined yet, and the transmission content of PRDCH is usually command, which means the transmission block size is not large. As described in the RFID, the structure of the DL command usually has a fixed length, i.e., 22 bits if the command is identified as ‘Query’. It seems that the length information of the DL command can be automatically acquired as long as the command is identified by IoT devices, and the transmission length is usually small, thus the necessity to define a postamble indicating the end occasion seems not critical. Moreover, since only ‘DT’ or ‘DO-DTT’ traffic will be studied based on SID, the DL command type will be naturally limited, such as only ‘paging’, ‘Read’, or other limited DL command types, hence the overhead to distinguish the command type and acquire the command length information is also acceptable. Therefore, if a fixed length and only several commands will be used in the PRDCH transmission, a postamble is not necessary.

Proposal 6: The option to determine or derive the end of PRDCH depends on the PRDCH structure design:

- Postamble shall not be used if the DL command has a specific structure with a fixed length or only several commands will be considered.

- Otherwise, the postamble can be considered.

Figure 5 Potential synchronization structure for R2D transmission

- Midamble

Based on the previous meeting, the midamble is not configured in the R2D transmission as a baseline if Manchester coding is used, and whether it can be introduced for PIE coding will be studied. For the Manchester coding, a potential rising or down edge can be tracked to compensate for the time offset. Besides, considering the duration of each chip length is equal, the compensation will be plausible since no additional judgment on whether the duration between two adjacent edges is one chip or three chips caused by different PIE encoding symbols. As shown in our simulation results, considering the payload size in R2D transmission will not be too large, e.g., 20bit or 96bit, the compensation can be complicated by the devices even if a large timing offset is introduced, no more than 0.5dB degradation can be observed between the idle transmission condition and the large timing offset transmission condition in the 7kbps data rate, and the performance is slimier if the data rate drops to 1.75kbps. Therefore, the midamble in the R2D transmission is not needed if Manchester coding is utilized.

Figure 6 Performance of R2D transmission in Manchester coding when midamble is not inserted

Observation 4: In 7kbps, the performance degradation between idle timing and large timing offset assumptions is less than 0.5dB.

Observation 5: In 1.75kbps, the performance between idle timing and large timing offset assumptions is similar.

For PIE coding, though encoded symbol duration is different between symbol ‘0’ and symbol ‘1’, rising or down edge still exists for the potential timing tracking. Considering the clock signal is provided by a highly accurate gNB clock, such a rising or down edge is also helpful for symbol-level timing tracking of devices, and the only thing different from Manchester coding is to determine the duration between two adjacent edges is one chip or three chips, but the inserted midamble is not helpful for the duration judgment process. Moreover, inserted midamble in the PRDCH shall be handled on the device side, which means the corresponding midamble handing work needs to be discussed in the other agenda item. Considering the TU is limited, issues requiring additional workload should be avoided. Hence, midamble is not needed in the R2D transmission.

Proposal 7: Inserting a midamble in the R2D transmission is not supported.

D2R synchronization procedure

- Postamble

In the last meeting, methods to indicate the end of PDRCH transmission were also discussed, 2 options are provided, which can be divided by postamble or based on control information indication. Similar to R2D transmission, the utilization of postamble is a more general method. Considering the message size in the D2R transmission is usually not fixed, adding a postamble at the end of PDRCH provides more flexibility in the transmission. Besides, there still exist some scenarios for the ‘configure-based’ schedule, such as the potential ‘RN16’ information scheduled by message 0, the TBs or length information will be provided by message 0 or just by pre-defined, and the end of PDRCH transmission is accordingly confirmed based on such indication. Moreover, since only 16bit information will be transmitted, the timing offset impact will not be so large, which means the indication of the end occasion may be relatively accurate, thus postamble in this scenario may be not necessary. Hence, the end occasion indicated by R2D control information should not be precluded.

Proposal 8: For the reader to acquire the end of PDRCH transmission, support using D2R postamble to indicate the end occasion of PDRCH.

- The end occasion indicated by R2D control information may be not precluded in some scenarios.

For the method that ends occasion indicated by D2R control information, a D2R control channel needs to be separately designed, or a separate D2R length field is designed in the PDRCH channel to assist gNB in obtaining the PDRCH length information, which makes the D2R transmission design more complex. Considering the postamble has similar functionality but is simpler than such indication, the D2R control information indication option should be deprioritized in the current stage.

Proposal 9: The end occasion indicated by D2R control information should be deprioritized in the current stage.

- Midamble

The midamble should be considered in the UL transmission based on the previous meeting, and the corresponding design needs to be studied. Since the D2R transmission is usually long, the timing offset will be naturally large and have significant impacts on the detection performance. The largest 10% timing offset means the chip length will easily shift during the detection process, leading to the incorrect judgment of the chip voltage level. Besides, though the chip length variation due to the time offset can be compensated by tracking the rising or falling edges of the line code, the residual timing error accumulated by such tracking still exists and will be accumulated. If a long-duration D2R transmission is performed, this accumulation error affects the judgment of all the subsequent chips. Therefore, the midamble is introduced to reduce the accumulated timing offset and provide the channel condition by its signal strength measurement, whose functionality is similar to the preamble.

Proposal 10: Support the midamble in the D2R transmission at least in the large payload size transmission configuration.

Moreover, considering that midamble is necessary for the high payload scenario, we may need to design the midamble overhead to choose an appropriate midamble pattern and avoid low communication efficiency. In the D2R transmission, the ‘chip’ will be the only time unit that the devices can identify, which may be regarded as a basic time unit in the transmission process. Hence we may use an amount of ‘X chip’ as a length unit to determine whether a midamble needs to be introduced. The configuration of value ‘X’ can be pre-defined by a fixed value for simplicity, while considering the PDRCH transmission can also be scheduled by R2D transmission, especially for the scenario that the length/TBs of PDRCH is scheduled by PRDCH, the midamble configuration information can also be included in the PRDCH to determine whether midamble is needed and its detailed length, pattern information.  In addition, since the chip is derived from information bits, the ‘bit’ number can also be considered another alternative time unit for the midamble overhead study if the ‘chip length’ definition is not supported.

Proposal 11: The midamble overhead and other information can be indicated at least based on the following methods:

- Based on a pre-configured chip length or bit length.

- Indicated by the R2D control information.

Figure 7 Two potential synchronization structures for D2R transmission

In addition, since the functionality of the midamble is similar to the preamble, the design method of the midamble can be referred to as the preamble design. As agreed in the last meeting, the D2R preamble shall have the SFO estimation, channel estimation, or other functionalities. Thus, the potential preamble design can be regarded as a fixed sequence so that gNB can determine the start occasion and timing offset level by cross-correlation, and acquire the judgment threshold by obtaining the power of the high voltage and low voltage level on the preamble. The midamble can be regarded as an inserted preamble that has the mentioned functionalities, therefore the midamble can have the same design as the midamble. The design of the midamble can be a binary sequence with a good correlation performance, and the length shall be limited to avoid the too-large midamble overhead.

Proposal 12: The midamble of D2R transmission can be referred to the D2R preamble design:

- The same sequence design of the D2R preamble can be reused to the midamble design

- The sequence design should have a good correlation performance with limited overhead.

Finally, considering the IoT buffer size is not confirmed in the current stage, there exists the possibility that IoT transmission blocks cannot be sent only by one D2R transmission. For example, if the IoT buffer size is X and 2X bits are expected to be transmitted by the target IoT device, at least 2 transmissions are needed to transmit this TB. To signal that the transmission of the target device is not completed yet, assistant information can be provided on the midamble part to help a better reception at gNB. The potential assistant information can be regarded as a data transmission identifier which may be a completion indication or the middle indication to indicate the current transmission state.

Proposal 13: Midamble can also be used as assistant information to identify the transmission status in the D2R transmission.

- The assist information can be a middle indication to indicate that the current TB transmission is not completed yet.

- The assist information can be a completion indication to indicate that the current TB transmission has been completed.

- The calibration of the device clock

In the last meeting, whether a device can calibrate the device clock frequency for D2R transmission by using an R2D signal and transmitting D2R with a potential residual SFO is discussed. Considering the initial SFO is up to 105, the time offset may be a tricky problem and hard to solve in the detection process, thus the device shall have some capabilities to correct its clock and reduce the potential time offset so that the reader can have a better detection performance. In our simulation evaluation, the higher SFO value can be solved by an appropriate midamble pattern configuration, which proves that the large time offset can be compensated by the midamble configuration. Moreover, considering other encoding methods in the D2R transmission will not have the timing tracking performance like the Manchester encoding, a performance gap may exist in those encoding methods. Hence, the calibration process of the device sides may have its necessary. Considering a harmonized design shall be regarded as in the device functionalities, the device clock calibration process shall be designed with limited power consumption and complexity to meet the performance requirements of device 1.

Figure 8 Performance of different payload sizes under an appropriate D2R midamble configuration

Observation 6: The large time offset can be compensated by an appropriate midamble configuration.

Proposal 14: The calibration of the device clock process can be studied under a limited power consumption value to meet the device 1 requirement.

Timeline design

In the previous meeting, the conclusion mentioned that the timeline gap should be studied, and four basic time duration gaps should be defined for ambient IoT transmission, including TR2D_min, TD2R_min, TR2D_R2D_min and TD2R_D2R_min. The timeline requirements are all the minimum time gap requirements to guarantee no timeline overlap will present and enough preparation time such as detection duration, processing duration, and generation duration are guaranteed to complete a normal interaction between gNB and device. However, only defining minimum time gap requirements may be not enough for the whole ambient IoT system, thus some maximum timeline requirements or other necessary timeline requirements can be studied and included in the timeline design.

New timeline requirement

Based on the four defined minimum timeline requirements, we can naturally associate its corresponding maximum requirements, such as TR2D_max, TD2R_max, TR2D_R2D_max, and TD2R_D2R_max, and the meaningful of these maximum requirements will be discussed.

TR2D_max: Defined as the maximum time between an R2D transmission and the corresponding D2R transmission following it. It can be regarded as a maximum time occasion a device can transmit its D2R information and can avoid the endless waiting on the reader side if the R2D transmission is not received by the device side. If the TR2D_max has arrived, the reader will identify a wrong transmission has taken place, and perform a repetition or just a new operation for the target device.

TD2R_max: Defined as the maximum time between a D2R transmission and the corresponding R2D transmission following it. It can be regarded as a maximum time occasion the reader side can transmit the R2D information and can avoid the endless waiting on the device side if the D2R transmission is not received by the reader side. If a response is needed for the reader side and the TD2R_max has arrived, the device will consider that a wrong transmission has taken place, and wait for the potential CW energy to perform a repetition or just change to the unavailable stage to save energy.

TR2D_R2D_max: Defined as the maximum time between two different consecutive R2D transmissions to the same A-IoT device. Seems that it doesn’t have too many application scenarios, but if the device charging stage can be aware by the reader side, this requirement can help complete the interaction process within the device ‘available’ stage duration to improve the communication efficiency. Such as a scenario in a ‘write’ command is following a ‘read’ command and the reader attempts to complete the two processes within the device ‘available’ stage to avoid the long charging time duration for the device side.

TD2R_D2R_max: Defined as the maximum time between two different consecutive D2R transmissions to the same A-IoT device. It can be regarded as a maximum time occasion the device can respond for the reader side toward the same R2D command and can be used in the ‘in-processing reply’ scenario where the device needs extra time to prepare a D2R response for the reader and provide some continuous reply information duration the preparing duration. The reader can wait for the D2R response if a response is received before the maximum time occasion, and will give up waiting if the maximum time occasion has arrived but no reply information is detected.

Observation 7: TR2D_max, TD2R_max, TR2D_R2D_max, and TD2R_D2R_max all have their application scenarios.

Moreover, the discussed eight requirements don’t contain the charging time, but the charging process exists in the backscatter communication process and will affect the whole timeline design, hence we can separately define the CW timeline and CW requirement to consider the charging process as well as avoid to introduce the charging process in the mentioned 8 requirements. The new requirement, defined as TCW_D2R, can be introduced.

TCW_D2R: Defined as the time between the CW has transmitted and the D2R transmission to the target A-IoT device has started. It can be regarded as a time duration between CW and the D2R transmission to guarantee the devices always have enough energy and avoid considering the charging time in the mentioned 8 requirements to complicate timeline requirements. A potential timeline design method can be illustrated as follows:

Figure 9 An example of the potential timeline design method

Therefore, the introduction of TCW_D2R has its application scenario and can be studied in the timeline design.

Proposal 15: Considering the following timeline intervals in the timeline design:

- Support the TR2D_max and TD2R_max requirements.

- Study the necessity of TR2D_R2D_max and TD2R_D2R_max.

- Introduce the CW timeline requirement TCW_D2R.

Timeline requirements for different device types

It is worth noting that the device structure is quite different among all IoT devices. Taking device 1/2a as an example, compared with device 1, device 2a has an extra amplifier, and multi-bit ADC is also used instead of a simpler comparator, thus the processing time is naturally different between these devices. Considering inaccurate timeline relationships will affect the gNB scheduling effectiveness, the collision between the devices and the interference at the gNB side will be exacerbated, which may degrade the transmission performance and affect the coverage range, the time duration gaps are better defined separately based on device type. Meanwhile, as illustrated in our contribution [5], the E2E latency definition should be considered based on traffic type. DT traffic and DO-DTT traffic are naturally different in the different transmission types, hence their processing time are also different.

Proposal 16: The study should consider at least the following aspects on time interval:

- Implementation restrictions for the existing BS/UE.

- Processing time is different for different A-IoT devices.

- Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use cases (e.g., Inventory or Command) is different.

Indication method for timing interval

The potential timeline indication method is also discussed in the last meeting. A D2R transmission can occur by defining a timing interval of [TR2D_min, TR2D_max] to guarantee no time overlap will be caused between this D2R transmission and the previous R2D transmission or the next D2R transmission, or the D2R transmission occasion can be indicated by scheduling information in the PRDCH. A similar discussion is also carried out in the R2D transmission. Considering the transmission occasion depends on Reader indication costs additional signalling overhead and a more precise occasion acquired by IoT devices is not necessary in most scenarios, the D2R transmission occasion can be just defined in the timing interval range. Besides, in some specific scenarios such as 2-step contention-free access, considering the D2R transmission has already been scheduled by Reader, an extra access occasion indication in the scheduling information will not introduce too much overhead. Meanwhile, the indication of access occasion will help IoT devices access the network with a lower collision probability, which will improve access efficiency.

Proposal 17: The indication of the starting timing of the D2R transmission after an R2D transmission depends on the scenarios:

- For contention-free access, based on reader’s indication.

- Otherwise, define a timing interval to limit the transmission occasion range.

For the R2D transmission after a D2R transmission, the timing interval definition is necessary. If no maximum timing interval is defined to limit the reception range, the IoT devices will wait for the Reader information until its energy has been entirely consumed, which causes unnecessary energy waste and is an unreasonable design. Therefore, it is necessary to define a maximum time TD2R_max between the end of the D2R transmission and the start of the expected R2D transmission following it.

Proposal 18: Support to define a maximum time TD2R_max between the end of the D2R transmission and the start of the expected R2D transmission following it, so that the start of the expected R2D transmission timing is within [TD2R_min, TD2R_max].

Scheduling and indication

Time unit definition

In the last meeting, the time unit of R2D of D2R transmission is also discussed. Considering several time intervals have been introduced in the previous meeting, the existing interval duration shall be defined, especially for the case that TR2D_max needs to be indicated by the reader, the time interval duration will be indicated based on the size of basic time units. Therefore, a definition of time unit should be discussed.

In the R2D transmission, the information in the PRDCH channel is mapped based on OFDM symbols. However, the devices can not understand the duration of ‘an OFDM symbol’ since only the envelope can be identified by it instead of the OFDM waveform, thus reusing OFDM symbols as the basic time unit and indicating it to devices is not realistic. Considering the identified envelope is the OOK chip within the OFDM symbols, the time unit granularity of R2D transmission can be regarded as an amount of ‘OOK chip’ for the time interval duration, and the time unit length is defined as the OOK chip length.

For the D2R transmission, the chip length can be regarded as the time unit as well based on a similar reason, but considering the modulation method in the D2R transmission will not be limited by OOK, the time unit granularity of D2R transmission can be named as ‘chip’ for the indication of time interval duration, and the time unit length is also regarded as chip length.

Proposal 19: The time unit granularity of R2D transmission or D2R transmission can be defined as ‘chip’, and the time unit length can be the chip length.

Meanwhile, a discussion of whether a different chip length between data information and control information in the R2D transmission is necessary is also been mentioned in the last meeting. The benefits of different chip lengths can be realized such as flexibility and low-reliability acquirement for the data transmission, but the indication process may be quite complicated. In that case, a specific chip length indication may be needed previous to the data transmission, causing a complete channel or signaling design. For simplicity, a different OOK chip duration between control or data transmission shall not be supported in the chip length design.

Proposal 20: Different chip length between data information and control information in the R2D transmission is not supported.

Indication format design

Notice that an RFID command has its specific indication field, the corresponding ambient IoT command and its indication field should also be defined and modified. Take the ‘Query’ command as an example, it has nine fields with 22bits, and the structure is shown as follows:

Figure 10 The ‘Query’ command structure

The ‘Command’ field is utilized to distinguish the command function, the ‘DR’ field is used to select a suitable transmission data rate or frequency, ‘M’ represents the coding rate and other fields have specific functionalities. Considering the transmission data rate and the MCS indication may be performed by a new command, whether such fields are also necessary in the ‘Query’ command for the Ambient IoT command design may need further discussion. From this perspective, the command field design may differ between the RFID command and the Ambient IoT command even if a similar procedure has been carried out. Hence, the detailed command field design needs thorough consideration.

Proposal 21: The potential scheduling content and scheduling command field should be studied.

Energy harvest operation

Based on the discussion in the last meeting, the device status due to charging by energy harvesting was studied. As agreed in the RAN Plenary #103, the duration of one device’s unavailability due to charging can be assumed up to several tens of seconds. That means, at least two device status can be identified, divided into the availability status which can be communicated with the reader, and the unavailability status which must process charging before communication. To further describe the device behavior in the communication process and improve the corresponding communication efficiency, the device status needs to be identified. Two main categorizations for the device status definition can be listed as follows:

Categorization 1: Only the ‘ON’ status and ‘OFF’ status are defined for the device status definition. In the ‘ON’ stage, the device can communicate with the reader, and the device will be fully shut down in the ‘OFF’ status.

Categorization 2: Apart from the ‘ON’ status and ‘OFF’ status, a ‘SLEEP’ status is also defined for the device status definition. During the ‘SLEEP’ status, the device can run its clock in a low-power method, and the device can receive some wake-up signal so that it can transform to the ‘ON’ status in an expected time interval when receiving the wake-up message.

The introduction of the ‘SLEEP’ status will make the devices work more positively, improving the communication efficiency between the reader and devices. Compared with the devices working in a purely negative method, the reader may control the status of devices by its control information and schedule the devices to be waken up in an expected occasion, avoiding the potential charging waiting time caused by the low-energy devices. Therefore, the introduction of the ‘SLEEP’ status is helpful and necessary in the ambient IoT system.

However, considering the maintenance of a running clock will cost relatively large power consumption for the devices, whether the introduction of the ‘SLEEP’ status still meets the power consumption requirements shall be studied. Based on the SID description, the power consumption of device 1 is only a few µW, the requirement is too strict to run a clock even if it is not so accurate, thus the ‘SLEEP’ status is hard to support for device 1. For device 2, since the power consumption requirement is up to several hundred µW, it seems that the ‘SLEEP’ status design has the feasibility to be supported.

Proposal 22: The device status for the ambient IoT devices can be defined based on the device type:

- For device 1, the device status only contains the ‘ON’ status and the ‘OFF’ status.

- For device 2, additionally introduce the ‘SLEEP’ status if the power consumption requirement can be met.

Conclusions

In this contribution, we discuss the potential design of synchronization, random access and scheduling, and have the following observations and proposals:

Observation 1: The end of the NR OFDM symbol boundary is naturally aligned if the division is an integer.

Observation 2: During R2D transmission, the end alignment of the NR OFDM symbol is not necessary.

Observation 3: If the end occasion of the last R2D transmission chip is not aligned with an OFDM symbol, additional impacts on the gNB implementation will be caused.

Observation 4: In 7kbps, the performance degradation between idle timing and large timing offset assumptions is less than 0.5dB.

Observation 5: In 1.75kbps, the performance between idle timing and large timing offset assumptions is similar.

Observation 6: The large time offset can be compensated by an appropriate midamble configuration.

Observation 7: TR2D_max, TD2R_max, TR2D_R2D_max, and TD2R_D2R_max all have their application scenarios.

Proposal 1: For the end of R2D transmission, the end occasion of the last R2D transmission chip shall be aligned with an OFDM symbol.

Proposal 2: The data rate value that is not divisible by the number of OFDM symbols per millisecond is better replaced by an approximate value that can be divided by the number of OFDM symbols per millisecond.

Proposal 3: The alignment for the NR slot/symbol boundary alignment is not necessary for the D2R transmission.

Proposal 4: Both the first and last symbol occupied by D2R transmission should be regarded as the allocated reception time unit of gNB.

Proposal 5: The time domain resource allocation of multiple D2R transmission detection shall be studied.

Proposal 6: The option to determine or derive the end of PRDCH depends on the PRDCH structure design:

- Postamble shall not be used if the DL command has a specific structure with a fixed length or only several commands will be considered.

- Otherwise, the postamble can be considered.

Proposal 7: Inserting a midamble in the R2D transmission is not supported.

Proposal 8: For the reader to acquire the end of PDRCH transmission, support using D2R postamble to indicate the end occasion of PDRCH.

- The end occasion indicated by R2D control information may be not precluded in some scenarios.

Proposal 9: The end occasion indicated by D2R control information should be deprioritized in the current stage.

Proposal 10: Support the midamble in the D2R transmission at least in the large payload size transmission configuration.

Proposal 11: The midamble overhead and other information can be indicated at least based on the following methods:

- Based on a pre-configured chip length or bit length.

- Indicated by the R2D control information.

Proposal 12: The midamble of D2R transmission can be referred to the D2R preamble design:

- The same sequence design of the D2R preamble can be reused to the midamble design

- The sequence design should have a good correlation performance with limited overhead.

Proposal 13: Midamble can also be used as assistant information to identify the transmission status in the D2R transmission.

- The assist information can be a middle indication to indicate that the current TB transmission is not completed yet.

- The assist information can be a completion indication to indicate that the current TB transmission has been completed.

Proposal 14: The calibration of the device clock process can be studied under a limited power consumption value to meet the device 1 requirement.

Proposal 15: Considering the following timeline intervals in the timeline design:

- Support the TR2D_max and TD2R_max requirements.

- Study the necessity of TR2D_R2D_max and TD2R_D2R_max.

- Introduce the CW timeline requirement TCW_D2R.

Proposal 16: The study should consider at least the following aspects on time interval:

- Implementation restrictions for the existing BS/UE.

- Processing time is different for different A-IoT devices.

- Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use cases (e.g., Inventory or Command) is different.

Proposal 17: The indication of the starting timing of the D2R transmission after an R2D transmission depends on the scenarios:

- For contention-free access, based on reader’s indication.

- Otherwise, define a timing interval to limit the transmission occasion range.

Proposal 18: Support to define a maximum time TD2R_max between the end of the D2R transmission and the start of the expected R2D transmission following it, so that the start of the expected R2D transmission timing is within [TD2R_min, TD2R_max].

Proposal 19: The time unit granularity of R2D transmission or D2R transmission can be defined as ‘chip’, and the time unit length can be the chip length.

Proposal 20: Different chip length between data information and control information in the R2D transmission is not supported.

Proposal 21: The potential scheduling content and scheduling command field should be studied.

Proposal 22: The device status for the ambient IoT devices can be defined based on the device type:

- For device 1, the device status only contains the ‘ON’ status and the ‘OFF’ status.

- For device 2, additionally introduce the ‘SLEEP’ status if the power consumption requirement can be met.

- References

- Draft Report of 3GPP TSG RAN WG1 #118 v0.3.0, 3GPP TSG-RAN WG1 Meeting #118, Maastricht, NL, August 19th-August 23rd, 2024.

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG-RAN Plenary Meeting #102, Edinburgh, UK, December 11th – December 15th, 2023

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz – 960 MHz v2.0.1, April, 2015

- TR 38.848 v18.0.0, Study on Ambient IoT (Internet of Things) in RAN, September, 2023.

- R1-2402510, Discussion on evaluation assumptions and results for Ambient IoT, 3GPP TSG-RAN WG1 Meeting #116-bis, Changsha, Hunan Province, China, April 15th-April 19th, 2024.

- Appendix: Evaluation assumption for link-level simulation

|  | Parameters |  | Assumptions | Company result1 |  |
| --- | --- | --- | --- | --- | --- |
|  | R2D/D2R common parameters |  |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template | 900MHz |  |
| [0b] | SCS |  | 15 kHz as baseline | 15kHz |  |
| [0c] | Block structure |  | Blocks as agreed in 9.4.2.3, or other blocks reported by companies | Preamble+data+CRC +Midamble+Postamble |  |
| [0d] | Channel model |  | <Editor’s Note: will be updated according to the agreements made for channel model> | TDL-A |  |
| [0e] | Delay spread |  | [30, 150] ns - An RMS delay spread of 30 ns (M) and [150] ns (O) is considered for TDL-A channel model. - An RMS delay spread of 30 ns is considered for TDL-D channel model. | 30ns |  |
| [0f] | Device velocity |  | 3 km/h | 3km/h |  |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 | 1 |  |
| [0h1] | BS | Number of antenna elements | 2 or 4 | 2 |  |
| [0h2] |  | Number of TXRUs | 2 or 4 | 2 |  |
| [0j1] | Intermediate UE | Number of antenna elements | 1 or 2 | - |  |
| [0j2] |  | Number of TXRUs | 1 or 2 | - |  |
| [0m] | Reference data rate |  | [0.1, 1, 5] kbps [0.1] kbps (M), [1] kbps (M), [2] kbps (O), [5 ~ 7] kbps (O M), [large value] (O) 1 kbps (M), 5 ~ 7 kbps (M), [large value] (O) FFS:0.1 kbps (M), - Note1: companies to report the exact data rate. - Note 2: the exact data rate is close to the values listed above. - Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads (e.g., CRC, pre/mid/post-ambles if present). - Note 4: the exact data rate may be related to coding scheme, repetition and etc. - Note 5: all data rates considered are for evaluation purpose only | 7.5kbps |  |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. - Note 1: companies to report the M value and chip length used for each message size - Note 2: CRC is not included for the message size | 20bit |  |
| [0p] | BLER target |  | 1%, 10% | - |  |
| [0q] | Sampling frequency |  |  | 1.92MHz |  |
| [0r] | Device 1/2a/2b |  | Options are as follows, - Device 1, RF-ED - Device 2a, RF-ED - Device 2b, RF-ED/IF-ED/ZIF <Editor’s Note: will be updated according to agreements from 9.4.1.2> | Device 1, RF-ED Device 2a, RF-ED |  |
|  | R2D specific parameters |  |  |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz as baseline. Other larger values are not precluded. | 180kHz |  |
| [1b] | FFS: ED bandwidth |  | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. | RF-ED, 20MHz |  |
| [1c] | FFS: BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at [Y] kHz, half of R2D transmission bandwidth. Companies to report X = {3, 5}. | 5 order Butterworth with cutoff frequency at 90KHz. |  |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator | OOK waveform |  |
| [1e] | Modulation |  | OOK Companies to report, e.g., OOK-1, OOK-4 with M chips per OFDM symbol | OOK |  |
| [1f] | Line code |  | Companies to report, e.g., Manchester, PIE | Manchester |  |
| [1g] | FEC |  | No FEC as baseline | No |  |
| [1h] | ADC bit width |  | 1-bit for device 1 4-bit for device 2 | 4 bit |  |
| [1j] | Detection/decoding method for Line code |  | Companies to report | - |  |
|  | D2R specific parameters |  |  |  |  |
| [2a1] | Transmission bandwidth (w.r.t. D2R data rate) |  | - [2a1]-Alt1 (M): - DSB - X kHz (M) and Y kHz (O) is considered for D2R transmission bandwidth. - The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz (M) and Y kHz (O). - [2a1]-Alt2: - SSB - X kHz (M) and Y kHz (O) is considered for D2R transmission bandwidth. - The value is for one sideband, i.e., the total transmission bandwidth for DSBSSB is X kHz (M) and Y kHz (O). - For device 2b only, FFS for device 2a. - The value of X and Y is as follows, to be down-select from alternative 1 and 2 - Alternative 1: - X = {15 (M), 180 (O)} - Y =180 - Alternative 2: - X and Y reported by companies, - the value may be related to, e.g., - Reference data rate - Coding scheme - Repetition - With or without SFS - SSB or DSB - X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies | DSB, 15kHz |  |
| [2a2] | [OOK/BPSK/BFSK chip rate] |  | Companies to report | OOK chip, chip rate = 15kbps |  |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. - Assume the receiver matches the transmitter's modulation, i.e., to receiver uses SSB when transmitter uses SSB, receiver uses DSB when transmitter uses DSB. Companies to report the value, and further down-selection of the values and DSB/SSB is not precluded. | 15KHz |  |
| [2b] | Waveform (CW) |  | Companies to report waveform, e.g., unmodulated single tone, multi-tone(multiple unmodulated single tone) | Unmodulated single tone |  |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK, BFSK | OOK |  |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, FM0 encoding, Miller encoding, no line coding | Manchester |  |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC | No |  |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit | 11 bit |  |
| [2j] | D2R receiver |  | FFS: Reader receiver, e.g., coherent receiver / non-coherent receiver Companies to report, e.g., coherent receiver / non-coherent receiver | Non-coherent receiver |  |
|  | Other assumptions |  |  |  |  |
| [3a] | Other assumptions |  | To be reported by company | - |  |