R1- 2407814

3GPP TSG RAN WG1 Meeting #118bis

Hefei, October 14-18, 2024

| Agenda Item (AI): | 9.2.3 |
| --- | --- |
| AI Description: | UL 3Tx Support UL 3Tx with non-coherent codebooks. |
| Source: | MediaTek Inc. |
| Title: | Support for 3-antenna-port codebook-based transmissions |
| Document for: | Discussion |

- Introduction

In the RAN (Plenary) Meeting #102, a new Work Item (WI) was approved targeting MIMO evolution for Downlink (DL) and Uplink (UL). The 4th objective of the aformentioned WI is:

The lagacy specifications include support of up to 1/2/4 UL TX ports, allowing up to maximum of 4 trasnmission layers with Codebook and Non-Codebook based schemes.

- The codebook transmission scheme entails providing the UE with a specific Transmit Precoder Matrix Indicator (TPMI) chosen from a predefined codebook, based on non-precoded SRS transmission.

- The non-codebook scheme, on the other hand, relies on channel reciprocity, to deduce general precoding weights. Those weights are in turn used to precode SRS. Up to 4 SRS resources in one SRS Resource Set are configured, with only one port per SRS resource. The Network (NW) implicitly indicates to UE which precoding weights to use, in addition to the trasnsmission rank, through SRS Resource Indicators (SRI).

However, the commercial UEs in the market only support a maximum of 2Tx ports with 2Rx or 4Rx capability. To enhance the commercial handset UEs from 2Tx capability, 3Tx in the UL is explored in this work item without significant modifications from the existing 4Tx design structure. Thanks to the innovations in RF, which allows a UE to support 3 parallel RF chains for UL transmission. Thus 3Tx is being viewed as trade-off between the improved performance with optimized RF architecture.

The agreements till the previous RAN1 meeting try to achieve majority of the objectives in the WID. Thus in the RAN plenary (RAN#105), it is agreed to update the WID to enhance the 3Tx with non codebook based transmission and SRS antenna switching features. RAN4 had proposed to study the SRS antenna switching for 3Tx UE and on the other side the specification impact for enabling non codebook based transmission is low. Considering these aspects, both of the features are incorporated into the WID objective as below.  In this document, we try to discuss our proposals on the updated WID objective.

WID Objective (updated as in RP#105): Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource.

- Note: UL full power transmission mode 1 and 2 are not supported.

- Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

- SRS antenna switching for 3Tx.

SRS antenna switching is configured to obtain DL CSI acquisition using the UL SRS signal in case of UL-DL reciprocity. Typically, receiver dimensions of a UE are larger than the transmitter dimensions. To spatially cover the receiver dimensions from the UL SRS transmission, SRS resources are switched over different Tx ports based on the transmitter capability. As it was concluded in RANP to support 3T6R, we propose to support 3T6R SRS antenna switching without the enhancements on SRS. Also, support for 3T3R is enabled based on the UE capability. Though, 3Rx and 6Rx is not typical commercial configuration (unlike 4Rx and 8Rx), we tend to support 3T6R and 3T3R SRS antenna switching as the spec impact would be higher with 3T4R or 3T8R. Because the current specification expects to have same number of SRS ports in each resource of a given resource set with Antenna switching usage. Thus, without any specification modification 3T6R can be supported. Also, the current agreements on SRS (in R1-116b) allows to support 2 SRS resources within a resource set which allows to operate 3T6R.

Agreement in RAN1#116b

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, support Alt1,

- Alt1: Support configuration of X 4-port SRS resources in a resource set where one the ports is muted.

  - FFS muting mechanism.

where X can be up to 2, subject to UE capability.

Two SRS resources within an SRS resource set with usage Antenna switching can be configured for 3T6R. Each of the resource is 4-port resource with the last port being muted. Thus, two uniform SRS resources with equal number of active ports can be enabled to support 3T6R without disturbing the existing specifications.

Proposal 1: Support SRS antenna switching of 3T6R with SRS resource set usage as Antenna switching.

- Non codebook based transmission.

The SRS design for non codebook based transmission is different from that of codebook based transmission. SRS resource set should be able to support three single port SRS resources within a set for non codebook transmission. As the Legacy 4Tx UE can support up to 4 single port SRS resources, 3 single port SRS resource configuration is possible with existing specification. Thus, enabling the support for 3 maximum layers enable the 3Tx UE with non-codebook transmission feature.

Proposal 2: Support non codebook transmission by enabling the max number of layers as 3 for 3Tx UE.

In non codebook transmission, SRS resource indication (SRI) corresponding to the number of layers is carried out using the 3 single port SRS resources. SRI indication in the Legacy for 4Tx UE can be reused for 3Tx UE. As there are only 3 ports for 3Tx UE, SRI tables can be reused from the existing SRI indication Tables of 4Tx by ignoring NSRS=4.

Table 1: SRI indication for Lmax=1

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 |
|  |  | 2 | 2 |
|  |  | 3 | reserved |

Table 2: SRI indication for Lmax=2

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 |
| 3 | reserved | 3 | 0,1 |
|  |  | 4 | 0,2 |
|  |  | 5 | 1,2 |
|  |  | 6-7 | reserved |

Table 3: SRI indication for Lmax=3

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 |
| 3 | reserved | 3 | 0,1 |
|  |  | 4 | 0,2 |
|  |  | 5 | 1,2 |
|  |  | 6 | 0,1,2 |
|  |  | 7 | reserved |

Proposal 3: Reuse the Legacy SRI indication mechanism for the 3Tx UE non codebook transmission. Reuse the SRI indication Tables of Legacy with NSRS={2,3}.

- Conclusions

The document discussed enhancememnts targeting codebook transmission to enable up to 3 Tx UL operation. Based on the observations, the proposals are presented as below.

Proposal 1: Support SRS antenna switching of 3T6R with SRS resource set usage as Antenna switching.

Proposal 2: Support non codebook transmission by enabling the max number of layers as 3 for 3Tx UE.

Proposal 3: Reuse the Legacy SRI indication mechanism for the 3Tx UE non codebook transmission. Reuse the SRI indication Tables of Legacy with NSRS={2,3}.

- References

[1] RP-23400 - New WID: NR MIMO Phase 5, Edinburgh, Scotland, December 11-15, 2023.