3GPP TSG RAN WG1 #118bis	R1-2407839

# Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | --- | rev | --- | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on default TCI state for PDSCH without TCI field in DCI in HST-SFN scheme (Rel-18 mirror) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-17 HST-SFN discussion, default TCI state determination rules have been agreed, accordingly if PDSCH is scheduled by DCI without TCI field if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL as follows. Agreement When SFN PDSCH and SFN PDCCH are configured by RRC, for PDSCH reception scheduled by DCI formats 1_1 and 1_2, and, if applicable the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the TCI state(s) of the scheduling CORESET when receiving the PDSCH - If there are two active TCI states for the CORESET, UE applies both QCL assumptions of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, if there is one active TCI state for the CORESET, UE applies the one active TCI state of the CORESET when receiving the PDSCH This feature is UE optional capability - If UE doesn’t support this capability, UE is expected to be configured with TCI state field - UEs supporting this feature and are not capable of dynamic switching between single TRP and SFN, the CORESET that schedules PDSCH by DCI formats 1_1 and 1_2 (FFS DCI format 1_0) should be activated with two TCI states. FFS for maintenance: if SFN PDCCH is not configured Agreement When SFN PDSCH is not configured by RRC and there is no TCI codepoint which indicates two TCI states activated for the PDSCH (i.e. Rel-16 MTRP PDSCH is not configured) and SFN transmission scheme 1 is configured for PDCCH, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2 without TCI field, if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL if applicable and the CORESET which schedules the PDSCH is indicated with two TCI states, the default TCI state is defined as the first TCI state of the CORESET.. However, in Rel-17 specification TS 38.214, “without TCI field” is missing after DCI format 1_0/1_1/1_2 for HST-SFN scheme, which would lead to the wrong behavior when determining the TCI state for PDSCH reception. Besides, sfn-DefaultDL-BeamSetup-r17 is defined to indicate whether the UE supports the following features in TS 38.306. - For FR2 only, PDSCH reception using default beam for enhanced SFN scheme when PDSCH is scheduled with offset less than threshold - For FR1 and FR2, PDSCH reception using default beam for enhanced SFN scheme when TCI field is not present in DCI format 1 0/1 1/1 2 when PDSCH is scheduled with offset equal or larger than the threshold, if applicable. - For FR2 only, aperiodic CSI-RS reception using default beam for enhanced SFN scheme when scheduling offset is less than threshold.. It can be noticed that sfn-DefaultDL-BeamSetup-r17 is not only for the case of TCI field not present in DCI format 1 0/1 1/1 2 when PDSCH is scheduled with offset equal or larger than the threshold. To remove the potential confusion, the unnecesary condition, i.e., for DCI scheduling without TCI field after sfn-DefaultDL-BeamSetup-r17 in TS 38.214 should be removed. |
| Summary of change: |  | 1. To add a phrase “without TCI field” after DCI format 1_1/1_2 2. To remove the condition “for DCI scheduling without TCI field” after sfn-DefaultDL-BeamSetup-r17 3. To modify “1_1, 1_2” to “1_1/1_2” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The default TCI state dermination is not correct if “without TCI field” is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.5	Antenna ports quasi co-location

The UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability maxNumberConfiguredTCIstatesPerCC. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH, the DM-RS port of PDCCH or the CSI-RS port(s) of a CSI-RS resource. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

-	'typeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'typeB': {Doppler shift, Doppler spread}

-	'typeC': {Doppler shift, average delay}

-	'typeD': {Spatial Rx parameter}

The UE can be configured with a list of up to 128 TCI-State configurations, within the higher layer parameter dl-OrJointTCI-StateList in PDSCH-Config for providing a reference signal for the quasi co-location for DM-RS of PDSCH and DM-RS of PDCCH in a BWP/CC, for CSI-RS, and to provide a reference signal with qcl-Type set to 'typeD', if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a BWP/CC, and SRS.

If the TCI-State or TCI-UL-State configurations are absent in a BWP of the CC, the UE can apply the TCI-State or TCI-UL-State configurations from a reference BWP of a reference CC configured by unifiedTCI-StateRef. The UE is not expected to be configured with tci-StatesToAddModList, SpatialRelationInfo or PUCCH-SpatialRelationInfo, except SpatialRelationInfoPos in a CC in a band, if the UE is configured with dl-OrJointTCI-StateList or ul-TCI-StateList in any CC in the same band. The UE can assume that when the UE is configured with tci-StatesToAddModList in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with dl-OrJointTCI-StateList or ul-TCI-StateList in any CC within the same band in the CC list.

The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs. When a set of TCI state IDs are activated for a set of CCs/DL BWPs and if applicable, for a set of CCs/UL BWPs, where the applicable list of CCs is determined by the indicated CC in the activation command, the same set of TCI state IDs are applied for all DL and/or UL BWPs in the indicated CCs. If the activation command maps TCI-State(s) and/or TCI-UL-State(s) to only one TCI codepoint, the UE shall apply the indicated TCI-State(s) and/or TCI-UL-State(s) to one or to a set of CCs /DL BWPs, and if applicable, to one or to a set of CCs /UL BWPs once the indicated mapping for the one single TCI codepoint is applied as described in [11, TS 38.133].

When the bwp-id or cell for QCL-TypeA/D source RS in a QCL-Info of the TCI state is not configured, the UE assumes that QCL-TypeA/D source RS is configured in the CC/DL BWP where TCI state applies.

When tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, a UE configured with dl-OrJointTCI-StateList with activated TCI-State or ul-TCI-StateList with activated TCI-UL-State receives DCI format 1_1/1_2/1_3 providing indicated TCI-State(s) and/or TCI-UL-State(s) for a CC or all CCs in the same CC list configured by simultaneousU-TCI-UpdateList1-r17, simultaneousU-TCI-UpdateList2-r17, simultaneousU-TCI-UpdateList3-r17, simultaneousU-TCI-UpdateList4-r17. The DCI format 1_3 provides indicated TCI state(s) and/or TCI-UL-State(s) for the CC(s) in a scheduledCellListDCI-1-3 if the UE is scheduled by the DCI format 1_3 to receive PDSCH at least on one serving cell in the scheduledCellListDCI-1-3. The DCI format 1_1/1_2 can be with or without, if applicable, DL assignment. If the DCI format 1_1/1_2 is without DL assignment, the UE can assume the following:

-	CS-RNTI is used to scramble the CRC for the DCI

-	The values of the following DCI fields are set as follows:

-	RV = all '1's

-	MCS = all '1's

-	NDI = 0

-	Set to all '0's for FDRA Type 0, or all '1's for FDRA Type 1, or all '0's for dynamicSwitch (same as in Table 10.2-4 of [6, TS 38.213]).

After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states:

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, otherwise.

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure

After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state or an initial higher layer configuration of ul-TCI-StateList where more than one TCI-UL-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states:

-	The UE determines the UL TX spatial filter, if applicable, for dynamic-grant based PUSCH that is not transmitted during the RACH procedure and configured-grant based PUSCH and PUCCH that are not transmitted during the RACH procedure, and for SRS applying the indicated TCI state, from the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, otherwise.

-	The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during the initial access procedure

After a UE receives a higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states:

-	The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

After a UE receives a higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state or a higher layer configuration of ul-TCI-StateList where more than one TCI-UL-State can be used as an indicated TCI state as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states:

-	The UE assumes that the UL TX spatial filter, if applicable, for dynamic-grant and configured-grant based PUSCH and PUCCH, and for SRS applying the indicated TCI state, is the same as that for a PUSCH transmission scheduled by a RAR UL grant or a MsgA PUSCH transmission during random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

If a UE receives a higher layer configuration of dl-OrJointTCI-StateList where only one TCI-State can be used as an indicated TCI state, the UE obtains the QCL assumptions from that TCI state for DM-RS of PDSCH and DM-RS of PDCCH, and the CSI -RS applying the indicated TCI state.

If a UE receives a higher layer configuration of dl-OrJointTCI-StateList where only one TCI-State can be used as an indicated TCI state or a higher layer configuration of ul-TCI-StateList where only one TCI-UL-State can be used as an indicated TCI state, the UE determines an UL TX spatial filter, if applicable, from that TCI state for dynamic-grant and configured-grant based PUSCH and PUCCH, and SRS applying the indicated TCI state.

When a UE configured with dl-OrJointTCI-StateList would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to one or more PDSCH(s) scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State(s) is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) should be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state for a CC/BWP to be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state carried in the latest DCI, for the corresponding coresetPoolIndex value when applicable, in time corresponding to positive HARQ-ACK value is applied. The first slot and the beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK.

When a UE is configured with dl-OrJointTCI-StateList and is having one indicated TCI-state, and if the UE is configured with unifiedTCI-StateType is set as ‘separate’, and if the UE receives a TCI codepoint mapped with either of {TCI-State, TCI-UL-State}, the UE shall update the one indicated {TCI-State, TCI-UL-State} and maintain the other {TCI-State, TCI-UL-State} that is not updated by the received TCI codepoint.

When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, if the UE receives a TCI codepoint mapped with a sub-set of first and second TCI-State(s) and/or a sub-set of first and second TCI-UL-State(s), the UE shall update the first/second TCI-State(s) and/or first/second TCI-UL-State(s) mapped to the TCI codepoint, when applicable, and keep the previously indicated first/second TCI-State(s) and/or first/second TCI-UL-State(s) that is/are not updated by the TCI codepoint.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, and the UE is receiving a DCI carrying the TCI-State indication and without DL assignment, the UE does not expect that the number of indicated SLIVs in the row of the pdsch-TimeDomainAllocationListForMultiPDSCH by the DCI is more than one.

If the UE is configured with SSB-MTC-AddtionalPCI and with PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE receives an activation command associated with each coresetPoolIndex, as described in clause 6.1.3.14 of [10, TS 38.321] or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP. When a set of TCI state IDs are activated for a coresetPoolIndex, the activated TCI states corresponding to one coresetPoolIndex is associated with the serving cell physical cell ID and activated TCI states corresponding to another coresetPoolIndex can be associated with another physical cell ID.

When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command.

When a UE configured with dl-OrJointTCI-StateList supports tci-JointTCI-UpdateMultiActiveTCI-PerCC-r18, the UE may receive an activation command, as described in clause 6.1.3.70 of [10, TS 38.321], the activation command is used to map up to 8 sets of TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs, where each set is comprised of up to two TCI state(s) for DL and UL signals/channels.

When a UE configured with dl-OrJointTCI-StateList supports tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-r18, the UE may receive an activation command, as described in clause 6.1.3.71 of [10, TS 38.321], the activation command is used to map up to 8 sets of TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs, where each set is comprised of up to two TCI state(s) for DL channels/signals and up to two TCI state(s) for UL channels/signals.

When the DCI field 'Transmission Configuration Indication' is present in DCI format 1_2 and when the number of codepoints S in the DCI field 'Transmission Configuration Indication' of DCI format 1_2 is smaller than the number of TCI codepoints that are activated by the activation command, as described in clauses 6.1.3.14, 6.1.3.24, 6.1.3.47, 6.1.3.70 and 6.1.3.71 of [10, TS38.321], only the first S activated codepoints are applied for DCI format 1_2.

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and for FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command,

-	the UE assumes that DM-RS of ports of PDSCH of a serving cell are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321], except during RACH procedure for RACH-based LTM, if applicable, otherwise

-	the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'.

If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling a PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 or format 1_3 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentDCI-1-2 for the CORESET scheduling the PDSCH, the UE assumes that the TCI field with a DCI field size indicated by tci-PresentDCI-1-2 is present in the DCI format 1_2 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the multicast PDSCH, the UE assumes that the TCI field is present in the DCI format 4_2 of the PDCCH transmitted on the CORESET. If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability [13, TS 38.306], for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

When a UE is configured with both sfnSchemePDCCH and sfnSchemePDSCH scheduled by DCI format 1_0 or by DCI format 1_1/1_2 without TCI field, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable:

-	if the UE supports sfn-DefaultDL-BeamSetup-r17, the UE assumes that the TCI state(s) or the QCL assumption(s) for the PDSCH is identical to the TCI state(s) or QCL assumption(s) whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell regardless of the number of active TCI states of the CORESET. If the UE does not support sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17, the UE should be activated with the CORESET with two TCI states.

-	else if the UE does not support sfn-DefaultDL-BeamSetup-r17, the UE shall expect TCI field present when scheduled by DCI format 1_1/1_2.

When a UE is configured with sfnSchemePDSCH and sfnSchemePDCCH is not configured, when scheduled by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE shall expect TCI field present.

For PDSCH scheduled by DCI format 1_0 or by DCI format 1_1/1_2 without TCI field, when a UE is configured with sfnSchemePDCCH set to 'sfnSchemeA' and sfnSchemePDSCH is not configured, and there is no TCI codepoint with two TCI states in the activation command, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL if applicable and the CORESET which schedules the PDSCH is indicated with two TCI states, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the first TCI state or QCL assumption which is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

If a UE is not provided dl-OrJointTCI-StateList-r17, and if a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. For a single slot PDSCH, the indicated TCI state(s) should be based on the activated TCI states in the slot with the scheduled PDSCH. For a multi-slot PDSCH or the UE is configured with higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, the indicated TCI state(s) should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

< Unchanged parts are omitted >