3GPP TSG RAN WG1 #118bis		                     R1-2407856

Hefei, China, October 14th – 18th, 2024

Source: 	vivo

Title:	Remaining issues on asymmetric DL sTRP/UL mTRP scenarios

Agenda Item	9.2.4

Document for:	Discussion and Decision

Introduction

In [1], one of the objectives is to support asymmetric DL sTRP/UL mTRP deployment scenarios to further improve UL throughput. And, objective b is added to support 2 TA in the revised WID [2].

5. Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 a. Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. b． Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Great progress has been achieved in previous few meetings and the basic function for the above objective has been achieved. In this contribution, we provide our opinions on remaining issues to support asymmetric DL sTRP/UL mTRP deployment scenarios. Besides, to address the objective b, we also propose necessary specification clarification to support 2 TA in this scenario.

Remaining issues on asymmetric DL sTRP/UL mTRP scenarios

- PL offset indication for PRACH

Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. Three options can be considered for the PDCCH order design:

- Option 1: 1-bit to indicate one of the indicated joint/UL TCI states

- Option 2: 1-bit to indicate whether to apply the PL offset in the indicated TCI state or not.

- Option 3: repurpose PRACH association indicator to indicate whether to apply the PL offset in the indicated TCI state or not.

Option 1 is not suitable for Rel-17 unified TCI/ICBM framework which is also supported for asymmetric DL sTRP/UL mTRP deployment scenarios, for which whether to apply the PL offset value configured in the indicated TCI state should be indicated. Since only one joint TCI state or one UL TCI state is indicated, indication of which one TCI state can be used is redundant.

Option 2 is feasible for both Rel-17 unified TCI/ICBM and Rel-18 unified TCI. Considering that only one joint or UL TCI state is configured with PL offset value no matter one or two joint or UL TCI states are indicated.

Regarding Option 3, PRACH association indicator field is present only when two TAGs are configured. It is not reasonable to bundle two TA and PDCCH-order PRACH towards UL-only TRP operation together. So, it is better to introduce a new 1-bit field indication instead of repurposing the PRACH association indicator field.

Support Option2: 1-bit to indicate whether to apply the PL offset in the indicated TCI state or not.

- On support of 2 TA

Two TAGs associated with different coresetPoolIndex values are introduced to support TRP-specific timing in mDCI-based mTRP transmission. In asymmetric DL sTRP/UL mTRP deployment scenarios, UL timing difference of UL transmission may exceed CP length. So, 2TA can be supported to enable flexible deployment of asymmetric DL sTRP/UL mTRP deployment scenarios. To support 2TA for sDCI-based mTRP, specification change is required. First step is to remove the restriction on association between coresetPoolIndex and TAG. Then, definition of DL reference timing is also different from current two TAGs because there is no DL timing for the UL-only TRP. LS is required to RAN4 to inform this issue.

Remove restriction on association between coresetpoolindex and TAG.

Send LS to RAN4 on DL timing definition of two TAGs.

Besides, if two UL timing are supported, two PUSCH transmission occasions in mTRP TDM repetition scheme would be overlapped due to different timing offset. How to deal with this situation should be further studied. As shown in Table 1, Maximum overlapping symbols derives from MTTD and number of overlapped OFDM symbols varies for different SCS. In general, maximum 1 or 2 OFDM symbols would be overlapped.

Maximum number of overlapped OFDM symbols based on Ran4 specification

| Μ | FR1 | FR2 | Maximum number of overlapped OFDM symbols |
| --- | --- | --- | --- |
| 0 | 34.6 |  | 1 |
| 1 | 34.6 |  | 1 |
| 2 | 34.6 | 8.5 | 2 for FR1, 1 for FR2 |
| 3 |  | 8.5 | 1 |

In Rel-18 mDCI-based PUSCH transmission, if two UL transmission overlap due to different UL timing, if UE is not capable to reduce the later transmission, UE does not expect to be scheduled two overlapped UL transmission. Considering sDCI based PUSCH or PUCCH repetition, overlapping can be avoided by scheduling for inter-slot repetition by at least one or two symbols scheduling gap between repetitions using different timing. NW has knowledge on the transmission timing difference because difference of two TACs from two TAGs equals to two UL timing difference. Flexible scheduling can be achieved according to UL timing difference. So, the handling rule defined in Rel-18 can be reused directly. However, for intra-slot repetition, overlapping symbol cannot be avoided because consecutive repetitions are scheduled. As shown in Figure 1, symbols dropping happens in all repetitions related to TA1, which will degrade mTRP transmission performance. It should be noted that this is not a corner case and should be seriously considered.

Intra-slot repetition scheduling when two TAG are supported

Reuse Rel-18 overlapping handling rule for inter-slot repetition scheme, that is UE does not expect repetitions associated with different TAGs to overlap.

FFS: how to handle overlapping for repetition intra-slot repetition scheme.

- Default CLI, TAG ID and PL offset indication

Agreement - Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state. - Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state. - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

Typically, SRS resource set with usage set to ‘beam management’ are not configured with any QCL assumption for it is UE implementation. How to determine closed-loop PC adjustment state for SRS transmission without TCI state indication in such case should be further studied. PL offset value is also configured in TCI state, so for BM SRS resource set without TCI state configuration, whether and how to determine PL offset value for SRS transmission should be discussed. Besides, if two TAGs are configured, which TAG id is used for UL timing should also be determined. For simplicity, unified design to configure CLI, TAG ID and PL offset value for SRS resource set without TCI state indication is preferred.

Support configure CLI, TAG ID and PL offset value for SRS resource set without TCI state indication.

- Design on PL offset update MAC CE

Agreement For the association between PL offset and joint/UL TCI state, support the following • Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

MAC CE update of the PL offset value(s) for joint or UL TCI states is to amend the PL offset between macro TRP and UL-only TRP due to UE mobility. For the usage of MAC CE, following three options can be considered and determine which one is the intended behavior:

- Option 1: the MAC CE can update PL offset value of any RRC configured TCI states

- Option 2: the MAC CE can update PL offset value of activated TCI states by TCI state activation MAC CE

- Option 3: the MAC CE can update PL offset value of the current applied TCI states

Option 1 is unnecessary. Latest PL offset value of the un-activated TCI states can be indicated by the TCI state activation MAC CE. So, there is no motivation to support MAC CE to update the PL offset value of the un-activated TCI states. Compared to option 3, option 2 can update all the PL offset value from all activated TCI states, which is more practical considering that all the PL offset will change due to UE movement. So, we prefer the MAC CE to update PL offset value of activated TCI states by TCI state activation MAC CE.

The PL offset update MAC CE can update PL offset for any activated TCI state for the UL-only TRP.

Regarding the MAC CE design, to update PL offset value of any activated TCI states, the MAC CE should indicate the target TCI states and associated PL offset values together to inform UE which PL offset is used to update which TCI state. Two alternatives can be considered:

- Option 1: the MAC CE indicates N set of {TCI state ID, PL offset value}

- Option 2: the MAC CE indicates N set of {TCI state codepoint indication, PL offset value}

Compared to Option 1, option 2 saves at least N*4 bits overhead. So, Option 2 should be supported. In addition， considering that maximum 8 TCI states codepoints is supported, maximum value of N is 8.

The MAC CE indicates N set of {TCI state codepoint indication, PL offset value}, N<=8.

- Type 3 PHR reporting

Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

- Three cases should be considered for type 3 PHR reporting. Case 1: For single cell operation，no Type 3 PHR is reported because there is no SRS transmission when PHR is reported. Case 2: For serving cell with SUL carrier configuration, type 3 PHR is reported only when type 1 PHR reporting is based on reference transmission and type 3 PHR is based on actual SRS transmission. However, SUL carrier and UL carrier is deployed on the same TRP because TCI state and PL-RS is shared for both carriers hence no PL offset is required. Case 3: For multiple cell operation, actual and virtual type 3 PHR can be reported for a serving cell without PUSCH-config which is not the UL-only TRP case. So, for all these cases, there is no need to introduce PL offset in the calculation of Type 3 PHR.

There is no need to introduce PL offset in the calculation of Type 3 PHR.

- Starting bit of DCI format 2_3

Agreement About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, down-select one from the following Alts in RAN1#118bis: - Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is applicable to any rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise. - Alt2: X = 44 (to be captured in RAN2 spec) - This feature is only applicable to UE who is configured with two separate SRS CLPC adjustment states. - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise. - Above extended value range is applied to startingBitOfFormat2-3.

- On the extended value X of starting bit of blocks in DCI format 2_3 in Rel-19, Alt2 is preferred. Alt1 should be excluded because it is out of scope. Motivation of extended value is to extend bit length of each block, so this capability should be bundled with Rel-19 two separate SRS CLPC adjustment states.

On the extended value X of starting bit of blocks in DCI format 2_3, Alt2 is preferred.

- Clarification on TCI state indication

- Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2. - Joint TCI state mode can be configured at least for FR1. Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: • When rel-17 unified TCI/ICBM is configured: o For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. o For FR2: {one DL TCI state + one UL TCI state} can be applied. • When rel-18 unified TCI is configured: o For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. o For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In last meeting, it has been concluded that up to two joint TCI states or {one DL TCI state + up to two UL TCI state} for FR1 can be applied and {one DL TCI state + up to two UL TCI states} for FR2 can be applied for Rel-18 unified TCI for the asymmetric DL sTRP/UL mTRP deployment scenario. To support single DL TCI state application, further clarification is required because it is different from Rel-18 TCI state framework.

Whether to reuse two indicated DL TCI states framework in Rel-18 for the asymmetric DL sTRP/UL mTRP deployment scenario should be further discussed because it may go beyond the UE capability, even though single DL TCI state application for DL transmission can be achieved by the higher layer parameter transmission configuration indication for PDCCH and TCI selection field indication in DCI for PDSCH by selecting the applied DL TCI state of two indicated TCI states. As shown in Figure 2, two indicated DL TCI states can be from two separate DL TRPs (case 1) or from one DL TRP (case 2) if two indicated DL TCI states are supported for the asymmetric DL sTRP/UL mTRP deployment scenario. Case 1 means the UE operates in the scenario with three TRPs, while case 2 means two TCI states are indicated for single TRP transmission. Both scenarios in case 1 and case 2 are beyond the UE capability, since they are not supported in release16-18 mTRP framework.

(case 1)                                           (case 2)

TCI states indication for the asymmetric DL sTRP/UL mTRP deployment scenario if two indicated DL TCI states are supported

To restrict mTRP capability, following two options can be considered for separate TCI state indication:

- Option 1: restrict that two indicated DL TCI states are same

- Option 2: support single indicated DL TCI state

  - transmission configuration indication for PDCCH and TCI selection field in DCI to indicate which TCI state of two indicated TCI states are not needed

Compared to Option 1, RRC and DCI signaling can be saved with option 2. So, we prefer Option 2.

Support single indicated DL TCI state for separate TCI indication for the asymmetric DL sTRP/UL mTRP deployment scenario.

- transmission configuration indication for PDCCH and TCI selection field in DCI are not needed.

For joint TCI state indication for FR1, two joint TCI states should be indicated for UL mTRP transmission. To restrict that only one TCI state is applied for the TRP with both DL and UL transmission, QCL info if configured for these two TCI states should be same. And to further reduce signaling overhead, transmission configuration indication for PDCCH and TCI selection field in DCI are also not needed.

TCI info for two indicated TCI state for joint TCI state indication should always be same.

- transmission configuration indication for PDCCH and TCI selection field in DCI are not needed.

•	PL offset measurement

PL offset can be acquired by gNB by measuring receiving power of SRS. As shown in Figure 3, one SRS transmitted by UE can be received by both TRPs for FR1 due to the omni-directional antenna assumption at UE. So, PL offset can be calculated by gNB based on the receiving power difference. And, this is purely gNB implementation and no spec impact is required for this measurement method. However, for FR2, two SRS resources are transmitted towards two TRPs. If both are configured in one SRS resource set, which means transmission power of the SRS resources are same, PL offset can also be measured based on the difference between receiving power at gNB. Therefore, no spec impact is envisioned for this method either because SRS with usage set to BM can be reused for PL offset measurement. However, this method has negative impact on SRS measurement itself, because the transmission power is derived neither suitable for macro TRP nor suitable for the UL-only TRP. Taking BM SRS resource as an example, if the transmission power targets for UL-only TRP, receiving RSRP for the macro TRP of all SRS resources in the same set will be very low. It means the beam quality of macro TRP is very low from gNB’s point of view, even though it is because of low transmission power. But, gNB has no knowledge on the real cause of low RSRP. This will impact UL beam selection. So, it is better to follow TRP-specific SRS power control and configure separate SRS resource sets for macro TRP and UL-only TRP. If two SRS resources are from two SRS resource sets, UE should inform transmission power of these two SRS resources for PL offset calculation at the gNB. This method enables gNB to configure SRS flexibly. So, whether to design specific PL offset measurement procedure and specific SRS configuration should be further discussed.

PL offset measurement by one SRS resource for FR1

PL offset measurement by two SRS resources for FR2

To enable PL offset measurement for FR2, reporting of transmission power of SRS can be supported.

Conclusion

In this contribution, following proposals are presented:

- Support Option2: 1-bit to indicate whether to apply the PL offset in the indicated TCI state or not.

Remove restriction on association between coresetpoolindex and TAG.

Send LS to RAN4 on DL timing definition of two TAGs.

Reuse Rel-18 overlapping handling rule for inter-slot repetition scheme, that is UE does not expect repetitions associated with different TAGs to overlap.

FFS: how to handle overlapping for repetition intra-slot repetition scheme.

Support configure CLI, TAG ID and PL offset value for SRS resource set without TCI state indication.

The PL offset update MAC CE can update PL offset for any activated TCI state for the UL-only TRP.

The MAC CE indicates N set of {TCI state codepoint indication, PL offset value}, N<=8.

There is no need to introduce PL offset in the calculation of Type 3 PHR.

On the extended value X of starting bit of blocks in DCI format 2_3, Alt2 is preferred.

Support single indicated DL TCI state for separate TCI indication for the asymmetric DL sTRP/UL mTRP deployment scenario.

- transmission configuration indication for PDCCH and TCI selection field in DCI are not needed.

TCI info for two indicated TCI state for joint TCI state indication should always be same.

- transmission configuration indication for PDCCH and TCI selection field in DCI are not needed.

To enable PL offset measurement for FR2, reporting of transmission power of SRS can be supported.

References

RP-234007, NR MIMO Phase 5, Samsung (Moderator).

RP-242394, WID revision: NR MIMO Phase 5, Samsung (Moderator).

RAN1#118 Chiarman Notes.