3GPP TSG RAN WG1 #118bis		R1-2407871

Hefei, China, 14th – 18th October, 2024

Source:	vivo

Title:	Discussion on LP-WUS operation in CONNECTED modes

Agenda Item:	9.6.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we present our views for the LP-WUS operation in CONNECTED modes.

- Procedures of LP-WUS triggering PDCCH monitoring

Following agreements were made in RAN1#118 meeting [4]:

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

We provide our views on above FFS points in the following.

Details on option 1-2 works alone

For Option 1-2, the PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer. Instead, the PDCCH monitoring is triggered based on the detection of LP-WUS. Since the legacy drx-onDurationTimer will not trigger the PDCCH monitoring, it is not necessary to start the legacy drx-onDurationTimer based on the legacy DRX configuration. Option 1-2 functions alone in the following way:

After RRC provides the LP-WUS monitoring configuration to a UE, the procedure/feature of PDCCH monitoring triggered by LP-WUS is activated. When the UE detects the LP-WUS at the configured monitoring occasion (MO), after a time gap (the time gap is discussed in following section 3), it starts a Timer (e.g., WUS-Timer) and also starts PDCCH monitoring. During the running time of the WUS-Timer, UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected once it starts PDCCH monitoring. For example,

- If the PDCCH indicating a new transmission is detected, the UE start/restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception;

- If the PDCCH indicating a DL retransmission, the UE starts/restarts the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback and the UE stops the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

- If the PDCCH indicating a UL retransmission,the UE starts/restarts the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the corresponding PUSCH transmission and the UE stops the drx-RetransmissionTimerUL for the corresponding HARQ process.

- If no PDCCH is detected until the WUS-Timer and/or drx-InactivityTimer expires, the UE starts LP-WUS monitoring and stops PDCCH monitoring.

Based on above procedure, on the FFS of the timer is existing timer and/or new timer during which UE monitors PDCCH triggered by WUS, the existing DRX timer such as drx-onDurationTimer is designed to cover the uncertainty of whether the traffic arrivals including traffic jitter. While the Timer for option 1-2 here is used for the confirmed traffic which initiates LP-WUS transmission to trigger PDCCH monitoring. Therefore, a new Timer (e.g., WUS-Timer) for cleaner design is preferred.

Proposal 1: Supporting following for Option 1-2.

- When LP-WUS is detected, LP-WUS triggers the start of a new timer during which UE monitors PDCCH

- The UE starts LP-WUS monitoring and stops PDCCH monitoring based on one of the following

  - No PDCCH is detected during the new timer running and the new timer expires

  - Expiration of drx-InactivityTimer

  - Explicit indication received to deactivate PDCCH monitoring and activate LP-WUS monitoring, e.g. by PDCCH skipping command.

Clarification on periodic CSI/L1-RSRP reporting

38.214 5.1.6 UE procedure for receiving reference signals 5.1.6.1 CSI-RS reception procedure […] If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' when drx-onDurationTimer in DRX-Config is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to cri-RSRP when drx-onDurationTimer in DRX-Config is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported.

For option 1-1 and option 1-2 of LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operaion is same as Rel-16 DCP. More specifically, the UE can be configured with one of the following for periodic CSI/L1-RSRP reporting when the UE is operating with LP-WUS option 1-1 or option 1-2:

- Configuration 1:

  - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is not reported.

  - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time.

- Configuration 2: Regardless if UE is indicated to wake-up or not, the periodic CSI/L1-RSRP is reported during the time duration indicated by drx-onDurationTimer in DRX-Config.

Proposal 2: Clarify that for both LP-WUS operation option 1-1 and option 1-2 of LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operaion is same as Rel-16 DCP, that is the UE can be configured with one of the following:

- Configuration 1:

  - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is not reported.

  - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time.

- Configuration 2:

  - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is reported during the time duration indicated by drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start.

  - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time.

Regarding the FFS on potential restriction for LP-WUS configuration in relation with C-DRX configuration, in our view, since the main motivation for option 1-2 is to enable more flexible triggering for PDCCH monitoring that is not restricted by C-DRX configuration, there should be no restriction for LP-WUS configuration in relation with C-DRX configuration. Network still has full flexibility to configure the LP-WUS MOs close to the legacy drx-onDurationTimer and/or the periodicity of legacy C-DRX cycle can be an integer multiple of the periodicity of LP-WUS MOs if needed.

Proposal 3: For LP-WUS operation option 1-2, there is no restriction for LP-WUS monitoring configuration (periodicity and offset) in relation with C-DRX configuration.

Regarding the simultaneous support of both Option 1-1 and Option 1-2 for the same UE, we believe this can be addressed once there is a clear understanding of how Option 1-2 functions independently.

One remaining issue raised initially for option 1-1 (e.g., LP-WUS achieves similar function as Rel-16 DCP) which can be common for both  LP-WUS operation option 1-1 and option 1-2 is that whether LP-WUS also needs to support SCell dormancy indication. Although it would be nice for LP-WUS to support it, it is not essential function/feature for LP-WUS, especially considering the limited payload size of LP-WUS compared to the DCI format 2_6. In addition, the SCell dormancy indication can be achieved by other DCI formats e.g., DCI format 0_1/0_2/0_3/1_1/1_2/1_3. Therefore, we prefer that LP-WUS does not support the SCell dormancy indication.

Proposal 4: For both LP-WUS operation option 1-1 and option 1-2, LP-WUS does not support SCell dormancy indication.

  - LP-WUS operation for UE configured with CA

Following agreement was made in RAN1#118 meeting [4]

Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC

There are two issues need further discussion on how LP-WUS works when UE is configured with CA in RRC CONNECTED mode:

Issue 1: on which cell(s) to monitor the LP-WUS

Issue 2: the cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

About on which cell(s) to monitor the LP-WUS, considering Case 1 that LP-WUS procedures to trigger PDCCH monitoring work with C-DRX configuration was agreed, there are two options.

- Option 1: UE monitors LP-WUS only on one serving cell per MAC entity

- Option 2: UE monitors LP-WUS only on one serving cell per DRX group

In our view, even if the secondary DRX group is configured, except drx-onDurationTimer and drx-InactivityTimer, other DRX parameters are common to the two DRX groups and the start of the drx-onDurationTimer is also the same for the two DRX groups. So, the active time between the two DRX group is actually overlapped. When there is traffic, it is highly probable that the active times of the two DRX groups will be aligned. In addition, LP-WUS monitoing on both primary and secondary DRX groups in parrelell will increase the UE complexity and power consumption. So, we slightly prefer not to optimize the case for dual DRX groups.

Proposal 5: For both LP-WUS operation option 1-1 and option 1-2, a UE monitors LP-WUS only on one serving cell per MAC entity when UE is configured with CA in RRC CONNECTED mode.

- The serving cell where to monitor LP-WUS is configured by the network.

- This applies to UEs configured with single and dual DRX groups

For issue 2 on which cell(s) the PDCCH monitoring triggered by a LP-WUS is applicable, there are two options.

- Option 1: one or more serving cells based on gNB indication/configuration

- Option 2: all activated serving cells

For option 1, further discussion is needed on how a LP-WUS is applicable to one or more serving cells based on gNB indication/configuration. One option, option 1-a is that the same bit in the WUS content is applied/corresponds to one or more serving cells configured by higher layers; The other option, option 1-b is that different bit(s) in the form of bitmap or codepoint corresponding to one or more serving cells. In addition, for option 1, it also needs to discuss whether/how to support a WUS or different WUS that can be applied to different one or more serving cells. Option 1 gives more flexibility, but such flexibility is at the cost of the WUS overhead and more discussion and standardization efforts. What’s more, the flexibility is not be essential or necessary for WUS operation combined with C-DRX. Therefore, option 2 should be supported.

Proposal 6: For both LP-WUS operation option 1-1 and option 1-2, when UE is configured with CA in RRC CONNECTED mode, the LP-WUS information is applied to all activated serving/scheduling cells.

- FFS for the case of dual DRX groups, LP-WUS received in the serving cell of primary DRX group indicates the triggering of PDCCH monitoring in primary DRX group only, or both DRX groups

Once supporting LP-WUS for a UE configured with CA is clear, we think it can be extended to the case where a UE configured with dual connectivity (DC) that is the LP-WUS monitoring on a serving cell per cell-group, depending on UE capability.

Proposal 7: For a UE configured with DC and for both LP-WUS operation option 1-1 and option 1-2, it can support LP-WUS monitoring on a serving cell per cell-group depending on UE capability.

- Timeline between LP-WUS reception and PDCCH monitoring

Following agreements were achieved on the timeline between LP-WUS reception and PDCCH monitoring in RAN1#116 and RAN1#118 meeting:

Agreement@RAN1#116 For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one Agreement@RAN1#118 Select one of the following alternatives in RAN1#118bis Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network

For connected mode, the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring that is reported by a UE should include the time for LP-WUS processing and the transition time for MR ramping up.

About whether the reported value(s) for the minimum time gap includes the duration for time/frequency synchronization of MR, for RRC connected mode, ultra-deep sleep is not allowed for MR and MR is still required to perform RLM/BFD/CSI and RRM measurements periodically (e.g. one measurement every 160ms) regardless if PDCCH monitoring is required or not. Therefore, at least for MR in micro and light sleep state, it is reasonable to assume MR already synchronizd once it wakes up. The same applies to MR deep sleep, i.e. MR does not lose coarse sync if periodic RLM/BFD/CSI/RRM measurement are maintained during connected mode. Even if additional CSI-RS/TRS may be needed for finer sync before MR PDCCH/PDSCH, it is highly dependent on the UE implementation and the reference signal (CSI-RS/TRS) availability thus difficult to make the aligned assumptions.

Therefore, we prefer the minimum time gap does not include the duration for time/frequency synchronization of MR for RRC connected mode, network can configure a larger gap considering MR time/frequency synchronization.

Proposal 8: For both LP-WUS operation option 1-1 and option 1-2 in or RRC CONNECTED mode, the reported value(s) of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring includes LP-WUS processing time and MR transition time for ramp up and does not include the duration for time/frequency synchronization of MR.

The time required to process LP-WUS can be defined based on the SCS of the carrier where the WUS is situated. The MR transition time for ramping up should be a fixed duration, depending on how deep the MR's sleep state is and irrespective of the SCS. The minimum interval is the combined time of the LP-WUS processing time and the MR transition time for ramping up. Given that for LP-WUS operation in CONNECTED mode, the MR CA may include carriers that are on different frequency bands than LR, the value(s) reported by the UE for each SCS in the agreements can be understood or specified as the SCS of the LR, which is the carrier where the LP-WUS resides.

Proposal 9: Clarify that the SCS for the reported value(s) for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is the respective SCS of the carrier where LP-WUS is monitored.

Between above two alternatives, each has pros and cons. Alt.1 offers simplicity in terms of signalling and ease of network configuration. However, providing a single value lacks adaptability to different scenarios concerning latency and power efficiency. The UE is forced to either compromise latency or power-saving performance all the time as the UE capability signalling is static and cannot be updated based on traffic needs. Alt.2, which allows a UE to report multiple gap values for MR waking up, offers more network flexibility in balancing latency and power saving according to the traffic needs, especially when there are significant differences in power-saving gains among these multiple values. Depending on the UE implementation, the UE can report one or multiple values, and if multiple values are reported, network can control the UE MR sleep behaviour by configuring the proper time gap considering tradeoff between traffic need and UE power saving. For example, if two values are reported by the UE, the one with smaller gap value (corresponding to MR micro or ligh sleep) can be used by the network when UE is operating with relatively latency sensitive traffics (e.g. XR), the one with lager gap value (corresponding to MR deep sleep) can be used by the network when UE is operating with non-latency sensitive traffics (e.g. FTP download).

To allow the above flexibility, up to two values per SCS seems sufficient.

Proposal 10: Support Alt.2 that for RRC CONNECTED mode for both LP-WUS operation option 1-1 and option 1-2, UE reports multiple values for each SCS from two candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

- Different minimum time gaps correspond to different sleep states

- The reported value(s) includes LP-WUS processing time and MR transition time for ramp up and does not include the duration for time/frequency synchronization of MR.

- Network can configure a proper time gap according to UE reporting, and considering the trade-off between traffic latency requirement and the UE power saving benefit.

- Activation and deactivation procedure for LP-WUS monitoring

Following agreements were achieved in RAN1#116 [1]:

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

As agreed, gNB RRC signaling should be used for enable/disable the procedure/feature of PDCCH monitoring triggered by LP-WUS. When UE correctly decodes the PDSCH carrying the RRC signalling for enabling the LP-WUS operation, UE will feedback ACK to network, then gNB can know the UE starts LP-WUS monitoring. In addition, it should be the common understanding that if the LP-WUS is not detected on a WUS MO by a UE, the UE is not required to wake up the MR for PDCCH monitoring. During LP-WUS monitoring, UE RRM/RLM/BFD/CSI measurements are still performed by MR in RRC connected mode. Therefore, based on the measurement report, gNB can disable the LP-WUS operation by RRC signalling before it loses the control of UE’s LP-WUS monitoring. Even if a UE autonomous fallback from LP-WUS monitoring to PDCCH monitoring, we think additional indication from UE is not necessary. Since such autonomous fallback case is rare and keeping connection with network is top priority and the UE should continue PDCCH monitoring if needed. Introducing additional report requires network to preconfigure/reserve some UL resource for the UE, which increase the unnecessary overhead.

Proposal 11: In RRC_CONNECTED mode, if the LP-WUS is not detected on a WUS monitoring occasion (MO) by the UE,

- For LP-WUS operation option 1-1, the UE is not required to trigger the starting of the drx-onDurationTimer.

- For LP-WUS operation option 1-2, the UE is not required to wake up the MR for PDCCH monitoring for option 1-2.

Proposal 12: In RRC_CONNECTED mode, for both LP-WUS operation option 1-1 and option 1-2, the indication from UE to indicate whether it monitors LP-WUS or PDCCH is not needed.

In case LP-WUS monitoring is already enabled by the RRC signaling, the options specifally for option 2 and 3 listed in above agreement from our understanding are about switching between LP-WUS monitoring and PDCCH monitoring. Therefore, Option 1 that No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring is not necessarily to be discussed. We can directly discuss the Option 2 and Option 3.

- Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

For Option 2, the L1/L2 signaling for activation/deactivation WUS monitoring can be a new signalling or can enhance Rel-17 PDCCH skipping signalling if Rel-17 PDCCH skipping is configured. For example, when LP-WUS monitoring is configured by RRC signalling, UE starts monitoring the LP-WUS. When LP-WUS is detected, the UE starts and continues monitoring the PDCCH until the UE receives the L1/L2 activation signalling for LP-WUS. Option 2 is straightforward and gNB can dynamically switch the PDCCH monitoring and LP-WUS monitoring.

- Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

Following conditions are discussed in previous meetings.

- Condition 1: the insufficient/sufficient coverage of LP-WUS

- Condition 2: Timer based activation/deactivation of WUS monitoring.

For Condition 1 of insufficient coverage of LP-WUS, as discussed, for RRC-connectd UE, RRM/RLM/BFD/CSI measurements are performed by MR and network can use RRC signalling or L1/L2 signalling to disable or deactiavte the WUS operation based on the measurement results. Therefore, condition 1 of insufficient/sufficient coverage of WUS is not needed for deactivation/activation of LP-WUS.

For Condition 2 of Timer based activation/deactivation. Following two types of timers are considered.

- First type of Timer can be viewed as LP-WUS activation Timer that is used to activate/resume LP-WUS monitoring. As discussed in section 2, When LP-WUS is detected, UE starts a new timer during which MR PDCCH monitoring is performed. At the expiration of this new timer (i.e. no PDCCH is detected during the new timer running), the UE stops monitoring PDCCH and starts/resumes LP-WUS monitoring. Another example is that UE can start/resume LP-WUS monitoring when drx-InactivityTimer expires, i.e. when the traffic delivering to the UE completed.

- Second type of Timer can be viewed as LP-WUS deactivation Timer that is used to avoid the case that UE does not receive the WUS for a long time due to bad channnel condition. The motivation for WUS deactivation Timer is the same as Condition 1. Therefore, in our view, the WUS deactivation Timer is not needed.

It should be noted that, not having a specified condition to deactivate LP-WUS monitoring due to poor channel condition does not prevent UE fallback from LP-WUS monitoring to PDCCH monitoring, due to worse channel condition, by UE implementation. In such case, it can be clarified that for LP-WUS operation option 1-1, the UE can fallback by implementation to PDCCH monitoring behavior according to C-DRX configuration. And for LP-WUS operation option 1-2, the UE can fallback by implementation to always-on PDCCH monitoring.

Proposal 13: After LP-WUS monitoring is enabled by RRC signalling, the specified conditions used for LP-WUS deactivation due to poor channel condition for LP-WUS monitoring is not needed for both LP-WUS operation option 1-1 and option 1-2.

Proposal 14: UE is allowed to fallback to MR PDCCH monitoring by implementation (e.g. due to poor channel condition) without specified condition.

- For LP-WUS operation option 1-1, the fallback PDCCH monitoring behavior is according to the C-DRX configuration

- For LP-WUS operation option 1-2, the fallback PDCCH monitoring behavior is always-on PDCCH monitoring.

- Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

In case uplink transmission such as SR, PRACH, Configured Grant (CG) PUSCH are triggered by UE self, the UL transmisisons should be transmitted irrespective WUS monitoring, and MR PDCCH monitoirng is started after the UL transmission according to the legacy UE behavior. From our understanding, it is the leagcy UE behavior and should not be impacted by the introduction of LP-WUS.

In addition, during LP-WUS opeartion in CONNECTED mode, gNB explicit indiction (L1/L2) can be used to indicate the UE to stop PDCCH monitoring and start LP-WUS monitoring, when the traffic delivering is completed for a given time period. Such indication can be built on PDCCH skipping command or seperate L1/L2 indication.

Based on above discussion, following proposals are made:

Proposal 15: For LP-WUS operation option 1-1 and option 1-2 in RRC_CONNECTED mode, UL transmissions such as SR, PRACH and CG PUSCH can trigger MR PDCCH monitoring according to the legacy UE behavior.

Proposal 16: For LP-WUS operation option 1-1 and option 1-2 in RRC_CONNECTED mode, UE switching from PDCCH monitoring to LP-WUS monitoring is supported by gNB L1/L2 signaling.

- LP-WUS Co-exists with existing features

Following agreements was achieved in RAN1#117 meetings [3]:

Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. - Rel-16 DCP - Rel-17 PDCCH skipping - Rel-17 SSSG switching - Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

RAN2 agreed in RAN2#126 that RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. In the following, we provide our views on how LP-WUS co-exists with PDCCH skipping, SSSG switching and cell-DTX.

LP-WUS co-exists with PDCCH skipping

During SI, PDCCH skipping indication used as LP-WUS activation signalling was studied. For example, when the UE receives Rel-17 PDCCH skipping indication, UE stops monitoring PDCCH and starts WUS monitoring. If this way of co-existence between LP-WUS and Rel-17 PDCCH skipping is adopted, following potential enhancemens should be discussed.

- Enhancement#1 on PDCCH skipping duration and UE behavior: Rel-17 PDCCH skipping indication also indicates how long time in unit of slots, the UE skips monitoring PDCCH. After the skipping duration, the UE should resume PDCCH monitoring. In case the PDCCH skipping indication is used to activate the LP-WUS monitoring, the PDCCH skipping duration should be assumed infinite since UE will resume PDCCH monitoring once it detects WUS.

- Enhancement#2 on applying PDCCH skipping indication to multiple cells: Rel-17 PDCCH skipping indication is applied only to single schedulling cell, a single DCI can NOT enable PDCCH skipping on multiple cells. In case the LP-WUS information is applied to all the scheduling cells as discussed in section 2.2, if PDCCH skipping is still applied per scheduling cell, it makes no or complex switching between PDCCH monitoring and WUS monitoring. In addition, it requires the UE to monitor the WUS and PDCCH at the same time which may put stringent requirements on UE implementation. For example, the detected WUS at time T1 indicating to resume PDCCH monitoring on both scheduling cell#1 and cell#2, while at time T2, only cell#2 receives PDCCH skipping indication which activates the WUS monitoring. Then the UE starts monitoring WUS for both cell #1 and cell #2 and it stops PDCCH monitoring on cell#2, but the UE is required to still keep PDCCH monitoring on cell#1.

Proposal 17: if PDCCH skipping indication is used as LP-WUS activation signalling, enhanced PDCCH skipping indication can be used to indicate UE stop PDCCH monitoring on one or mulitiple serving cells and starts LP-WUS monitoring.

LP-WUS co-exists with Rel-17 SSSG switching

On co-existence between the LP-WUS and Search Space Set Group (SSSG) switching, in our view, SSSG switching is performed after successful detection of LP-WUS triggering PDCCH monitoring and SSSG switching is stopped after successful detection of LP-WUS reactivation signalling. In detail, when LP-WUS is detected, the UE starts/resumes PDCCH monitoring on search space sets with all group indexes provided by the searchSpaceGroupIdList-r17. Then during the PDCCH monitoring period, before LP-WUS monitoring is reactivated by a new L1/L2 signalling or PDCCH skipping indication, Rel-17 SSSG switching can be performed following existing way, e.g., either by a DCI including the search space sets with group index or by a Timer configured by searchSpaceSwitchTimer-r17. Once LP-WUS monitoring is reactivated, the UE stops PDCCH monitoring on search space sets with all group indexes provided by the searchSpaceGroupIdList-r17 and also stops the search space switch Timer if running.

Proposal 18: For a UE configured with both LP-WUS monitoring and Rel-17 SSSG switching, Rel-17 SSSG switching is applied during the time period when MR performs PDCCH monitoring. There should be no impact due to LP-WUS monitoring.

LP-WUS co-exists with cell DTX

On co-existence between the LP-WUS and Rel-18 cell DTX operation, following aspects should be discussed.

- Aspect#1: LP-WUS monitoring on a cell configured with cell DTX

- Aspect#2: LP-WUS information applicability to a scheduling cell configured with cell DTX

For Aspect#1, we think it makes sense that UE monitor WUS only within cell DTX active time for network energy saving.

Proposal 19: A UE does not expect to monitor the LP-WUS during the non-active periods of cell DTX of a serving cell, if cell DTX is activated for the serving cell with the LP-WUS monitoring.

For Aspect#2, following options can be considered.

- Option 1: WUS triggering PDCCH monitoring applies only to the active periods of cell DTX of a serving cell.

- Option 2: WUS triggering PDCCH monitoring irrespective of the active periods or non-active periods of cell DTX of a serving cell

Option 1 means during the active periods of a cell DTX, UE does not need to monitor the PDCCH unless LP-WUS for the cell is detected. During the non-active period of a cell DTX, since the network is expected not to transmit the PDCCH for dynamic scheduling of the new transmissions, it is reasonable that the detected WUS information for this cell if any, does not apply to the non-active periods of cell DTX.

Option 2 means once LP-WUS for a cell is detected, UE starts PDCCH monitoring on the cell even if during the cell DTX non-active period.

Comparing the two options, option 1 has more power saving benefits and less configuration restrictions at the network side. Therefore, following is proposed:

Proposal 20: As in legacy, UE monitors PDCCH on a cell during the cell-DTX active period in case LP-WUS for the cell is detected.

- WUS monitoring occasions

In RAN1#117 and #118 meetings, following agreements were achieved:

Agreement@RAN1#117 meeting LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for MOs - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured Agreement@RAN1#117 meeting For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement@RAN1#118 meeting For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s)

It was agreed to at least support that a UE can monitor MOs with a periodicity. For the periodicity of LP-WUS MO , the network can configure it properly accoding to traffic latency requirement. For example, the periodicity of LP-WUS MO can be much smaller than C-DRX cycle length e.g., 3ms or 5ms for the sake of reducing the wake up latency and improving the UPT gain. Besides, multiple LP-WUS MOs within a time window with a periodicity can also be configured for following:

- To ensure LP-WUS has at least one chance to be transmitted considering potential collison between LP-WUS MOs and UL symbols or other high priority NR signals/channels

- To enable LP-WUS transmission with beam-sweeping

As long as the above can be achieved, whether to have an explicit or implicit time window for LP-WUS MOs is not the issue.

Proposal 21: LP-WUS monitoring occasions (MOs) are defined by a network-configured Periodicity and Offset. The configuration should allow multiple LP-WUS MOs within each periodicity.

- For LP-WUS operation option 1-1, the periodicity equals to the C-DRX periodicity

- For LP-WUS operation option 1-2, the periodicity is separately configured from C-DRX periodicity

NR defines following four QCL Types:

| QCL Type | Parameters/Description |
| --- | --- |
| QCL Type A | Doppler Shift; Doppler Spread; Average Delay, Delay Spread |
| QCL Type B | Doppler Shift; Doppler Spread |
| QCL Type C | Doppler Shift; Average Delay |
| QCL Type D | Spatial Receiver Parameters |

It was agreed that SSB and/or CSI-RS can be the QCL source of LP-WUS. It is clear that for FR2, QCL type D of spatial receiver parameters is applicable between the SSB and/or CSI-RS and LP-WUS. It is not clear whether/how QCL Type A, B and C is/are used for LP-WUS processing in time-domain.

Proposal 22: For RRC CONNECTED mode, for LP-WUS monitoring in FR2, QCL Type D is applicable for LP-WUS QCLed with SSB/CSI-RS.

- FFS QCL Type A, B and C for FR1 and FR2

Regarding to the QCL/TCI configuration/update for LP-WUS, it can be achieved by Rel-15 TCI framework or Rel-17/18 unified TCI framework that the TCI consists of a source reference signal (RS) and an intended quasi co-location (QCL) type to be applied. A subset of TCI states from the TCI state table from “tci-StatesToAddModList” or “dl-OrJointTCI-StateList” defined in PDSCH-Config can be selected and configured for a LP-WUS resource/MO. In addition to RRC signalling assigning a TCI state Id to a LP-WUS resource, MAC CE or DCI can also be used to update the TCI states from the subset of TCI states for the LP-WUS resource.

Proposal 23: Re-use existing TCI state with or without unified framework to support the TCI state configuration/indication for a LP-WUS resource.

- LP-WUS Payload and Content

In RAN1#116bis and 118 meeting, following agreements were achieved [2] – [4]:

Agreement For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits - FFS value X, which is no more than [8 or 16] Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA

Unlike RRC idle/inactive state, gNB has more flexibility to provide proper UE-specific LP-WUS configuration, to accommodate UE-specific traffic and data arrival time as well as reasonable overhead for LP-WUS. The LP-WUS can carry bitmap or codepiont indication to wake-up one or multiple UEs. In case of UEs with totally different traffic characteristics, using UE-specific LP-WUS, e.g., with a codepoint value corresponding to one or all UEs, can achieve minimum latency and desirable power saving gain. However, with increase of number UEs, such UE-specific LP-WUS would consume large network overhead. As evaluated in [5], for LP-WUS operation option 1 (i.e.,WUS may replace DCP), bitmap scheme can achieve much lower overhead than codepoint scheme, e.g., up to 50% overhead reduction, when the probability of UE traffic per C-DRX cycle is in the range of 30%~ 80%, which is typical for a reasonable C-DRX configuration.

As evaluated in [5], for LP-WUS operation option 1-1, bitmap scheme can achieve much lower overhead than codepoint scheme, e.g., up to 50% overhead reduction, when the probability of UE traffic per C-DRX cycle is in the range of 30%~ 80%, which is typical for a reasonable C-DRX configuration. Codepoint scheme requires a large number of MOs to be monitored by a UE to achieve sufficiently low false-wake up rate caused by the codepoint waking up multiple UEs, which increases UE complexity, and it would be challenge to find proper time domain locations to transmit all these LP-WUSs, especially for TDD system.

For LP-WUS operation option 1-2, codepoint scheme can be more efficient than bitmap scheme, if LP-WUS is transmitted immediately after traffic arrival. However, such timely transmission would result in almost continuous LP-WUS monitoring, which increases UE complexity and results in large false alarm rate. It is more practical to configure duty-cycle based LP-WUS monitoring with a reasonable periodicity. Based on this assumption, overhead for bitmap is comparable to codepoint as analyzed in [5].

Therefore, we proposed to support at least bitmap for RRC connected mode.

Proposal 24: Support at least bitmap for RRC connected mode and the maximum number of LP-WUS information bits is up to 16 bits.

- Conclusion

In this contribution, we provide our views on LP-WUS operation for UEs in RRC CONNECTED modes. The proposals are summarized as follows.

Proposal 1: Supporting following for Option 1-2.

- When LP-WUS is detected, LP-WUS triggers the start of a new timer during which UE monitors PDCCH

- The UE starts LP-WUS monitoring and stops PDCCH monitoring based on one of the following

  - No PDCCH is detected during the new timer running and the new timer expires

  - Expiration of drx-InactivityTimer

  - Explicit indication received to deactivate PDCCH monitoring and activate LP-WUS monitoring, e.g. by PDCCH skipping command.

Proposal 2: Clarify that for both LP-WUS operation option 1-1 and option 1-2 of LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operaion is same as Rel-16 DCP, that is the UE can be configured with one of the following:

- Configuration 1:

  - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is not reported.

  - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time.

- Configuration 2:

  - If the UE is not indicated to wake-up, the periodic CSI/L1-RSRP is reported during the time duration indicated by drx-onDurationTimer in DRX-Config, even if the drx-onDurationTimer does not start.

  - If the UE is indicated to wake-up, the periodic CSI/L1-RSRP is reported during the DRX active time.

Proposal 3: For LP-WUS operation option 1-2, there is no restriction for LP-WUS monitoring configuration (periodicity and offset) in relation with C-DRX configuration.

Proposal 4: For both LP-WUS operation option 1-1 and option 1-2, LP-WUS does not support SCell dormancy indication.

Proposal 5: For both LP-WUS operation option 1-1 and option 1-2, a UE monitors LP-WUS only on one serving cell per MAC entity when UE is configured with CA in RRC CONNECTED mode.

- The serving cell where to monitor LP-WUS is configured by the network.

- This applies to UEs configured with single and dual DRX groups

Proposal 6: For both LP-WUS operation option 1-1 and option 1-2, when UE is configured with CA in RRC CONNECTED mode, the LP-WUS information is applied to all activated serving/scheduling cells.

- FFS for the case of dual DRX groups, LP-WUS received in the serving cell of primary DRX group indicates the triggering of PDCCH monitoring in primary DRX group only, or both DRX groups

Proposal 7: For a UE configured with DC and for both LP-WUS operation option 1-1 and option 1-2, it can support LP-WUS monitoring on a serving cell per cell-group depending on UE capability.

Proposal 8: For both LP-WUS operation option 1-1 and option 1-2 in or RRC CONNECTED mode, the reported value(s) of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring includes LP-WUS processing time and MR transition time for ramp up and does not include the duration for time/frequency synchronization of MR.

Proposal 9: Clarify that the SCS for the reported value(s) for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is the respective SCS of the carrier where LP-WUS is monitored.

Proposal 10: Support Alt.2 that for RRC CONNECTED mode for both LP-WUS operation option 1-1 and option 1-2, UE reports multiple values for each SCS from two candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

- Different minimum time gaps correspond to different sleep states

- The reported value(s) includes LP-WUS processing time and MR transition time for ramp up and does not include the duration for time/frequency synchronization of MR.

- Network can configure a proper time gap according to UE reporting, and considering the trade-off between traffic latency requirement and the UE power saving benefit.

Proposal 11: In RRC_CONNECTED mode, if the LP-WUS is not detected on a WUS monitoring occasion (MO) by the UE,

- For LP-WUS operation option 1-1, the UE is not required to trigger the starting of the drx-onDurationTimer.

- For LP-WUS operation option 1-2, the UE is not required to wake up the MR for PDCCH monitoring for option 1-2.

Proposal 12: In RRC_CONNECTED mode, for both LP-WUS operation option 1-1 and option 1-2, the indication from UE to indicate whether it monitors LP-WUS or PDCCH is not needed.

Proposal 13: After LP-WUS monitoring is enabled by RRC signalling, the specified conditions used for LP-WUS deactivation due to poor channel condition for LP-WUS monitoring is not needed for both LP-WUS operation option 1-1 and option 1-2.

Proposal 14: UE is allowed to fallback to MR PDCCH monitoring by implementation (e.g. due to poor channel condition) without specified condition.

- For LP-WUS operation option 1-1, the fallback PDCCH monitoring behavior is according to the C-DRX configuration

- For LP-WUS operation option 1-2, the fallback PDCCH monitoring behavior is always-on PDCCH monitoring.

Proposal 15: For LP-WUS operation option 1-1 and option 1-2 in RRC_CONNECTED mode, UL transmissions such as SR, PRACH and CG PUSCH can trigger MR PDCCH monitoring according to the legacy UE behavior.

Proposal 16: For LP-WUS operation option 1-1 and option 1-2 in RRC_CONNECTED mode, UE switching from PDCCH monitoring to LP-WUS monitoring is supported by gNB L1/L2 signaling.

Proposal 17: if PDCCH skipping indication is used as LP-WUS activation signalling, enhanced PDCCH skipping indication can be used to indicate UE stop PDCCH monitoring on one or mulitiple serving cells and starts LP-WUS monitoring.

Proposal 18: For a UE configured with both LP-WUS monitoring and Rel-17 SSSG switching, Rel-17 SSSG switching is applied during the time period when MR performs PDCCH monitoring. There should be no impact due to LP-WUS monitoring.

Proposal 19: A UE does not expect to monitor the LP-WUS during the non-active periods of cell DTX of a serving cell, if cell DTX is activated for the serving cell with the LP-WUS monitoring.

Proposal 20: As in legacy, UE monitors PDCCH on a cell during the cell-DTX active period in case LP-WUS for the cell is detected.

Proposal 21: LP-WUS monitoring occasions (MOs) are defined by a network-configured Periodicity and Offset. The configuration should allow multiple LP-WUS MOs within each periodicity.

- For LP-WUS operation option 1-1, the periodicity equals to the C-DRX periodicity

- For LP-WUS operation option 1-2, the periodicity is separately configured from C-DRX periodicity

Proposal 22: For RRC CONNECTED mode, for LP-WUS monitoring in FR2, QCL Type D is applicable for LP-WUS QCLed with SSB/CSI-RS.

- FFS QCL Type A, B and C for FR1 and FR2

Proposal 23: Re-use existing TCI state with or without unified framework to support the TCI state configuration/indication for a LP-WUS resource.

Proposal 24: Support at least bitmap for RRC connected mode and the maximum number of LP-WUS information bits is up to 16 bits.

# References

- 3GPP RAN1#116 meeting, Chairman notes

- 3GPP RAN1#116bis meeting, Chairman notes

- 3GPP RAN1#117 meeting, Chairman notes

- 3GPP RAN1#118 meeting, Chairman notes

- R1-2407869, Discussion on LP-WUS and LP-SS design, vivo