3GPP TSG RAN WG1 #118bis	                                           R1-2407877

Hefei, China, 14th – 18th October, 2024

Source:	vivo

Title:	Discussion on enabling data transmissions for XR during RRM measurements

Agenda Item:	9.10.1

Document for:	Discussion and Decision

- Introduction

In RAN1#118 meeting, great process was achieved regarding enabling transmission/reception in gaps/restrictions that are caused by RRM measurements in RAN1 and the following agreements and working assumption were made [1].

Agreement For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select one among the following options: Option 1: Support Alt. 1-1: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. Option 2: Support Alt. 3-1: - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

In this contribution, we will discuss some remaining issues about the enhancements to enable transmissions/receptions in gaps/restrictions.

- Discussion

  - Solutions to enable transmissions/receptions in gaps/restrictions

  - For different alternatives to enable transmissions/receptions in gaps/restrictions, it was agreed as working assumption in the last meeting that Alt.1 is selected with an LS to RAN4 to inform them of the working assumption and ask them if there is any issue with it.

  - For Alt. 1, the indication is included as part of a scheduling DCI and the bit-field size is one bit. One remaining issue is which DCI format(s) are supported with such one-bit indication. During the discussion in last meeting, it was proposed that both DL scheduling non-fallback DCI formats, i.e., DCI formats 1_1 and 1_2, and UL scheduling non-fallback DCI formats 0_1 and 0_2 are supported. However, the necessity of supporting UL scheduling DCI formats needs further discussion. Note that the mainly use case for this feature of enabling transmissions/receptions in gaps/restrictions is when the gNB is aware of emergency demand for traffic delivery, e.g. when DL XR packets have arrived at the gNB, and these packets could not be delivered within PDB, so the gNB can indicate the UE to skip a particular MG/scheduling restriction. For XR traffic of UL pose/control, its packet size is usually small, and the gNB can finish the scheduling within a very short duration which may be shorter than the timeline for Alt. 1. For AR UL stream(s), the baseline PDB is 30ms which is much longer than that of DL (3 times), the gNB can transmit corresponding packets outside the gaps/restrictions and there is no need to cancel a gap/restriction occasion to transmit them urgently. So, this feature of enabling transmissions/receptions in gaps/restrictions is mainly useful for XR DL traffic and supporting DL scheduling DCI with one-bit indication is sufficient.

  - Once more than one DCI format is supported with such one-bit indication, presence of this DCI indication field should be configured per DCI format. For example, new RRC parameter kipGapRestrictionDCI-1-1 and kipGapRestrictionDCI-1-2 can be added in PDSCH-Config for DCI format 1_1 and 1_2, respectively. When the parameter is set to ‘enabled’, it means there is one-bit indication for enabling transmissions/receptions in gaps/restrictions in the corresponding DCI format.

Proposal 1: To enable Tx/Rx in gaps/restrictions that are caused by RRM measurements with Alt. 1, DCI formats 1_1 and 1_2 are supported.

- The presence of the one-bit indication in different DCI formats is configured per DCI format.

- FFS DCI formats 0_1 and 0_2.

  - Another issue is whether the scheduling DCI has to schedule a PDSCH reception or not. Considering that the main use case for this feature of enabling transmissions/receptions in gaps/restrictions is when the gNB is aware of emergency demand for traffic delivery, it means that typically there is traffic to be transmitted when the gNB sends a DCI to indicate the UE to skip a particular gap/restriction occasion. Currently, for several cases, e.g. SCell dormancy indication and Type-3 HARQ-ACK codebook triggering, a DL scheduling DCI with invalid FDRA is supported and HARQ-ACK for the DCI without scheduling PDSCH reception are specified. Considering the marginal specification impact and more flexibility, it is slightly preferred that the DL scheduling DCI can indicate invalid FDRA.

Proposal 2: For a DCI indicating to skip a particular gap/restriction occasion, the DCI can indicate invalid FDRA without scheduling a PDSCH reception.

  - Currently, for a UE, it can be configured with multiple concurrent measurement gap configurations. When the UE is configured with concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than [4]ms.

In case of collision between two occasions based on two measurement gap configurations with different priorities, the UE shall perform measurements in the occasion corresponding to the measurement gap configuration with higher priority, and the occasion corresponding to the measurement gap configuration with lower priority is considered to be dropped.

In a case with multiple concurrent measurement gap configurations, the UE can handle collision among multiple measurement gap occasions as legacy firstly, and then determine the occasion indicated by a DCI as shown in Figure 1. In other words, the bit in the DCI is used to indicate a survived measurement gap occasion after collision handling.

Figure 1 Example for DCI indication in a case with concurrent measurement gap configurations

Proposal 3: When a UE is configured with multiple concurrent measurement gap configurations, if there are collided measurement gap occasions, the UE first solves collision among different measurement gap occasions as legacy, and DCI indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements is applied to a survived measurement gap occasion after collision handling.

In addition, in RAN1#116 meeting, it was agreed as WA that RAN1 aims to develop/identify solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements agnostic in RAN1 normative work to types of gaps/restrictions that are caused by RRM measurements. RAN4 is discussing which types of gaps/restrictions can be skipped and whether UAI is supported to indicate which gap/restriction occasions can be skipped. It can be understood that if RAN4 decides that only some particular type(s) of gaps/restrictions can support this feature of enabling transmissions/receptions in gaps/restrictions, e.g. only measurement gaps used for L3 measurement, the DCI indication should only apply to gaps/restrictions of these particular type(s). For a particular gap/restriction configuration, if only a subset of occasions for the configuration can be skipped, e.g. a pattern of protected/non-skippable gap/restriction occasions is configurated by the gNB, it can be understood that the DCI should only apply to the measurement gap/restriction occasions that can be skipped. Our understanding on the operation of DCI indicating to skip a particular gap/restriction occasion is illustrated in Figure 2.

Figure 2 Example for the operation of DCI indicating to skip a gap/restriction occasion

Proposal 4: For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements, the bit in a DCI is used to indicate whether to skip the first skippable gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

- FFS: How to determine whether an occasion is skippable or not

  - Interaction with DRX

When a UE is configured with DRX for power saving, the UE does not need to wake up to monitor PDCCH(s) outside DRX active time. For Alt 1-1 as described above, some proponents propose to use a DCI to indicate whether one or more gaps, e.g. a next measurement gap after the DCI is skipped or not. One issue is if a measurement gap is outside DRX active time, whether it can be counted as a next measurement gap indicated by the DCI. For example, as shown in following Figure 1, MG1 is fully in DRX off duration, and MG2 is outside DRX off time. At the same time, Alt 1-1 is adopted by using DCI1 to indicate a next measurement gap after DCI1. One interpretation is DCI1 is used to indicate whether MG1 is skipped or not. Another interpretation is DCI1 is used to indicate whether MG2 is skipped or not. Note that outside DRX active time, UE still needs to perform transmission/reception based on configuration/scheduling following current DRX mechanism, but the scheduling DCI should be transmitted before DRX off.

In our view, the mechanism of enabling transmission/reception in gaps/restrictions that are caused by RRM measurement and the existing mechanism of DRX can be independent, that is, when determining the next measurement gap indicated by DCI1, there is no need to take consideration on DRX configuration, therefore, DCI1 is used to indicate the skipping status of MG1. gNB can send another indication to skip MG2 after MG1 if needed.

Figure 3 Example for interaction with DRX

Another issue is outside DRX active time, if the gNB indicates transmission/reception within a measurement gap, the UE should follow the behavior of the indication to perform transmission/reception within the measurement gap including PDCCH monitoring, or the UE should follow the behavior of DRX and does not need to perform PDCCH monitoring within the measurement gap. In our view, such case is not a typical case, and there is no motivation to change the existing DRX mechanism. That is, the UE should follow the behavior of DRX and does not need to perform PDCCH monitoring within the measurement gap.

Proposal 5: Enhancement on enabling Tx/Rx in gaps/restrictions that are caused by RRM measurements outside DRX active time is not considered in R19.

- Conclusion

In this contribution, some remaining issues about enhancements to enable transmissions/receptions in gaps/restrictions are discussed, with the following proposals:

Proposal 1: To enable Tx/Rx in gaps/restrictions that are caused by RRM measurements with Alt. 1, DCI formats 1_1 and 1_2 are supported.

- The presence of the one-bit indication in different DCI formats is configured per DCI format.

- FFS DCI formats 0_1 and 0_2.

Proposal 2: For a DCI indicating to skip a particular gap/restriction occasion, the DCI can indicate invalid FDRA without scheduling a PDSCH reception.

Proposal 3: When a UE is configured with multiple concurrent measurement gap configurations, if there are collided measurement gap occasions, the UE first solves collision among different measurement gap occasions as legacy, and DCI indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements is applied to a survived measurement gap occasion after collision handling.

Proposal 4: For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements, the bit in a DCI is used to indicate whether to skip the first skippable gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

- FFS: how to determine whether an occasion is skippable or not

Proposal 5: Enhancement on enabling Tx/Rx in gaps/restrictions that are caused by RRM measurements outside DRX active time is not considered in R19.

# References

- Chair’s notes RAN1#118 meeting, Maastricht, Netherland, August 19 – 23, 2024

- LS on FS_XRM Ph2, S2-2407351, SA WG2 Meeting #163, Jeju, Korea, May 27 – May 31, 2024