- 3GPP TSG RAN WG1 #118bis                                                                                  R1-2407933

Hefei, China, October 14th-18th, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on DL coverage enhancement for NR NTN

Agenda Item:	9.11.1

Document for:    Discussion

- Introduction

              - Based on the results of DL coverage evaluation at system level in RAN1#118 [1], it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints. Accordingly, following agreements have been achieved for further consideration.

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

In this contribution, further analysis on the system level and link level enhancements are provided.

- System level analysis on the DL coverage issues

  - Analysis on extended SSB periodicity

    - Values for extended SSB periodicity

It is observed in RAN1#118 [1] that the common messages overhead for Set 1-1/3 can be reduced from 40% to less than 14%, if the SSB periodicity is increased from 20ms to 160ms. And the common messages overhead for Set 1-2 can be reduced from more than 100% to 25.8%, if the SSB periodicity is increased from 20ms to 640ms.

Observation Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints: - With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used. - With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used. - Note: the overhead of SIB19 was included in some of the results - Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation

Besides, to support initial access in an FDD NTN, DL/UL beams are needed to simultaneously illuminate the same beam footprint. Therefore, further SSB periodicity extension is needed due to the longer dwell time with consideration on RACH procedure. A minimum dwell time of 31.5ms is needed to support a complete initial access procedure. In this case, only 0.5 footprint can be served by a beam in 20ms (or 1 footprint served in 40ms). For Set 1-1/3, 53 (=0.5*106) beam footprints can be served by 106 simultaneously active beams in 20ms. Consequently, a minimum SSB periodicity of 640 ms is also needed to support a complete RACH procedure.

Therefore, at least 640ms should be supported as the default value for extended SSB periodicity assumed by NTN UE for initial access.

Additionally, by assuming that the same behavior (e.g., hopping) is expected for  both DL and UL, for all assocaited RO to one specific SSB, it’s obvious that only part of the Ros are valid for transmission (e.g., within the service/dwell time of UL beam).

Proposal 1: At least 640ms should be supported as the default value for extended SSB periodicity assumed by NTN UE for initial access.

Proposal 2: The assumption on the UL beam (e.g., simultaneously actived) should be confirmed.

Additionally, considering the flexibility of the association between SSB and satellite beams for the implementation, esepcailly, to support the scenario, e.g., different traffic loads among multiple beams, which are mapped to different geological region,  the proper periodicity for each SSB may be different. Moreover, in some cases, e.g., dedicated beams are allocated for connected UEs, the periodicities of SSBs transmissted over these beam will be different compared to the SSBs (e.g., same or different SSB index) for initial access stage.  To address the above needs, enhancement to support the different periodicty per SSB seems necessary.

Proposal 3: Different periodicity per SSB can supported with the consideration on uneven traffic load among different beams and over different time.

    - Cell search procedure

For initial cell search, a UE may assume that half frames with SS/PBCH blocks occur with one or more predefined periodicities, e.g., {20ms, 640ms}. After successful reception of SIB1, the UE can obtain the SSB periodicity indicated by the gNB.

According to TS 38.105, a global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The parameters defining the SSREF and GSCN for all the frequency ranges are quoted in Table 1. For a 5MHz system bandwidth, the SSB may located around only 4 (=5MHz/1.2MHz) frequency position.

Table 1 GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5}1 | 3N + (M-3)/2 | 2 – 7498 |
| NOTE: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Besides, there are following ways to further improve the cell search efficiency:

- The network parameters stored in the USIM can be used to reduce cell search complexity by providing the exact channel frequency with the resolution of 200 kHz independent of any band information [2].

- It is common to use multiple branches for parallel SSB detection to reduce the processing delay at UE side.

- It is common to used a wide band energy detection to roughly locate the SSB  at UE side.

In addition, after a UE accesses the network, it generally retains key network information to facilitate quick access in the future. For example, the UE saves details about the last cell it connected to, such as cell ID and frequency information. In current NTN, the cell definition is based on geographic location like that in TN deployment. Therefore, the cell ID and the frequency used will be the same no matter which satellite serves the cell. This key network information is typically stored in the UE's non-volatile memory, such as the SIM card or built-in device storage, so that it remains available after device restarts or battery replacements. Therefore, when a new satellite comes into coverage, a UE can use the retained information to speed up the SSB detection.

Though the extended SSB periodicity of 640ms increases the cell search latency, considering one-shot PSS detection (at least workable for LEO-600 Set 1-1 and Set 1-2), an approximate latency of 0.7s is acceptable for accessing NTN with inherent long propagation delay.

Observation 1: Cell search complexity and latency can be reduced by using stored frequency information in USIM, advanced receiver processing and retained key network information at the UE side.

    - Backward compatibility

Regarding the backward compatibility issue, the main concern from companies is that the legacy UE (e.g., pre-Rel-19) may not be able to access the network (i.e., initial access) since the SSB periodicity is extended. However, on this aspect, the following points should be clarified:

- P-1: As clarified in the previous discussion, the intention to extend the SSB periodicity is to match the selected deployment of brand-new service by interested operator. Such kind of extension is the direct consequence of the deployment. In this case, typically, there is no “legacy subscriber”  to access the new satellite network and the backward compatibility issue does not exist. Furthermore, similar as another WI (e.g., NES), it can be even further clarified that this feature only allows Rel-19 device to access.

- P-2: For the pre-Rel-19, as also argued by other companies that the behaviour for SSB searching, i.e., assumption on the periodicity, can be further upgraded via OTA updates, e.g., from 20 ms to other value. In this case, the corresponding device can also be able to access the network without any issues.

Proposal 4: Backward compatibility should not be considered for extended SSB periodicity in NTN scenario.

  - Analysis on wide/narrow beam

    - DL CNR analysis with wide beam

In RAN1#118 [1], wider beam of SSB has also been discussed as an alternative to improve coverage ratio. However, wide beam leads to insufficient CNR problems as analyzed in this section. Without loss of generality, a wide beam with coverage of 4 original beams is used as an example. There are following two possible ways to form the wide beam.

- The first way is based on decreased transmission power of each original beams, which can be approximate to a wide beam with larger HPBW.

For a phased array antenna, each original beam is formed using a phase shift weight vector for antenna elements. If a wide beam with coverage of 4 original beams is formed, these 4 original beams transmits the same SSB simultaneously. As a result, the transmission power on each original beam will be decreased by 6dB and impact on the DL link level performance is expected. The comparison of DL CNR with/without wide beam usage for LEO600km with 30-degree elevation angle is provided in Table 2.

Table 2 DL CNR for LEO600km with 30-degree elevation angle

| Wide beam usage | Satellite parameter set | Frequency [GHz] | UE antenna gain [dBi] | TX: EIRP density [dBW/MHz] | RX: G/T [dB/T] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | 1-1 1-2 | 2 | -5.5 | 34 | -37.1 | 159.1 | 0.1 | 3.0 | 2.2 | 3.0 | 0 | -1.9 |
|  | 1-3 | 2 | -5.5 | 26 | -37.1 | 159.1 | 0.1 | 3.0 | 2.2 | 3.0 | 0 | -9.9 |
| Yes 4 footprints | 1-1 1-2 | 2 | -5.5 | 28 | -37.1 | 159.1 | 0.1 | 3.0 | 2.2 | 3.0 | 0 | -7.9 |
|  | 1-3 | 2 | -5.5 | 20 | -37.1 | 159.1 | 0.1 | 3.0 | 2.2 | 3.0 | 0 | -15.9 |

Compared with the required SNR provided in Section 3, all necessary DL common channels cannot be correctly received with the reduced CNR values. Accordingly, all necessary DL channel should be enhanced to meet the link budget. Then, the minimum dwell time for the served region for each beam will be further increased and the needs to extend the periodicity of the common channel still holds.

Observation 2:  If a wide beam covering 4 footprints is formed based on decreased transmission power of each original beams, the CNR will be decreased by about 6dB. At a result, necessary DL common channels cannot be correctly received for Set 1-1/2/3.

- The second way is based on increased coverage of one original beam, i.e., directly increase footprint diameter (or adjacent beam spacing) by keeping the same HPBW.

This way may be difficult due to the antenna gain loss with larger azimuth angle required by the extended footprint diameter. The antenna gain of the phased array antenna for S-band is provided in Figure 1. It is observed that the antenna gain decreases rapidly with the growth of azimuth angle.

Figure 1 Antenna gain pattern of the phased array antenna (S-band)

To cover original beams’ footprint (with beam radius of 25km), the beam radius is changed to 50km. The corresponding azimuth angle is ±4.76 degree instead of ±2.39 degree. For the nadir beam, the antenna gain loss of the wide beam edge is about 20dB. In Table 3, DL CNR with/without increased beam coverage for LEO600km nadir beam is provided. It is observed that the CNR at the wide beam edge is too low to support the DL common channels transmission.

Table 3 DL CNR comparison for LEO600km nadir beam without/with wide beam usage-2

| Wide beam usage | Satellite parameter set | Azimuth angle （degree） | Frequency [GHz] | UE antenna gain [dBi] | TX: EIRP density [dBW/MHz] | RX: G/T [dB/T] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | 1-1/2 | 0 | 2 | -5.5 | 34 | -37.1 | 154.03 | 0.1 | 3 | 2.2 | 3 | 0 | 3.17 |
|  |  | +/-2.39 | 2 | -5.5 | 28.96 | -37.1 | 154.04 | 0.1 | 3 | 2.2 | 3 | 0 | -1.88 |
|  | 1-3 | 0 | 2 | -5.5 | 26 | -37.1 | 154.03 | 0.1 | 3 | 2.2 | 3 | 0 | -4.83 |
|  |  | +/-2.39 | 2 | -5.5 | 20.96 | -37.1 | 154.04 | 0.1 | 3 | 2.2 | 3 | 0 | -9.88 |
| Yes 4 footprints | 1-1/2 | 0 | 2 | -5.5 | 34 | -37.1 | 154.03 | 0.1 | 3 | 2.2 | 3 | 0 | 3.17 |
|  |  | +/-4.76 | 2 | -5.5 | 14.14 | -37.1 | 154.06 | 0.1 | 3 | 2.2 | 3 | 0 | -16.72 |
|  | 1-3 | 0 | 2 | -5.5 | 26 | -37.1 | 154.03 | 0.1 | 3 | 2.2 | 3 | 0 | -4.83 |
|  |  | +/-4.76 | 2 | -5.5 | 6.14 | -37.1 | 154.06 | 0.1 | 3 | 2.2 | 3 | 0 | -24.72 |

Observation 3: If a wide beam covering 4 footprints is formed based on increased coverage of one original beam, the CNR will be decreased by about 15dB at the edge of the nadir beam. At a result, necessary DL common channels cannot be correctly received for Set 1-1/2/3 at the wide beam edge.

To deal with the insufficient CNR values caused by the wide beam usage, repetition of DL common channels is needed. Consequently, extended SSB periodicity is still necessary since the DL common channels transmission per wide beam needs longer time.

Proposal 5: If wide beam is used, extended SSB periodicity is still necessary since the longer transmission/reception duration is required for DL/UL common channels transmission per wide beam to compensate the link budget.

    - Impact of asymmetrical DL/UL beams

In RAN1#118 [1], it is proposed to study potential enhancements for transmitting the DL common channels using a wider beam footprint, while transmitting the DL/UL dedicated channels (incl. PRACH) using a narrower beam footprint.

In Figure 2, possible DL/UL beam layouts are illustrated, in which a wide DL beam with coverage of 4 narrow UL beams is used as an example.

Figure 2 Possible DL/UL beam layouts

- In Figure 2 (a), symmetrical DL/UL beam is used. In this case, the simultaneous service are provided for both DL and UL and  all UEs in the DL beam coverage can use the associated RO for SSB0 for their initial access within the dwell time for the both DL and UL.

- In Figure 2 (b), although the same number of UL/DL beams is assumed, the narrow beam is assumed for UL. Since the coverage of UL beam is only part of the DL beam, only UEs in one of the UL beams can use the associated RO for SSB0 for their initial access. To serve all UEs in the DL beam coverage, UL beam sweeping can be used. Consequently, 4*SSB periodicity is needed to support all UEs’ initial access within the DL beam coverage, which means that the dwell time of the DL wider beam is further extended and merits of the introduction wider beam is not valid.

- In Figure 2 (c), more simultaneous UL beams than DL beams is assumed. From the viewpoint of a UE, it can perform initial access using the RO associated with the SSB0 as usual. The BS needs to distinguish the UL beam that is used by the UE for its RACH. However, it is a not pratical to support much more simultaneous UL beams than the DL beams in satellite. In addition, if the BS can not correctly distinguish the UL beam used by the UE, the performance of following RACH procedure will be largely impacted.

Observation 4: If asymmetrical DL/UL beams and the same number of UL/DL beams is assumed, the larger dwell time of DL is still required to enable the transmission of  all UEs due to the UL beam hopping.

Observation 5: If asymmetrical DL/UL beams more simultaneous UL beams than DL beams is assumed, undesirable complexity is required at satellite side with RACH performance degradation if incorrect UL beam is identified.

Proposal 6: The asymmetrical DL/UL beam layout should not be supported, since there is no performance merits with the assumption of the same number of UL/DL beams, and it is not practical with the assumption of more simultaneous UL beams than DL beams.

- Link level analysis on the DL coverage issues

In RAN1#118 [1], RAN1 has agreed to consider link-level enhancements for the following channels:

- PDCCH

- PDSCH with Msg 4

- PDSCH with SIB1/SIB19.

And RAN1 has also noted that this does not imply that all the channels above will be enhanced. In our view, RAN1 should first determine the target SNR for link level enhancement. Then, by comparing the required SNRs for above channels with the target SNR, whether a channel need to be enhanced can be justified.

  - SSB

In previous meetings, the CNR and required SNR for SSB were considered to be target SNR for link level enhancement. It should be noted that SSB channel enhancement other than SSB periodicity extension is not considered as illustrated in the WID. Therefore, the required SNR of SSB detection should be the bottleneck of DL coverage enhancement, since correct SSB detection is the basis for following transmissions. It is not necessary to further enhance a DL channel if its required SNR is lower than SSB. Hence, the target SNR for link level enhancement should not be lower than the required SNR for SSB detection. The satellite parameter with CNR lower than required SNR for SSB detection should be re-justified instead of directly using its CNR as target for link level enhancement.

Proposal 7: The required SNR for SSB detection can be considered as the lower bound of target SNR for link level enhancement.

In RAN1#116bis, the assumptions for SSB evaluations are discussed as shown in Table 4, which is used in following simulations. For PSS detection, the large Doppler and time drift should be considered in evaluation since SSB is detected in initial access without any compensation, which is different from other physical channels.

Observation 6: For PSS detection, large frequency offset and timing drift should be considered since no compensation can be performed in initial DL synchronization.

Table 4 Simulation assumptions for SSB

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 |
| Combination of SSBs | With and without 4 combinations |
| Maximum Frequency offset for PSS | 24 ppm |
| Maximum Time drift for PSS | 48 ppm |
| CFO for PBCH | 0.1 ppm |
| Maximum Doppler frequency drift | 0.27 ppm/s based on TR 38.821 |
| Periodicity | 20 ms |

    - PSS

The simulation results for PSS are as illustrated in Figure 3 with assumption of FA=1%. And the margin of SSB detection performance and link budget are listed in Table 5, where CNR for set 1-1/1-2 is -1.9dB and CNR for set 1-3 is -9.9dB. Note that multi-branch detection is applied here to handle the large frequency offset. It can be observed that the required SNR for both 1-SSB detection and 4-SSB detection are lower than CNR for Set 1-1 and Set 1-2 but higher than that for Set 1-3, no matter which periodicity is used.  Hence, for Set 1-1 and Set 1-2, single SSB detection is enough. While for Set 1-3, even 4-SSB combination cannot satisfy the required performance at the CNR.

Figure 3 Performance of PSS detection

    - PBCH

The simulation results for PBCH are as illustrated in Figure 4.

Figure 4 Performance of PBCH detection

The margin of SSB (for PSS and PBCH separately) detection performance and link budget are listed in Table 5. It can be observed that the required SNR for 1-SSB detection are lower than CNR for Set 1-1 and Set 1-2, but higher than that for Set 1-3. For 4-SSB detection, the required SNR for PBCH detection can be further lower than CNR for Set 1-3. However, it should be noted that the required SNR of PSS, which is higher than CNR for Set 1-3 as marked in red, will be the bottleneck of SSB detection. Therefore, for Set 1-1 and 1-2, single SSB detection is enough. While for Set 1-3, even with combination of 4 SSBs, the required SNR cannot be satisfied.

Table 5 Margin of required SNR for SSB

| SSB detection | Required SNR | Margin w.r.t CNR for LEO-600 Set 1-1 and Set 1-2 | Margin w.r.t CNR for LEO-600 Set 1-3 |
| --- | --- | --- | --- |
| 1-PSS | -3.6 dB | 1.7 dB | -6.3 dB |
| 4-PSS | -5.9 dB | 4.0 dB | -4.0 dB |
| 1-PBCH | -7.2 dB | 5.3 dB | -2.7 dB |
| 4-PBCH | -13.2 dB | 11.3 dB | 3.3 dB |

Observation 7: For PSS detection, the required SNR for single-shot SSB detection is -3.6 dB, which shows an SNR margin of 1.7 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Even with 4-SSB combination, the required SNR is around -5.9 dB, which is still higher than CNR for Set 1-3.

Observation 8: For PBCH detection, the required SNR for single-shot SSB detection is -7.2 dB, which shows an SNR margin of 5.3 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3.

Observation 9: For PBCH detection, with 4-SSB combination, the required SNR is around -13.2 dB, which is lower than CNR for Set 1-3. However, the required SNR for PSS detection, which is -5.9 dB, will be the bottleneck for SSB detection.

Proposal 8: The target SNR for link level enhancement should not be lower than -5.9 dB with consideration of required SNR for PSS detection.

Proposal 9: The required SNR for SSB detection is larger than the CNR for Set 1-3 even with 4-SSB combination, i.e., CNR for LEO-600 Set 1-3 should not be target for link level enhancement.

  - PDCCH

In this section, the required SNR for PDCCH detection is firstly evaluated. By comparing the required SNR with target SNR, the necessity of PDCCH enhancement is justified.

    - Evaluation

The simulation assumptions for PDCCH are as agreed in RAN1#116bis and shown in Table 6.

Table 6 Simulation assumptions for PDCCH

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 |
| Aggregation level | 8 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 24 PRBs |
| Target BLER | 1% BLER |

The simulation results are as illustrated in Figure 5. It can be observed that the required SNR for PDCCH detection is about -6.8 dB. Therefore, the SNR margin of PDCCH is as listed in Table 7. Based on the results, PDCCH enhancement is not necessary.

Figure 5 Performance of PDCCH

Table 7 Margin of required SNR for PDCCH

| Required SNR | Margin w.r.t CNR for LEO-600 Set 1-1 and Set 1-2 | Margin w.r.t required SNR for 1-SSB detection | Margin w.r.t required SNR for 4-SSB detection |
| --- | --- | --- | --- |
| -6.8 dB | 4.9 dB | 3.2 dB | 0.9 dB |

Observation 10: The required SNR for PDCCH is about -6.8 dB with single repetition, which shows an SNR margin of 4.9 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, 3.2 dB with respect to 1-SSB detection, 0.9 dB with respect to 4-SSB detection.

Proposal 10: Link level enhancement on PDCCH is not needed.

  - PDSCH with SIB1/SIB19

In this section, the required SNRs for PDSCH with SIB1 and SIB19 are firstly evaluated. By comparing the required SNR with target SNR, the necessity of enhancement is justified. And the potential solution of enhancement is further discussed after justification.

    - Evaluation

3.4.1.1 PDSCH with SIB1

The simulation assumptions are in Table 8.

Table 8 Simulation assumptions for SIB1

| Parameter | Value |
| --- | --- |
| Target BLER | 10% |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 |
| HARQ configuration | Without HARQ |
| DMRS configuration | 2 DMRS symbols Type I, no multiplexing with data |
| Payload size for PDSCH of SIB1 | Option 1: 800 bits Option 2: 1280 bits |
| PRB | 24 |
| MCS | Option 1: Index 1 of MCS index table 1 for PDSCH (Table 5.1.3.1-1 in TS 38.214), Target code rate 157/1024, QPSK Option 2: Index 3 of MCS index table 1 for PDSCH (Table 5.1.3.1-1 in TS 38.214), Target code rate 251/1024, QPSK |
| TBS | Option 1: 888 Option 2: 1416 |
| PDSCH duration | 12 OS (including DMRS) |

The simulation results are as illustrated in Figure 6. It can be observed that the required SNR for SIB1 is about -5.6 dB for option 1 and -3.4 dB for option 2. The SNR margin of SIB1 is as listed in Table 9. Based on the results, SIB1 may not be needed considering that option 1 has similar performance as 4-SSB detection performance and much lower than CNR for set 1-1 and set 1-2.

Figure 6 Performance of SIB1

Table 9 Margin of required SNR for SIB1

|  | Required SNR | Margin w.r.t CNR for LEO-600 Set 1-1 and Set 1-2 | Margin w.r.t required SNR for 1-SSB detection | Margin w.r.t required SNR for 4-SSB detection |
| --- | --- | --- | --- | --- |
| Option 1, 800 bits | -5.6 dB | 3.7 dB | 2.0 dB | -0.3 dB |
| Option 1, 800 bits, 2 repetitions | -8.1 dB | 6.2 dB | 4.5 dB | 2.2 dB |
| Option 2, 1280 bits | -3.4 dB | 1.5 dB | -0.2 dB | -2.5 dB |
| Option 2, 1280 bits, 2 repetitions | -6.2 dB | 4.3 dB | 2.6 dB | 0.3 dB |

Observation 11: The required SNR for SIB1 is about -5.6 dB for option 1, which shows an SNR margin of 3.7 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, 2.0 dB with respect to 1-SSB detection, -0.3 dB with respect to 4-SSB detection.

Observation 12: The required SNR for SIB1 is about -3.4 dB for option 2, which shows an SNR margin of 1.5 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, -0.2 dB with respect to 1-SSB detection, -2.5 dB with respect to 4-SSB detection.

Proposal 11: Link level enhancement on SIB1 may be needed if required SNR for 4-SSB detection is considered as target SNR, while not needed if the required SNR for 1-SSB detection or CNR for set 1-1/1-2 is considered as target SNR.

3.4.1.2 PDSCH with SIB19

The simulation assumptions are in Table 10.

Table 10 Simulation assumptions for SIB19

| Parameter | Value |
| --- | --- |
| Target BLER | 10% |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 |
| HARQ configuration | Without HARQ |
| DMRS configuration | 2 DMRS symbols Type I, no multiplexing with data |
| Payload size for PDSCH of SIB1 | 616 |
| PRB | 24 |
| MCS | Index 0 of MCS index table 1 for PDSCH (Table 5.1.3.1-1 in TS 38.214) Target code rate 120/1024, QPSK |
| TBS | 672 |
| PDSCH duration | 12 OS (including DMRS) |

The simulation results are as illustrated in Figure 7. It can be observed that the required SNR for SIB19 is about -6.5 dB. The SNR margin of SIB19 is as listed in Table 11. Based on the results, SIB19 enhancement is not necessary.

Figure 7 Performance of SIB19

Table 11 Margin of required SNR for SIB19

|  | Required SNR | Margin w.r.t CNR for LEO-600 Set 1-1 and Set 1-2 | Margin w.r.t required SNR for 1-SSB detection | Margin w.r.t required SNR for 4-SSB detection |
| --- | --- | --- | --- | --- |
| 616 bits | -6.5 dB | 4.6 dB | 2.9 dB | 0.6 dB |

Observation 13: The required SNR for SIB19 is about -6.5 dB, which shows an SNR margin of 4.6 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, 2.9 dB with respect to 1-SSB detection, 0.6 dB with respect to 4-SSB detection.

Proposal 12: Link level enhancement on SIB19 with TBS = 672 is not needed.

    - Potential enhancement

As evaluated above, enhancement on PDSCH with SIB1 may be needed if required SNR of 4-SSB detection is considered as target SNR for link level enhancement. Regarding detailed enhancement solution, SIB1 repetition can be considered.

Proposal 13: For PDSCH with SIB1, repetitions can be considered to mitigate the SNR gap if required SNR for 4-SSB detection is considered as target SNR for link level enhancement.

Regarding the configuration, considering that MIB still has 1 spare bit, whether SIB1 repetition is performed can be indicated in MIB. In case MIB indicates SIB1 repetition is performed, a predefined repetition number, e.g., 2 repetitions, can be performed.

Proposal 14: If SIB1 repetition is supported, whether SIB1 repetition is performed can be indicated in MIB. When SIB1 repetition is indicated, a predefined repetition number can be performed.

  - PDSCH with Msg4

In this section, the required SNR for PDSCH with Msg4 is firstly evaluated. By comparing the required SNR with target SNR, the necessity of enhancement is justified. And the solution of enhancement is further discussed after justification.

    - Evaluation

The simulation assumptions are in Table 12.

Table 12 Simulation assumptions for PDSCH with Msg4

| Parameter | Value |
| --- | --- |
| Target BLER | 10% |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 |
| HARQ configuration | Without HARQ |
| DMRS configuration | 2 DMRS symbols Type I, no multiplexing with data |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| PRB | 25 |
| MCS | Index 2 of MCS index table 1 for PDSCH (Table 5.1.3.1-1 in TS 38.214) Target code rate 193/1024, QPSK |
| TBS | 1128 |
| PDSCH duration | 12 OS (including DMRS) |

The simulation results are as illustrated in Figure 8. It can be observed that the required SNR for PDSCH with Msg4 is about -4.5 dB. The SNR margin of PDSCH with Msg4 is as listed in Table 13. Based on the results, Msg4 PDSCH enhancement may be needed if the required SNR of 4-SSB detection is considered as target.

Figure 8 Performance of PDSCH with Msg4

Table 13 Margin of required SNR for PDSCH with Msg4

|  | Required SNR | Margin w.r.t CNR for LEO-600 Set 1-1 and Set 1-2 | Margin w.r.t required SNR for 1-SSB detection | Margin w.r.t required SNR for 4-SSB detection |
| --- | --- | --- | --- | --- |
| 1 repetition | -4.5 dB | 2.6 dB | 0.9 dB | -1.4 dB |
| 2 repetitions | -7.2 dB | 5.3 dB | 3.6 dB | 1.3 dB |

Observation 14: The required SNR for PDSCH with Msg4 is about -4.5 dB, which shows an SNR margin of 2.6 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, 0.9 dB with respect to 1-SSB detection, -1.4 dB with respect to 4-SSB detection.

Proposal 15: Link level enhancement on PDSCH with Msg4 may be needed if the required SNR for 4-SSB detection is considered as target SNR, while not needed if the required SNR for 1-SSB detection or CNR for set 1-1/1-2 is considered as target SNR.

    - Potential enhancement

As evaluated above, enhancement on PDSCH with Msg4 may be needed if required SNR of 4-SSB detection is considered as target SNR for link level enhancement. Regarding detailed enhancement solution, Msg4 repetition can be considered. With only 2 repetitions, the required SNR Msg4 can be lower than that for 4-SSB detection.

Proposal 16: For PDSCH with Msg4, repetitions can be considered to mitigate the SNR gap if required SNR for 4-SSB detection is considered as target SNR for link level enhancement.

Regarding the configuration, considering that the NTN is dominated by LoS, the channels for UEs within same cell are similar and a broadcast signaling is enough. Hence, the Msg4 repetition can be configured via a SIB.

Proposal 17: If Msg4 repetition is supported, the repetition configuration can be signalled via SIB.

- Conclusions

In this contribution, the system level and link level analysis are provided with following observation and proposals.

Proposal 1: At least 640ms should be supported as the default value for extended SSB periodicity assumed by NTN UE for initial access.

Proposal 2: The assumption on the UL beam (e.g., simultaneously actived) should be confirmed.

Proposal 3: Different periodicity per SSB can supported with the consideration on uneven traffic load among different beams and over different time.

Observation 1: Cell search complexity and latency can be reduced by using stored frequency information in USIM, advanced receiver processing and retained key network information at the UE side.

Proposal 4: Backward compatibility should not be considered for extended SSB periodicity in NTN scenario.

Observation 2:  If a wide beam covering 4 footprints is formed based on decreased transmission power of each original beams, the CNR will be decreased by about 6dB. At a result, necessary DL common channels cannot be correctly received for Set 1-1/2/3.

Observation 3: If a wide beam covering 4 footprints is formed based on increased coverage of one original beam, the CNR will be decreased by about 15dB at the edge of the nadir beam. At a result, necessary DL common channels cannot be correctly received for Set 1-1/2/3 at the wide beam edge.

Proposal 5: If wide beam is used, extended SSB periodicity is still necessary since the longer transmission/reception duration is required for DL/UL common channels transmission per wide beam to compensate the link budget.

Observation 4: If asymmetrical DL/UL beams and the same number of UL/DL beams is assumed, the larger dwell time of DL is still required to enable the transmission of  all UEs due to the UL beam hopping.

Observation 5: If asymmetrical DL/UL beams more simultaneous UL beams than DL beams is assumed, undesirable complexity is required at satellite side with RACH performance degradation if incorrect UL beam is identified.

Proposal 6: The asymmetrical DL/UL beam layout should not be supported, since there is no performance merits with the assumption of the same number of UL/DL beams, and it is not practical with the assumption of more simultaneous UL beams than DL beams.

Proposal 7: The required SNR for SSB detection can be considered as the lower bound of target SNR for link level enhancement.

Observation 6: For PSS detection, large frequency offset and timing drift should be considered since no compensation can be performed in initial DL synchronization.

Observation 7: For PSS detection, the required SNR for single-shot SSB detection is -3.6 dB, which shows an SNR margin of 1.7 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3. Even with 4-SSB combination, the required SNR is around -5.9 dB, which is still higher than CNR for Set 1-3.

Observation 8: For PBCH detection, the required SNR for single-shot SSB detection is -7.2 dB, which shows an SNR margin of 5.3 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, but not lower than CNR for LEO-600 Set 1-3.

Observation 9: For PBCH detection, with 4-SSB combination, the required SNR is around -13.2 dB, which is lower than CNR for Set 1-3. However, the required SNR for PSS detection, which is -5.9 dB, will be the bottleneck for SSB detection.

Proposal 8: The target SNR for link level enhancement should not be lower than -5.9 dB with consideration of required SNR for PSS detection.

Proposal 9: The required SNR for SSB detection is larger than the CNR for Set 1-3 even with 4-SSB combination, i.e., CNR for LEO-600 Set 1-3 should not be target for link level enhancement.

Observation 10: The required SNR for PDCCH is about -6.8 dB with single repetition, which shows an SNR margin of 4.9 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, 3.2 dB with respect to 1-SSB detection, 0.9 dB with respect to 4-SSB detection.

Proposal 10: Link level enhancement on PDCCH is not needed.

Observation 11: The required SNR for SIB1 is about -5.6 dB for option 1, which shows an SNR margin of 3.7 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, 2.0 dB with respect to 1-SSB detection, -0.3 dB with respect to 4-SSB detection.

Observation 12: The required SNR for SIB1 is about -3.4 dB for option 2, which shows an SNR margin of 1.5 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, -0.2 dB with respect to 1-SSB detection, -2.5 dB with respect to 4-SSB detection.

Proposal 11: Link level enhancement on SIB1 may be needed if required SNR for 4-SSB detection is considered as target SNR, while not needed if the required SNR for 1-SSB detection or CNR for set 1-1/1-2 is considered as target SNR.

Observation 13: The required SNR for SIB19 is about -6.5 dB, which shows an SNR margin of 4.6 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, 2.9 dB with respect to 1-SSB detection, 0.6 dB with respect to 4-SSB detection.

Proposal 12: Link level enhancement on SIB19 with TBS = 672 is not needed.

Proposal 13: For PDSCH with SIB1, repetitions can be considered to mitigate the SNR gap if required SNR for 4-SSB detection is considered as target SNR for link level enhancement.

Proposal 14: If SIB1 repetition is supported, whether SIB1 repetition is performed can be indicated in MIB. When SIB1 repetition is indicated, a predefined repetition number can be performed.

Observation 14: The required SNR for PDSCH with Msg4 is about -4.5 dB, which shows an SNR margin of 2.6 dB with respect to CNR for LEO-600 Set 1-1 and Set 1-2, 0.9 dB with respect to 1-SSB detection, -1.4 dB with respect to 4-SSB detection.

Proposal 15: Link level enhancement on PDSCH with Msg4 may be needed if the required SNR for 4-SSB detection is considered as target SNR, while not needed if the required SNR for 1-SSB detection or CNR for set 1-1/1-2 is considered as target SNR.

Proposal 16: For PDSCH with Msg4, repetitions can be considered to mitigate the SNR gap if required SNR for 4-SSB detection is considered as target SNR for link level enhancement.

Proposal 17: If Msg4 repetition is supported, the repetition configuration can be signalled via SIB.

Reference

- Chair’s notes for RAN1 #118

- TS 31.102, V18.5.0, 3GPP TSG Core Network and Terminals; Characteristics of the Universal Subscriber Identity Module (USIM) application (Release 18)