3GPP TSG RAN WG1 #118b		R1-2407997

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.5.3

Source:	Google

Title:	Adaptation of Common Signals

Document for:	Discussion/Decision

# Introduction

In RAN1 #118 meeting, the following agreements on adaptation of common signal have been achieved.

Agreement For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 1-1: Scaled/adjusted PRACH configuration period - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Opt 1-3: Muting/masking ROs - Opt 1-4: additional timing offset(s) - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources, - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Opt 2-2: Additional timing offset(s) - FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) - FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset) Agreement Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1 - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs RAN1#117 Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs. Agreement For SSB-RO mapping rule for additional PRACH resources for Case 2. - Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format. Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For DCI-based adaptation for additional PRACH resources, - Select from the following DCI format(s) to carry the adaptation indication. - DCI format 1_0 - DCI format 2_7 - DCI format 2_9 - FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated. - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available. - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period - FFS: details

In this contribution, we provide some discussion on adaptation of common signal/channel transmissions.

# Adaptation of SSB in time domain

## Control signal to update the SSB periodicity

For NES, the NW transmit some SSBs associated with active TCI state frequently but other SSBs less frequently as shown in Figure 1. Since the active TCI state can be dynamically changed, it is necessary to consider dynamic update of periodicities for some SSBs.

Figure 1: SSBs with different usages

Since the SSBs are shared by multiple UEs, it is better to dynamically update the periodicities for SSBs by a group-cast signal, e.g., group-cast DCI. With regard to the idle mode UEs that need to decode MIB, it is better to update the SSB periodicity based on the MIB periodicity. Then, the UEs can update the periodicity for the SSBs at the next MIB periodicity, e.g., 80ms, after receiving the group-cast DCI.

Proposal 1: Support to update the periodicity for SSBs by group-cast DCI

- UE updates the periodicity for the SSBs at the next MIB periodicity, e.g., 80ms, after receiving the group-cast DCI

## RLM/BFD based on updated periodicity

After update of the periodicity of SSB for RLM/BFD, UE may further update the RLM/BFD interval. If the NW increases the periodicity of SSB for RLM/BFD, to update the RLM/BFD interval may increase the latency for RLM/BFD. Therefore, one possible way is to update the RLM/BFD RS based on a CSI-RS with the same periodicity as previous periodicity for the SSB. Therefore, it is necessary to support a MAC CE based RLM/BFD RS configuration with regard to dynamic periodicity update of SSB.

Proposal 2: Support MAC CE based RLM/BFD RS configuration with regard to dynamic periodicity update of SSB.

## Rate matching based on updated periodicity

For UE-dedicated signals, the rate matching can be based on the SSB with the updated periodicity. For non-UE dedicated signals, since legacy UE cannot receive the periodicity update signal, the rate matching pattern can be based on the SSB periodicity configured by RRC signaling as shown in Figure 2.

Figure 2: Rate matching pattern for different type of signals

Proposal 3: For UE-dedicated signal, the rate matching pattern should be based on the updated periodicity for the SSB.

Proposal 4: For UE-dedicated signal, the rate matching pattern should be based on the initial periodicity for the SSB configured by RRC.

## Collision handling between SSB and PRACH

Since the RO validation is based on the RRC configured SSB pattern, if the SSB configuration is dynamically changed, some valid RO may collide with SSB as shown in Figure 3. Then it is necessary to define certain collision handling rule between SSB and valid RO. One possible way is to determine such valid RO as unavailable RO for PRACH transmission. Then the UE can select the next available valid RO for PRACH transmission.

Figure 3: SSB and RO collision due to SSB time-domain adaptation

Proposal 5: With regard to SSB time-domain adaptation, a valid RO that overlaps with SSB should be considered as an unavailable RO

- UE transmits the PRACH on the available valid RO

## Cell DTX impact on SSB adaptation

In RAN1 #118 meeting, the following agreement on cell DTX impact on SSB adaptation has been achieved.

Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

The SSB adaptation should be a separate feature, which does not rely on the cell DTX. Therefore, option 3 above should be supported.

Proposal 6: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

# Adaptation of PRACH

## PDCCH ordered PRACH

For PRACH time-domain adaptation, it has been agreed that additional PRACH resources for NES are configured and separate SSB-RO mapping for the additional PRACH resources for NES are supported as follows. Then for the PDCCH ordered PRACH, it is necessary to introduce an indicator indicating whether the triggered PRACH is based on legacy PRACH resources or additional PRACH resources for NES.

Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources

Proposal 7: Support to introduce one DCI field for the PDCCH used to trigger PRACH indicating whether the triggered PRACH is from the legacy PRACH resources or the additional PRACH resources for NES.

## Configuration for the additional PRACH resources for NES

For PRACH time-domain adaptation, the following on the PRACH configuration has been agreed.

Agreement For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 1-1: Scaled/adjusted PRACH configuration period - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Opt 1-3: Muting/masking ROs - Opt 1-4: additional timing offset(s) - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources, - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Opt 2-2: Additional timing offset(s) - FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) - FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

From configuration perspective, both alternatives should be allowed. With regard to orthogonal ROs, the Alt1 can be prioritized. For NES, one possible way is to configure multiple PRACH resources in consecutive frames but with a higher periodicity. Then it can help to reduce the ratio of collision and reduce the NW energy. Therefore, one possible way is to configure multiple y value, e.g., a value set Y, then the NW and UE can determine PRACH can be present in frame nf if it can meet the condition of nf mod x ∈Y.

Further, currently the NW can configure the multiple PRACH configurations for multiple cells to support the LTM and inter-cell mTRP. For NES, it is necessary to support that the NW can also configure multiple additional PRACH configurations, where different additional PRACH configurations for different cells.

Proposal 8: Support both Alt1 and Alt2, i.e., it is up to the NE whether to configure the same or different PRACH configuration index for the additional PRACH resources compared to the legacy PRACH resources

- Support to configure multiple y value, e.g., a value set Y, for additional PRACH resources for NES

  - NW and UE can determine PRACH can be present in frame nf if it can meet the condition of nf mod x ∈Y

Proposal 9: Support the NW to configure multiple additional PRACH configurations for NES, where different additional PRACH configurations for different cells

## RO selection

Since the NW can configure two sets of PRACH resources – additional PRACH resources for NES and legacy PRACH resources. Then how to select the RO for initial transmission and retransmission could be one open issue.

For CFRA, the NW can configure whether UE should select the RO from additional PRACH resources for NES or legacy PRACH resources.

For CBRA, to reduce the probability of collision, one simple way is that the NES-capable UEs select the additional PRACH resource and other UEs select the legacy PRACH resources. However, as the number of NES-capable UE increases, such simple way could lead to high probability of collision for NES-capable UEs. Therefore, one possible way is to select the PRACH resources based on the measured RSRP. If the NES-capable UEs measures a RSRP for an SSB that is higher than a threshold, it can select the RO from the additional PRACH resources, otherwise it selects the RO from the legacy PRACH resources.

For retransmission of CBRA, one simple way is that the UE selects the RO from the same set of PRACH resources. This could simplify the power ramping procedure. However, if the UE can select the RO from a different set of PRACH resources, the probability of collision for retransmission can be reduced. Then how to perform the power ramping when the UE performs retransmission by RO from a different set of PRACH resources could be studied.

Proposal 10: For CFRA, support the NW to configure whether the UE should select the RO from additional PRACH resources for NES or legacy PRACH resources.

Proposal 11: For initial transmission of CBRA, support the UE to select the PRACH resource set based on the measured RSRP

- If the measured RSRP is above a threshold, UE selects the RO from the additional PRACH resources for NES; otherwise, UE selects the RO from legacy PRACH resources

Proposal 12: For retransmission of CBRA, support the UE to select the RO either from the additional PRACH resources for NES or legacy PRACH resources

- FFS: how to perform the power ramping for the retransmission of PRACH

## Interaction with SBFD

For SBFD, the NW can also configure additional PRACH resources for SBFD symbols/slots. Then for PRACH adaptation for NES, it should be clarified that whether the additional PRACH resources can be applicable to SBFD symbols/slots. One possible way is to configure two additional PRACH resource sets for NES, where one is not applicable to the SBFD symbols/slots and the other one is applicable to the SBFD symbols/slots, with regard to UEs that support NES only and UEs that support both SBFD and NES.

Proposal 13: With regard to UEs that support NES only and UEs that support both SBFD and NES, support to configure two additional PRACH resource sets for NES

- The first set is not applicable to the SBFD symbols/slots and the second set is applicable to the SBFD symbols/slots

# Adaptation of PO

## SSB-specific PO

Currently one PO includes the PDCCH MOs associated all the actually transmitted SSBs. However, since the load for different beams can be different, it is unnecessary to include all the PDCCH MOs within one PO. With regard to NES, one possible way is to divide the SSBs into several groups, and configures different PO for different SSB groups. There are similar issues for the PEI-O. Thus, the PEI-O can also be configured in a SSB group specific manner. Then as shown in Figure 4, the NW can configure more POs/PEI-Os for SSB group with high load and less POs/PEI-Os for SSB group with low load.

Figure 4: SSB group specific PO/PEI-O configuration

Proposal 14: Support SSB-group-specific PO/PEI-O configuration

- One PO/PEI-O may include the MOs associated one group of SSBs

## PO activation/deactivation

With regard to NES, the NE may dynamically activate/de-activate some POs or shift some POs. The NW may update the MOs corresponding to one PO. For example, the NW may update the first MO for a PO. Similarly, the NW can also activate/deactivate some PEI-Os or updates the MOs corresponding to one PEI-O. The NW can transmit the dynamic update of PO/PEI-O configuration by L1/L2 signaling.

Proposal 15: Support dynamic update of PO/PEI-O configuration, including

- Activation/deactivation of PO/PEI-O

- Activation/deactivation of MOs of a PO/PEI-O

# Conclusion

In this contribution, we provided discussion on adaptation of common signals. Based on the discussion, the following proposals are provided.

SSB time-domain adaptation

Proposal 1: Support to update the periodicity for SSBs by group-cast DCI

- UE updates the periodicity for the SSBs at the next MIB periodicity, e.g., 80ms, after receiving the group-cast DCI

Proposal 2: Support MAC CE based RLM/BFD RS configuration with regard to dynamic periodicity update of SSB.

Proposal 3: For UE-dedicated signal, the rate matching pattern should be based on the updated periodicity for the SSB.

Proposal 4: For UE-dedicated signal, the rate matching pattern should be based on the initial periodicity for the SSB configured by RRC.

Proposal 5: With regard to SSB time-domain adaptation, a valid RO that overlaps with SSB should be considered as an unavailable RO

- UE transmits the PRACH on the available valid RO

Proposal 6: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

PRACH time-domain adaptation

Proposal 7: Support to introduce one DCI field for the PDCCH used to trigger PRACH indicating whether the triggered PRACH is from the legacy PRACH resources or the additional PRACH resources for NES.

Proposal 8: Support both Alt1 and Alt2, i.e., it is up to the NE whether to configure the same or different PRACH configuration index for the additional PRACH resources compared to the legacy PRACH resources

- Support to configure multiple y value, e.g., a value set Y, for additional PRACH resources for NES

  - NW and UE can determine PRACH can be present in frame nf if it can meet the condition of nf mod x ∈Y

Proposal 9: Support the NW to configure multiple additional PRACH configurations for NES, where different additional PRACH configurations for different cells

Proposal 10: For CFRA, support the NW to configure whether the UE should select the RO from additional PRACH resources for NES or legacy PRACH resources.

Proposal 11: For initial transmission of CBRA, support the UE to select the PRACH resource set based on the measured RSRP

- If the measured RSRP is above a threshold, UE selects the RO from the additional PRACH resources for NES; otherwise, UE selects the RO from legacy PRACH resources

Proposal 12: For retransmission of CBRA, support the UE to select the RO either from the additional PRACH resources for NES or legacy PRACH resources

- FFS: how to perform the power ramping for the retransmission of PRACH

Proposal 13: With regard to UEs that support NES only and UEs that support both SBFD and NES, support to configure two additional PRACH resource sets for NES

- The first set is not applicable to the SBFD symbols/slots and the second set is applicable to the SBFD symbols/slots

PO/PEI-O adaptation

Proposal 14: Support SSB-group-specific PO/PEI-O configuration

- One PO/PEI-O may include the MOs associated one group of SSBs

Proposal 15: Support dynamic update of PO/PEI-O configuration, including

- Activation/deactivation of PO/PEI-O

- Activation/deactivation of MOs of a PO/PEI-O