3GPP TSG RAN WG1 #118bis                                                                                      R1- 2408005

Hefei, China, October 14th – 18th, 2024

Title:	Draft reply LS on applicable functionality reporting for beam management UE-sided model

Response to:	R1-2407604 (R2-2407848)

Release:	Rel-19

Work Item:	NR_AIML_air

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:	N/A

Contact Person:

Name:	Yongqiang Fei

E-mail Address:	Feiyongqiang(at)catt(dot)cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN1 thanks RAN2 for the LS. RAN1 has discussed the questions from RAN2:

On General - Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? On NW-side additional condition and configuration - Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? - Q3: Is NW-side additional condition functionality specific? - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition? - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities? - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc? - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities? - Q5: What is the content of applicable functionality reporting in Step 4? - Q6: What is the content of inference configuration in Step 5? On Functionality Activation - Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? - Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5? - Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? - Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

RAN1 would like to provide the response as following:

Reply to Q1:

- According to TR 38.843, ‘Functionality’ refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

- In RAN1 understanding, the granularity of functionality, if introduced, may be in FG level, or in combination of configuration(s) level. RAN1 still needs to further discuss the granularity of functionality.

- In RAN1 understanding, the term ‘functionality’ may not need to be explicitly specified in specification, if functionality-based LCM can be realized equivalently by management of FG or combination of configuration(s).

Reply to Q2:

- RAN1 agreed to support associated ID to ensure the consistency between training and inference for UE-sided model in beam management. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

- However, associated ID is not NW-side additional condition itself, but to be an implicit indication on whether NW-side additional condition is changed or not. So far, RAN1 hasn’t agreed on specifying any specific content of NW-side additional conditions.

Reply to Q3:

- So far RAN1 has no much discussion on this issue. As working assumption, RAN1 agreed that the associated ID can at least be configured within CSI framework. It is possible that NW-side additional condition is common to some or all functionalities of beam management. More discussion is needed in RAN1.

Reply to Q4-1:

- RAN1 understands that associated ID (implicit indication on the change of NW-side additional condition) is optional.

- It is feasible for UE to decide the applicable functionalities without NW-side additional condition.

Reply to Q4-2:

- If functionality is introduced with a granularity of FG level, it is feasible for NW to provide some configuration in addition to associated ID in Step 3. The configurations provided in Step 3 may only be a subset of full configurations in Step 5, e.g. only those related to model input/output (e.g. configuration related to Set B and Set A) are provided. However, for simplicity, it is also possible (though redundant) to provide full inference configurations (e.g. CSI-ReportConfig) to UE in Step 3.

- If functionality is introduced with a granularity of combination of configuration(s) level, it is redundant to provide configuration in Step 3 anymore, since the supported/required configuration(s) is already implicitly indicated by UE capability report in Step 2.

Reply to Q4-3:

- In Step 5, as working assumption, RAN1 agreed that associated ID can at least be configured in CSI framework, e.g. in CSI-ReportConfig. Thus associated ID can at least be part of inference configuration.

- In Step 3, if functionality is introduced with a granularity of FG level, and if configuration is provided in Step 3, associated ID may be part of inference configuration, or separate from inference configuration.

- In Step 3, if functionality is introduced with a granularity of combination of configuration(s) level, the answer is ‘No’.

Reply to Q4-4:

- In Step 3, if functionality is introduced with a granularity of FG level, and if configuration is provided in Step 3, only configurations impacting the input and output of the AI/ML model are necessary for UE to determine applicable functionalities. It is also possible if complete inference configuration (e.g. CSI-ReportConfig) is provided to UE for simplicity. RAN1 has no agreement on detailed configuration yet.

- In Step 3, if functionality is introduced with a granularity of combination of configuration(s) level, the answer is ‘No’.

Reply to Q5:

- If functionality is introduced with granularity of FG level, the UE may report the applicable functionality, the preferred/supported configuration(s), and associated ID (if multiple associated IDs are provided by NW in Step 3) in Step 4.

- If functionality is introduced with granularity of combination of configuration(s), the UE may report the applicable functionality and associated ID (if multiple associated IDs are provided by NW in Step 3) in Step 4.

Reply to Q6:

- In Step 5, RAN1 assumes CSI report configurations are provided in Step 5, e.g. in CSI-ReportConfig.

Reply to Q7:

- RAN1 do not think configurations (if provided) in Step 3 activates the functionality immediately since the applicability of functionality is unclear and not reported to NW yet.

Reply to Q8:

- RAN1 thinks configuration in Step 5 can activate the functionality immediately.

- However, the actual AI/ML-based CSI report still needs to follow legacy non-AI/ML behaviour, e.g. periodic, semi-persistent, or triggered by dynamic signaling, which are configured in CSI-ReportConfig, with potential additional indication using MAC-CE or DCI.

Reply to Q9:

- Yes, RAN1 thinks multiple/all applicable functionalities can be activated within the same AI/ML use cases or across different AI/ML use cases.

Reply to Q10:

- As long as the activation/deactivation of functionality is independent of actual CSI reporting which is controlled by legacy CSI reporting mechanism, RAN1 does not identify the strong need to activate/deactivate functionality using L1/L2 signaling.

# 2	Actions

To SA2:

ACTION: 	RAN1 respectfully requests RAN2 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #119	18th – 22nd Nov. 2024	Orlando, the USA

TSG RAN WG1 Meeting #120	17th – 21st Feb. 2025	Athens, Greece