3GPP TSG RAN WG1 #118bis		R1-2408006

Hefei, China, October 14th – 18th, 2024

Source:	CATT, Nokia

Title:	Continued discussions on UE procedure for applying transform precoding on PUSCH

Agenda Item:	7

Document for:	Decision

Introduction

In RAN1#118, UE procedure for applying transform precoding on PUSCH was discussed [1][2][3] with the following agreement.

In this contribution, we discuss the remaining issues on UE procedure for applying transform precoding on PUSCH.

Discussion

In order to achieve early identification for different features, RACH partitioning was introduced in Rel-17. A list of AdditionalRACH-Configs can be configured in additionalRACH-ConfigList to provide additional RACH resources for different features/feature combinations. The higher layer signaling structure is illustrated in Figure 1.

Figure 1: signaling structure of RACH configurations from Rel-17

With introduction of additionalRACH-ConfigList in Rel-17, which msg3-transformPrecoder/ msgA-TransformPrecoder are used for PUSCH waveform determination is unclear because multiple configurations of msg3-transformPrecoder/msgA-TransformPrecoder can be provided. And in TS 38.331, it is assumed that msg3-transformPrecoder can be different in rach-ConfigCommon included directly within BWP configuration and in rach-ConfigCommon included in additionalRACH-ConfigList.

The issue was discussed in RAN1#118 discussions. The following text proposal was agreed for PUSCH transmission scheduled by a DCI format 0_0 with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI.

Agreement Adopt the following TP for clause 6.1.3, TS38.214 for alignment in principle from Rel-17. For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder from rach-ConfigCommon included directly within BWP configuration. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder..

In addition, companies agreed with the following proposals and think no spec change is needed [3].

Proposal 2: For a PUSCH scheduled by RAR UL grant and for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, UE enables or disables transform precoding according to msg3-transformPrecoder in the RACH-ConfigCommon corresponding to the RACH procedure. Proposal 3: For a MsgA PUSCH, UE enables or disables transform precoding according to msgA-TransformPrecoder in the MsgA-ConfigCommon corresponding to the RACH procedure.

The following two cases were discussed without conclusion.

- PUSCH scheduled by fallbackRAR UL grant

- MsgA PUSCH when msgA-TransformPrecoder is not configured

1) PUSCH scheduled by fallbackRAR UL grant

According to existing TS38.214, the waveform for PUSCH scheduled by fallbackRAR UL grant is determined according to msg3-transformPrecoder.

For a PUSCH scheduled by RAR UL grant, or for a PUSCH scheduled by fallbackRAR UL grant, or for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

Regarding which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant, two alternatives were discussed in RAN1#118.

  - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

  - Alt 2:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config.

Proponents proposed Alt 1 in [2]. Four companies agree with Alt 2, in which three companies think Alt 2 is more logical and one company thinks Alt 2 follows the principle of RAN2 spec and propose to send an LS to RAN2 if it is not clear. One company needs further internal checking with RAN2 colleagues [3].

2) MsgA PUSCH when msgA-TransformPrecoder is not configured

According to existing TS38.214, the waveform for PUSCH scheduled by fallbackRAR UL grant is determined according to msg3-transformPrecoder.

For a MsgA PUSCH, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msgA-TransformPrecoder. If higher layer parameter msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

Regarding which msg3-transformPrecoder is used to determine waveform for MsgA PUSCH when msgA-TransformPrecoder is not configured, four alternatives were discussed in RAN1#118.

  - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

  - Alt 2:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config.

  - Alt 3: UE does not expect higher layer parameter msgA-TransformPrecoder is not configured.

  - Alt 4: If the higher layer parameter msgA-TransformPrecoder is not configured, UE expects the same value of msg3-transformPrecoder between that in rach-ConfigCommon included directly within BWP configuration and that in AdditionalRACH-Config.

Views are quite divergent among the four alternatives. Alt 1, Alt 2 and Alt 3 are supported by two companies respectively. Alt 4 is proposed by one company [3].

Based on the discussions, it is observed that RAN1 cannot make consensus on which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH when msgA-TransformPrecoder is not configured. As some companies commented, this issue is related to RAN2 RACH procedure. It is more efficient to send an LS to RAN2 to ask for their views instead of debating in RAN1 among the alternatives.

Proposal: Send an LS to RAN2 to ask which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH when msgA-TransformPrecoder is not configured.

- 1 Overall description In RAN1#118 and RAN1#118bis, RAN1 discussed UE procedure for applying transform precoding on PUSCH with introduction of additionalRACH-ConfigList in R1-2406324 and R1-2408006. For a PUSCH scheduled by fallbackRAR UL grant, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder according to TS38.214. For a PUSCH scheduled by RAR UL grant, or for a PUSCH scheduled by fallbackRAR UL grant, or for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.. For this case, the following two alternatives were discussed on which msg3-transformPrecoder should be used without conclusion in RAN1. - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration. - Alt 2: - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration. - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config. For MsgA PUSCH if msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder according to TS38.214. For a MsgA PUSCH, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msgA-TransformPrecoder. If higher layer parameter msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.. For this case, the following alternatives were discussed on which msg3-transformPrecoder should be used without conclusion in RAN1. - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration. - Alt 2: - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration. - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config. - Alt 3: UE does not expect higher layer parameter msgA-TransformPrecoder is not configured. - Alt 4: If the higher layer parameter msgA-TransformPrecoder is not configured, UE expects the same value of msg3-transformPrecoder between that in rach-ConfigCommon included directly within BWP configuration and that in AdditionalRACH-Config. - 2 Actions To RAN2 ACTION: RAN1 respectfully asks RAN2 to answer the following questions. Q1: From RAN2 point of view, which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH when msgA-TransformPrecoder is not configured respectively? Q2: Is the related UE behavior already defined in RAN2 spec?

Conclusion

In this contribution, we discuss the remaining two cases on waveform determination for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH when msgA-TransformPrecoder is not configured. It is proposed to send and LS to RAN2 and a draft LS is provided as a starting point for discussions.

Proposal: Send an LS to RAN2 to ask which msg3-transformPrecoder is used to determine waveform for PUSCH scheduled by fallbackRAR UL grant and MsgA PUSCH when msgA-TransformPrecoder is not configured.

Reference

- R1-2406324, UE procedure for applying transform precoding on PUSCH from Rel-17, CATT, Nokia

- R1-2406325, Clarification on UE procedure for applying transform precoding on PUSCH, CATT, Nokia

- R1-2407483, Summary#2 of UE procedure for applying transform precoding on PUSCH, Moderator (CATT)