3GPP TSG RAN WG1 #118bis		                                                      R1-2408008

Hefei, China, October 14th – 18th, 2024

Source:	CATT

Title:	Discussion on SRS power scaling and transmission

Agenda Item:	7

Document for:	Discussion and Decision

- Introduction

This contribution discusses specification change to 38.213 subclause 7.5 when multiple SRS resources for usage ‘nonCodebook’ are transmitted simultaneously.

- Discussion

From RAN1#115 to RAN1#118 meetings, the issues on SRS power scaling and transmission for the scenarios with multiple SRS resources transmitted simultaneously had been discussed.

On SRS power scaling and transmission for SRS resources for usage ‘nonCodebook’, the following agreement and conclusion have been achieved in previous meetings:

Agreement(RAN1#116) For a given CC, multiple SRS resources in a set with usage “nonCodebook” are not expected to be partially overlapped in time - Specification change is only for Rel-17 and onwards Conclusion(RAN1#116bis) The transmit power of each of the simultaneously transmitted SRS resources for usage ‘nonCodebook’ is equal in the fully overlapping time/frequency case, regardless of whether calculated transmission power across all SRS resources is higher or lower than Pcmax. - No specification change Agreement(RAN1#118) The following TP is agreed for Rel-18 TS38.213. Final CR in R1-2407562. - Further discuss specification change to 38.213 subclause 7.5 in RAN1#118bis. If no consensus, no future discussions beyond RAN1#118bis. ## 7.3 Sounding reference signals For SRS, - if a UE is provided nrofSRS-Ports-n8 = ‚ports8tdm‘ for an SRS resource with 8 ports in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, the UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antennaSRS ports of an SRS resource of an SRS resource set on each symbol for SRS transmission. - else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS SRS ports of each SRS resource of an SRS resource set in a symbol for SRS transmission. ### 7.3.1 UE behaviour If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) of each SRS resource in SRS transmission occasion i as [dBm] where, - PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2], [TS 38.101-3] and [8-5, TS 38.101-5] for carrier f of serving cell c in SRS transmission occasion i.

Based on the agreement in RAN1#118, the SRS transmission power PSRS,b,f,c(i,qs,l) in TS 38.213 is the transmission power for one SRS resource. Therefore when multiple SRS resources in an SRS resource set with usage set to ‘nonCodebook’ are transmitted simultaneously, it is possible that the total power across the simultaneously transmitted SRS resources is higher or lower than Pcmax.

Carrier aggregation and operation with two uplink carriers were not considered when RAN1 made the conclusion in RAN1 #116bis meeting that the transmit power of each of the simultaneously transmitted SRS resources for usage ‘nonCodebook’ is equal in the fully overlapping time/frequency case, regardless of whether calculated transmission power across all SRS resources is higher or lower than Pcmax. Therefore it was agreed in RAN1#118 meeting to further discuss specification change to 38.213 subclause 7.5 in RAN1#118bis, where subclause 7.5 is prioritizations for transmission power reductions for single cell operation with two uplink carriers or for operation with carrier aggregation.

Since the power control parameters for SRS are determined per SRS resource set, it is definitely the transmit power of each of the simultaneously transmitted SRS resources in an SRS resource set with usage set to ‘nonCodebook’ is equal if the calculated transmit power for the SRS resources are used directly.

For single cell operation with two uplink carriers or for operation with carrier aggregation, if the total transmit power of the UL transmissions in the same transmission occasion exceeds Pcmax, for the following two cases, the UE cannot use the calculated transmit power for the SRS resources in an SRS resource set with usage set to ‘nonCodebook’ as the actual transmit power:

- Case 1: The total power of the calculated transmission power across the simultaneously transmitted SRS resources in the SRS resource set with usage set to ‘nonCodebook’ exceeds Pcmax.

- Case 2: The total power of the calculated transmission power across the simultaneously transmitted SRS resources in the SRS resource set with usage set to ‘nonCodebook’ is less than Pcmax, but is larger than Pcmax minus Ph, where Ph is the total transmit power of UL transmissions with higher priority than the SRS resources in the SRS resource set with usage set to ‘nonCodebook’ on the serving cells in the same transmission occasion.

Similar as that for SRS transmission in single carrier, for the above two cases, it can be clarified that the transmit power of each of the simultaneously transmitted SRS resources for usage ‘nonCodebook’ is equal. In order to make the specification more clear, we prefer to capture the clarification in subclause 7.5 in TS 38.213. Similar as that for the clarification of SRS transmission power PSRS,b,f,c(i,qs,l) in TS 38.213, this clarification can start from Rel-18.

Proposal 1: Clarify in subclause 7.5 in TS38.213 that for single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for UL transmissions on serving cells in a frequency range in a respective transmission occasion i would exceed PCMAX(i) and there are multiple SRS resources in an SRS resource set with usage set to ‘nonCodebook’ to be transmitted in the same transmission occasion, the transmit power of each of the simultaneously transmitted SRS resources in the SRS resource set with usage set to ‘nonCodebook’ is equal, regardless of whether calculated transmission power across all SRS resources is higher or lower than Pcmax.

The companion CR for TS 38.213 is provided in [1] with the following changes:

## 7.5 Prioritizations for transmission power reductions For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i. If SRS resource(s) in an SRS resource set with usage set to ‘nonCodebook’ is to be transmitted, and there are multiple SRS resources in the SRS resource set with usage set to ‘nonCodebook’ in the same transmission occasion i, the UE transmits all the SRS resources in the SRS resource set with usage set to ‘nonCodebook’ and allocates equal power to the SRS resources. If the UE transmits SRS on multiple SRS resources according the XYZ [6, TS 38.214], the UE allocates power so that all Res of the SRS transmission have same power. <Unchanged parts are omitted>

- Conclusions

In this contribution, we provide our views on SRS power control when a total UE transmit power for UL transmissions on serving cells in a frequency range in a respective transmission occasion i would exceed PCMAX(i) and multiple SRS resources in an SRS resource set with usage set to ‘nonCodebook’ are to be transmitted in the transmission occasion simultaneously. We also provide the following proposal:

Proposal 1: Clarify in subclause 7.5 in TS38.213 that for single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for UL transmissions on serving cells in a frequency range in a respective transmission occasion i would exceed PCMAX(i) and there are multiple SRS resources in an SRS resource set with usage set to ‘nonCodebook’ to be transmitted in the same transmission occasion, the transmit power of each of the simultaneously transmitted SRS resources in the SRS resource set with usage set to ‘nonCodebook’ is equal, regardless of whether calculated transmission power across all SRS resources is higher or lower than Pcmax.

The companion CR for TS 38.213 is provided in [1].

- Reference

- R1-2408009, Clarification on SRS power scaling for non-codebook based UL transmission, CATT, 3GPP TSG RAN1#118bis, Hefei, China, October 14th – 18th, 2024.