3GPP TSG-RAN WG1 Meeting #118b	R1-2408017

Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Type-3 HARQ-ACK codebook triggered by DCI format 1_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_Enh-Core |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When a DCI format 1_3 tregger an enhanced Type-3 HARQ-ACK codebook transmission, it was agreed that an invalid FDRA field values can indicate the index of enhanced Type-3 HARQ-ACK codebook, and cells with valid FDRA fields can be scheduled. Agreement For an enhanced Type-3 HARQ-ACK codebook triggered by a DCI format 1_3, if the enhanced Type-3 HARQ-ACK codebook indicator is not configured, the MCS field of TB1 corresponding to a cell with smallest serving cell index among the co-scheduled cells with invalid FDRA field values is used to indicate the index of the enhanced Type-3 HARQ-ACK codebook. - Note: Cells with valid FDRA fields are scheduled. It means that if the DCI format does not include the enhanced Type 3 codebook indicator field and there is no invalid FDRA field in the DCI fortmat, the pdsch-HARQ-ACK-EnhType3Index value is zero. But current spec does not capture it. In additional, if DCI indicates the index of enhanced Type-3 HARQ-ACK codebook using one cell with invalid FDRA and schedules one or more PDSCH reception(s) on other cell(s), the timeline of N symbols cann’t be applied to the DCI foramat 1_3. However, the above cann’t be reflected in current specification. |
| Summary of change: |  | 1: Clarify that If the DCI format does not include the enhanced Type 3 codebook indicator field and there is no invalid FDRA field in the DCI fortmat, the pdsch-HARQ-ACK-EnhType3Index value is zero, 2: If format 1_3 scheduling one or more PDSCH reception(s) and an invalid FDRA field values can indicate the index of enhanced Type-3 HARQ-ACK codebook, clarify that value of N is not referred in clause 10.2 by replacing "SPS PDSCH release" with "DCI format". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback or pdsch-HARQ-ACK-EnhType3ToAddModList, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and a DCI format scheduling one or more PDSCH reception(s)  and triggering the Type-3 HARQ-ACK codebook includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a size of a set of indicated serving cells NcellsDL,ind and a size of a set of indicated HARQ process numbers NHARQ,cDL,ind for each indicated serving cell and each indicated HARQ process number from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. Each bit from MSB to LSB provided by perCC corresponds to a serving cell in ascending order of serving cell index, where value '1' or value '0' indicate HARQ-ACK for the corresponding serving cell is included or not included in the Type 3 HARQ-ACK codebook, respectively. Each bit string provided by perHARQ corresponds to a serving cell in ascending order of serving cell index, and each bit from MSB to LSB within a bit string corresponds to a HARQ process number on a corresponding serving cell in ascending order of HARQ process number, where value '1' or value '0' indicate HARQ-ACK for the corresponding HARQ process number on the corresponding serving cell is included or not included in the Type 3 HARQ-ACK codebook, respectively. If the DCI format does not include the enhanced Type 3 codebook indicator field and there is no invalid FDRA field in the DCI fortmat, the pdsch-HARQ-ACK-EnhType3Index value is zero.

Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind.

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH or nrofHARQ-ProcessesForPDSCH-v1700 for serving cell c, if provided; else, set NHARQ,cDL=8 . When applicable, set NHARQ,cDL to NHARQ,cDL,ind.

Set NTB,cDL to the maximum value of maxNrofCodeWordsScheduledByDCI in PDSCH-config and PDSCH-configMulticast for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1.

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-EnhType3CBG corresponding to the pdsch-HARQ-ACK-EnhType3Index value is provided; else, set NHARQ-ACK,cCBG/TB,max=0. A UE provided with pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-EnhType3CBG does not expect to be provided with different values of maxCodeBlockGroupsPerTransportBlock for different priority indexes in pdsch-CodeBlockGroupTransmissionList for serving cell c.

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-EnhType3NDI is provided; else set NDIHARQ=1.

Set c=0 – serving cell index in the set of serving cells

Set h=0 – HARQ process number index in the set of numbers of HARQ processes

Set t=0 – TB index

Set g=0 – CBG index

Set j=0

while c<NcellsDL

while h<NHARQ,cDL

if downlinkHARQ-FeedbackDisabled is not provided, or is provided and indicates enabled HARQ-ACK information for h, or harq-feedbackEnablingforSPSactive is provided and enabled and h corresponds to a transport block in a first SPS PDSCH reception after an activation of SPS PDSCH receptions

if NDIHARQ=0

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

while g<NHARQ-ACK,cCBG/TB,max

ojACK= HARQ-ACK information bit for CBG g of TB t for HARQ process number index h in the set of numbers of HARQ processes of serving cell c, if any; else, ojACK=0

j=j+1

g=g+1

end while

ojACK= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number index h in the set of numbers of HARQ processes on serving cell c, if any; else, ojACK=0

g=0

j=j+1

t=t+1

end while

else

while t<NTB,cDL

ojACK= HARQ-ACK information bit for TB t for HARQ process index h in the set of numbers of HARQ processes of serving cell c, if any; else, ojACK=0

j=j+1

ojACK= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number index h in the set of numbers of HARQ processes on serving cell c, if any; else, ojACK=0

j=j+1

t=t+1

end while

end if

t=0

else

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

if UE has obtained HARQ-ACK information for TB t for HARQ process number index h in the set of numbers of HARQ processes on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

while g<NHARQ-ACK,cCBG/TB,max

ojACK= HARQ-ACK information bit for CBG g of TB t for HARQ process number index h in the set of numbers of HARQ processes of serving cell c

j=j+1

g=g+1

end while

else

while g<NHARQ-ACK,cCBG/TB,max

ojACK=NACK

j=j+1

g=g+1

end while

end if

g=0

t=t+1

end while

else

while t<NTB,cDL

if UE has obtained HARQ-ACK information for TB t for HARQ process number index h in the set of numbers of HARQ processes on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

if harq-ACK-SpatialBundlingPUCCH is not provided

ojACK= HARQ-ACK information bit for TB t for HARQ process number index h in the set of numbers of HARQ processes of serving cell c

else

ojACK= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process number index h in the set of numbers of HARQ processes of serving cell c. If the UE receives one transport block, the UE assumes ACK for the second transport block

end if

j=j+1

t=t+1

else

ojACK=NACK

j=j+1

t=t+1

end if

end while

end if

t=0

end if

end if

h=h+1

end while

h=0

c=c+1

end while

If NTB,cDL>1, when a UE receives a PDSCH with one transport block, the HARQ-ACK information is associated with the first transport block.

If a UE receives a SPS PDSCH, or a PDSCH that is scheduled by a DCI format that does not support CBG-based PDSCH receptions for a serving cell c, and if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c, and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-EnhType3CBG corresponding to the pdsch-HARQ-ACK-EnhType3Index value is provided, the UE repeats NHARQ-ACK,cCBG/TB,max times the HARQ-ACK information for the transport block, if any, in the PDSCH.

If a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, the UE determines a PUCCH or a PUSCH to multiplex a Type-3 HARQ-ACK codebook for transmission in a slot as described in clauses 9.2.3 and 9.2.5. If the UE is provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE determines the slot and a corresponding cell based on the periodic cell switching pattern as described in clause 9.A. The UE multiplexes only the Type-3 HARQ-ACK codebook in the PUCCH or the PUSCH for transmission in the slot. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList, the UE expects that HARQ-ACK information in a Type-1 or Type-2 HARQ-ACK codebook in a slot is associated with HARQ process(es) of the Type-3 HARQ-ACK codebook in the slot.

If

-	a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, and

-	the CRC of the DCI is scrambled by a C-RNTI or an MCS-C-RNTI, and

if for one or more serving cells

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in the DCI format are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in the DCI format are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in the DCI format are equal to 0 or 1

the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception on the one or more serving cells. If the UE is provided pdsch-HARQ-ACK-EnhType3ToAddModList and the DCI format includes an enhanced Type 3 codebook indicator field that provides a value for pdsch-HARQ-ACK-EnhType3Index, the UE determines a size of a set of indicated serving cells NcellsDL,ind and a size of a set of indicated HARQ process numbers NHARQ,cDL,ind for each indicated serving cell and each indicated HARQ process number from the entry in pdsch-HARQ-ACK-EnhType3ToAddModList corresponding to the pdsch-HARQ-ACK-EnhType3Index value. If the DCI format does not include the enhanced Type 3 codebook indicator field, the pdsch-HARQ-ACK-EnhType3Index value is provided by the value of

-	the MCS field for transport block 1 if the DCI format is DCI format 1_1,

-	the MCS field if the DCI format is DCI format 1_2,

-	the MCS field for transport block 1 of a serving cell with smallest index among the one or more serving cells if the DCI format is DCI format 1_3.

The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format other than format 1_3 scheduling one or more PDSCH reception(s), where the value of N is provided in clause 10.2 by replacing "SPS PDSCH release" with "DCI format".

If a UE multiplexes HARQ-ACK information in a PUSCH transmission, the UE generates the HARQ-ACK codebook as described in this clause except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

< Unchanged parts are omitted >