3GPP TSG RAN WG1 #118bis                 .                                                            R1-2408039

Hefei, China, October 14th – 18th, 2024

Source:	CATT

Title:	Enhancement for UE-initiated/event-driven beam management

Agenda Item:	9.2.1

Document for:	Discussion and Decision

Introduction

In RAN1 #118 meeting, following agreements and working assumptions were achieved on UE-initiated/event-driven beam management [1].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH. - FFS: PUCCH as the second channel - FFS: Whether the PUSCH can be with UL data Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported. CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #N. L1-RSRP #1. Differential L1-RSRP #2. …. Differential L1-RSRP #N. Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC. Note: Other contents are not precluded. . - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1 - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP. - FFS: range and step size of differential L1-RSRP - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID). Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, select one of the following options - Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - FFS: DCI format 0_3 - Option-2: Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - FFS: DCI format 0_3 - FFS: Whether/how to handle the case that multiple CSI report configuration(s) for the UE-initiated/event-driven beam report are associated with the same first PUCCH resource and/or the same scheduled PUSCH Conclusion On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, there is no RAN1 consensus on additionally supporting that the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH. Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4} - FFS: additional candidate value(s) of {5, 6, 7, 8} - FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default. Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further down-select one of the following for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement Note: If there is no consensus in RAN1 on one of Options 1/2/3, Option 4 will be taken. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least one or both of the following should be supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, the following is provided for down-selection. - Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting. - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel. - Option-1B: No restriction in terms of periodicity. - Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. - FFS: Whether the first and second channels can be from the same/different CC. Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A and/or Mode-B, further study the following for first PUCCH transmission - UCI multiplexing/dropping/prioritization rule - Conditions for the transmission of the first PUCCH. - Whether the PUCCH resource in the first PUCCH channel can be associated with multiple CSI report configurations for UE-initiated/event-driven beam reporting from one or multiple CC(s). - Whether/how to re-transmit the first PUCCH channel. Whether/how to apply prohibit-timer or maximum number of (re)transmission(s) for first PUCCH channel.

In this contribution, related issues on UE-initiated/event-driven beam management are discussed, including triggering events, RS resources for beam measurement, UL report contents and UL report container.

Discussion

  - Triggering events

In RAN1#118 meeting, Event-1 and Event-7 were remained as working assumption [1]:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

It is essential to decide whether the surviving events should be supported or not. In the following, analysis and views with respect to the events are provided.

Both Event-1 and Event-7 are beneficial to UEIBM. Event-1 can offer additional knowledge on current beam for gNB before BFR happens. When Event-1 is satisfied and reported by UE, gNB can initiate a beam sweeping to find a new beam. In this case, BFR can be avoided. Event-7 can help fast beam switching regarding the activated TCI state list. For the two events, the conclusions on current beam and new beam for Event-2 can be reused.

Proposal 1: On UE-initiated/event-driven beam management, confirm the following working assumption to support Event-1 and Event-7:

Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

  - RS resources for beam measurement

Regarding RS measurement for the new beam for Event-2, it was agreed that the RS(s) for new beam(s) were explicitly configured in one RS resource set associated with an CSI reporting configuration in RAN1#118 meeting [1]. One open issue is whether the RS in the RS resource set can be updated by MAC-CE or not. Updating the TCI state via MAC CE can reduce the latency compared with the re-configuration of RRC. Thus, it is beneficial to support the update via MAC-CE.

Proposal 2: Regarding explicit RS configuration for new beam management for Event-2, support updating the RS resources in the RS resource set for new beam by MAC-CE

In legacy CSI reporting framework, the maximum number of CSI-RS resources in a CSI-RS resource set can be reported by a UE while the maximum value among the candidate values for the UE capability can be 64. For UEIBM, the maximum number of CSI-RS resources may be relatively small, thus, the same scheme can be followed while a UE capability indicating the maximum supported number of the configured RS(s) in the RS resource set for UEIBM is introduced.

Proposal 3: Regarding explicit RS configuration for new beam management for Event-2, support to introduce a UE capability of indicating the maximum number of the configured RS(s) in the RS resource set for UEIBM.

Regarding RS measurement for the current beam for Event 2, for enabling one of either Scheme-1 or Scheme-2 by NW in Option-2a, two options are remained for down-selection. One is the implicit manner while the other is the explicit manner. The implicit manner is that if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. While, the explicit manner introduces a new RRC parameter to indicate which scheme is used. As the RS types of the new beam and the current beam should be the same is not decided yet, an explicit RRC parameter to enable the scheme is the simplest and clearer way.

Proposal 4: Regarding explicit RS configuration for new beam management for Event-2, support explicit manner, i.e., enabling one of either Scheme-1 or Scheme-2 via an explicit RRC parameter.

Regarding RS measurement for the new beam for Event-2, another pending issue is whether support the implicit methods. Two implicit methods have been remained for further discussion:

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

For option-3b, one possible use case is to update the indicated current serving beam among the activated beams, these QCL source RS signals of the activated TCI-state can be implicitly used as measurement RS resource rather than explicitly signaled. However, one concern for option-3b is that currently the QCL source RS for PDSCH in the unified TCI state can be only CSI-RS for beam management or TRS, if the RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s), it seems that only CSI-RS can be used as RSs for new beams for option-3b, which is contradictory with the agreement that support L1-RSRP as a measurement quantity on SSB.

For Option-3c, with the agreement that supporting L1-RSRP as a measurement quantity on SSB, applying the QCL RS(s) of the configured TCI-states as the measurement resource may lead to an unfair comparison between CSI-RS beam and SSB beam which have different beam width and transmission power, number of ports, etc.

As above, it is not necessary to support implicit methods for RS measurement for new beam for event-2.

Proposal 5：For RS measurement for new beam for event-2, do not support option-3b or option 3c.

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, it is agreed to further study the case that only one TRS is configured in the indicated TCI state. Currently, only CSI-RS for beam management and SSB can be used as the measurement RS while TRS cannot be used. In RAN1#118 meeting, following options were proposed:

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement

Option-1 introduces additional scheme that the TRS for current beam must be associated with a CSI-RS for beam management derived from the QCL RS in the indicated TCI state. However, TRS is mostly used for wide beam case while CSI-RS is used for narrow beam cases. Thus, such an association may not be able to achieve. Option-2 supports TRS as the measurement RS for current beam, which is not used in current spec. Additionally, the scope of UEIBM states to leverage the legacy CSI framework, thus, it is not essential to support TRS as the measurement RS in this case. Option-3 introduces additional scheme that the RS for current beam can be explicitly configured by RRC or MAC-CE, however, this follows a scheme (explicitly configuration for measurement RS for current beam) which is already being precluded in a previous agreement. It is not essential to re-discuss a precluded scheme. Option-4 proposes to introduce no further enhancement. In this case, UE can suspend the UEIBM report when only one TRS is configured in the indicated TCI state. As the RS of the current beam is from the indicated TCI state, UE can still perform UEIBM report as long as the indicated TCI state has changed to be one not configured with only one TRS.

Proposal 6：Regarding RS measurement for the current beam for event-2, when only one TRS is configured in the indicated TCI state, support one of the following schemes:

- Alt1: No further enhancement, i.e., UE suspends the measurement for UEIBM until the indicated TCI state has changed to be one not configured with only one TRS.

- Alt2: If there exists a CSI-RS QCLed with the TRS configured in the indicated TCI state, uses the CSI-RS as the measurement RS, otherwise, UE suspends the measurement for UEIBM until the indicated TCI state has changed to be one not configured with only one TRS or to be one configured with only one TRS but there exists a CSI-RS associated with the TRS.

  - UL report content

In RAN1#118 meeting, it was agreed to use the differential reporting. One remaining issue is that the range and step size of differential L1-RSRP. The reporting of the L1-RSRP for UEIBM is merely the same as the legacy L1-RSRP reporting, thus, the existing range and the step size (4-bit value with 2dB step size) of differential L1-RSRP can be re-used. Another issue is whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. As gNB knows the differential values between other reported beams and the best new beam while the best reported new beam must satisfy the triggering condition of the Event-2 it is not necessary to further indicate other satisfied new beam(s). On the other hand, it is necessary to indicate the current beam when the RRC parameter that the current beam is always reported is disabled. Otherwise, gNB did not know whether the current beam is reported or not.

Proposal 7: For the differential L1-RSRP report in UE-initiated/event-driven beam management, for the range and the step size of the differential L1-RSRP, re-use the existing range and the step size of differential L1-RSRP, i.e., 4-bit value with 2dB step size

Proposal 8: For the differential L1-RSRP report in UE-initiated/event-driven beam management, it is not necessary to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

Proposal 9: For the differential L1-RSRP report in UE-initiated/event-driven beam management, support to indicate whether the current beam is reported when the RRC parameter that the current beam is always reported is disabled.

In RAN1#117 meeting, it was agreed that the UE initiated beam reporting can occur when a number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M. However, it is not mentioned which RSRP value is reported through the M satisfied values. As this condition is within a configured time window, the last satisfied RSRP values should be reported as it can show the latest current channel status.

Proposal 10: For event-2 in UE-initiated/event-driven beam management, when M event-2 instance(s) for one same new beam are used to trigger beam report within a configured time window, the latest RSRP value is reported for this new beam.

  - UL report container

In RAN1#118 meeting, regarding the triggering procedure in Step-2 of Mode-A, two options were remained for down-selection:

- Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

- Option-2: Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

Option-1 is a straightforward way to trigger the transmission of the UEI beam report, if the 1-bit belong to the reserved bits, the spec effect is small. Otherwise, extra effect on the spec have to be conducted. Option-2 can be implemented by introducing a specific report quantity for UEIBM in CSI-ReportConfig. By doing so, the CSI request field in DCI format 0_1/0_2 can be re-used to trigger the transmission of the UEI beam report. In this case, the spec effect is negligible.

Proposal 11: For the triggering procedure in Step-2 of Mode-A, reuse CSI request field in DCI format 0_1/0_2.

In RAN1#118 meeting, regarding the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, two options were remained for further discussion:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report.

If option-1 is supported, the issue of multiplexing UL-SCH and the beam report needs specified. This leads to further signaling overhead which is controversial to the intention of the UEIBM. Additionally, from gNB’s point of view, it is not clear how to distinguish between data and the beam report for a pre-configured CG PUSCH. In this case, option-2 is a straightforward solution to carry the beam report.

Proposal 12: Regarding the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, support option-2:

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report.

In RAN1#118 meeting, regarding Mode-B, it was agreed that the UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. In this case, the definition of the first available transmission occasion needs clarification.

Proposal 13: For the Mode-B of UL report container for UEIBM, further study the definition of the “first available transmission occasion” of the second channel.

In RAN1#118 meeting, regarding the resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, two options were remained for down-selection:

- Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

  - Option-1B: No restriction in terms of periodicity.

- Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

Option-1 is a straightforward solution. As it is agreed to have a timeline between the first channel and the second channel, a one-to-one mapping between them can help gNB acquiring the knowledge of the reporting in an efficient manner. Also, restriction of the periodicity is not necessary as a fixed timeline is used.

Proposal 14: Regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, support option-1B.

In RAN1#118 meeting, regarding the dedicated RRC signaling for first PUCCH channel configuration, two options were remained for down-selection for both mode-A and mode-B.

- Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration.

Although the design can be different for mode-A and mode-B, strive a unified solution is preferred. Alt 1 is a straightforward solution for mode-B. For mode-B, as a pre-configured PUSCH is used as the second channel, UE can send a simple notification and transmit the beam report on the pre-configured slot. This can work well in this case. Similarly, the same scheme works for mode-A as well. UE sends a request using a dedicated SR and waits a response from gNB. By contrast, supporting alt-2 is not necessary as UEIBM is not frequently triggered. As WID describes to use the legacy scheme as much as possible, it is not essential to introduce a new UCI type when alt1 can work for both mode-A and mode-B. Besides, introducing a new UCI type will lead to substantial spec impact, which is not necessary.

Proposal 15: For UL report container of Mode A and Mode B in UEIBM, support to have a unified solution, i.e., alt 1: a dedicated SR for both mode-A and mode-B. Do not support to introduce a new UCI type.

In RAN1#118 meeting, it has been agreed that for Event-2 Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. One further issue to be discussed is whether the first and second channel can be from the same/different CC. In the legacy BFR mechanism, the CC where the dedicated BFR SR transmitted can be different from the CC where the BFR MAC CE transmitted. Similar mechanism can be applied in UEIBM, i.e., the CC of the first channel can be different from CC of the second channel. Specifically, for UEIBM Mode-B, since the UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel, if the second UL channel of Type-1 CG PUSCH is on the same CC as the first uplink channel, UE might need to wait for a long time for the first available transmission occasion of the second UL channel, to avoid such case, it will be beneficial if the second Type-1 CG PUSCH channel is from a different CC with a near first available transmission occasion.

Proposal 16: On beam report transmission procedure for UEIBM, regarding Event-2 for Mode-B, the first and the second channels can be from the different CC.

Conclusions

In this contribution, we provide our views on the enhancements for UE-initiated beam management, including triggering events, RS resources used for beam measurement, UL report contents and UL report container. We have the following proposals:

Proposal 1: On UE-initiated/event-driven beam management, confirm the following working assumption to support Event-1 and Event-7:

Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

Proposal 2: Regarding explicit RS configuration for new beam management for Event-2, support the RS in the RS resource set can be updated by MAC-CE

Proposal 3: Regarding explicit RS configuration for new beam management for Event-2, support to introduce a UE capability of indicating the maximum number of the configured RS(s) in the RS resource set for UEIBM.

Proposal 4: Regarding explicit RS configuration for new beam management for Event-2, support explicit manner, i.e., enabling one of either Scheme-1 or Scheme-2 via an explicit RRC parameter.

Proposal 5：For RS measurement for new beam for event-2, do not support option-3b or option 3c.

Proposal 6：Regarding RS measurement for the current beam for event-2, when only one TRS is configured in the indicated TCI state, support one of the following schemes:

- Alt1: No further enhancement, i.e., UE suspends the measurement for UEIBM until the indicated TCI state has changed to be one not configured with only one TRS.

- Alt2: If there exists a CSI-RS QCLed with the TRS configured in the indicated TCI state, uses the CSI-RS as the measurement RS, otherwise, UE suspends the measurement for UEIBM until the indicated TCI state has changed to be one not configured with only one TRS or to be one configured with only one TRS but there exists a CSI-RS associated with the TRS.

Proposal 7: For the differential L1-RSRP report in UE-initiated/event-driven beam management, for the range and the step size of the differential L1-RSRP, re-use the existing range and the step size of differential L1-RSRP, i.e., 4-bit value with 2dB step size

Proposal 8: For the differential L1-RSRP report in UE-initiated/event-driven beam management, it is not necessary to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

Proposal 9: For the differential L1-RSRP report in UE-initiated/event-driven beam management, support to indicate whether the current beam is reported when the RRC parameter that the current beam is always reported is disabled.

Proposal 10: For event-2 in UE-initiated/event-driven beam management, when M event-2 instance(s) for one same new beam are used to trigger beam report within a configured time window, the latest RSRP value is reported for this new beam.

Proposal 11: For the triggering procedure in Step-2 of Mode-A, reuse CSI request field in DCI format 0_1/0_2.

Proposal 12: regarding the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, support option-2:

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report.

Proposal 13: For the Mode-B of UL report container for UEIBM, further study the definition of the “first available transmission occasion” of the second channel.

Proposal 14: Regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, support option-1B.

Proposal 15: For UL report container of Mode A and Mode B in UEIBM, support to have a unified solution, i.e., alt 1: a dedicated SR for both mode-A and mode-B. Do not support to introduce a new UCI type.

Proposal 16: On beam report transmission procedure for UEIBM, regarding Event-2 for Mode-B, the first and the second channels can be from the different CC.

References

- 3GPP TSG RAN WG1 Meeting RAN1 #118 Chairman's Notes, August. 2024.