3GPP TSG RAN WG1 #118bis		R1-2408043

Hefei, China, October 14th – 18th, 2024

Source:	CATT

Title:	Discussion on 	SBFD TX/RX/measurement procedures

Agenda Item:	9.3.1

Document for:	Discussion and Decision

Introduction

For Rel-19 NR duplex evolution, the followings were agreed in last meeting for SBFD TX/RX/measurement procedures [1].

Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured Conclusion If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols. Agreement For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition. Agreement Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2. - Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols. Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH. - Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols. - Support separate configurations for DCI format 0_2 and other DCI formats as legacy - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions. FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc. Agreement Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc. Working Assumption For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. - For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC. - It is not applicable to PDCCH - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. - For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options: - Option 1: - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig. - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig. - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config. - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource]. - Option 2: - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation. - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation. Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1, - For PUSCH repetition type A with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type. - For CG PUSCH and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped. Agreement For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols - The numbers of PRBs are the same for PUSCH transmissions in SBFD symbols and PUSCH transmissions in non-SBFD symbols Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters. - Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively - FFS: No RRC impact - FFS: New MAC CE or reuse existing MAC CE - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively - FFS: Impact on MAC CE - Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS - FFS: Same offset or different offsets for all channels - FFS: RRC or MAC CE introduced to convey power offset - FFS whether the maximum number of closed power control loops is increased to be more than 2. - FFS: Whether/How to handle the case where UL power control is not associated within TCI state

In this contribution, further discussions on SBFD TX/RX/measurement procedures in Rel-19 NR duplex evolution are given.

Discussion

Semi-static indication of SBFD subbands locations

Based on RAN4’s agreement in [2], 10us and 3us were agreed to be the transition period length for FR1 and FR2 respectively between the non-SBFD slot and SBFD slot. From RAN1 point of view, two options can be considered on whether transition period is indicated to SBFD aware UEs.

- Option 1: Transition period is transparent to UE

- Option 2: Transition period is predefined or explicitly indicated to UE

For Option 1, it is up to gNB configuration/scheduling to make sure that there is no DL/UL transmissions during transition period. However, for SPS PDSCH, CG PUSCH and repetitions, it would impose additional gNB configuration/scheduling restrictions.

For Option 2, transition period is assumed between SBFD and non-SBFD symbols at UE side based on predefined rules or explicit indication from gNB. For example, if duration of transition period is defined in the spec, SBFD aware UEs can always assume transition period between SBFD and non-SBFD symbols according to predefined duration. Or gNB can explicitly indicate the presence and/or the duration of transition period to SBFD aware UEs. SBFD aware UEs would assume no transmissions and receptions during transition period which helps to increase gNB’s configuration/scheduling flexibility.

The issue of SBFD transition period is similar to TX-RX transition period in TDD system, i.e. reserving enough time for gNB transition. Therefore, existing scheme for gNB TX-RX transition period handling can be referred. The existing gNB TX-RX transition period in TDD system is configured in F symbol(s) and configured UL/DL transmission can be performed on the F symbol(s) if no DCI scheduling DL/UL transmission or no SFI indicating conflicted direction. It means that gNB implementation is used to avoid configured UL/DL transmission on the RX-TX transition period. From this point of view, Option 1 can be considered.

Proposal 1: No explicit indication of transition period between SBFD and non-SBFD symbols.

Enhancements to frequency resource allocation in SBFD symbols

For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4}, Option 1-1 was agreed in last meeting while the DMRS sequence mapping was FFS.

- Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only

  - FFS: DMRS sequence mapping

In existing specification, DMRS sequence is mapped to REs within CRBs allocated for PDSCH transmission. For Option 1-1, DMRS sequence should be mapped to REs within allocated CRBs within DL usable PRBs.

Proposal 2: DMRS sequence is mapped to REs within allocated CRBs within DL usable PRBs for Option 1-1.

The issue of PRG(s) overlapping with subband boundary was studied in SI. Based on the study, RAN1 agreed that, if the part of DL PRG inside the DL subband can be used, better scheduling flexibility and resource utilization can be achieved, however degraded channel estimation quality in the partial PRG is expected compared to a PRG due to limited RBs in the partial PRG.  Besides, UE complexity could increase if this feature is supported. Considering that partial RBG in case of unaligned boundaries between RBG and SBFD subbands is supported, when RBG size is same as PRG size, not supporting transmission in partial PRG means partial RBG is not supported, which contradicts the existing agreement. From perspectives of scheduling flexibility and resource utilization, we prefer to utilize the part of DL PRG inside the DL usable PRBs. Furthermore, at most two partial PRG within a BWP is supported in current specification. In order to avoid introducing additional UE complexity, partial RBG can be supported in case the number of partial RBGs in SBFD symbols does not exceed two.

Proposal 3: For PRG overlapping with subband boundary, the part of DL PRG inside the DL usable PRBs can be used in case the number of partial RBGs in SBFD symbols does not exceed two.

Transmissions and receptions across SBFD symbols and non-SBFD symbols

For transmissions and receptions across SBFD symbols and non-SBFD symbols in different slots, it was agreed that one of the following two configurations can be provided to UE:

- Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

- Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

Remaining issues for Configuration 1

For configuration 1, UE is only allowed to transmit or receive in one symbol type. It should be decided in which symbol type the transmissions/receptions are restricted to and what the UE behaviour is in SBFD symbols. For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, it was agreed to down-select from the following options:

- Option 1:

  - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig.

  - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig.

  - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config.

  - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource].

- Option 2:

  - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation.

  - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI.

  - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

  - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

Considering that Option 2 is more flexible since the valid symbol type can be changed by activation, and Option 2 does not bring additional effort by using the same mechanism as dynamically scheduled transmissions/receptions, Option 2 is proposed for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS.

Proposal 4: For configuration 1, the valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS is defined based on the symbol type of the first transmission/reception occasion after activation.

If the valid symbol type used for transmissions/receptions across SBFD symbols and non-SBFD symbols is determined as SBFD symbols, then UE behaviour in SBFD symbols should be defined. Similar as enhancements for single slot transmissions/receptions, enhancements for transmissions/receptions across multiple slots and valid only in SBFD symbols can be considered.

For example, for configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SPS PDSCH, CG PUSCH, PDSCH/PUSCH repetition, multi-PDSCH/PUSCH scheduled by a single DCI and TBoMS is SBFD symbols, then when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission, the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively. SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

For configuration 1 and frequency resource allocation Type 1, if the valid symbol type for SPS PDSCH, PDSCH repetition and multi-PDSCH scheduled by a single DCI is SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH, and assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. Existing RB indexing and VRB-to-PRB mapping are reused. The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only.

Proposal 5: For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SPS PDSCH, CG PUSCH, PDSCH/PUSCH repetition, multi-PDSCH/PUSCH scheduled by a single DCI and TBoMS is SBFD symbols, then

- when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission;

- the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively;

- SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

Proposal 6: For configuration 1 and frequency resource allocation Type 1, if the valid symbol type for SPS PDSCH, PDSCH repetition and multi-PDSCH scheduled by a single DCI is SBFD symbols, then

- only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH;

- assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping;

- existing RB indexing and VRB-to-PRB mapping are reused;

- the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only.

Remaining issues for Configuration 2

For configuration 2, both SBFD symbols and non-SBFD symbols can be used for transmissions/receptions across SBFD symbols and non-SBFD symbols in different slots. The UE behaviour when the frequency resources of transmissions/receptions overlap with RBs outside UL/DL usable PRBs in SBFD symbols should be decided.

For SPS PDSCH, PDSCH repetitions and multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, Option 5 was agreed as working assumption in last meeting, which means only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. We prefer to confirm the working assumption since rate matching is not new for PDSCH and the frequency resource in usable PRBs in SBFD symbols can be utilized when partial frequency resources overlapping with RBs outside UL usable PRBs.

The FFS issues are how to determine the TBS for each case. SPS PDSCH is for semi-persistent services, the packet size would not change with the symbol type, and hence TBS should be determined based on assigned PRBs. For a PDSCH repetition, only one TBS should be determined, considering gNB could know the valid resources for transmission and indicate a proper MCS, TBS can be determined based on assigned PRBs. For multi-PDSCH scheduled by a single DCI, TBS in each slot are independently determined, hence it is preferred that TBS for multi-PDSCH scheduled by a single DCI in each slot are independently determined based on available PRBs.

Proposal 7: Confirm the following working assumption and update the FFS as below:

Working Assumption

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

  - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only

  - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition.

  - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs in each slotonly.

For CG PUSCH, PUSCH repetitions, multi-PUSCH scheduled by a single DCI and TBoMS across SBFD symbols and non-SBFD symbols, it was agreed that single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols. The remaining issue is how to determine the RB offset(s). The following options can be considered:

- Option 1: RB offset is configured by RRC

- Option 2: RB offset is indicated by DCI

- Option 3: Several RB offsets are configured by RRC and one of the value is indicated/determined

- Option 4: RB offset is determined by the offset between the start RB of UL BWP and the start RB of UL subband

For Option 1, new RRC parameters would be introduced, and different RB offsets values can be configured for different cases. For example, different RB offsets values can be configured for different CG PUSCH configurations, and another RB offset value can be configured for other PUSCHs.

For Option 2, the DCI overhead would increase if new DCI field is introduced. Reuse existing DCI field would introduce some limitation on scheduling, e.g., if a new column is defined in TDRA table, the time domain scheduling flexibility would be impacted.

Option 3 introduces both RRC and DCI impact, but the DCI overhead is reduced compared with Option 2.

Based on Option 4, only one RB offset value is determined, this option does not introduce signalling overhead, but it is not flexible since only limited resource in UL symbols can be indicated to make sure that UE could get resource in UL usable PRBs in SBFD symbols.

Comparing these options, Option 1 is preferred since it does not impact DCI and different RB offsets values can be configured for different cases.

Proposal 8: For CG PUSCH, PUSCH repetitions, multi-PUSCH scheduled by a single DCI and TBoMS across SBFD symbols and non-SBFD symbols, RB offset(s) to determine frequency resource for SBFD symbols can be configured by RRC.

How to provide/determine configuration 1 or configuration 2

To provide configurations 1 or configuration 2 to SBFD-aware UE, the following options were proposed by companies:

- Option 1: SBFD-aware UE is provided with configuration 1 or configuration 2 by RRC

- Option 2: SBFD-aware UE is provided with configuration 1 or configuration 2 by RRC or DCI indication

For Option 1, new RRC signaling should be introduced to provide configuration 1 or configuration 2 to the SBFD-aware UE. It can be used for both configured transmissions/receptions and dynamic transmissions/receptions.

For Option 2, RRC configuration can be applied for configured transmissions/receptions and DCI indication can be applied for dynamic transmissions/receptions. For DCI indication, new DCI field can be introduced or reuse existing DCI field, the DCI overhead would increase if new DCI field is added and some limitation on scheduling can be introduced if reusing existing DCI field.

Hence, it is preferred that SBFD-aware UE is provided with configuration 1 or configuration 2 by RRC.

Proposal 9: SBFD-aware UE is provided with configuration 1 or configuration 2 by RRC.

For the granularity of the configuration, the following options were proposed by companies as summarized in [3]:

- Option 1: SBFD-aware UE is provided with configuration 1 or configuration 2 per UE

- Option 2: SBFD-aware UE is provided with configuration 1 or configuration 2 per channel/signal

- Option 3: SBFD-aware UE is provided with configuration 1 or configuration 2 per cell

- Option 4: SBFD-aware UE is provided with configuration 1 or configuration 2 per time domain type of across SBFD symbols and non-SBFD symbols

- Option 5: SBFD-aware UE is provided with configuration 1 or configuration 2 per link direction

Option 1 is simple but there is limitation in gNB scheduling/configuration, since the requirement of different channels/signals to adopt configuration 1 or configuration 2 may be different, and gNB may not be able to ensure that all UL channels could get proper resources in both SBFD symbols and non-SBFD symbols when configuration 2 is applied.

Option 2 is more flexible. Different channels/signals could have different configurations, then gNB have more flexibility in resource allocation. With this option, it is necessary to define how different configurations can be made for different channels/signals. For CG PUSCH, it is preferred that different CG PUSCH configurations have different configurations, then when the bandwidth of UL usable PRBs is limited, gNB could configure part of CG PUSCH configurations with configuration 2 and the other CG PUSCH configurations with configuration 1. Other PUSCHs could have a separate configuration. Similar for SPS PDSCH, SBFD-aware UE can be provided with configuration 1 or configuration 2 for each SPS PDSCH configuration.  Also other PDSCHs could have a separate configuration. For PUCCH, different PUCCH resources may have different requirement for configuration 1 or configuration 2, e.g., configuration 2 is proper for PUCCH with SPS HARQ-ACK/SR which could avoid UCI dropping and ensure low latency, and configuration 1 is more proper for PUCCH with CSI which has larger payload and occupy more frequency resources. Hence, SBFD-aware UE can be provided with configuration 1 or configuration 2 for each PUCCH resource.

In general, with Option 2, SBFD-aware UE can be provided with configuration 1 or configuration 2 per channel/signal as below:

- For  CG PUSCH, whether configuration 1 or 2 is applied  is configured per CG PUSCH configuration;

- For SPS PDSCH, whether configuration 1 or 2 is applied  is configured per SPS PDSCH configuration;

- For PUSCHs except for CG PUSCHs, whether configuration 1 or 2 is applied is configured per BWP;

- For PDSCHs except for SPS PDSCHs, whether configuration 1 or 2 is applied is configured per BWP;

- For PUCCH, whether configuration 1 or 2 is applied is configured per PUCCH resource.

Option 3/4/5 is not as flexible as Option 2, and can be implemented through the specific configuration of Option 2.

Hence, Option 2 is preferred, SBFD-aware UE is provided with configuration 1 or configuration 2 per channel/signal.

Proposal 10: SBFD-aware UE is provided with configuration 1 or configuration 2 per channel/signal as below:

- For  CG PUSCH, whether configuration 1 or 2 is applied  is configured per CG PUSCH configuration;

- For SPS PDSCH, whether configuration 1 or 2 is applied  is configured per SPS PDSCH configuration;

- For PUSCHs except for CG PUSCHs, whether configuration 1 or 2 is applied is configured per BWP;

- For PDSCHs except for SPS PDSCHs, whether configuration 1 or 2 is applied is configured per BWP;

- For PUCCH, whether configuration 1 or 2 is applied is configured per PUCCH resource.

In addition, whether configuration 2 is subject to UE capability can be discussed. Considering that supporting configuration 2 would introduce more complexity, we support that configuration 2 is defined as an optional UE capability for SBFD aware UEs.

Proposal 11: For transmissions and receptions across SBFD symbols and non-SBFD symbols in different slots, define configuration 2 as an optional UE capability for SBFD aware UEs.

CSI report with CSI-RS instances in SBFD and non-SBFD symbols in different slots

For CSI report associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots, the following options for SBFD-aware UEs were agreed for discussion:

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

  - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only.

    - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

For Option A, new RRC signalling can be added in CSI-ReportConfig to indicate whether it is applicable to SBFD symbols or for non-SBFD symbols. Accordingly, only CSI-RS transmission occasions within the applicable symbol type are used for CSI derivation.

For Option B, following Rel-18 NES CSI reporting framework, two sub-configurations can be configured in one CSI-ReportConfig, where each sub-configuration can be configured with a CSI-RS resource for channel measurement. The symbol type corresponding to each sub-configuration can be predefined, e.g. the first sub-configuration is for non-SBFD symbols and the second sub-configuration is for SBFD symbols. For interference measurement, considering that interference level can be different in SBFD symbols and non-SBFD symbols, separate CSI-IM and NZP CSI-RS for different symbol types should be supported. Separate CSI-IM resources for interference measurements can be supported based on existing resource-wise association between CSI-RS resources for channel measurement and CSI-IM resources for interference measurements. However, only one NZP CSI-RS resource can be configured in the resource set for interference measurement as in current specification, which limits the configurability of CSI reports when the gNB needs interference measurements based on NZP CSI-RS for different symbol types. Enhancements can be considered to support two NZP CSI-RS resources contained in the resource set for interference measurement for SBFD UE.

Observation 1: If Rel-18 NES CSI reporting framework is reused for CSI reporting for different symbols types, only one NZP CSI-RS resource can be configured in the resource set for interference measurement.

Proposal 12: For CSI report associated with CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots, support Option A and Option B.

- For Option A, new RRC signalling can be added in CSI-ReportConfig to indicate whether only CSI-RS transmission occasions in SBFD symbols or in non-SBFD symbols are used for CSI derivation;

- For Option B, the following enhancements can be supported:

  - The first sub-configuration is for non-SBFD symbols and the second sub-configuration is for SBFD symbols;

  - Support two NZP CSI-RS resources in the resource set for interference measurement in one CSI-ReportConfig.

Separate configurations for SBFD and non-SBFD symbols

In this section, separate resources, FH parameters, UL power control parameters and/or beam/spatial relation for SRS, PUCCH and PUSCH are discussed.

Separate UL power control parameters and/or spatial relation

It was agreed that separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols are supported for single TRP scenario. A remaining issue is that whether separate UL power control is supported based on UL spatial relation info framework and/or unified TCI state framework.

In Rel-17, unified TCI state framework was introduced, which uses a unified mechanism to determine the beam and uplink power for all channels and simplifies the original UL spatial relation info framework. It is the most likely way to be adopted in actual deployment, and is also the design direction of 6G in the future. Hence, we prefer to support separate UL power control for FR2 based on unified TCI state framework.

Proposal 13: For a single TRP scenario, support separate UL power control for FR2 based on unified TCI state framework.

Considering that the interference is different in SBFD symbols and non-SBFD symbols, it is likely that the interference caused by inter-gNB CLI and/or clutter reflection in SBFD symbols from one direction is more severe than the interference from the same direction in UL symbols. Hence it is preferred to apply different beams for UL transmission in different symbol types. Similarly, different beams for DL receptions in SBFD symbols and non-SBFD symbols can be applied. Based unified TCI framework, separate unified TCI states for SBFD symbols and non-SBFD symbols can be supported.

Proposal 14: Support separate unified TCI states for transmissions/receptions in SBFD symbols and non-SBFD symbols respectively.

Based on unified TCI state framework, the following options were agreed to be down-selected to support separate configurations of UL power control parameters:

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

  - FFS: No RRC impact

  - FFS: New MAC CE or reuse existing MAC CE

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

  - FFS: Impact on MAC CE

- Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols

  - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS

    - FFS: Same offset or different offsets for all channels

  - FFS: RRC or MAC CE introduced to convey power offset

For Option 1, two TCI states for one TCI codepoint are activated by MAC CE. As shown in Figure 1, one TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively. DCI indicates one of the TCI codepoints to determine two unified TCI states for SBFD symbols and non-SBFD symbols. UE determines unified TCI state including UL power control parameters based on symbol type of the UL transmission. There is no RRC impact with this option. New MAC CE can be defined to active two TCI states for one TCI codepoint.

Figure 1: Separate TCI states for non-SBFD symbols and non-SBFD symbols

Option 2 is illustrated in Figure 2, where two sets of UL power control parameters for SBFD symbols and non-SBFD symbols are configured for one unified TCI state. UL power control parameters can be determined based on symbol type of the UL transmission.

Figure 2: Separate UL power control parameters for SBFD symbols and non-SBFD symbols

Option 3 introduces new RRC or MAC CE to indicate power offset for transmission in SBFD symbols for PUSCH, PUCCH, and SRS.

Option 1 supports separate unified TCI states including beam related parameters and UL power control parameters for SBFD and non-SBFD symbols. Option 2 and Option 3 support separate UL power control for SBFD and non-SBFD symbols. Although separate beam for SBFD and non-SBFD symbols of Option 2 and Option 3 can be realized by additional schemes, e.g., unified TCI states for SBFD symbols is indicated by DCI transmitted in SBFD symbols and unified TCI states for non-SBFD symbols is indicated by DCI transmitted in non-SBFD symbols, we think it is less flexible and efficient than Option 1, in addition extra specification effort is required, hence it is preferred to adopt Option 1.

Proposal 15: For separate PUCCH power control parameters for SBFD and non-SBFD symbols, one TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively. Unified TCI states including UL power control parameters are determined based on symbol type of the UL transmission.

For the maximum number of closed power control loops, it was agreed in NR Rel-15 that at most 2 closed loop processes can be configured for UE, and different closed loops can be used for the following aspects:

- PUSCH beam indication (if present) for grant-based PUSCH

- slot sets (if supported)

- grant-free PUSCH and grant based PUSCH

- for two uplinks of SUL band combination

Slot sets are not defined in NR, and different symbol types are similar as different slot sets in power control. In addition, complexity of UE implementation is not increased if up to 2 closed power control loops is maintained. Hence, it is proposed that the maximum number of closed power control loops is not increased, i.e. up to 2 closed loops as legacy.

Proposal 16: For separate PUCCH power control parameters for SBFD and non-SBFD symbols, the maximum number of closed power control loops is not increased, i.e. up to 2 closed loops as legacy.

For the case unified TCI state is configured and UL power control parameters are not configured within unified TCI state, UE would use the UL power control parameters configured in UL BWP associated with unified TCI state. In this case, to support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols, separate UL power control parameters for SBFD symbols and non-SBFD symbols can be configured in UL BWP associated with unified TCI state.

Proposal 17: For separate PUCCH power control parameters for SBFD and non-SBFD symbols, if unified TCI state is configured and UL power control parameters are not configured within unified TCI state, separate UL power control parameters for SBFD symbols and non-SBFD symbols can be configured in UL BWP associated with unified TCI state.

For mTRP scenario, separate SRS resource sets are configured corresponding to different TRPs, and power control parameters and beam/spatial relations are configured separately for each SRS resource set. The same mechanism can be applied to SRS transmissions in SBFD symbols and non-SBFD symbols in different slots, where two SRS resource sets can be configured for SBFD symbols and non-SBFD symbols respectively.

Proposal 18: Support separate SRS resource sets for SBFD symbols and non-SBFD symbols.

Separate PUCCH resources

For PUCCH resources, separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols were agreed in last meeting. There are two FFS issues for separate PUCCH resources in SBFD symbols and non-SBFD symbols.

The first issue is whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2. The main point for this issues is whether PUCCH intra-slot frequency hopping should be separately configured for SBFD symbols or non-SBFD symbols. Considering that the interference can be different in SBFD symbols and non-SBFD symbols, and PUCCH frequency resource may be different in SBFD symbols and non-SBFD symbols, it is preferred to enable separate PUCCH intra-slot frequency hopping for SBFD symbols or non-SBFD symbols for both Configuration 1 and 2. To realize separate PUCCH intra-slot frequency hopping, the following options can be considered:

- Option 1: separate configurations of intraSlotFrequencyHopping for SBFD symbols or non-SBFD symbols;

- Option 2: existing intraSlotFrequencyHopping is only applied for non-SBFD symbols, and whether frequency hopping for SBFD symbols is enabled is determined based on whether secondHopPRB for SBFD symbols is configured;

- Option 3: if intraSlotFrequencyHopping is not configured, PUCCH intra-slot frequency hopping is not enabled for both SBFD symbols and non-SBFD symbols, otherwise whether PUCCH intra-slot frequency hopping is enabled for a symbol type is determined based on whether secondHopPRB is configured for the corresponding symbol type;

- Option 4: if intraSlotFrequencyHopping is not configured, PUCCH intra-slot frequency hopping is not enabled for both SBFD symbols and non-SBFD symbols, otherwise PUCCH intra-slot frequency hopping is enabled for both SBFD symbols and non-SBFD symbols. It should be noted that with this option, separate PUCCH intra-slot frequency hopping for SBFD symbols and non-SBFD symbols can be realized by different configurations of startingPRB and/or secondHopPRB for SBFD symbols and non-SBFD symbols. For example, when startingPRB and secondHopPRB for a symbol type are the same, then PUCCH intra-slot frequency hopping is not enabled for the symbol type.

All of Option 1/2/3/4 support separate configuration of PUCCH intra-slot frequency hopping for SBFD symbols and non-SBFD symbols. Considering that Option 4 does not introduce additional RRC parameters and it is also consistent with the existing PUCCH frequency hopping scheme, Option 4 is preferred.

Observation 2: Separate PUCCH intra-slot frequency hopping for SBFD symbols and non-SBFD symbols can be realized by different configurations of startingPRB and/or secondHopPRB for SBFD symbols and non-SBFD symbols.

Proposal 19: For PUCCH resource configuration for SBFD aware UEs, configuration of intraSlotFrequencyHopping is applied for both SBFD symbols and non-SBFD symbols.

The second issue is how to determine the UE behaviour when single startingPRB and/or secondHopPRB is configured for a SBFD aware UE. To avoid introducing additional RRC parameters, the single configured startingPRB and secondHopPRB can be applied for non-SBFD symbols by default. Then, the following can be considered for UE behavior in SBFD symbols:

- Option 1: PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected

- Option 2: configurations for non-SBFD symbols are applied for SBFD symbols

Considering that separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols has been supported, there is no reason to support the gNB configure single startingPRB and secondHopPRB for non-SBFD symbol and make UE use this resource in another symbol type. In addition, gNB may not be able to ensure that the PUCCH frequency resources for non-SBFD symbols are valid for SBFD symbols. Hence, Option 1 is preferred.

Proposal 20: For SBFD aware UEs, a PUCCH resource with single configured startingPRB and/or secondHopPRB is applied for non-SBFD symbols by default, and PUCCH transmissions in SBFD symbols for this PUCCH resource is not expected.

Separate PUSCH FH parameters

For PUSCH frequency hopping, separate configurations of FH offsets/FH offset lists for SBFD symbols and non-SBFD symbols were agreed, the first FFS issue is whether the number of bits used to indicate FH offset in DCI is determined as legacy.

In current specification, the number of bits used to indicate FH offset is determined based on the number of FH offset values configured by higher layer parameter frequencyHoppingOffsetLists, while the higher layer parameter frequencyHoppingOffsetLists contains two or four values which is determined based on the size of active UL BWP as referenced below.

TS38.212 - Frequency domain resource assignment - number of bits determined by the following: - bits if neither of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, where is defined in clause 7.3.1.0 - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] TS38.214 For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1/0_3 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config. - When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant. - When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

Assuming that the number of FH offset values configured in FH offset list for SBFD symbols is determine based on the size of UL usable PRBs, then it is possible that the number of bits required to indicate the FH offset for non-SBFD symbols is different from the number of bits required to indicate the FH offset for SBFD symbols. However, DCI size should not change based on the scheduled symbol type, and considering that the bandwidth of UL usable PRBs would not be larger than the bandwidth of UL BWP, the maximum number of FH offset values in FH offset list should be determined based on the bandwidth of UL BWP. Therefore, it is preferred that the number of bits used to indicate FH offset in DCI is determined as legacy.

Proposal 21: For SBFD aware UEs, the number of bits used to indicate FH offset in DCI is determined as legacy.

The second FFS issue is how to define the UE behaviour when FH offset/FH offset list is not provided for SBFD symbols. Similar as PUCCH, we prefer to support separate PUSCH frequency hopping for SBFD symbols and non-SBFD symbols.

For type-1 CG PUSCH, PUSCH frequency hopping is performed if FH offset is provided, otherwise no PUSCH frequency hopping is performed. Hence, whether PUSCH frequency hopping is performed for SBFD symbols can be determined based on whether FH offset is provided for SBFD symbols.

For type-2 CG PUSCH and dynamic PUSCH, PUSCH frequency hopping is performed if the frequency hopping field in a corresponding detected DCI format is set to 1, otherwise no PUSCH frequency hopping is performed. For single slot PUSCH, separate PUSCH frequency hopping for SBFD symbols and non-SBFD symbols can be realized by DCI indication. But for PUSCH across SBFD symbols and non-SBFD symbols in different slots, separate PUSCH frequency hopping cannot be realized by the 1-bit indication in DCI. In addition, possible FH offset values in FH offset list does not include 0, separate PUSCH frequency hopping for SBFD symbols and non-SBFD symbols cannot be realized by configuring different FH offset list. Considering that FH offset list may not be provided for SBFD symbols, PUSCH frequency hopping can be disabled in case FH offset list is not provided for SBFD symbols.

Proposal 22: For SBFD aware UEs, when FH offset/FH offset list is not provided for SBFD symbols, PUSCH frequency hopping is disabled for PUSCH transmissions in SBFD symbols.

For PUSCH frequency hopping in SBFD symbols, it was proposed to update the PUSCH frequency hopping formula according to one of the following options:

- Option 1: RBUL SB start+(RBstart+RBoffset)modNBWPsizeNUL SBsize

- Option 2: RBUL SB start+(RBstart-RBUL SB start+RBoffset)modNBWPsizeNUL SBsize

- Option 3: (RBstart+RBoffset)modNBWPsizeNUL SBsize

  - For Option 3, the above result provides the RB offset with reference to the start of UL usable PRB

- Option 4: (RBstart-RBUL SB start+RBoffset)modNBWPsizeNUL SBsize

  - For Option 4, the above result provides the RB offset with reference to the start of UL usable PRB

Where RBUL SB start is the starting PRB index of the UL usable PRB within UL BWP and NUL SBsize is the size of UL usable PRB.

It should be noted that these updated formulas are used for determine the PUSCH starting RB of the second hop. For Option 1 and Option 2, the reference point of the result is the starting PRB index of the UL BWP, which is the same as the definition for PUSCH starting RB of the first hop. Besides, RBoffset in Option 2 is more representative of the offset between the two hops, since the starting PRB index of the first hop and the starting PRB index of the second hop are same when RBoffset=0.

For Option 3 and Option 4, the reference point of the result is the starting PRB index of the UL usable PRB, but its essence is the same as Option 1 and Option 2 respectively. Different descriptions are needed in specification to define the reference point of PUSCH starting RB of the second hop for different cases, e.g., intra-slot frequency hopping, inter-slot frequency hopping without DMRS bundling, inter-slot frequency hopping with DMRS bundling and inter-repetition frequency hopping.

Considering that the reference point of Option 2 is the same as the definition for PUSCH starting RB of the first hop, the description can be simpler and clearer. In addition, RBoffset in Option 2 is more representative of the offset between the two hops. Hence, it is proposed to adopt Option 2 for PUSCH frequency hopping.

Proposal 23: For PUSCH frequency hopping in SBFD symbols, update the PUSCH frequency hopping formula to determine the second hop as follow:

- RBUL SB start+(RBstart-RBUL SB start+RBoffset)modNBWPsizeNUL SBsize

Collision handling in SBFD symbols

For SBFD operation, half-duplex operation is assumed at UE side. How to resolve the collision between transmissions and receptions, and order of different collision types are discussed in this section.

Collision between transmissions and receptions

The following two schemes were agreed to be considered to resolve the issue of time domain conflict of transmission and reception in the same SBFD symbol for SBFD aware UE:

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Option 1 follows the design principles of legacy NR UEs in flexible symbols and it is proposed to be supported.

For Option 2, the transmission direction can be indicated to UE by RRC or DCI, reusing TDD-UL-DL-ConfigDedicated or dynamic SFI. The indicated link directions can be different for different SBFD-aware UEs. The potential benefits of Option 2 are UE complexity reduction and/or UE power consumption reduction. From specification point of view, both options can be supported.

Proposal 24: For SBFD aware UEs link direction determination, support Option 1 and Option 2 if the benefits of Option 2 are agreed.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

For Option 1, six collision cases were agreed for discussion. Collision handling rules should be defined for each collision case.

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 6: Dynamic or semi-static DL vs. valid RO

It was agreed to reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum for Case 1 to Case 4.

For Case 3, it was agreed that an SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol; and an SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol. For cell-specifically configured DL reception, PDCCH in Type-0B CSS set should also be included in addition to PDCCH in Type-0/0A/1/2 CSS set.

Proposal 25: If link direction indication is not supported nor provided for a SBFD symbol, cell-specifically configured DL reception defined in collision Case 3 refers to PDCCH in Type-0/0A/0B/1/2 CSS set.

For Case 5, it was agreed in RAN1#117 to re-use the existing collision handling principle for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission.

The remaining issue is whether/how UL subband in SSB symbols can be used. It is obvious that if only DL receptions within DL usable PRBs are allowed for SBFD aware UEs in SSB symbols configured with SBFD subbands, it would cause reduced resource utilization efficiency. The percentage of wasted resources/DL total resources is shown in Table 1. It can be observed that more than 5% DL resources may be wasted for short SSB periods of 5 and 10ms.

Table 1: Percentage of wasted resources/DL total resources

| SSB configuration/period | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- | --- |
| Case A (L=4) | 5.71% | 2.86% | 1.43% | 0.71% | 0.48% | 0.36% |
| Case A (L=8) | 11.43% | 5.71% | 2.86% | 1.43% | 0.95% | 0.71% |
| Case B (L=4) | 2.86% | 1.43% | 0.71% | 0.36% | 0.24% | 0.18% |
| Case B (L=8) | 5.71% | 2.86% | 1.43% | 0.71% | 0.48% | 0.36% |
| Case C (L=4) | 2.86% | 1.43% | 0.71% | 0.36% | 0.24% | 0.18% |
| Case C (L=8) | 5.71% | 2.86% | 1.43% | 0.71% | 0.48% | 0.36% |
| Case D (L=64) | - | 5.71% | 2.86% | 1.43% | 0.95% | 0.71% |
| Case E (L=64) | - | 2.86% | 1.43% | 0.71% | 0.48% | 0.36% |
| Assumptions: 1. All SSB symbols are configured as SBFD symbols; 2. UL subband occupies 20% carrier bandwidth; 3. Frame structure is XXXXU. |  |  |  |  |  |  |

Observation 3: If only DL receptions within DL usable PRBs are allowed for SBFD aware UEs in SSB symbols, more than 5% DL resources may be wasted for short SSB periods of 5 and 10ms.

Alternatively, DL receptions within UL usable PRBs can be allowed, i.e. SSB symbols fallback to full DL symbols. However, the problem is that if only SSB symbols fallback to full DL symbols, there are too many transition points. Therefore, if DL receptions within DL usable PRBs are allowed, a longer duration including SSB symbols and other adjacent symbols should be considered for fallback, e.g. a slot consisting of SSB symbols.

Proposal 26: For SSB symbols configured with SBFD subbands, discuss and decide whether SSB symbols can fall back to full DL symbols. If supported, a longer duration including SSB symbols and other adjacent symbols should be considered for fallback.

For case 6, collision handling rule for half-duplex FDD RedCap UE can be reused. For example, for PDCCH order triggered PRACH, it can be considered as dynamically scheduled UL transmission. For higher layer triggered PRACH, whether prioritize DL reception or PRACH transmission can be up to UE implementation.

Proposal 27: For SBFD aware UEs, if UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any), the following collision handling rules for HD-FDD RedCap UE is reused:

- UE does not expect collision between PRACH triggered by PDCCH order and DL channel/signal with scheduling DCI in the same SBFD symbol;

- For collision between PRACH triggered by PDCCH order and PDCCH, PDSCH, CSI-RS or DL PRS configured by higher layers in the same SBFD symbol, UE does not receive the PDCCH, PDSCH, CSI-RS or DL PRS configured by higher layers;

- For collision between PRACH triggered by higher layer and DL channel/signal in the same SBFD symbol, whether to prioritize DL channel/signal or PRACH transmission is up to UE implementation.

Order of different collision types

In general, there are four types of collision for a SBFD aware UE as below, the processing order for these collision types should be considered:

1) Collision between transmissions/receptions and unusable time/frequency resources;

2) Collision between transmissions with same transmission direction;

3) Collision between transmissions and receptions;

4) Collision between transmissions/receptions and the link direction indicated by gNB (if supported);

Figure 3: Different collision types in SBFD symbol

Collision type 4 has not been agreed yet, hence we discuss the first three collision types. As shown in Figure 3, there are three kinds of collision in slot n. The collision between PUSCH and DL usable PRBs is the first kind of collision, the collision between HARQ-ACK and PUSCH is the second kind of collision, and the collision between HARQ-ACK and PDSCH-1 or between PUSCH and PDSCH-2 is the third kind of collision. It is necessary to determine the execution order of the solutions to resolve these collisions. Different execution order may lead to different channels to be transmitted at last as shown below:

- Case 1: If the first collision type is resolved first, PUSCH is dropped at the first step, then the collision between HARQ-ACK and PDSCH-1 should be resolved, assuming dynamic HARQ-ACK is prioritized over PDSCH-1, PUCCH with HARQ-ACK and PDSCH-2 can be transmitted at last in this case.

- Case 2: If the second collision type is resolved first, the HARQ-ACK would be multiplexed on PUSCH, then there are still two kinds of collision, collision between PUSCH and DL usable PRBs, and collision between PUSCH and PDSCH-2.

  - Case 2-1: assuming the collision between PUSCH with HARQ-ACK and PDSCH-2 should be resolved first and dynamic HARQ-ACK is prioritized over PDSCH-2, then PDSCH-2 should be dropped, then the PUSCH with HARQ-ACK is dropped for collision with DL usable PRBs, only PDSCH-1 could be transmitted at last.

  - Case 2-2: assuming the collision between PUSCH with HARQ-ACK and DL usable PRBs should be resolved first, then PUSCH with HARQ-ACK should be dropped, PDSCH-1 and PDSCH-2 could be transmitted at last.

- Case 3: If the collision between downlink and uplink is resolved first, then for collision between PDSCH-1 and HARQ-ACK, assuming dynamic HARQ-ACK is prioritized over PDSCH-1, then PDSCH-1 should be dropped, for collision between PDSCH-2 and PUSCH, assuming PUSCH is prioritized over PDSCH-2, then PDSCH-2 should be dropped. Then there are still two kinds of collision, collision between PUSCH and DL usable PRBs, and collision between PUSCH and HARQ-ACK.

  - Case 3-1: assuming the collision between PUSCH with HARQ-ACK should be resolved first and HARQ-ACK is multiplexed on PUSCH, then the PUSCH with HARQ-ACK is dropped for collision with DL usable PRBs, no channel could be transmitted at last.

  - Case 3-2: assuming the collision between PUSCH and DL subband should be resolved first, then PUSCH should be dropped, only PUCCH with HARQ-ACK could be transmitted at last.

It can be seen that gNB and UE may have different understanding on which channel could be transmitted based on different understanding on the order of resolution of different collision types, hence it is proposed to study collision handling rules for the order of different collision types.

Proposal 28: For SBFD aware UEs, study the order of collision handling of the following collision types:

- Type 1: Collision between transmissions/receptions and unusable time/frequency resources;

- Type 2: Collision between transmissions with same transmission direction;

- Type 3: Collision between transmissions and receptions;

- Type 4: Collision between transmissions/receptions and the link direction indicated by gNB (if supported).

Conclusion

In this contribution, we discuss SBFD TX/RX/measurement procedures and give the following observations and proposals.

Observation 1: If Rel-18 NES CSI reporting framework is reused for CSI reporting for different symbols types, only one NZP CSI-RS resource can be configured in the resource set for interference measurement.

Observation 2: Separate PUCCH intra-slot frequency hopping for SBFD symbols and non-SBFD symbols can be realized by different configurations of startingPRB and/or secondHopPRB for SBFD symbols and non-SBFD symbols.

Observation 3: If only DL receptions within DL usable PRBs are allowed for SBFD aware UEs in SSB symbols, more than 5% DL resources may be wasted for short SSB periods of 5 and 10ms.

Proposal 1: No explicit indication of transition period between SBFD and non-SBFD symbols.

Proposal 2: DMRS sequence is mapped to REs within allocated CRBs within DL usable PRBs for Option 1-1.

Proposal 3: For PRG overlapping with subband boundary, the part of DL PRG inside the DL usable PRBs can be used in case the number of partial RBGs in SBFD symbols does not exceed two.

Proposal 4: For configuration 1, the valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS is defined based on the symbol type of the first transmission/reception occasion after activation.

Proposal 5: For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SPS PDSCH, CG PUSCH, PDSCH/PUSCH repetition, multi-PDSCH/PUSCH scheduled by a single DCI and TBoMS is SBFD symbols, then

- when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission;

- the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively;

- SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

Proposal 6: For configuration 1 and frequency resource allocation Type 1, if the valid symbol type for SPS PDSCH, PDSCH repetition and multi-PDSCH scheduled by a single DCI is SBFD symbols, then

- only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH;

- assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping;

- existing RB indexing and VRB-to-PRB mapping are reused;

- the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only.

Proposal 7: Confirm the following working assumption and update the FFS as below:

Working Assumption

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

  - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only

  - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition.

  - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs in each slotonly.

Proposal 8: For CG PUSCH, PUSCH repetitions, multi-PUSCH scheduled by a single DCI and TBoMS across SBFD symbols and non-SBFD symbols, RB offset(s) to determine frequency resource for SBFD symbols can be configured by RRC.

Proposal 9: SBFD-aware UE is provided with configuration 1 or configuration 2 by RRC.

Proposal 10: SBFD-aware UE is provided with configuration 1 or configuration 2 per channel/signal as below:

- For  CG PUSCH, whether configuration 1 or 2 is applied  is configured per CG PUSCH configuration;

- For SPS PDSCH, whether configuration 1 or 2 is applied  is configured per SPS PDSCH configuration;

- For PUSCHs except for CG PUSCHs, whether configuration 1 or 2 is applied is configured per BWP;

- For PDSCHs except for SPS PDSCHs, whether configuration 1 or 2 is applied is configured per BWP;

- For PUCCH, whether configuration 1 or 2 is applied is configured per PUCCH resource.

Proposal 11: For transmissions and receptions across SBFD symbols and non-SBFD symbols in different slots, define configuration 2 as an optional UE capability for SBFD aware UEs.

Proposal 12: For CSI report associated with CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots, support Option A and Option B.

- For Option A, new RRC signalling can be added in CSI-ReportConfig to indicate whether only CSI-RS transmission occasions in SBFD symbols or in non-SBFD symbols are used for CSI derivation;

- For Option B, the following enhancements can be supported:

  - The first sub-configuration is for non-SBFD symbols and the second sub-configuration is for SBFD symbols;

  - Support two NZP CSI-RS resources in the resource set for interference measurement in one CSI-ReportConfig.

Proposal 13: For a single TRP scenario, support separate UL power control for FR2 based on unified TCI state framework.

Proposal 14: Support separate unified TCI states for transmissions/receptions in SBFD symbols and non-SBFD symbols respectively.

Proposal 15: For separate PUCCH power control parameters for SBFD and non-SBFD symbols, one TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively. Unified TCI states including UL power control parameters are determined based on symbol type of the UL transmission.

Proposal 16: For separate PUCCH power control parameters for SBFD and non-SBFD symbols, the maximum number of closed power control loops is not increased, i.e. up to 2 closed loops as legacy.

Proposal 17: For separate PUCCH power control parameters for SBFD and non-SBFD symbols, if unified TCI state is configured and UL power control parameters are not configured within unified TCI state, separate UL power control parameters for SBFD symbols and non-SBFD symbols can be configured in UL BWP associated with unified TCI state.

Proposal 18: Support separate SRS resource sets for SBFD symbols and non-SBFD symbols.

Proposal 19: For PUCCH resource configuration for SBFD aware UEs, configuration of intraSlotFrequencyHopping is applied for both SBFD symbols and non-SBFD symbols.

Proposal 20: For SBFD aware UEs, a PUCCH resource with single configured startingPRB and/or secondHopPRB is applied for non-SBFD symbols by default, and PUCCH transmissions in SBFD symbols for this PUCCH resource is not expected.

Proposal 21: For SBFD aware UEs, the number of bits used to indicate FH offset in DCI is determined as legacy.

Proposal 22: For SBFD aware UEs, when FH offset/FH offset list is not provided for SBFD symbols, PUSCH frequency hopping is disabled for PUSCH transmissions in SBFD symbols.

Proposal 23: For PUSCH frequency hopping in SBFD symbols, update the PUSCH frequency hopping formula to determine the second hop as follow:

- RBUL SB start+(RBstart-RBUL SB start+RBoffset)modNBWPsizeNUL SBsize

Proposal 24: For SBFD aware UEs link direction determination, support Option 1 and Option 2 if the benefits of Option 2 are agreed.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Proposal 25: If link direction indication is not supported nor provided for a SBFD symbol, cell-specifically configured DL reception defined in collision Case 3 refers to PDCCH in Type-0/0A/0B/1/2 CSS set.

Proposal 26: For SSB symbols configured with SBFD subbands, discuss and decide whether SSB symbols can fall back to full DL symbols. If supported, a longer duration including SSB symbols and other adjacent symbols should be considered for fallback.

Proposal 27: For SBFD aware UEs, if UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any), the following collision handling rules for HD-FDD RedCap UE is reused:

- UE does not expect collision between PRACH triggered by PDCCH order and DL channel/signal with scheduling DCI in the same SBFD symbol;

- For collision between PRACH triggered by PDCCH order and PDCCH, PDSCH, CSI-RS or DL PRS configured by higher layers in the same SBFD symbol, UE does not receive the PDCCH, PDSCH, CSI-RS or DL PRS configured by higher layers;

- For collision between PRACH triggered by higher layer and DL channel/signal in the same SBFD symbol, whether to prioritize DL channel/signal or PRACH transmission is up to UE implementation.

Proposal 28: For SBFD aware UEs, study the order of collision handling of the following collision types:

- Type 1: Collision between transmissions/receptions and unusable time/frequency resources;

- Type 2: Collision between transmissions with same transmission direction;

- Type 3: Collision between transmissions and receptions;

- Type 4: Collision between transmissions/receptions and the link direction indicated by gNB (if supported).

Reference

- Chairman's Notes RAN1#118, 3GPP TSG RAN WG1 Meeting #118, Maastricht, NL, August 19th – 23rd, 2024.

- R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung, WG4 Meeting #112，Maastricht, Netherlands, 19th – 23rd August, 2024.

- R1-2407220, Summary #3 of SBFD TX/RX/measurement procedures, Moderator (CATT), 3GPP TSG RAN WG1 Meeting #118, Maastricht, NL, August 19th – 23rd, 2024.