3GPP TSG RAN WG1 #118bis	R1-2408082

Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UE capability alignment for two TAs in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-10-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is specified in the current version that if the UE is not provided enableSTx2PofMDCI and operates with two TAGs on a serving cell, the UE does not expect transmissions associated with different TAGs to overlap unless the UE indicates XYZ; if the UE indicates XYZ, the UE reduces in duration a latter transmission using a first TAG to avoid overlapping with a former transmission using a second TAG. According to the agreement endorsed in RAN1#113, XYZ represents the UE capability of supporting the optional feature that the overlapping duration of the later of the two UL transmissions is reduced. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, - for the baseline feature, the UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) - as an optional feature, the overlapping duration of the later of the two UL transmissions is reduced. - FFS: for the optional feature, whether or not the overlapping duration needs to be specified as 1 (in case 2) or 2 (in case 1) OFDM symbols where - Case 1 applies when UE is capable of supporting MRTD > CP, SCS=60 kHz and frequency range is FR1. - Case 2 applies in all other cases. Since the corresponding UE capability has been captured in TS 38.306 as follows, the placeholder of XYZ should be replaced with overlapUL-TransReduction accordingly. overlapUL-TransReduction-r18 Indicates whether the UE supports reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is not configured with UL STx2P for multi-DCI based multi-TRP operation with two TA enhancement. A UE supporting this feature shall indicate support of multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18. NOTE: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions).. Furthermore, according to TS 38.331, the RRC parameter tag2-Id is contained in tag2 which indicates the second TAG information for the serving cell. tag2-Id Timing Advance Group ID, as specified in TS 38.321 [3], which this cell or set of TCI-States of this cell are associated with.. The temporary terminology corresponding to tag2-Id has already been captured in TS 38.213 clause 8.3 by editor, and then, per RRC update, revisions on the RRC parameter should be made for alignment. |
| Summary of change: |  | Regarding the UE capability of supporting the optional feature that the overlapping duration of the later of the two UL transmissions is reduced, replace XYZ with overlapUL-TransReduction to align with the UE capability definition is TS 38.306. Replace RRC parameter [twoTAGs] to tag2-Id. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Presence of misalignment of the UE capability of supporting the optional feature that the overlapping duration of the later of the two UL transmissions is reduced and misalignment of RRC parameter for two TAs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has no isolated impact on network and UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of this CR. |  |  |  |  |  |  |  |  |

4.2	Transmission timing adjustments

<Unchanged part is omitted>

For operation with single TAG on a serving cell, if two adjacent slots overlap due to a TA command or due to update of NTA,adjUE or NTA,adjcommon, when applicable, the latter slot is reduced in duration relative to the former slot. The UE does not change NTA during an actual time domain window for a PUSCH or a PUCCH transmission [6, TS 38.214]. If the UE is not provided enableSTx2PofMDCI and operates with two TAGs on a serving cell, the UE does not expect transmissions associated with different TAGs to overlap unless the UE indicates overlapUL-TransReduction; if the UE indicates overlapUL-TransReduction, the UE reduces in duration a latter transmission using a first TAG to avoid overlapping with a former transmission using a second TAG.

<Unchanged part is omitted>

8.2	Random access response - Type-1 random access procedure

<Unchanged part is omitted>

If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for the SpCell [11, TS 38.321], the UE may assume that the PDCCH that includes the DCI format 1_0 and the PDCCH order have same DM-RS antenna port quasi co-location properties. If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for a secondary cell, or if the UE is configured with tag2-Id for the SpCell and the CORESET where the UE receives the PDCCH order that triggers a contention-free random access procedure for the SpCell is not associated with the physical cell ID for the serving cell, the UE may assume the DM-RS antenna port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set for receiving the PDCCH that includes the DCI format 1_0 and the PDSCH scheduled by the DCI format 1_0.

<Unchanged part is omitted>