3GPP TSG RAN WG1 Meeting #118bis	R1-2408111

Hefei, China, October 14th – 18th, 2024

Source:	Fujitsu

Title:	Discussion on UL-only mTRP operation

Agenda item:	9.2.4

Document for:	Discussion and Decision

- Introduction

In RAN1 #118 meeting, the following agreements were achieved on asymmetric DL sTRP/UL mTRP operation [1].

Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not. Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR. Agreement About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, down-select one from the following Alts in RAN1#118bis: - Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is appliable to any rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise - Alt2: X = 44 (to be captured in RAN2 spec) - This feature is only applicable to UE who is configured with two separate SRS CLPC adjustment states. - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise - Above extended value range is applied to startingBitOfFormat2-3. Agreement - Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state - Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

In this contribution, we provide further discussions on the UL-only mTRP operation in Rel-19.

- Discussion on UL-only mTRP operation

2.1 Pathloss offset

For asymmetric DL sTRP/UL mTRP operation, it has been agreed that the pathloss offset factor could be configured for joint or UL TCI states and MAC-CE could be used to update the pathloss offset factor value.

In physical layer processing, usually a timeline operation is applied for MAC-CE, i.e., the update provided by the MAC-CE is applied after a period from the receiving of the MAC-CE. Therefore, whether the timeline operation should be applied for the MAC-CE for pathloss offset factor update should be discussed, i.e., whether the update of the pathloss offset factor is applied immediately after the receiving the MAC-CE, or the update of the pathloss offset factor is applied after a period from the receiving of the MAC-CE.

Proposal 1:

- For asymmetric DL sTRP/UL mTRP operation, RAN1 to discuss the timeline operation of the MAC-CE to update the pathloss offset factor, i.e., whether the update of the pathloss offset factor is applied immediately after the receiving the MAC-CE, or the update of the pathloss offset factor is applied after a period from the receiving of the MAC-CE.

2.2 Impact on PHR

In RAN1 #118 meeting, it has been agreed that the pathloss offset factor will be included in Type-1 PHR calculation and RAN1 will further discuss whether to support Type-3 PHR and whether to include pathloss offset factor in the calculation of Type-3 PHR.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

For Type-3 PHR, it is applied to the PUSCH-less cell, i.e., for SRS carrier switching. The SRS antenna switching is for DL CSI acquisition based on channel reciprocity of TDD system.

However, in asymmetric DL sTRP/UL mTRP deployment, for the site only capable of uplink reception, it seems SRS carrier switching is not applicable since there is no DL from the UL-only site and hence there is no channel reciprocity. Hence, Type-3 PHR seems not necessary for the UL-only site.

Proposal 2:

- In asymmetric DL sTRP/UL mTRP deployment, Type-3 PHR seems not necessary for the site only capable of uplink reception.

In legacy spec, two PHR mode has been introduced for multi-TRP operation, which means two PHR could be reported by the UE, and one PHR for one TRP. The two PHRs are calculated based on different pathloss reference signal, i.e., one pathloss reference signal from one TRP respectively.

In asymmetric DL sTRP/UL mTRP deployment, the uplink is multi-TRP operation. But in this case, only one pathloss reference signal is configured to the UE which is configured to be transmitted from the macro site capable of both DL transmission and UL reception. For the UL-only site, it’s not possible to configure pathloss reference signal since there is no DL transmission from the UL-only site at all.

Hence, it should be discussed and clarified whether two PHR mode can be supported for asymmetric DL sTRP/UL mTRP operation.

Proposal 3:

- For asymmetric DL sTRP/UL mTRP operation, RAN1 to discuss whether/how to support two PHR mode.

2.3 DCI enhancement for SRS power control

In RAN1 #116bis meeting, it was agreed that DCI 2_3 will be enhanced for SRS power control, including both Type A and Type B. The 1-bit closed-loop-indicator field will be added for each TPC command in DCI format 2_3.

In NR Rel-17, for aperiodic SRS transmission, the available slot operation was introduced for DCI format 0_1/0_2/1_1/1_2. With available slot operation, the aperiodic SRS resource set could be configured with RRC parameter availableSlotOffsetList which contains one or multiple values. If multiple values are contained in the availableSlotOffsetList, then SRS offset indicator is present in DCI to indicate the value of the available slot.

However, during Rel-17 discussion, DCI 2_3 was not enhanced for available slot operation since DCI 2_3 is used for SRS carrier switching.

But in RAN1 #115 meeting, in the maintenance discussion, it was agreed that DCI 1_1/1_2 could also be used for SRS carrier switching. And in RAN1 #116 meeting, in the maintenance discussion, it was concluded that for SRS carrier switching, available slot operation could be used for DCI 1_1/1_2/2_3, and if availableSlotOffsetList are configured with multiple values, it’s error case to use DCI 2_3 for SRS carrier switching. The conclusion from RAN1 #116 meeting is copied as below.

Conclusion - Available slot operation can be used with SRS carrier switching triggered by DCI formats 1_1, 1_2, and 2_3. - It is an error case if DCI format 2_3 is used with more than one entry in availableSlotOffsetList (no spec change needed).

In Rel-19, DCI 2_3 is used for asymmetric DL sTRP/UL mTRP operation, i.e., the DCI 2_3 is decoupled with SRS carrier switching operation. For example, DCI 2_3 could also be used for SRS with usage of beam management. However, with the conclusion mentioned above, if the SRS are configured with more than one value of available slots, then DCI 2_3 can’t be used. Therefore, the use of DCI 2_3 is very restrictive.

Considering the limited size of DCI 2_3, introducing new DCI field to indicate the available slot value similar as scheduling DCI is not preferred.

In this case, one way to apply available slot without adding new DCI field to DCI 2_3 is that the first available slot value could be applied to the triggered SRS if more than one available slot value is configured. In this way, more flexibility could be achieved without any changing on DCI 2_3 format.

Proposal 4:

- For asymmetric DL sTRP/UL mTRP operation, RAN1 to discuss the support of available slot operation for DCI 2_3 without adding new field into DCI 2_3, if more than one available slot values are configured for SRS. For example, the first available slot value could be applied to the SRS triggered by DCI 2_3.

- Conclusion

In conclusion, we have the following proposals on the UL-only mTRP operation in Rel-19.

Proposal 1:

- For asymmetric DL sTRP/UL mTRP operation, RAN1 to discuss the timeline operation of the MAC-CE to update the pathloss offset factor, i.e., whether the update of the pathloss offset factor is applied immediately after the receiving the MAC-CE, or the update of the pathloss offset factor is applied after a period from the receiving of the MAC-CE.

Proposal 2:

- In asymmetric DL sTRP/UL mTRP deployment, Type-3 PHR seems not necessary for the site only capable of uplink reception.

Proposal 3:

- For asymmetric DL sTRP/UL mTRP operation, RAN1 to discuss whether/how to support two PHR mode.

Proposal 4:

- For asymmetric DL sTRP/UL mTRP operation, RAN1 to discuss the support of available slot operation for DCI 2_3 without adding new field into DCI 2_3, if more than one available slot values are configured for SRS. For example, the first available slot value could be applied to the SRS triggered by DCI 2_3.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #118, August 19th – 23rd, 2024