3GPP TSG RAN WG1 #118b                                 R1-2408154

Hefei, China, October 14th – 18th, 2024

Source:	OPPO

Title:          Discussion on ISAC deployment scenarios

Agenda Item:   9.7.1

Document for:  Discussion and Decision

# Description

In RAN1 #118 meeting, the following agreement for evaluation parameters for UAV sensing scenarios was achieved.

Agreement For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#117 as a baseline. Note: Additional parameters, value/value ranges are not precluded. The detailed scenario description in this clause can be used for channel model calibration. ISAC-UAV Details on ISAC-UAV scenarios are listed in Table x. Table x. Evaluation parameters for UAV sensing scenarios Parameters; Value. Applicable communication scenarios; UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV. Sensing transmitters and receivers properties; Rx/Tx Locations; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. Note1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.. Sensing target; Outdoor/indoor; Outdoor. 3D mobility; Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity.. 3D distribution; Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. Note2: target(s) are outside the minimum distance to the Tx/Rx. Orientation; Random in horizontal domain. Physical characteristics (e.g., size); Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s). Minimum 3D distances between pairs of Tx/Rx and sensing target; Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a target Option 2: 10 meters. [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.]; FFS. Note: further down-selection between the options in the table is not precluded.

In this contribution, we will discuss some remaining issues for UAV as well as the evaluation parameters for other scenarios.

# Discussion

## Remaining parameters for UAV scenario

3D mobility

According to the previous agreements, the horizontal velocity of UAV is uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0.  However, the probability function for UAV to have a fixed zero horizontal velocity is missing from the table. Without this probability function, it is almost impossible, from probability theory perspective, to make an event for UAV to have either zero or non-zero horizontal speed. An UAV may have a movement on horizontal direction when it is in neither take-off/landing state nor hovering state. Then, mathematically,

Probnonzero horizontal speed=1-Probtakeoff & landing1-Probhovering

where both Probtakeoff & landing and Probhovering can be height-dependent, for example,

Probtakeoff & landing=1hUAV≤h0exp-hUAV-h0a0hUAV>h0

Probhovering=0hUAV≤h1p0hUAV>h1

The constant parameters {h0, a0,h1, p0} can be further determined if the above formulation is agreeable.

Proposal 1: A Note is attached to “if horizontal velocity is not fixed to 0” in 3D mobility, providing the following probability of non-zero horizontal velocity

Probnonzero horizontal speed=1-Prob11-Prob2

where

- Prob1=1hUAV≤h0exp-hUAV-h0a0hUAV>h0

- Prob2=0hUAV≤h1p0hUAV>h1

- {h0, a0,h1, p0} are constants and their values are FFS.

## Deployment parameters for indoor case

Indoor scenario is considered with human to be the sensing target in indoor office, and with automated guided vehicle to be the sensing target in indoor factory. The sensing-performing entity (i.e., Tx/Rx) can be either TRP or UE. The layout of TRP is same with communication case. The location distributions of sensing target and environment object are similar with the distribution of UE in communication case. The details assumptions for indoor scenarios are given in Table 1. For Indoor office scenario, the environment object could be furniture or small living animal such as pets. Human, metal robot and clutter could be the environment objects in indoor factory scenario.

Table 1 Assumptions for Indoor scenarios

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Indoor office (InH) | Indoor factory (InF) |
| Sensing transmitters and receivers properties |  | Layout： - 120m*50m*3m Vertical height: - 1.5m (UE), 3m (TRP) Horizontal: - Uniformly random, if height=1.5m - fixed location with ISD=20m, if height=3m Mobility: - Horizontal: 0, 3km/h, if height=1.5m | Layout： - 20-160000 m2 Vertical height: - 1.5m (UE), [5~10]m (TRP) Horizontal: - Uniformly random, if height=1.5m - fixed location with ISD=[20m], if height=[5~10]m Mobility: - Horizontal: 0, 3km/h, if height=1.5m |
| Supported sensing modes |  | mono-static, bi-static, multi-static | mono-static, bi-static, multi-static |
| Sensing target | Outdoor/indoor | Indoor | Indoor |
|  | 3D mobility | 0, 3km/h | 3km/h |
|  | 3D distribution | Vertical height: 0.8~1m Horizontal: arbitrary | Vertical height: 1m Horizontal: arbitrary |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° | Vertical: 0-180° Horizontal: 0-360° |
|  | Physical characteristics (e.g., size) | Human: - Material: Mass of water - Size: 1.8m*0.3m*0.36m (adult), 1m*0.2m*0.25m (child) | vehicle: - Material: Steel - Size: 5m*2m*1.6m (passenger vehicle); 13m*2.6m*3m (truck) |
| Unintended/Environment objects | Types | furniture, pet | human, robot, clutter |
|  | 3D mobility | 0, 3km/h | 0, 3km/h |
|  | 3D distribution | Vertical height: 0.2~1 m Horizontal: arbitrary | Vertical height: 1m Horizontal: arbitrary |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° | Vertical: 0-180° Horizontal: 0-360° |
|  | Physical characteristics (e.g., size) | Furniture: - Material: Wood - Size: 1.8m*1m*0.5m Pet: - Material: Mass of water - Size: 0.5m*0.5m*0.5m | Human: - Material: Mass of water - Size: 1.8m*0.3m*0.36m Robot : - Material: Steel - Size: 1.5m*0.3m*0.36m Clutter: - Material: Steel - Size: 1m*1m*1m |
| Sensing area |  | full area of target distribution | full area of target distribution |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] |  | 0m (for Tx-Rx in mono-static sensing only); 1m (for all other pairs) | 0m (for Tx-Rx in mono-static sensing only); 1m (for all other pairs) |

## Deployment parameters for outdoor case

The human and vehicle could be the sensing targets in outdoor scenarios. The details assumptions for outdoor scenarios with UMi/UMa/RMa are given in Table 2. The environment object could be human, vehicle or buildings. For the use case of vehicle, the assumptions are given in Table 3 with Urban gird and High-way scenarios.

Table 2 Assumptions for UMi/UMa/RMa scenarios

| Parameters |  | Value |  |  |
| --- | --- | --- | --- | --- |
| Applicable communication scenarios |  | UMi | UMa | RMa |
| Sensing transmitters and receivers properties |  | Layout: - Hexagonal cell - ISD=200m(UMi), 500m(UMa), 1732m(RMa) Vertical height: - As a UE (outdoor only): 1.5m - As a TRP: 10m(UMi), 25m(UMa), 35m(RMa) Horizontal: - As a TRP: Fixed location at hexagonal cell centers; - As a UE: Uniformly random per hexagonal cell. Mobility: - Horizontal: 0, 3km/h for a UE |  |  |
| Sensing target | Outdoor/ indoor | Outdoor human, vehicle |  |  |
|  | 3D mobility | Horizontal: 0, 3km/h, [20km/h [6]], 60km/h |  |  |
|  | 3D distribution | Vertical height: 1.5m Horizontal: arbitrary in hexagonal cell. |  |  |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° |  |  |
|  | Physical characteristics (e.g., size) | Human: - Material: Mass of water - Size: 1.8m*0.3m*0.36m (adult), 1m*0.2m*0.25m (child) Vehicle: - Material: Steel - Size: 5m*2m*1.6m (passenger vehicle), 13m*2.6m*3m (truck) |  |  |
| Unintended/Environment objects | Types | Human, animal, vehicle, obstacle |  |  |
|  | 3D mobility | Horizontal: 0, 3km/h, [20km/h [6]], 60km/h |  |  |
|  | 3D distribution | Vertical height: 1m, 1.5m Horizontal: arbitrary in hexagonal cell. |  |  |
|  | Orientation | Vertical: 0-90° Horizontal: 0-360° |  |  |
|  | Physical characteristics (e.g., size) | Human/animal: - Material: mass of water - Size: 1.8m*0.3m*0.36m (adult); 1.5m*0.5m*1m (animal) Vehicle: - Material: Steel - Size: 5m*2m*1.6m (passenger vehicle), 13m*2.6m*3m (truck) Obstacle: - Material: Wood, Cardboard - Size: 0.5m*0.5m*0.1m |  |  |
| Sensing area |  | full area of target distribution |  |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] |  | 0m (for Tx-Rx in mono-static sensing only); 10m (for all other pairs) |  |  |

Table 3 Assumptions for Urban grid and High-way scenarios

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Urban grid | High-way/Railway |
| Sensing transmitters and receivers properties |  | TRP Layout: - Hexagonal cell - ISD=500m - Road grid=250m*433m Vertical height: - 1.5m(UE, outdoor only) - 25m(TRP) Horizontal: - Uniformly random in area not occupied by rectangular buildings, if height=1.5m - Fixed location at hexagonal cell centers, if height=25m Mobility: - Horizontal: 0, 3km/h for a UE - Horizontal: 0, 3km/h, 30km/h for Vehicle-UE | High-way: HST: TRP Layout: - Hexagonal cell - ISD=1732m Vertical height: - 1.5m(UE, outdoor only) - 35m(TRP) Horizontal: - Uniformly random over straight-line, if height=1.5m - Fixed location as figure shows, if height=35m Mobility: - Horizontal: 0, 3km/h for a UE - Horizontal: 0, 30km/h, 120km/h for Vehicle -UE |
| Supported sensing modes |  | mono-static, bi-static, multi-static | mono-static, bi-static, multi-static |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor |
|  | 3D mobility | 0, 3km/h, 30km/h, 60km/h | 0, 3km/h, 60km/h, 120km/h |
|  | 3D distribution | Vertical height: - 1m Horizontal: - arbitrary over straight-line. | Vertical height: - 1m Horizontal: - arbitrary over straight-line. |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° | Vertical: 0-180° Horizontal: 0-360° |
|  | Physical characteristics (e.g., size) | Vehicle: - Material: Steel - Size: 5m*2m*1.6m (passenger vehicle); 13m*2.6m*3m (truck) | Vehicle: - Material: Steel - Size: 5m*2m*1.6m (passenger vehicle); 13m*2.6m*3m (truck) |
| Unintended/ Environment objects | Types | Human, animal | Human, animal, rocks |
|  | 3D mobility | Horizontal: 0, 3km/h | Horizontal: 0, 3km/h |
|  | 3D distribution | Vertical height: 1m Horizontal: arbitrary | Vertical height: 1m Horizontal: arbitrary |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° | Vertical: 0-180° Horizontal: 0-360° |
|  | Physical characteristics (e.g., size) | Human/animal: - Material: mass of water - Size: 1.8m*0.3m*0.36m (adult); 1.5m*0.5m*1m (animal) Obstacle: - Material: Wood, Cardboard - Size: 0.5m*0.5m*0.1m | Human/animal: - Material: mass of water - Size: 1.8m*0.3m*0.36m (adult); 1.5m*0.5m*1m (animal) Obstacle: - Material: Wood, Soil - Size: 1m*1m*0.3m |
| Sensing area |  | full area of target distribution | full area of target distribution |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] |  | 0m (for Tx-Rx in mono-static sensing only); 10m (for all other pairs) | 0m (for Tx-Rx in mono-static sensing only); 10m (for all other pairs) |

Proposal 2: RAN1 takes Table 1~3 in this contribution as reference to define ISAC deployment scenarios other than UAV.

# Conclusion

Proposal 1: A Note is attached to “if horizontal velocity is not fixed to 0” in 3D mobility, providing the following probability of non-zero horizontal velocity

Probnonzero horizontal speed=1-Prob11-Prob2

where

- Prob1=1hUAV≤h0exp-hUAV-h0a0hUAV>h0

- Prob2=0hUAV≤h1p0hUAV>h1

- {h0, a0,h1, p0} are constants and their values are FFS.

Proposal 2: RAN1 takes Table 1~3 in this contribution as reference to define ISAC deployment scenarios other than UAV.

# Reference

- RP-234069 New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

- TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz

- TR 36.777 Study on Enhanced LTE Support for Aerial Vehicles

- TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR

- TR 38.802 Study on New Radio Access Technology Physical Layer Aspects

- R1-2405964 LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC, 5GAA WG4