3GPP TSG RAN WG1 #118-bis		                                                R1-2408167

Hefei, China, Oct. 14th – 18th, 2024

Source:	OPPO

Title:	Enhancements on asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	9.2.4

Document for:	Discussion

# Introduction

The WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#102 meeting. The objectives related to this agenda item is listed below

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In this contribution, we present our view on the enhancement of asymmetric DL sTRP and UL mTRP scenario.

# Scenarios and assumptions

The deployment of heterogeneous networks (HetNets) is a strategy to improve user experience by combining macro cells with smaller cells (e.g., micro, pico, and femtocells). This setup caters to the differing needs of users and helps improve overall network coverage and capacity, particularly for uplink (UL) throughput.

This scenario provides a strategic advantage in network topology, catering to dense urban environments where uplink data traffic is significantly high. By offloading the uplink communication to dedicated UL TRPs, NW can efficiently manage the increased uplink traffic without overburdening the regular TRP, which manages both uplink and downlink. Such a deployment scenario is instrumental in improving uplink throughput and enhances network capacity and coverage.

The basic scenario of asymmetric deployment is illustrated in Figure 1. UE is engaged in downlink communication with a regular TRP capable of both downlink and uplink transmission. However, for UL communication, the UE targets a separate TRP, designated as an UL TRP.

- : the asymmetric deployment scenario in which a UE communicates with one single TRP for DL and with one or more TRPs for UL

In the WID [1], the assumptions for the enhancement in this agenda item was explicitly listed in what follows. From our point of view, those assumptions underlying this deployment are constrained to ensure focus and manageability in standardization efforts.

  - DL sTRP/UL mTRP deployment

  - Intra-band & intra DU & non-co-located mTRP

  - No definition change on cell

  - Reuse Rel-17/18 unified TCI framework

  - Applicable to both FR1 and FR2

# PL offset(s) for UL TRP

For UL TRP, since no DL RS transmitted, UE cannot measure its path loss (PL) from the UL TRP. In legacy, the UE measures the PL from the DL/UL TRP. Hence, a PL offset (based on the PL RS from DL/UL TRP) should be introduced according to the WID.

In RAN1#118, the value range of PL offset has been determined mainly as positive values. For asymmetric multi-TRP scenario, consider the fact that the UL PL to UL TRP is normally smaller than that of DL PL to DL/UL TRP.

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB

- FFS: Extending the range to lower than -10dB

- Step size is 4dB

Given the current progress, it is time to present the detailed formulas on how to reflect the PL offset in determining UL Tx power for PUSCH/PUCCH/SRS/PRACH toward UL TRP. Note that the unit is in dBm.

PPUSCH,b,f,c,ki,j,qd,l=minPCMAX,f,c,kiPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙(PLb,f,cqd-PLOk)+∆TF,b,f,ci+fb,f,ci,l

PPUCCH,b,f,c,ki,qu,qd,l=minPCMAX,f,c,kiPO_PUCCH,b,f,cj+10log102μ∙MRB,b,f,cPUCCHi+PLb,f,cqd-PLOk+∆F_PUCCHF+∆TF,b,f,ci+gb,f,ci,l

PSRS,b,f,c,ki,qs,l=minPCMAX,f,ciPO_SRS,b,f,c(qs) +10log10(2μ⋅MSRS,b,f,ci)+αSRS,b,f,c(qs)⋅(PLb,f,cqd-PLOk)+hb,f,c(i)

PPRACH,b,f,ci=minPCMAX,f,ci,PPRACH,target,f,c+PLb,f,c-PLOk

where PLOk and stands for the PL offset associated with the k-th indicated UL/joint TCI state.

With above being said, our first proposal is to consider how to implement the PL offset for UL Tx power calculation for UL channels and signals in current specification, i.e. TS 38.213.

- Support to reflect the impact of PL offset as above for Tx power calculation for PUSCH/PUCCH/SRS/PRACH in TS 38.213.

## PHR impacted by PL offset

In RAN1#118, PL offset is supported to be reflected in Type 1 PHR. RAN1 continued to study whether to PL offsets should be reflected in Type 3 PHR too.

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Agreement

Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

- Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

For Type 3 PHR (for SRS) with actual and reference transmission, one may argue that in legacy UE reports Type 3 PHR when the UE is not configured for PUSCH transmission on that carrier f of serving cell c. Now for UL TRP, there is PUSCH configured. Then the question in one’s mind could be that should UE report Type 3 PHR for UL TRP for any other good reason?

From our understanding, one use case is to facilitate NW for PL offset determination. Specifically, UE sends two separate SRS resources (SRS resource #1 and SRS resource #2) to UL/DL TRP and UL TRP respectably. If the same Tx power used for SRS resource #1 and #2, then NW can determine the UL PL difference by using SRS RSRP #1 – SRS RSRP #2. However, if different Tx powers (Pc,max - PH) applied for SRS resource #1 and 2#, then NW needs to know estimate UL PL toward DL/UL TRP and UL TRP separately. In other words, UE has to report Type 3 PHR to give NW enough information to determining the PL offset between DL/UL TRP and UL TRP.

- Type 3 PHR could be helpful for NW to determine PL offset value between DL/UL TRP and UL TRP.

With above being said, we suggest to support Type 3 PHR along with Type 1 PHR on the same carrier f of the serving cell c. Details impacting Type 3 PHR can be listed as below for actual and reference SRS transmission, respectively.

PHtype3,b,f,ci,qs=PCMAX,f,ci - PO_SRS,b,f,c(qs) +10log10(2μ⋅MSRS,b,f,ci)+αSRS,b,f,c(qs)⋅(PLb,f,cqd-PLOk)+hb,f,c(i)[dB]

PHtype3,b,f,ci,qs=PCMAX,f,ci- PO_SRS,b,f,c(qs) +αSRS,b,f,c(qs)⋅(PLb,f,cqd-PLOk)+hb,f,c(i)[dB]

where PLOk stands for the PL offset associated with the k-th indicated UL/joint TCI state.

- For carrier f of serving cell c configured with PUSCH, support both actual and reference Type 3 PHR with PL offset considered.

## PHR triggering condition

In TS 38.321, it specifies whether/how to trigger PHR based on events at UE side. Part of TS 38.321 on PHR is captured below for reference. The very 1st condition on PHR triggering is that the path loss has changed significantly (larger than a pre-defined threshold) since last PHR transmission.

Next, what matters is how to understand the pathloss variance in RAN2 specification. From the note under the first condition, one may find that the pathloss variance is between the pathloss measured at present time and the pathloss measured at previous time when last PHR transmitted. In short, the pathloss variance is based on path loss RS measurement.

- For one of the PHR triggering conditions, the pathloss variance is calculated based on the measurement of path loss RS.

### 5.4.6	Power Headroom Reporting

<Unrelated parts are omitted>

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

NOTE 1:	The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

-	phr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink of which firstActiveDownlinkBWP-Id is not set to dormant BWP;

-	activation of an SCG;

-	addition of the PSCell except if the SCG is deactivated (i.e. PSCell is newly added or changed);

<Unrelated parts are omitted>

For UL TRP in the asymmetric multi-TRP scenario, there is no DL PL RS from the UL TRP. UE has to measure PL RS from DL/UL TRP and adopt a PL offset associated with an indicated TCI state on path loss.

From this sense, it is reasonable to consider the impact of PL offset along with measured path loss into PHR triggering condition. The PL offset can be configured in UL/joint TCI state, and it can be updated by MAC CE.

One example could be that for a particular UE, the PL from DL/UL TRP remain stable, but its PL offset has been updated by MAC CE with a largely different value, e.g. from -30dB to -10dB. Hence the PHR triggering condition should consider the change of pathloss and PL offset in total, rather than path loss variance by measurement.

- On PHR triggering condition, support the case when the total change of path loss (by measurement) and PL offset is more than the pre-configured threshold (e.g. phr-Tx-PowerFactorChange dB).

## SRS not configured to follow indicated UL/Joint TCI state

In RAN1#118, the following agreement was achieved to study the case when SRS resource set is not configured with TCI state.

Agreement

- Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state

- Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state

  - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

From our understanding, NW can avoid the case when SRS is not configured with any unified TCI state. In Rel-17, SRS can be applied with two types of unified TCI states. The first type is the indicated TCI state shared with other UL channel/signals, and the second type is dedicatedly configured for each SRS resource when the first case doesn’t hold.

More specifically, for the second case, when the RRC parameter followUnifiedTCI-StateSRS is not configured for a SRS resource set, NW can configure UL/joint TCI state for each SRS resource within the SRS resource set. Each TCI state includes a set of UL power control parameters, e.g. P0, alpha, CLI and PL RS. To determine one set of UL PC parameters for all SRS resources within the SRS resource set, the current spec (highlighted as yellow) applies the UL power control parameter associated with the SRS resource with lowest ID in the SRS resource set. For more detail information, one may refer to the following specification.

# 7	Uplink Power control

-----------------------------------------<unrelated part>----------------------------------------------

-	in clause 7.3.1, if p0AlphaSetforSRS is provided,

-	if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of  PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State

-	else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set

PO_SRS,b,f,cqs is the sum of the component PO_UE_SRS,b,f,cqs and a component p0 provided by SRS-ResourceSet corresponding to the SRS resource set.

-----------------------------------------<unrelated part>----------------------------------------------

Following the same rule, when one SRS resource set is not configured with followUnifiedTCI-StateSRS, we could adopt the same rule (applying the PL offset associated with the SRS resource with lowest ID) to determine the PL offset for all the SRS resources within the SRS resource set.

- When a SRS resource set is not configured with followUnifiedTCI-StateSRS, and for a SRS resource from the SRS resource set

- Adopt the PL offset associated with the UL/Joint TCI state of an SRS resource with lowest SRS-ResourceId in the SRS resource set

## PL offset(s) for PDCCH order PRACH

To support PDCCH order PRACH, RAN1 made down a selection on how to indicate PL offset for PRACH in RAN1#118 as in the following agreement. Finally, PDCCH order PRACH also applies the PL offset associated with indicated UL/joint TCI state, which aligns well with other UL channels/signals.

Agreement

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3:

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

  - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

The remaining issue is with which indicated joint/UL TCI states the PL offset is associated. Specifically, UE should know which TRP (UL TRP or DL/UL TRP) it sends PRACH toward. In the PDCCH order, NW should provide the such indication to UE.

One simple way is to add one additional bit in the PDCCH order. Particularly, the newly added 1 bit can indicate which one out of two PL offsets associated with two indicated TCI states can be applied for PDCCH order PRACH. Analogous to TCI selection field designed for PDSCH in Rel-18 unified TCI extension for multi-TRP, we may reuse the same field in PDCCH order.

- On selection of PL offset for PRACH from the indicated UL/joint TCI states, support to add 1-bit new field in PDCCH order.

In RAN1#116bis, along with PUCCH/PUSCH/SRS, PDCCH order PRACH @FR1 has been supported to be compensated with PL offset(s). Operating at lower frequency band, UE doesn’t have to consider beam-specific UL transmission.

Agreement

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification

- Above is subject to a separate UE capability signaling

To have a unified design of PL offset for PDCCH order PRACH at both FR1 and FR2, we would like to propose as below.

- For PDCCH-order PRACH transmission toward UL TRP, support PL offset associated with indicated UL/joint TCI state at FR2.

# Enhancements on dynamic power control for SRS

In the asymmetric deployment scenario, UE may transmit SRS towards multi-TRP including DL/UL TRP and UL TRP. However, in legacy releases, dynamic power control for SRS was inherited from the design of single TRP, i.e. single close-loop power control state. Specifically, the close-loop index (CLI) was not independently introduced along with the TPC command for SRS, e.g. in DCI format 2_3. But for PUSCH and PUCCH, two separated close-loop power control states have been already supported and SRS can be configured to follow that of PUSCH.

In RAN1#116bis, RAN1 achieved a few of key agreements on two CLPC adjustment states for SRS.

Configuration-wise, the RRC parameter, closedLoopIndex-r17 in UL/joint TCI state can be reused to indicate one out of two CLPC adjustment states for SRS.

Agreement

For a UE configured with two SRS CLPC adjustment states, support Alt2 for indicating one of the SRS CLPC adjustment states to SRS:

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 for SRS in the TCI state indicates one of the SRS CLPC adjustment states

  - The candidate value of i0 and i1 in closedLoopIndex-r17 for SRS refers to the first and the second CLPC adjustment state separate from PUSCH, respectively

Signaling-wise, DCI format 2_3 with both Type A and Type B are supported with introducing 1-bit CLI field for each TPC command in DCI format 2_3.

Agreement

For indicating TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured with two SRS CLPC adjustment states, support Option3:

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

Agreement

For enhancing DCI format 2_3 for indicating TPC command for two SRS CLPC adjustment states, support Alt2:

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

Note: this 1-bit indicator is present for the CC where two SRS CLPC adjustment states are configured.

But there are still remaining issues to be addressed in upcoming RAN1 meetings.

## Additional DCI format for two CLPC of SRS

In RAN1#116bis, the following agreement is made to further study additional DCI format to convey separate TPC command(s) for SRS.

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, study whether and how to indicate TPC command for SRS CLPC adjustment states through DCI format 1_1 and/or 0_1 when the UE is configured two SRS CLPC adjustment states.

Within the UL DCI format 0_1 commonly used for scheduling PUSCH, there is already one or two TPC command fields for PUSCH. To support two CLPC adjustment states for SRS, the existing way is to set SRS to follow one of PUSCH TPC commands. It seems not necessary to additional introduce one new field for the same purpose.

-	TPC command for scheduled PUSCH - 2 bits as defined in Clause 7.1.1 of [5, TS38.213]

-	Second TPC command for scheduled PUSCH - 2 bits as defined in Clause 7.1.1 of [5, TS38.213] if higher layer parameter SecondTPCFieldDCI-0-1 is configured; 0 bit otherwise.

- For UL DCI e.g. format 0_1, the TPC command(s) for scheduled PUSCH can be reused for SRS with two CLPC adjustment states.

For DL DCI format 1_1 commonly used for scheduling PDSCH, there is one or two TPC command fields for scheduled PUCCH. But there is no link between SRS and scheduled PUCCH, i.e. SRS following that of PUCCH TPC commands.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	Second TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213] if higher layer parameter SecondTPCFieldDCI-1-1 is configured; 0 bit otherwise.

- For DL DCI e.g. format 1_1, the TPC command(s) for scheduled PUCCH cannot be reused for SRS with two CLPC adjustment states in current specification.

However, in our understanding, either DCI format 0_1 or format 1_1 are commonly used for scheduling UL or DL data respectively. Closed loop power adjusting for SRS is not the main target of data scheduling DCI as mentioned above. Such purpose has already been dedicatedly supported by the group-common DCI format 2_3. Same as legacy, the original intention of DCI 2_3 is to send CLIs and TPCs to a group of UEs for SRS power control.

- On additionally introducing DCI format(s) for indicating TPC command for SRS CLPC adjustment states, it seems unnecessary to support DCI format 0_1 and 1_1.

# 2 TA for S-DCI MTRP

In RANP#105, 2 TA enhancement for asymmetric MTRP was added as one of the objectives. One may note the asymmetric MTRP scenario as S-DCI based MTRP, since there is no DL channel from UL TRP to UE. Moreover, in Rel-18, 2 TA has been enhanced for M-DCI MTRP where two CORESETPoolIndex are configured to UE. It becomes clear that RAN1 should remove the restriction of applying 2 TA only for M-DCI MTRP.

- To support 2TA for asymmetric MTRP, suggest to remove the particular restriction of configuration of two different CORESETPoolIndex values for M-DCI MTRP.

To determine the TA value for UL TRP, NW may send the PDCCH-order PRACH. For FR2, the determination of Tx beam for PRACH seems a key issue to be addressed. Notice that the DL RS for QCL-TypeD from UL TRP doesn’t exist according to the asymmetric setting. So, it seems the SSB indicated in PDCCH-order cannot be used to determine Tx beam for PRACH in legacy release.

To make it workable, one may have to replace it with actual Tx beam toward the UL TRP. Two signaling approaches can be considered.

Specifically, one way to this end is to ignore the SSB index in PDCCH order and replace it with an UL resource, i.e. SRS resource. UE can transmit the PRACH according to the Tx beam of the indicated SRS resource.

Another way for this purpose is to take one more step forward. That is to replace it with an indicated UL/Joint TCI state. Consider the fact that the PL offset is already associated with UL/joint TCI state, it seems convenient to use spatial information from the indicated TCI state too. No additional signaling is needed. The newly added 1-bit field in the PDCCH order (if supported) could be reused for Tx beam determination. Note that the UL/joint TCI state could contain one SRS resource as QCL source RS.

- To determine Tx beam toward UL TRP at FR2, UE refers to one of the indicated UL/joint TCI states for PRACH, rather than the SSB index in PDCCH order.

# Conclusion

In this section, allow us to repeat our proposals and observations.

- Support to reflect the impact of PL offset as above for Tx power calculation for PUSCH/PUCCH/SRS/PRACH in TS 38.213.

- For carrier f of serving cell c configured with PUSCH, support both actual and reference Type 3 PHR with PL offset considered.

- On PHR triggering condition, support the case when the total change of path loss (by measurement) and PL offset is more than the pre-configured threshold (e.g. phr-Tx-PowerFactorChange dB).

- When a SRS resource set is not configured with followUnifiedTCI-StateSRS, and for a SRS resource from the SRS resource set

- Adopt the PL offset associated with the UL/Joint TCI state of an SRS resource with lowest SRS-ResourceId in the SRS resource set

- On selection of PL offset for PRACH from the indicated UL/joint TCI states, support to add 1-bit new field in PDCCH order.

- For PDCCH-order PRACH transmission toward UL TRP, support PL offset associated with indicated UL/joint TCI state at FR2.

- On additionally introducing DCI format(s) for indicating TPC command for SRS CLPC adjustment states, it seems unnecessary to support DCI format 0_1 and 1_1.

- To support 2TA for asymmetric MTRP, suggest to remove the particular restriction of configuration of two different CORESETPoolIndex values for M-DCI MTRP.

- To determine Tx beam toward UL TRP at FR2, UE refers to one of the indicated UL/joint TCI states for PRACH, rather than the SSB index in PDCCH order.

- Type 3 PHR could be helpful for NW to determine PL offset value between DL/UL TRP and UL TRP.

- For one of the PHR triggering conditions, the pathloss variance is calculated based on the measurement of path loss RS.

- For UL DCI e.g. format 0_1, the TPC command(s) for scheduled PUSCH can be reused for SRS with two CLPC adjustment states.

- For DL DCI e.g. format 1_1, the TPC command(s) for scheduled PUCCH cannot be reused for SRS with two CLPC adjustment states in current specification.

# References

- RP-234007, New WID: NR MIMO Phase 5, Samsung, RAN#102, Dec. 2023