3GPP TSG RAN WG1#118bis                                                             R1-2408199

Hefei, Anhui, China, October 14th – 18th, 2024

Agenda Item:	9.2.1

Source:	Lenovo

Title:	Enhancements for UE-initiated/event-driven beam management

Document for:	Discussion

Introduction

UE-initiated/event-driven based beam management is one of the Rel-19 MIMO objective as follows [1]:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

The following agreements and working assumptions were achieved in RAN1 #118 [2].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH. - FFS: PUCCH as the second channel - FFS: Whether the PUSCH can be with UL data Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported. CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #N. L1-RSRP #1. Differential L1-RSRP #2. …. Differential L1-RSRP #N. Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC. Note: Other contents are not precluded. . - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1 - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP. - FFS: range and step size of differential L1-RSRP - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID). Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, select one of the following options - Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - FFS: DCI format 0_3 - Option-2: Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - FFS: DCI format 0_3 - FFS: Whether/how to handle the case that multiple CSI report configuration(s) for the UE-initiated/event-driven beam report are associated with the same first PUCCH resource and/or the same scheduled PUSCH Conclusion On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, there is no RAN1 consensus on additionally supporting that the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH. Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4} - FFS: additional candidate value(s) of {5, 6, 7, 8} - FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default. Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further down-select one of the following for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement. Note: If there is no consensus in RAN1 on one of Options 1/2/3, Option 4 will be taken. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least one or both of the following should be supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report. - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, the following is provided for down-selection. - Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting. - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel. - Option-1B: No restriction in terms of periodicity. - Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. - FFS: Whether the first and second channels can be from the same/different CC. Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A and/or Mode-B, further study the following for first PUCCH transmission - UCI multiplexing/dropping/prioritization rule - Conditions for the transmission of the first PUCCH. - Whether the PUCCH resource in the first PUCCH channel can be associated with multiple CSI report configurations for UE-initiated/event-driven beam reporting from one or multiple CC(s). - Whether/how to re-transmit the first PUCCH channel. Whether/how to apply prohibit-timer or maximum number of (re)transmission(s) for first PUCCH channel.

In this contribution, we provide our views on the enhancements to support UE initiated beam measurement.

Discussion

Events for UE-initiated/event-driven beam reporting

The following working assumption about the additional trigger events besides for Event-2 was reached in RAN1#118.

Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

As discussed in our previous contributions, Event 1 can be used to monitor the quality of the current beam with less UE complexity. Event-7 is valuable for the activated TCI state updating by configuring proper M value. Thus, we support both Event-1 and Event-7 in addition to Event-2.

Proposal 1: Confirm the working assumption to support both Event-1 and Event-7 besides for Event-2.

Resources for beam measurement of UE-initiated/event-driven beam reporting

Regarding the RS measurement for the current beam, four options were provided to the case when TRS is configured as QCL-Type A and QCL-Type D RS in the indicated joint or DL TCI state because there is no specification on how to measure the L1-RSRP based on TRS according to TS38.215.

Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further down-select one of the following for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement Note: If there is no consensus in RAN1 on one of Options 1/2/3, Option 4 will be taken.

The agreed scheme 1 and scheme 2 seems enough. When TRS is configured as QCL-Type A and QCL-Type D RS in the indicated joint or DL TCI state, the NW can indicate the UE to use the SSB which is QCLed with the TRS for the current beam measurement because each TCI state can be associated with a SSB according to the current QCL chain.  However, SSB resources can not be directly configured in joint or DL TCI state which can be indicate as the unified TCI state for a BWP of a serving cell. If SSB resource is taken as the RS for current beam and new beam evaluation, the NW cannot directly obtain the quality on the TRS which may be directly configured in the joint or DL TCI state. If the spec allow the UE to obtain the L1-RSRP TRS, after receiving the UEI beam report, the corresponding joint or DL TCI state can be directly indicated as unified TCI state. Thus, we support option 2 for this case.

Proposal 2: Further support TRS as measurement RS of current beam for determining L1-RSRP when only one TRS is configured in the indicated TCI state for the RS determination for current beam for Event 2.

The NW may indicate the UE to use either CSI-RS or SSB for current beam measurement. Different beam patterns may be used for the CSI-RS transmission and SSB transmission, to ensure the fair comparison between current beam and the new beams, we support to confirm the working assumption to ensure the same RS type for RS measurement for current bean and new beams. Regarding to the comments in the RAN1#118, we would like to clarify that the RS type in this proposal refers to SSB, CSI-RS for beam management and TRS.

Proposal 3: Confirm the following working assumption with a little updating

  - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

    - The RS type refers to SSB, CSI-RS for beam management and TRS.

Regarding the RS measurement for new beams for event 2, it has been agreed to support explicit configuration on the new beam set. In our opinion, Option-3b, where the RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s), can be used for the UE to identify one of the activated TCI state which is better than the current indicated TCI state for potential dynamic TCI state switching by DCI.

Proposal 4: Support Option-3b for the RS determination/configuration on RS measurement for the new beam for Event-2.

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

UL resources for event-triggered beam report

Remaining issues in Mode A

Regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, two alternatives were provided in RAN1#118 as follows for down-selection.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt 1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case.

Considering that the first PUCCH channel in Mode-A is used to request UL grant for the transmission of UEI beam report whose function is the same to legacy SR mechanism, , we support Alt-1 that the UCI bit carried in first PUCCH channel for Mode-A is a dedicated SR.

Proposal 5: Support Alt 1 for first PUCCH channel configuration for Mode-A.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, two options were provided in RAN1#118 as follows for down-selection:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, select one of the following options - Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - FFS: DCI format 0_3 - Option-2: Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - FFS: DCI format 0_3 - FFS: Whether/how to handle the case that multiple CSI report configuration(s) for the UE-initiated/event-driven beam report are associated with the same first PUCCH resource and/or the same scheduled PUSCH

Compared with Option-1, there is no need to enhance DCI format 0_1/0_2 but just reuse CSI request field in DCI format 0_1/0_2 in Option-2. Therefore, we support Option-2 considering the spec impact.

Proposal 6: Support Option-2 for triggering procedure in Step-2 of Mode-A.

Remaining issues in Mode-B

Similarly with Mode A, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, the following two alternatives were provided in RAN1#118 for down-selection:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case.

Considering that the first PUCCH channel in Mode-B is used to notify that there is a UL transmission for  UEI beam report which is different from the SR mechanism and new timing relation between the first PUCCH channel and the second UL channel for Mode-B may be required, we prefer Alt-2 that the UCI bit carried in first PUCCH channel for Mode-B is a new type of UCI.

Proposal 7: Support Alt 2 for first PUCCH channel configuration for Mode-B.

Another issue for Mode-B is the resource mapping/configuration between first and second channel in Mode-B, and the following two alternatives were provided for down-selection:

- Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

  - Option-1B: No restriction in terms of periodicity.

- Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

Among the two options, we thinks option 1 should be enough because the UE only need to report the beam report corresponding to one event when one or more events occurs. In this case, one pre-configured resource for second UL channel is sufficient. One issue for option 2 is that additional mechanism may be needed to determine one of the type-1 PUSCH resources for the beam report transmission.

Proposal 8: Regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, support that only one first PUCCH resource and only one e pre-configured resource for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

Regarding to the case that pre-configured Type-1 CG PUSCH carry the beam report for Mode B, one issue whether the PUSCH can carry UL-SCH. To simplify the UE and NW behavior, we prefer that the Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report. When there is a beam report, the PUSCH can only carry the triggered beam report, i.e., the PUSCH can not carry UL-SCH and other UCI in this case. However, if the PUSCH is not indicated by the first PUCCH for beam report, it should be allowed to carry the UL-SCH or other UCI to improve the resource utilization.

Proposal 9: For mode B, if the pre-configured Type-1 CG PUSCH is indicated to carry a UEI beam report, the PUSCH only carries the UEI beam report. Otherwise, the pre-configured Type-1 CG PUSCH can be used for UL-SCH or other UCI.

Remaining issues in both Mode A and Mode B

Besides, the priority of the UCI bit in the first PUCCH resource should be determined for UCI multiplexing. If the UCI bit in the first PUCCH is a dedicate SR, e.g., for Mode A, then the priority of the UCI bit in the first PUCCH should be higher than normal SR but lower than LRR. Because the UEI beam report should be has higher priority than regular UL-SCH while the event-2 should not be more urgent than BFRQ.

Proposal 10: The priority of the UCI in the first PUCCH resource is higher than normal SR and lower than LRR if the UCI in the first PUCCH is a dedicated SR.

If the UCI bit in the first PUCCH is a new type of UCI, then the priority of the UCI bit is lower than SR but higher than CSI considering the urgency of UEI beam report compared to legacy CSI report.

Proposal 11: The priority of the UCI in the first PUCCH resource is lower than SR and higher than CSI if the UCI in the first PUCCH is a new type of UCI.

When the first PUCCH resource is overlapped with other PUCCH resources, the multiplexing rule of the UCI in the first PUCCH follows the multiplexing rule of SR if the UCI in the first PUCCH is a dedicated SR. But the UCI in the first PUCCH is multiplexed in a determined PUCCH resource as an independent part different from HARQ-ACK, SR and CSI if the UCI in the first PUCCH is a new type of UCI.

Proposal 12: The UCI in the first PUCCH is multiplexed in a determined PUCCH resource as an independent part different from HARQ-ACK, SR and CSI if the UCI in the first PUCCH is a new type of UCI.

Another issue is whether acknowledge information is needed in each of the steps. The DCI in step 2 for Mode A is the acknowledgement for the first step. The UE can directly send the beam report without gNB’s response for Mode B. The remaining issue is whether the acknowledgement for the beam report in step 3 of Mode A and step 2 of Mode B is needed. Different from NW controlled beam report, where the NW has total control on the UE reporting via configuration and triggering, in UE initiated beam report the UE needs to determine whether and when to send the beam report. It’s better to send an acknowledgement for the UE when the beam report is successfully received by the NW to help the UE to determine whether to perform the retransmission of some beam report. Otherwise, the UE may frequently trigger the beam report for some event if the expected beam switching/activating is not followed-up after the beam report.

Proposal 13: Support NW’s acknowledgement with response to the PUSCH carrying the beam report.

Content of event-triggered beam report

Event 1 and Event 7 may be further supported in addition to Event-2. For Event-1, the NW can configure the UE to report the measured L1-RSRP of the current beam when beam report corresponding to Event 1 is triggered. For Event-7, the beam report format agreed for Event-2 can be reused, i.e., the NW can indicate the UE to report the RSRP corresponding to N new beams as well as the L1-RSRP corresponding to the current beam. How to select the N new beam for reporting can be further studied.

Proposal 14: Regarding UL signaling content for Event 1 and Event-7,

- Regarding UL signaling content for Event 1, the UE can be indicated to report the measured L1-RSRP of the current beam.

- Regarding UL signaling content for Event 7, the UE can be indicated to report N new beams as well as the L1-RSRP of the current beam.

Another issue is how to differential the event triggering the beam report when more than one event is configured for the UE. It has been agreed that the first PUCCH only has one bit to indicate whether an event is occurred in a PUCCH occasion, the following schemes can be considered for event differential.

- Scheme 1: Different event is configured with different SR or notification resource in the first PUCCH resource.

- Scheme 2: An event related information is contained in the beam report carried in the second UL resource.

    - To have a fixed UCI size for all events, the UE can set a special/default value on the contents corresponding to new beams in the beam report.

Proposal 15: Support to report event id related information either by different SR resources or in the beam report if multiple events are configured for a UE.

Current beam determination for event-triggered beam report

It is known that the current beam of Event 2  is associated with the indicated joint or DL TCI state for a BWP of a serving cell. However, the indicated TCI state can be different in different slot due to the updating of the indicated TCI state. Therefore, for event-triggered beam report based on Event 2 which is reported in a first UL resource (PUCCH resource) with a second UL channel followed, how to interpret the current beam when receiving the first UL resource in gNB side should be clarified. As show in Figure 1, the first UL resource is transmitted in slot n+9 which means UE detects that Event 2 happens. As we known that Event 2 is based on the measurement of current beam which is associated with the indicated TCI state. However, the indicated TCI state is TCI state 1 from slot n to slot n+7 and the indicated TCI state is changed to TCI state 2 from slot n+8 to slot n+15. If the first UL resource on slot n+9 is based on Event 2 which is detected on slot n+6, then the first UL resource is based on the indicated TCI state on slot n+6 which is TCI state 1. If the first UL resource on slot n+9 is based on Event 2 which is detected in slot n+8, then the first UL resource is based on the indicated TCI state which is TCI state 2. Therefore, which slot is used to determine the indicated TCI state when receiving the first UL resource and second UL resource for Event 2 should be further studied and determined. Therefore, we propose that:

Proposal 16: Determine the slot of the indicated TCI state when receiving the first UL resource and second UL resource for Event 2.

slot n

slot n+1

slot n+2

slot n+3

slot n+4

slot n+5

slot n+6

slot n+7

slot n+8

slot n+9

slot n+10

slot n+11

slot n+12

slot n+13

slot n+14

slot n+15

TCI state 1

TCI state 2

First UL resource

Second UL resource

Event 2

Event 2

Figure 1 Illustration of relationship between indicated TCI state and event triggered beam report

CSI reference resource determination for event-triggered beam report

For a legacy CSI report, the time domain of CSI reference resource is according to the slot of PUSCH or PUCCH carrying the CSI report as shown in Figure 2 to align the understanding on the report CSI. While for event-triggered beam report based on Event 2, a first UL resource is used to request a second UL resource for beam report in Mode A or notify that a second UL resource will transmit beam report in Mode B as shown in Figure 2. The calculation results of beam report are already known in UE side when it transmits the first UL resource, therefore, for Event-2, the time domain of CSI reference resource for event-triggered beam report should be according to the slot of the first UL resource.

Second UL resource carrying beam report

First UL resource

PUSCH/PUCCH carrying CSI report

CSI reference resource

CSI reference resource

Slot l

Slot m

Slot n

RS

RS

RS

RS

Legacy CSI report

Event-triggered beam report

Figure 2 Illustration of CSI reference resource

Proposal 17: The time domain resource of CSI reference resource is determine according to the slot of the first UL resource for Event-2 for both mode A and mode B.

Besides, in legacy L1-RSRP report, the RS occasion(s) for a beam report is either the latest RS occasion no later than the CSI reference resource of the beam report or the RS occasions no later than the CSI reference resource of the beam report according to a RRC parameter timeRestrictionForChannelMeasurements. However, whether there is a same or similar RRC parameter to determine the RS occasion(s) for the event triggered beam report should be studied. Besides,  the latest RSs associated with event triggered beam report no later than the corresponding CSI reference resource may not always satisfy the condition Therefore, we propose that:

Proposal 18: Further study the determination of RS occasions for the event triggered beam report.

Conclusion

In this contribution, we provide the following proposals:

Proposal 1: Confirm the working assumption to support both Event-1 and Event-7 besides for Event-2.

Proposal 2: Further support TRS as measurement RS of current beam for determining L1-RSRP when only one TRS is configured in the indicated TCI state for the RS determination for current beam for Event 2.

Proposal 3: Confirm the following working assumption with a little updating

  - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

    - The RS type refers to SSB, CSI-RS for beam management and TRS.

Proposal 4: Support Option-3b for the RS determination/configuration on RS measurement for the new beam for Event-2.

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

Proposal 5: Support Alt 1 for first PUCCH channel configuration for Mode-A.

Proposal 6: Support Option-2 for triggering procedure in Step-2 of Mode-A.

Proposal 7: Support Alt 2 for first PUCCH channel configuration for Mode-B.

Proposal 8: Regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, support that only one first PUCCH resource and only one e pre-configured resource for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

Proposal 9: For mode B, if the pre-configured Type-1 CG PUSCH is indicated to carry a UEI beam report, the PUSCH only carries the UEI beam report. Otherwise, the pre-configured Type-1 CG PUSCH can be used for UL-SCH or other UCI.

Proposal 10: The priority of the UCI in the first PUCCH resource is higher than normal SR and lower than LRR if the UCI in the first PUCCH is a dedicated SR.

Proposal 11: The priority of the UCI in the first PUCCH resource is lower than SR and higher than CSI if the UCI in the first PUCCH is a new type of UCI.

Proposal 12: The UCI in the first PUCCH is multiplexed in a determined PUCCH resource as an independent part different from HARQ-ACK, SR and CSI if the UCI in the first PUCCH is a new type of UCI.

Proposal 13: Support NW’s acknowledgement with response to the PUSCH carrying the beam report.

Proposal 14: Regarding UL signaling content for Event 1 and Event-7,

- Regarding UL signaling content for Event 1, the UE can be indicated to report the measured L1-RSRP of the current beam.

- Regarding UL signaling content for Event 7, the UE can be indicated to report N new beams as well as the L1-RSRP of the current beam.

Proposal 15: Support to report event id related information either by different SR resources or in the beam report if multiple events are configured for a UE.

Proposal 16: Determine the slot of the indicated TCI state when receiving the first UL resource and second UL resource for Event 2.

Proposal 17: The time domain resource of CSI reference resource is determine according to the slot of the first UL resource for Event-2 for both mode A and mode B.

Proposal 18: Further study the determination of RS occasions for the event triggered beam report.

References

- RP-242394 WID revision: NR MIMO Phase 5, Samsung, 3GPP RAN #105, September 2024.

- 3GPP RAN1 #118 Chairman Notes, August 2024