3GPP TSG RAN WG1 #118bis		                                         R1-2408202

Hefei, Anhui, China, October 14th – 18th, 2024

Agenda Item:	9.2.4

Source:	Lenovo

Title:	Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion

Introduction

In RAN1 #118 meeting [1], the following agreements were made on enhancement for asymmetric DL sTRP/UL mTRP scenarios:

Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not. Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR. Agreement About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, down-select one from the following Alts in RAN1#118bis: - Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is appliable to any rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise - Alt2: X = 44 (to be captured in RAN2 spec) - This feature is only applicable to UE who is configured with two separate SRS CLPC adjustment states. - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise - Above extended value range is applied to startingBitOfFormat2-3. Agreement - Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state - Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

In this contribution, we provide our views on the related issues to support asymmetric DL sTRP/UL mTRP scenarios.

Discussion

Closed-loop PC for SRS

In current specification, only DCI format 2_3 can indicate TPC command for SRS if srs-PowerControlAdjustmentStates indicates a different power control adjustment states between SRS transmissions and PUSCH transmissions. Since DCI format 2_3 has already been agreed to indicate TPC command for SRS when two separate SRS power control adjustment states are configured, we don’t see the need to specify other methods for the same purpose. If DCI format 0_1/1_1 is used to indicate TPC command for SRS, different TPC fields shall be used to indicate TPC command for the scheduled PUSCH/indicated PUCCH and for SRS transmission. Then the payload size of DCI format 0_1/1_1 will be increased. Besides, whether one or two TPC fields are used for SRS also needs to be discussed. Therefore, we think DCI format 2_3 is enough to indicate TPC command for SRS with two separate CLPC adjustment states from PUSCH.

Proposal 1: Only support DCI format 2_3 to indicate TPC command for SRS configured with two CLPC adjustment states.

Up to Rel-18, for srs-TPC-PDCCH-Group = typeA, DCI format 2_3 can only trigger SRS resource set(s) for ‘antennaswitching’ as shown in Table 7.3.1.1.2-24 in TS 38.212 [2].

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field, or value of ‘SRS request’ index for each cell in the scheduled cell set indicated by SRS request field in DCI 0_3 or 1_3 | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 0_2, 0_3, 1_1, 1_2, 1_3, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 1 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList set to 2 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 2 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerList set to 3 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 3 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |

In Rel-19 asymmetric DL sTRP/UL mTRP scenarios, two closed-loop PC adjustment states for SRS both separate from PUSCH are introduced. Different SRS resource sets may use different PC adjustment states and the different SRS resource sets can be used for antenna switching, or for other purpose for example for beam management. In Rel-19, the feature of two SRS CLPC adjustment states should not tie with SRS antenna switching. Therefore, the restriction ‘antennaswitching’ should be removed when DCI format 2_3 is used for triggering aperiodic SRS resource set(s) for type A SRS TPC command. That is if a UE is configured with two separate closed loops for SRS transmission, Table 7.3.1.1.2-24A is used for interpreting SRS request.

Table 7.3.1.1.2-24A: SRS request

| Value of SRS request field, or value of ‘SRS request’ index for each cell in the scheduled cell set indicated by SRS request field in DCI 0_3 or 1_3 | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 0_2, 0_3, 1_1, 1_2, 1_3, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 1 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured by SRS-ResourceSet and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList set to 2 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 2 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured by SRS-ResourceSet and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerList set to 3 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 3 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured by SRS-ResourceSet and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |

Proposal 2: Support to remove the ‘antenna switching’ restriction for aperiodic SRS resource sets triggered by DCI format 2_3 if two CLPC adjustment states are configured for SRS.

In RAN1 #117 meeting [3], the issue of SRS available slot for DCI format 2_3 in asymmetric DL sTRP/UL mTRP scenario was discussed. In RAN1 #116 meeting [4], it was concluded that available slot operation could be used for SRS carrier switching triggered by DCI 2_3, but it is error case to use DCI format 2_3 if availableSlotOffsetList is configured with multiple values in the Rel-17 MIMO maintenance. That means the availableSlotOffsetList configured for a SRS resource set for antenna switching shall contain only one entry when we apply this conclusion for Rel-19. The flexibility may not be a big issue in Rel-17 since DCI format 2_3 is only used for triggering SRS for antenna switching. However in Rel-19, considering that DCI format 2_3 does not tie to antenna switching, it will be too restrictive if all SRS resource sets for antenna switching and beam management are configured with only one entry in availableSlotOffsetList. Otherwise, a DCI other than DCI format 2_3 may need to be used to trigger aperiodic SRS resource set for antenna switching or for beam management and a DCI format 2_3 is used to indicate TPC command for SRS transmission. This two-level DCI mechanism will increase UE’s decoding complexity. One simple method is to allow the gNB configures multiple values in availableSlotOffsetList for the SRS resources which can be triggered by DCI format 2_3 and introduce SRS offset indicator field in DCI format 2_3.

Proposal 3: Support DCI format 2_3 to trigger aperiodic SRS resource set configured with multiple entries in availableSlotOffsetList in asymmetric DL sTRP/UL mTRP scenario and introduce SRS offset indicator field in DCI format 2_3 in this case.

In RAN1 #117 meeting, it was agreed to extend the value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19. In RAN1 #118 meeting, two alternatives were discussed on the specific value of X. Alt1 is X=45 regardless whether a UE supports two separate SRS CLPC adjustment states or not and Alt 2 is X=44 applicable to UE who is configured with two separate SRS CLPC adjustment states. We think the issue of extending the starting bit is caused by the introduced 1-bit closed-loop indicator field in a block in Rel-19. Therefore the condition of configuring two separate SRS CLPC adjustment states shall be satisfied. But considering DCI format 2_3 is a group common DCI format which can indicate SRS TPC command for UEs with or without configuring two separate SRS CLPC adjustment states. We propose to modify Alt 2 as:

- Alt2: X = 44 (to be captured in RAN2 spec)

  - This feature is only applicable to when at least one UE who is configured with two separate SRS CLPC adjustment states.

  - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise

Proposal 4: On value of X, support the Alt2 with following update:

- Alt2: X = 44 (to be captured in RAN2 spec)

  - This feature is only applicable to when at least one UE who is configured with two separate SRS CLPC adjustment states.

  - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise

## 2.2 Beam indication for asymmetric DL sTRP/UL mTRP deployment scenarios

It is agreed that when Rel-18 unified TCI framework is configured, up to two joint TCI states (joint mode) or {one DL TCI state + up to two UL TCI state}(separate mode) can be configured in a BWP of a serving cell. If two joint TCI states are indicated, only one joint TCI state can be used for DL transmission since only one TRP support DL transmission in asymmetric DL sTRP/UL mTRP deploy scenarios. In Rel-18 unified TCI framework, a TCI selection field can be configured in the scheduling DCI to indicate the TCI state for PDSCH when two TCI states are indicated. And for PDCCH and CSI-RS, a RRC parameter applyIndicatedTCI-State is used to indicate the applied TCI state. These indications may not be needed in asymmetric DL sTRP/UL mTRP deploy scenario for separate TCI mode. However, there may be  ambiguity when two joint TCI states are indicated in asymmetric DL sTRP/UL mTRP deploy scenario. One simple way is that the first indicated joint TCI state is always used for DL channels/signals which follow the indicated TCI state.

Proposal 5: If two joint TCI states are indicated for asymmetric DL sTRP/UL mTRP deployment scenarios in FR1, the first joint TCI state is applied for DL channels/signals which follow the indicated TCI state. And the TCI selection field in DCI format 1_1/1_2 as well as higher layer parameter applyIndicatedTCI-State for PDCCH and CSI-RS are not needed.

When two joint or UL TCI states are indicated for UL transmission, then one indicated TCI state can be associated with a Macro TRP which performs both DL transmission and UL reception and the other indicated TCI state can be associated with a Micro TRP which only perform UL reception. Therefore, the priority of a Macro TRP should be higher than a Micro TRP. While one UL transmission associated with the Macro TRP and another UL transmission associated with the Micro TRP are overlapped, whether the power priority and transmission priority of the two UL transmissions needs to be enhanced can be further studied.

Proposal 6: Study whether the power priority of two overlapped UL transmissions associated with a Macro TRP and a Micro TRP respectively should be enhanced considering the priority of the Macro TRP and the Micro TRP.

Proposal 7: Study whether the transmission priority of two overlapped UL transmissions associated with a Macro TRP and a Micro TRP respectively should be enhanced considering the priority of the Macro TRP and the Micro TRP.

It is agreed to study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. Type 3 PHR reporting is applied for a serving cell without configured PUSCH/PUCCH transmission but only have SRS transmission in the serving cell. There is no need to support Type 3 PHR reporting since it always support PUSCH/PUCCH transmission in the serving cell in asymmetric DL sTRP/UL mTRP deploy scenarios.

Proposal 8: Do not support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

If a UE is near to a Micro TRP, it can be scheduled to transmit PRACH to the Micro TRP to save the transmit power. It is beneficial to transmit PRACH to Micro TRP in both FR1 and FR2. In RAN1#118, to indicate the PL offset for a PDCCH-order PRACH transmission, Alt 3 was agreed at least for FR1, that is the PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. We think the PL offset determination is also appliable to FR2. In FR2, one issue is how to determine the Tx beam of a PDCCH-order PRACH transmission. In legacy S-DCI based system, PDCCH-order PRACH is transmitted with a beam corresponding to the indicated SSB. Since only Macro gNB can transmit DL RS, PRACH can only be transmitted to Macro gNB. To support PDCCH-order PRACH transmission to a Micro TRP in Rel-19, the Tx beam can be determined by one of the indicated joint/UL TCI state for UL TRP in unified TCI framework as the PL offset.

Proposal 9: Support PDCCH-order PRACH transmission to UL TRP in FR2.

Proposal 10: The Tx beam of a PDCCH-order PRACH transmission is determined by one of the indicated joint/UL TCI state in FR2.

Another issue is  which one of the indicated joint/UL TCI state should be used to determine the PL offset/Tx beam. A 1-bit new field can be introduced in the PDCCH order to indicate the triggered PRACH is transmitted to a Macro TRP or a Micro TRP. If the PRACH is transmitted to a Macro TRP, the PL offset is 0dB and the Tx beam of the PRACH transmission is determined as legacy; while if the PRACH is transmitted to a Micro TRP, the PL offset/Tx beam should be determined by a new rule that is PL offset/Tx beam is determined by the indicated joint/UL TCI state corresponding to the Micro TRP. For example, if two joint/UL TCI states are indicated for UL transmission, and the first joint TCI state is used for UL transmission to a Macro TRP and the second joint/UL TCI state is used for UL transmission to a Micro TRP. If the new field in a PDCCH order equals ‘0’, then the PL offset/Tx beam of the PRACH transmission is determined as legacy; if the new field in a PDCCH order equals ‘1’, then the PL offset/Tx beam for the PRACH transmission is determined by the the second joint TCI state.

Proposal 11: Support to introduce a 1-bit new field to indicate PL offset/Tx beam is determined by which one of the indicated joint/UL TCI states.

## 2.3 Two TA issue

In RAN #105 meeting [5],  it was agreed to specify two TAs in asymmetric DL sTRP/UL mTRP scenario as following revised WID:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Different from multi-DCI based multi-TRP transmission with two TAs, there is only one NTA,offset and one DL reference timing corresponding to the DL transmission from the Macro gNB in asymmetric DL sTRP/UL mTRP scenario. Thus, the single NTA,offset and single DL reference timing  will be applied to the two TAs.

Proposal 12: When two TAs are configured for the serving cell in asymmetric DL sTRP/UL mTRP, the single NTA,offset and single DL reference timing  will be applied to the two TAs.

Conclusion

As a summary, we have the following proposals on enhancement for asymmetric DL sTRP/UL mTRP scenarios:

Proposal 1: Only support DCI format 2_3 to indicate TPC command for SRS configured with two CLPC adjustment states.

Proposal 2: Support to remove the ‘antenna switching’ restriction for aperiodic SRS resource sets triggered by DCI format 2_3 if two CLPC adjustment states are configured for SRS.

Proposal 3: Support DCI format 2_3 to trigger aperiodic SRS resource set configured with multiple entries in availableSlotOffsetList in asymmetric DL sTRP/UL mTRP scenario and introduce SRS offset indicator field in DCI format 2_3 in this case.

Proposal 4: On value of X, support the Alt2 with following update:

- Alt2: X = 44 (to be captured in RAN2 spec)

  - This feature is only applicable to when at least one UE who is configured with two separate SRS CLPC adjustment states.

  - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise

Proposal 5: If two joint TCI states are indicated for asymmetric DL sTRP/UL mTRP deployment scenarios in FR1, the first joint TCI state is applied for DL channels/signals which follow the indicated TCI state. And the TCI selection field in DCI format 1_1/1_2 as well as higher layer parameter applyIndicatedTCI-State for PDCCH and CSI-RS are not needed.

Proposal 6: Study whether the power priority and transmission priority of two overlapped UL transmissions associated with a Macro TRP and a Micro TRP respectively should be enhanced considering the priority of the Macro TRP and the Micro TRP.

Proposal 7: Study whether the transmission priority of two overlapped UL transmissions associated with a Macro TRP and a Micro TRP respectively should be enhanced considering the priority of the Macro TRP and the Micro TRP.

Proposal 8: Do not support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

Proposal 9: Support PDCCH-order PRACH transmission to UL TRP in FR2.

Proposal 10: The Tx beam of a PDCCH-order PRACH transmission is determined by one of the indicated joint/UL TCI state in FR2.

Proposal 11: Support to introduce a 1-bit new field to indicate PL offset/Tx beam is determined by which one of the indicated joint/UL TCI states.

Proposal 12: When two TAs are configured for the serving cell in asymmetric DL sTRP/UL mTRP, the single NTA,offset and single DL reference timing  will be applied to the two TAs.

References

- Chairman notes, RAN1#118, Maastricht, NL, August, 2024

- 3GPP TS 38.212 V18.3.0, “Physical layer procedures for data”.

- Chairman’s notes, RAN1#117 , Fukuoka, Japan, May 2024.

- Chairman’s notes, RAN1#116 , Athens, Greece, February 2024.

- Chairman notes, RAN#105, Melbourne, Australia, September, 2024