3GPP TSG RAN WG1 #118bis	R1-2408207

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.4.2.2

Source:	NEC

Title:	Discussion on frame structure and timing for ambient IoT

Document for:	Discussion and Decision

1	Introduction

In RAN1 #118 meeting [2], some agreements are achieved and listed below.

Agreement When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices Agreement Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following - How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 - How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1 Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

In this paper, we discuss and give our views on frame structure, timing and scheduling related aspects for ambient IoT.

2	Discussion

2.1 	PDRCH guard period

For ambient IoT transmission and reception, especially for device type 2, the maximum transmission and reception duration may be needed for the device’s energy harvesting. In that sense, if the duration of the transmission for a large number of payload bits is larger than this maximum duration, the ambient IoT device may not have enough power for transmission and reception. Some guard period within the transmission burst should be introduced for the device’s energy harvesting. The whole transmission is divided into multiple sub-transmissions with a guard period between them, and for each sub-transmission, there should be a pre-amble and post-amble at the beginning and at the end of the sub-transmission separately. To distinguish between the first and the last sub-transmission and the others, the pre-amble at the first sub-transmission and the post-amble at the last sub-transmission could be different from other pre-ambles and post-ambles. We can also call the other pre-ambles and post-ambles as mid-ambles.

Proposal 1: For PDRCH transmission, it could be divided into multiple sub-transmission with guard period between them, when the transmission duration is larger than a threshold that ambient IoT device needs to do energy harvesting during guard period.

To allow for efficient division of PDRCH transmission into multiple sub-transmission with a guard period, both the length of the guard period and the length of sub-transmission can be designed to be in line with the energy harvesting/storing capability of the device. For example, if a device has higher energy storage capability, it should have a larger length of sub-transmission. Likewise, a device with strong energy harvesting capability would require a shorter guard period. Therefore, to assist in the division, the device can share the capability related to energy harvest/storing to the reader or network, and a predefined level with indexes can be applied to control the overhead/complexity of sharing the information.

Proposal 2: For PDRCH transmission, capability information related to energy harvesting/storing of device could be shared to reader or network to facilitate the division of sub-transmission when it’s necessary.

2.2	The end of transmission

In the RAN1#116bis meeting, both control information and the postamble located at the end of the transmission were discussed to indicate the end of transmission. If control information is used to indicate the length of transmission, the device should store the receiving signal first before decoding the control information, which increases the complexity of the device's hardware. However, it also increases the probability of decoding successfully by avoiding the error detection of postamble. To consider the overhead of control information further, we prefer to apply control information to indicate the end of transmission with a low enough overhead.

Proposal 3: For both PDRCH and PRDCH transmission, we prefer to apply control information to indicate the end of transmission with a low enough overhead.

To control the overhead of control information, the transmission payload or transmission time length could be defined in terms of a fixed granularity, such as in terms of 50 bits, milliseconds and so on. If we take the sub-transmission division into consideration, only the payload or time length of the last sub-transmission needs to be indicated with the same length for the other sub-transmissions, wherein different midamble types are located at the front of the last sub-transmission and the other sub-transmission, based on which, the receiving node can determine whether the present sub-transmission is the last one.

Proposal 4: To realize a low enough overhead for control information, payload or time length could be allocated in terms of fixed granularity, or only indicate the payload or time length for the last sub-transmission with same length for the other sub-transmissions when multiple sub-transmissions are existed.

2.3	Scheduling related aspects

2.3.1	Indication of device message size

In previous RAN2#126 meeting [4], there is an agreement on functionality shown as below.

Agreements on functionality 1 Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. 2 legacy NR BSR/SR is not needed for A-IoT communication. 3 FFS whether further indication of device message size/status is needed 4 AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed)

From the agreement, we can see legacy NR BSR/SR is not needed for A-IoT communication, and whether to further indicate device message size/status is still for further study. Generally this issue is within RAN2’s working scope, but the detailed procedure as well as necessary signalling like physical layer indication/reporting may have RAN1 impact. Therefore, in this section, we would like to discuss this issue from RAN1’s perspective.

First of all, on the necessity of indication of device message size/status, we think it is necessary since reader can not identify subsequent procedures as continuous data scheduling or completion without this indication. Secondly, regarding on the indication content, we prefer message size report. The reason is that, with status reporting on whether there is remaining data or not, reader only knows the following procedure as continuous data scheduling or completion. However, due to lack of specific data size information, reader can only support blind scheduling. This will bring three subsequent cases. Further message status report and further round of blind data scheduling, large padding bits due to large resource allocation blindly, and the near prefect data scheduling. The first two cases will make blind scheduling quite inefficient. Therefore, message status report is not enough and message size report with higher efficiency is preferred from our point of view.

Regarding high efficiency scheduling, BSR/SR mechanism is applied with BSR table locating the range of message date size in legacy. However, BSR/SR is not needed in AIoT at least due to the relatively small data size of AIoT device. Therefore, for AIoT system, simple and direct mechanism of message data size reporting/indication is especially needed. Targeting this, a ratio based indication scheme is proposed, where a ratio defined as device remaining data size over the previous indicated/scheduled data size is introduced. By introducing this metric, the general data scheduling procedure of AIoT system can be illustrated in Figure 1.

Figure 1. Data scheduling procedure with ratio indication

Denoting TBS indicated in the first/initial PRDCH for following PDRCH scheduling as TBS1 in Figure.1, after device successfully received the first PRDCH, TBS1 can be obtained. Then, device will calculate ratio r=remaining data size/TBS1=(total R2D data size-TBS1)/TBS1 when the total D2R data size at device side is larger than TBS1, and report it together with D2R data of size TBS1 in the corresponding PDRCH. After reader receives the PDRCH successfully, reader can deduce the remaining D2R data size with reported r, then reader will decide TBS2 for the next round PDRCH scheduling and indicate it over the following PRDCH. This procedure will repeat until the completion of all D2R data transmission.

Observation 1: Ratio based message size indication can improve the scheduling efficiency of AIoT system.

Proposal 5: Study ratio based message size indication for data scheduling in AIoT system.

2.3.2	Ehanced RACH procedure

In the discussion of RACH procedure for AIoT system, it is captured in FL summary [5] from previous RAN1#117 meeting that, RAN1 can study the physical resources and physical channel(s)/signal(s) for contention-based and contention-free random access procedures. While the RACH procedure discussion is within the scope of RAN2. Meanwhile, with the discussion in previous RAN2#126 meeting [4], 4-step and 2-step RACH procedures had finally agreed as below.

FL summary (R1-2405381) Based on the chair’s guidance, it is not in the scope of RAN1 to define the number of steps and the function of the message for each step in random access procedure. RAN1 can study contention resolution aspects at physical layer (in case of contention-based access) and how to use physical resources (in case of contention-free access), i.e., to study physical resources and physical channel(s)/signal(s) for contention-based and contention-free random access procedures.

Agreements on “4 step” RA 1 A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information 2 A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements 3 A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) - The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. - “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed

From above 2-step and 4-step RACH procedure, upper layer data can be transmitted in msg1 and msg3 respectively. Therefore, enhanced RACH can be achieved by embedding the proposed ratio based data scheduling mechanism. Specifically, for enhanced 2-step RACH procedure illustrated in Figure 2, with small data transmission (size of TBS1) in AIoT msg1, the ratio of remaining data size over TBS1 assisting next round scheduling can be transmitted/reported together. For enhanced 4-step RACH procedure illustrated in Figure 3, similarly, after TBS1 indicated in AIoT msg2, to assist a better data scheduling in next round, the ratio of remaining data size over TBS1 can be calculated and reported by device through AIoT msg3.

Figure 2. Enhanced 2-step RACH procedure with ratio indication for AIoT system

Figure 3. Enhanced 4-step RACH procedure with ratio indication for AIoT system

Observation 2: Ratio based message size indication can improve the data scheduling efficiency of RACH in AIoT system.

Proposal 6: Study ratio based message size indication as enhanced 2-step and 4-step RACH for AIoT.

2.3.3	IoT Sync Requirements during Random Access Procedure

During RAN1#117, there was discussion on how IoT device can maintain sync with the reader when RACH procedure is triggered. The major issue is related to transmission of Msg1 (PDRCH) from IoT device, where the time gap between Msg0 (RACH trigger message) and Msg1 (subsequent PDRCH) message can be quite large due to slotted ALOHA based RACH procedure. In such a case, the IoT device may not be fully synchronized with the reader when it performs PDRCH transmission, and we can encounter a scenario where IoT device may end up performing transmission in a different slot as intended. Please find the below figure as an example for this case, here we assume that sampling offset is close to 10^5 ppm. In this case, the IoT device gains sync with reader after receiving the R2D trigger message, but after a few slots of random access there can be a large difference between the IoT device clock and reader clock (due to sampling offset) such that if IoT device intends to perform transmission in slot-7, the transmission may end up being performed in the middle of slot-6 for the reader.

Such a behaviour defeats the slotted ALOHA principle and will end up having significantly lower random-access performance. Hence, there is a need for IoT device to remain in sync with the reader for all the available slots of random access.

Observation 3: There is a need to ensure that how IoT device remains in sync with the reader for all the available slots of random access.

One mechanism which was discussed in RAN1#117 was that reader should transmit PRDCH message before each slot, however this will result in large signalling overhead. We need to investigate more efficient mechanisms (in terms of signalling overhead) for maintaining synchronization.

We align with this understand that there should be a R2D transmission before each slot of PDRCH transmission, however this should only contain the clock acquisition part (for IoT device to regain sync) along with only necessary information required for performing PDRCH transmission in the upcoming slot.

Given that IoT device may not have the capability to count slot times, it would be beneficial that the R2D transmission from the reader before each slot can incorporate the slot identifier so that IoT device can determine (after receiving the R2D transmission) whether it should perform PDRCH transmission within the upcoming slot or not.

Proposal 7: Study R2D transmission before start of each random-access slot. The R2D transmission should contain at least following information:

- Clock acquisition part

- Slot indicator

2.3.4	Resource allocation for Msg.1

Two options were discussed during RAN1#118 meeting for number of resources in paging message determined for the corresponding Msg.1 transmission:

Option 1: one resource is determined by one A-IoT paging message

Option 2: one or more resources is determined by one A-IoT paging message

When we take that there’re multiple A-IoT devices need to response to a same commend into consideration, one or more resources in option 2 may save more signalling overhead. And the number of resources can be indicated in the paging message, wherein time duration for one resource can be fixed at 0.5ms or several chip duration predefined, to facilitate the resource counting. Within the more than one resources indicated for multiple devices, the resource for each device can be determined according to the random ID or device ID, to reduce the probability of collision transmission among different devices.

If we further consider the transmission of Msg.1 may be failure due to collision or channel quality or insufficient energy stored, the one or more multiple resources can be used for re-transmission of Msg.1 for a single device too. In this case, the resources for both initial transmission and re-transmission are indicated in a single paging message to save signalling overhead. And a gap can be predefined between two consecutive transmissions for the device to determine whether the latest transmission is failure or success, which is larger than the maximum gap defined for the response R2D transmission and the related D2R transmission.

Observation 4: One or more resources is determined by paging message can save the signalling overhead.

Proposal 8: For random access, study one or more resources is determined by one A-IoT paging message to be used for D2R transmission(s) for Msg.1 corresponding to the paging message, and the one or more resources can be used for:

- Msg.1 transmission for multiple A-IoT devices, and the resource’s location for Msg.1 transmission of each device can be determined according to the random ID or device ID.

- Initial transmission and retransmission of Msg.1 for a same device, and a gap larger than the maximum gap defined for R2D transmission response to the D2R transmission is located between the two resources for two consecutive transmissions.

2.4	Energy harvest on device availability for Tx/Rx procedures

In previous RAN1#118 meeting [2], there is an agreement with two directions for the study of device availability/unavailability.

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection. For Direction 1, following can be the template to capture the solution details proposed by companies, if any. Source; Details. Source [x]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. Source [y]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. For Direction 2, following can be the template to capture the solution details proposed by companies, if any. Source; Details. Source [x]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. Source [y]; Solution description Observations or Analysis or Evaluations Specification impacts, if any.

In this section, we proposed solutions along with Direction 2, where Reader will provide information to a device based on which the device may become available/unavailable. Since different states represent different level of device availability/unavailability, device becoming available/unavailable can be interpreted as state transitions. In the following, we assume three states are supported at device side to facilitate the illustration of proposed solutions.

2.4.1	State transition from ON to OFF

In this sub-section, we consider the case in which device will become unavailable based on information provided by Reader. That is to say, device will transit from ON state to OFF state by the indicated information. One typical scenario for this is shown in Fig. 4. To be specific, after one round of inventory/command task, Reader envisions no more inventory/command task in a short period for this device. In this case, regardless of device's power level, state transition directly to OFF state is more power efficient than putting it to Sleep. To achieve this, disabling of Sleep state can be indicated from Reader to guide device state transition from ON to OFF directly. The necessity of this indication is that ON state can transit to either Sleep state or OFF state, if without the disabling indication, device may first by-default change to Sleep state then OFF state when the power level of device is not sufficient.

Fig. 4 State transition from ON to OFF with information provided by Reader

With company’s proposals on study of state transition from ON to Sleep in RAN1#118 meeting, this sub-section focus on state transition from ON to OFF for specific scenarios. Regarding the spec impact of this solution, an extra disabling of Sleep state indication is explicitly needed from Reader.

Proposal 9: Reader can provide disabling of Sleep state indication to device based on which the device becomes unavailable/OFF state.

2.4.2	State transition from ON to Sleep

In this sub-section, we consider device transiting from ON state to Sleep state by information provided from Reader. This has already been studied by companies in last meeting, and we would like to propose an alternative solution with potentially less latency for study. Considering scenario with two subsequent inventory rounds paging same devices in Fig. 5, some devices may access and finish the inventory task late in previous round or near the start of next round. In order to save another round of RA for this device in the subsequent round of inventory/command, indicating device transits to Sleep state after the end of first round task is preferred. Targeting this, an end indication can be provided so that device can immediately change to Sleep state upon successful reception of that PRDCH. Compared to existing solutions in [6], the additional monitoring of PRDCH in an extra time window can be saved, thereby further saving the latency of state transition. Regarding the spec impact of this solution, an extra end indication is explicitly needed from Reader.

Fig. 5 State transition from ON to Sleep with information provided by Reader

Proposal 10: Reader can provide end indication to device based on which the device becomes Sleep state.

2.4.3	Harmonized design among Device types

Recalling that there was an online discussion about states in the last meeting [7],

FL1 High Priority Proposal 4.1-1b RAN1 studies energy harvesting impacts in the following way: - Case 1: Device is assumed to have two states: ON, OFF - Case 2: Device is assumed to have three states: ON, OFF, SLEEP - Identify function(s) of the device that can be assumed supported and assumed not supported in each of the above device states, subject to: - ON state supports at least: transmission, reception for communication - OFF state does not support at least: transmission, reception for communication, running a clock - OFF state supports at least: energy harvesting - SLEEP state supports at least: - Maintaining a memory content from ON state - Running a clock or maintaining a timer (RAN1 to discuss purpose(s) of clock) - Energy harvesting - FFS: monitoring some reception - SLEEP state does not support at least: transmission, reception for communication - Identify reader knowledge / control of the above states for case 2 - RAN1 is not required to establish a consensus model for the duration of each of the states

Besides, there is also one SI target of studying a harmonized framework with minimized differences (where necessary), and RAN reiterates it including the study on minimized differences (where necessary) between devices in RANP#105 meeting [8]. With several company’s preference on device type 1 supporting case 1 and device type 2 supporting case 2 due to their different capabilities, enabling/disabling Sleep state indication from Reader to device can serve as an effective/flexible mechanism for the two cases within a harmonized framework.

Proposal 11: Enabling/disabling Sleep state indication by Reader can be applied to support harmonized design among different device types and cases.

2.5	Random Access

In RAN1#116 meeting [9], there are agreements on study of CBRA for multiple devices.

Agreement From RAN1 perspective, at least when a response is expected from multiple devices that are intended to be identified, an A-IoT contention-based access procedure initiated by the reader is used. Agreement For A-IoT contention-based access procedure, at least slotted-ALOHA based access is studied.

In RAN2#126 meeting [4], there is another agreement on study of CFRA for one and multiple devices.

Agreement on CFRA From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

With multi-device random access case, when only CBRA is applied, due to resource collisions among devices, some devices may fail the access to Reader during one round of RA, then another round of RA for these failed devices may be scheduled. This mechanism will introduce large access latency for failed devices. To facilitate failed devices accessing Reader successfully within the same round as succeeded devices for latency reduction, we propose a CFRA embedded CBRA mechanism.

As shown in Fig. 6 one case of multi-device RA within the i-th round, some devices like device #1 and device #3 may succeed in accessing Reader after multiple RA trigger messages, while some devices like device #2 and device #4 still failed. Therefore, to assist failed devices accessing successfully within the i-th round instead of delaying to the (i+1)-th round, CFRA towards these failed devices can be scheduled in the last RA trigger message. Without collision in CFRA, device #2 and device #4 may have chance to access Reader successfully within the i-th round.

Fig. 6 CFRA embedded CBRA mechanism for multi-device RA

Proposal 12: Study CFRA embedded CBRA mechanism for multiple device random access.

3	Conclusion

In this contribution, we give our views on frame structure and timing for ambient IoT and propose that:

Observation 1: Ratio based message size indication can improve the scheduling efficiency of AIoT system.

Observation 2: Ratio based message size indication can improve the data scheduling efficiency of RACH in AIoT system.

Observation 3: There is a need to ensure that how IoT device remains in sync with the reader for all the available slots of random access.

Observation 4: One or more resources is determined by paging message can save the signalling overhead.

Proposal 1: For PDRCH transmission, it could be divided into multiple sub-transmission with guard period between them, when the transmission duration is larger than a threshold that ambient IoT device needs to do energy harvesting during guard period.

Proposal 2: For PDRCH transmission, capability information related to energy harvesting/storing of device could be shared to reader or network to facilitate the division of sub-transmission when it’s necessary.

Proposal 3: For both PDRCH and PRDCH transmission, we prefer to apply control information to indicate the end of transmission with a low enough overhead.

Proposal 4: To realize a low enough overhead for control information, payload or time length could be allocated in terms of fixed granularity, or only indicate the payload or time length for the last sub-transmission with same length for the other sub-transmissions when multiple sub-transmissions are existed.

Proposal 5: Study ratio based message size indication for data scheduling in AIoT system.

Proposal 6: Study ratio based message size indication as enhanced 2-step and 4-step RACH for AIoT.

Proposal 7: Study R2D transmission before start of each random-access slot. The R2D transmission should contain at least following information:

- Clock acquisition part

- Slot indicator

Proposal 8: For random access, study one or more resources is determined by one A-IoT paging message to be used for D2R transmission(s) for Msg.1 corresponding to the paging message, and the one or more resources can be used for:

- Msg.1 transmission for multiple A-IoT devices, and the resource’s location for Msg.1 transmission of each device can be determined according to the random ID or device ID.

- Initial transmission and retransmission of Msg.1 for a same device, and a gap larger than the maximum gap defined for R2D transmission response to the D2R transmission is located between the two resources for two consecutive transmissions.

Proposal 9: Reader can provide disabling of Sleep state indication to device based on which the device becomes unavailable/OFF state.

Proposal 10: Reader can provide end indication to device based on which the device becomes Sleep state.

Proposal 11: Enabling/disabling Sleep state indication by Reader can be applied to support harmonized design among different device types and cases.

Proposal 12: Study CFRA embedded CBRA mechanism for multiple device random access.

References

[1]	RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #102, December 11-15, 2023.

[2]	Draft_Minutes_report_RAN1#118_v010

[3]	3GPP TR 22.840 V2.0.0, Study on Ambient power-enabled Internet of Things (Release 19), September, 2023

[4]          Draft_R2_126_Meeting_Report_v1

[5]          R1-2405381, FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT.

[6]	R1-2406071, Discussion on frame structure and physical layer procedures for Ambient IoT

[7]	Chair notes RAN1#118 (9.4 R19 Ambient IoT) v08

[8] 	RP-242360, Moderator's summary for offline discussion on Ambient IoT.

[9]	Draft_Minutes_report_RAN1#116_v100