3GPP TSG RAN WG1 #118bis											R1-2408222

Hefei, China, October 14th – 18th, 2024

Agenda item:	9.1.4.2

Source:	NEC

Title: 	Discussion on other aspects of AI/ML model and data

Document for:	Discussion and Decision

- Introduction

Rel-19 Work Item (WI) is targeting on providing specification support for the general framework as well as the selected representative use cases, and in the revise version the study objectives are more focused on two-sided model CSI compression sub use case [2]. In the last meeting, the following agreements have been achieved [3]. In this contribution, we provide our views on the general issues of AI/ML model and data.

Conclusion From RAN1 perspective, model identification is at least applicable to some of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). Conclusion The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work. Conclusion The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work. Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study) Agreement From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information Conclusion From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). Agreement RAN1 is recommending extending the study of the Model identification, and Model transfer/Model delivery based on RAN1 understanding the study is not completed.

- Discussion on LCM

  - Model Identification

The revised WID [2] updated the study objectives as follows, which excludes the study of model identification for one-sided model. However, in previous RAN1 discussion and agreement, associated ID is used to ensure consistency at least for beam management use case, and associated ID is determined during the model identification procedure, more specifically, MI-Option1. If model identification is not supported for one-sided model use cases, associated ID may not be able to be used to ensure consistency. Therefore we propose to clarify the support of model identification for one-sided model use cases, at least when associated ID is needed to ensure the consistency between model training and model inference.

Revise WID [2] - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] RAN1 118 agreement Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list RAN1 116bis agreement Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

To ensure consistency, the option based on associated ID is supported. And the associated ID is determined during model identification.

Clarify the support of model identification for one-sided model use cases, at least when associated ID is needed to ensure the consistency between model training and model inference.

At this stage, there are two options available for model identification: MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) and MI-Option 3 (Model identification in model transfer from NW to UE). Which of the given options should be adopted is also highly dependent on the progress of CSI compression use case to address the inter-vendor training collaboration.

Following agreement was reached on RAN1#116 for addressing the inter-vendor training collaboration.

Agreement

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Note that among the given options, Option-3, 4 and 5 are still being discussed. Additionally, more progress was made on the given options in RAN1#116bis as indicated below.

Agreement

- For Option 3, further define the two sub-options:

  - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 5, further define the two sub-options:

  - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

Note that Option-3a/5a is further divided into different options: whether model/parameters exchanged from the NW-side to UE-side is corresponding to CSI generation part or CSI reconstruction part or both CSI generation and CSI reconstruction part.

For these available options, different methods may need to be considered for model identification. For instance, for the case of Option-3b/5b, we can adopt MI-Option 3 directly without any major enhancement. However, for Option-4 which does not involve any model transfer from network to UE, it may be beneficial to still consider MI-Option 1 where dataset transferred from gNB to UE can be seen as a data collection procedure where data collection configuration is directly mapped to dataset identity.

MI-Option 1 is suitable for adoption if Option-4 (Standardized data / dataset format + Dataset exchange between NW-side and UE-side) is agreed to address inter-vendor collaboration for CSI compression use case.

MI-Option 3 is suitable for adoption if Option-3b/5b (Parameters/models received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations) is agreed to address inter-vendor collaboration for CSI compression use case.

For Option-3a/5a, model identification needs to consider both model transfer from gNB to UE and the possibility of UE performing data collection for model training. Let us take an example of the case where gNB transfers CSI reconstruction model to the UE. Here, UE is expected to develop CSI generation model which can work with CSI construction model passed from network side. However, the CSI generation model development may also highly depend on the cell conditions and topology. For instance, it has already been established in TR38.843 [1] that AI/ML generalization may not always work when we consider different cell topologies (e.g. Uma vs Umi). Hence, UE would need to develop CSI generation model based on CSI reconstruction model/parameters and data collection related to network side additional conditions. Thereby, the final CSI generation model developed by UE may depend on both CSI reconstruction model as well as data collection configuration (which includes NW side additional conditions). Hence, we will probably need a combination of MI-Option-1 and MI-Option-3 to address the scenario of Option-3a/5a of addressing inter-vendor collaboration for CSI compression use case.

A combination of MI-Option 1 and MI-Option 3 is suitable for adoption if Option-3a/5a (Parameters/models received at the UE or UE-side goes through offline engineering at the UE-side) is agreed to address inter-vendor collaboration for CSI compression use case.

Hence, the final model identification method which should be adopted should be based on what option is agreed for addressing the inter-vendor collaboration issues for CSI compression use case.

RAN1 to select the model identification procedure based on the outcome of the study to address inter-vendor collaboration issues for CSI compression use case.

MI-Option 1 to be adopted if Option-4 (Standardized data / dataset format + Dataset exchange between NW-side and UE-side) is agreed to address inter-vendor collaboration issues for CSI compression use case.

MI-Option 3 to be adopted if Option-3b/5b (Parameters/models received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations) is agreed to address inter-vendor collaboration issues for CSI compression use case.

A combination of MI-Option 1 and MI-Option 3 to be adopted if Option-3a/5a (Parameters/models received at the UE or UE-side goes through offline engineering at the UE-side) is agreed to address inter-vendor collaboration issues for CSI compression use case.

The association among those different data collection configurations may need to be provided to UE, for example, via the associated ID, since UE needs to be aware that these configurations are for different LCM stages of the a same AI/ML model, and to maintain some consistency in measurement and report for those data collections. As discussed in each use case, at least the consistency across training and inference is beneficial from performance perspective. In addition, the consistency across model inference and performance monitoring may also be needed. In this case, one or more associated ID(s) can be attached to one same model ID to reflect different NW side additional conditions.

One or more associated ID(s) can be attached to one same model ID to reflect different NW side additional conditions.

In addition, as discussed in step D in AI-Example1 for MI-Option 1, how model ID is determined/assigned needs to be specified. Considering that different types of model IDs may be used during model identification procedure, for example, a global model ID and a local model ID, at least the combination of Alt.1: NW assigns Model ID and Alt.2: UE assigns/reports Model ID can be supported. A global model ID is usually much more complicated than a local model ID and can be used for the initial identification, and after the models being identified, a simpler ID, e.g., local ID can be used for the following managements including model activation, deactivation, switching and selection. The figure below illustrates a model identification procedure initiated by the UE (Alt.2: UE reports a global Model ID) and NW may respond the request by assigning a local ID (Alt.1: NW assigns a local Model ID) for the future use. Meanwhile, the alternatives that associated ID(s) is assumed as model ID(s) may not work noticed that one or more associated ID(s) can be attached to one same model ID to reflect different NW side additional conditions

Figure 1 an example of model ID translation

In the model identification procedure, support the combination of Alt.1: NW assigns Model ID and Alt.2: UE assigns/reports Model ID, which is used to link a UE reported global model ID to a NW assigned local model ID.

  - Conditions, Additional Conditions and Ensure the Consistency

The concept of additional conditions was introduced during the later stage of the study item where the need for the additional conditions was identified due to presence of AI/ML training variables which may not be standardised. For instance, in AI/ML based beam management use case, network topology elements like cell layout or antenna configuration, or UE internal variables like UE speed can impact the selection of the AI/ML model which should be used during AI/ML operation. However, even though these variables may be used for training of AI/ML model and thereby the AI/ML model operation require these variables as input, these variables may not be standardised due to disclosure of proprietary information. Hence, classifying these variables as additional conditions can be an enabler for optimal AI/ML operation.

Among the methods which have been considered for ensuring consistency between network and UE for these additional conditions, the methodology of “Consistency assisted by monitoring” seems to be the least optimal solution. This method is expected to result in high delay in selection of optimal AI/ML model for a UE. For instance, if multiple set of beam management models are developed for different UE speeds and network wants to use the monitoring procedure to select the optimal model, then network needs to activate multiple models at UE one by one associated to different UE speeds until it determines an AI/ML model which results in the best performance. For the duration of time when network is activating non-optimal model(s) at UE, the UE performance suffers and results in delay in selection of the most optimal model at UE.

Ensuring consistency of additional conditions using monitoring procedure results in high delay in identification of the suitable AI/ML model to run at UE, during which system performance suffers.

For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options should be considered as priority:

Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

Model training at NW and transfer to UE, where the model has been trained under the additional condition

Information and/or indication on NW-side additional conditions is provided to UE

The associated ID is configured by the NW and is used to reflect NW-side additional conditions which may not be specified. However, not every difference in NW-side additional conditions or NW implementation has impacts on UE side assumption of the consistency, UE side model training and model inference, and therefore not every different NW-side additional conditions require to be assigned with a separated associated ID. It is difficult for NW to decide how to assign the associated IDs for data collection configurations, e.g., under which circumstance NW needs to assign a different associated ID. For UE-sided models, it is more reasonable for UE to decide how to develop the model and whether the model can be used for different NW additional conditions. For example, UE can develop independent models or a general model based on data collection configurations with different associated IDs. If a general model is developed, the associated IDs may not be very useful. One possible solution could be that NW provides configurations with different associated IDs during model training, and let UE to feedback which associated IDs are needed, or the grouping of associated IDs. NW can then at least decide whether associated ID is needed for the data collection configuration of model inference based on UE feedback. Moreover, NW can also adjust the configuration of associated IDs for model training, for example, releasing some IDs not useful.

In addition, the working assumption only suggests UE assumption of the consistency within a cell, it implies cell identifications may be inserted into the associated ID and UE may need to train and maintain a large number of models, which introduces unnecessary complexities for UE. On the other hand, if associated ID is PLMN unique then it may introduce quite some unnecessary difficulties considering inter-vendor coordination of many networks. As one solution, grouping of a subset of cells can be considered. Meanwhile, the UE feedback procedure above can also be used to check whether the consistency holds across cells, at least when UE switches to a new cell and receives data collection configurations with associated ID, it can send NW indications that this associated ID is needed or not, at least for model inference.

Not every difference in NW-side additional conditions requires a data collection configuration with a separated associated ID.

If the associated ID is only for UE assumption of the consistency within a cell, it may introduce unnecessary complexities for UE to train and to maintain a large number of models. If the associated ID is valid for UE assumption of the consistency within PLMN, it may introduce unnecessary difficulties considering inter-vendor coordination.

To ensure the consistency within a cell and across multiple cells, support UE to feedback whether associated ID is needed, at least for model inference.

Study the grouping of cells that can ensure the consistency within a subset of cells

  - Performance Monitoring

For AI/ML model performance, methods should be identified to support the monitoring of AI/ML model performance and the required feedback signalling. The TR concluded several methods for AI/ML model monitoring, based on inference accuracy, system performance, data distribution, applicable condition respectively.

It can be observed that each method has its own application scenario, since the cause of model failure may be different. Supporting more than one of monitoring methods seems inevitable. If the monitoring method is determined at NW, configuration information of model monitoring method should be provided to UE. If the monitoring method is determined at UE, together with the model monitoring results, it may report the applied monitoring method, or it may report the cause of model failure if the monitoring results implying the model has been failed. In addition, considering the different candidate monitoring methods, UE capability on the supported methods of model monitoring shall also be reported.

Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

RAN1 had also considered the mechanism of monitoring inactive model/functionality for the purpose of model management decision making (e.g. activation/deactivation), however more discussions are required for the details of the procedure. Monitoring of inactive model/functionality is useful when a new model needs to be deployed in the network whose real time performance over different cells still needs to be validated. In such a case, the model/functionality shall be run at UE only for monitoring purpose without using the model/functionality for actual radio operation. For instance, in UE based beam prediction, UE may run legacy operation of beam measurements (e.g. by measuring full set of SSBs/CSI-RS) while at the same time a model/functionality could be run at UE (performing beam prediction) whose monitoring is performed to determine the performance of the given model/functionality.

Specify monitoring of inactive model/functionality for the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities for Rel-19 AI/ML.

Subsequently, it needs to be discussed, when an inactive model/functionality is being monitored, whether UE should run legacy operation or can UE activate a different model/functionality simultaneously for radio operations. For example, model-1 may be inactive but being monitored for beam prediction and at the same time model-2 may be activated at the same UE for beam prediction through which radio operations are being enabled. Such operation can allow testing of newly deployed AI/ML models and at the same time continuing the radio operation using older well-established AI/ML models. The UE capability and UE Assistant Information(UAI) for supporting such procedure needs to be investigated to ensure feasibility.

Concurrent inference operation of two or more models/functionalities at a UE (where one model/functionality is inactive but being monitored and other model/functionality is activated at UE) allows testing of newly deployed AI/ML model/functionality (using inactive model operation) and at the same time continuing the radio operation using older well-established AI/ML model/functionality.

Discuss whether a UE can perform inference of two models/functionalities concurrently where one model/functionality is inactive but being monitored and other model/functionality is activated at UE.

  - Management

Although explicit LCM signalling can be used to indicate AI/ML functionality/model selection, activation/deactivation/switching, adaptive selection of applied AI/ML functionality/model can greatly reduce signalling overhead, and possibly latency. For example, if different AI/ML functionalities/models are available for LOS/NLOS, high/low SINR, high/low velocity, more/less antenna ports/beams respectively, model/functionality switching can be adaptive to the change of configurations or the detection of change of the scenarios.

Support adaptive model/functionality selection, activation, deactivation, switching, and fallback.

At least for UE side AI/ML functionality/model management, event triggered method may be considered, a timer and a counter can be used to control the management of the AI/ML functionality/model as follows. UE could activate candidate model(s) for performance assessment before switching to it.

- For AI/ML performance to be monitored, UE higher layer can initialize a counter to 0

- For an assessment duration, UE could monitor the performance of the AI/ML

- Based on assessment in the assessment duration, UE PHY layer may generate an indication to UE higher layer to indicate the performance issue of the AI/ML

- UE higher layer may start a timer when receives the indication from PHY layer for the first time

- While the timer is running,

  - when UE higher layer receives the indication from PHY layer, UE higher layer may increase the counter by 1 and restart the timer

  - If the counter reaches a threshold value, UE higher layer can make decision to deactivate the current AI/ML model

  - If the counter reaches another threshold value, UE higher layer can make decision to assess candidate model(s), if the candidate model(s) is inactive, it can be activated

- When the timer expires, the counter is set to 0

Support event triggered AI/ML functionality/model activation/deactivation/switching.

- Discussion on Collaboration Levels and Model Transfer

Network-UE collaboration levels x, y and z are considered during the Rel-18 study phase, and for Level z: signalling-based collaboration with model transfer, 5 different assumptions for model transfer are discussed, as in the following table.

[1] Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization..

Several benefits of model transfer were identified during the study phase for AI/ML operation. From our perspective, supporting scenario/configuration specific models has one of the highest priorities. Although scenario/configuration specific models have not been actively considered in 3GPP, as the deployment of AI/ML accelerates, there is expected to be significant need of such models to achieve the highest possible network performance given large variations of network deployment from cell to cell. For instance, beam management study has already identified the impact of cell topology/deployment on the performance of AI/ML algorithms and various mechanisms are being considered e.g. additional conditions, dataset identifiers to handle different deployment scenarios. However, as each cell deployment is a combination of various factors (cell layout, antenna configuration, types of UEs, etc.) each of which individually impacting the AI/ML performance, covering all such combinations requires deployment of large number of AI/ML models which introduces significant burden at UE in terms of model maintenance and memory consumption. Hence, model transfer is essential to cater to such deployments where scenario/configuration specific models can be deployed at UE by gNB on demand.

Supporting model transfer is essential when considering cell/scenario-specific AI/ML deployment which is expected to happen when AI/ML deployment accelerates.

In RAN1#118, companies agreed to focus on case z4 to enable model transfer to cater for the case of 2-sided model operation for CSI compression.

For model transfer methodology z4, some following agreement in RAN1#117 was reached to study Alt. A and Alt. B. The main difference between the two alternatives is that in Alt. A the indication of UE supported AI/ML models is the first step while in Alt. B, UE indicates its supported models after gNB indicates candidate models. Note that for Alt. A, UE would need to provide indication of all supported AI/ML model structures to gNB, this may also include AI/ML models which might not be supported by the gNB (e.g. applicable for different network vendor). Exchanging this information can result in high signaling overhead for UE capability exchange which is already quite congested. Hence, we think it is more appropriate for gNB to first indicate its candidate model/model structures (based on initial information from UE in Step B0), and thereby UE responding which of the candidate models are supported.

Support Alt. B for model transfer methodology z4.

In Step B-0, UE reports to NW (within UE capability information) that model transfer is supported for which AI/ML features (in Rel-19 only CSI compression use case)

Step B-1: NW indicates to UE the candidate known model structure(s)

Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported

Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2

- UE Capability Reporting and UE Assistance Information Reporting

During the SI, RAN1 also discussed the need to study how to handle UE’s internal conditions such as memory, battery which may impact AI/ML operation. Given that AI/ML models are expected to be more intensive as compared to legacy operation and may require dedicated UE hardware to optimise its operations, it seems natural to support handling UE internal conditions properly. While for the case of UE-based AI/ML operation, such kind of intervention can be performed locally at UE without any awareness of 3GPP network, for the case when AI/ML model operation is controlled by the network (e.g., activation/deactivation) it is important to discuss how UE can indicate its internal conditions to the network for optimal AI/ML operation.

It is important to discuss how UE can indicate its internal restrictions to activate or run an AI/ML model/functionality to the network for optimal AI/ML operation.

There are two types of indications from UE to network which can be supported for such application. In the first approach, UE can indicate to the network its detailed status of memory size, battery level and other hardware limitations to the network like UE Assistance Information message. However, this may involve proprietary information disclosure of UE’s operation which may be undesirable. Further it is not clear how network would be able to ascertain based on the assistance information whether UE can run an AI/ML model/functionality as AI/ML operation footprint shall be UE implementation specific.

Reporting of UE’s internal conditions such as memory size, battery level and other detailed hardware limitations to gNB for AI/ML operation may lead to UE’s proprietary information disclosure and may be hard for network to determine AI/ML applicability for a UE based on the provided information.

In other approach, UE can indicate whether it can run an AI/ML functionality/model after gNB indicates an AI/ML model/functionality for configuration or activation. UE, after receiving AI/ML information, shall determine whether it can run the AI/ML model/functionality or not based on its internal conditions. If the UE determines that it shall not be able to run an AI/ML model/functionality, it can indicate that to network about the restriction and can also provide a relevant cause value (e.g., memory limitation) which can allow network to understand the reason of failure and appropriate next action. Such a procedure keeps implementation simple and efficient.

Specify UE indication to network about its inability to run a configured/activated AI/ML model/functionality due to UE’s internal condition along with a relevant cause value for the failure.

- Conclusion

In this contribution, we provided our views on Rel-19 AI/ML for air-interface, and we have the following observations and proposals:

Observation 1:	To ensure consistency, the option based on associated ID is supported. And the associated ID is determined during model identification.

Observation 2:	MI-Option 1 is suitable for adoption if Option-4 (Standardized data / dataset format + Dataset exchange between NW-side and UE-side) is agreed to address inter-vendor collaboration for CSI compression use case.

Observation 3:	MI-Option 3 is suitable for adoption if Option-3b/5b (Parameters/models received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations) is agreed to address inter-vendor collaboration for CSI compression use case.

Observation 4:	A combination of MI-Option 1 and MI-Option 3 is suitable for adoption if Option-3a/5a (Parameters/models received at the UE or UE-side goes through offline engineering at the UE-side) is agreed to address inter-vendor collaboration for CSI compression use case.

Observation 5:	Ensuring consistency of additional conditions using monitoring procedure results in high delay in identification of the suitable AI/ML model to run at UE, during which system performance suffers.

Observation 6:	Not every difference in NW-side additional conditions requires a data collection configuration with a separated associated ID.

Observation 7:	If the associated ID is only for UE assumption of the consistency within a cell, it may introduce unnecessary complexities for UE to train and to maintain a large number of models. If the associated ID is valid for UE assumption of the consistency within PLMN, it may introduce unnecessary difficulties considering inter-vendor coordination.

Observation 8:	Concurrent inference operation of two or more models/functionalities at a UE (where one model/functionality is inactive but being monitored and other model/functionality is activated at UE) allows testing of newly deployed AI/ML model/functionality (using inactive model operation) and at the same time continuing the radio operation using older well-established AI/ML model/functionality.

Observation 9:	Supporting model transfer is essential when considering cell/scenario-specific AI/ML deployment which is expected to happen when AI/ML deployment accelerates.

Observation 10:	It is important to discuss how UE can indicate its internal restrictions to activate or run an AI/ML model/functionality to the network for optimal AI/ML operation.

Observation 11:	Reporting of UE’s internal conditions such as memory size, battery level and other detailed hardware limitations to gNB for AI/ML operation may lead to UE’s proprietary information disclosure and may be hard for network to determine AI/ML applicability for a UE based on the provided information.

Proposal 1:	Clarify the support of model identification for one-sided model use cases, at least when associated ID is needed to ensure the consistency between model training and model inference.

Proposal 2:	RAN1 to select the model identification procedure based on the outcome of the study to address inter-vendor collaboration issues for CSI compression use case.

−	MI-Option 1 to be adopted if Option-4 (Standardized data / dataset format + Dataset exchange between NW-side and UE-side) is agreed to address inter-vendor collaboration issues for CSI compression use case.

−	MI-Option 3 to be adopted if Option-3b/5b (Parameters/models received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations) is agreed to address inter-vendor collaboration issues for CSI compression use case.

−	A combination of MI-Option 1 and MI-Option 3 to be adopted if Option-3a/5a (Parameters/models received at the UE or UE-side goes through offline engineering at the UE-side) is agreed to address inter-vendor collaboration issues for CSI compression use case.

Proposal 3:	One or more associated ID(s) can be attached to one same model ID to reflect different NW side additional conditions.

Proposal 4:	In the model identification procedure, support the combination of Alt.1: NW assigns Model ID and Alt.2: UE assigns/reports Model ID, which is used to link a UE reported global model ID to a NW assigned local model ID.

Proposal 5:	For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options should be considered as priority:

−	Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

−	Model training at NW and transfer to UE, where the model has been trained under the additional condition

−	Information and/or indication on NW-side additional conditions is provided to UE

Proposal 6:	To ensure the consistency within a cell and across multiple cells, support UE to feedback whether associated ID is needed, at least for model inference.

Proposal 7:	Study the grouping of cells that can ensure the consistency within a subset of cells

Proposal 8:	Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

Proposal 9:	Specify monitoring of inactive model/functionality for the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities for Rel-19 AI/ML.

Proposal 10:	Discuss whether a UE can perform inference of two models/functionalities concurrently where one model/functionality is inactive but being monitored and other model/functionality is activated at UE.

Proposal 11:	Support adaptive model/functionality selection, activation, deactivation, switching, and fallback.

Proposal 12:	Support event triggered AI/ML functionality/model activation/deactivation/switching.

Proposal 13:	Support Alt. B for model transfer methodology z4.

−	In Step B-0, UE reports to NW (within UE capability information) that model transfer is supported for which AI/ML features (in Rel-19 only CSI compression use case)

−	Step B-1: NW indicates to UE the candidate known model structure(s)

−	Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported

−	Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2

Proposal 14:	Specify UE indication to network about its inability to run a configured/activated AI/ML model/functionality due to UE’s internal condition along with a relevant cause value for the failure.

- References

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Chair’s Notes, 3GPP TSG RAN WG1 #118, Maastricht, NL, August 19th – 23rd, 2024