3GPP TSG RAN WG1 #118-bis		                                                    R1-2408229

Hefei, China, 19th – 23rd October, 2024

Agenda Item:	9.11.4

Source:	NEC

Title:	IoT-NTN uplink capacity/throughput enhancement

Document for:	Discussion

- Introduction

In RAN1#118, the following agreements on IoT-NTN uplink capacity/throughput enhancement have been established[1]:

Agreement RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity - UL gaps for synchronization (from Rel-13) - Gaps around NPRACH occasions - UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) - TDM DMRS that are muted - Guard periods for 3.75kHz UL transmissions Agreement The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. Agreement For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

In this contribution, we provide further views on the uplink capacity enhancement for IoT-NTN.

- NPUSCH

For the single carrier NPUSCH with SCS 3.75 kHz, where 5 slots are a frame, the slot duration is 61440 Ts or 2 ms, and each slot contains 7 symbols [2]. Thus, the gap between any two DMRS symbols will be an integer multiplied by 2/7 ms.

Based on the agreements in RAN1#116-bis, the frequency error is a uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs, and the carrier frequency is 2 GHz [1]. Thus, the maximum frequency error is 400 Hz.

To ensure the gNB could estimate the frequency error based on the DMRS symbols without phase rotation ambiguity, the maximum phase rotation between two DMRS symbols should be smaller than 2π. As the maximum frequency error is 400Hz, the gap between two DMRS symbols should be smaller than 2.5 ms, which is 8.75 symbols for SCS 3.75 KHz and 35 symbols for 15 KHz. Thus, a maximum distance between two DMRS symbols of 8 symbols for 3.75 KHz and 35 symbols for 15 KHz could meet the CFO estimation requirement of the current simulation assumption.

Observation 1: To meet the CFO estimation requirement of the current simulation assumption, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 2.5 ms.

Observation 2: For the single carrier, NPUSCH with 3.75 KHz SCS, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 8 symbols to meet the CFO estimation requirement.

Observation 3: For the single carrier, NPUSCH with 15 KHz SCS, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 35 symbols (5 slots) to meet the CFO estimation requirement.

Proposal 1: For the single carrier, NPUSCH with 3.75 KHz SCS, a new DMRS pattern should be studied to meet the CFO estimation requirement and the following options should be considered:

- Option 2: OCC2, Symbol-level, CDM DMRS with new pattern

- Option 6: OCC4, Symbol-level, CDM DMRS with new pattern

Proposal 2: For the single carrier, NPUSCH with 15 KHz SCS, Rel-17 DMRS pattern could meet the CFO estimation requirement  and the following options should be considered:

- Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

- Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern

The two-level gap is proposed in [3], which is similar to the NPRACH “two-level hopping”. Within a DMRS cluster, the gap between two multiplexed consecutive DMRS symbols is x1. Between two DMRS clusters, the gap between two multiplexed consecutive DMRS symbols is x2. For symbol-level OCC of 3.75 KHz SCS NPUSCH, the x1 is strictly constrained to meet the CFO estimation requirement, and the x2 is constrained to retain the same DMRS overhead as the legacy.

Proposal 3: For symbol-level OCC, the multiplexed DMRS pattern contains the first level gap between two consecutive DMRS symbols within a DMRS cluster and the second level gap between two consecutive DMRS symbols across two DMRS clusters, where the first level gap should meet the CFO estimation requirement and the second level gap should keep the DMRS overhead same as the legacy NPUSCH without OCC.

- For 3.75 KHz SCS, the first level gap should be smaller or equal to 8 symbols.

- NPRACH

The legacy Random Access procedure is performed on the anchor carrier or one of the non-anchor carriers for which the PRACH resource has been configured in system information. For a legacy NB-IoT UE, when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities [4]:

- the selection probability of the anchor carrier PRACH resource for the given enhanced coverage level, nprach-ProbabilityAnchor, is given by the corresponding entry in nprach-ProbabilityAnchorList

- the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

Proposal 4: Support study if the anchor carrier and non-anchor carrier selection probability could be enhanced to multiplex NPRACH with OCC and legacy NPRACH  in uplink capacity enhancement for IoT-NTN.

For the legacy NPRACH preamble component, each preamble repetition contains P symbol groups, each group contains one CP + N symbols as illustrated in Fig. 1 and Fig. 2.

Fig. 1 NPRACH symbol

Fig. 2 NPRACH symbol group

For intra-symbol group OCC, additional CP might be required to ensure the orthogonal property. In an NPRACH symbol group, all the symbols are identical. Thus, the third symbol of NPRACH Format 1 can be treated as the second CP for the NPRACH symbols multiplexed with the second OCC code word as illustrated in Fig. 3. for OCC length = 2 cases.

Fig. 3 NPRACH intra-symbol group OCC

From the UE perspective, UE multiplex the S1 and S2 with the first code word; S3, S4, and S5 with the second code word, and then added the CP as legacy. There is no need to specify a new NPRACH symbol group structure or add new procedures for processing the additional CP.

From the gNB perspective, for any NPRACH with intra-symbol group OCC, the receiver could treat the third symbol of an NPACH symbol group as a CP.

Proposal 5: For NPRACH with OCC, the intra-symbol group OCC should be supported at least with the OCC length =2 cases.

For legacy NPRACH, the supported preamble repetition numbers are 1, 2, 4, 8, 16, 32, 64, and 128. During the NPRACH transmission, there are three different levels of frequency hopping for preamble format 0 / 1 in frame structure type 1 as illustrated in Fig. 3:

- Level 1 Hopping: between the 1st and 2nd symbol group, and the 3rd and 4th symbol group, hopping one subcarrier

- Level 2 Hopping: hopping 6 subcarriers between the 2nd and 3rd symbol group

- Level 3 Hopping: hopping between each repetition in a pseudo-random fashion within 12 contiguous subcarriers

For inter-symbol group(s) OCC or inter-repetition OCC, the hopping pattern/rule might be different to fit with the multiplex symbol group(s). The potential frequency hopping patterns are illustrated in Fig. 4 and Fig. 5 for inter-symbol group(s) OCC and inter-repetition OCC, respectively.

Fig. 4 Inter-symbol group(s) OCC frequency hopping

Fig. 5 Inter-repetition OCC frequency hopping

Proposal 6: Support inter-symbol group(s) OCC and inter-repetition OCC for NPRACH with a modified frequency hopping mechanism.

- FFS: the hopping mechanisms

For PUCCH with OCC, the codebook index is configured by the Network. During a random access procedure, the codebook index will be selected randomly.  The UEs might select the same codebook index resulting in codebook index conflicts.

Proposal 7: Support study of the performance of different codebook index hopping mechanisms for NPRACH with OCC in uplink capacity enhancement for IoT-NTN.

Without losing the orthogonal properties of the OCC, more accurate timing and frequency synchronizing might be required. For legacy NPRACH transmission, the segment transmission and gap are configured by the network for UE to handle the precompensations [4].

Proposal 8: Support study on whether the timing and frequency synchronizing should be enhanced to support NPRACH with OCC in uplink capacity enhancement for IoT-NTN.

- Conclusion

From the discussion, we have the following observations and proposals.

Observation 1: To meet the CFO estimation requirement of the current simulation assumption, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 2.5 ms.

Observation 2: For the single carrier, NPUSCH with 3.75 KHz SCS, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 8 symbols to meet the CFO estimation requirement.

Observation 3: For the single carrier, NPUSCH with 15 KHz SCS, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 35 symbols (5 slots) to meet the CFO estimation requirement.

Proposal 1: For the single carrier, NPUSCH with 3.75 KHz SCS, a new DMRS pattern should be studied to meet the CFO estimation requirement and the following options should be considered:

- Option 2: OCC2, Symbol-level, CDM DMRS with new pattern

- Option 6: OCC4, Symbol-level, CDM DMRS with new pattern

Proposal 2: For the single carrier, NPUSCH with 15 KHz SCS, Rel-17 DMRS pattern could meet the CFO estimation requirement  and the following options should be considered:

- Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

- Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern

Proposal 3: For symbol-level OCC, the multiplexed DMRS pattern contains the first level gap between two consecutive DMRS symbols within a DMRS cluster and the second level gap between two consecutive DMRS symbols across two DMRS clusters, where the first level gap should meet the CFO estimation requirement and the second level gap should keep the DMRS overhead same as the legacy NPUSCH without OCC.

- For 3.75 KHz SCS, the first level gap should be smaller or equal to 8 symbols.

Proposal 4: Support study if the anchor carrier and non-anchor carrier selection probability could be enhanced to multiplex NPRACH with OCC and legacy NPRACH  in uplink capacity enhancement for IoT-NTN.

Proposal 5: For NPRACH with OCC, the intra-symbol group OCC should be supported at least with the OCC length =2 cases.

Proposal 6: Support inter-symbol group(s) OCC and inter-repetition OCC for NPRACH with a modified frequency hopping mechanism.

- FFS: the hopping mechanisms

Proposal 7: Support study of the performance of different codebook index hopping mechanisms for NPRACH with OCC in uplink capacity enhancement for IoT-NTN.

Proposal 8: Support study on whether the timing and frequency synchronizing should be enhanced to support NPRACH with OCC in uplink capacity enhancement for IoT-NTN.

References

- Chair’s Notes RAN1 #118

- TS 36.211

- R1-2405175, “IOT-NTN uplink capacity/throughput enhancement”, Qualcomm Incorporated

- TS 36.321