3GPP TSG RAN WG1 #118bis	 R1- 2408236

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.6.1

Source:	HONOR

Title:	Discussion on LP-WUS and LP-SS design

Document for:	Discussion and Decision

Introduction

In RAN#102, a new Rel-19 WID “Low-power wake-up signal and receiver for NR (LP-WUS/WUR)” has been approved with the following objective as one of the objectives [1]:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported

In this contribution, we provide input for the above objective. The following aspects of LP-WUS design will be discussed: waveform generation, overlaid OFDM sequences and content.

- Discussion

  - LP-WUS design

Waveform

Waveform generation

For the waveform, RAN1# 118 meetings have made the following agreements [2].

Agreement [H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4.

Supporting M=4 for OOK-4 can provide a more flexible spectrum efficiency range for fixed LP-WUS resources. In addition, from the simulation results below, there is no obvious performance degradation when M=4. Therefore, it is recommended to support M=4 for 30KHz SCS.

Figure 1. Detection performance of OOK-4 without time-frequency errors

Figure 2. Detection performance of OOK-4 with time error of 0.5us

Proposal 1：Support M=4 for 30kHZ SCS.

From the following simulation results, we can see that the simulation performance of OOK-1 is better than that of M=1 for OOK-4. In addition, from section 2.1.1.2, we can see that OOK-1 is obviously simpler to implement than OOK-4. Based on the above analysis, it is recommended not to support M=1 for OOK-4.

Figure 3. Comparison of detection performance of OOK-1 and OOK-4 for M=1

Proposal 2: Do not support M=1 for OOK-4.

Different subcarrier spacings will affect the OOK symbol length, thus having a certain impact on the performance. However, considering that M has only two cases, 2 and 4, and the simplification of implementation, it is recommended to decouple the value of M from the subcarrier spacing.

Proposal 3: The value of M is independent of SCS.

Specifying OOK-1 and OOK-4

RAN1# 116 meetings have made the following agreements [5].

Agreement Support both OOK-1 and OOK-4 for LP-WUS. - FFS how OOK-1 and OOK-4 are specified - For OOK-4, M<=4, FFS supported values - The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol FFS different SCS.

Currently, there are only two generation methods of LP-WUS waveforms: OOK-1 and OOK-4. As described in [6]:

-	Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are

-	OOK=1 means all SCs are modulated

-	OOK=0 means all SCs are zero power (from base-band point of view)

- Option OOK-4: Transform M-bit OOK in time domain

-	N SCs of OOK-1 are generated by a transformation (DFT/Least square)

-	N’ samples are generated from M-bits

-	signal modification may or may NOT be used

-	truncation or other additional modification may or may NOT be used, if not used, N is the same as N’

-	N’ can be the same as K

The primary design target of LP-WUS is to simplify the implementation and reduce the power consumption of the receiver. Therefore, when considering the specific design of OOK-1 and OOK-4, it is necessary to simplify the reception of the UE as much as possible. Therefore, only necessary steps need to be retained, and other performance improvements are left to the base station side and transparent to the receiving side.

Proposal 4：Specify only the necessary steps for the design of OOK-1 and OOK-4.

According to the above principles, for OOK-1, only the modulated overlaid sequence needs to be specified. For OOK-4, only the sequence mapping and DFT steps are required. The purpose of sequence mapping is to map the time domain information to the overlaid sequence, and then convert the time domain signal to the frequency domain through DFT for OFDM modulation.

Proposal 5: Specifies only the overlaid sequence for OOK-1.

Proposal 6: Specifies the two steps of sequence mapping and DFT for OOK-4.

Since both OOK-1 and OOK-4 are supported, the UE side needs to know in advance the specific OOK waveform generation scheme used by the network side. Therefore, it is necessary to discuss how the UE obtains the OOK waveform generation scheme adopted by the network side.

Proposal 7: Further discuss how the UE obtains the OOK waveform generation scheme.

Overlaid OFDM sequences

Sequence type

RAN1# 117 meetings have made the following agreements [3].

Agreement Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS - Down select among gold sequence, m sequence and ZC sequence - FFS the overlaid OFDM sequence is time or frequency domain sequence. - FFS how to reuse the existing sequences - Note: Strive to minimize the impact on OOK detection performance - If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS

According to the evaluation results of R18 LP-WUS SI, OOK-1 and OOK-4 can improve detection performance by using overlaid OFDM sequences. On the one hand, the OFDM sequence can make the spectrum of the OOK waveform more uniform in the frequency domain. On the other hand, the receiving side can use known sequence information to improve detection performance. It can be seen from the existing evaluation results that the receiver using sequences for correlation is simpler and has lower power consumption than the OFDM receiver with the I/Q branches.

Observation 1: Using the correlation of sequences, the implementation complexity and power consumption of the receiver can be reduced.

When designing the NR PSS, the performance of the M sequence and the ZC sequence was evaluated, and the M sequence was finally selected based on the performance advantage of anti-frequency offset. In Figure 4, we also simulated the performance without frequency errors, and the performance of M sequence is slightly better than that of ZC sequence. Furthermore, some terminals still use PSS/SSS for synchronization for LP-WUS detection. Therefore, it can be considered to use the same sequence for LP-WUS and PSS/SSS to reduce the complexity of the receiver. In addition, it is best to use the same sequence type for the design of LP-SS, which not only ensures performance but also enables unified design.

Observation 2: The M sequence not only has better anti-frequency offset performance but also can reduce the complexity of the receiving side.

Figure 4. Comparison of detection performance of M, Gold and ZC sequences

Proposal 8: Prioritize M sequence for overlaid sequence.

For OOK-4, we compared the direct use of M sequence (BPSK modulation) and the generation of random phase based on M sequence. As shown in Figure 5 below, the performance of the two solutions is not much different, so we recommend that the existing M sequence can be used directly.

Figure 5. Comparison of detection performance using M sequence directly and generating random phase based on M sequence

Observation 3: There is no obvious benefit in generating overlaid sequences based on M sequences to generate random phases.

Proposal 9: Directly use the existing M sequence as the overlaid sequence.

Specifying overlaid OFDM Sequences for OOK

RAN1# 118 meetings have made the following agreements.

Agreement For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1. - Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing Agreement For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported) - Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing - Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing - Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary

Based on the conclusions of the last meeting, only the overlaid sequence generation method of OOK-1 and OOK-4 with M=1 needs to be discussed. However, based on the analysis in section 2.1.1.1, we suggest that OOK-4 with M=1 does not need to be supported. Therefore, here we only discuss the overlaid sequence generation method of OOK-1.

The generation method of the overlaid sequence needs to be selected on the premise of first adapting to the OOK modulation scheme. For OOK-1, as described in Section 2.1.1.2, the generation method in the frequency domain is the simplest and most effective. Therefore, the overlaid sequence obviously also needs to be generated in the frequency domain. Introducing DFT processing does not bring any performance benefits and only increases the implementation complexity. Therefore, we recommend that OOK-1 adopt option 2.

Proposal 10: The overlaid sequence generation method of OOK-4 with M=1 is not supported.

Proposal 11: OOK-1 adopts the overlaid sequence generation method of option 2.

Methods of carrying information

RAN1# 116-bis meetings have made the following agreements [4].

Agreement Regarding the overlaid OFDM sequence(s) of LP-WUS, consider the following options: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol or OFDM symbol duration. OFDM-based LP-WUR can obtain the whole information bits by the presence of the overlaid sequence. - Option 1-2: The overlaid OFDM sequence is pre-determined from multiple sequences. This sequence carry NO information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the OOK ON/OFF pattern. - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - Option 3: One sequence is selected from multiple candidates overlaid OFDM sequences on one or more OOK ‘ON’ symbols, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). - Option 4: Use of modulated overlay sequence with constellation point: overlay sequence acting as a spreading sequence and constellation point carrying information for OFDM-based LP-WUR. Other options are not precluded.

Option 1 has the simplest implementation complexity for the receiving side, and only requires correlation of a single sequence. However, since each OOK ‘ON’ symbol needs to be detected and the OOK information bit transmission time is long, the power consumption is also high. Option 1-2 is similar to option 1, except that the number of sequences is greater. More sequences can be used for coordination between multiple cells, but also increase the complexity.

In Option 2, different bits of information are matched through sequences, which can reduce the number of overlaid sequence detections compared to Option 1. Since the sequence carries less bit information, it is also necessary to obtain all the information bits through the combination of sequence position information. This increases the complexity of the receiver implementation.

Option 4 increases the complexity of implementation and demodulation and does not bring any additional benefits. Therefore, it is not recommended.

Option 3 completely uses sequences to correspond to different bit information. Compared with option 2, it does not need to combine position information, but the number of sequences during sequence detection increases significantly. However, Option 3 can obtain information in a shorter time by only detecting the overlaid sequence. It is recommended to give priority to Option 3.

Proposal 12：Support option 3 for the overlaid OFDM sequence(s) of LP-WUS.

Content

For RRC idle/inactive state, the following agreements have been reached in the last meeting.

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.

Regarding X, when X is greater than 1, it can be applied to the scenario where multiple subgroups are paged at the same time. That is, each subgroup corresponds to an MO, and different MOs can indicate different subgroups.

The number of subgroups paged at the same time is related to the paging rate and the number of users, so X is not a fixed value. Therefore, based on flexibility considerations, X can be configured by the network side.

Proposal 13: Support X can be configured by the network side.

Since the paging probability is usually very low, the number of subgroups that are paged simultaneously is less than or equal to 1 in most scenarios. In this case, if the value of X is greater than 1, if the UE needs to detect all MOs, most detections are unnecessary.

Observation 4: Since the number of subgroups that are paged simultaneously is less than or equal to 1 in most scenarios, if the UE detects all X MOs, then most detections are necessarily invalid and result in increased energy consumption.

In order to reduce the number of UE blind detections and energy consumption, it is necessary to further study the sending rules and design of multiple code points. For example, sending in ascending order of code point values ​​can allow UEs with small code point values ​​to end detection early, thereby reducing energy consumption.

Proposal 14: Further study the sending rules and detailed design of multiple code points to reduce the number of UE blind detections and energy consumption e.g., sent in the order of code point values.

Figure 6. Multiple code points are sent in different MOs

For the RRC connection state, RAN1# 116-bis has reached the following agreements.

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA

For RRC connected UEs, since the network side has obtained the user's location and other information, the indication of the WUS information is only related to the number of users. When the number of users is large, the number of users awakened at the same time may be large, and more information needs to be carried. Conversely, less information can be sent. Therefore, we recommend supporting both option 1 and option 3, so that the network side can flexibly indicate which solution to use.

Proposal 15: Support both option 1 and option 3, and the network side can flexibly indicate the specific solution.

Regarding CA, for option 1, since the bitmap carries more information, the carrier information can be carried only on the overlaid sequence. For option 3, since the number of information bits of the code point is small, the carrier information can be carried through OOK bits or overlaid sequences. However, to maintain a unified design, it can also be carried only through the overlaid sequence. Therefore, the carrier information of CA is carried through the overlaid sequence.

Proposal 16: Carrier information is carried through the overlaid sequence in the CA scenario.

  - LP-SS design

Waveform

For the waveform, RAN1# 116-bis meetings have made the following agreements.

Working assumption Support the following options for LP-SS - Option 1: OOK-1 - Option 2: OOK-4 with M=2,4, FFS:1,8,16 - FFS whether value of M depends on SCS - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation FFS how OOK-1 and OOK-4 are specified

In order to simplify the implementation complexity, it is recommended that the LP-SS and LP-WUS signals use the same waveform generation method. In addition, although M=8 and 16 can improve synchronization accuracy, the tolerance to time errors is poor and the performance cannot be guaranteed. Therefore, it is not recommended to support. Regarding the relationship between the value of M and SCS, referring to the analysis in LP-WUS, it is recommended to decouple the two.

Proposal 17: Confirm the following working assumption:

Support the following options for LP-SS

- Option 1: OOK-1

- Option 2: OOK-4 with M=2,4

- The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation

Sequence type

RAN1# 117 and RAN1# 118 meetings have made the following agreements.

Agreement For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following: - Gold sequence - M sequence - Computer searched sequence - FFS: the length of LP-SS sequence Agreement The number of binary LP-SS sequences is 4. Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption

From the generation principle of gold sequence and M sequence, the performance difference between the two is not obvious. Gold sequence is proposed to solve the problem that there are few m sequences and the cross-correlation function value between m sequences is not ideal. It is obtained by adding a pair of m sequences with the same period and rate modulo 2. However, the number of sequences required here is not large, so the demand for cross-correlation is weaker than autocorrelation. In addition, M sequence has a very good balance characteristic, that is, the number of 1 and 0 is almost equal. Therefore, from the above analysis, M sequence is more suitable than gold sequence.

For the computer search sequence, we refer to the sequence in [7]. Then, as shown in Figure 7-13, we compare the performance of correlation, synchronization and RRM measurement of M sequence, gold sequence and computer search sequence. From the simulation results, there is no obvious difference. Therefore, to simplify the implementation and reduce the workload, we recommend that LP-SS use M sequences.

Proposal 18: Prioritize M sequence for LP-SS.

Figure 7. Comparison of autocorrelation of M sequence, gold sequence and computer search sequence

Figure 8. Comparison of synchronization accuracy among M sequence, gold sequence and computer search sequence under OOK-1

Figure 9. Comparison of synchronization accuracy among M sequence, gold sequence and computer search sequence under OOK-4 with M=2

Figure 10. Comparison of synchronization accuracy among M sequence, gold sequence and computer search sequence under OOK-4 with M=4

Figure 11. Comparison of RSRP measurement performance of M sequence, gold sequence and computer search sequence under OOK-1

Figure 12. Comparison of RSRP measurement performance of M sequence, gold sequence and computer search sequence under OOK-4 with M=2

Figure 13. Comparison of RSRP measurement performance of M sequence, gold sequence and computer search sequence under OOK-4 with M=4

The following is a simulation verification of the synchronization and RRM measurement performance of the M sequence based on the above simulation conditions. Figures 14-16 show the simulation results of the synchronization performance, and Figures 17-22 show the simulation results of the RRM measurement.

As can be seen from figures 14 and 15, for OOK-1 and OOK-4 with M=2, an M sequence with a length of 8 can meet the synchronization performance of 2us and increasing the length can further improve the synchronization performance. As can be seen from Figure 16, for OOK-4 with M=4, an M sequence with a length of 32 can meet the synchronization performance of about 1us and increasing the length can further improve the synchronization performance.

Observation 5: The synchronization performance is related to the sequence length:

- to achieve the synchronization requirement of 2us, a sequence length of more than 8 is required,

- while to achieve the synchronization requirement of 1us, a sequence length of more than 32 is required.

Figure 14. Synchronization performance of the M sequence based on OOK-1

Figure 15. Synchronization performance of the M sequence based on OOK-4 M=2

Figure 16. Synchronization performance of the M sequence based on OOK-4 M=4

As shown in Figure 17, for OOK-1, an M sequence of length 4 can meet the RSRP measurement accuracy requirement of 4db and increasing the sequence length can significantly improve the measurement accuracy. As shown in Figure 18, increasing the number of samples can also significantly improve the RSRP measurement accuracy. As shown in Figure 19-22, under the same sequence length and sample, the larger the M value, the worse the measurement accuracy. Therefore, through observation we can draw the following conclusions.

Observation 6: The RRM measurement accuracy is related to the OOK chip length and can be improved by increasing the M sequence length and the number of samples.

- For OOK-1, an M sequence length of 4 in a single sample can meet the measurement accuracy requirements.

- - For OOK-4 with M=2, an M sequence length of at least 8 in a single sample can meet the measurement accuracy requirements.

- - For OOK-4 with M=4, an M sequence length of at least 16 in a single sample to meet the measurement accuracy requirements.

Figure 17. RRM measurement performance under single sampling based on OOK-1

Figure 18. RRM measurement performance under multi-sampling based on OOK-1

Figure 19. RRM measurement performance under single sampling based on OOK-4 with M=2

Figure 20. RRM measurement performance under multi-sampling based on OOK-4 with M=2

Figure 21. RRM measurement performance under single sampling based on OOK-4 with M=4

Figure 22. RRM measurement performance under multi-sampling based on OOK-4 with M=4

Overlaid OFDM sequences

RAN1# 116 meetings have made the following agreements.

Agreement For the overlaid OFDM sequence(s) for LP-SS, consider the following options for further down-selection: - Option 1: Do not specify the overlaid OFDM sequences(s) - Option 2: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation without targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - Option 3: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation and also targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - For Option 3, it is up to RAN4 to make decision on whether/how to define the RRM measurement requirement for OFDM-based LP-WUR using the overlaid sequence of LP-SS.

Option1 is the simplest to implement, but according to the research results of R18 SI, there will be a decrease in performance without the overlaid sequence. Since LP-SS is used for synchronization and RRM measurement purposes, it has higher performance requirements. Therefore, Option 1 is not recommended.

Option3 is optimal in terms of performance. However, since the overlaid sequence needs to be used for synchronization and RRM measurement, the complexity of the transmitting side and the receiving side is increased at the same time. Considering that some LP-WURs have poor capabilities and do not support PSS/SSS detection, LP-SS is introduced. The complexity of using overlaid sequences for synchronization and RRM measurement purposes is not much different from directly measuring PSS/SSS. Therefore, option3 is not recommended.

Option2 will increase the implementation complexity on the transmitting side. However, since the overlaid sequence is not used for synchronization and RRM measurement, it is transparent to the receiving side. Therefore, the complexity of the receiving side is not affected, and the receiving performance of LP-SS can be improved. Therefore, option 2 is a compromise between performance and complexity, and it is recommended to adopt Option 2.

Proposal 19: Adopt option 2 for the overlaid sequence of LP-SS.

- Conclusions

In this contribution, we provide our views on waveform, detection scheme, and information carrying. The following observations and proposals are given:

Observation 1: Using the correlation of sequences, the implementation complexity and power consumption of the receiver can be reduced.

Observation 2: The M sequence not only has better anti-frequency offset performance but also can reduce the complexity of the receiving side.

Observation 3: There is no obvious benefit in generating overlaid sequences based on M sequences to generate random phases.

Observation 4: Since the number of subgroups that are paged simultaneously is less than or equal to 1 in most scenarios, if the UE detects all X MOs, then most detections are necessarily invalid and result in increased energy consumption.

Observation 5: The synchronization performance is related to the sequence length:

- to achieve the synchronization requirement of 2us, a sequence length of more than 8 is required,

- while to achieve the synchronization requirement of 1us, a sequence length of more than 32 is required.

Observation 6: The RRM measurement accuracy is related to the OOK chip length and can be improved by increasing the M sequence length and the number of samples.

- For OOK-1, an M sequence length of 4 in a single sample can meet the measurement accuracy requirements.

- - For OOK-4 with M=2, an M sequence length of at least 8 in a single sample can meet the measurement accuracy requirements.

- - For OOK-4 with M=4, an M sequence length of at least 16 in a single sample to meet the measurement accuracy requirements.

Proposal 1：Support M=4 for 30kHZ SCS.

Proposal 2: Do not support M=1 for OOK-4.

Proposal 3: The value of M is independent of SCS.

Proposal 4：Specify only the necessary steps for the design of OOK-1 and OOK-4.

Proposal 5: Specifies only the overlaid sequence for OOK-1.

Proposal 6: Specifies the two steps of sequence mapping and DFT for OOK-4.

Proposal 7: Further discuss how the UE obtains the OOK waveform generation scheme.

Proposal 8: Prioritize M sequence for overlaid sequence.

Proposal 9: Directly use the existing M sequence as the overlaid sequence.

Proposal 10: The overlaid sequence generation method of OOK-4 with M=1 is not supported.

Proposal 11: OOK-1 adopts the overlaid sequence generation method of option 2.

Proposal 12：Support option 3 for the overlaid OFDM sequence(s) of LP-WUS.

Proposal 13: Support X can be configured by the network side.

Proposal 14: Further study the sending rules and detailed design of multiple code points to reduce the number of UE blind detections and energy consumption e.g., sent in the order of code point values.

Proposal 15: Support both option 1 and option 3, and the network side can flexibly indicate the specific solution.

Proposal 16: Carrier information is carried through the overlaid sequence in the CA scenario.

Proposal 17: Confirm the following working assumption:

Support the following options for LP-SS

- Option 1: OOK-1

- Option 2: OOK-4 with M=2,4

- The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation

Proposal 18: Prioritize M sequence for LP-SS.

References

- RP-234056, “Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, RAN#102, Dec. 11 - 15, 2023.

- Draft_Minutes_report_RAN1#118_v030

- Draft_Minutes_report_RAN1#117_v020

- Draft_Minutes_report_RAN1#116b_v030

- Draft Report of 3GPP TSG RAN WG1 #116 v0.3.0

- 3GPP TR 38.869

- Miroslav Dimitrov, Tsonka Baitcheva Member, IEEE, and Nikolay Nikolov, “Efficient Generation of Low Autocorrelation Binary Sequences,”in IEEE Signal Processing Letters, vol. 27, pp. 341-345, 2020