3GPP TSG RAN WG1 #118bis                                                                                                R1-2408256

Hefei, China, October 14th – 18th, 2024

Source:               TCL

Title:                    Enabling TX/RX for XR during RRM measurements

Agenda item:      9.10.1

Document for:    Discussion and Decision

- Introduction

In the RAN #102 meeting [1], a new WID for enabling Tx/Rx for XR during RRM measurement has been approved, and detailed objectives were shown in the following:

The Rel-19 XR ph3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Note: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - If justified, define a mechanism for transmitter to inform the receiver of SN gap (or missing SNs) in PDCP. - Specify Core requirements related to the above objectives as necessary [RAN4]

In RAN1#118[1] meeting, enabling Tx/Rx for XR during RRM measurements has been discussed, and the following agreements were achieved.

Agreement

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select one among the following options:

Option 1: Support Alt. 1-1:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s);

    - Indication is included as part of scheduling DCI:

      - FFS: Bit-field size is one bit;

      - FFS: Bit-field size is >1 bit;

    - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced.

  - FFS: DCI format, DCI content, DCI bit-field size;

  - FFS: Whether indication is for one or more occasions;

  - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped.

Option 2: Support Alt. 3-1:

- Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements.

  - Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions;

    - FFS: Details of pattern:

      - FFS: Pattern is based on periodicity, offset and duration;

      - FFS: Pattern is based on a bitmap;

    - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions.

Agreement

If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

Working Assumption

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction;

    - Indication is included as part of scheduling DCI:

      - Bit-field size is one bit;

        - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

In this contribution, we mainly provide our views on enabling Tx/Rx for XR during RRM measurements.

- Discussion

Enabling Tx/Rx for XR during measurements

In the current specification, measurement gaps are configured to allow UE to do inter-frequency neighbor cell measurement and the corresponding Radio frequency (RF) tuning for RRM purposes (e.g. mobility, CA set-up, etc.). During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331:

1>	not perform the transmission of Hybrid Automatic Repeat request (HARQ) feedback, Scheduling Require (SR), and Channel State Information (CSI);

1>	not report Sounding Reference Signal (SRS);

1>	not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running:

2>	monitor the Physical Downlink Control Channel (PDCCH) as specified in clauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH;

2> not receive on DL-SCH.

In other words, during a measurement gap, a UE is restricted from transmitting or receiving any data except during a random access procedure. As discussed above, XR services have low latency, jitter, and large packet size, when XR traffic arrives close to or during a measurement gap, after UE performs RRM measurement during the measurement gap, the remaining time of latency budget is not enough for the UE to transmit an XR traffic whole, as shown in figure 1. Thus, the performance of XR service will be decreased, to handle this, enabling Tx/Rx for XR during the measurement gap can be considered.

Figure 1. A packet with low latency arrival close to or during the measurement gap

To handle this, a direct way is to relax the scheduling restrictions caused by RRM measurements, a scheduling DCI is used with explicit signaling to indicate upcoming gaps/restrictions. A 1-bit field in the scheduling DCI can be used to indicate whether the upcoming gap/restriction is skipped or not. Additionally, we believe that it is unnecessary to introduce a new DCI format, and instead, reusing the current UE-specific DCI format could be considered.

Proposal 1. Confirming the working assumption, for solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction;

    - Indication is included as part of scheduling DCI:

      - Bit-field size is one bit;

        - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Proposal 2: The current UE-specific DCI format (e.g. DCI format 0_x/1_x) can be used for dynamic indication to enable Tx/Rx within measurement gap(s)/restriction(s).

In addition, to reserve sufficient processing time for UE to skip an occasion of measurement gap/restrictions, a time gap is introduced between the end of the skipping command and the start of the skipped measurement occasion. In the current specification, a processing timeline for PUSCH or PDSCH has been defined. In our view, reusing the current processing timeline for PDSCH or PUSCH is suitable.

Proposal 3: Reusing the current processing timeline for PDSCH or PUSCH as the minimum time gap between the end of the skipping command and the start of the skipped measurement occasion.

UE assistance information for enabling Rx/Tx for XR during RRM measurement

In Rel-18, an enhanced configured grant (eCG) with multiple transmission occasions has been introduced to match the low latency and high data rate requirements of XR services, made possible to eliminate the need for a scheduling request (SR) and buffer status report (BSR) procedure. With eCG, multiple transmission occasions can be configured within a CG configuration, which can be used for transmitting multiple transport blocks (TB) simultaneously. Similarly, in Rel-19, to match the low latency and high data rate, Rx/Tx enabling within RRM has been studied, for DL traffic, gNB can know the actual traffic arrival time and the packet size, then based on gNB indication, trade-off between XR service capacity and RRM measure can be achieved. However, for UL service, gNB may not know the actual size and delay budget for a packet if UE does not report the BSR timely, for example, in some cases, due to resource limitations (e.g., frame structure), there is no opportunity for a UE to report BSR and PDB in time before a measurement restriction. In addition, when the CG configuration is used for XR services, and the CG configuration is located within an RRM measurement gap/SMTC, with this case, it’s very difficult for gNB to make any decision to determine whether performing data transmission/reception or measurement within measurement gap/SMTC, thus, UE assistance information to notify gNB whether Tx/Rx need to perform within RRM measurement gap/SMTC can be considered.

Proposal 4：UE assistance information/indication to notify gNB whether enabling Tx/Rx for XR during RRM measurements restriction can be considered.

Interaction with DRX

When c-DRX is configured to a UE for power saving, during c-DRX with idle time, UE doesn’t wake up to monitor PDCCH(s), thus, within a DRX cycle idle time, how does gNB/UE to indicate data transmission/reception enabling within a measurement gap/SMTC. For the DL scenario, within DRX idle time, a signal to notify whether Rx/Tx can be performed within measurement gap/SMTC is needed. For the UL scenario, similar scenario in section 2.2, due to resource limitations (e.g., frame structure), there may be no opportunity for an UE to report BSR and PDB timely before a measurement restriction. Thus, signaling within DRX idle time to notify gNB whether Rx/Tx can be performed within measurement gap/SMTC is needed.

Proposal 5: Interaction between DRX and solutions to enable Tx/Rx during measurement restrictions can be studied.

- Conclusion

In this contribution, the following proposals have been made:

Proposal 1. Confirming the working assumption, for solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction;

    - Indication is included as part of scheduling DCI:

      - Bit-field size is one bit;

        - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Proposal 2: The current UE-specific DCI format (e.g. DCI format 0_x/1_x) can be used for dynamic indication to enable Tx/Rx within measurement gap(s)/restriction(s).

Proposal 3: Re-use the current processing timeline for PDSCH or PUSCH as the minimum time gap between the end of the skipping command and the start of the skipped measurement occasion.

Proposal 4：UE assistance information/indication to notify gNB whether enabling Tx/Rx for XR during RRM measurements restriction can be considered.

Proposal 5: Interaction between DRX and solutions to enable Tx/Rx during measurement restrictions can be studied.

4. References

[1] RP-234057, XR (eXtended Reality) for NR Phase 3.

[2] 3GPP RAN1 #118 meeting, Chairman notes.