3GPP TSG RAN WG1 #118bis						  R1- 2408302

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.11.5

Source: 	LG Electronics

Title: 	Discussion on IoT-NTN TDD mode

Document for:	Discussion and decision

- Introduction

According to WID [1], the objectives for Rel-19 IoT-NTN TDD mode are as follows:

The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

In this contribution, we discuss mechanism for TDD mode in IoT NTN.

- Discussion

  - Impact of TDD mode on DL synchronization procedure

According to the current NB-IoT specification, NPSS is transmitted in every frame, and NSSS is transmitted in every even frame, and NPBCH is transmitted in every frame with the same MIB content with 640 msec periodicity. In case of NSSS, different cyclic shift values are used across different frame, and this pattern is repeated every 80msec. In this case, the UE can determine the frame location within 80msec based on the detected NSSS. In case of NPBCH, 800 complex coded-symbols are first partitioned in to 8 parts, and each part is repeated 8 times. The UE can determine 6 LSB of SFN based on the received NPBCH by assuming proper scrambling sequence and complex coded-symbols. Then, the UE can know 4 MSB of SFN based on the decoded MIB. In practice, it is assumed that the UE combines up to 8 NPBCH with the same complex coded symbols in symbol-level to decode the NPBCH reliably. However, if we consider that only a single DL subframe is present every N=9 radio frames, the symbol-level combining would not be possible since there is at most one chance to transmit NPBCH every 80 msec. In this case, the NPBCH detection performance would be degraded by roughly 9dB compared to the IoT NTN without TDD mode. Moreover, since the NSSS will be transmitted in even frame, the effective period of the NSS after applying the TDD pattern would be changed into 180msec, and it will further decrease the detection performance.

Observation 1: If a single DL subframe is present every N (=9) radio frame, the detection performance of NPSS, NSSS, and NPBCH would not be sufficient due to the lack of symbol-level combining. Decoding NPBCH without symbol-level combining could cause 9 dB loss compared to decoding NPBCH with symbol-level combining of 8 NPBCH transmissions.

In case of NSSS, when the TDD mode is enabled, it can be considered to allow to transmit it even in odd frames to enhance the detection chance. In this case, it would be necessary to newly define how to set the cyclic shift value for the new occasions. For simplicity, the same cyclic shift values can be repeated twice across contiguous two frames.

In case of NPSS and NPBCH, TDD pattern would need to have the sufficiently large number of DL subframes are present within N radio frame to guarantee symbol-level combining to boost up the detection performance. In this case, the value of N may need to be further increase. For instance, 40 or 80 contiguous DL subframes can be present every 64 radio frames. Alternatively, NPBCH could be mapped on additional subframe within a frame. However, in this case, due to the lack of DL resources for dedicated signaling, the DL throughput performance could be degraded.

Proposal 1: For IoT-NTN TDD mode, following enhancements can be considered for DL synchronization:

- NSSS is mapped on all or a subset of odd frames on top of even frames.

- TDD pattern includes a large number of contiguous DL subframes at the expense of the increased value for N.

The periodicity of MIB signaling is 640 msec, and the periodicity of SIB1-NB signaling is 2560msec. On the other hand, when we consider the periodicity of the TDD pattern is 90msec according to the WID, the location of the contiguous DL subframes would not be aligned with the system information periodicity. To be specific, depending on the TDD pattern period, it would be possible that the time location to be used for system information signaling is not set to DL subframe. In this case, the UE may not receive the system information properly. For instance, if the first 10 subframes every N=9 radio frames are set to DL subframe, then the first NPBCH transmission in the second period would not be transmitted as shown in Figure 1. Moreover, since the NPBCH would be transmitted on a subset of frames, cell randomization would need to be modified accordingly. To be specific, cell randomization could target the NPBCH transmissions with the same TDD pattern.

Figure 1: Example of NPBCH transmissions after applying TDD pattern.

In case of SIB1-NB, the starting frame is determined by PCID, and the repetitions are equally spaced within the 2560 msec period. When we consider the value of N is 9, then the some SIB1-NB repetition would not be transmitted since there is no DL subframe. Moreover, depending on the value of PCID, the TDD pattern also needs to be changed to ensure the DL subframe for the SIB1-NB transmission. Moreover, SIB1-NB transmission can occur in every other frame within 16 radio frames depending on the value of I_SF. In this situation, different coded bits of SIB1-NB will be transmitted in another frame as shown in Figure 2. When the UE does not receive all the combinations of the coded bits due to the lack of DL subframes, it would be more critical problem compared to the case when the UE does not receive some repetitions.

Figure 2: Example of SIB1-NB with I_SF=2 and its repetition transmissions after applying TDD pattern.

Observation 2: Depending on the TDD pattern, some system information cannot be transmitted since the MIB-NB or SIB1-NB transmissions are mapped on non-DL subframe.

Meanwhile, since the SFN is represented by 10 bits, the TDD pattern may need to be repeated every 10240 msec as well. If the hyper SFN is enabled, the TDD pattern may need to be repeated every 10240*1024 msec. On the other hand, depending on the value of N, there could be orphan subframes or frames after applying the TDD pattern repeatedly. For instance, if the value of N is 9, then 70 subframes are remaining. It is necessary to investigate how to set the TDD pattern for these orphan subframes or frames.

Figure 3: Example of applying TDD pattern when SFN wraps around.

Observation 3: Depending on the TDD pattern period, when SFN wraps around, there could be some orphan subframes or frames. TDD pattern for these orphan subframes needs to be defined.

Proposal 2: For IoT-NTN TDD mode, one or more of followings can be considered to define DL subframes and their locations for the TDD pattern:

- Set the value of N, TDD pattern periodicity, to be a form of powers of 2.

- DL subframe locations can be different across different TDD pattern periods based on:

  - MIB signaling periodicity.

  - SIB1-NB transmission occasions.

- Modifying MIB periodicity, SIB1-NB mapping.

- Introduce separate TDD pattern for orphan subframes or frames within 10240 msec or 1024*10240msec.

  - Impact of TDD mode on random access procedure

For initial access procedure, UEs needs to transmit NPRACH after receiving relevant system information. Meanwhile, the TDD pattern will restrict the possible locations of UL subframe where the UE can transmit UL channels/signals. According to TS36.331, the NPRACH periodicity can be 40, 80, 160, 240, 320, 640 1280, and 2560 msec, and the NPRACH start time can be 8, 16, 32, 64, 128, 256, 512, 1024 msec within the configured NPRACH period. When we consider that the TDD pattern periodicity is N=9 radio frames, then the UL subframe locations would not be quite aligned with NPRACH occasions depending on the TDD pattern period as shown in Figure 4. In this case, it would be also possible that some TDD pattern does not have any NPRACH resources, and it will cause roughly additional 90 msec latency for NPRACH transmission. To bypass this inefficiency, it can be considered that the UL subframe locations can be different across different TDD pattern to ensure that NPRACH occasions are on UL subframes. Alternatively, we can modify NPRACH resource occasions to be aligned with the TDD pattern and its periodicity. Moreover, since the reduced chance of NPRACH transmission, we may also need to consider UL capacity increasement for NPRACH as discussed in Agenda 9.11.4.

Figure 4: Example of NPRACH resources after applying TDD pattern.

Observation 4: Depending on the TDD pattern, NPRACH cannot be transmitted since the NPRACH resources are mapped on non-UL subframe.

Proposal 3: For IoT-NTN TDD mode, one or more of followings can be considered to define UL subframes and their locations for the TDD pattern:

- Set the value of N, TDD pattern periodicity, to be a form of powers of 2.

- UL subframe locations can be different across different TDD pattern periods based on:

  - NPRACH transmission occasions.

- Modifying NPRACH periodicity and start time.

Next, due to the reduced chance of UL transmissions and DL receptions, it would be necessary to update all the timelines for the random-access procedure accordingly including NPRACH-to-RAR window timing, RAR-to-Msg3 Tx timing, and Msg3-to-conention resolution window timing.

According to TS36.321, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 3 subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level. Moreover, the maximum RA window size is currently 10.24s for FDD. Meanwhile, depending on the TDD pattern, it would be possible that the determined RAR window does not include any DL subframes or includes insufficient DL subframes. To bypass this problem, it would be necessary to have additional timing offset to derive the location of RAR window. For simplicity, it can be considered that the RAR window starts the earliest DL subframe 3+UE-eNB RTT after the subframe that contains the end of the last preamble repetition. However, in this case, multiple RAR windows associated with different NPRACH repetitions that starts in different timing could be overlapping. In this case, it would be necessary to distinguish different NPRACH transmission by adjusting RAPID or RA-RNTI.

Regarding RAR-to-Msg3 TX timing, depending on the TDD pattern, the existing timeline might be sufficient especially when the TDD pattern includes UL subframes after DL subframes in time within a period. If necessary, additional subframe offset could be used to ensure that Msg3 transmission is scheduled on the configured UL subframes. Similarly, the Msg3 can be transmitted in the earliest UL subframe after the indicated Msg 3 TX timing.

Regarding Msg3-to-Contention resolution window, it can be considered to postpone the starting time of the contention resolution window to be located in the configured DL subframe.

Proposal 4: For IoT-NTN TDD mode, one or more of followings can be considered for initial access procedure:

- RAR window starts in DL subframe provided by the TDD pattern subject to the existing required time.

- Subframe offset to indicate RAR-to-Msg3 TX timing is prolonged to ensure that Msg3 is mapped on UL subframe provided by the TDD pattern.

- UE (re)starts the contention resolution timer at the DL subframe provided by the TDD pattern subject to the existing required time.

- Conclusions

In this contribution, we discussed TDD mode for IoT-NTN. Based on the above discussion, our observations and proposals are given as follows:

Observation 1: If a single DL subframe is present every N (=9) radio frame, the detection performance of NPSS, NSSS, and NPBCH would not be sufficient due to the lack of symbol-level combining. Decoding NPBCH without symbol-level combining could cause 9 dB loss compared to decoding NPBCH with symbol-level combining of 8 NPBCH transmissions.

Observation 2: Depending on the TDD pattern, some system information cannot be transmitted since the MIB-NB or SIB1-NB transmissions are mapped on non-DL subframe.

Observation 3: Depending on the TDD pattern period, when SFN wraps around, there could be some orphan subframes or frames. TDD pattern for these orphan subframes needs to be defined.

Observation 4: Depending on the TDD pattern, NPRACH cannot be transmitted since the NPRACH resources are mapped on non-UL subframe.

Proposal 1: For IoT-NTN TDD mode, following enhancements can be considered for DL synchronization:

- NSSS is mapped on all or a subset of odd frames on top of even frames.

- TDD pattern includes a large number of contiguous DL subframes at the expense of the increased value for N.

Proposal 2: For IoT-NTN TDD mode, one or more of followings can be considered to define DL subframes and their locations for the TDD pattern:

- Set the value of N, TDD pattern periodicity, to be a form of powers of 2.

- DL subframe locations can be different across different TDD pattern periods based on:

  - MIB signaling periodicity.

  - SIB1-NB transmission occasions.

- Modifying MIB periodicity, SIB1-NB mapping.

- Introduce separate TDD pattern for orphan subframes or frames within 10240 msec or 1024*10240msec.

Proposal 3: For IoT-NTN TDD mode, one or more of followings can be considered to define UL subframes and their locations for the TDD pattern:

- Set the value of N, TDD pattern periodicity, to be a form of powers of 2.

- UL subframe locations can be different across different TDD pattern periods based on:

  - NPRACH transmission occasions.

- Modifying NPRACH periodicity and start time.

Proposal 4: For IoT-NTN TDD mode, one or more of followings can be considered for initial access procedure:

- RAR window starts in DL subframe provided by the TDD pattern subject to the existing required time.

- Subframe offset to indicate RAR-to-Msg3 TX timing is prolonged to ensure that Msg3 is mapped on UL subframe provided by the TDD pattern.

- UE (re)starts the contention resolution timer at the DL subframe provided by the TDD pattern subject to the existing required time.

Reference

[1] RP-242415, “New WID on introduction of IoT-NTN TDD mode”, Moderator (RAN1 Vice-Chair), Melbourne, Australia, September 9-12, 2024.