3GPP TSG RAN WG1 #118bis			R1-2408321

Hefei, China, October 14th – 18th, 2024

Agenda item:	9.5.2

Source:	KT Corp.

Title:	Discussion on on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

1. Introduction

Many techniques to achieve energy savings for NR BS is discovered during the study period [1], and adopting adaptation of common signals is considered as objectives of the Rel. 19 enhanced NES with its high NES gain. Especially, on-demand based SIB1 transmission is studied and agreed to be specified with the following objective [2]:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

  - RAN1 specification impact to support this feature should be minimized

After the meeting of RAN #118, some agreements about this agenda are made [3]. Agreements and conclusions for this agenda are at the appendix of this contribution. Especially, limiting scenarios for the operations on on-demand SIB1 is discussed indicating the results in [2].

In this contribution, we give our views for this agenda, especially in the view about the UL WUS configuration and transmission.

2. Aspects on the UL WUS and RAR procedure

It is agreed to study the options about the sharing resources of WUS with the legacy RACH occasion. In our view, the separated RO may provide relatively simple procedures, however, no critical misoperation will occur with the shared RO, even when the sequence indices are not separately configured. For many cases, the BS may be able to distinguish the two signals, RA preamble and UL WUS, sharing same RO and sequence indices, especially with the limiation of usage case (e.g., only for the UEs attempting to camp on). For the other cases the BS can still accomplish feasible operation, e.g., BS may send both Msg2 and SIB1 if it cannot be distinguished.

Proposal 1: For the dedicated PRACH resource usage of UL WUS, the resource management is left to the BS implementation, i.e., both shared and separated RO with the conventional PRACH resource pool are supported.

It is agreed that the RAR for the UL WUS is supported, while the contents are still in discussion. Many of the contents of conventional msg2 can be omitted like msg3 scheduling grant, while others are still valuable to preserve. For this issue, we give our views on the contents of RAR for the UL WUS.

Proposal 2: Beam operation is supported for the UL WUS, that is, the identification information of UL WUS is included in the RAR for the response of UL WUS.

Proposal 3: Discuss futher for the contents of RAR of UL WUS whether including the following contents are beneficial.

- TA related information for further UL transmissions

- Timing related information for the transmission of the SIB1 (instead of in the WUS configuration)

The last issues we discuss on this aspect is regarding the coverage aspects, the UL WUS will require at least similar coverage with the PRACH. The NES cell with OD-SIB1 needs to be feasible with the Rel. 18 coverage enhancement feature.

Proposal 4: The repetition for the UL WUS is supported. The same mechanism with the PRACH repetition in Rel. 18 coverage enhancement can be adopted.

3. Aspects on the WUS configuration procedure

In [4], many elements for the WUS configuration are discussed. As a reference, the followung is agreed in [5] for the starting point on this issue.

It is agreed to specify only for case 2 (1+B+X) and the configuration is delivered to UE via SIBx. We give our views on the contents of the UL WUS configuration.

Proposal 5: Beam operation is supported for the UL WUS, that is, configuring multiple occasions for UL WUS transmission is supported for the UL WUS configuration.

Proposal 6: The followings are supported for the UL WUS configuration as mandatory.

- ARFCN-ValueNR for the SSB of NES cell

- Parameters about the UL resources for the transmission of UL WUS

- Parameters for the reception of RAR of UL-WUS

On the other hand, we see the NES cell can be all the cells being operated at specific frequency bands and independent with the position of the cell. With the absense of the physical cell ID of NES cell, the UE canidistinguish a cell as NES with OD-SIB1 with the absense of SIB1 and proper notification.

Proposal 7: The followings are supported for the UL WUS configuration as optional.

- PhysCellId for the limitation of accessible cells to SIB1 request

- Repetition related parameters for the UL WUS transmission

- prach-RootSequenceIndex separating the signa space with the conventional PRACH

The last issue we discuss for the WUS configuration is about the triggering condition of the SIBx transmission containing the WUS configuration. Periodical transmission can be inefficient for many deployment scenarios, so the transmission is needed to be based on the request of UE. Considering the scenario, a UE may request when it finds cell without SIB1 transmission.

Proposal 8: The transmission of the SIB containing UL WUS configuration is based on the UE request.

4. Conclusion

We give our proposals in this contribution as follows:

Proposal 1: For the dedicated PRACH resource usage of UL WUS, the resource management is left to the BS implementation, i.e., both shared and separated RO with the conventional PRACH resource pool is supported.

Proposal 2: Beam operation is supported for the UL WUS, that is, the identification information of UL WUS is included in the RAR for the response of UL WUS.

Proposal 3: Discuss further for the contents of RAR of UL WUS whether including following contents are beneficial.

- TA related information for further UL transmissions

- Timing related information for the transmission of the SIB1 (instead of in the WUS configuration)

Proposal 4: The repetition for the UL WUS is supported. The same mechanism with the PRACH repetition in Rel. 18 coverage enhancement can be adopted.

Proposal 5: Beam operation is supported for the UL WUS, that is, configuring multiple occasions for UL WUS transmission is supported for the UL WUS configuration.

Proposal 6: The followings are supported for the UL WUS configuration as mandatory.

- ARFCN-ValueNR for the SSB of NES cell

- Parameters about the UL resources for the transmission of UL WUS

- Parameters for the reception of RAR of UL-WUS

Proposal 7: The followings are supported for the UL WUS configuration as optional.

- PhysCellId for the limitation of accessible cells to SIB1 request

- Repetition related parameters for the UL WUS transmission

- prach-RootSequenceIndex separating the signa space with the conventional PRACH

Proposal 8: The transmission of the SIB containing UL WUS configuration is based on the UE request.

5. References

[1] 3GPP TR 38.864 V18.0.0, Study on network energy savings for NR.

[2] RP-242354, Revised WID for Enhancements of network energy savings for NR.

[3] RAN1 #118 Chairman’s note, final.

[4] R1-2403729, FL summary 5 for on-demand SIB1 in idle/inactive mode, Moderator (MediaTek)

[5] RAN1 #117 Chairman’s note, final.

6. Appendix: Agreements on the previous meeting

Agreement

For further work on on-demand SIB1 in idle/inactive mode, as a baseline, it is assumed that the transmit power control of UL WUS transmission based on PRACH is applied in the same manner as the legacy PRACH transmission.

- FFS: Potential optimization of the power ramp-up procedure.

Agreement

For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options:

- Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages.

- Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages.

Agreement

RAN1 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Agreement

RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and durationof the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options:

- Option 1: starting time and duration are indicated inRAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on reference time pointto determine the window starting time, RAN1 to down select from the following two options:

- Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission

  - FFS: Definition of RAR reception time

- Option 2: The reference time point is defined based on the UL-WUS transmission time

- Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

Agreement

For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective.

- RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

Conclusion

For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective.

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.

Agreement

For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options:

Option 1: searchSpaceZerofor on-demand SIB1 is provided from MIB on NES cell

Option 2: searchSpaceZerofor on-demand SIB1 is provided from UL WUS configuration

Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS.

Combination of multiple options is not precluded.