3GPP TSG RAN WG1 #118bis	                             R1-2408344

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.6.1

Source:	Panasonic

Title:	Discussion on the LP-WUS and LP-SS design

Document for:	Discussion/Decision

# Introduction

In RAN1 #117 and #118 meeting, the agreements listed the appendix were achieved for LP-WUS and LP-SS design [1] [4].

In this contribution, we further share our views on the waveform design of the LP-WUS and LP-SS.

# Discussion

## Waveform design

### On LP-WUS design

Symbol rate design

In the previous meeting, below agreement was achieved for LP-WUS design.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) FFS M=1 for OOK-4

Regarding whether to support M=1 for OOK-4, as long as OOK-1 is sufficient to support 1 chip/segment per OFDM symbol, we do not see the need to duplicate and specify M=1 for OOK-4.

Proposal 1: If OOK-1 is supported and specified, no need to support M=1 for OOK-4, which duplicates the feature design.

The supported M values for OOK-4 should depend on the designed symbol rate of OOK and applicable SCS. There are two reasons to limit the support option. One is the supported number of symbols rates scales the complexity of the LP-WUR. The second one is the high OOK symbol rate is, more complexity and higher cost for LP-WUR implementation. Therefore, we support 60kHz symbol rate, which corresponds to OOK-4 with M=4 for 15 kHz SCS and M=2 for 30kHz SCS. Regarding the working assumption of M=4 for 30 kHz SCS, which requires 120 kHz symbol rate and additional LP-WUR complexity and cost, our first preference is not to support, although we can be okay to confirm it if majority support.

For OOK-1, if specified separately, either 15 or 30 kHz symbol rate and SCS would be supported.

Proposal 2: For FR1, OOK-4 M=4 for 15kHz and M=2 for 30kHz are supported. Either 15 or 30kHz is specified for OOK-1.

For the support of FR2, to limit the additional complexity, below can be considered.

Proposal 3: For FR2, OOK-4 with M=2 for 60 kHz SCS and OOK-1 for 60kHz SCS are supported.

Channel bandwidth

Regarding channel bandwidth of LP-WUS/LP-SS, below agreement were achieved in RAN1#117 meeting.

Agreement Update agreement in last meeting as below: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2

For possible other number of PRBs than 11, the only motivation should be for RAN4 to decide how large GB is needed for ACLR/ACS and CFO handling. No need to further decide other values in RAN1.

For 15 kHz SCS, same number of PRBs to have same number of sub-carriers with 30 kHz, or 2X PRBs to have same bandwidth should be decided. Performance-wise, it is tradeoff between utilizing larger bandwidth or longer duration. In order not to increase the LP-WUR complexity by multiple bandpass filters, same bandwidth with 15kHz SCS should be supported.

Proposal 4: For both 15 kHz and 30 kHz, same bandwidth should be supported for LP-WUS for LP-WUS RF complexity reduction.

For FR2, it is proposed to utilize 60kHz SCS, which is always different band of 30 kHz/15 kHz SCS of FR1. Then separate filter can be used for FR2. Then the number of PRBs used for LP-WUS and LP-SS can be separately defined. 11 PRBs can be baseline.

Proposal 5: For FR2, 60 kHz SCS is supported. 11 PRBs for LP-WUS and LP-SS is supported as baseline.

Overlaid sequence design

Below agreements were achieved regarding the candidate overlaid OFDM sequence:

Agreement Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS - Down select among gold sequence, m sequence and ZC sequence - FFS the overlaid OFDM sequence is time or frequency domain sequence. - FFS how to reuse the existing sequences - Note: Strive to minimize the impact on OOK detection performance - If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS

For the time error and residual frequency error, following was agreed.

Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. - Note: Companies can use any value within the above range with justification on the value (including impact on power consumption)

NR PSS/SSS sequences design were discussed in RAN1#88bis and RAN1#89. After the big discussion between ZC sequence and m-sequence, frequency domain-based pure BPSK M sequence was selected. The argument to select m-sequence instead of ZC sequence was ZC sequence has high sidelobes in the time/frequency offset detection ambiguity function [5] in spite that ZC sequence allows better PAPR/CM. As the possibility to reuse NR transmitter in gNB and receiver in UE, to use frequency domain-based pure BPSK M sequence is one design direction. On the other hand, as shown in the evaluation of section 2.2.1, better PAPR allows more power boosting of LP-WUS and can reduce the amount of resource usage for LP-SS/LP-WUS.

Regardless of the sequence choice, larger CFO decrease the detection performance. Therefore, the target CFO for LP-WUS reception should be less than 5 ppm.

Proposal 6: We are ok to use either ZC sequence or frequency domain-based M sequence.

Proposal 7: The residual frequency error after frequency error correction should be less than 5 ppm.

On overlaid sequence is in time or frequency domain

In previous meeting, below agreements were achieved for how to specify overlaid sequence.

Agreement For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1. - Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing Agreement For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported) - Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing - Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing - Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary

As discussed before, for OOK-1 and OOK-4 with M=1, it is still controversial how to specify, especially whether the overlaid sequence should be in time domain (before DFT/LS spreading) or frequency domain (before IFFT). Below is a full set of possible solutions that we can see so far:

- Solution 1: Only support OOK-1. No support of OOK-4 with M=1

  - 1.1: For OOK-1 overlaid sequence, same as OOK-4 M>1, overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

  - 1.2: For OOK-1 overlaid sequence, overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing.

- Solution 2: Support both OOK-1 and OOK-4 with M = 1. OOK-4 with M=1 follows above agreement for M>1.

  - 2.1: For OOK-1 overlaid sequence, same as OOK-4 M>1, overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

  - 2.2: For OOK-1 overlaid sequence, overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing.

Firstly, we prefer Solution 1 to avoid design duplication. Between 1.1 and 1.2, 1.1 has less specification efforts and has same framework with OOK-4, while 1.2 has less impact to gNB implementation. We slightly prefer 1.1 but can also accept with 1.2.

Proposal 8: For overlaid OFDM sequences for LP-WUS for OOK-1, we firstly prefer to generate overlaid sequence in an OOK symbol before DFT/LS processing but can also be flexible and accept to specify the overlaid sequence in frequency domain before IFFT processing.

On whether/how the overlaid OFDM sequence carry LP-WUS information

On whether/how the overlaid OFDM sequence carry LP-WUS information, below agreement was achieved in the RAN1#116-bis meeting.

Agreement Regarding the overlaid OFDM sequence(s) of LP-WUS, consider the following options: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol or OFDM symbol duration. OFDM-based LP-WUR can obtain the whole information bits by the presence of the overlaid sequence. - Option 1-2: The overlaid OFDM sequence is pre-determined from multiple sequences. This sequence carry NO information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the OOK ON/OFF pattern. - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - Option 3: One sequence is selected from multiple candidates overlaid OFDM sequences on one or more OOK ‘ON’ symbols, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). - Option 4: Use of modulated overlay sequence with constellation point: overlay sequence acting as a spreading sequence and constellation point carrying information for OFDM-based LP-WUR. Other options are not precluded.

In our view, the merit to support overlaid sequence for LP-WUS is to allow possible early detection and termination of the processing by LP-WUR to acquire longer sleep duration, rather than to enhance the coverage performance. Network side handling complexity between OOK or OFDM-based receiver type was raised in RAN [2]. Then WID was concluded as following [3]. It means network is not required to distinguish two types of receivers. Enhance the coverage performance specific to OFDM-based receiver type without network identification does not have specific merit as the required resource for LP-WUS needs to be dimensioned to OOK case. Therefore, termination of the processing by LP-WUR to acquire longer sleep duration should be used for OFDM-based receiver.

•The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

Therefore, given this criterion, not all the above options can achieve such merit, while below options may benefit from the early detection.

- Option 2-2 is more straightforward to support. The standardization effort is only for the sequence selection.

  - Option 3 is similar with Option 2-2. If the “one or more OOK ‘ON’ symbols” is at the beginning of all the OOK symbols, early termination is also possible.

- Option 4 payload modulation on spreading sequence fits to larger payload and less UE multiplexing capacity, compared with Option 2-2 sequence selection. Such comparison is similar with the discussion for PUCCH format 0 in Rel.15.

- However, Option 4 may need more standardization effort, which is from both spreading sequence design and also sequence modulation. The receiver also needs to jointly detect the spreading sequence and the constellation point, which would increase the complexity and degrade the performance compared with Option 2-2.

Therefore,

Proposal 9: Option 2-2 should be supported for the overlaid OFDM sequence(s) of LP-WUS.

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options.

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

On LP-WUS information design

Below agreements were achieved in the previous meeting regarding the LP-WUS information design.

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.

Our views on the pros/cons of bitmap VS. codepoint are summarized in the Table.1. Furthermore:

- When smaller number of UEs or UE groups are indicated, bitmap can be simpler solution.

- When larger number of UEs or UE groups are indicated, codepoint may be tradeoff between flexibility and overhead

- CONNECTED mode operation for LP-WUS foresees smaller number of UEs/UE group(s) for a particular MO, as more flexible RRC configuration of LP-WUS resource is possible.

- Although codepoint was agreed for DLE/INACTIVE mode operation, it may not fit to the case for CONNECTED mode. As all the UEs/UE groups(s) within a whole tracking area need to be handled in each cell, codepoint is suitable to cover larger granularity of UE groups, considering the tradeoff of overhead and flexibility. However, it is not the same case for CONNECTED mode.

Table.1 Pros and Cons for bitmap and codepoint design for LP-WUS info triggering PDCCH monitoring

| LP-WUS info for PDCCH monitoring triggering | Pros | Cons | Comments |
| --- | --- | --- | --- |
| Bitmap | - Refined/smaller granularity of UE/UE groups when the number of candidate UEs/UE groups is not large - Less specification effect - When extending to CA, it is simpler to use bitmap for cell or cell group | - Larger overhead when the number of candidate UEs/UE groups is large | - If the required indication capacity is not large, bitmap is simpler solution. |
| Codepoint | - By RRC configuration, the indication per codepoint is flexible in terms of the target UEs/UE groups. - Less overhead on the indication if efficient grouping is done by RRC configuration of codepoints | - More specification efforts but one would argue differently that the framework is same with IDLE/INACTIVE UEs so the efforts could be less. | - If the required indication needs to cover large range number of UEs, codepoint is more flexible. - Whether UE receiver side can enjoy the benefit of unified design with IDLE/INACTVE is not obvious, as neither design requires complicated decoding and processing. |

Therefore,

Proposal 10: for CONNECTED LP-WUS, we support multiple bit blocks, each of which is a bitmap, where each bit points to one or more UEs. Each block can refer to one or more CCs.

### On LP-SS design

- In RAN1#116-bis meeting, below agreements were achieved for LP-SS design.

Working Assumption Support the following options for LP-SS - Option 1: OOK-1 - Option 2: OOK-4 with M=2,4, FFS:1,8,16 - FFS whether value of M depends on SCS - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation FFS how OOK-1 and OOK-4 are specified

- Firstly, we support that the SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation as gNB may typically utilize only one SCS except PRACH and SSB. Although LP-SS and LP-WUS can be TDMed, to use different SCSs increases gNB complexity. In addition, we also propose to use the same symbol rate between LP-SS and LP-WUS for envelop detection.

- When M = 8 or 16, even for 15kHz SCS, the equivalent symbol rate would be up to 120 or 240kHz. In our view, it is much higher symbol rate than that of LP-WUS and would add further complexity for LP-WUR when detecting LP-SS.

- Therefore,

- Proposal 11: The supported symbol rate(s) and SCS value(s) of LP-SS should be aligned with that of LP-WUS, i.e., not to support M = 8 and 16 for LP-SS.

In previous meetings, below agreements were achieved for LP-SS design.

Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration Agreement The number of binary LP-SS sequences is 4.

To decide the binary sequence of LP-SS, using sequence hopping allows more interference randomization. This is beneficial for synchronization and RRM measurement performance against inter-cell interference. Evaluation is performed for binary sequence hopping and the results are shown in the Section 2.2.2.4.

With above consideration,

Proposal 12:

- When LP-SS is only required to calibrate certain timing error within an OOK symbol/chip duration, simple design to employ some candidates of Gold sequence (as the pseudo random code defined in TS38.211) by configuration is sufficient.

- Sequence hopping should be studied to immigrate the inter-cell interference, while the complexity of the design is managed with sufficient simplicity.

## LLS Evaluations

### Comparison of PAPR on different overlaid sequence types

The purpose of this evaluation is to evaluate time-domain PAPR for different overlaid sequence types (ZC sequence and Gold sequence) of LP-WUS with PSD boosting.

The evaluation assumptions are as follows:

- For LP-WUS, OOK-4 with M=4 is assumed and Manchester coding with coding rate of 1/2 is applied to OOK sequence. LP-WUS occupied BW is 4.68 MHz(13 RBs for 30kHz SCS), which consist of LP-WUS transmission BW (11RBs) and GB between LP-WUS and PDSCH (1RB for one side, totally 2RBs). Regarding overlaid sequence, ZC sequence and Gold sequence are evaluated. For comparison, all-ones sequence is also evaluated. The overlaid sequence for ZC and Gold sequence, respectively, is randomly selected for each OOK symbol.  LP-WUS is PSD boosted with the value of {0, 3, 4.77, 10} dB before DFT precoding.

- System BW is 20MHz. Other than LP-WUS occupied BW, the rest BW is mapped with QPSK modulated PDSCH.

- PAPR is calculated for time-domain signal after IFFT in the entire system BW, i.e., includes both LP-WUS transmission and PDSCH transmission. For comparison, PAPR of the case that all system bandwidth 20MHz is allocated to PDSCH is also shown.

PAPR evaluation results for different overlaid sequences with PSD boosting values for LP-WUS are shown in Figure.1. In Figure.1, each graph represents different value of PSD boosting for LP-WUS. From these graphs, following points are observed:

- In Figure.1(a) which is the case LP-WUS is not PSD boosted, regardless of sequence type, PDSCH+LP-WUS has almost the same PAPR as PDSCH only. This is because the occupied bandwidth of PDSCH (15.32MHz) is larger than that of LP-WUS (4.68 MHz) in the system bandwidth (20MHz), PAPR of the PDSCH is dominant in the PAPR for total transmission.

- In the results with PSD boosting of larger value (in Figure.1(b) to (d)), we can observe the gap between PDSCH only and PDSCH+LP-WUS, as the impact of the power of LP-WUS become larger on PAPR of total transmission power.

- With Gold sequence, PAPR of PDSCH+LP-WUS (Gold) increase with larger PSD boosting. This indicates that higher peaks can be occurred with PSD boosting in Gold sequence.

- With ZC sequence, PAPR of PDSCH+LP-WUS (ZC) can maintain almost the same as PDSCH only up to 4.77dB of PSD boosting. In the case of 10 dB boosted, PAPR of PDSCH+LP-WUS (ZC) can be smaller than that of PDSCH only and the gain of 0.4dB can be obtained at 90%CDF. Therefore, it can be said that even with PSD boosting of larger value, ZC sequence can keep Maximum power peak of the signal lower.

From the above observations, we can conclude as follows:

Observation 1: When PSD boosting is adopted, ZC sequence has better time-domain PAPR than Gold sequence.

(a) LP-WUS is not PSD boosted				(b) LP-WUS is PSD boosted by 3dB

(c) LP-WUS is PSD boosted by 4.77 dB				(d) LP-WUS is PSD boosted by 10 dB

Figure.1 PAPR evaluation results for different overlaid sequence types with PSD boosting

### LP-SS performance in terms of time synchronization accuracy and RRM measurement accuracy

In this section, we provide LLS evaluations on time synchronization accuracy and RRM measurement accuracy to determine the LP-SS length.

The general evaluation assumptions are shown in Table.2 and some specific assumptions are listed as follows:

- Gold sequence is used as the binary LP-SS sequences for the ‘ON-OFF’ pattern and carried by OOK-4 with M=1,2,4.

- Zadoff-Chu sequence is used as the overlaid sequence, and randomly selected for each OOK symbol.

- Interferences from neighboring cells are not assumed.

- For the evaluation of the time synchronization accuracy,

  - Sample-level timing estimation is performed by time-domain correlation peak detection with sliding window.

  - According to the agreement in RAN1#118, 20 ppm carrier frequency error and SNR=-3dB are assumed, and the timing estimation error to achieve (T us for P=90 %) is assumed for both AWGN and TDL-C channel as follows:

    - T=5us for OOK-4 with M=1 (OOK-1)

    - T=2us for OOK-4 with M=2

    - T=1us for OOK-4 with M=4

  - From this evaluation, required LP-SS length to achieve the timing estimation error of T us is detected for each of M and channels, respectively.

- For the evaluation of RRM measurement accuracy,

  - LP-RSRP measurement accuracy is evaluated by |delta RSRP| with CDF curves.

  - LP-RSRP is calculated from liner average power difference between ON chip and OFF chip in the received OOK-4 sequence, assuming OOK based receiver. That is:

RSRP [dB] =10log10|(avg. power of ON chips) – (avg. power of OFF chips)|

  - |Delta RSRP| is calculated by:

|Delta RSRP| [dB] = |estimated RSRP – ideal RSRP|

Note: in ideal RSRP, any channel estimation and noise are not considered.

  - The LP-SS length detected by the evaluation of time synchronization is used as the LP-SS length per each LP-RSRP measurement sample.

  - According to the agreement in RAN1#118, 10 ppm carrier frequency error and SNR=-3dB are assumed. In addition, required timing estimation error of T in the evaluation of time synchronization is assumed as timing error.

  - As the LP-RSRP measurement accuracy to achieve, ± 4dB for 90% measurement is adopted.

  - From this evaluation, required measurement samples to achieve ± 4dB for 90% measurement with the LP-SS length detected by the evaluation of time synchronization for each of M and channel, respectively.

Table.2 General evaluation assumptions for time synchronization and LP-RSRP measurement

| Carrier Frequency | 2.6 GHz |
| --- | --- |
| LP-SS structure | Binary Sequence - Sequence type: Gold sequence (From Sec.5.2.1 of TS38.211) |
| Waveform | OOK-4 |
| Configuration for LP-SS signal | M=1,2,4 for 30kHz SCS |
| Overlaid sequence | ZC sequence |
| SCS of OFDM generator for NR signal | same as LP-SS signal |
| LP-SS duration | Number of OFDM symbols: (LP-SS sequence length / M) symbols |
| Channel BW | 20 MHz |
| LP-SS BW | 4.68 MHz (13 PRBs for 30 kHz SCS) for LP-WUS transmission including outer GBs - LP-SS transmission: 11 PRBs - Outer GBs between LP-WUS and adjacent PDSCH: 2 RBs in total |
| Filter | 5-th Order Butterworth filter with 4.68 MHz bandwidth |
| Adjacent subcarrier Interference | PDSCH mapped on resources other than that for LP-SS and outer GB in Channel BW; EPRE of PDSCH / LP-WUS=ρ, where ρ=0 dB |
| Sampling Rate | Tx: 30.72 MHz / Rx: 7.68 MHz |
| ADC | Ideal |
| Channel Model | AWGN/TDL-C - In TDL-C channel, delay spread: 300 ns and UE velocity: 3 km/h |

Evaluation results of time synchronization and LP-RSRP measurement for M=1,2, and 4 in AWGN channel are shown in Figure.2, 3, and 4, respectively. In each figure, the orange line means timing estimation error accuracy and LP-RSRP accuracy to achieve. From these figures, following points are observed:

- On time synchronization, OOK-4 with M=1/OOK-1 has poor timing estimation performance compared to M=2 and 4 and requires the longer sequence length to achieve T=5 us. This may be because chip length of OOK-4 is longer than that of M=2 and 4.

- On RSRP measurement, OOK-4 with M=1/OOK-1 has quite better performance than M=2 and 4, because of longer sequence length to achieve timing estimation error of 5 us.

- In AWGN channel, minimum LP-SS length and measurement samples required to achieve the target time synchronization accuracy and LP-RSRP measurement accuracy are:

  - For OOK-4 with M=1 / OOK-1,

    - 48 bits are required to achieve the target timing estimation error.

    - 1 sample is sufficient to achieve the target RSRP measurement accuracy with LP-SS length of 48 bits.

  - For OOK-4 with M=2,

    - 6 bits are required to achieve the target timing estimation error.

    - 1 sample is sufficient to achieve the target RSRP measurement accuracy with LP-SS length of 6 bits.

  - For OOK-4 with M=4,

    - 4 bits are required for the target timing estimation error.

    - 2 samples are required for the target RSRP measurement accuracy with LP-SS length of 4bits.

- Time synchronization						(b) LP-RSRP measurement

Figure.2 Results of time synchronization and LP-RSRP measurement for OOK-4 with M=1 / OOK-1 in AWGN

- Time synchronization						(b) LP-RSRP measurement

Figure.3 Results of time synchronization and LP-RSRP measurement for OOK-4 with M=2 in AWGN

- Time synchronization						(b) LP-RSRP measurement

Figure.4 Results of time synchronization and LP-RSRP measurement for OOK-4 with M=4 in AWGN

The results in TDL-C channel are shown in Figure.5, 6, and 7, respectively. From these figures, following points are observed:

- On time synchronization, as similar as the case in AWGN, OOK-4 with M=1/ OOK-1 has poor timing estimation performance compared to M=2 and 4, due to longer OOK chip length.

- On RSRP measurement, at least 2 measurement samples are required for averaging the fading.

- In TDL-C channel, minimum LP-SS length and measurement samples required to achieve the target time synchronization accuracy and LP-RSRP measurement accuracy are:

  - For OOK-4 with M=1 / OOK-1,

    - 24 bits are required to achieve the target timing estimation error.

    - 2 sample are required to achieve the target RSRP measurement accuracy with LP-SS length of 48 bits.

  - For OOK-4 with M=2,

    - 4 bits are required to achieve the target timing estimation error.

    - 4 sample are required to achieve the target RSRP measurement accuracy with LP-SS length of 6 bits.

  - For OOK-4 with M=4,

    - 12 bits are required for the target timing estimation error.

    - 4 samples are required for the target RSRP measurement accuracy with LP-SS length of 4bits.

(a) Time synchronization						(b) LP-RSRP measurement

Figure.5 Results of time synchronization and LP-RSRP measurement for OOK-4 with M=1 / OOK-1in TDL-C

(a) Time synchronization						(b) LP-RSRP measurement

Figure.6 Results of time synchronization and LP-RSRP measurement for OOK-4 with M=2 in TDL-C

(a) Time synchronization						(b) LP-RSRP measurement

Figure.7 Results of time synchronization and LP-RSRP measurement for OOK-4 with M=4 in TDL-C

From the above observations, we can conclude as follows:

Observation 2: For OOK-4 with M=1 / OOK-1, 48 bits and 24 bits are required to achieve timing estimation error of 5 us in AWGN and TDL-C, respectively. To achieve ± 4dB for 90% RSRP measurement with these LP-SS length, 1 sample and 2 samples are required in AWGN and TDL-C, respectively.

Observation 3: For OOK-4 with M=2, 6 bits and 4 bits are required to achieve timing estimation error of 2 us in AWGN and TDL-C, respectively. To achieve ± 4dB for 90% RSRP measurement with these LP-SS length, 1 sample and 4 samples are required in AWGN and TDL-C, respectively.

Observation 4: For OOK-4 with M=4, 4 bits and 12 bits are required to achieve timing estimation error of 1 us in AWGN and TDL-C, respectively. To achieve ± 4dB for 90% RSRP measurement with these LP-SS length, 2 sample and 4 samples are required in AWGN and TDL-C, respectively.

Table.3 in below summaries our preliminary evaluation results of RRM measurement accuracy of LP-SS.

Table.3 Summaries on our preliminary evaluation results of RRM measurement accuracy of LP-SS

| Summary of RRM measurement accuracy in AWCN channel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | LP-SS length (bits) | X | Y | TO | CFO |
| OOK-4 with M=1/ OOK-1 | 48 | 4 | 1 | 5 us | 10 ppm |
| OOK-4 with M=2 | 6 | 4 | 1 | 2 us | 10 ppm |
| OOK-4 with M=4 | 4 | 4 | 2 | 1 us | 10 ppm |
| Summary of RRM measurement accuracy in TDL-C channel |  |  |  |  |  |
|  | LP-SS length (bits) | X | Y | TO | CFO |
| OOK-4 with M=1/ OOK-1 | 24 | 4 | 2 | 5 us | 10 ppm |
| OOK-4 with M=2 | 4 | 4 | 4 | 2 us | 10 ppm |
| OOK-4 with M=4 | 12 | 4 | 4 | 1 us | 10 ppm |
| Note: RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples. Z is not directly assumed. |  |  |  |  |  |

It is noted that, for both synchronization and RRM measurement performance, in some cases, OOK-4 with longer sequence (number of bits) does not generate better results. In our observation, it is impacted by CFO, which leads to sidelobe effects particularly for some of the ZC sequence root indices. This will cause the correlation operation to generate unstable value of peaks due to undesired sidelobes, especially in the fading channel.

Observation 5: For LP-SS, if ZC sequence is used as overlaid sequence, CFO would degrade the performance for both time synchronization and RRM measurement accuracy, which leads to the need of using longer binary sequence, especially in the fading channel.

On the other hand, from the above evaluations, we think timing estimation error of 5 us could be too strict requirement for OOK-4 with M=1 / OOK-1, as too long sequence is needed to achieve it and LP-RSRP measurement accuracy with these sequence lengths is much better compared to M=2 and 4. Therefore, we think the timing estimation requirement for OOK-4 with M=1/OOK-1 could be relaxed a little more. We evaluated LP-RSRP measurement accuracy with LP-SS length to achieve timing estimation error of 8 and 10 us, relaxed from 5us. Figure.8 shows the results in AWGN and Figure.9 shows the results in TDL-C. The sequence length used in this evaluation are follows:

- In AWGN channel, 22 bits and 4 bits are required to achieve timing estimation error of 8 and 10 us, respectively.

- In TDL-C channel, 20 bits and 4 bits are required to achieve timing estimation error of 8 and 10 us, respectively.

From these figures, following points are observed:

- By relaxing the requirement for timing estimation error, required LP-SS sequence length can be shortened. Accordingly, LP-RSRP measurement accuracy get worse and larger number of measurement samples would be needed.

- Compared to AWGN channel, the performance degradation in TDL-C channel due to shortened sequence is less obvious. Because in TDL-C channel, it is less effective to use longer sequence for averaging out fading, compared with AWGN channel.

- The values of timing estimation error can be relaxed to around 8 us and less than 10 us for both AWGN and TDL-C, considering the number of required measurement samples can be 2 to 4.

(a) Timing error=5 us		(b) Timing error=8 us (relaxed)		(c) Timing error=10 us (relaxed)

Figure.8 Results of LP-RSRP measurement with LP-SS length for relaxed timing estimation error requirement for OOK-4 with M=1 / OOK-1 in AWGN

(a) Timing error=5 us		(b) Timing error=8 us (relaxed)		(c) Timing error=10 us (relaxed)

Figure.9 Results of LP-RSRP measurement with LP-SS length for relaxed timing estimation error requirement for OOK-4 with M=1 in TDL-C

Moreover, for the discussion on how much timing estimation error could be relaxed, the LP-WUS detection performance should also be considered, assuming time synchronization by LP-SS before detection of LP-WUS in LR.

Figure.10 shows BLER performance of OOK-4 with M=1 / OOK-1 to evaluate the impact of relaxed timing error in TDL-C channel. From the figures, TO=10 degrades by about 3 dB and TO=7 degrade within about 2dB compared to TO=0 at 1%BLER. Therefore, considering tolerance of LP-WUS detection performance for timing error in addition to RSRP measurement, relaxation of timing estimation error to around 8 us could be reasonable.

Figure.10 The impact of timing error on the LP-WUS detection performance of OOK-4 with M=1 / OOK-1 in TDL-C

From the above observations, we can conclude as follows:

Observation 6: For OOK-4 with M=1 / OOK-1, timing estimation error of 5 us could be relaxed for both AWGN and TDL-C.

Proposal 13: For decision of LP-SS length, acceptable timing/frequency tolerance for LP-WUS detection should also be considered.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: If OOK-1 is supported and specified, no need to support M=1 for OOK-4, which duplicates the feature design.

Proposal 2: For FR1, OOK-4 M=4 for 15kHz and M=2 for 30kHz are supported. Either 15 or 30kHz is specified for OOK-1.

Proposal 3: For FR2, OOK-4 with M=2 for 60 kHz SCS and OOK-1 for 60kHz SCS are supported.

Proposal 4: For both 15 kHz and 30 kHz, same bandwidth should be supported for LP-WUS for LP-WUS RF complexity reduction.

Proposal 5: For FR2, 60 kHz SCS is supported. 11 PRBs for LP-WUS and LP-SS is supported as baseline.

Proposal 6: We are ok to use either ZC sequence or frequency domain-based M sequence.

Proposal 7: The residual frequency error after frequency error correction should be less than 5 ppm.

Proposal 8: For overlaid OFDM sequences for LP-WUS for OOK-1, we firstly prefer to generate overlaid sequence in an OOK symbol before DFT/LS processing but can also be flexible and accept to specify the overlaid sequence in frequency domain before IFFT processing.

Proposal 9: Option 2-2 should be supported for the overlaid OFDM sequence(s) of LP-WUS.

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options.

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

Proposal 10: for CONNECTED LP-WUS, we support multiple bit blocks, each of which is a bitmap, where each bit points to one or more UEs. Each block can refer to one or more CCs.

- Proposal 11: The supported symbol rate(s) and SCS value(s) of LP-SS should be aligned with that of LP-WUS, i.e., not to support M = 8 and 16 for LP-SS.

Proposal 12:

- When LP-SS is only required to calibrate certain timing error within an OOK symbol/chip duration, simple design to employ some candidates of Gold sequence (as the pseudo random code defined in TS38.211) by configuration is sufficient.

- Sequence hopping should be studied to immigrate the inter-cell interference, while the complexity of the design is managed with sufficient simplicity.

Observation 1: When PSD boosting is adopted, ZC sequence has better time-domain PAPR than Gold sequence.

Observation 2: For OOK-4 with M=1 / OOK-1, 48 bits and 24 bits are required to achieve timing estimation error of 5 us in AWGN and TDL-C, respectively. To achieve ± 4dB for 90% RSRP measurement with these LP-SS length, 1 sample and 2 samples are required in AWGN and TDL-C, respectively.

Observation 3: For OOK-4 with M=2, 6 bits and 4 bits are required to achieve timing estimation error of 2 us in AWGN and TDL-C, respectively. To achieve ± 4dB for 90% RSRP measurement with these LP-SS length, 1 sample and 4 samples are required in AWGN and TDL-C, respectively.

Observation 4: For OOK-4 with M=4, 4 bits and 12 bits are required to achieve timing estimation error of 1 us in AWGN and TDL-C, respectively. To achieve ± 4dB for 90% RSRP measurement with these LP-SS length, 2 sample and 4 samples are required in AWGN and TDL-C, respectively.

Observation 5: For LP-SS, if ZC sequence is used as overlaid sequence, CFO would degrade the performance for both time synchronization and RRM measurement accuracy, which leads to the need of using longer binary sequence, especially in the fading channel.

Observation 6: For OOK-4 with M=1 / OOK-1, timing estimation error of 5 us could be relaxed for both AWGN and TDL-C.

Proposal 13: For decision of LP-SS length, acceptable timing/frequency tolerance for LP-WUS detection should also be considered.

# Reference

[1] 3GPP RAN1#117 meeting Chairman’s notes

[2] RP-233291, Views on LP-WUS/WUR for R19, Vodafone

[3] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[4] 3GPP RAN1#118 meeting Chairman’s notes

[5] R1-1706156, NR PSS and SSS design

# Appendix

Agreements from RAN1#117 meeting

Agreement

Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS

- Down select among gold sequence, m sequence and ZC sequence

  - FFS the overlaid OFDM sequence is time or frequency domain sequence.

  - FFS how to reuse the existing sequences

  - Note: Strive to minimize the impact on OOK detection performance

- If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS

Agreement

For the LP-SS sequence used in a cell,

- Option 1: the information necessary for determining the sequence is explicitly configured

- FFS: Additional support of determining the sequence by predefined rule without configuration

Agreement

The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR.

- FFS X which is no larger than 20ppm

- Initial frequency error assumption: up to company report

For the overlaid OFDM sequence design of LP-WUS, it is assumed that the residual frequency error for OFDM-based LP-WUR after frequency error correction without considering impact of drift is not larger than Y.

- FFS Y which is no larger than 20ppm and lower than X

- Initial frequency error assumption: up to company report

Agreement

For overlaid OFDM sequence(s) for LP-WUS in time or frequency domain, down-selection from the following:

- Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS and the value of M

- Option 1-2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before DFT/LS processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS

- Option 2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS

- Option 3: overlaid sequence(s) are the sequence(s) of an OOK on symbol in time domain after IFFT processing

  - The length of overlaid sequence(s) depends on IFFT size and the value of M

FFS: same or different options are applied for OOK-1 and OOK-4 M>1.

Agreement

Update agreement in last meeting as below:

From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz

- X to be down-selected between = 11 and 12 PRBs

- FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz

FFS: Whether the above is applicable to FR2

Agreement

Further down-select the number of binary LP-SS sequences for the ‘ON-OFF’ pattern:

- 3

- 4

- 8

- 16

Agreement

For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following:

- Gold sequence

- M sequence

- Computer searched sequence

- FFS: the length of LP-SS sequence

Agreement

The following SNR values for LP-WUR are reported by companies to achieve coverage of PUSCH for message3 for difference noise figures for calibration purposes:

- NF of LR = NF of MR+ 8dB: the reported SNR value range is [-9.05,2.94] dB

- NF of LR = NF of MR+ 5dB:  the reported SNR value range is [-6.5, 5.58] dB

- NF of LR = NF of MR+ 2dB: the reported SNR value range is [-4.04,7.95] dB

- Note 1: The above is observed based on the following assumptions used for calibration:

-	Carrier frequency: 2.6 GHz

-	The number of Tx chains: 1

-	MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE

-	Transmit antenna gain correction factors for WUS: up to company report

-	Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

Agreement

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

Agreement

For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS:

- Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24

  - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups

- Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

  - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N.

- Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

  - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N.

  - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1

- How to satisfy FAR is reported by companies, e.g., FEC/CRC

- Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option.

- Note: Y% effective paging rate per PO is reported by companies

- The followings are considered when down-select among options:

  - The number of supported UE subgroups per PO: M

  - Average network overhead to indicate the number of UE subgroups M per PO

  - False wake up rate due to subgroup-based indication, which will impact the power saving gain

  - Paging latency

Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

Agreements from RAN1#118 meeting

Agreement

For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS:

- Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs

  - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported

    - Value of X is larger than 1. FFS on additional details of X.

Agreement

[H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

Agreement

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

- Option 3: A codepoint value corresponding to [one or more] UEs

- FFS details for extension of option 1 and/or 3 when UE is configured with CA

Agreement

For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1.

- Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

Agreement

For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported)

- Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

- Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing

- Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary

Agreement

The number of binary LP-SS sequences is 4.

Agreement

To determine the binary sequences for LP-SS, the evaluation assumes the following:

- Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority),

  - T=2us(optional), 5us for OOK-1

  - T=1us(optional), 2us for OOK-4 with M=2

  - T=0.5us(optional), 1us for OOK-4 with M=4

  - Other values of SNR, T, M are up to company report.

  - Additionally, companies can submit results on SINR with details on how the interference was modelled.

  - Assume one-shot estimation

  - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS

  - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified

- RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report.

  - As a starting point, the time error is based on 5-10ppm residual frequency error.

  - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance

  - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified

- The frequency error: up to company report

- Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS.

- Channel: AWGN and TDL-C 300ns is assumed.

- Consider cross-correlation for 4 binary sequences under time and frequency error

- Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption

Agreement

For OOK-based LP-WUR, the LP-WUS design assumes the following:

- As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift.

- The frequency error: up to company report.

- Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance

- Note: Companies can assume other values outside of 5-10ppm as long as the values are justified

Agreement

As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift.

- Note: Companies can use any value within the above range with justification on the value (including impact on power consumption)

Agreement

Support Manchester coding for LP-WUS

- FFS other coding schemes