3GPP TSG RAN WG1 #118bis 	R1- 2408347

Hefei, China, October 14th – 18th, 2024

Agenda item:		9.11.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on IoT-NTN TDD mode

Document for:		Discussion and Decision

# Introduction

In RANP # 105 meeting, a new WI for Rel19 IoT NTN TDD mode [1] was approved with assumptions and objective in RAN1 as:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands.

The study and work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band.

This work item includes the following objectives:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

For new NT-IoT TDD NTN mode, we will provide our view in this contribution.

# Discussion

## Cell differential delay

As discussed and concluded in the SI phase of IoT NTN release 17 (see TR 36.763), the maximum differential delay of UEs in one LEO cell is more than 3ms. Then for this release 19 TDD NTN system, UEs receive the NPDCCH will have different TA for the NPDCCH scheduled NPUSCH and compensate based on UE specific TA compensation and cell specific TA compensation for these UL channels.

For NPDCCH scheduled NPUSCH, although UE can adjust the TA based on GNSS and satellite ephemeris, UE with different delay may arrive at satellite on different time (e.g. different slot) considering the different RTT.

Especially for NPRACH, if there is TA error, the UE with different differential delay may arrive at satellite with an even larger range in time domain. This will request that the resource for NPRACH need to be larger than real request so that to cover all the UE with different differential delay.

In this Rel19 TDD mode, considering there are only a set of contiguous UL subframes in each active period for every N radio frame. This set of contiguous UL subframes should cover all UE’s differential delay.

Observation 1: The set of contiguous UL subframe should cover all UE’s UL transmission with different differential delay.

Proposal 1: RAN1 should study how to allocate the period of active contiguous subframe for UL to cover all the UE with different differential delay.

## Uplink and downlink periods

The WID assumptions define the uplink and downlink subframes shall we periodically available very N radio frames. The baseline is assumed to be N=9. In our view, the use of N=9 has significant impact on the specification, because it is not a power of 2 or a scaling of the 10 ms radio frame i.e. on the contrary to most of the legacy specification time elements. As an example, the UE DRX cycle in TS 36.331 is given in number of subframes as “sf256, sf512, sf1024, sf1536, sf2048, sf3072, sf4096, sf4608, sf6144, sf7680, sf8192, sf9216, sf1280, sf2560, sf5120, sf10240”. This means that if the UL/DL subframes are available every 90 ms the DRX cycle needs to be reconfigured frequently to re-align with the pattern.

Observation 2: The use of UL/DL subframe availability every 9 radio frames does not fit legacy specification time elements.

A simpler alternative may be to use N=8 or N=10. This fits at least some of the DRX cycles and other timers.

Proposal 2: RAN1 to study N=8 and N=10 as alternatives to the N=9 baseline for UL/DL subframe availability.

## Features potentially not needed in TDD NTN

Considering the 1:N or k:N active radio frame, the delay between transmission and reception of data and to confirmation from the other side (i.e. HARQ feedback) will be larger than in FDD. One way to reduce the delay is to disable HARQ feedback as already defined in IoT NTN release 18. However this will impact the reliability of the control channel, higher layer signalig and NPUSCH performance.

Proposal 3: RAN1 to discuss whether HARQ feedback is needed between two active radio frames separated according to the periodicity N.

## Cell Search

In legacy (FDD) specification, the NPSS is sent in the 6th subframe of every radio frame, while the NSSS is sent in the 10th subframe of even radio frames. Considering the 1:N or k:N active radio frame, the periodicity of the PSS and SSS will also increase e.g. to at most N times. Then the UE can only search the PSS/SSS with a larger period than legacy FDD and the latency for performing cell search & identification will increase. This will eventually result in the UE having reduced remaining serving time from the NTN cell. It may also impact the UE’s ability to maintain synchronized to the cell.

We think this issue shall be studied also by RAN4 and propose to send an LS to RAN4.

Observation 3: The 1:N or k:N active radio frame with contiguous UL subframe and contiguous DL subframe may impact to the DL synchronization and cell search.

Proposal 4: RAN1 should study how to have accurate DL synchronization and cell search in this IoT NTN TDD mode.

Proposal 5: RAN1 to send LS to RAN4 to study both DL synchronization and cell search issue in this IoT NTN TDD mode.

## Link budget impact

The proposed IoT NTN frame structure will spread the available UL and DL subframes in time. This means that the use of repetitions will also result in a repetition duration that scales with N. Thus it may not be possible to use the currently specified maximum number of repetitions, because the total duration will be too long (10s of seconds potentially) compared to the satellite-based cell availability. The configuration of repetitions is up to network implementation, but restricting the maximum number of repetitions effectively also restricts the link budget and thus we think it is important for RAN1 to discuss.

Proposal 6: RAN1 to discuss the impact on the achievable link budget if the number of repetitions is restricted due to the use of every Nth radio frame.

## System capacity

As noted above, the repetition duration may extend in time and across the available UL and DL subframes. This will impact the resource availability. Furthermore, there is a fundamental signaling overhead due to the requirement of providing NPSS, NSSS, NRS, MIB and relevant System Information Blocks (at least SIB1-5 and SIB31). We consider it beneficial for RAN1 to evaluate the actual capacity of the cell when accounting for the aforementioned transmissions.

Proposal 7: RAN1 to evaluate the cell capacity when accounting for at least NPSS, NSSS, NRS, MIB and relevant System Information Blocks (at least SIB1-5 and SIB31).

Likewise, there are restrictions on uplink capacity because a certain minimum number of Random Access Occasions need to be configured to ensure UEs can timely access the cell.

Proposal 8: RAN1 should evaluate the UL capacity for NPRACH and NPUSCH considering the active radio frame per N radio frame.

# Conclusion

In this contribution, we discussed our view on IoT NTN TDD mode. Our observations ans proposals are as follows:

Observation 1: The set of contiguous UL subframe should cover all UE’s UL transmission with different differential delay.

Observation 2: The use of UL/DL subframe availability every 9 radio frames does not fit legacy specification time elements.

Observation 3: The 1:N or k:N active radio frame with contiguous UL subframe and contiguous DL subframe may impact to the DL synchronization and cell search.

Proposal 1: RAN1 should study how to allocate the period of active contiguous subframe for UL to cover all the UE with different differential delay.

Proposal 2: RAN1 to study N=8 and N=10 as alternatives to the N=9 baseline for UL/DL subframe availability.

Proposal 3: RAN1 to discuss whether HARQ feedback is needed between two active radio frames separated according to the periodicity N.

Proposal 4: RAN1 should study how to have accurate DL synchronization and cell search in this IoT NTN TDD mode.

Proposal 5: RAN1 to send LS to RAN4 to study both DL synchronization and cell search issue in this IoT NTN TDD mode.

Proposal 6: RAN1 to discuss the impact on the achievable link budget if the number of repetitions is restricted due to the use of every Nth radio frame.

Proposal 7: RAN1 to evaluate the cell capacity when accounting for at least NPSS, NSSS, NRS, MIB and relevant System Information Blocks (at least SIB1-5 and SIB31).

Proposal 8: RAN1 should evaluate the UL capacity for NPRACH and NPUSCH considering the active radio frame per N radio frame.

# References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024