3GPP TSG-RAN WG1 Meeting #118bis	R1-2408366

Hefei, China, October 14th–18th, 2024

Agenda Item:	9.13

Source:	Ericsson

Title:	Link adaptation TEI-19

Document for:	Discussion, Decision

# 1	Problem description

In our testing of link adaptation with commercial NW and UE devices we encountered a problem with link adaptation for some devices, that degrades the downlink throughput in the system.

On the NW side, the target BLER for PDSCH transmissions is a parameter choice for the scheduler and the NW side target BLER is achieved using outer loop link adaptation (OLA), where the scheduled MCS for a UE is adjusted/fine-tuned using e.g., the historic ACK/NACK reported by the UE.

On the UE side, the UE shall report CQI according to the highest possible MCS with a BLER not exceeding 10% (if CQI Table 1 and 2 is configured) or 0,001% (if Table 3 is configured).

Our tests indicate that when the NW side target BLER target is different from the CQI BLER target, then throughput degrades for some devices compared to the case of aligned BLER targets. This should not be the case, since the CQI feedback according to BLER target, that is defined by specifications, should not be affected by the success or failure of ongoing PDSCH receptions.

Some UE devices seem to adapt the reported CQI based on the success/failure of ongoing PDSCH transmissions, which should not be the case as it causes interference with the convergence of the NW side outer loop link adaptation (OLA)

In Figure 1, is a time trace plot for the rank indicator (RI) and CQI for a commercial UE shown, captured at low SINR, using CQI Table 1 (10% BLER target) where the NW side BLER target is set to 10% and 1% respectively. It can be seen when NW BLER target and CQI BLER target is aligned (10%), then CQI report is stable (blue plot) and rank is also stable at RI=4 reporting.

However, when there is a mismatch as in the 1% case (orange plot), then the CQI and RI in the CSI report fluctuates heavily, which implies that the UE may (also) take the PDSCH ACK/NACK into account, since NW side OLA will lower the MCS to reach its 1% target, and the UE reacts to this and increase the CQI (which it shouldn’t).

Figure 1 Rank indicator and time averaged CQI for a commercial UE when the CQI BLER target is 10% and the NW side BLER target is set to 10% (blue) and 1% (orange).

Another example is shown below where Table 3 is used (0,001% BLER target for CQI) and where the NW side BLER target is 10%. The reason why the NW side would like to use Table 3 is to extend the coverage, while still run with the “eMBB-range” BLER targets of 0,1% 1% or 10%.

Here it is seen that the UE is trying to lower the reported CQI (green curve) to reach its target 0,001% while at the same time the NW side is increasing the OLA offset (turquoise curve) to reach its target of 10%, hence the efforts by the UE and NW side adjustments are contradicting which causes unexpected events.

Figure 2 Traces of parameters where CQI,RI is reported by the UE and OLA,BLER and MCS is parameters set by the NW side OLA.

Hence, there is a tug of war between NW OLA and UE CQI adaptation which causes disturbances and throughput degradation for the UE.

It is useful for app coverage (see Section 2) and cell edge UEs to use CQI table 3 (currently only for the 0,001% BLER target) without the very low URLLC based BLER target.

Also, based on the above observations of real world performance, it is useful if the NW side OLA BLER target and the UE side CQI BLER target is the same. Hence, we propose as a TEI-19:

- Proposal 1 As a TEI-19, introduce an RRC parameter that indicates one target from a set of BLER targets [0.1, 0.01, 0.001, 0.0001, 0.00001] to be used for CQI reporting, independently of which CQI table is configured

# 2 App Coverage for CQI table 3 with 10% BLER target

In below, we show an evaluation of the DL throughput as a function of SNR for a 10% CQI BLER target (and same for NW side BLER target) while using CQI Table 3 (which currently only support 0,001% BLER target). It can be seen that the cell edge DL throughput increases. Hence, this improves the throughput at cell edge, which benefits the use of apps, hence better app coverage.

Figure 3 DL throughput as a function of SNR

# 3	TEI-19 solution

The specification change is related to introducing a new RRC parameter, e.g.,  BLER_target-r19 with range [10, 1, 0.1, 0.01, 0.001, 0.0001, 0.00001]% (and the associated UE capability) and update specification as follows for TS 38.214:

[..]

A single PDSCH transport block with a combination of modulation scheme, target code rate and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding the value configured by the higher layer parameter BLER_target-r19, if configured, otherwise:

-     0.1, if the higher layer parameter cqi-Table in CSI-ReportConfig configures 'table1' (corresponding to Table 5.2.2.1-2), or 'table2' (corresponding to Table 5.2.2.1-3), or

-     0.00001, if the higher layer parameter cqi-Table in CSI-ReportConfig configures 'table3' (corresponding to Table 5.2.2.1-4).

[..]

# 4.  Conclusion

Observation 1	Some UE devices seem to adapt the reported CQI based on the success/failure of ongoing PDSCH transmissions, which should not be the case as it causes interference with the convergence of the NW side outer loop link adaptation (OLA)

Hence, we propose

Proposal 1 As a TEI-19, introduce an RRC parameter that indicates one target from a set of BLER targets [10, 1, 0.1, 0.01, 0.001, 0.0001, 0.00001]% to be used for CQI reporting, independently of which CQI table is configured