3GPP TSG RAN WG1 #118bis		R1-2408407

Hefei, China, October 14th – 18th, 2024

Agenda Item: 	9.3.1

Source: 	Sony

Title: 	SBFD Tx/Rx/Measurement Procedures

Document for: 	Discussion / decision

# Introduction

In RAN1#118 we agreed the following:

Agreement

For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported

- A starting slot index

- A starting symbol index within the starting slot

- An ending slot index

- An ending symbol index within the ending slot

Working Assumption

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

Conclusion

If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

Agreement

For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition.

Agreement

Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2.

- Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols.

Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH.

- Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols.

  - Support separate configurations for DCI format 0_2 and other DCI formats as legacy

  - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy

UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions.

FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

Agreement

Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource.

- pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols

- Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols

  - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols

- FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2

- No change on the maximum number of PUCCH resources supported by a UE

- Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource

FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

Working Assumption

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

  - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only

  - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition.

  - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

Agreement

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following.

- For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC.

  - It is not applicable to PDCCH

- For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception.

- For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options:

  - Option 1:

    - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig.

    - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig.

    - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config.

    - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource].

  - Option 2:

    - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation.

    - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI.

    - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

    - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1,

- For PUSCH repetition type A with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type.

- For CG PUSCH and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

Agreement

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols

  - The numbers of PRBs are the same for PUSCH transmissions in SBFD symbols and PUSCH transmissions in non-SBFD symbols

Agreement

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters.

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

  - FFS: No RRC impact

  - FFS: New MAC CE or reuse existing MAC CE

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

  - FFS: Impact on MAC CE

- Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols

  - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS

    - FFS: Same offset or different offsets for all channels

  - FFS: RRC or MAC CE introduced to convey power offset

- FFS whether the maximum number of closed power control loops is increased to be more than 2.

- FFS: Whether/How to handle the case where UL power control is not associated within TCI state

This contribution discusses some SBFD operations aspects.

# Discussions

## Frequency Domain Configurations

### Useable PRB

For determining UL/DL useable PRBs, we consider the following options in RAN1#116:

- Option 1: UL useable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL useable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

- Option 2: UL/DL useable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

In Option 1, the SBFD subbands frequency locations are firstly configured in a cell specific manner, and the subset of the cell specific PRBs forming the active BWP determines the subbands of the active BWP.  For example, in Figure 1, in Option 1, the cell specific carrier bandwidth is from f0 to f6, where DL Subband#1 is from f0 to f1, UL Subband#1 is from f1 to f3, and DL Subband#2 is from f3 to f6.  The active BWP is configured to occupy frequencies f2 to f5, which in Option 1, the active BWP intersects the cell specific UL Subband#1 between f2 to f3, thereby forming an UL Subband#2 with UL useable PRBs from f2 to f3.  Similarly, the active BWP intersects the cell specific DL Subband#2 between f3 to f5, thereby forming a DL Subband#3 with DL useable PRBs from f3 to f5.

In Option 2, the SBFD subbands frequency locations are independently configured.  For example, in Figure 1, the active BWP in Option 2, also ranges from f2 to f5 but it consists of DL Subband#4 with DL useable PRBs from f2 to f4, and UL Subband#3 with UL useable PRBs from f4 to f5.  Since, it is a working assumption that the SBFD subbands frequency locations are cell specifically configured, the subbands frequency locations of the active BWP in Option 2 is configured in a UE dedicated and may be configured such that they are not aligned.  That is, in the example in Figure 1, the DL useable PRBs from f2 to f3 in DL Subband#4 have opposite link direction to the UL PRBs from f2 to f3 in the cell specific configuration.  Similarly, UL useable PRBs from f4 to f5 of UL Subband#3 have opposite link directions to the DL PRBs in f4 to f5 of the cell specific configuration.  This may cause intra subband CLI if the cell specific subbands are used by another UE.  On the other hand, Option 2 provides full flexibility for the gNB to manage the subbands of individual UEs.

Figure 1: Useable PRB

Currently, the BWPs are independently configured and the RAN2 specifications already have the mechanism to configure the frequency locations of individual BWP, that is, the configuration mechanism known as “Common Frequency Resource” used for MBS can be used as a basis for configuring subbands in BWP supporting SBFD.  Hence, it the configuration of subband can reuse the “Common Frequency Resource” mechanism which would reduce the complexity in specifying the frequency resources of the subband configurations.

Observation 1: The explicit configuration of UL/DL useable PRBs within active UL/DL BWP in SBFD symbols (Option 2), can reuse the existing RAN2 specification signalling mechanism for MBS Common Frequency Resource.

Proposal 1: The UL/DL useable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols (Option 2).

For the indication of UL and DL subbands, a working assumption was made to use the RIV.  This is a straightforward method with flexibility on the locations of each subband, and hence the working assumption should be confirmed.

Proposal 2: Confirm the following working assumption:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

## Time Domain Configurations

### Signalling

It was agreed that the SBFD time locations are cell specifically configured.  In previous meeting, we discussed whether this cell specific configuration is performed in the SIBs or in UE dedicated RRC message.  Since PRACH is supported at least for CFRA, the SBFD time configuration should at least be indicated in the SIBs.

Proposal 3: SBFD time domain configurations is indicated in the SIBs.

### Interactions with TDD-UL-DL-ConfigDedicated and SFI

In RAN1#116, it was agreed to further discuss the SBFD UE behaviour under legacy slot format configurations TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0.  These legacy configurations are used to configure Flexible OFDM symbols to DL or UL.  For Flexible OFDM symbols, we do not see any need to change the behaviour for SBFD UE and hence the legacy behaviour should be used.

For SBFD OFDM symbols that are configured from Flexible OFDM symbols, TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0 indicates which subbands are activated and deactivated.  That is, the subband with the same link direction as that of the indicated by TDD-UL-DL-ConfigDedicated and SFI, are activated and hence can be used, whilst the subband with the opposite link direction is deactivated, that is it is not used for transmission or reception.  The guardbands are also used for transmission or reception based on the indicated link direction.  An example is shown in Figure 2, where TDD-UL-DL-ConfigCommon configures a {DDDSU} TDD pattern, which is further configured into SBFD pattern {XXXXU}.  An SFI signals an index = 0, which indicates that all Flexible OFDM symbols in a slot are used as DL for legacy UE.  For SBFD UE, the DL subbands are active and the UL subband is deactivated, that is, the SBFD UE will not transmit in the UL subband.  If guardbands are configured, they are used for DL.  This reduces CLI to legacy UEs, since legacy UEs may receive DL in frequencies that overlap the UL subband of SBFD UEs, and so by deactivating the UL subband, the CLI is removed.

For SBFD OFDM symbols that are configured from DL OFDM symbols, the SBFD UE should ignore the indications from TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0, since these legacy slot format configurations are only applicable to Flexible OFDM symbols.

Figure 2: Deactivating UL subband using SFI

Proposal 4: The behaviour of SBFD capable UE on legacy slot format configurations, TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0 configurations are:

- For Flexible OFDM symbols: Follow legacy behaviour, i.e., use the Flexible OFDM symbols as DL or UL as per the indication in the legacy slot format configurations.

- For SBFD OFDM symbols configured on Flexible OFDM symbols: Deactivate the subband with the opposite link direction as the link direction indicated by the legacy slot format configurations. If guardbands are configured, the guardbands are used for DL or UL if the legacy slot format configurations indicate the OFDM symbol to be DL or UL respectively.

- For SBFD OFDM symbols configured on DL OFDM symbols: Ignore the indication in the legacy slot format configurations.

## Transmissions/receptions across SBFD & non-SBFD slots

A transmission that is periodic or with repetition may have instances of that transmission in SBFD OFDM symbols and other instances in non-SBFD OFDM symbols in different slots.  If the UL and DL transmissions fully resides within the UL subband and DL subband respectively, there isn’t any issue in resource allocation (there may be issues with different transmission parameters).  In the previous meeting we agreed on two configurations to handle this issue:

- Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

- Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

The granularity of Configuration 1 and Configuration 2 was FFS.  Different transmission/reception has different requirements.  For example, URLLC transmission may benefit from Configuration 2 since it has low latency whilst eMBB is delay tolerant and can use Configuration 1.  Hence, the granularity should be per signal basis.

Proposal 5: The granularity of Configuration 1 and Configuration 2 in handling transmissions/receptions across SBFD and non-SBFD slots is per signal basis.

### Configuration 1

It was agreed that the valid symbol type used for transmissions/receptions for semi-statically configured transmissions/receptions without activation DCI are explicitly indicated by RRC, whilst for dynamically scheduled transmissions/receptions, the valid symbol type is the symbol type used for the first transmission/reception.  For semi-persistent transmissions/receptions such as CG-PUSCH Type 2 or SPS where activation DCI is used to start and stop the transmission/reception, the valid symbol type can either be RRC configured as for semi-static transmissions/receptions or based on the symbol type of the first transmission/reception.  The benefit of using an activation DCI is that it provides flexibility in some of the transmissions/receptions parameters.  This flexibility should be maintained for Configuration 1 and therefore the valid symbol type of semi-persistent transmissions/receptions is based on the symbol type of the first transmission/reception.

Proposal 6: For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, is determined as:

- Option 2:

  - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation.

  - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI.

  - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

  - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

### Configuration 2

#### PDSCH

For Configuration 2, it was agreed that for DL transmissions across SBFD and non-SBFD slots, including SPS, PDSCH with repetitions, and multi-PDSCH scheduled by a single DCI, only assigned PRBs within DL usable PRBs in SBFD symbols are considered valid.  That is the PDSCH is rate matched around non-DL usable PRBs.  It was FFS on whether the TBS is determined based on assigned PRBs or assigned PRBs within DL usable PRBs only.  For all cases using the assigned PRBs would have less specs impact.  Although this may lead to higher code rate for cases where the number of assigned PRBs within DL usable is less than assigned PRBs, the gNB can compensate that with lower MCS it deems necessary.

Proposal 7: For SPS PDSCH without repetitions, PDSCH with repetitions and multi-PDSCH scheduled by a single DCI under Configuration 2 (i.e. the PDSCH instances can be in SBFD or non-SBFD symbols), the TBS is determined based on assigned PRBs.

### CSI Reporting

In RAN1#117, we agreed the following options for CSI reporting:

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

  - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only.

  - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

It should be noted that it should be possible that more than one of the above options can be implemented and up to the gNB to configure one or more options for the UE.

Observation 2: The gNB should be able to configure one or two CSI-ReportConfig with separate CSI measurements, where one CSI measurement is derived from CSI-RS in SBFD symbols and another CSI measurement is derived from CSI-RS in non-SBFD symbols.

Proposal 8: The gNB can configure the following CSI report for SBFD UE:

- Configuration A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

- Configuration B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

### TCI State

The gNB may change antenna panel for transmission/reception in SBFD and non-SBFD slots, which will change the QCL assumption of a transmissions with multiple instances, such as repetitions and periodic transmissions.  For example, in Figure 3, a PDSCH with 2× repetitions is transmitted in Slot n and Slot n+1, which is non-SBFD symbol and SBFD slot respectively.  For the transmission in non-SBFD Slot n, the gNB uses the traditional “TDD” panel which is used for DL and UL.  For the transmission SBFD Slot n+1, the gNB uses the SBFD “DL” panel to provide spatial isolation with the SBFD “UL” panel.  That is, the 1st PDSCH repetition and the 2nd PDSCH repetition are transmitted from different antenna panels and therefore they have different QCL assumptions.

Figure 3: Different QCL assumptions for SBFD and non-SBFD slots

Observation 3: For repetitive and periodic transmissions/receptions across SBFD and non-SBFD OFDM symbols, the QCL assumption may be different for SBFD OFDM symbols and non-SBFD OFDM symbols.

It may be beneficial to indicate different TCI state indications for transmission in SBFD and non-SBFD OFDM symbols, notably for cases where the channel contains multiple instances, e.g., repetitions or periodic transmissions, that crosses SBFD and non-SBFD slots.

Proposal 9: Support different TCI state indications for transmissions in SBFD and non-SBFD OFDM symbols.

### UL Power Control

Since an UL transmission may experience different radio condition in SBFD and non-SBFD OFDM symbols, it was agreed that different UL power control is supported for such transmissions.  It was agreed that for a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, and the following options were considered:

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols

The gNB may use different antenna panel for UL reception in SBFD and non-SBFD OFDM symbols, which may have a different TCI states as described in previous section.  The UE may also use different UL beams for SBFD and non-SBFD OFDM symbols, and hence among Option 1 where separate TCI states are used for SBFD and non-SBFD OFDM symbols is the most appropriate.

Observation 4: gNB may use different antenna panel for UL reception in SBFD and non-SBFD OFDM symbols, which may result in different TCI states.

Proposal 10: For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework,

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

## Transmissions across SBFD & non-SBFD OFDM symbols in a slot

It was agreed in the SI that SBFD and non-SBFD OFDM symbols can be configured within a slot.  However, the transmission across SBFD and non-SBFD OFDM symbols within a slot has not been concluded.  The main issue with a single instance of a transmission across SBFD and non-SBFD OFDM symbols within a slot is the gNB may change hardware, particularly the antenna panels when the transmission moves from SBFD to non-SBFD OFDM symbols and vice-versa.  That is the gNB may use separate antenna panels for DL and UL when transmission and receiving in SBFD OFDM symbols for SBFD aware UE to reduce self-interference caused by CLI from DL transmission into UL reception at the gNB.  This change of antenna panels between SBFD and non-SBFD OFDM symbols cause discontinuity in the channel at the switching point as shown Figure 4 at time t1 for the PDSCH and time t3 for the PUSCH.

Figure 4: Discontinuity in channel between SBFD & non-SBFD OFDM symbols

One way to avoid channel discontinuity is for the gNB to maintain the same antenna panel and transmission parameters, for the entire transmission of a channel within the slot.  This is similar to the case for long PRACH format that crosses SBFD and non-SBFD slots agreed in Agenda 9.3.2 [1].  Figure 5 shows four cases where a PUSCH and PDSCH transmission crosses SBFD and non-SBFD OFDM symbols within a slot, and the antenna panel is not changed.  Here we assume that the start of the transmission uses the “TDD” antenna panel, i.e., the existing one used for both DL & UL, if it starts in non-SBFD OFDM symbols, and the transmission uses SBFD DL or SBFD UL antenna panel if the transmission starts in DL subband or UL subband of SBFD OFDM symbols respectively.  It is also likely that the “TDD” antenna panel is used for DL transmission and UL reception for legacy UE.  In the four cases in Figure 5, it is observed that for an UL transmission that starts in UL OFDM symbols (non-SBFD) and proceeds to the UL subband of SBFD OFDM symbols, and if the antenna panel is maintained as the “TDD” antenna panel, it will cause self-interference at the gNB, if a legacy UE, e.g., UE2, is being scheduled with a DL transmission in the SBFD OFDM symbols.  For the other cases, maintaining the same antenna panel does not cause any additional self-interference at the gNB.

Figure 5: Transmission across SBFD and non-SBFD OFDM symbols without changing antenna panel

Observation 5: Similar to long PRACH format across SBFD and non-SBFD OFDM symbols, maintaining the same antenna panel for transmission or reception of DL or UL channel that crosses SBFD and non-SBFD OFDM symbols may avoid channel discontinuity as the transmission/reception switches from SBFD to non-SBFD OFDM symbols and vice-versa.

Observation 6: In some cases, the gNB may not be able to maintain the same antenna panel for the duration of a channel that crosses SBFD and non-SBFD OFDM symbols.  For example, the gNB may need to change antenna panel to reduce self-interference for an UL reception that starts in UL OFDM symbols and switches to SBFD OFDM symbols.

One way to take the cases in Figure 5 into account is to allow the UE to receive all DL channels that crosses SBFD and non-SBFD OFDM symbols since it is feasible for the gNB to maintain the same antenna panel, and UL channel that starts in SBFD OFDM symbols.  For UL channel that starts in UL OFDM symbols (non-SBFD OFDM symbols), the UE will drop the UL transmission.

There may be other reasons for the gNB to use different antenna panels and transmission parameters in some cases and maintain the same antenna panel and transmission parameters for other cases, for a channel that crosses SBFD and non-SBFD OFDM symbols.  Hence, the gNB should be provided with the flexibility on whether to change antenna panels and transmission parameters or not for a transmission that crosses SBFD and non-SBFD OFDM symbols in a slot.  Here, the gNB can indicate whether there is a change in antenna panel or transmission parameters or configurations in the DCI scheduling the transmission.  If the gNB indicates that a transmission uses different transmission parameters due to changes in antenna panels, the gNB provides DMRS and different TCI state indications for the SBFD and non-SBFD portion of the channel.

Observation 7: It is beneficial to provide flexibility for the gNB to decide whether a transmission that crosses SBFD and non-SBFD OFDM symbols in a slot uses a different or the same antenna panels and transmission parameters.

Whether the gNB maintains the same antenna panel and transmission parameters or different antenna panels and transmission parameters for transmission across SBFD and non-SBFD OFDM symbols in a slot, it is beneficial that such transmission is allowed to provide flexibility in scheduling.  The alternative is to drop ALL transmissions & receptions that crosses SBFD and non-SBFD OFDM symbols in a slot, which would significantly reduce the resource usage, thereby defeating the purpose of SBFD and is against the justification of the WI.

Observation 8: Dropping all transmissions or receptions that crosses SBFD and non-SBFD OFDM symbols in a slot will reduce the resource usage, thereby defeating the purpose of SBFD and is against the justification of the WI.

Proposal 11: Transmission across SBFD and non-SBFD OFDM symbols in a slot is allowed.

Proposal 12: For transmission across SBFD and non-SBFD OFDM symbols in the same slot, consider the following options:

- Option 1: The UE drops an UL transmission that starts in UL OFDM symbols and switches to SBFD OFDM symbols within a slot, however the UE transmits UL transmissions that starts in SBFD OFDM symbols (i.e., similar to long PRACH format across SBFD and non-SBFD OFDM symbol), and the UE receive all DL reception that crosses SBFD and non-SBFD OFDM symbols regardless if it starts with SBFD OFDM symbol or non-SBFD OFDM symbol.

- Option 2: The gNB indicates whether the transmission parameters, e.g., due the different antenna panel, change in the portions of the channel that are in SBFD and non-SBFD OFDM symbols.  If there is a change in transmission parameters, the gNB further provides DMRS and different TCI state indications for the SBFD and non-SBFD portions of the channel.

## Link Direction Indicator

Link direction indicator has been proposed in the previous meetings where a UE is indicated to use only the UL subband or only the DL subband in a slot.  One of the proposed methods for link direction indicator is to reinterpret the TDD-UL-DL-ConfigDedicated parameter to indicate link direction in SBFD OFDM symbols [2] for SBFD aware UE.  That is, in the TDD-UL-DL-ConfigDedicated is applicable to all OFDM symbols types instead of just flexible OFDM symbols.  Link direction indicator can be beneficial to resolve DL & UL collisions, especially for the case of semi-static DL reception (e.g. SPS) colliding with semi-static UL transmission (e.g. CG-PUSCH) in a UE, since currently this case is considered an error case and it may be difficult for the gNB to avoid such collisions if there is a large number of SPS and CG-PUPSCH being configured.

Observation 9: Link direction indicator can be beneficial to resolve collision in a UE between semi-static DL reception (e.g. SPS) against semi-static UL transmission (e.g. CG-PUSCH), by explicitly indicating whether the UE should be receiving DL or transmitting in the UL.

The parameter TDD-UL-DL-ConfigDedicated is semi-statically configured, which means that the UE is denied significant portion of frequency resources.   For example, in a {XXXXU} configuration, if the link direction indicates an SBFD slot “X” as UL, the UE DL resources is reduced for that slot in every TDD period.  If the UL subband occupies 20% of the frequency bandwidth as per the scenarios considered in the Rel-18 SI, then 80% of downlink resources are denied from the UE in the indicated slot.  This will significantly reduce the UE accessible resources, which defeats the purpose of introducing SBFD as one of the main objectives is to improve UE capacity.  If there are no collision in an SBFD slot, then the link direction indicator needlessly prevents the UE from accessing a significant portion of frequency resources.

Observation 10: Since TDD-UL-DL-ConfigDedicated is a semi-static signal, using TDD-UL-DL-ConfigDedicated as a link direction indicator would deny a UE from accessing a significant portion of resources, which defeats the purpose of introducing SBFD for capacity enhancement.

Observation 11: If there is no DL & UL collision in an SBFD slot, a semi-statically configured link direction indicator such as TDD-UL-DL-ConfigDedicated would unnecessarily deprive a UE from significant portion of frequency resources.

Instead of using TDD-UL-DL-ConfigDedicated, the SFI can be used instead which is dynamic.  However, the SFI targets a group of UE, instead of individual UEs, which reduces the flexibility of the gNB in controlling individual UEs.  The SFI is also not as reliable as TDD-UL-DL-ConfigDedicated.

Observation 12: Since SFI is a GC-DCI, using SFI as link direction indicator reduces the gNB flexibility in controlling the link direction of individual UEs.

One way to resolve this issue is to allow a semi-statically configured link direction indicator to be overwritten. Notably a dynamic grant should be able to overwrite a link direction indicator.  An example is shown in Figure 6, where in a {DXXXU} TDD pattern, the TDD-UL-DL-ConfigDedicated indicates that the link direction for the 4th slot of the TDD pattern to be DL, meaning the UL subband for that SBFD slot is disabled, as in Slot n+3 and Slot n+8 in Figure 6.  In Slot n+3, there is a DL & UL collision where a CG-PUSCH collides with an SPS, where without a link direction indicator this would be an error case causing both SPS & CG-PUSCH to be dropped.  However, since the link direction indicator, indicates DL, the CG-PUSCH is dropped and the SPS is received.  In Slot n+5, the gNB sends an UL grant to the UE to schedule a PUSCH in Slot n+8, where the UL subband is disabled by the link direction indicator.  However, in this case, the PUSCH is transmitted as the UL grant overwrites the instruction from the link direction indicator.  This is beneficial especially for URLLC UL transmission where the gNB may wish to schedule a dynamic URLLC PUSCH in a UL subband that has been disabled by the link direction indicator.

Figure 6: Dynamic grant overwrites link direction indicator

Since the link direction indicator is only beneficial to resolve collision, if there is no DL & UL collision in a slot, the link direction indicator should be overwritten.  An example is shown in Figure 7, where TDD-UL-DL-ConfigDedicated is used as a link direction indicator and indicate that the direction is UL for every 4th slot of a {DXXXU} TDD pattern.  In Slot n+3, there is a collision between CG-PUSCH and SPS, and since the link direction indicator indicates UL, the SPS is dropped and the CG-PUSCH is transmitted.  Although the link direction indicator indicates UL for Slot n+8, since there is no DL & UL collision, the SPS in that slot is received by the UE.

Figure 7: Absence of a collision means link direction indicator is not applied

In addition to dynamic grants and the absence of collisions, link direction indicator should also be overwritten if there is a PRACH transmission.  That is if a UE with a PRACH transmission in a valid RO that resides in an UL subband that has been disabled by a link direction indicator can overwrite it and transmit that PRACH.  This is because typically PRACH transmission in Connected Mode is used to maintain the radio link, e.g., in beam management, synchronization and radio link recovery, and therefore they are urgent messages.

Proposal 13: Support a UE dedicated semi-static link direction indicator.

Proposal 14: The link direction indicator is overwritten by:

- A dynamic grant:

  - If the SBFD OFDM symbols are indicated as DL by the link direction indicator, an UL grant or a DL grant would overwrite the link direction indicator and transmit the PUSCH or PUCCH respectively in the UL subband in these SBFD OFDM symbols.

  - If the SBFD OFDM symbols are indicated as UL by the link direction indicator, a DL grant would overwrite the link direction indicator and the UE would receive the PDSCH in the DL subband in these SBFD OFDM symbols.

- An absence of DL & UL collision.  If there is no DL & UL collision then the link direction indicator is not applicable.

- A valid RO with a PRACH transmission.

# Conclusion

In this contribution, we discuss some operations issues on SBFD, and we observe the following:

Observation 1: The explicit configuration of UL/DL useable PRBs within active UL/DL BWP in SBFD symbols (Option 2), can reuse the existing RAN2 specification signalling mechanism for MBS Common Frequency Resource.

Observation 2: The gNB should be able to configure one or two CSI-ReportConfig with separate CSI measurements, where one CSI measurement is derived from CSI-RS in SBFD symbols and another CSI measurement is derived from CSI-RS in non-SBFD symbols.

Observation 3: For repetitive and periodic transmissions/receptions across SBFD and non-SBFD OFDM symbols, the QCL assumption may be different for SBFD OFDM symbols and non-SBFD OFDM symbols.

Observation 4: gNB may use different antenna panel for UL reception in SBFD and non-SBFD OFDM symbols, which may result in different TCI states.

Observation 5: Similar to long PRACH format across SBFD and non-SBFD OFDM symbols, maintaining the same antenna panel for transmission or reception of DL or UL channel that crosses SBFD and non-SBFD OFDM symbols may avoid channel discontinuity as the transmission/reception switches from SBFD to non-SBFD OFDM symbols and vice-versa.

Observation 6: In some cases, the gNB may not be able to maintain the same antenna panel for the duration of a channel that crosses SBFD and non-SBFD OFDM symbols.  For example, the gNB may need to change antenna panel to reduce self-interference for an UL reception that starts in UL OFDM symbols and switches to SBFD OFDM symbols.

Observation 7: It is beneficial to provide flexibility for the gNB to decide whether a transmission that crosses SBFD and non-SBFD OFDM symbols in a slot uses a different or the same antenna panels and transmission parameters.

Observation 8: Dropping all transmissions or receptions that crosses SBFD and non-SBFD OFDM symbols in a slot will reduce the resource usage, thereby defeating the purpose of SBFD and is against the justification of the WI.

Observation 9: Link direction indicator can be beneficial to resolve collision in a UE between semi-static DL reception (e.g. SPS) against semi-static UL transmission (e.g. CG-PUSCH), by explicitly indicating whether the UE should be receiving DL or transmitting in the UL.

Observation 10: Since TDD-UL-DL-ConfigDedicated is a semi-static signal, using TDD-UL-DL-ConfigDedicated as a link direction indicator would deny a UE from accessing a significant portion of resources, which defeats the purpose of introducing SBFD for capacity enhancement.

Observation 11: If there is no DL & UL collision in an SBFD slot, a semi-statically configured link direction indicator such as TDD-UL-DL-ConfigDedicated would unnecessarily deprive a UE from significant portion of frequency resources.

Observation 12: Since SFI is a GC-DCI, using SFI as link direction indicator reduces the gNB flexibility in controlling the link direction of individual UEs.

We therefore propose the following:

Proposal 1: The UL/DL useable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols (Option 2).

Proposal 2: Confirm the following working assumption:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

Proposal 3: SBFD time domain configurations is indicated in the SIBs.

Proposal 4: The behaviour of SBFD capable UE on legacy slot format configurations, TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0 configurations are:

- For Flexible OFDM symbols: Follow legacy behaviour, i.e., use the Flexible OFDM symbols as DL or UL as per the indication in the legacy slot format configurations.

- For SBFD OFDM symbols configured on Flexible OFDM symbols: Deactivate the subband with the opposite link direction as the link direction indicated by the legacy slot format configurations. If guardbands are configured, the guardbands are used for DL or UL if the legacy slot format configurations indicate the OFDM symbol to be DL or UL respectively.

- For SBFD OFDM symbols configured on DL OFDM symbols: Ignore the indication in the legacy slot format configurations.

Proposal 5: The granularity of Configuration 1 and Configuration 2 in handling transmissions/receptions across SBFD and non-SBFD slots is per signal basis.

Proposal 6: For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, is determined as:

- Option 2:

  - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation.

  - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI.

  - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

  - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

Proposal 7: For SPS PDSCH without repetitions, PDSCH with repetitions and multi-PDSCH scheduled by a single DCI under Configuration 2 (i.e. the PDSCH instances can be in SBFD or non-SBFD symbols), the TBS is determined based on assigned PRBs.

Proposal 8: The gNB can configure the following CSI report for SBFD UE:

- Configuration A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

- Configuration B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

Proposal 9: Support different TCI state indications for transmissions in SBFD and non-SBFD OFDM symbols.

Proposal 10: For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework,

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

Proposal 11: Transmission across SBFD and non-SBFD OFDM symbols in a slot is allowed.

Proposal 12: For transmission across SBFD and non-SBFD OFDM symbols in the same slot, consider the following options:

- Option 1: The UE drops an UL transmission that starts in UL OFDM symbols and switches to SBFD OFDM symbols within a slot, however the UE transmits UL transmissions that starts in SBFD OFDM symbols (i.e., similar to long PRACH format across SBFD and non-SBFD OFDM symbol), and the UE receive all DL reception that crosses SBFD and non-SBFD OFDM symbols regardless if it starts with SBFD OFDM symbol or non-SBFD OFDM symbol.

- Option 2: The gNB indicates whether the transmission parameters, e.g., due the different antenna panel, change in the portions of the channel that are in SBFD and non-SBFD OFDM symbols.  If there is a change in transmission parameters, the gNB further provides DMRS and different TCI state indications for the SBFD and non-SBFD portions of the channel.

Proposal 13: Support a UE dedicated semi-static link direction indicator.

Proposal 14: The link direction indicator is overwritten by:

- A dynamic grant:

  - If the SBFD OFDM symbols are indicated as DL by the link direction indicator, an UL grant or a DL grant would overwrite the link direction indicator and transmit the PUSCH or PUCCH respectively in the UL subband in these SBFD OFDM symbols.

  - If the SBFD OFDM symbols are indicated as UL by the link direction indicator, a DL grant would overwrite the link direction indicator and the UE would receive the PDSCH in the DL subband in these SBFD OFDM symbols.

- An absence of DL & UL collision.  If there is no DL & UL collision then the link direction indicator is not applicable.

- A valid RO with a PRACH transmission.

# References

[1] R1-2405656, “Summary#4 on SBFD random access operation,” Moderator (CMCC), RAN1#117

[2] R1-2404516, “Discussion on SBFD TX/RX/measurement procedures,” MediaTek, RAN1#117

subband non-overlapping full duplex for NR,” Nokia, RAN1#112