3GPP TSG RAN WG1 Meeting #118bis		R1-2408424

Hefei, China, Oct. 14th – 18th, 2024

Agenda Item: 	9.10.1

Source: 	Sony

Title: 	Views on enabling TX/RX for XR during RRM measurements

Document for: 	Discussion & Decision

# Introduction

eXtended Reality (XR) for NR phase 3 work item (WI) was revised in RANP#105 [1]. The WI is also in conjunction with the work during the study item phase [2]. The WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including also multi-modal applications. The WI also aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited. The Rel-19 XR ph3 objective that being addressed in this contribution is:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In RAN1#118 meeting, the following agreements were made [3]:

Agreement Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

This contribution provides our views on enabling transmission/reception in measurement gaps / scheduling restrictions period to support XR application.

# Discussion

Apart from performing data transmission and reception, a UE is also required to perform RRM measurements. For example, if the signal quality of the serving cell is deteriorating, such as below certain threshold then the UE starts neighbor-cell measurements. A UE shall measure the neighboring cells signal and other carrier components. In the legacy procedure, once the UE is configured to perform RRM measurement in a given time, the UE shall perform RRM measurement and drop any uplink / downlink data transmission during the RRM measurement time. In Figure 1, we provide an example where the Measurement Gap with certain Length (MGL) collides with Uplink Configured Grant (CG). Based on the legacy procedure, the CG transmission is dropped and the UE performs RRM measurement.

Figure 1: Collision of Measurement During MGL and Data Tx/Rx

To support XR application, the UE may require continuing the transmission and skip the RRM measurement. Several proposals / solutions have been proposed in the last few meetings. In RAN1#118, it was agreed as a working assumption on the solution based on the dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. The dynamic indication is provided from gNB to UE via downlink control information (DCI) carried out by PDCCH. This approach is illustrated in Figure 2.

Figure 2: Illustration of Skip / Release Measurement to support data Tx/Rx

The skip measurement here means the UE can continue CG transmission during the affected measurement gap. The other solution is to provide the indication semi-statically. We consider this solution may not be able to support XR characteristics. The gNB may not be known the UE traffic far in advance. Hence, we support to confirm the working assumption to support the dynamic indication.

Proposal 1: Confirm the working assumption to support dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. The dynamic indication is an explicit indication by DCI to skip a particular gap/restriction.

Regarding the dynamic indication information in DCI, our preference is that indication is represented in multiple bits. Hence, the gNB has more flexibility in indicating which gap / restriction to be skipped. However, we agreed that the DCI’s bit-field size is one bit. In this case, the gNB can only indicate the upcoming / nearest gap / restriction to be skipped. In practice, this bit-field can be carried out in a new DCI or a new field in the legacy DCI. Considering the size is relatively small (i.e., only one bit). This new bit-field can be carried out in the legacy DCI which has been configured for this purpose. This bit-field can be introduced, for example in DCI X_1.

Proposal 2: Support to introduce a new bit-field in the existing DCI format (e.g., DCI x_1).

The XR traffic that may collide with RRM measurement may not necessarily be a video transmission which may require a large payload. It can also be the motion control information which require a small payload size. For example, it can be transmitted within one slot. If the UE perform transmission / reception for such short period then in practice, the UE can still continue to perform RRM measurement for the remaining measurement gap. The UE is not necessarily needed to skip the entire measurement gap. The UE can still perform both XR transmission / reception and RRM measurement with reduced measurement gap length. The RRM measurement with reduced measurement gap length is basically the operation of partial skipping of gap(s) / restriction(s).

The partial skipping of gap(s)/restriction(s) has been discussed and being proposed by several companies and the direction so far is to postpone such operation pending the decision on the general framework (i.e., whether dynamic or semi-static solutions) [4]. Since that one has been agreed (at least as working assumption in RAN1#118), we should make the progress whether to support partial skipping operation. Our view is to support it.

Proposal 3: Support partial skipping of gap(s)/restriction(s) to allow more scheduling opportunities.

For the case of uplink transmission, UE may have better information than gNB in terms of the uplink transmission characteristics. The UE has direct interaction with the upper layer (e.g., application layer) so that the UE can predict the upcoming uplink transmission characteristics. The uplink transmission can be in a form of an aperiodic transmission that require spontaneous transmission from the UE. It can also be in a form of semi-static / periodic transmission. Furthermore, the UE may have various types of uplink transmission. Some uplink transmission can be compromised, such that the legacy gap/restriction can still be applied. In this case, the UE can prioritize the measurement over the transmission. Some other uplink transmission (e.g., XR traffic for video, control, data) has higher priority over RRM measurement. In this case, the UE skips RRM measurement and performs UL transmission instead. Based on these reasonings, UE assistance information to serving gNB on indicating an upcoming temporary measurement gap modification (e.g., skipping) shall be supported. The gNB can take that information into account and decide whether to grant the temporary measurement gap modification or not. This mechanism would also ensure that the gNB is still in full control of the UE behaviour. Hence, any potential system performance degradation (e.g., due to the measurement gap skipping) can be avoided.

In RAN1#117, RAN1 could not reach consensus to support UE assistance information (UAI) related to measurements occasions. The reason is due to lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). Hence, RAN1 sends an LS to RAN4 requesting their views on UAI [5]. Here, we express our observation related to UAI.

Observation 1: UE has better knowledge, particularly for the UL traffic. Hence, UE assistance information could be beneficial in assisting gNB to allow XR traffic when there is a collision between XR traffic and RRM measurement.

Observation 2: UE assistance information indicating the number of gap(s) / restriction(s) that can be skipped during a configured RRM measurement is beneficial in the operation of XR transmission during RRM measurement.

Proposal 4: Support UE Assistance Information (UAI) indicating the gNB on the MG(s) to be skipped by the UE.

In case of the RRM measurement is affected by the temporary measurement gap modification, for example when one or more of the measurement gaps has been skipped, then the UE can also indicate that the reported RRM measurement has been affected. Skipping one or more measurement gaps is expected to reduce the quality of the reported RRM measurement from the UE, such as reducing the mobility performance. Alternatively, the UE is actually postponing the measurement. Hence, it may affect the latency in reporting the RRM measurement. This may require further study in other WG(s), such as RAN2 and RAN4.

Observation 3: Skipping RRM measurement may affect the quality and/or latency of the reported RRM measurement. It would be beneficial for gNB to know whether the RRM measurement has been compromised or not.

# Conclusion

In this contribution, we provide our views on enabling transmission/reception during measurement gap. The observations and proposals are listed below.

Observation 1: UE has better knowledge, particularly for the UL traffic. Hence, UE assistance information could be beneficial in assisting gNB to allow XR traffic when there is a collision between XR traffic and RRM measurement.

Observation 2: UE assistance information indicating the number of gap(s) / restriction(s) that can be skipped during a configured RRM measurement is beneficial in the operation of XR transmission during RRM measurement.

Observation 3: Skipping RRM measurement may affect the quality and/or latency of the reported RRM measurement. It would be beneficial for gNB to know whether the RRM measurement has been compromised or not.

Proposal 1: Confirm the working assumption to support dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. The dynamic indication is an explicit indication by DCI to skip a particular gap/restriction.

Proposal 2: Support to introduce a new bit-field in the existing DCI format (e.g., DCI x_1).

Proposal 3: Support partial skipping of gap(s)/restriction(s) to allow more scheduling opportunities.

Proposal 4: Support UE Assistance Information (UAI) indicating the gNB on the MG(s) to be skipped by the UE.

# References

- RP-241771, “Revised WID: XR (eXtended Reality) for NR Phase 3”, RANP #105, September 2024

- TR 38.838, “Study on XR (Extended Reality) Evaluations for NR”, v17.0.0, December 2021

- RAN1 #118 Chairman’s notes, Maastricht, Netherland, August 2024

- R1-2407511 “Moderator summary #3 - Enabling TX/RX for XR during RRM measurements”, Nokia

- R1-2405736 “LS on UE assistance information related to measurement occasions”, Nokia