3GPP TSG RAN WG1 #118bis						          R1-2408438

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.1.4.2

Source:	InterDigital, Inc.

Title:	On other aspects of AI/ML model and data

Document for:	Discussion and Decision

# Introduction

In RAN #102, the normative work on AI/ML for NR Air Interface has been approved with some additional studies which were not finalized during the study phase [1].  The work builds on the Rel-18 studies in FS_NR_AIML_Air [2].

The Rel-19 AI/ML air interface work item will provide support for the general framework for AI/ML for air interface discussed in Rel-18, and addresses some of the outstanding issues identified in TR 38.843 [3].  The study objectives of the work item were updated at the RAN #105 checkpoint ([13]), as shown below.

Study objectives ----------------------------------------------------------Text omitted -------------------------------------------------- - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

This contribution discusses aspects related to the model identification in the context of LCM for two-sided models, as well as aspects related to model transfer/delivery and contents of UE data collection.

# Discussion

## Model identification for two-sided models

RAN1 #116 [4] agreed to study several options for model identification Type B, as follows.

Agreement - To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases. - To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose - Note: other options are not precluded Observation The other options are proposed for model identification type B by companies during the discussion: - MI-Option 4. Model identification via standardization of reference models. (for CSI compression) - MI-Option 5. Model identification via model monitoring

Subsequently, in RAN1 #116b [10] it was clarified that MI-Option 4 refers to Option 1 of CSI compression inter-vendor training collaboration.

Conclusion - It is clarified that MI-Option 4 refers to the Option 1 of CSI compression - Option 1: Fully standardized reference model (structure + parameters)

During RAN1 #116b it was also observed that Option 1 corresponds to RAN4-Option3 or RAN4-Option4, as shown below.

Conclusion: - Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors). - It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4. Observation - Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed - Option 1 and 2 may require high specification effort from RAN1 perspective.

Given the potentially high specification effort in RAN1 for Option 1, and that further study and final conclusions for the corresponding RAN4-Option3, or RAN4-Option4 are up to RAN4, it may be beneficial to defer discussions on MI-Option4 until more progress is made in RAN4.

Proposal 1: Defer discussions on MI-Option4 until more progress is made in RAN4 (on RAN4-Option3, or RAN4-Option4).

RAN1 #118 [14] further reached the following conclusions regarding MI-Option2 for one-sided model and for MI-Option5, respectively.

Conclusion The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work. Conclusion The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work.

In this section, we provide our views regarding MI-Option2.

For the UE-part of a two-sided model, RAN1 #117 [11] agreed to further study an example of MI-Option2, as follows.

Agreement From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset

Based on the above description, including step A, it appears that MI-Option2 is refers to Option 4 of the CSI compression sub-agenda item for the inter-vendor training collaboration, where Option 4 is further defined in RAN1 #116b [10] as follows.

Agreement (amended as shown in red in Wednesday session) - For Option 3/4/5, focus further discussion on the following assumptions: - Option 4: - The dataset exchange originates from the NW-side and ends at the UE-side. - Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback). - Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI). - Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI). - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

Proposal 2: Clarify if MI-Option2 refers to Option 4 in the CSI compression sub-agenda item (“Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side”).

Under the assumption of NW-first training, the NW trains its NW-side proprietary CSI reconstruction model using the dataset and a generic (proxy) CSI generation model (i.e., encoder). At the end of Step A of AI-Example2-1, the dataset is available at the UE/UE-side. During Step B, the UE part of the two-sided model (i.e., the UE-side CSI generation model) is developed and trained at the UE-side based on at least the exchanged dataset.   The dataset transferred in Step A may not be sufficient for the UE/UE-side/UE vendor to train a UE-side CSI generation model that meets the performance requirement. To achieve this, additional information may need to be exchanged from the NW-side to the UE-side, for example intermediate KPI performance targets.   Using the exchanged dataset and the intermediate KPI performance targets, in Step B, the UE/UE vendor may develop and train the UE-side CSI generation model, using a UE-side proxy decoder.  However, it may still be possible that the UE-side CSI generation model is not fully compatible with the NW-side CSI reconstruction model (for example, if the backbone of the UE-side CSI generation and UE-side proxy reconstruction model does not match the backbone of the NW-side CSI reconstruction).  This may lead to considerable inter-vendor collaboration complexity to align the UE-side and NW-side models.

Observation 1: For MI-Option2, transferring only a dataset from the NW/NW-side to UE/UE-side via standardized signaling is not sufficient to ensure that the UE-part of the two-sided model (e.g., UE-side CSI generation) trained at the UE-side is compatible with the NW-part of the two-sided model (e.g., NW-side CSI reconstruction).

Observation 2:  MI-Option2 may lead to considerable inter-vendor collaboration complexity to align the UE-side and NW-side models.

## Data collection for model training

One of the objectives of the recently approved WI [1] is to study the CN/OAM/OTT collection of UE-sided model training data, as shown below.

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

As captured in TR 38.843 [3], data collection may be performed for different purposes, such as model training, model monitoring, model inference (where applicable), as well as model selection, and model update. Each of these goals result in different requirements for the data collection, at least in terms of latency.

For model training purposes, given the assumption of offline training, there is no latency requirement for data collection. In general, the collected data should include the model input and the corresponding ground truth, configuration information for the model input (including formatting), as well as site specific information (where/if applicable), for example to enable finetuning for site specific models.

In the reminder of this section, we present our views on the contents of the UE data collection for UE-sided model training data for the Beam Management, Positioning and CSI enhancements use cases.

### CSI enhancements use case

For CSI prediction using UE-sided model, data collection for model training should include at least the CSI measured during the observation window, and the corresponding ground truth (measured CSI) during the prediction window.  The collected data should also include configuration information relevant to the model input, such as pre-processing type (if applicable), raw channel matrix or eigenvector, as well as information that impacts the size of the model input, such as the number of Tx antenna ports, the BWP size, the sub-band configuration. It is assumed that the CSI-RS configuration used for generating the collected data should also be reflected in the dataset.

Proposal 3: For model input and ground truth for CSI prediction model training dataset, the collected data could include the measured CSI during the observation and the prediction window.

As indicated in [8], the sizes of the observation and prediction windows are important factors in the design, training, and operation of CSI prediction models. For each combination of window lengths, different prediction models may need to be trained, which impacts the configuration of the UE-collected data for model training purposes; thus, the length of the observation and prediction windows needs to be captured in the other information associated with the collected data.

Proposal 4:  Other information for the CSI prediction model training dataset could include the sizes of the observation and prediction windows, CSI format (raw or eigenvector), pre-processing (if any), CSI-RS configuration, the number of Tx antenna ports and BWP and sub-size.

To enable cleaning/curating the collected training dataset, metrics to indicate the quality of the measured data should also be reported. These may include at least the RSRP and TDCP.

Proposal 5:  Quality indicators for the CSI prediction model training dataset could include at least the RSRP and TDCP.

### Positioning use case

Overview

For positioning sub-use case, training data for AIML based positioning consists of ground truth and inputs to an AIML model where inputs may consist of measurements made on received DL-PRS. Some examples of measurements are Channel Impulse Response (CIR), Power Delay Profile (PDP), Delay Profile(DP), timing or power measurements as identified during the study item phase [3]. For UE-based AIML based positioning with one-sided model, how the training data can be acquired needs to be discussed.

Quality of training data

In TR 38.853, the following agreement made during the study item phase is captured; PRU and/or UE can generate the ground truth label and label quality indicator can be generated by the UE which may use RAT dependent or RAT independent positioning method to estimate its location.

- The following options of entity and mechanisms to generate ground-truth label are identified: - UE with estimated/known location generates ground-truth label and corresponding label quality indicator - Based on non-NR and/or NR RAT-dependent and/or NR RAT-independent positioning methods - At least for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified: - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP

In addition, in RAN1#116b [10], the following agreement was made which identified label generation entities for Case 1 (UE-side model, UE based positioning, direct AIML positioning), namely PRU, non-PRU UE and LMF.

Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

As quality of training data determines quality of the trained AIML model, integrity of training data becomes critical. For UE-sided model, the UE needs to determine the quality of the training data based on the ground truth label quality indicator and there is a need to define a ground truth label quality indicator associated with one value of a ground truth label. For example, one PRU location can be associated with a ground truth label quality indicator. In addition, since density of deployment of PRUs is not clear, UE location information can be also used as the ground truth and corresponding ground truth label quality indicator should be specified.

Proposal 6: A ground truth label quality indicator is associated with a UE or PRU location

Delivery of training data to the UE

How training data can be delivered to the UE needs to be discussed. During Release 18, measurement and assistance data forwarding functionality by the LMF for NRCP (NR Carrier Phase) positioning was supported [4]. Using the supported forwarding functionality, a UE can request for measurements made by PRU (Positioning Reference Unit) to the network. As a response for the request made by the target UE, the LMF can send timing, power and phase measurements (e.g., NRCP, RSTD, RSRP) made by the PRU to the target UE who made the request.

The measurement forward functionality can be used to deliver training data to a UE. As the location of the PRU is known by the LMF, the measurements and location information, i.e., the ground truth, can be  reliable and used as training data. An additional benefit of the functionality, in the context of training data for AIML positioning, is that the LMF can check the content of the data, i.e., training data.

As the current specification supports forwarding only measurements and PRU location, the content of forwarded information needs to be improved for the purpose of AIML positioning. For example, the hard or soft ground truth label quality indicator needs to be associated with the ground truth, i.e., PRU or UE location information.

In addition, the number of deployed PRUs may be limited, or PRUs may be stationary. Thus location information and associated measurements about PRUs may not be enough to create diverse training data. The UE should therefore obtain location information of non-PRUs from the LMF. Obtaining location information and associated measurements of UE, which is not a PRU, can increase the volume of the training data at the UE. Thus the following proposals are made.

Proposal 7: For case 1 for positioning, support LMF to forward location information of PRUs, measurements made by PRUs and ground truth label quality indicator with the PRU location to a target UE

Proposal 8: For case 1 for positioning, support LMF to forward location information of a UE, which is not a PRU, measurements made by the UE and ground truth label quality indicator associated with the UE location to a target UE

Obtaining ground truth quality label

In TR 38.843, the following options are identified as entities which can generate a ground truth label quality indicator:

- UE with estimated/known location generates ground-truth label and corresponding label quality indicator

- Network entity generates ground-truth label and corresponding label quality indicator

In positioning, uncertainty for UE location estimate is specified [4]. However uncertainty is determined by the UE if UE based positioning is implemented. Thus, in NLOS heavy environment assumed for the positioning use case, the UE could be locked on an inaccurate position estimate as illustrated in Figure 1.

Figure 1 An example of uncertainty related to UE location estimate

If the UE determines the ground truth label quality indicator based on the uncertainty derived at the UE, the generated ground truth label quality indicator may be unreliable as the estimate UE location may be inaccurate.

Observation 3: A ground truth label quality indicator generated by a UE or PRU may be unreliable as the estimate UE location may be inaccurate

To solve this issue, we propose that the network is the only entity that generates ground truth label quality indicators. It is assumed throughout NR positioning work that the LMF can collect a large amount of data from UEs and generate assistance information (e.g., LOS status between TRP and UE, or along PRS). Thus, the LMF should be able to generate a ground truth label quality indicator by comparing reported measurements and UE estimate against its database of collected measurements and UE location estimates.

Proposal 9: The LMF is the only entity that can generate a ground truth label quality indicator associated with location information of UE or PRU

### Beam management use case

For beam management use case, different approaches need to be considered based on position of AI/ML model and use cases. For a UE-side AI/ML model, additional specification impact of UE reporting is not needed as UE can measure beams by itself and train an AI/ML model by using the measured values. Instead, support of a procedure to measure RS transmissions for whole Set A over multiple time instances is needed to acquire the necessary measurements. On the other hand, for gNB-side AI/ML model, enhancement of UE reporting is needed as gNB needs to acquire UE side measurements for whole Set A beams over multiple time instances. For example, in BM-Case 1, current beam reporting with up to 4 CRIs/SSBRIs and corresponding L1-RSRP values is not enough for AI/ML model training, so that beam reporting up to 64 RSRPs should be supported considering maximum number of beams supported in the current specification. Compared to RSRPs, additional reporting of CRIs/SSBRIs are not needed as the UE need to report all measured RSRPs from configured RSs (but needs an association between reported RSRPs and measured RSs). For BM-Case 2, time domain characteristics of beam measurements are essential for time domain prediction. Especially, such time domain information will be crucial for certain predictable scenarios such as highway or HST. Information about measured past instances (e.g., time stamp) for reported beam related information should be considered.

Observation 4: For UE side model, additional specification impact for UE reporting is not needed, but a procedure to measure whole Set A over multiple time instances is needed.

Observation 5: For gNB side model, enhancement of UE reporting is needed as gNB needs to acquire UE side measurements.

Proposal 10: For UE side model, support a common procedure to measure whole Set A over multiple time instances for both BM-Case 1 and BM-Case 2.

Proposal 11: For gNB side model, support enhanced UE reporting to report up to 64 RSRP values for whole Set A over multiple time instances.

- No CRIs/SSBRIs are reported and implicit beam indexes (e.g., by association with RSs and reported RSRPs) are used.

- Information on measured past instances (e.g., time stamp) is supported.

Compared to data collection for inference, data collection for training requires huge overhead for BM-Case 1 and BM-Case 2. In case of BM-Case 1, for data collection for inference, only limited sets of measured beam quality (i.e., RSRPs from Set B (e.g., 16 beams)) need to be reported. On the other hand, for data collection for training, measured beam quality from whole set of supported beams in Set A (e.g., 64 beams) need to be reported. The overhead difference of data collection between training and inference is becoming larger if we consider BM-Case 2 in temporal domain. For BM-Case 2, while UE needs to report measured information only for selected time instances for inference, UE needs to report measured information for both potential inference instances as well as potential prediction instances. In that regard, a proper mechanism to compress data for training is essential for both BM-Case 1 and BM-Case 2.

Figure 2 shows examples of measured RSRPs within one UE. While Figure 2(a) shows measured RSRPs with different beams in spatial domain, Figure 2(b) shows measured RSRPs within a same beam in temporal domain. As shown in Figure 2(a), measured RSRP values of neighbouring beams in spatial domain are correlated for one UE at a given time and do not make a sudden change. As well as spatial domain, as shown in Figure 2(b), measured RSRP values within one beam in temporal domain are correlated over time and do not make a sudden change. Based on the observations, it can be identified that UE beam reporting compression (e.g., differential RSRP) by using neighbouring beams in spatial domain and RSRP values within a same beam in temporal domain can be beneficial.

(a) RSRPs across different beams			(b) RSRPs across different measurement instances

Figure 2 An example of measured RSRPs within one UE

Observation 6: Compared to data collection for inference, data collection for training requires huge overhead for both BM-Case 1 and BM-Case 2.

Observation 7: According to the evaluation results, measured RSRPs within one UE do not significantly change over different beams in spatial domain and different time instances within one beam.

Proposal 12: Support beam reporting compression mechanism for training to reduce overhead by using RSRPs in neighboring beams in spatial domain and RSRPs within a same beam in temporal domain.

## Considerations on model transfer/delivery

It is possible that a single model may not generalize well for all the use cases/deployment scenarios or lead to undesirable tradeoff between model size and performance. One option is then to train multiple smaller models for specific scenario/configuration/site. But in this case, the UE may have limitations in terms of storage space. It can be argued that the UE can finetune/update the model to match the operating condition. But it is not clear if the latency, processing capability and overhead associated with model finetuning on the UE is desirable. Another option is to use model delivery/transfer to relax the UE requirements. However, signaling overhead, specification impact, and/or latency associated with model delivery/transfer should be carefully considered.

Observation 8: In cases where model generalization, model finetuning or model storage/switching is not feasible, model delivery/transfer can be beneficial.

While model transfer/delivery may be beneficial in the examples discussed above, for UE-side models with Functionality based LCM, the 3GPP specification transparent model delivery seems to be enough to address the issues mentioned above. However, for two-sided models, due to model pairing when a specific model training type is used, model transfer can further be studied.

Proposal 13: Model transfer for UE-side models with functionality-based LCM is not supported and 3GPP specification transparent model delivery is only considered.

# Conclusion

In this contribution, views on various LCM aspects are discussed, and the following observations and proposals are made.

Observation 1: For MI-Option2, transferring only a dataset from the NW/NW-side to UE/UE-side via standardized signaling is not sufficient to ensure that the UE-part of the two-sided model (e.g., UE-side CSI generation) trained at the UE-side is compatible with the NW-part of the two-sided model (e.g., NW-side CSI reconstruction).

Observation 2:  MI-Option2 may lead to considerable inter-vendor collaboration complexity to align the UE-side and NW-side models.

Observation 3: A ground truth label quality indicator generated by a UE or PRU may be unreliable as the estimate UE location may be inaccurate

Observation 4: For UE side model, additional specification impact for UE reporting is not needed, but a procedure to measure whole Set A over multiple time instances is needed.

Observation 5: For gNB side model, enhancement of UE reporting is needed as gNB needs to acquire UE side measurements.

Observation 6: Compared to data collection for inference, data collection for training requires huge overhead for both BM-Case 1 and BM-Case 2.

Observation 7: According to the evaluation results, measured RSRPs within one UE do not significantly change over different beams in spatial domain and different time instances within one beam.

Observation 8: In cases where model generalization, model finetuning or model storage/switching is not feasible, model delivery/transfer can be beneficial.

Proposal 1: Defer discussions on MI-Option4 until more progress is made in RAN4 (on RAN4-Option3, or RAN4-Option4).

Proposal 2: Clarify if MI-Option2 refers to Option 4 in the CSI compression sub-agenda item (“Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side”).

Proposal 3: For model input and ground truth for CSI prediction model training dataset, the collected data could include the measured CSI during the observation and the prediction window.

Proposal 4:  Other information for the CSI prediction model training dataset could include the sizes of the observation and prediction windows, CSI format (raw or eigenvector), pre-processing (if any), CSI-RS configuration, the number of Tx antenna ports and BWP and sub-size.

Proposal 5:  Quality indicators for the CSI prediction model training dataset could include at least the RSRP and TDCP.

Proposal 6: A ground truth label quality indicator is associated with a UE or PRU location

Proposal 7: For case 1 for positioning, support LMF to forward location information of PRUs, measurements made by PRUs and ground truth label quality indicator with the PRU location to a target UE

Proposal 8: For case 1 for positioning, support LMF to forward location information of a UE, which is not a PRU, measurements made by the UE and ground truth label quality indicator associated with the UE location to a target UE

Proposal 9: The LMF is the only entity that can generate a ground truth label quality indicator associated with location information of UE or PRU

Proposal 10: For UE side model, support a common procedure to measure whole Set A over multiple time instances for both BM-Case 1 and BM-Case 2.

Proposal 11: For gNB side model, support enhanced UE reporting to report up to 64 RSRP values for whole Set A over multiple time instances.

- No CRIs/SSBRIs are reported and implicit beam indexes (e.g., by association with RSs and reported RSRPs) are used.

- Information on measured past instances (e.g., time stamp) is supported.

Proposal 12: Support beam reporting compression mechanism for training to reduce overhead by using RSRPs in neighboring beams in spatial domain and RSRPs within a same beam in temporal domain.

Proposal 13: Model transfer for UE-side models with functionality-based LCM is not supported and 3GPP specification transparent model delivery is only considered.

# Reference

RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

3GPP TR38.843 V18.0.0 (2023-12) Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

RAN1 #116 Chairman’s note

3GPP TS38.805 V18.0.0 (2023-12) Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 18)

3GPP TS37.355 V18.0.0 (2023-12) LTE Positioning Protocol (LPP) (Release 18)

R1-2401042, “Discussion on support for AIML positioning”, InterDigital, RAN1#116, Athens, Greece, Feb 2024.

R1-2400908, “Discussion on AI/ML-based CSI prediction”, InterDigital, RAN1 #116, Athens, Greece, Feb. 2024.

R1-2402609, “Discussion on AIML for beam management”, InterDigital, RAN1#116bis, Changsha, China, April 2024.

RAN1 #116b Chairman’s note

RAN1 #117 Chairman’s note

R2-2406381, “Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2”

RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

RAN1 #118 Chairman’s note