3GPP TSG RAN WG1 #118b                                                                   R1- 2408463

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.3.3

Source:	Apple Inc.

Title:	Discussion on CLI handling

Document for:	Discussion/Decision

# Introduction

In RAN#102, a new work item was endorsed in RP- 234035, later revised in RP-241614 to specify Evolution of NR duplex operation: Sub-band full duplex (SBFD) in Rel-19, where one of the objectives is related to CLI handling as follows:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD.

In this contribution, we share our views on CLI handling aspects.

# UE-to-UE CLI management

## L1/L2 based UE-to-UE CLI measurement and reporting

In Rel-16, UE-to-UE cross link interference (CLI) framework is specified based on periodic CLI measurement and a L3 filtering/reporting. L3 filtering has by nature some latency on the CLI report availability to the gNB. As a result, a quick CLI report which better reflects current CLI profile is not achievable by R16 procedure. L1/L2 CLI reporting is potentially faster than L3 filtering/reporting and may be more useful when a quick scheduling coordination between aggressor and victim UEs is needed. As a result, Rel-19 aims to specify L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, where aperiodic reporting shall be supported.

Existing CSI framework already supports aperiodic report on PUSCH where the report is of course triggered by an UL DCI. Alternatively, or additionally aperiodic CLI report can be over PUCCH where the report is triggered by a DL DCI. Here we discuss about the motivation and the specification effort over the existing CSI framework.

- First on justification, we should note that aperiodic CLI report is sent by a victim UE, and a victim UE is a UE in downlink reception. When NW is suspicious of a UE in DL being a victim UE, aperiodic CLI report can be triggered on DL DCI at the time UE is being scheduled. It is not well justified to use an UL DCI for AP-CLI, when there is already and certainly a DL DCI for the victim UE. Now one may think that similar argument could be made for AP-CSI, where AP-CSI is eventually used for downlink, but AP-CSI on PUCCH was not specified in NR, so why should AP-CLI on PUCCH is specified. We should note that AP-CSI is not as critical as AP-CLI, given that if CLI exists it potentially blocks communication toward victim UE. In addition, the purpose of L1-based CLI report is to quickly turn around if a nearby aggressor UE is making CLI to victim UE, which is a UE being scheduled in downlink anyway.

- On the specification effort, the change to existing framework is marginal. Indeed, in terms of CLI resource setting there is no difference between A-CLI on PUCCH vs A-CLI on PUSCH. For CLI report setting, the procedure follows CSI framework but instead of CLI report setting on any UL CC for A-CLI on PUSCH, A-CLI report on PUCCH is simplified when CLI report setting is only configured for the CC with pucch-Config. Of course, the trigger bit-field, i.e. CSI request, is added to DL DCI. The specification change to existing CSI framework for A-CLI on PUCCH is shown by red in Figure 1.

Figure 1: Aperiodic measurement and report triggered by UL DCI (left) vs DL DCI (right)

Based on the above discussion, we have the following proposal:

Proposal 1: AP-CLI report can be triggered through UL DCI or DL DCI

For CLI reportQuantity, the following agreement is achieved in RAN1#118.

Agreement

For L1 UE-to-UE CLI measurement and reporting, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity.

- FFS: configuration of number of reported CLI resources in the reportConfig

- FFS: reporting criteria

For CLI report setting and CSI resource setting, we can simply reuse existing CSI report and resource setting, where resources for CLI measurement are added to existing CSI-ResourceConfig. Example of RRC signaling flow is shown in Figure 2, where the new procedure for CLI measurement and reporting within CSI framework is shown in red.

Figure 2: CLI resource configuration within CSI-ReportConfig

Based on the above discussion, the following is proposed:

Proposal 2: Introduce cli-ResourceForCrossLInkInterference in CSI-ReportConfig, as shown in Figure 2, where the corresponding CSI-ResourceConfigId is associated with CLI-RS-ResourceList.

As reflected in WID, Rel-16 SRS-RSRP resource or CLI-RSSI resource are baseline for CLI measurement resources. Obviously, some enhancement over Rel-16 is needed, for example to add smei-persistent and aperiodic measurement resources but there is no strong justification to introduce new measurement resource format just for CLI.

Proposal 3: CLI-RS-ResourceList is made of one or more SRS-ResourceConfigCLI-r16 and/or one or more RSSI-ResourceConfigCLI-r16 resources, as shown in Figure 2.

For L1 based CLI report indication, a CLI report occasion may cover more than one CLI measurement occasions.

Proposal 4: If UE is aperiodically indicated to report CLI, each CLI report occasion may cover O CLI measurement occasions, where O>=1 and is subject to UE capability.

## Indication to victim UE on resources impacted by CLI

Although it is expected that scheduler maintains enough separability between victim and aggressor UEs (e.g., in frequency and/or spatial domain), there could be scenarios where UE-to-UE CLI is still inevitable. For example, imagine the case that UL transmission on aggressor UEA carries URLLC and thus cannot be cancelled. In this scenario, gNB should be able to indicate to the victim UEV, which is receiving in DL, about the probable existence of CLI from aggressor UE(s) on resources that overlap in time between UEV and UEVA ‘s allocations. Although this problem is not exactly same as DL preemption indication, which was specified in Rel-15 for URLLC, but there are strong similarities between the two cases. In both cases, we have a UE in DL which its data reception has been impacted by another UE (a URLLC UE in DL in Rel-15 and a URLLC UE in UL in future releases). Given that no additional signaling to legacy UE is promoted, we propose the interrupted transmission indication as specified in Rel-15 to be repurposed to indicate resources impacted by CLI. Of course, the reason of preemption indication is transparent to victim UE, i.e. it does not matter to UEV whether there exists a URLLC UE in UL or DL transmission.

Proposal 5: DL CLI indication, e.g., based on DL-PI, indicates which symbols were impacted by cross-link interference from aggressor UE(s).

## Timing advance techniques

An aspect related to coordinated CLI measurement is about potential misalignment of CLI from aggressor UEA and desired signal from gNB received at the victim UEV. Such misalignment at the victim UEV’s reception window, can result inter-carrier interference (ICI), and/or inaccurate CLI measurement. One solution to have both desired and interfering signals to be received at the victim UE within CP, is the aggressor UEA to keep to TAs, one for legacy non-SBFD symbols where UL transmissions from different UEs is supposed to be received at gNB’s CP, and another TA in SBFD symbols, where UL transmissions from aggressor UEA targets to be received at victim UEV’s CP, as shown in Fig. 3.

Proposal 6: To assure symbol level alignment at victim UEV, aggressor UEA is indicated to hold two different TAs:

- one TA for symbols on which TRP is doing legacy TDD, another TA for symbols on which TRP is doing SBFD or dynamic TDD.

Figure 3: Different TAs for UL transmissions in SBFD and non-SBFD ssymbols

## UL resource muting

For the time location configuration of non-transparent UL resource muting for a PUSCH, it was agreed in RAN1#118 that:

Agreement

For the time location configuration of UL resource muting for a PUSCH, one of the following options is selected

- Option 1: Semi-statically configure the position for each of the up to two UL muting symbols within a slot

- Option 2: Semi-statically configure X (X>=1) possible positions for each of the up to two UL muting symbols within a slot

Note: Above has no implication on how UL muting symbol indication/determination is done.

Note: For both options, there will at most up to 2 UL muting symbols for the PUSCH.

From specification effort, and ease of UE implementation, Option 1 is preferred given that Option 2 needs further indication to UE, which of the configured muting patterns should be applied. In our view, although Option 2 gives a bit flexibility to the scheduler, but the essence of such flexibility is not well justified. Thus, we have the following proposals:

Proposal 7: Each muting resource is defined e.g. as a bitmap of size 14 bits, where up to two bits set to 1 representing symbol location(s) of muting resource(s)

In addition to time domain indication of non-transparent UL muting resource, it is essential to identify which REs within in comb2 pattern over the indicated symbol(s) will be muted. In other words, a reference for the first muting RE is needed.

Proposal 8: For subcarrier spacing corresponding to an UL BWP, frequency location of the first muting RE in the corresponding muting resource is aligned with the first subcarrier of the starting RB of this UL BWP.

# Conclusion

In this contribution, we shared our views on CLI handling. Based on what we discussed, the following proposals are made:

Proposal 1: AP-CLI report can be triggered through UL DCI or DL DCI

Proposal 2: Introduce cli-ResourceForCrossLInkInterference in CSI-ReportConfig, as shown in Figure 2, where the corresponding CSI-ResourceConfigId is associated with CLI-RS-ResourceList.

Proposal 3: CLI-RS-ResourceList is made of one or more SRS-ResourceConfigCLI-r16 and/or one or more RSSI-ResourceConfigCLI-r16 resources, as shown in Figure 2.

Proposal 4: If UE is aperiodically indicated to report CLI, each CLI report occasion may cover O CLI measurement occasions, where O>=1 and is subject to UE capability.

Proposal 5: DL CLI indication, e.g., based on DL-PI, indicates which symbols were impacted by cross-link interference from aggressor UE(s).

Proposal 6: To assure symbol level alignment at victim UEV, aggressor UEA is indicated to hold two different TAs:

- one TA for symbols on which TRP is doing legacy TDD, another TA for symbols on which TRP is doing SBFD or dynamic TDD.

Proposal 7: Each muting resource is defined e.g. as a bitmap of size 14 bits, where up to two bits set to 1 representing symbol location(s) of muting resource(s)

Proposal 8: For subcarrier spacing corresponding to an UL BWP, frequency location of the first muting RE in the corresponding muting resource is aligned with the first subcarrier of the starting RB of this UL BWP.