3GPP TSG RAN WG1 #118bis	R1-2408518

# Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.212 | CR | xxxx | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on rate matching in TS 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to current specification TS 38.214, numRefSymbolLength would be used for TBS determination if 2 candidate starting symbols are configured regardless the value of sl-PSFCH-Period. But there may be a case that both 2-candidate-starting-symbol(BWP level configuration) and sl-PSFCH-Period=1 ( resource pool level configuration) are configured for a SL resource pool. It would lead to use numRefSymbolLength for rate matching in a resource pool with sl-PSFCH-Period =1. On the other hand, according to the agreements of RAN1#111 meeting, legacy sl-LengthSymbols instead of numRefSymbolLength should be used for a resource pool with sl-PSFCH-Period =1, since such resource pool only includes slots with PSFCH symbols, and numRefSymbolLength is only applied to a resource pool which includes slots with 2 candidate starting symbols. Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference symbol length, down-select one of the followings: - Option 1: The reference symbol length is dynamically indicated by Tx UE - Option 2: The reference symbol length is determined based on 1st starting symbol - Option 3: The reference symbol length is determined based on 2nd starting symbol - Option 4: The reference symbol length is (pre-)configured Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.. |
| Summary of change: |  | Updating the rate matching text to align with RAN1 agreements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between 38.212 specification and RAN1 agreements on rate matching. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.4.4	Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer if present, denoted as QSCI2', is determined as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R, αl=0NsymbolPSSCH−1MscSCI2(l)+γ

where

<Unchanged parts are omitted>

-	MscSCI2(l) is the number of resource elements that can be used for transmission of the 2nd-stage SCI in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH−1 and for NsymbolPSSCH=Nsymbsℎ−NsymbPSFCH−NsymbSL PRS, in PSSCH transmission, where NsymbsℎNsymbslot = sl-lengthSymbols - 2, where sl-lengthSymbols is the number of sidelink symbols within the slot provided by higher layers as defined in [6, TS 38.214]. NsymbSL PRS is the number of symbols for SL PRS provided by the higher layer parameter numSym-SL-PRS-2ndStageSCI if the 2nd-stage SCI is SCI format 2-D, and NsymbSL PRS = 0 otherwise. If sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided for the SL-BWP and if higher layer parameter sl-PSFCH-Period = 0, 2 or 4, NsymbsℎNsymbslot = sl-NumRefSymbolLength - 2, where sl-NumRefSymbolLength is provided by higher layers. If higher layer parameter sl-PSFCH-Period = 2 or 4, NsymbPSFCH = 3 if "PSFCH overhead indication" field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise. If higher layer parameter sl-PSFCH-Period = 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-	MscSCI2(l) = MscPSSCH(l) - MscPSCCH(l)

<Unchanged parts are omitted>