3GPP TSG-RAN WG1 Meeting #118-bis	R1-2408525

Hefei, P.R. China, October 14th – 18th 2024

Agenda Item:	9.3.1

Source:	Ericsson

Title:	SBFD Tx/Rx/measurement procedures

Document for:	Discussion, Decision

# Introduction

As a part of the WI on Duplex Evolution, RAN1 are commissioned to specify time and frequency location of SBFD subbands as well as procedures for transmission, reception, and measurement for SBFD-aware UEs [1]:

- Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

Below, we present our detailed views on SBFD for these topics.

# Discussion

## SBFD time and frequency configuration

### Frequency domain configuration

In the latest meeting, RAN1 made a working assumption about a RIV-based frequency domain subband configuration [2]:

Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. One or two SBFD DL subbands can be configured

While we acknowledge that a few bits may be saved by omitting indication of the far ends of the outermost subbands, there may also be advantages by explicitly also indicating these to the UE. In particular, if SBFD would need to co-exist, e.g., with SFFD, having a well-defined subband configurations would be an advantage. Consequently, we support the WA.

Endorse WA on frequency domain configuration of SBFD subbands.

In addition, RAN1 made the following agreement in RAN1 #117 regarding frequency domain location of SBFD subbands [3]:

Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

RAN1 has already agreed on using cell-specific and only cell-specific configuration in the time domain. In order to avoid coinciding DL transmissions (in time and frequency) from neighboring gNBs while a UE is transmitting in the UL subband, the SBFD configuration must not only be cell specific, in fact it must be aligned over an area far greater than a cell, see Figure 1. That is, gNB A cannot configure a part of the spectrum as an DL subband of an SBFD symbol while gNB B has configured the same part of the spectrum as an UL subband.  In that case, gNB A will be a direct aggressor in the UL subband, interfering with received signal in gNB B and thereby victimizing it. Hence, the only frequency domain configuration that is practically feasible is a cell-specific frequency configuration of SBFD subbands.

Figure 1. Illustration of interference between UE-specific and cell-specific SBFD configurations.

The SBFD frequency configuration needs to be area-specific for interference reasons.

Another topic that has been discussed, so far without any agreement, is UE-specific guard bands. In our view, there are several reasons why UE-specific guard bands should not be specified.

The main reason to introduce UE-specific guard bands is if there is some additional (compared to legacy UEs) UE filtering of DL/UL subbands requiring guard bands to handle the filter roll-off. However, the WID does not include any RAN4 objectives on new UE RF requirements, thus it can’t even be assumed that such additional filtering is done by the UE. Even if a SBFD-aware UE performs such filtering, what kind of UL subband suppression that can be expected is not known to the gNB. Thus, from an RF requirement point of view, a SBFD-aware UE would anyway need to be treated in the same way as a legacy UE.

Because the WID does not include new UE RF requirements for SBFD-aware UEs, they cannot be expected to have improved spectral filtering capabilities and thus there is no benefit with UE-specific guard bands.

If a UE anyway performs additional RF filtering, such filtering would presumably also be performed during the random-access procedure from RRC_IDLE mode, that was agreed to be supported in the latest revision of the WID. Hence, a UE would still need to support a cell-specific guard band configuration as provided in the cell-specific configuration. In our understanding, that could mean both analog and digital bandpass filtering. To introduce yet another set of filters for UE-specific filtering when the cell-specific filtering anyway needs to be supported is not justified, in our opinion, but will only lead to increased complexity and cost.

Since random access in RRC_IDLE is supported, a UE performing subband filtering needs to support the cell-specific guard band configuration.

Another reason for not specifying UE-specific guard bands is that SBFD already partitions the carrier into two or three subbands, substantially restricting SSB and CORESET 0 placement to within a single DL subband. Hence, any further bandwidth reduction from UE-specific guard bands will limit the configurability even more and thereby reducing the usefulness of the feature.

Limitations in SSB placement and associated CORESET 0 placement cause any further narrowing of a DL subband to be infeasible.

A fourth reason is that UE-to-UE CLI may oftentimes be solved by scheduling, such that highly interfering UEs are simply not co-scheduled but time multiplexed. In that case, there is no need for UE-specific guard bands. However, we acknowledge that may not always be the case, either because the network is unable to keep track of a large number of UEs, or that time multiplexing may otherwise not be feasible.

UE-to-UE CLI can mostly be avoided by scheduling.

Yet another reason that has been put forward for introducing UE-specific SBFD frequency configurations is to handle UEs that only support one DL subband. In our view, such restrictions should not be allowed, given that RAN1 has agreed to support placing the UL subband in the middle of the carrier. That said, in case RAN1 agrees to such restrictions, those UEs can be handled by configuring a BWP that only covers the UL subband and one DL subband.

Based on the discussion above we make the following proposal:

No UE-specific frequency configuration of SBFD symbols is supported.

### Time domain configuration

The TDD pattern configuration from tdd-UL-DL-ConfigurationCommon is specified in TS 38.213, §11.1:

If a UE is provided tdd-UL-DL-ConfigurationCommon, the UE sets the slot format per slot over a number of slots as indicated by tdd-UL-DL-ConfigurationCommon. The tdd-UL-DL-ConfigurationCommon provides - a reference SCS configuration by referenceSubcarrierSpacing - a pattern1. The pattern1 provides - a slot configuration period of msec by dl-UL-TransmissionPeriodicity - a number of slots with only downlink symbols by nrofDownlinkSlots - a number of downlink symbols by nrofDownlinkSymbols - a number of slots with only uplink symbols by nrofUplinkSlots - a number of uplink symbols by nrofUplinkSymbols … A slot configuration period of msec includes slots with SCS configuration . From the slots, a first slots include only downlink symbols and a last slots include only uplink symbols. The symbols after the first slots are downlink symbols. The symbols before the last slots are uplink symbols. The remaining are flexible symbols.

From here, it is clear that the cell common configuration of flexible symbols appear in a contiguous segment between DL and UL symbols.

Additionally, RAN4 has made the following agreement about transmitter transient periods :

- Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. #### Issue 2-1-2: Transient period length between SBFD and non-SBFD - Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1.

In brief, the RAN4 agreement states that a transmitter transient period of 10 µs for FR1 and 3 µs for FR2 is required to/from SBFD subbands. These values correspond to less than a symbol for both FR1 and FR2, regardless of numerology, but also exceeds the cyclic prefix implying that a symbol needs to be allocated for the transient.

The transients will affect both legacy UEs and SBFD-ware UEs. Hence, specifying a guard period that requires signaling is not a feasible solution since it will exclude legacy UEs from receiving and thereby accounting for it. Furthermore, there is no real need to inform UEs about it, since, for all numerologies, it is constantly one symbol in duration. Instead, the transmitter transient can be handled by gNB by implementation such that the last DL symbol before the SBFD symbols is avoided by scheduling and thereby anyway used as a guard period.

The transmitter transient period, agreed by RAN4, will affect both legacy and SBFD-aware UEs and for that reason cannot easily be solved by signaling. Instead, a guard symbol can be implemented by the gNB in the DL symbol preceding the SBFD subband without specification impact.

In addition to the transmitter transient period, a longer guard period can be introduced between DL and UL operation to avoid DL-to-UL or UL-to-DL interference due to timing advance, propagation delays and gNB synchronization inaccuracies. This timing misalignment may cause DL and UL slots to be mixed at both a receiving UE and a receiving gNB, see Figure 2. Without the guard period, the DL transmission from a gNB to UE A may be interfered by an UL transmission from UE B due to the timing advance of UE B. Alternatively, the UL transmission from UE A towards gNB A may be interfered by a DL transmission from gNB B towards UE B, due to a difference in propagation delay.

Figure 2: Interference scenarios for, left, UE-to-UE interference and, right, gNB-to-gNB interference.

The legacy NR guard period is implemented as flexible symbols causing a gap between DL and UL symbols, see Figure 3. If properly selected, the guard period will cause a sufficiently long gap between DL and UL such that any effects arising from timing advance, or propagation delays are either avoided or have sufficiently worn off. In SBFD, interference may not only arise between DL and UL symbols but also between DL symbols and UL subbands. However, such a guard period is only needed for the fraction of the spectrum that constitutes the UL subband for the above interference cases.

Figure 3: Interference at receiver without and with guard period for (left) UE A and (right) gNB A from Figure 2.

While configuring flexible symbols may strictly not be necessary, since the guard period can be solved by gNB implementation, there are some strong reasons to still do so. First, flexible symbols may be included in mandated, country specific TDD pattern configurations. Second, flexible symbols are a way for an operator to configure different durations of guard periods for different deployment scenarios with different interference distances or to align gNBs from different network vendors that may otherwise use different, incompatible strategies.

Figure 4 presents to the left a legacy flexible symbol configuration, effectively establishing a guard period in-between DL and UL. In the center figure, an SBFD configuration with an additional SBFD guard period spanning the whole carrier has been introduced in DL symbols (dotted lines), and the right figure shows an additional SBFD guard subband spanning only the UL subband. A corresponding interference effect as the DL-to-UL symbol transition in legacy NR is found between the DL-to-SBFD symbol transition in the UL subband. A difference, though, is that for legacy, the whole carrier is affected, whereas for SBFD, only the UL subband will suffer from the transient interference. In fact, for DL subbands, it is desirable to not introduce any additional guard band to what is already introduced by RAN4 per the above since that would only decrease DL resources without any benefit to it. Doing so at the start of a slot would be particularly ill-timed since that is typically when CORSETs are configured. Furthermore, legacy UEs scheduled in the DL would not be aware of such a guard band and would not be able to adapt to it.

Figure 4: Example of, left, legacy configuration with a single period of flexible symbols providing a DL-to-UL gap, center, an SBFD configuration with an SBFD-specific and gNB managed guard period for transmission transients in addition to the legacy configuration, and right, in addition to the center configuration, a guard band, separating the DL symbols from the UL subband.

A complication to the above is that the present specification only supports the configuration of a single contiguous block of flexible symbols. Although NR allows for the configuration of a large contiguous block of flexible symbols, from which two sets of guard periods could be derived, the guard subband preceding the UL subband would not be possible to configure. Furthermore, such a configuration would require significant network reconfigurations that is highly risky and therefore undesirable, e.g., legacy ROs for random access would likely cause significant problems.

Whereas the guard period arising from transmission transients only affects the serving cell and for that reason may be solved by network implementation, the guard subband ahead of the UL subband may be a bit more complicated since it may also affect neighbor cells. For some scheduling situations it may be beneficial or even necessary if the UE to knows about the guard subband, e.g., configured UL grants. Hence, the specification should allow for configuring a guard subband ahead of the UL subband in SBFD.

The above discussion brings us to the following observation and proposal:

For interference reasons, it would be beneficial to separate DL and UL spectrum in the intersection between DL and SBFD symbols, i.e., to introduce a guard band in the first symbol(s) of the UL subband.

Introduce a guard subband in the first symbol(s) of the UL subband. FFS: guard band duration(s).

## Collision handling

### Link direction determination

Regarding link direction determination, RAN1 made the following agreements in RAN1 #116 and RAN1 #116-bis, respectively [6]:

Agreement (RAN1 #116) For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded. Agreement (RAN1 #116-bis) A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

With respect to the first agreement above, we note that Option 1 is already supported by current agreements and does not require further specification effort. Moreover, Option 2 is associated with challenges. First, it would involve unnecessary dynamic signaling overhead for doubtful gains. Second, introducing further restrictions by help of TDD-UL-DL-ConfigDedicated and DCI format 2_0 would result in restrictions in the scheduling of the UE and is not desirable.

Option 1 for link direction is already supported by current agreements.

UE direction is resolved by configured/scheduled operations, i.e., support Option 1.

With respect to the second agreement above, a natural consequence of agreeing to Option 1 (in the first agreement above) would be that for a SBFD capable UE, tdd-UL-DL-ConfigurationDedicated (TS 38.213, §11.1) is only valid for flexible symbols that are not configured as SBFD symbols. That is, the ambiguity arising from the flexible configuration is resolved by the SBFD configuration in SBFD symbols whereas it can still be resolved by tdd-UL-DL-ConfigurationDedicated for non-SBFD symbols.

An SBFD configured UE is not expected to be provided with a slot configuration for SBFD symbols using tdd-UL-DL-ConfigurationDedicated. Alternatively, the UE ignores such configuration, if provided.

For the same reason as for the dedicated semi-static TDD pattern configuration, there is no need to have any ambiguity in SBFD symbols resolved by SFI in DCI format 2_0, and the UE can consequently disregard it for SBFD symbols. However, the DCI format 2_0 still applies to flexible, non-SBFD symbols. Different compared to the dedicated semi-static TDD pattern configuration, DCI format 2_0 is group common and thus it can be sent to non-SBFD-aware UEs. Thus, we think it is better that an SBFD-aware UE ignores such signaling when not applicable instead of not expecting it.

An SBFD-configured UE disregards SFI in DCI format 2_0 for SBFD symbols.

In RAN1 #116-bis, some companies proposed to introduce UE-specific restrictions for the UL subband, specifically for power saving reasons. The rationale would be that distant UEs would have a greater need for UL resources compared to more nearby UEs and would therefore benefit from more UL resources whereas the more nearby UEs could benefit from power saving by a priori knowing they would not be scheduled in the UL in certain SBFD slots. However, such behavior would differ from the typical legacy UE behavior. In particular, in FDD there are no such restrictions in UL scheduling and presumably the problem is more pronounced there. Furthermore, it would restrict the use of SBFD UL resources, complicate scheduling and, as a result, limit network capacity.

By introducing further UE-specific restrictions in UL subband utilization, the gNB would be constrained in its resource utilization, thereby limiting network utilization.

Neither TDD nor FDD have any UL scheduling restrictions. SBFD should not introduce such restrictions unless it is already supported in the spec.

### Case 3: semi-static DL vs. semi-static UL

In RAN1 #118, collision handling rules for repetitions in Case 1, 2 and 4 were agreed by applying the same rules and timeline to repetitions as for DL receptions and UL transmissions without repetitions. The same agreement should be made for Case 3. The argument for Case 3 was that PDCCH risk being blocked putting the UE in a situation where it cannot receive control information. While we acknowledge that this is a severe state for the UE, in particular if no pure DL slots are configured by the network, it is entirely avoidable by proper network configuration. Hence, the same collision handling rules should be agreed for repetitions in Case 3 as have already been agreed for Case 1, 2, and 4.

For collision Case 3 (with semi-statically configured DL Rx vs. semi-statically configured UL Tx with repetitions), the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition.

### Case 5: SSB vs. dynamically scheduled or configured UL transmission

This case was discussed in RAN1 #116 and RAN1 #117, and the following agreements were made [3], [5]:

Agreement (RAN1 #116) A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details Agreement (RAN1 #117) Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs

Apart from the FFS, we think it needs to be clarified if this agreement applies to SSBs transmitted in other cells for the purpose of RRM measurements or only SSBs transmitted in the serving cell. Prioritization between UL transmissions and SSBs for the purpose of RRM measurements are typically specified by RAN4 in TS 38.133 and referred to as scheduling availability. Thus, in our view, the RAN1 agreement above is only applicable to SSBs transmitted in the serving cell and the discussion on scheduling availability due to RRM measurements should be left to RAN4. Based on this we make the following proposal

Clarify that the agreement on SSB prioritization only covers the case of cell defining SSBs transmitted by the serving cell as indicated by ssb-PositionsInBurst and leave the discussion on scheduling availability due to RRM measurements to RAN4.

Regarding the FFS in the above agreement, interpreting SSB slots as DL slots and thus allowing DL transmission over the full bandwidth, would cause significant complications and interrupt the SBFD pattern. This, in turn, would imply that the gNB and potentially also the UE would need to reconfigure filters, and guard periods would need to be introduced in order to do so. Additionally, it would jeopardize the agreement from RAN1 #116 that “SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period” as highlighted in the agreement above. Instead, keeping SSB symbols as SBFD symbols with restriction of only using the DL subbands, is a preferable solution. The gNB can avoid scheduling UL in SSB symbols.

Configuring an SSB slot as DL jeopardizes the RAN1 agreement that “SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period.”

SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD-aware UEs in such symbols.

### Case 6: dynamic or semi-static DL vs. valid RO

The standard NR rule is to prioritize ROs regardless of if the colliding DL is dynamically scheduled or semi-statically configured as the following text in TS 38.213, §11 states:

For a set of symbols of a slot corresponding to a valid PRACH occasion and symbols before the valid PRACH occasion, as described in clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols.

For a half-duplex TDD base station this UE rule makes perfect sense because it can anyway not transmit DL while receiving PRACH. For a SBFD capable base station on the other hand, this rule would effectively mean that no UE would receive DL in symbols colliding with valid ROs and thus those DL resources will be wasted. HD-UEs have a similar issue and for those it was left to UE implementation if they receive the DL or transmit PRACH:

If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks within the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the PL RS, or the PDCCH, or the SS/PBCH blocks.

Leaving the prioritization for UE implementation has the drawback that there could be instances where the base station transmits DL, but the UE does not attempt to receive because it is simultaneously transmitting PRACH. That said, we believe those cases should be rare and for that reason mandating an RO to always be prioritized is not justified. It should also be noted that it is only left for UE implementation in the case of PRACH triggered by higher layers. For PDCCH ordered PRACH, the UE would prioritize PRACH.

Another option could be to treat this case under Case 1 (dynamic DL) and Case 3 (semi-static DL). That said, due to the rather inflexible PRACH configuration, we think it would be hard to guarantee that semi-static DL does not overlap with valid ROs. Even if Case 1 in principle could be applied, we see a risk that such behavior could affect RACH MAC procedures.

In our view, the best option is if the UE prioritizes valid ROs when it would transmit PRACH and receives DL otherwise. That is somewhat of a compromise between the legacy NR behavior (DL not received) and the HD-UE behavior (up to UE implementation). Based on the above discussion we make the following proposal:

For dynamic or semi-static DL vs valid RO, the behavior is left to UE implementation with the side condition that the UE prioritizes valid ROs when it would transmit PRACH, otherwise it receives the DL.

## Channel- and signal-specific configuration

### Physical channel/signal across SBFD and non-SBFD symbols in same slot

Proposal 2-1 [7] Proposed Agreement: For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, except for PRACH and PUSCH repetition type B, SBFD aware UE does not transmit or receive the physical channel/signal within the slot. - PRACH is separately discussed in AI 9.3.2. - For PUSCH repetition type B in case a nominal repetition is mapped to SBFD and non-SBFD symbols within a slot, FFS whether/how SBFD aware UE transmits multiple actual repetitions segmented from the nominal repetition, where each actual repetition includes only SBFD symbols or non-SBFD symbols.

In our contribution paper to the previous RAN1 meeting we provided discussion in great details on physical channel/signal across SBFD and non-SBFD symbols within a slot. By considering all the standardization and implementation complexities and negative impact on performance we suggested transmissions across SBFD symbols and non-SBFD symbols within a slot should NOT be supported, with exception of RACH and PUSCH repetition type B. Therefore, we propose to confirm the main body of the above proposed agreement captured in FL summary from the last meeting [7].

In PUSCH repetition Type B in the existing specifications, a set of PUSCH "nominal repetitions" are partitioned to a set of "actual repetitions" with splitting at slot boundaries (i.e., no cross-slot boundary coherent demodulation is supported) and gaps of invalid symbols. In Figure 2 an example is demonstrated for PUSCH repetition Type B in a SBFD carrier. The PUSCH repetition is indicated with S = 7, L = 14 and number of nominal repetitions is 2. Nominal repetition 0 is segmented into actual repetition 0 and 1 at the first slot boundary, and Nominal repetition 1 is segmented into actual repetition 2 and 3.

From the example in Figure 2, one can see that if PUSCH repetitions are further segmented around the transition between SBFD and UL symbols (i.e., actual repetition 2), then larger DMRS overhead would be required due to lack of phase coherency across the transition (see analogous discussion above in the case of DL). Moreover, such a design would impose significant complexity increase in standardization and implementation. Altogether, it is unclear whether the benefits of PUSCH repetition Type B can be achieved or not by such segmentation. For this reason, we do not support enhancements to PUSCH repetition Type B for the case that a nominal repetition span both SBFD and non-SBFD symbols.

Figure 5. PUSCH Repetition Type B across SBFD and non-SBFD symbols

If we recall Configuration 1 for PUSCH reception, i.e., the transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. If an SBFD-aware UE operating in Configuration 1, it is unlikely that it can support the transmission/reception across different symbol types within the slot as well. In light of this, it is essential to restrict the PUSCH repetition type B repetitions as well to be within the same symbol type. Specifically, in the example illustrated in Figure 5, only actual repetitions 0 and 1 are allowed or actual repetition 3 is allowed. This situation can be avoided by gNB scheduling.

If any SBFD-aware UE operating in Configuration 2, i.e., i.e., the transmissions/receptions can be in SBFD symbols and non-SBFD symbols, then it is capable to receive transmissions in different symbol types across different slots. If PUSCH repetition Type B is to be supported, after segmentation of actual repetitions is done according to the current specification, actual repetitions involving different symbol types can be dropped. In the above example in Figure 4, only actual repetitions 0, 1, and 3 are al

owed while actual repetitions 2 is invalid.

For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, SBFD aware UE does not transmit or receive the physical channel/signal within the slot.

    - RACH is separately discussed in AI 9.3.2.

    - PUSCH repetition type B is supported with a restriction that actual repetition involving different symbol types are invalidated.

### Physical channel/signal across SBFD and non-SBFD symbols in multiple slots

#### Granularity of Configurations

Agreement (cf. RAN#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability

There were different opinions on how to provide the configuration to be used (i.e., configuration 1 or configuration 2) by the SBFD-aware UE with respect to the transmission/reception across SBFD and/or non-SBFD symbols. Though the exact signalling details of providing the configuration is up to RAN2, RAN1 needs to discuss at least whether it is configured semi-statically or in a dynamic manner. It is advantageous to provide which configuration to use semi statically via RRC than in dynamic manner via DCI. Dynamic indication via DCI requires the introduction of a new field in the DCI and also the discussion related to DCI formats to be supported for this purpose depending on whether the UE is RRC_IDLE or RRC_CONNECTED mode. However, it is unclear in

terms of the benefits to change the configuration options dynamically given the signaling overhead. Depending on the use case the SBFD-aware UE operates in, the gNB can select the appropriate configuration beforehand specific to the UE and provide it semi-statically.

SBFD-aware UE operates by default in Configuration 1, if the UE is not provided which configuration to use.

SBFD-aware UE can be provided with Configuration 2 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) via dedicated RRC signalling, subject to UE capability reporting.

Also, the granularity of the configuration is to be discussed further. If the network indicates this configuration explicitly for each physical channel and signal, then this requires introducing an additional information element in the corresponding “config”, e.g., PUSCH-Config, PUCCH-Config, etc. This may allow to have one specific configuration (e.g., configuration 1) for PDSCH while another configuration (e.g., configuration 2) for PUSCH, but with new RRC parameter per channel/signal.  Alternatively, the network can choose to provide the configuration per-UE basis, i.e., the same configuration across all physical channels and signals to a specific UE, which involves lesser specification changes, but reduced flexibility. In terms of the implementation complexity, configuration 1 can be seen as simpler compared to configuration 2. Based on this, one can see configuration 1 as basic feature for SBFD-aware UE while configuration 2 can be an additional capability on top of configuration 1.

SBFD-aware UE is provided configuration semi statically with a granularity of per-channel/signal (e.g., in the config information element of the corresponding channel/signal).

For SBFD-aware UE, Configuration 2 is considered as an additional capability on top of basic feature Configuration 1 with a granularity of per-UE.

#### Configuration 1

Agreement (cf. RAN#118) For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. - For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC. - It is not applicable to PDCCH - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. - For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options: - Option 1: - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig. - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig. - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config. - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource]. - Option 2: - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation. - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

Between the two options considered for determining the valid symbol type, option 1 is rather straightforward. Though option 2 appears to be flexible but difficult to facilitate the utilization of the same configuration settings for different symbol types that can be activated across different instances. It is unclear whether the flexibility of Option 2 can be achieved in practice or not. Hence, it is preferrable to support option 1 for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS.

Support Option 1 to explicitly indicate the valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS in Configuration 1.

#### TCI state and UL power control in SBFD and non-SBFD symbols

Agreement (cf. RAN1#118) For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters. - Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively - FFS: No RRC impact - FFS: New MAC CE or reuse existing MAC CE - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively - FFS: Impact on MAC CE - Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS - FFS: Same offset or different offsets for all channels - FFS: RRC or MAC CE introduced to convey power offset - FFS whether the maximum number of closed power control loops is increased to be more than 2. - FFS: Whether/How to handle the case where UL power control is not associated within TCI state

In the current 3GPP specifications, TCI state and UL power control for PUSCH, PUCCH and SRS transmission are tightly coupled. Therefore, in this section we discuss TCI state and UL power control together to derive a solution through a holistic approach. The current 3GPP specification supports both spatial relation (from Rel-15 onwards) and unified TCI (from Rel-17 onwards) configuration for determining UL TX spatial filter (beamforming). To avoid the discussion from becoming cluttered, the analysis provided in this section mainly focuses on the unified TCI framework. Similar, if not the same, technical concepts can be applied to the legacy spatial relation framework.

According to the unified TCI framework in the Rel-17 specification, a UE is configured with a list of unified TCI states for UL transmission of PUSCH, PUCCH and SRS, which can be “joint DL/UL” or “UL only” TCI states as per RRC configuration. The UE receives beam indication (DCI format 1_1/1_2) providing a TCI codepoint referring to the activated unified TCI states, mapping to an indicated TCI state (joint DL/UL TCI state) or pair of TCI states (separate DL/UL TCI state) for DL reception and UL transmission. Any PUSCH, PUCCH or SRS transmission from the UE towards a TRP can be associated with the indicated common TCI state, which provides:

- Spatial reference signal for UL beamforming

- Uplink power control parameters, comprising of

  - Path-loss reference signal (PL-RS)

  - P0 (target received power)

  - Alpha (fractional factor for path loss compensation)

  - Closed loop index (index of the closed loop power control process)

For a UL transmission towards a single TRP, the associated TCI state is mapped to one spatial reference signal and one set of UL power control parameters. From Rel-18 the unified TCI framework starts to support multi-TRP transmission, in which case one TCI codepoint is mapped to two joint DL/UL TCI states or two pairs of separate DL/UL TCI states, implying separate UL TX spatial filters and power control parameters, one for transmission towards each TRP. The association between TCI state or pair of TCI states and a particular UL transmission is RRC configured for PUCCH, SRS and configured-grant based PUSCH, and dynamically indicated for dynamic-grant based PUSCH. It is worth noting that, to support separate power control closed loops in multi-TRP transmission scenarios, second TPC commands for PUSCH and PUCCH are added to various DCI formats.

At the previous meetings, some companies proposed that separate spatial relations/TCI states should be used for UL transmission in SBFD and non-SBFD symbols, on which we have different opinions. The discussion so far is mainly related to conventional deployment with single-TRP serving the UEs in a cell. In such scenarios, it is not motivated to use different UL TX spatial filters across different types of symbols and the reason is obvious: why would a UE transmit an UL channel or signal with one beamforming direction in a UL symbol and another beamforming direction in a SBFD symbol that are close to each other in time and to same/similar position of received panel(s) (of the same TRP), especially considering the UE has very limited beamforming capability?

It is not motivated to use different UL TX spatial filters for UL transmission in SBFD and non-SBFD symbols towards the same TRP.

In our view the argument to use different TCI states for UL transmission in SBFD and non-SBFD symbols is mainly to enable different UL power control parameters and different closed loops. By inspecting the open loop power control (OLPC) parameters associated to a TCI state as listed above, it becomes obvious that the same PL-RS and Alpha could be used in both SBFD and non-SBFD symbols simply judged from their physical implications. On the other hand, different P0 may be used to account for potentially different interference levels at SBFD and non-SBFD symbols. In this sense a separate TCI state with a higher P0 value can be configured for UL transmission in SBFD symbols. An alternative solution could be to introduce a power offset (or delta-P0) between UL transmission in SBFD and non-SBFD symbols while using the same TCI state. This alternative solution is more efficient in terms of specification impact and signalling complexity. Similar mechanism has been introduced for rel-16 URLLC, where a different P0 (instead of a different OLPC parameter set) could be used for URLLC traffic when switching from eMBB traffic.

For the open loop power control parameters, the same PL-RS and Alpha configuration should be used for UL transmission in SBFD and non-SBFD symbols. Different P0 values may be used to account for potentially different interference levels at SBFD and non-SBFD symbols.

Another argument for supporting separate TCI states for UL transmission in SBFD and non-SBFD symbols is about closed loop power control. Given the fact that the interference level in SBFD and non-SBFD symbols can be significantly different, it makes sense to consider two different power control loops one for each type of symbols.

We acknowledge that using different sets of open loop power control parameters and separate power control loops for UL transmission in SBFD and non-SBFD symbols can be beneficial to tackle different interference levels in different types of symbols. However, implementing such extensions to the current specifications implies significant changes in many aspects. As mentioned in the introduction to the unified TCI framework provided in the beginning of this section, the current specification supports UL transmission towards multiple TRPs by mapping one TCI codepoint to two TCI states (joint DL/UL TCI state) or two pairs of TCI states (pairs of separate DL and UL TCI states), with one TCI state or one pair of TCI states for transmission towards on TRP. Applying different TCI states for UL transmission in SBFD and non-SBFD symbols towards the same TRP implies that four TCI states or pairs of TCI states should be configured in UL multi-TRP scenarios. This requires significant changes to the current specifications. Some of the anticipated changes in RRC, MAC and PHY layers are:

- In RRC configuration, additional lists of Joint/DL and/or UL TCI states need to be configured to the UE for UL transmission in SBFD symbols, on top of the existing lists of TCI states used for UL transmission in legacy UL symbols. Accordingly, the RRC configured associations between TCI state and TRP for SRS, PUCCH and PUSCH transmission need to be updated.

- In MAC layer, new versions of the existing MAC-CEs for activation/deactivation of unified TCI states and indication of unified TCI states need to be introduced.

- Moreover, because different closed loops are used for UL transmission in different types of symbols, there ought to be additional PUSCH TPC command fields in UL DCI formats and PUCCH TPC command fields in DL DCI formats to indicate TPC commands to UL transmission in SBFD symbols.

Using separate power control loops for UL transmission in SBFD and non-SBFD symbols implies significant changes to the current specifications in many aspects.

Alternatively, we can stick to one closed power control loop being used for UL transmission in both SBFD and non-SBFD symbols towards the same TRP to minimize the specification impact (different closed loops are still used for transmission towards different TRPs as in the current specifications). In some cases, if it is necessary to transmit with higher power to account for higher interference level in SBFD symbols, a proper power offset or delta-P0 can be configured to the UE as mentioned previously. If needed the power offset or delta-P0 can be further updated by MAC-CE or RRC signalling to tackle fast variation of the interference level. Even though such a simplified solution might not achieve optimal power control performance in theory in some cases, we believe it is a good balance between technical complexity and performance.

At the last meeting the discussion on SBFD separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework was summarized by the three options as quoted in the beginning of this section. Option 1 requires significant specification changes as analysed above. On the other hand, neither Option 2 nor Option requires separate unified TCI states for UL transmission in SBFD and non-SBFD symbols. Option 2 requires introducing an additional set of UL power control parameters and an additional closed loop power control process to a TCI state, which also implies significant specification impact. Option 3 only adds a power offset for UL transmission in SBFD symbols while reusing the same set of power control parameters and closed loop process. Based on the above discussion, we support Option 3 with the same offset for all channels within RRC message.

For separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, support Option 3 (same unified TCI state is associated with same UL power control parameters, a power offset is introduced for transmission in SBFD symbols and non-SBFD symbols). Same offset is applied for all channels.

### PDCCH

Proposal 2-18 [7] Proposed Agreement: For a CORESET and a search space configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols, down-select from the following options. - Option 1: No enhancements for SBFD-aware UEs in Rel-19. - It is up to gNB to avoid the configuration - Option 2: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL usable PRBs in SBFD symbols. - Applicable to at least USS

The proposed agreement tries to address potential enhancements for PDCCH monitoring in a SBFD carrier for SBFD-aware UEs.

First of all, we need to understand what can be done with the existing specifications to support SBFD operation in a carrier. According to the existing specifications, PDCCH MOs are periodic, with frequency domain allocation specified in the associated CORESET and time locations determined by the periodicity, offset, and duration configured for the search space. By appropriate configuration of the periodicity, offset, and duration for a search space, it is possible to avoid the situation where the MOs occur in both SBFD and non-SBFD slots if that is preferrable.

In the current specification CORESET0 comprises a number of consecutive resource blocks in a number of consecutive symbols as specified in TS 38.213 Clause 13. For backward compatibility purpose, in a SBFD carrier, CORESET0 can be configured with the following two options:

- Option 1, frequency domain resource for CORESET0 is confined within a DL subband.

- Option 2, all PDCCH MOs associated with CORESET0 are mapped to non-SBFD symbols.

Both options can be supported with the existing specifications.

In the current specification other CORESETs than CORESET0 are configured in frequency domain in a bitmap with a granularity of 6 RBs, allowing intermittent configuration around an UL subband in a SBFD carrier. More specifically, non-contiguous frequency domain resource allocation for a CORESET provides significant flexibility to avoid the CORESET overlapping with an UL subband by appropriate configuration of the bitmap freqDomainResources to ensure that groups of 6 RBs are fully contained within a DL subband. As the configuration of multiple search spaces and multiple CORESETs is supported, it is also possible to have one CORESET/search space combination that is tailored for SBFD symbols and another that is tailored for non-SBFD symbols.

Since the existing spec. is already very flexible and the required efforts to make any type of enhancements to PDCCH at this point are significant, our strong preference is to avoid going down the road of PDCCH enhancement to avoid such major spec effort.

In the proposed agreement, Option 2 addresses the situation where some PDCCH candidates are mapped to one or more REs that overlap with REs outside DL subband. In our view, this situation can be avoided by proper CORESET and search space configuration given all the PDCCH configuration flexibilities as mentioned above. In the worst case, if such a situation does occur, SBFD-aware UEs can skip evaluating such PDCCH candidates while legacy UEs will still have to monitor all PDCCH candidates. In our view this is sufficient to support SBFD operation. Further PDCCH enhancement should not be pursued.

The existing specifications for CORESET and search space configuration are sufficiently flexible to support SBFD operation.

RAN1 shall not pursue further PDCCH enhancements other than facilitating SBFD-aware UEs to skip evaluating PDCCH candidates mapped to one or more REs that overlap with REs outside DL subband(s).

### PDSCH

Working Assumption (cf. RAN1#118) For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

In the current specifications for SPS PDSCH, different TBs are transmitted in different PDSCH reception occasions and the TBS are determined separately, one for each individual occasion. Similarly, in multi-PDSCH transmission different TBs are transmitted in different PDSCHs with TBS for each PDSCH being determined separately. In RAN1#117 it was agreed that for single-slot PDSCH transmission in SBFD symbols, the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs. We propose to leverage this agreement and apply it to an individual PDSCH reception occasion in an SPS PDSCH and an individual PDSCH in a multi-PDSCH transmission, such that only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs.

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid, and the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs.

The situation is different for PDSCH repetition across SBFD and non-SBFD symbols, where different redundancy versions of the same TB are transmitted in different PDSCH repetition occasions. The TBS for the TB can be determined in different ways, based on the assigned PRBs (Alt-1), or the assigned PRBs within the DL subbands (Alt-2), or the assigned PRBs within the DL subbands in the first repetition (Alt-3).

In Alt-1 where the TBS is determined based on the number of assigned PRBs, when a PDSCH repetition occasion is mapped to SBFD symbols, rate-matching around non-usable PRBs can cause higher effective code rate and lead to decoding performance degradation. The problem is more severe if a Redundancy Version with systematic bits (i.e., RV 0 or 3) is transmitted in the PDSCH repetition occasion in SBFD symbols. On the other hand, Alt-2 is a conservative solution where the TBS is determined based on the number of assigned PRBs within the DL usable PRBs. However, the effective code rate can become too low for the PDSCH repetition occasions mapped to non-SBFD symbols, resulting in reduced the spectrum efficiency. Alt-3 can be considered as a middle ground solution that tries to address the problems with Alt-1 and Alt-2 as above mentioned. In our view Alt-3 should be selected.

For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid, and the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs of the first repetition.

### CSI-RS

Agreement (cf. RAN1#117) For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

For CSI report associated with periodic/semi-persistent CSI-RS, Option A and Option B achieve the same outcome. However, it is straightforward to implement Option A based on separate reports on SBFD and non-SBFD symbols by leveraging the existing CSI reporting framework with minimum specification updates compared to Options B. Option A offers better scheduling flexibility in the sense that gNB can dynamically configure UE to report CSI measurements on the legacy symbols, SBFD symbols, or both. On the other hand, Rel-18 NES CSI reporting framework requires more specification updates to introduce sub-configurations specifically for each type of symbol.

For CSI report associated with periodic/semi-persistent CSI-RS, Option A is supported.

Working Assumption (cf. RAN1#117) For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

In the legacy specifications, though CSI-RS is scheduled in contiguously in frequency domain, the gNB can configure the UE to report the CSI explicitly for certain subbands that can be non-contiguous in frequency domain. There is no discrimination to this scenario in terms of the CSI processing timeline. Based on this, we do not see the need to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands.

RAN1 to confirm the working assumption for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, by excluding the study of CSI processing timeline to process the CSI-RS across the two DL subbands.

### SRS

A UE in SBFD operation needs to transmit SRS in both SBFD and non-SBFD symbols. At the last meeting it was discussed whether to support separate configuration of SRS resource sets (SRS-ResourceSet), or to support separate configuration of SRS resources (SRS-Resource), or to reuse the same SRS resource but updated with new parameters, for SRS transmission in SBFD and non-SBFD symbols. When evaluating the different design choices, one fundamental question that need to be answered is, for an SRS transmission in SBFD symbols that serves the same usage (i.e., beam management, codebook, non-codebook or antenna switching) with the same time domain characteristic (i.e., periodic, semi-persistent or aperiodic), what need to be configured differently compared to the corresponding SRS transmission in legacy non-SBFD symbols.

First of all, the frequency domain resource allocation of the SRS transmission in SBFD symbols might need be different. An SRS transmission in SBFD symbols must be confined within in the UL subband in frequency domain. Therefore, the frequency resource as well as the frequency hopping pattern for an SRS transmission in a legacy non-SBFD symbols might not be suitable for a transmission in SBFD symbols. The problem is somewhat analogous to CG PUSCH with transmission occasions in SBFD and non-SBFD symbols. At the last meeting, it was agreed that for CG PUSCH with transmission occasions across SBFD and non-SBFD symbols same configuration could be used with different frequency hopping offsets for different frequency domain resource mappings for SBFD and non-SBFD symbols. In our view, it makes a lot of sense to use similar scheme for SRS transmission. To be more specific, if an SRS resource is to be transmitted both in SBFD and non-SBFD symbols, same SRS resource configuration could be used with different sets of frequency domain parameters (i.e., freqDomainPosition, freqDomainShift, c-SRS, b-SRS and b-hop) can be configured for SRS transmissions in SBFD and non-SBFD symbols. The gNB should ensure the PRB allocation for the SRS transmission in SBFD symbols, including all the frequency hopping versions, is confined within the UL subband.

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

...

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

resourceMapping2                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

freqDomainPosition2                      INTEGER (0..67),

freqDomainShift2                         INTEGER (0..268),

freqHopping2                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

...

}

Secondly, the legacy time domain configuration for SRS transmission (startPosition, nrofSymbols and repetitionFactor) in non-SBFD symbols can be reused for transmission in SBFD symbols in some cases. In some other cases it might be preferable to introduce a separate set of time domain parameters for SRS transmission in SBFD Symbols.

Thirdly, regarding TCI state and UL power control parameter, we don’t think there is a need to have different TCI states and UL power control parameters for the same SRS transmission but in SBFD and non-SBFD symbols, discussed in 2.3.3.3.

Therefore, in our view it is most efficient to reuse the current SRS configuration framework. Same SRS resource can be used for transmission in SBFD and non-SBFD symbols. New set of frequency domain and time domain parameters can be introduced to the legacy SRS resource configuration for SRS transmission in SBFD symbols.

On the other hand, in the current specs there is limitation on maximum number of SRS resource sets and maximum number of SRS resources in a set that can be configured to a UE for each usage. Therefore, having separate configurations of SRS resource sets or SRS resources for SBFD symbols and non-SBFD symbols is expected to have larger impacts.

An SRS resource can be transmitted in SBFD and non-SBFD symbols. New parameters can be introduced to the legacy SRS resource configuration to specify frequency domain and/or time domain configuration for SRS transmission in SBFD symbols.

### PUCCH

Agreement (cf. RAN1#118) Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

A separate RRC parameter to control intra slot frequency hopping for SBFD symbols adds the flexibility to turn on/off intra slot frequency hopping for SBFD slots independently of non-SBFD symbols. This is useful if intra slot frequency hopping for some reason is not desirable in SBFD symbols or if the usefulness of frequency hopping in the likely narrower SBFD UL band is questionable. To not complicate the meaning of the current intraSlotFrequencyHopping parameter, the new parameter should be defined and valid for both Configuration 1 and Configuration 2.

For simplicity and maximum flexibility, the following is proposed.

Support separate configurations of intraSlotFrequencyHopping for SBFD and non-SBFD symbols. Introduce a new RRC parameter in PUCCH-Resource to configure intra-slot frequency hopping for SBFD symbols.

If no separate configuration is provided for SBFD symbols, then PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected.

#### PUCCH inter-slot frequency hopping and slot counters

The resources used for PUCCH inter-slot frequency hopping are indicated by RRC parameters startingPRB and secondHopPRB. Which of these to use in a specific slot is determined by the slot repetition counter, even values use startingPRB and odd values use secondHopPRB, according to Figure 6.

In RAN1#118 it was agreed to introduce a second set of parameters to control the PRB starting locations in SBFD symbols. This second set, here denoted startingPRB_SBFD and secondHopPRB_SBFD, makes it possible to select frequency resources differently in SBFD and non-SBFD symbols, see Figure 7. Note that using a single slot repetition counter could prevent optimal frequency separation of resources across slots since both non-SBFD slot allocations in Figure 7 end up in the same resources, see slot repetitions 3 and 7.

To improve on this, two slot counters can be used, one counting repetitions on SBFD slots and one counting repetitions on non-SBFD slots, this would ensure optimal frequency hopping allocation across slots, see Figure 8.

Two slot repetition counters, one for SBFD slots and one for non-SBFD slots, should be used for PUCCH inter-slot frequency hopping to ensure optimal frequency resource allocation across the slots.

Figure 6. Legacy resource indication and the slot repetition counter for PUCCH inter-slot frequency hopping.

Figure 7: Example of PUCCH inter-slot frequency hopping resource allocation using one slot counter.

Figure 8: Example of PUCCH inter-slot frequency hopping resource allocation using two slot repetition counters, one for SBFD and one for non-SBFD slots.

### PUSCH

In the following, we discuss on how to configure, signal, and determine different frequency resources (i.e., different frequency hopping offsets) for PUSCH repetitions/transmissions across SBFD symbols and non-SBFD symbols in different slots where each repetition/transmission has either all SBFD or all non-SBFD symbols.

Regarding configuration different FH offsets for SBFD symbols and non-SBFD symbols, we have the following agreement in RAN1#118 meeting [2].

Agreement (cf. RAN1#118) Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2. - Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols. Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH. - Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols. - Support separate configurations for DCI format 0_2 and other DCI formats as legacy - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions. FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

For PUSCH scheduled by DCI and type 2 CG PUSCH, to support two separate FH offsets for SBFD symbols and non-SBFD symbols, new RRC parameters in PUSCH-Config are introduced to configure a set of FH offsets for SBFD symbols.

Regarding FFS: The number of bits used to indicate FH offset in DCI is determined as legacy, in our view, to minimize specification impact if more than one FH offset lists are configured, the number of entries in both FH offset lists should be the same. Moreover, the entries in 2 RRC FH offset lists for transmissions in non-SBFD symbols and SBFD symbols are linked together into FH offset pairs, which could be indicated/scheduled by a single NUL_hop field in DCI. The bitwidth of NUL_hop field could be the same as legacy NR, i.e., up to 2 bits.

Regarding FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc., in our view, if FH offset list is not provided for SBFD or non-SBFD symbols, FH should not be used for repetition/transmissions in that symbol type.

If two FH offset lists for SBFD symbols and non-SBFD symbols are configured, support the same number of entries for both FH offset lists and linked together into FH offset pairs, which could be indicated/scheduled by a single NUL_hop field in DCI. The bit width of NUL_hop field is the same as legacy NR, i.e., up to 2 bits.

If FH offset list is not provided for SBFD or non-SBFD symbols, FH should not be used for repetition/transmissions in that symbol type.

For determining different FH offsets and frequency resources for SBFD symbols and non-SBFD symbols, we have the following proposal in RAN1#118 meeting.

Proposal 2-13a [8] For PUSCH FH in SBFD symbols, update the PUSCH frequency hopping formula according to one of the following options. - Option 1: RBUL SB start+(RBstart+RBoffset)modNUL SBsize - Option 2: RBUL SB start+(RBstart-RBUL SB start+RBoffset)modNUL SBsize - Option 3: (RBstart+RBoffset)modNUL SBsize - For Option 3, the above result provides the RB offset with reference to the start of UL usable PRB Where RBUL SB start is the starting PRB index of the UL usable PRB within UL BWP and NUL SBsize is the size of UL usable PRB - Applicable at least to intra-slot FH and inter-slot FH - FFS inter-repetition FH

In our view, option 1 and option 3 are similar with respect to different reference PRBs (starting PRB in UL BWP for option 1 and starting PRB of UL usable PRB in option 3) and could be supported. However, details of FH patterns and repetition counters for transmissions in SBFD symbols and non-SBFD symbols should be considered. Considering RBoffset and RBoffset2 as FH offsets indicated for PUSCH transmissions in non-SBFD and SBFD symbols, following is detail on how to determine frequency resources for SBFD symbols and non-SBFD symbols.

In order to determine starting RB in non-SBFD symbols, legacy formula could be used with NBWPsize is number of RBs in UL BWP

RBstart(i)=RBstarti mod 2=0RBstart+RBoffsetmodNBWPsizei mod 2=1,

where index i represents ith PUSCH repetition among PUSCH repetitions in non-SBFD symbols only (separating repetition counters for PUSCH transmissions in non-SBFD and SBFD symbols). RBstart is the starting PRB for first transmission in non-SBFD symbols.

In order to determine staring RB in SBFD symbols, legacy formula could be re-used with NUL SBsize is number of usable RBs in UL subband

RBstart_sbfd(isbfd)=RBstartmodNUL SBsizeisbfdmod 2=0RBstart+RBoffset2modNUL SBsizeisbfdmod 2=1,

where index isbfd represents isbfdth PUSCH repetition among PUSCH repetitions in SBFD symbols only. Moreover, the starting RB in SBFD symbol representing the position of the starting RB in SBFD symbol within the usable UL RBs in UL subband, i.e., the relative position of the staring RB in SBFD symbol compared to the first usable RB within UL subband. This formula corresponds to Option 3 in Proposal 2-13a with more details on how to define FH patterns and separate repetition counters for PUSCH transmissions in non-SBFD and SBFD symbols.

Following Figure 9 describes details on how to determine different starting points and frequency hopping offsets for PUSCH repetitions in SBFD symbols and non-SBFD symbols

Figure 9. Different starting points and frequency hopping offsets for PUSCH repetitions in SBFD symbols and non-SBFD symbols.

For PUSCH FH formula for non-SBFD and SBFD symbols, support separate repetition counters for PUSCH repetitions in non-SBFD and SBFD symbols.

For PUSCH FH in SBFD symbols, support Option 1 or Option 3 in Proposal 2-13a, RAN1#118 meeting.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The SBFD frequency configuration needs to be area-specific for interference reasons.

Observation 2	Because the WID does not include new UE RF requirements for SBFD-aware UEs, they cannot be expected to have improved spectral filtering capabilities and thus there is no benefit with UE-specific guard bands.

Observation 3	Since random access in RRC_IDLE is supported, a UE performing subband filtering needs to support the cell-specific guard band configuration.

Observation 4	Limitations in SSB placement and associated CORESET 0 placement cause any further narrowing of a DL subband to be infeasible.

Observation 5	UE-to-UE CLI can mostly be avoided by scheduling.

Observation 6	The transmitter transient period, agreed by RAN4, will affect both legacy and SBFD-aware UEs and for that reason cannot easily be solved by signaling. Instead, a guard symbol can be implemented by the gNB in the DL symbol preceding the SBFD subband without specification impact.

Observation 7	For interference reasons, it would be beneficial to separate DL and UL spectrum in the intersection between DL and SBFD symbols, i.e., to introduce a guard band in the first symbol(s) of the UL subband.

Observation 8	Option 1 for link direction is already supported by current agreements.

Observation 9	By introducing further UE-specific restrictions in UL subband utilization, the gNB would be constrained in its resource utilization, thereby limiting network utilization.

Observation 10	Neither TDD nor FDD have any UL scheduling restrictions. SBFD should not introduce such restrictions unless it is already supported in the spec.

Observation 11	Configuring an SSB slot as DL jeopardizes the RAN1 agreement that “SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period.”

Observation 12	It is not motivated to use different UL TX spatial filters for UL transmission in SBFD and non-SBFD symbols towards the same TRP.

Observation 13	For the open loop power control parameters, the same PL-RS and Alpha configuration should be used for UL transmission in SBFD and non-SBFD symbols. Different P0 values may be used to account for potentially different interference levels at SBFD and non-SBFD symbols.

Observation 14	Using separate power control loops for UL transmission in SBFD and non-SBFD symbols implies significant changes to the current specifications in many aspects.

Observation 15	The existing specifications for CORESET and search space configuration are sufficiently flexible to support SBFD operation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse WA on frequency domain configuration of SBFD subbands.

Proposal 2	No UE-specific frequency configuration of SBFD symbols is supported.

Proposal 3	Introduce a guard subband in the first symbol(s) of the UL subband. FFS: guard band duration(s).

Proposal 4	UE direction is resolved by configured/scheduled operations, i.e., support Option 1.

Proposal 5	An SBFD configured UE is not expected to be provided with a slot configuration for SBFD symbols using tdd-UL-DL-ConfigurationDedicated. Alternatively, the UE ignores such configuration, if provided.

Proposal 6	An SBFD-configured UE disregards SFI in DCI format 2_0 for SBFD symbols.

Proposal 7	For collision Case 3 (with semi-statically configured DL Rx vs. semi-statically configured UL Tx with repetitions), the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition.

Proposal 8	Clarify that the agreement on SSB prioritization only covers the case of cell defining SSBs transmitted by the serving cell as indicated by ssb-PositionsInBurst and leave the discussion on scheduling availability due to RRM measurements to RAN4.

Proposal 9	SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD-aware UEs in such symbols.

Proposal 10	For dynamic or semi-static DL vs valid RO, the behavior is left to UE implementation with the side condition that the UE prioritizes valid ROs when it would transmit PRACH, otherwise it receives the DL.

Proposal 11	For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, SBFD aware UE does not transmit or receive the physical channel/signal within the slot.

i.	RACH is separately discussed in AI 9.3.2.

ii.	PUSCH repetition type B is supported with a restriction that actual repetition involving different symbol types are invalidated.

Proposal 12	SBFD-aware UE operates by default in Configuration 1, if the UE is not provided which configuration to use.

Proposal 13	SBFD-aware UE can be provided with Configuration 2 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) via dedicated RRC signalling, subject to UE capability reporting.

Proposal 14	SBFD-aware UE is provided configuration semi statically with a granularity of per-channel/signal (e.g., in the config information element of the corresponding channel/signal).

Proposal 15	For SBFD-aware UE, Configuration 2 is considered as an additional capability on top of basic feature Configuration 1 with a granularity of per-UE.

Proposal 16	Support Option 1 to explicitly indicate the valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS in Configuration 1.

Proposal 17	For separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, support Option 3 (same unified TCI state is associated with same UL power control parameters, a power offset is introduced for transmission in SBFD symbols and non-SBFD symbols). Same offset is applied for all channels.

Proposal 18	RAN1 shall not pursue further PDCCH enhancements other than facilitating SBFD-aware UEs to skip evaluating PDCCH candidates mapped to one or more REs that overlap with REs outside DL subband(s).

Proposal 19	For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid, and the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs.

Proposal 20	For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid, and the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs of the first repetition.

Proposal 21	For CSI report associated with periodic/semi-persistent CSI-RS, Option A is supported.

Proposal 22	RAN1 to confirm the working assumption for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, by excluding the study of CSI processing timeline to process the CSI-RS across the two DL subbands.

Proposal 23	An SRS resource can be transmitted in SBFD and non-SBFD symbols. New parameters can be introduced to the legacy SRS resource configuration to specify frequency domain and/or time domain configuration for SRS transmission in SBFD symbols.

Proposal 24	Support separate configurations of intraSlotFrequencyHopping for SBFD and non-SBFD symbols. Introduce a new RRC parameter in PUCCH-Resource to configure intra-slot frequency hopping for SBFD symbols.

Proposal 25	If no separate configuration is provided for SBFD symbols, then PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected.

Proposal 26	Two slot repetition counters, one for SBFD slots and one for non-SBFD slots, should be used for PUCCH inter-slot frequency hopping to ensure optimal frequency resource allocation across the slots.

Proposal 27	If two FH offset lists for SBFD symbols and non-SBFD symbols are configured, support the same number of entries for both FH offset lists and linked together into FH offset pairs, which could be indicated/scheduled by a single NUL_hop field in DCI. The bit width of NUL_hop field is the same as legacy NR, i.e., up to 2 bits.

Proposal 28	If FH offset list is not provided for SBFD or non-SBFD symbols, FH should not be used for repetition/transmissions in that symbol type.

Proposal 29	For PUSCH FH formula for non-SBFD and SBFD symbols, support separate repetition counters for PUSCH repetitions in non-SBFD and SBFD symbols.

Proposal 30	For PUSCH FH in SBFD symbols, support Option 1 or Option 3 in Proposal 2-13a, RAN1#118 meeting.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Huawei, RAN #104, June 2024.

R1-2407587, “Report of RAN1#118 meeting,” ETSI MCC, RAN1 #118-bis, October 2024.

R1-2405782, “Report of RAN1#117 meeting,” ETSI MCC, v 1.0.0, RAN1 #118, August 2024.

R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General,” Samsung, RAN4 #112, August 2024.

R1-2401937, “Report of RAN1#116 meeting,” ETSI MCC, RAN1 #116-bis, April 2024.

R1-2403821, “Report of RAN1#116bis meeting,” ETSI MCC, RAN1 #117, May 2024.

R1-2405425, “Summary #4 of 	SBFD TX/RX/measurement procedures,” Moderator (CATT), RAN1#117, May 2024.

R1-2407220, “Summary#3 of SBFD TX/RX/measurement procedures,” Moderator (CATT), RAN#118, August 2024.