3GPP TSG RAN WG1 #118bis		  R1-2408530

Hefei, China, October 14th – 18th, 2024

Source:	Panasonic

Title: 	Discussion on SBFD TX/RX/measurement procedures

Agenda Item:		9.3.1

Document for:	Discussion

Introduction

A work item on Evolution of NR duplex operation has been approved for Rel.19 [1]. This document provides our view on SBFD TX/RX/measurement procedures.

Discussion

Link direction indication

In RAN1#116, the following agreement was reached on link direction for SBFD-aware UE in SBFD symbol.

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded.

For Option 2, the two benefits were argued. One is collision handling, i.e., the collision between UL transmission and DL reception can be resolved by the dynamic indication or semi-static configuration of the link direction. The other is UE power saving, i.e., UE power consumption can be reduced by avoiding the frequent switching of the link direction and/or the PDCCH monitoring.

For collision handling, in RAN1#117, it was agreed that collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) is assumed as error case if link direction indication is not supported nor provided as follows. For half-duplex FDD, collision Case 3 is also assumed as an error case, and the link direction indication is not introduced, but there would be no critical issue. Thus, the link direction indication would not be essential to handle the collision.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases

For UE power saving, if needed, gNB can control the link direction by scheduling/configuring UE's transmission/reception on SBFD symbols, and can reuse Rel.17 PDCCH monitoring reduction schemes (i.e., search space set group switching, PDCCH skipping). Therefore, the explicit indication of link direction would not be essential.

Proposal 1: For link direction for SBFD-aware UE in SBFD symbol, support Option 1 (UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any)).

Physical channels/signals across SBFD and non-SBFD symbols

In RAN1#117, the following agreement was reached, i.e., two configurations were supported for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability

For Configuration 1, there are two interpretations. One is that the restriction is applied by gNB scheduling. The other is that UE disables certain transmissions/receptions based on the symbol type (i.e., SBFD symbol or non-SBFD symbol). We assume the latter handling because Configuration 1 was based on Option 4 of the agreement in RAN1#116bis. Option 4 for SPS is that "Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid ". Since Option 4 describes that PDSCH in the other symbol type is invalid, we assume the latter handling (i.e., UE disables certain transmissions/receptions based on the symbol type).

As described in FFS, granularity of the configuration needs to be clarified. One possible motivation to select Configuration 1 or Configuration 2 would be the difference of the link condition between SBFD and non-SBFD symbols. If the link conditions between SBFD and non-SBFD symbols are very different, Configuration 1 would be better in order to apply appropriate parameters for SBFD and non-SBFD symbols (e.g., MCS). If the link conditions between SBFD and non-SBFD symbols are similar, Configuration 2 can be used. If whether Configuration 1 or Configuration 2 is selected depends on the link condition, "per UE" granularity can work because the link condition is not significantly different with channels/signals. CLI can be different by the link direction (DL or UL) because UL subband is affected by gNB-to-gNB CLI, and DL subband is affected by UE-to-UE CLI. Thus, "per link direction" can be a compromise between flexibility and simplicity.

Proposal 2: Granularity of the configuration can be per UE or per link direction (i.e., DL or UL).

Configuration 1

In RAN1#118, the following agreement was reached on valid symbol type for Configuration 1.

Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. - For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC. - It is not applicable to PDCCH - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. - For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options: - Option 1: - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig. - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig. - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config. - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource]. - Option 2: - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation. - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, since the first transmission/reception timing can be uniquely determined, the explicit indication would not be needed (i.e., Option 2 can be used). One unclear point of option 2 is how to determine the symbol type when the first transmission is dropped due to other transmission/reception for SP-CSI on PUCCH or PUSCH, and semi-persistent SRS. The following interpretations can be considered.

- Interpretation 1: The valid symbol type is determined based on the symbol type of the first actual transmission.

- Interpretation 2: The valid symbol type is determined based on the symbol type of the first transmission occasion.

For Interpretation 1, symbol type can be different depending on whether the transmission is dropped. For Interpretation 2, symbol type is the same regardless of dropping. We think that Interpretation 2 should be used so that first transmission/reception timing can be uniquely determined.

Proposal 3: For the valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH, and semi-persistent SRS for Configuration 1, support Option 2 with clarification that the the symbol type is determined based on the first transmission occasion (not the first actual transmission).

Configuration 2

In RAN1#118, the following agreement was reached on PDSCH reception for Configuration 2.

Working Assumption For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

As described in FFS, how to determine TBS needs to be clarified for each transmission scheme.

For SPS PDSCH, the main purpose is that the same TBS is used and repeated like voice service. Different TBSs based on DL usable PRBs with the same coding rate are not useful for this purpose. When the link quality is not sufficient, retransmission is carried out as usual operation of SPS. Thus, it would be simple that TBS determination is based on assigned PRBs.

For PDSCH repetitions,the total coding rate can be adjusted based on the number of repetitions. Thus, it would be simple that TBS determination is based on assigned PRBs.

For multi-PDSCH scheduled by a single DCI, different TBSs based on DL usable PRBs, i.e., keeping the same code rate for each, would be similar to allocating different time domain resources in the current specification. To have same code rate with adjusted TBSs are more aligned with the channel condition.

Proposal 4: For SPS PDSCH for Configuration 2, TBS determination is based on assigned PRBs.

Proposal 5: For PDSCH repetitions for Configuration 2, TBS determination is based on assigned PRBs.

Proposal 6: For multi-PDSCH scheduled by a single DCI for Configuration 2, TBS determination is based assigned PRBs within DL usable PRBs only.

Collision handling

Collision cases

In RAN1#116, the following collision cases were identified on the collision between DL reception and UL transmission in a SBFD symbol.

Agreement For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

In our understanding, the existing collision handling for Rel.17 half-duplex FDD is as follows.

- Case 1 and case 2: Dynamically scheduled transmission/reception is prioritized.

- Case 3 and case 4: Not assumed (i.e., error case).

- Case 5: SSB is prioritized.

- Case 6: Up to UE implementation except for PRACH triggered by PDCCH order.

For Case 1 and Case 5, to reuse the existing collision handlings were basically agreed in RAN1#116bis and RAN1#117. For Case 6, although the collision handling depends on UE implementation in half-duplex FDD, there would be no critical issue. Therefore, reusing the existing handling rules can be reused for Case 6.

Proposal 7: Reuse the existing collision handling rules for half-duplex FDD for collision Case 6.

SBFD operation in SSB symbols

In RAN1#117, the following agreement was reached that SSB is prioritized over UL transmission (collision Case 5).

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

On whether/how to utilize the symbol where SBFD symbol and SSB symbol collide (FFS point in the agreement), the following options were summarized in the FL summary in RAN1#118 [2].

- Option 1: A slot consisting of SSB symbols is considered as a full DL slot

- Option 2: UL subband in SSB symbols are reserved

- Option 3: The SSB symbol is considered as full DL symbol

For Option 1 and Option 3, SBFD aware UE can use UL usable PRBs and PRBs in guard band for DL reception. In this sense, resources can be used efficiently. We think that the following two issues need to be further discussed.

1) Transition points between SBFD and non-SBFD symbol

In SI phase, the limitation on the maximum number of transition points between SBFD and non-SBFD symbol was discussed and concluded to be up to two transition points within TDD UL-DL pattern period [3]. From the transition point perspective, Option 1 would be better than Option 3 because SSBs within a SSB burst are non-contiguously allocated. Even for Option 1, if SSBs are allocated in non-contiguous slots, the number of transition points can be increased.

Figure 1 shows an example for Option 3. (a) shows the slot configuration for non-SBFD aware UE. "DDDDU" slot pattern is configured. SSBs are transmitted in slot 0 and slot 2. (b) shows the slot configuration for SBFD aware UE based on Option 3. "DXXXU" slot pattern is repeated, but slot 2 is considered as full DL slot since slot 3 conteins SSB symbols (Note: "X" means SBFD slot). In this case, the number of transition points between SBFD and non-SBFD symbol is 4 within the period. In order to reduce the transition points, further enhancements would be needed, e.g., some consecutive slots are considered as full DL slots, or Option 2 is applied for such case.

Figure 1 Example of non-contiguous SSBs

2) Slot consisting of SSB symbols and UL symbols

If a slot contains SSB symbols and UL symbols, the slot cannot be considered as a full DL slot. Instead of a full DL slot, we think that the slot can be considered as a non-SBFD slot (i.e., a slot consisting of non-SBFD symbols).  Figure 2 shows an example. Slot 3 consists of SBFD symbols and UL symbols. Slot 3 cannot be considered as a full DL slot.

Figure 2 Example of slot consisting of SSB symbols and UL symbols

For Option 2, SBFD aware UE cannot use UL usable PRBs and PRBs in guard band. Non-SBFD aware UE can still use all PRBs, but schedulable UEs are limited. Besides, since the same Tx beam as SSB would be used in SSB symbols, schedulable UEs can be limited in the SSB symbols. Thus, resources in SBFD symbol cannot be used efficiently.

Therefore, our preference is basically Option 1, but we think that a slot consisting of SSB symbols should be considered as a non-SBFD slot (not a full DL slot). In addition, the transition point aspect needs to be further considered.

Proposal 8: For the collision between SSB and SBFD symbols, a slot consisting of SSB symbols is considered as a non-SBFD slot.

Proposal 9: How to reduce the number of transition points between SBFD and non-SBFD symbol should be further discussed such as,

- If the number of transition points exceeds two within TDD UL-DL pattern period, UL subband in SSB symbols are reserved (i.e., the slot is not considered as a non-SBFD slot), or

- Some consecutive slots are considered as non-SBFD slots so that the number of transition points does not exceed two.

Conclusion

In this contribution, we made the following proposals.

Proposal 1: For link direction for SBFD-aware UE in SBFD symbol, support Option 1 (UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any)).

Proposal 2: Granularity of the configuration can be per UE or per link direction (i.e., DL or UL).

Proposal 3: For the valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH, and semi-persistent SRS for Configuration 1, support Option 2 with clarification that the the symbol type is determined based on the first transmission occasion (not the first actual transmission).

Proposal 4: For SPS PDSCH for Configuration 2, TBS determination is based on assigned PRBs.

Proposal 5: For PDSCH repetitions for Configuration 2, TBS determination is based on assigned PRBs.

Proposal 6: For multi-PDSCH scheduled by a single DCI for Configuration 2, TBS determination is based assigned PRBs within DL usable PRBs only.

Proposal 7: Reuse the existing collision handling rules for half-duplex FDD for collision Case 6.

Proposal 8: For the collision between SSB and SBFD symbols, a slot consisting of SSB symbols is considered as a non-SBFD slot.

Proposal 9: How to reduce the number of transition points between SBFD and non-SBFD symbol should be further discussed such as,

- If the number of transition points exceeds two within TDD UL-DL pattern period, UL subband in SSB symbols are reserved (i.e., the slot is not considered as a non-SBFD slot), or

- Some consecutive slots are considered as non-SBFD slots so that the number of transition points does not exceed two.

- Reference

- RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)", Huawei

- R1-2407220, "Summary #3 of SBFD TX/RX/measurement procedures", Moderator (CATT)

- 3GPP TR 38.858 V18.1.0 (2024-03)