3GPP TSG RAN WG1 #118bis                                       R1-2408573

Hefei, China, October 14th – 18th, 2024

Source: 	ETRI

Title:	On-demand SIB1 for idle/inactive mode UEs

Agenda Item:	9.5.2   On-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion/Decision

Introduction

At RAN#105 held in Melbourne in the last month, regarding the Rel-19 NES WI, it was agreed to move the study sub-item “on-demand SIB1 for idle/inactive mode UEs” to the normative work phase. The updated WID includes the following objectives [1]:

2. Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In the last RAN1 meeting, the following agreements were made regarding the on-demand SIB1 operation [2]:

Agreement

For further work on on-demand SIB1 in idle/inactive mode, as a baseline, it is assumed that the transmit power control of UL WUS transmission based on PRACH is applied in the same manner as the legacy PRACH transmission.

- FFS: Potential optimization of the power ramp-up procedure

Agreement

For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options:

- Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages.

- Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages.

Agreement

RAN1 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Agreement

RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options:

- Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options:

- Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission

  - FFS: Definition of RAR reception time

- Option 2: The reference time point is defined based on the UL-WUS transmission time

- Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

Agreement

For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective.

- RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

Conclusion

For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective.

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.

Agreement

For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options:

- Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell

- Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration

- Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS.

Combination of multiple options is not precluded.

RAN2 made the following agreements in RAN2#127 [3]:

Agreements on OD-SIB1 Scenarios: 1. No consensus for RAN1 case 3 in RAN2. OD-SIB1 Validity: 2. We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell). 3. Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement). WUS configuration of NES cell from NES cell: 4. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures. MSG3 based OD-SIB1 REQ: 5. Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell). Access restriction for legacy UEs: 6. Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells. Impacts to RRC connected UEs: 7. RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell). OD-SIB1 SI conclusion: 8. RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

In this contribution, we provide our views on enhancements to support on-demand SIB1 operation with triggering based on UL WUS for idle/inactive mode UEs.

Discussion

Identification of NES cell

Before triggering an on-demand SIB1, UE shall first check whether the cell is transmitting SIB1. For the identification of NES cell, the followings were agreed:

Agreement (@RAN1#116bis) RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options: - Option 1: By WUS configuration - Option 2: By PBCH payload of NES cell RAN2 agreement on on-demand SIB1 (@RAN2#125bis): 1. The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

Basically, UE can identify the presence of regular SIB1 according to the MIB in the received PBCH for a cell (i.e., NES cell). A legacy field (e.g., k_SSB=30 for FR1 and k_SSB =14 for FR2) or a reserved bit in the MIB may be reinterpreted for that purpose by Rel-19 OD-SIB1-capable UEs. If UE is camping on Cell A and receives the UL WUS configuration of the NES cell, the UL WUS configuration can additionally be used to validate the SIB1 presence or absence. For example, the reinterpretation of the certain field in MIB may be performed in a NES cell indicated by the UL WUS configuration. Therefore, the NES cell identification can be done based on both UL WUS configuration (option 1) and PBCH payload of NES cell (option 2). If the k_SSB field is used for the NES cell indication, the actual k_SSB value required to derive the frequency location of CORESET 0 can be provided by the UL WUS configuration.

As captured in the previous section, RAN2 is discussing access restriction of legacy UEs for Rel-19 NES cells. The legacy UEs should be blocked from the NES cell which is not transmitting SIB1. One way to do this is by using the cellBarred bit in the MIB. Then, it should be guaranteed that this cellBarred bit does not block Rel-19 OD-SIB1-capable UEs as well. Therefore, UE behavior on the cellBarred bit needs to be clarified. For example, if a Rel-19 UE determines that a NES cell is not transmitting regular SIB1, the UE may ignore the cellBarred bit in the MIB. This allows the network to reuse the existing access control mechanism for Rel-19 NES cells.

Proposal 1: The presence or absence of regular SIB1 transmission of a NES cell is acknowledged based on both the UL WUS configuration (option 1) and the PBCH payload of the NES cell (option 2).

Proposal 2: UE reinterprets a legacy field (e.g., k_SSB=30 for FR1 and k_SSB =14 for FR2) or a reserved bit in MIB of the NES cell as indicating the presence or absence of SIB1 transmission.

Proposal 3: If k_SSB field is used for NES cell indication, the actual k_SSB value required to derive the frequency location of CORESET 0 is provided by the UL WUS configuration.

Proposal 4: If UE (i.e., Rel-19 OD-SIB1-capable UE) identifies a NES cell that is not transmitting regular SIB1, the UE ignores the cellBarred bit in the MIB (i.e., the UE is allowed to camp on or access the NES cell even if the cellBarred bit in the MIB is set to “barred”).

Transmission of on-demand SIB1

After transmitting the UL WUS, UE monitors one or more Type 0 PDCCH monitoring occasions within a time window to receive the OD-SIB1. The followings were agreed for further study on how to determine the OD-SIB1 window:

Agreement (@RAN1#117) At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement (@RAN1#117) At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission Agreement (@RAN1#117) For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded.

In the legacy msg1-based on-demand OSI, UE is expected first to receive RAR as a response to the msg1 PRACH and next to monitor the regular OSI window. However, for SIB1, according to the agreements it is highly probable that a separate time window for the OD-SIB1 which is different from the regular SIB1 window will be configured. As a result, there is a timeline issue between the OD-SIB1 reception and the RAR reception.

In our view, it is not desirable to impose a fixed reception order between RAR and OD-SIB1. To support both operations, i.e., ‘RAR first and OD-SIB1 later’ and ‘OD-SIB1 first and RAR later’, the starting time and duration needs to be indicated in the UL WUS configuration (option 2). Option 1 forces UE to receive the RAR first and may bring larger specification impact than option 2. For the same reason, it is desirable to define the reference time point based on the UL WUS transmission time (option 2).

The issue of the search space zero configuration in the last agreement is related to how UE identifies the NES cell. If k_SSB or a reserved bit in MIB is used to indicate the presence of regular SIB1, the controlResourceSetZero and the searchSpaceZero fields in MIB are still available and can be used to indicate the CORESET 0 and Type 0-PDCCH CSS set (option 1). Other options just increase the signaling overhead unnecessarily.

Proposal 5: Starting time and duration of OD-SIB1 window are indicated in UL WUS configuration (option 2).

Proposal 6: The reference time point for OD-SIB1 window is defined based on UL WUS transmission time (option 2).

Proposal 7: There is no fixed order forced by the specification between RAR reception and OD-SIB1 reception (up to gNB implementation).

Proposal 8: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell (option 1).

UL WUS configuration

There is ongoing discussion on essential parameters/contents to be included in the UL WUS configuration. From our understanding, the minimum set of information required for the UL WUS configuration should include the followings:

| Parameter/content | Opinion |
| --- | --- |
| List of cell IDs | To indicate applicable NES cell(s). |
| PRACH configuration for UL WUS | Time and frequency resources, the set of preamble(s), and ss-PBCH-BlockPower for the UL WUS transmission need to be included. For the set of preamble(s), a single preamble per SSB may be sufficient since the contention resolution is not required. |
| Frequency location of NES cell | UE needs to know UL frequency location of the NES cell to derive the the UL WUS frequency resource. At least pointA and offsetToCarrier needs to be included. |
| [Initial UL BWP] | Depending on how to define the reference of the UL WUS frequency resource, information for initial UL BWP may or may not be included in the UL WUS configuration. |
| Initial PDCCH monitoring | If k_SSB is used to indicate the presence of regular SIB1, the actual k_SSB value required to derive the frequency location of CORESET 0 needs to be included in the UL WUS configuration. For early RAR reception before receiving OD-SIB1, parameters for Type 1-PDCCH CSS set need to be included. Other parameters for CORESET 0 (controlResourceSetZero) and Type 0-PDCCH CSS set (searchSpaceZero) need not be included if they are delivered via the MIB as legacy. |
| SSB-positionsInBurst | SSB-positionsInBurst needs to be included, from which UE derives the association between the actually transmitted SSBs and the UL WUS ROs. |
| OD-SIB1 window | At least starting time and duration of the OD-SIB1 window can be included as discussed above. |

Proposal 9: The UL WUS configuration includes the following parameters/contents:

- List of cell IDs

- PRACH configuration for UL WUS

  - At least time and frequency resources, set of preamble(s), and ss-PBCH-BlockPower

- Frequency location of NES cell

  - At least pointA and offsetToCarrier

- [Initial UL BWP]

- Initial PDCCH monitoring

  - At least Type 1-PDCCH CSS set

  - k_SSB depending on NES cell identification scheme

- SSB-positionsInBurst

- OD-SIB1 window

  - At least starting time and duration of OD-SIB1 window

Conclusion

In this contribution, we discuss our views on enhancements for on-demand SIB1 transmission for idle/inactive mode UEs, from which the following observations and proposals are drawn:

Proposal 1: The presence or absence of regular SIB1 transmission of a NES cell is acknowledged based on both the UL WUS configuration (option 1) and the PBCH payload of the NES cell (option 2).

Proposal 2: UE reinterprets a legacy field (e.g., k_SSB=30 for FR1 and k_SSB =14 for FR2) or a reserved bit in MIB of the NES cell as indicating the presence or absence of SIB1 transmission.

Proposal 3: If k_SSB field is used for NES cell indication, the actual k_SSB value required to derive the frequency location of CORESET 0 is provided by the UL WUS configuration.

Proposal 4: If UE (i.e., Rel-19 OD-SIB1-capable UE) identifies a NES cell that is not transmitting regular SIB1, the UE ignores the cellBarred bit in the MIB (i.e., the UE is allowed to camp on or access the NES cell even if the cellBarred bit in the MIB is set to “barred”).

Proposal 5: Starting time and duration of OD-SIB1 window are indicated in UL WUS configuration (option 2).

Proposal 6: The reference time point for OD-SIB1 window is defined based on UL WUS transmission time (option 2).

Proposal 7: There is no fixed order forced by the specification between RAR reception and OD-SIB1 reception (up to gNB implementation).

Proposal 8: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell (option 1).

Proposal 9: The UL WUS configuration includes the following parameters/contents:

- List of cell IDs

- PRACH configuration for UL WUS

  - At least time and frequency resources, set of preamble(s), and ss-PBCH-BlockPower

- Frequency location of NES cell

  - At least pointA and offsetToCarrier

- [Initial UL BWP]

- Initial PDCCH monitoring

  - At least Type 1-PDCCH CSS set

  - k_SSB depending on NES cell identification scheme

- SSB-positionsInBurst

- OD-SIB1 window

  - At least starting time and duration of OD-SIB1 window

References

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson, RAN#105.

[2] Chair’s Notes RAN1#118.

[3] Draft RAN2#127 Meeting Report.