3GPP TSG RAN WG1 #118bis                                       R1-2408577

Hefei, China, Meeting, October 14-18, 2024

Source: 	ETRI

Title:	Discussion on measurements related enhancements for LTM

Agenda Item:	9.9.1

Document for:	Discussion/Decision

Introduction

The revised Rel-19 WID for measurement enhancement in LTM is as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management[RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

We address the part of the Rel-19 WID focused on updating the CSI-RS measurement list to reduce unnecessary power consumption from the UE's perspective and minimize downlink radio resource waste from the network's perspective. Additionally, we share our views on the issue of acquiring CSI before the LTM cell switch. Although not directly related to the WID, we also emphasize the need for uplink beam management, in addition to downlink beam management. Lastly, we discuss the potential ambiguities that may arise when allocating CSI-RS resources for candidate cells.

Discussion

2.1 Dynamic update of measurement RS

As shown in Fig. 1, when the UE moves from the serving cell (Cell A) to a neighboring cell (C cell) managed by CU3 (gNB3), the CSI-RS set for candidate cells consists of CSI-RS2, CSI-RS3, CSI-RS4, and CSI-RS5. CSI-RS2 and CSI-RS3 are signals from TRPs belonging to DU2 of the neighboring cell (B cell) managed by CU2 (gNB2), while CSI-RS4 is a signal from TRP5 belonging to DU3 of the neighboring cell (C cell) managed by CU3 (gNB3). CSI-RS5 is a signal from TRP6 belonging to DU4 of the neighboring cell (C cell) managed by CU3 (gNB3). When the UE is instructed by the serving cell to perform CSI-RS based measurements for candidate cells, it measures all four periodic CSI-RS signals and reports the results to the serving cell.

Figure 1. Example of Wireless Environment Configuration for LTM cell switching using CSI-RS based measurement

In Fig. 1, as the UE moves toward C cell, it detects a weakening of the CSI-RS2 signal from TRP3, which belongs to B cell, while the initially weak CSI-RS5 signal strengthens. To quickly reflect these changes, the UE could transmit all L1-RSRP measurement results of periodic CSI-RS signals to the serving cell, enabling an update of the candidate cell's CSI-RS list. However, using both periodic and event-triggered reporting would significantly increase the UE's power consumption. To mitigate this, instead of periodic reporting, the UE can identify when certain CSI-RS signals should be excluded and report this to the serving cell, allowing it to update the CSI-RS measurement list accordingly. This approach helps reduce both UE power consumption and network radio resource waste. The updated list can then be delivered to the UE via RRC or MAC-CE and shared with candidate cells, ensuring that the excluded CSI-RS is no longer transmitted. It remains FFS how the UE will determine the criteria for requesting the serving cell to exclude certain CSI-RS from the candidate CSI-RS list.

Proposal 1: We propose that the UE requests the serving cell to exclude certain CSI-RS from the candidate CSI-RS list, and the serving cell updates the list accordingly. The criteria for determining which CSI-RS to exclude remains FFS.

2.2 Pre-CSI acquisition for LTM

If a CSI acquisition for the corresponding beam is not performed even if the beam management procedure is completed before cell switching, the BS would transmit data very conservatively. As a result, it is difficult to provide exactly the same quality of data service before and after cell switching. For example, during a high-resolution video streaming service, the resolution might decrease when the cell switches. This is because the BS typically decides which MCS (Modulation and Coding Scheme) to apply for data transmission based on the beams configured through beam management. To do this, the BS transmits CSI-RS signals to the UE to acquire CSI information for the downlink beam, and after receiving the CSI values, it applies the appropriate MCS value for data transmission. Thus, if CSI values could be acquired before cell switching, similar service quality could be maintained before and after the switch. In typical beam management procedures, CSI acquisition is not included, but in LTM, cell switching is performed based on CSI-RS measurements. Therefore, if the CSI acquisition process is an extension of these CSI-RS measurements, separate CSI acquisition procedures after cell switching may not be necessary.

However, since the CSI-RS patterns and procedures suitable for CSI reporting differ from those for beam management, additional CSI-RS resource allocation and procedures are required. CSI-RS for CSI acquisition deals with more comprehensive channel information, including channel quality, precoding information, and multipath characteristics. Information reported during CSI measurement includes Channel Quality Indicator (CQI), Precoding Matrix Indicator (PMI), and Rank Indicator (RI). As a result, CSI-RS for CSI acquisition may be transmitted over a wider frequency and time span compared to CSI-RS for beam management. Therefore, obtaining CSI for all candidate CSI-RS signals involves significant overhead, so it is preferable to acquire CSI only for the specific beam once the target cell and target beam are determined.

Figure 2. The procedure of pre-CSI acquisition for LTM

In Fig. 2, the source cell first determines the best beam from the candidate cells based on CSI-RS measurements. Then, a CSI-measurement request is immediately sent, allowing the UE to measure the CSI. After the source cell relays the CSI report results to the candidate cell, it sends a cell switch command MAC CE to the UE. The UE performs cell switching to the downlink beam of the candidate cell indicated by the cell switch command MAC CE. Since the network already knows the CSI values for the beam after the cell switch, it can provide the UE with the same quality of service both before and after the switch.

Proposal 2: Obtaining CSI for all candidate CSI-RS signals involves significant overhead, so it is preferable to acquire CSI only for the specific beam(s) once the target cell and target beam are determined.

2.3 Early beam management for LTM

Beam management is the process of aligning downlink and uplink beams between the UE and the TRP. It typically consists of three stages: beam measurement, reporting, and beam indication. Beam measurement can be divided into downlink and uplink measurements. We have referred to performing beam management before cell switching and completing the three steps mentioned above as "early beam management." Currently, in Rel-19, beam management is only performed for the downlink. However, if a UE is using high-quality streaming services on the uplink, it would be more effective to perform uplink beam management rather than just managing the downlink beams before cell switching. The network can easily determine, through traffic analysis, whether a moving UE is using uplink or downlink services. Therefore, the network can decide which beam management to prioritize based on the traffic. While it is possible to apply the downlink beam to the uplink using channel reciprocity, this approach does not ensure precise beam management, which could lead to a degradation in service quality after cell switching.

Proposal 3: We propose prioritizing uplink beam management before cell switching when a UE is engaged in high-quality uplink services, as this would be more effective than focusing solely on downlink beam management. Relying on channel reciprocity to apply downlink beams to the uplink may result in inaccurate beam management and degrade service quality after the switch.

2.4 Ambiguity in CSI-RS resource allocation for candidate cells

In RAN2#127 [2], the following agreement was reached for CSI-RS configuration in L1 MR event evaluation.

Agreements on L1 MR event evaluation - ... - ... - Any beam in candidate RS configuration can be used for LTM event evaluation. - ... - ... - ...

The aforementioned approval can be interpreted as meaning that NW allocates CSI-RS resources to all candidate cells it has determined. The larger the number of determined candidate cells, the more CSI-RS resource sets (e.g., # candidate cells×M). To this end, it is pivotal for NW to exactly indicate each candidate cell.

Figure 3. Ambiguity in CSI-RS transmission belonging to a non-serving CU.

Fig. 3 shows an inter-CU deployment scenario where the UE switches cells from the serving cell, TRP 1-1, to the 1st best candidate cell, TRP 2-1. Here, it is assumed that NW has determined TRP 2-1 and TRP 3-1 as candidate cells based on the SSB measurements reported by the UE and has already transferred this to the UE. As seen in Fig. 3, before the UE switches to the candidate cell TRP 2-1, which is controlled by gNB 2 of a different CU, it cannot distinguish whether the candidate cell is TRP 2-1 or TRP 2-2 based on the SSB measurements it reported to the serving cell, TRP 1-1. Accordingly, NW faces ambiguity as to whether it should have TRP 2-1 transmit the CSI-RS in the direction of the UE or TRP 2-2 transmit the CSI-RS in the direction of the UE. Therefore, a TRP indication method or a procedural solution that can avoid this ambiguity through TRP indication is required. One example of the TRP indication method is the SSB-grouping-based TRP indication method, in which the SSBs used by each TRP are grouped differently so that the TRP can be directly identified from the estimated SSB [3],[4],[5]. On the other hand, an example of the procedural method is that, before NW determines the candidate cells and allocates CSI-RS to each candidate cell, it performs PDCCH-order-based UL TA acquisition at the early sync stage as a mandatory step. In this way, NW can indicate each candidate cell somewhat. For reference, in the early sync stage, UE-based RACH-less UL TA acquisition makes it very difficult for NW to indicate each candidate cell because no UL signals are sent to the expected candidate cells.

Proposal 4: Specify a method to prevent ambiguity in CSI-RS resource allocation for each candidate cell.

- FFS: Study a candidate cell indication method, e.g., SSB-grouping based TRP indication or mandatory PDCCH-order-based UL TA acquisition in early sync stage.

Conclusion

In this contribution, we note the following proposals.

Proposal 1: We propose that the UE requests the serving cell to exclude certain CSI-RS from the candidate CSI-RS list, and the serving cell updates the list accordingly. The criteria for determining which CSI-RS to exclude remains FFS.

Proposal 2: Obtaining CSI for all candidate CSI-RS signals involves significant overhead, so it is preferable to acquire CSI only for the specific beam(s) once the target cell and target beam are determined.

Proposal 3: We propose prioritizing uplink beam management before cell switching when a user is engaged in high-quality uplink services, as this would be more effective than focusing solely on downlink beam management. Relying on channel reciprocity to apply downlink beams to the uplink may result in inaccurate beam management and degrade service quality after the switch.

Proposal 4: Specify a method to prevent ambiguity in CSI-RS resource allocation for each candidate cell.

- FFS: Study a candidate cell indication method, e.g., SSB-grouping based TRP indication or mandatory PDCCH-order-based UL TA acquisition in early sync stage.

References

- RP-242356, “Revised Work Item: NR mobility enhancements Phase 4,” Apple Inc, China Telecom, 3GPP TSG RAN meeting #105, Sep 9-12, 2024.

- R2-2407571, “Report from session on V2X/SL, R18/19 MOB, and R19 NES,” Samsung, 3GPP TSG RAN WG2 Meeting #127, Aug 19-23, 2024.

- R1-2300094, “Study on TA enhancement for UL M-TRP transmission,” Huawei, 3GPP TSG RAN WG1 Meeting #112, Feb. 27-March 3, 2023.

- R1-2307660, “Views on two TAs for m-DCI,” Samsung, 3GPP TSG RAN WG1 Meeting #114, August 21-25, 2023.

- R1-2307737, “Discussion on two TAs for mTRP operation,” ETRI, 3GPP TSG RAN WG1 Meeting #114, August 21-25, 2023.