3GPP TSG RAN WG1 #118-bis		R1-2408596

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.4.2.2

Source:	IIT Kharagpur

Title:	Frame Structure and Timing Aspects for Ambient IoT

Document for:	Discussion

# Introduction

At RAN#102, a new study item, “Study on solutions for Ambient IoT (Internet of Things) in NR” (FS_Ambient_IoT_solutions), was approved [1]; the SID was most recently revised at RAN#103 [1]. The following objective(s) is/are relevant for the present agenda item:

2. Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

# Discussions

Given the device's complexity, the AIoT device is not expected to perform the sophisticated downlink and uplink synchronization as the NR UE, where the UE wakes up and searches through a frequency raster and looks for a synchronization signal. For AIoT, a very simple approach is required for the operation of these devices. Due to the limited power storage capabilities of AIoT devices, long-term synchronization is not possible. These devices are expected to operate asynchronously, as all these devices may have different power-harvesting cycles.

# D2R Transmission

The following agreements were made in the previous RAN1 meeting on the D2R synchronization-related aspect.

RAN1#116 [2] Agreement At least the following time domain frame structure is studied for A-IoT R2D and D2R transmission. - For R2D transmission, - A R2D timing acquisition signal (e.g. R2D preamble) is included at least for timing acquisition and for indicating the start of the R2D transmission in time domain. - For D2R transmission, - A D2R timing acquisition signal (e.g. D2R preamble) is included at least for timing acquisition and for indicating the start of the D2R transmission in time domain. - FFS other necessary component(s), e.g. midamble, postamble, periodic sync signal, control fields, guard period RAN1#116bis [3] Agreement For D2R transmission, study the necessity of midamble at least for the purpose of performing timing/frequency tracking or channel estimation or interference estimation, considering at least the following: - Modulation and Coding schemes, e.g., data modulation, line/channel coding - Receiving methods, e.g., coherent or non-coherent - D2R transmission length/packet size - Midamble overhead - Timing/frequency accuracy - Phase accuracy RAN1#117[4] Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D RAN1#118 [5] Agreement When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices

Based on the RAN-level study conclusion [6], the max TB size is less than 1000 bits. However, depending upon the A-IoT applications as in [7], it can start as low as 100 bits. Besides, considering repetition or retransmission is supported in the current NR system, whether such a mechanism can be supported in the Ambient IoT system should be further studied with the consideration of device complexity.  It is evident that, for a long payload, additional synchronization signals called midamble may be required to maintain the time synchronization. Meanwhile, for short payload sizes, the PDRCH should not be overloaded with the midamble. With the presence of midamble the Reader tracks the timing/frequency or estimates the channel estimation or estimates the interference, that increases the processing and energy consumption. To signal that the transmission of the target device is not completed yet, assistant information can be provided on the preamble or postamble part to help a better reception at Reader.

Observation 1: The processing and energy consumption overhead is required minimized through the knowledge of the existence of the midamble.

Observation 2: The payload size can be used as a presence of midamble or postamble.

Proposal 1: Use of TBS for indicating the presence of midamble.

Proposal 2: One-bit indicator in the preamble for the existence of the midamble depending upon the TBS.

Proposal 3: If the midamble is present, the payload, say X bits is before the midamble. The ‘X’ can be decided upon discussion.

The preamble includes the indication of the starting of the PDRCH. However, due to an SFO error, the A-IoT devices may not be able to perform cross-correlation operation efficiently, so a baseline approach for all types of A-IoT devices on the correction of the time/frequency estimation would be energy efficient with the midamble. With the size of the payload, the presence of the midamble can be indicated to the reader. A simple method will be to use the size of the payload or end of the PDRCH that is already present in the preamble, which will be sufficient for it. So the overall PDRCH frame size is also incorporating the midamble size.

Observation 3: The existence of the midamble also contributes to D2R transmission time.

Proposal 4: While calculating the [TD2R_min, TD2R_max], the existence of the midamble needs to be considered.

# Device unavailability and its impact

The following agreements were made in the previous RAN1 meeting on device unavailability

RAN1#117[4] Conclusion RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2. RAN1#118 [5] Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection. For Direction 1, following can be the template to capture the solution details proposed by companies, if any. Source; Details. Source [x]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. Source [y]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. For Direction 2, following can be the template to capture the solution details proposed by companies, if any. Source; Details. Source [x]; Solution description Observations or Analysis or Evaluations Specification impacts, if any. Source [y]; Solution description Observations or Analysis or Evaluations Specification impacts, if any.

In energy-constrained A-IoT systems, the availability of devices is dictated by the residual energy, which may fluctuate due to varying environmental conditions, including signal attenuation, device mobility, and physical blockages during the energy harvesting cycle. It is overly optimistic to assume that sufficient energy has been harvested during the last cycle, as these factors can significantly impact the available energy for data transmission.

A critical aspect of A-IoT communication is ensuring reliable data transmission even under energy constraints. Based on the available residual energy, an A-IoT device may not be able to transmit the complete PDSCH frame. In such cases, a dynamic approach to payload segmentation can be employed, wherein the payload is divided based on the energy available at the device.

- Payload Segmentation: This approach allows the payload to be divided into smaller segments, ensuring energy-efficient transmission. In the worst-case scenario, the entire payload is appended with a Cyclic Redundancy Check (CRC) and then segmented into smaller chunks. Each segment is also attached to its own CRC for independent error detection.

- Energy-Based Segmentation Strategy: Different types of A-IoT devices can have varying payload sizes depending on their operational requirements. The payload segmentation technique can be adapted to match the energy level, allowing the system to ensure transmission without requiring complete frame retransmission, thereby conserving energy. The rest of the segments can be transmitted once sufficient energy is harvested.

When employing segmentation, attaching individual CRCs to each segment allows for partial retransmission in case of error detection, similar to the mechanism used in NR frames. This segmentation ensures that only the corrupted or lost segments are retransmitted, significantly reducing the energy overhead compared to retransmitting the entire frame.

Observation 5: The time duration for which the device is charging or communicating with the reader is not quantified in solutions for A-IoT.

Observation 6: The time required for full energy harvesting of the device depends on the type of energy-storing components, environmental conditions, and residual energy of the device after the last operation.

Observation 7: Dividing payloads based on residual energy ensures that A-IoT devices can transmit data without excessive energy. This adaptive strategy is essential in maximizing the device's operational life in energy-limited scenarios.

Observation 8: By attaching CRCs to individual payload segments, the system enables partial retransmissions. This drastically reduces the energy cost of resending entire frames when errors are detected, which is crucial for A-IoT.

Proposal 5: Study on A-IoT charging duration awareness to the reader.

Proposal 6: Enabling the adaptive duty cycle design for the A-IoT devices.

The adaptive duty cycle will depend upon the following:

- Energy forecast

- historical data and environmental conditions of energy harvesting

- historical data and environmental conditions of data transmission

- Knowledge of TR2D_min, TR2D_max, TD2R_min, and TD2R_max

- FFS for other parameters

Proposal 7: Dynamic segmentation of the payloads at A-IoT, depending upon the residual energy, will help exit PDRCH next to the NR frame.

Proposal 8: Each segment of the A-IoT will have CRC.

# Conclusion

In this contribution, we discuss A-IoT frame structure and unavailability issues and have the following proposals:

Proposal 1: Use of TBS for indicating the presence of midamble.

Proposal 2: One-bit indicator in the preamble for the existence of the midamble depending upon the TBS.

Proposal 3: If the midamble is present, the payload, say X bits is before the midamble. The ‘X’ can be decided upon discussion.

Proposal 4: While calculating the [TD2R_min, TD2R_max], the existence of the midamble needs to be considered.

Proposal 5: Study on A-IoT charging duration awareness to the reader.

Proposal 6: Enabling the adaptive duty cycle design for the A-IoT devices.

The adaptive duty cycle will depend upon the following:

- Energy forecast

- historical data and environmental conditions of energy harvesting

- historical data and environmental conditions of data transmission

- Knowledge of TR2D_min, TR2D_max, TD2R_min, and TD2R_max

- FFS for other parameters

Proposal 7: Dynamic segmentation of the payloads at A-IoT, depending upon the residual energy, will help exit PDRCH next to the NR frame.

Proposal 8: Each segment of the A-IoT will have CRC.

# References

- RP-240826,” Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103

- 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #116, Athens, Greece, February, 2024

- 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #116bis, Changsha, China, April, 2024

- 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #117, Fukuoka, Japan, May, 2024

- 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #118, Maastricht, Netherlands, August, 2024

- TR 38.848 v18.0.0, Study on Ambient IoT (Internet of Things) in RAN, September 2023.

- TR 22.840 v19.0.0, Technical Specification Group Services and System Aspects; Study on Ambient power-enabled Internet of Things, December 2023.