3GPP TSG RAN WG1 #118bis		R1-2408655

Hefei, China, Octorber 14th – 18th, 2024

Agenda item:    9.6.2

Source:	Samsung

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE modes

Document for:	Discussion and decision

- Introduction

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In the approved new WID for low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1], one important direction is to specify solutions for LP-WUS operation in IDLE/INACTIVE modes.

This contribution discusses aspects related to LP-WUS operation in IDLE/INACTIVE modes.

- LP-WUS information bits

In RAN1 #117, it has been agreed that the maximum number of information bits (excluding CRC) in a LP-WUS could be less than or equal to [8 or 16], and then in the last meeting, we have agreed that those information bits of LP-WUS can be interpreted as a codepoint value to indicate the subgroup for one or more PO, and the maximum number of subgroups per PO can be down-select from 6 typical options, which can be shown as below:

Agreement For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16]. - FFS the exact value of Z Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X. Agreement For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis: - Option 1: X = 8 - Option 2: X = 16 - Option 3: X = 32 - Option 4: X = 64 - Option 5: X = 128 - Option 6: X = 256 For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option.

In general, the maximum number of information bits carried by a LP-WUS shall be limited by the detection performance and amount of resource allocated for LP-WUS. Typically, a larger amount of information implies higher requirement on the signal design to reliably carry such information, e.g., with reasonable false alarm rate and mis-detection rate. Hence, RAN1 shall carefully determine whether 16 bits can be supported according to the signal design.

Refer to the functionality of LP-WUS, when LP-WUS is used for an indication to trigger the paging monitoring for one or more subgroups, such indication should be carried by the LP-WUS. For the codepoint value indicated by LP-WUS, it can be message-based or sequence-based [2]. Assuming that per UE subgroup paging rate is no larger than 3%, when the number of subgroups is less than or equal to 32, with the 74% probability, at most one subgroup is expected to be waked up by a LP-WUS in one DRX cycle, e.g., at most 5 information bits can be used to trigger paging monitoring for the UEs belong to one of 32 subgroups, as shown in Table 1. In this case, one LP-WUS indicates a codepoint value corresponding to one subgroup, and the UE may only need to monitor one MO for the same beam to determine whether it belongs to the indicated subgroup or not. Actually, the relationship between LP-WUS and PO is not the main issue, the key problem is that how many subgroups can be indicated by a LP-WUS. With this understanding, the relationship between LP-WUS and PO depend on the maximum number of subgroups per PO and the maximum number of information bits in a LP-WUS.

With a same assumption, if the number of subgroups to be indicated is larger than 32, e.g., 64, 128 or 256, more than one subgroup is expected to be waken up in one DRX cycle as shown in Table 1. Taking 256 subgroups as an example, calculate from a statistical perspective, at most 8 subgroups were awakened simultaneously with a 63% probability. To achieve a full mapping function, the number of combinations can be C256,1+C256,2+C256,3+C256,4+C256,5+C256,6+C256,7+C(256,8) =4.23×1014, which means at least 49 information bits should be used to indicate one of the combinations. Considering the limitation for the maximum number of information bits (excluding CRC) in a LP-WUS, multiple LP-WUS should be used to indicate such large information bits, which increase the false alarm rate and the resource overhead. Considering that UE should monitor multiple MOs for the same beam to obtain the whole information for a complete codepoint value, the power consumption at the UE side and the resource overhead from the NW side will be relatively higher.

Table 1. The relationship between maximum number of information bits and the maximum number of subgroupsNote1

| The maximum number of subgroups | The probability of not paged | The probability of only one subgroup being paged | The probability of two subgroups being paged | The probability of three subgroups being paged | The probability of four subgroups being paged | The probability of eight subgroups being paged | Number of subgroups awakened in one DRX cycle with the cumulative probability | Maximum number of information bits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X=8 | 78% | 19.4% | 2% | -<1% | <1% | <1% | 1 with >90% | 3 |
| X=16 | 61% | 30.4% | 7% | <1% | <1% | <1% | 1 with >90% | 4 |
| X=32 | 37% | 37.3% | 18% | 5.5% | 1.2% | <1% | 1 with >74% | 5 |
| X=64 | 14% | 28% | 27.5% | 17.5% | 17.5% | <1% | 2 with >69% | 12 |
| X=128 | 2% | 8% | 15.8% | 20.4% | 19.8% | 2.42% | 4 with >66% | 24 |
| X=256 | 0.04% | 0.3% | 1.28% | 3.3% | 6.6% | 14% | 8 with 63% | 49 |

*Note1: Per UE subgroup paging rate is 3%

Observation 1: If the number of subgroups is no larger than 32, at most one subgroup is expected to be waken up in one DRX cycle with larger than 74% cumulative probability.

- For the maximum number of subgroups per PO (X), the cumulative possibility for wake up at most one subgroup in one DRX cycle is larger than 90% for X=8 and X=16 cases.

Observation 2: The relationship between LP-WUS and PO depend on the maximum number of subgroups per PO and the maximum number of information bits in a LP-WUS.

Companies supporting large subgroup value per PO declare that large subgroup can reduce the false alarm rate, and thus the corresponding UE power consumption can be reduced accordingly. To achieve this goal with minimum cost, subgroup set or a mapping relationship between resources and subgroups can be introduced to realize the subgroup indication. More specifically, different MO for a same beam can be associated with different subgroup sets (or different multiple subgroups as an equivalent terminology) by a predefined rule, and then the LP-WUS transmitted in a MO can be used to indicate one subgroup in the specific subgroup set (or multiple subgroups). The benefit of this design is that UE may only need to monitor one MO for a same beam which can reduce the power consumption at the UE side, while the drawback is that the resource overhead still higher than small subgroup value per PO cases and the paging latency will be relatively large when more than one subgroup to be indicated associated in a same subgroup set or MO since one codepoint can only indicate one subgroup in one DRX cycle. To reduce the impact for paging latency, we should at least reduce the possibility of waken up more than one subgroup in one subgroup set or a MO. That is to say, at most 32 subgroups can be associated with a subgroup set or a MO based on the result from Table 1. For example, for X=64 cases, UEs corresponding to 32 subgroups among 64 subgroups only monitor the first MO for the same beam, while UEs corresponding to the remaining 32 subgroups are monitoring the second MO for that beam. In this case, at most 5 information bits can be used to indicate one of the associated 32 subgroups in a subgroup set or a MO, which can be shown in Table 2.

Table 2. The relationship between maximum number of information bits and the maximum number of subgroupsNote1

| The maximum number of subgroups | Maximum number of subgroups awakened in one DRX cycle with the cumulative probability > 60% | The number of MOs for same beam | Maximum number of subgroups awakened in one DRX cycle with the cumulative probability >74% per MO | Maximum number of subgroups associated with a LP-WUS | Maximum number of information bits |
| --- | --- | --- | --- | --- | --- |
| X=64 | 2 | 2 | 1 | 32 | 5 |
| X=128 | 4 | 4 | 1 | 32 | 5 |
| X=256 | 8 | 8 | 1 | 32 | 5 |

*Note1: Per UE subgroup paging rate is 3%

In addition, due to the higher resource overhead from network perspective, such as 4 times MOs for a same beam for X=128 and 8 times MOs for a same beam for X=256, we still think large number of subgroups per PO should not be supported, at least 128 and 256 is not preferred.

Depending on the further functionality of LP-WUS, additional information can be studied to be carried by the LP-WUS, such as indication for wake up all the UEs for the modification of system information SI change and ETWS/CMAS information. For carrying such information, the trade-off between the payload size and detection performance shall be further investigated.

Proposal 1: For RRC_IDLE/RRC_INACTIVE mode, a LP-WUS indicates a codepoint value corresponding to one subgroup, and the maximum number of subgroups associated with a LP-WUS should be no larger than 32. The number of information bits (excluding CRC) to trigger the paging monitoring in a LP-WUS can be down-selected among 3, 4, 5.

- FFS: other/remaining information bit can be used to support additional functionality such as SI change and ETWS/CMAS information.

Proposal 2: For the maximum number of subgroups per PO, option 1 and/or option 2 is preferred.

- Configuration of LO

In this section, we further discuss the configuration of LO, the relationship between LO and PO, and the configuration for the wake-up delay.

LO configuration and the mapping relationship between LO and PO

In RAN1 #117, it has been agreed that LP-WUS can provide wake-up indication for each subgroup, which can be shown as:

Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered.

For the Option 2 in the above, it should be further clarified whether it defines from network perspective or from UE perspective. If it defined from network perspective, it means one LO can be associated with multiple POs as we discussed for a long time. But from UE perspective, it means that UE in different POs, while these POs are belonged to different DRX cycle, can determine whether the UE should be waked up to monitor paging in those associated POs across multiple DRX cycle by monitoring a same LO, the illustrate figure can be shown as Fig.1. Considering that the size of paging message is relatively large, it may divide to multiple packages, which can be transmitted in multiple consecutive DRX cycles. In this case, it may not be efficient for the UE to enable LP-WUS monitoring between the multiple consecutive DRX cycles. A more efficient way is, when UE receives LP-WUS indicating wake-up, UE is waked up to monitor multiple consecutive POs across the DRX cycles, and not expect to monitor the LOs associated with the remaining POs other than the first PO. Since UE no longer needs to switch back and forth between MR and LR with extra sharp impulse response, the power consumption and paging latency is reduced accordingly.

LO

DRX cycle

LO

PO

DRX cycle

PO

Fig.1 LP-WUS trigger multiple POs monitoring across multiple DRX cycle

Proposal 3: For the relationship between LO and PO, further clarified whether different POs in option2 are defined from network perspective or from UE perspective.

- different POs in the network perspective means different POs in one DRX cycle (or one PF), which can further reduce the power consumption and paging latency.

- different POs in the UE perspective means different POs across multiple DRX cycles.

For the mapping relationship between LO and PO, when Option 1 and Option 2 from network perspective are adopted, if the UE detects LP-WUS and the LP-WUS indicates the UE to monitor its associated PO, the MR will be waked up to monitor the associated one or more PO(s). Considering the limited information bits carried by LP-WUS, Option 1 can be supported. And given that the benefit of Option 2 is that it can reduce the resource overhead to monitor LP-WUS, Option 2 should also be considered to build the association relationship between LO and PO, if the maximum number of information bits allows.

Based on the common understanding for Option 3 discussed in the last meeting, we think Option 3 should be revised as “UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same MO for the same beam within an LO.” With this understanding, it may be applied to 2 different designs. One design is that when one LO associated with more POs with larger number of subgroups, those subgroups can be divided to multiple subgroup sets, UEs belong to same subgroup set monitor same MO for the same beam within an LO. And another design is that when the maximum number of subgroups per PO is larger than 32, the possibility of wake up more than one subgroup increases, considering the limitation of information bits for a LP-WUS, those subgroups can be divided to multiple subgroup sets, the UEs belong to same subgroup set monitor same MO for the same beam within an LO. Compared with option 1 and 2, the resource overhead is much higher for option 3. That is to say, option 3 should be deprioritized.

The configuration of LO can be delivered to the UE before the MR is turned to sleep mode, for example, by system information for RRC_IDLE and RRC_INACTIVE mode. Such configuration can include the gap between two neighboring monitoring occasions, the offset from the start of the LO to the first monitoring occasion or the offset from the end of the LO to the last monitoring occasion, the duration for each monitoring occasion, etc. An illustration of the configuration is shown in Fig. 2.

Proposal 4: The configuration of LO can be provided to the UE by system information for RRC_IDLE and RRC_INACTIVE mode.

Regarding the details on the LO configuration for iDRX, the following agreement was agreed in the RAN1 #118:

Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded Note: The PO mentioned above refers to legacy PO configured for the UE.

The time location of a LP-WUS occasion should be determined by a reference point and an offset, wherein the reference point is the start of a reference PF associated with the LO if LO associated with all the POs in a PF, or the reference point is the start of the one or more PO(s) associated with the LO if LO associated with one or more PO(s) in one DRX cycle. The offset can be a frame level and/or a slot-level and/or a symbol-level offsets from the reference point to the start or the end of the LO.

Proposal 5: The time location of LP-WUS occasion can be determined by a reference point and an offset, wherein the reference point is the start of a reference PF associated with the LO if LO associated with all the POs in a PF, or the reference point is the start of the one or more PO(s) associated with the LO if LO associated with one or more PO(s) in one DRX cycle.

Referring to the periodicity of LP-WUS occasions, the working assumption should be confirmed.

Proposal 6: Confirm the working assumption for the periodicity of LO.

For the specific method for determining the location of LO, we think Alt 1 in the agreement above is better. To make Alt 2 feasible, NW may need to transmit multiple paging messages to accommodate possibly different reported wake-up delay from UEs in the cell. In this sense, the resource overheads for paging transmission in Alt 1 at the network side is much lower than Alt 2.

Proposal 7: If a UE receives a wake-up indication in a LP-WUS, it monitors the PO associated with the offset (i.e., Alt 1 in RAN1#118 agreement).

From UE perspective, to reduce the power consumption and false alarm rate at the UE side, UE may only need to monitor one MO for a same beam and monitor multiple beams for beam sweeping in principle, which can be shown in Fig.2. In Fig. 2, we can find 2 different design directions:

- In direction (a), MOs configured in a LO can correspond to N beams wherein each beam further corresponds to one or multiple MOs, and the gNB perform one round of beam sweeping to transmit all the MOs. For this direction, a UE may monitor at least one MO targeting for the subgroup set it belongs to in at least one beam, up to its implementation. That is to say, UE monitor whether there is an LP-WUS in one beam first and then perform beam sweeping. Under this assumption, UE only monitor one MO in each beam and when UE detect the LP-WUS, UE stops monitoring the subsequent MO in the LO. It should be noted that which MO can detect LP-WUS is uncertain. Therefore, we can only ensure the worst case that the minimum gap time should be the offset between the end of LO and the start position of the associated PO.

- In direction (b), MOs configured in a LO can correspond to one or multiple rounds of beam sweeping, and each round of beam sweeping may further include one or multiple MOs. For this direction, a UE may monitor one or multiple MOs targeting for the subgroup it belongs to. The benefit of direction (b) is that the LP-WUS subgrouping method can be further enhanced to reduce the paging latency in this case. To achieve this goal, the UE with larger wake up time should be divided in one or more subgroups and these subgroups can be configured associated with the MO(s) in the first half of the LO and the UE with shorter wake up time can be divided in one or more subgroups and these subgroups can be configured associated with the MO(s) in the latter half of the LO. To determine the location of the LO with direction (b), NW should configure the minimum gap time according to the maximum value between T1 and T2 by implementation, where T1 refers to the largest wake-up delay UE reported minus the duration of the remaining MO targeting for UE subgroups with shorter wake-up delay, and T2 refers to the shortest wake-up delay UE reported.

- UE monitor one MO targeting for the subgroup set it belongs to in one beam

- UE monitor continuous multiple MOs targeting for the subgroup it belongs to for beam sweeping

Fig. 2 Illustration of the configuration of LO.

Besides, we think it is unreasonable to retransmit the same LP-WUS in multiple MOs, because UE will not be forced to combine all the repetitions. So, the coverage will not be enhanced by increase the number of LP-WUS transmissions.

Proposal 8: From UE perspective, when more than one MO is monitored for a same beam, further study supports gNB select the ideal MO to transmit LP-WUS, thereby introducing gNB scheduling flexibility from a network perspective.

Proposal 9: Do not support repetitions of LP-WUS in multiple MOs corresponding to the same beam in a LO.

Wake-up delay

In RAN1 #118, it has been agreed that minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced for RRC IDLE/INACTIVE mode, which can be shown as:

Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

For the minimum time gap, it should be ensured that there is enough time considering LP-WUS processing time, MR transition time for ramp up, and time/frequency synchronization of MR (e.g., at least > 400ms) so that MR can be ready for the reception of paging.

Proposal 10: The time offset between the end position of the LP-WUS monitoring and the reference point should be larger than LP-WUS processing time, MR transition time for ramp up and time/frequency synchronization of MR (e.g., at least > 400ms).

Refer to the minimum time offset between LP-WUS reception and PO monitoring, when UE reports the supported ramp-up time, gNB should guarantee the MR can be waked up to monitor the corresponding PO after the configured minimum time offset. Then, the network and UE can align their understanding on the minimum time offset. In addition, even though different MR ramp-up time and different sleep state may be supported, considering that this is in RRC IDLE/INACTIVE modes, the simplest way is to ensure the UE with the longer ramp-up time can be waked up after the configured minimum time offset. And UE with shorter ramp-up time can turn on its MR later within the configured minimum time offset based on UE implementation. To reduce the paging latency for the UEs with shorter ramp-up time, a separate subgroup set can be introduced according to the reported wake up delay, and the UEs with longer ramp-up time can be mapped to the MOs in the front part of the LO and the UEs with shorter ramp-up time can be mapped to the MOs in the latter part of the LO (how to define such mapping relationship between the subgroup and MO can be further discussed in RAN2). And in this case, NW also only needs to configure one offset value by implementation.

Proposal 11: NW can configure one offset value based on the reported UE capability.

- The fallback condition for LP-WUS monitoring

In RAN1 #116bis, the following working assumption has been approved for the entry/exit conditions for LP-WUS monitoring in IDLE/INACTIVE modes. In this section, we further analyze the fallback condition based on the FFS part. Before discussing the fallback condition, it should be noted that LP-WUS transmission is up to gNB and it can be activated/deactivated by a gNB signaling. For example, it can be LP-WUS/LP-SS configuration itself or it can be a separate signaling from LP-WUS/LP-SS configuration.

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

According to the working assumption above, when the serving cell measurement performed by the MR is above entry threshold, UE may start LP-WUS monitoring. However, it should be noted that in some cases, even though the serving cell measurement performed by the MR is above entry threshold, UE is not expected to start LP-WUS monitoring. For example, in legacy, while the SDT procedure is ongoing in RRC_INACTIVE state, the UE monitors the PO in order to receive only the Short Message as specified in TS 38.331. When only PEI is configured to be monitored, the receiver does not need to turn to ultra-deep sleep state, so the SDT procedure will not be affected. But when LP-WUS is enabled and the MR turn to ultra-deep sleep state, in which UE cannot transmit or receive SDT anymore. Therefore, if the SDT procedure is ongoing in RRC_INACTIVE state, UE is not expected to monitor LP-WUS even though the entry condition is satisfied. In addition, UE should stop LP-WUS monitoring and start a PUSCH transmission when there is an CG-SDT. A same logic should be applied when there is an UL transmission, which means when UE expect to send RACH relevant signals/channels such as MSG1 and MSG3 or MSG A, and/or UE expect to transmit an RA-SDT, UE should stop monitor LP-WUS.

Another case that the UE should stop LP-WUS monitoring is that when the UE haven’t received LP-WUS for a long time (e.g., configured by the gNB), to avoid missing the paging message, MR should be waked up to find if there are any paging message for the UE.

Proposal 12: If the SDT procedure is ongoing in RRC_INACTIVE state, UE is not expected to monitor LP-WUS even though the entry condition is satisfied. Besides, UE should stop LP-WUS monitoring when UE expects to transmit RACH relevant signal/channel or SDT.

Proposal 13: A UE stops monitoring LP-WUS when the UE haven’t received LP-WUS for a configured time duration.

- Association between LP-WUS MO(s)/LP-SS and SSB

In the last meeting, the following agreement was achieved to establish the association relationship between LP-WUS MO(s)/LP-SS transmissions and SSB beams, which can be shown as:

Agreement At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

Refer to the association between LP-WUS MO(s) and LP-SS transmission, we feel that the number of beams for LP-WUS and LP-SS transmission can be the same, which is a simple and easy way for UE’s operation. The association between LP-WUS MO(s) and SSB beams should be decided by the predefined criteria shown as Fig. 2. In addition to support 1:1 association, one LP-WUS MO can be associated with multiple SSB beam indexes to improve the beam sweeping efficiency of LR and to reduce the resource overhead.

For the association relationship between LP-SS transmissions and SSB beams, the association can be decided by a predefined criteria according to the mapping between resource for LP-SS transmission and SSB beam index or by gNB configuration. Assuming that LP-SS is also transmitted in an LP-SS burst like SSB, one LP-SS transmission in a LP-SS burst can be also associated with multiple SSB beam indexes to reduce resource overhead for LP-SS burst transmission considering the number of SSB for the current SSB beam operation and the coverage for LP-SS can be smaller than that for SSB., and how to configure the LP-SS burst can be further discussed.

Proposal 14: One to multiple association between LP-WUS MO(s)/LP-SS transmissions and SSB beams can also be supported.

In order to increase the reduced coverage for LP-SS when one to multiple association between LP-SS and SSB is supported, LP-SS can be power-boosted and then power ratio between LP-SS and SSB can be indicated. Also, to use LR to assistant the pathloss measurement, it is benefit to associate the power control parameter of LP-SS with SSB. That is to say, LP-SS power control parameter can be determined by a power control offset relative to ss-PBCH-BlockPower, which is defined as the average EPRE of the resources elements that carry secondary synchronization signals. The EPRE ratio between LP-SS and the SSB shall be configured by system information. After the association relationship is established, the AGC tuning time will be reduced during the transition between MR and LR, and the measurement result can be shared between MR and LR in case there is a RACH transmission during LP-WUS monitoring.

Proposal 15: Support configuring the EPRE ratio between LP-SS and SSB.

- Conclusion

This contribution discusses the solutions for LP-WUS operation in RRC_IDLE/RRC_INACTIVE modes. Proposals are summarized as follows:

Observation 1: If the number of subgroups is no larger than 32, at most one subgroup is expected to be waken up in one DRX cycle with larger than 74% cumulative probability.

- For the maximum number of subgroups per PO (X), the cumulative possibility for wake up at most one subgroup in one DRX cycle is larger than 90% for X=8 and X=16 cases.

Observation 2: The relationship between LP-WUS and PO depend on the maximum number of subgroups per PO and the maximum number of information bits in a LP-WUS.

Proposal 1: For RRC_IDLE/RRC_INACTIVE mode, a LP-WUS indicates a codepoint value corresponding to one subgroup, and the maximum number of subgroups associated with a LP-WUS should be no larger than 32. The number of information bits (excluding CRC) to trigger the paging monitoring in a LP-WUS can be down-selected among 3, 4, 5.

- FFS: other/remaining information bit can be used to support additional functionality such as SI change and ETWS/CMAS information.

Proposal 2: For the maximum number of subgroups per PO, option 1 and/or option 2 is preferred.

Proposal 3: For the relationship between LO and PO, further clarified whether different POs in option2 are defined from network perspective or from UE perspective.

- different POs in the network perspective means different POs in one DRX cycle (or one PF), which can further reduce the power consumption and paging latency.

- different POs in the UE perspective means different POs across multiple DRX cycles.

Proposal 4: The configuration of LO can be provided to the UE by system information for RRC_IDLE and RRC_INACTIVE mode.

Proposal 5: The time location of LP-WUS occasion can be determined by a reference point and an offset, wherein the reference point is the start of a reference PF associated with the LO if LO associated with all the POs in a PF, or the reference point is the start of the one or more PO(s) associated with the LO if LO associated with one or more PO(s) in one DRX cycle.

Proposal 6: Confirm the working assumption for the periodicity of LO.

Proposal 7: If a UE receives a wake-up indication in a LP-WUS, it monitors the PO associated with the offset (i.e., Alt 1 in RAN1#118 agreement).

Proposal 8: From UE perspective, when more than one MO is monitored for a same beam, further study supports gNB select the ideal MO to transmit LP-WUS, thereby introducing gNB scheduling flexibility from a network perspective.

Proposal 9: Do not support repetitions of LP-WUS in multiple MOs corresponding to the same beam in a LO.

Proposal 10: The time offset between the end position of the LP-WUS monitoring and the reference point should be larger than LP-WUS processing time, MR transition time for ramp up and time/frequency synchronization of MR (e.g., at least > 400ms).

Proposal 11: NW can configure one offset value based on the reported UE capability.

Proposal 12: If the SDT procedure is ongoing in RRC_INACTIVE state, UE is not expected to monitor LP-WUS even though the entry condition is satisfied. Besides, UE should stop LP-WUS monitoring when UE expects to transmit RACH relevant signal/channel or SDT.

Proposal 13: A UE stops monitoring LP-WUS when the UE haven’t received LP-WUS for a configured time duration.

Proposal 14: One to multiple association between LP-WUS MO(s)/LP-SS transmissions and SSB beams can also be supported.

Proposal 15: Support configuring the EPRE ratio between LP-SS and SSB.

References

- RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC, RAN#102, December 2023.

- R1-2408654 Discussion on LP-WUS and LP-SS design, Samsung, RAN1#118bis.

- R1-2408656 Discussion on LP-WUS operation in CONNECTED modes, Samsung, RAN1#118bis.