3GPP TSG RAN WG1 #118bis				R1-2408671

Hefei, China, October 14th – 18th, 2024

Agenda Item:	9.4.1.2

Source: 	LG Electronics

Title: 	Discussion on Ambient IoT device architectures

Document for:	Discussion and decision

- Introduction

The Re1-19 SI on solutions for Ambient IoT was approved in RAN#102 meeting. The SI objective relevant to this agenda item is as follows [1].

## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI … - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. …

- Discussion

In this contribution, we present our views on some of the remaining issues of Ambient IoT device architectures.

  - Ambient IoT device architectures

On the Ambient IoT device architectures for study, the agreements so far are summarized in Table 1:

Table 1 Ambient IoT device architectures for study

|  | Device 1 | Device 2a | Device 2b |
| --- | --- | --- | --- |
| Device architectures | RF-ED receiver | RF-ED receiver | RE-ED receiver IF-ED receiver ZIF receiver |

According to the original work plan in [3], down-selection on the device architectures is to be discussed from RAN1#117. In our view, if down-selection is needed, at least RF-ED receiver architectures for all Ambient IoT devices (1/2a/2b) should be kept in the interest of unified design and minimize the workload.

Proposal 1: For Ambient IoT device architectures, prioritize, if needed, RF-ED receiver architectures for all Ambient IoT devices (1/2a/2b).

In RAN1#116bis, it was agreed to study the feasibility of large frequency shift (large FS, i.e. between DL/UL spectrum of an FDD band) for device 2a. In our view, the large FS is infeasible at least for Device 1 mainly due to the power constraint. For Device 2a, as of RAN1#118 meeting, there is no conclusion yet on the feasibility of the large FS. And based on the observations that we agreed in RAN1#118 meeting, whether the frequency uncertainty ([e.g., 5kHz~50kHz]) caused by the large FS is acceptable, and how to handle it (if needed, e.g., using larger gap compared to the small FS) needs to be further discussed. Given the situation, we think the discussion on the scenarios that require the large FS can hold until there is a progress on the feasibility study on the large FS.

Proposal 2: De-prioritize the discussion and evaluation on the scenarios that require the large FS, e.g., the scenarios where CW is transmitted in DL spectrum and backscattered in UL spectrum.

Based on RAN plenary decision in RAN#103 on energy harvesting [2], which is “The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study”, energy harvesting aspects should be studied in RAN1. As the energy harvesting time is also related to the energy storage capacity, the evaluation assumptions on the energy storage capacity for devices 1/2a/2b need to be discussed.

Proposal 3: For RAN1 study purpose, consider RF energy harvesting time and its impact on device availability.

Proposal 4: Include in the study the case where different device types have different energy storage sizes.

- FFS: different energy storage sizes for the same device type

- FFS: assumption on the energy storage sizes for each device type/capability

Recent study in IEEE P802.11 on AMP IoT devices in WLAN reveals that for a device that has the capability of power storage and can work using the RF power that is harvested and stored, the minimum received RF power can be relaxed to -30dBm. Taking into account the device technology advances even if it may be slow, taking -35 to -30 as the typical RF EH sensitivity seems to be a reasonable compromise. We may also further check if the RF energy harvesting threshold can be different between device types.

Proposal 5: For Ambient IoT study, assume RF energy harvesting sensitivity of -35 to -30 dBm.

- FFS: if the RF energy harvesting sensitivity can be different depending device types

In RAN1#118 meeting, the following agreement was made on Ambient IoT clocks/LOs.

Agreement Adopt the following table for clock Description; Applicable device types; Clock speed; Power consumption; Initial clock accuracy; Accuracy after clock sync / calibration; Clock drift. Purpose #1 of the clock; Sampling; Device 1, 2a, 2b; a few MHz; < [1]uW for device 1 FFS for device 2a/2b; [10^4 ~ 10^5] ppm for device 1 [10^3~ 10^4] ppm for device 2a/2b; FFS (if applicable for device 1); FFS. Purpose #2 of the clock; Small frequency shift; At least for device 1 and device 2a. [Purpose #3 of the clock]; [Time counting (if supported)]; [Device 1, 2a, 2b]; FFS; FFS the same or different for different devices; FFS; FFS (if applicable); FFS. Purpose #4 of the clock; Large frequency shift (if supported for device 2a); Device 2a; 10s MHz; [10s] uW; FFS; FFS; FFS. Purpose #5 of the clock; LO for carrier frequency (for up/down conversion); Device 2b; e.g., [900] MHz; [10s ~ 100s] uW; FFS; FFS; FFS. Note: It does not necessarily imply that different purposes of LOs/clocks correspond to separate discrete LOs/clocks, which is up to implementation..

During the meeting, based on the table above, FL slightly updated the table format e.g., clarifying that the last 5 columns (Clock speed, Power consumption, Initial clock accuracy, Accuracy after clock sync / calibration, Clock drift) belong to clock requirements. In our view, we don’t have to make those columns as clock requirements for now given there is no consensus yet on whether Ambient IoT devices can or need to perform clock calibration and there are possibilities that some of those characteristics are left to device implementation. And also there is no need to further divide the rows within the Purpose #1 as it is not quite clear what is meant by the different rows within Purpose #1 and we don’t think we’ll have to take time to try a converged view on this because we don’t think it is essential. The references being part of an agreement is okay for us.

Proposal 6: On the table form for Ambient IoT clocks/LOs, we prefer to stick to the table form that we agreed in RAN1#118 meeting. In particular,

- we do not support to change some of the clock characteristics to clock requirements

- there is no need to sub-divide rows within Purpose #1

There has been a discussion on whether to define device energy state in relation to the impact of energy harvesting on device availability for transmission and reception procedures. In our view, there can be three states which are On, Off and Sleep states. On and Off states may be essential while on the necessity of the Sleep state may need some discussion. How to define each of the states, how transitions b/w states occur hopefully under reader’s control, how in each state an Ambient IoT device behaves, and how they are related to the description on the clock/LO in the  above can be further discussed if there is an enough consensus on the study of device energy states. Related to the impact of energy harvesting on device availability for transmission and reception procedures, duty cycle-based operation can also be studied.

Proposal 7: Related to the impact of energy harvesting on device availability for transmission and reception procedures, study the following aspects:

- whether/how to define energy states (e.g., On, Off, [Sleep])

- duty cycle-based operation

Given previous agreement on the antenna architecture of Ambient IoT devices, it is our understanding that Ambient IoT devices cannot perform reception and energy harvesting at the same time. During sleep state, if devices need to support synchronization, not just clock counting operation, this can only be supported in a TDM manner. It may be possible to leave the decision on when devices switch to Rx mode for synchronization to the device implementation, but reader may still want to configure the occasions for synchronization, e.g., to provide the signals for synchronization in a more efficient manner. Outside the occasions for sync monitoring, device can switch the Rx antenna to energy harvesting module.

Proposal 8: For a single antenna for both communication (Tx/Rx) and energy RF energy harvesting, study feasibility of supporting Rx (e.g., for synchronization) and RF energy harvesting in a TDM manner during the Sleep state.

For Ambient IoT devices, if the frequency selectivity can be improved, device multiplexing capacity can be greatly improved by using FDMA also in D2R reducing the overall latency for an inventory round. As companies have different understanding on feasibilities of RF BPF, BB filter, antenna selectivity, etc., at least for some device types, we propose to study whether/how to improve the frequency selectivity.

Proposal 9: Study whether/how to improve frequency selectivity of Ambient IoT device, e.g., feasibility of RF BPF, BB filter, antenna selectivity, etc.

- Conclusion

In this contribution, we shared our views on Ambient IoT device architectures.

Proposal 1: For Ambient IoT device architectures, prioritize, if needed, RF-ED receiver architectures for all Ambient IoT devices (1/2a/2b).

Proposal 2: De-prioritize the discussion and evaluation on the scenarios that require the large FS, e.g., the scenarios where CW is transmitted in DL spectrum and backscattered in UL spectrum.

Proposal 3: For RAN1 study purpose, consider RF energy harvesting time and its impact on device availability.

Proposal 4: Include in the study the case where different device types have different energy storage sizes.

- FFS: different energy storage sizes for the same device type

- FFS: assumption on the energy storage sizes for each device type/capability

Proposal 5: For Ambient IoT study, assume RF energy harvesting sensitivity of -35 to -30 dBm.

- FFS: if the RF energy harvesting sensitivity can be different depending device types

Proposal 6: On the table form for Ambient IoT clocks/LOs, we prefer to stick to the table form that we agreed in RAN1#118 meeting. In particular,

- we do not support to change some of the clock characteristics to clock requirements

- there is no need to sub-divide rows within Purpose #1

Proposal 7: Related to the impact of energy harvesting on device availability for transmission and reception procedures, study the following aspects:

- whether/how to define energy states (e.g., On, Off, [Sleep])

- duty cycle-based operation

Proposal 8: For a single antenna for both communication (Tx/Rx) and energy RF energy harvesting, study feasibility of supporting Rx (e.g., for synchronization) and RF energy harvesting in a TDM manner during the Sleep state.

Proposal 9: Study whether/how to improve frequency selectivity of Ambient IoT device, e.g., feasibility of RF BPF, BB filter, antenna selectivity, etc.

- References

RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

RP- 240854, Moderator's summary on R19 Ambient IoT

R1-2400328, Ambient IoT Study Item work plan