3GPP TSG RAN WG1 #118bis                                                            R1- 2408690

Hefei, China, October 14th – 18th, 2024

Source:	Kyocera

Title:	Specification Support for AI/ML for Beam Management

Agenda item:	9.1.1

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss AI/ML applications for Beam Management (BM) in the physical layer, focusing on the objectives outlined in the Rel-19 New Radio (NR) work item on Artificial Intelligence (AI)/Machine Learning (ML) [1]. During RAN#102, the Rel-19 work item titled ‘New Work Item Description (WID) on AI/ML for NR Air Interface’ received endorsement. The primary objective of this work item is as follows:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE - NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

We explore various sub-use cases for an AI/ML-assisted beam management framework, along with an analysis of findings from the Rel-18 study item on AI/ML-based beam management. Additionally, we provide an initial assessment of the potential specification impact required to enable these sub-use cases for 5G NR Rel-19.

# Configuration of Set A/B for NW sided and UE sided AI/ML models

In RAN1 116bis meeting, the following agreements were made regarding the configuration of Sets A and B for NW side and UE side AI/ML models respectively:

Agreement For network-sided AI/ML model for BM-Case1 and BM-Case2, - support using existing CSI framework for configuration of Set A as the starting point. - support using existing CSI framework for configuration of Set B as the starting point. - Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specification.

In a NW-sided AI/ML model for BM-Case1 and BM-Case2, the gNB may not need to configure Set A for the UE. This is because the inference is performed at the gNB side, where beam prediction occurs transparently to the UE. The gNB’s responsibilities may include configuring reporting parameters, such as the number of beams to be reported and the reporting periodicity.

However, Set B must be configured for the UE to facilitate the collection of L1 measurements. This configuration enables the UE to determine the resources needed to capture and provide the required L1 measurements essential for the NW’s AI/ML model’s operation. The L1 Set B-related measurements are reported to the gNB on UL-related data or control channels. This configuration ensures that the UE can effectively gather the necessary data inputs for the NW-sided AI/ML model.

- For a NW-sided AI/ML model for BM-Case1 and BM-Case2, for the configuration of inference results, RAN1 should consider the following options:

- Set A: No configuration for Set A is required for the UE, as the inference process is conducted transparently from the UE.

- The gNB should configure reporting-related parameters, including the report size and the number of beams that the UE should report.

- Explicit configuration of Set B is necessary, with resources for Set B explicitly defined for the UE.

In RAN1#118 meeting, the following agreement was made regarding the configuration of Sets A and B for a UE-side AI/ML model:

Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

In this agreement, a single CSI-ResourceConfigId is defined for both Sets A and B. Sets A and B can be distinguished at the resource set level, meaning that Set A could be defined using a different ResourceSetId than Set B. For UE-side AI/ML model inference, Set B needs to be explicitly configured for the UE to perform measurements and derive its AI/ML model. However, Set A does not need to be configured for measurements. It can be virtually configured for the UE to provide a reference for mapping its AI/ML model output, in the case of classification models, to the corresponding CRIs that the gNB can interpret.

Based upon that, we have the following proposal:

- For a UE-side AI/ML model, support configuring Sets A and B for inference results reporting, where:

  - Set A is virtually configured. It is not intended to provide measurements to the UE. Instead, it serves as a reference to map the UE-side AI/ML model output, in the case of classification models, to the corresponding CRIs that the gNB can interpret.

  - Set B is explicitly configured for measurements for the UE. It enables the UE to perform measurements that derive the UE’s AI/ML model.

## The association between Set A and Set B with or without additional IE

When both Sets A and B are configured (UE side AI/ML model case), the UE needs to know which resources to use for measurements during both inference and performance monitoring (Set B resources) for the AI/ML model input, and which resources to use for gathering reference/ground truth data (Set A resources) during the performance monitoring phase. Therefore, as we understand it, the association/mapping of Sets A and B defines which resources belong to each set. Generally, we do not see a way to define this association without introducing an additional Information Element (IE). This new IE could be introduced within (and not limited to) CSI-ResourceConfig or within a new AI/ML specific configuration. Hence, we propose the following:

- For UE-side AI/ML models, the association provides the UE with information about the beams that belong to Sets A and B. To facilitate this association, an IE is required. This IE could be defined within the CSI-ResourceConfig or within a new AI/ML specific configuration.

## An additional discussion

In the Feature Lead (FL) notes for RAN1#117 meetings, among of the questions to discuss were the following:

Whether using current configuration for Set A measurement and report have any restriction to gNB scheduling and/or require additional capability from UE? considering combination of AP, SP, P of resource config and report config. If, yes, any enhancement to resolve it? For BM case 2, using current configuration for Set B and Set A (for data collection and or training) measurement and report have any restriction to gNB scheduling and/or require additional capability from UE?

As we understand it, the question is whether the configuration and reporting of a Reference Signal (RS) measurement (from either Set A or Set B resources) would impose constraints on the gNB scheduler. In our opinion, the answer is YES. For example, a UE scheduled for a Set B measurement cannot be scheduled for an UL transmission in the same time slot. However, we believe this is a scheduler implementation issue and may not require any specification effort. Hence, we propose the following:

- For BM Case 1 and Case 2, in a NW-sided or UE sided AI/ML models, for any LCM aspect (data collection, training, inference, and performance monitoring), the configuration of either Set A or Set B for the UE for measurement and reporting may impose a restriction on the gNB scheduling. However, it will not require any additional capability from the UE. This is an implementation issue and does not necessitate further specification effort.

# Report for training for NW-Sided AI/ML Models

In this section, we discuss the procedure for training sample collection for a network-side AI/ML model. According to the definitions in [1], Set B may differ from Set A, be a subset of Set A, or be equal to Set A. Assuming the former case where Set B is different from Set A, during the data collection and deployment phases, the network configures the UE with "CSI-AIML-ResourceSet(s)," which could be defined as a new IE within the current CSI framework for Set B transmission. This allows Set B to be flexibly defined by the network (it may or may not be SSB wide beams). Additionally, the network configures "NZP-CSI-RS-ResourceSet(s)" within the "nzp-CSI-RS-ResourceSetToAddModList" for Set A transmission. The report configuration of the former may include a new IE "AIML-info-RSRP" as the report quantity, allowing the L1-RSRP measurements to be collected for the data sample input, particularly if Set B is defined arbitrarily by the network. In contrast, the report configuration of the latter may include “cri-RSRP” as the report quantity for the data sample label.

The data collection phase could be conducted offline, where UL UCI payload overhead reduction is not a concern. Therefore, there may be no need to consider UL reporting overhead reduction mechanisms currently being discussed in inference reports by other companies. Consequently, during the reporting of L1-RSRP measurements from Set B transmissions, the entire Set B measurements should be reported by the UE to the gNB without any omissions. This approach could allow the model to better represent the input sample data distribution and provide improved generalization when only part of the input measurements is reported during the inference stage.

Based on that, we have the following proposal:

- For training data collection of a NW side AI/ML model, in BM Cases 1 and 2, RAN1 should further study the following:

- Support using the CSI framework for inputs and labels collection of data samples for the purpose of training the AI/ML model. New IEs may be introduced, e.g., “CSI-AIML-ResourceSet” and “AIML-info-RSRP” to provide the gNB the flexibility to define any arbitrary Set B.

- Support reporting all Set B measurements during the data collection and training phase.

# Report for Inference

In this section, we discuss various aspects of the UE L1 Set B measurements reporting in the context of the NW-sided AI/ML model, as well as the inference results reporting for a UE-sided AI/ML model. This includes considerations for reporting quantities, mechanisms and potential overhead reduction techniques.

## NW Sided AI/ML Models

In RAN1#117, the following agreement was made:

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specification

This agreement addresses the mechanisms for input data sample collection for the AI/ML model located on the NW side. Ideally, the gNB transmits Set B of beams and receives measurements from the UE for those beams. However, during the SI phase, it was demonstrated that good prediction accuracy could be achieved even with the reporting of L1-RSRP for only a subset of Set B beams, significantly reducing the reporting overhead [1][2]. Therefore, this agreement discusses potential methods to minimize the size of Set B reporting from the UE to decrease the reporting overhead.

It is reasonable to assume that the NW-side AI/ML model is designed to accommodate full Set B measurements as its input. Therefore, if not all L1-RSRP measurements are reported to the NW as part of the overhead reduction, it is necessary to determine the indices of the reported beams to properly map them to the corresponding pin of the AI/ML model. Consequently, in the first bullet, if the configured value (M) is smaller than the size of the measurement resource set, it makes sense to report the corresponding SSBRI/CRI of the selected beams that are a subset of Set B. However, regarding the associated sub-bullet, if (M) is equal to the size of the measurement resource set, i.e., all of Set B is configured for reporting, then there may be no need to report the beam-related information. Therefore, we have the following observation:

- For a NW side AI/ML model, for BM Case1 and Case 2, regarding the reporting of Set B measurements for inference purpose, when (M) is equal to the size of the measurement resource set, and Set B is defined solely by the resources of this resource set, it may not be necessary to report the beam index of the largest L1-RSRP value. The measured absolute/differential L1-RSRPs alone may be sufficient for the AI/ML model to operate effectively.

### The number of reported beams, i.e., the value of M

In BM-Case 2, Set B could be the same as Set A; therefore, the size of Set A should be used as a starting point for the maximum number of reported beams (256 beams). In prior Technical Documents (Tdocs), companies discussed that with the limit of polar codes (1706 bits), and assuming differential L1-RSRP reporting of the beams of Set B with 4 bits, the maximum number of supported beams can be up to 1706/4 = 426 beams.

### The beam information

To minimize specification effort and maintain the CSI framework, we support using the CRI/SSBRI as the starting point for the beam information resulting from the NW-side AI/ML model output.

Based on the above discussion, we have the following proposal:

- For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support:

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

- If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- For the maximum value of M, and in order to support the cases when Set B is equal to Set A, support reporting the size of Set A (256 beams) as a starting point.

- For the beam information, support reporting CRI/SSBRI as a starting point.

Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specification

## UE Sided AI/ML Models

For a UE side AI/ML model the following agreement, in RAN1#116 meeting, about the content of the model inference, is made:

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

The agreement above provides different options for the UE-side AI/ML model inference output. Options 3 and 4 offer additional probability and confidence information about the Top-K selected beams and predicted RSRP from the AI/ML model. This information could be beneficial for gNB scheduling decisions, as it could allow the gNB to configure the UE with the appropriate value of K measurements and even revert to legacy configurations when necessary. However, a trade-off may exist between the potential benefits of this information in enhancing system performance (after accounting for quantization loss) and the additional overhead incurred due to the reporting of extra probability/confidence information, as well as the increased computational complexity from quantizing this information.

- For the UE-side AI/ML model, regarding the content of the report inference results, support using options 1 through 3. Further study the benefits and gains of adopting option 3, considering the effects of quantization methods on system throughput and considering the additional overhead incurred.

### Definition of the confidence information

Based on our understanding, the confidence information is the statistical confidence interval. A confidence interval represents the range within which an estimate is expected to fall a certain percentage of the time if the experiment is repeated, or the population is resampled in the same manner. By adopting this definition, the confidence information reported by the UE in Option #4 could include a probability value and a range, defined by minimum and maximum values, for each beam in Set A. At each inference instance, the probability value indicates the confidence level of the predicted RSRP by the AI/ML model for the corresponding beam. If a beam in Set A has a narrow range and a high probability, and the predicted RSRP for that beam falls within this narrow range, it implies that the UE is highly confident that this predicted RSRP is accurate, i.e., it is the value that would be measured if this beam were transmitted by the gNB.

Given the definition of confidence information, an additional overhead is incurred as the UE must report a probability value and an interval for each beam. However, if the UE is confident in its reported RSRP value, this can assist the gNB in configuring the performance monitoring set, where RS are transmitted to provide the UE with the ground truth for assessing its AI/ML model performance. This is particularly beneficial when the performance monitoring set size needs to be smaller than Set A, especially in cases where Set A is large. Configuring a performance monitoring set of the same size as a large Set A may not be the most efficient use of DL resources and could also impose a computational burden on the UE by requiring measurements on the entire large set.

Based on the above discussion, we have the following proposals:

- Regarding the definition of the confidence information, for option 4 of a UE side AI/ML model report content for inference, consider the following definition:

  - The confidence information represents a range within which the predicted RSRP of a beam in Set A is expected to fall a certain percentage of the time if the same input sample is reused to derive the AI/ML model.

FFS on the details of the calculation of the confidence interval

- For a UE-side AI/ML model report content for inference, further study the applicability of adopting Option 4, including its pros and cons, in terms of the associated UL overhead and its potential to provide the gNB with information that could assist in configuring the performance monitoring set.

### Definition of the beam information

Similar to the NW side AI/ML models, and in order to reduce the specification impact while maintain the CSI framework, we support using CRI/SSBRI as a starting point.

- For a UE side AI/ML model, for the FFS on beam information, support using CRI/SSBRI as a starting point.

### Definition of the predicted top-K beams

The definition of the predicted beams depends solely on how the labels in the data set, on which the AI/ML model is trained, are defined and collected. For example, a data set might be collected such that the label is the CRI that provides the highest L1-RSRP value. In another case, the label might be the CRI that provides the highest SINR. Therefore, the inference output should generally be independent once specified (i.e., CRI/SSBRI), of how the data label is collected and defined, as the AI/ML model operates blindly. Hence, we propose the following:

- For a UE-side AI/ML model, the definition of the predicted top-K beams is related to the definition of the data set labels and the method of label collection. If the model output is specified (e.g., beam information such as CRI/SSBRI), it may be unnecessary to define the top-K predicted beams, and how they are generated during training, explicitly, and this could be left to implementation.

### Bit width of the reported probability information

To optimize the quantization of probability information generated by the AI/ML model and reduce uplink overhead, it is proposed that the UE rounds the probability values to one decimal place. This approach would require a maximum of 4 bits to represent the probability information per beam. Therefore, we present the following proposal:

- For a UE side AI/ML model probability information reporting, support one decimal precision probabilities where only 4 bits (bit width) are required to represent the probability information per beam. The choice of rounding up/down the probability values is left up to the UE’s implementation.

### Ranking of the reported top-K beams

Upon inference, the UE is expected to report the top-K beams based on its AI/ML model predictions. During this process, the UE must inform the gNB about the ranking of the beams within the top-K set. The method of reporting this ranking depends on how the UE conveys the top-K beams. If the UE reports the top-K beams using a CRI vector, it should order the CRIs in descending order. This means the UE will provide a K-dimensional vector of CRIs, where the ith entry corresponds to the ith strongest beam.

Alternatively, the UE might report the beams using a bitmap instead of CRI information. In this case, the UE would report the bitmap K times, each time corresponding to a different beam. For instance, if the UE reports the top-K beams out of N beams in Set A, it will provide a matrix with K rows and N columns, where the ith row contains zeros except for the entry corresponding to the ith strongest beam.

While the bitmap approach may introduce more UCI overhead due to the need to report the bitmap multiple times, vector and matrix compression techniques could be employed to mitigate this, albeit at the cost of increased computational complexity for the UE. A more detailed study is required to assess the feasibility of this approach compared to CRI vector signalling.

Based on the above discussion, we have the following proposal:

- For a UE-side AI/ML model inference report on beam ranking, where the UE is required to report the top-K beams out of N beams in Set A, the following methods can be adopted:

  - CRI-based Reporting: The UE should report the beam information in a ranked manner using a CRI vector. This involves reporting a K-dimensional vector of CRIs, where the ith entry corresponds to the ith strongest beam.

  - Bitmap Reporting: The UE should report the bitmap K times, with each instance corresponding to a different beam. For example, the UE would report a matrix with K rows and N columns, where the ith row contains zeros except for the entry corresponding to the ith strongest beam.

FFS: The feasibility of the bitmap compared to the CRI vector signaling with respect to the associated UL overhead.

# Consistency and additional conditions

## UE Sided AI/ML Models

In RAN1#118 meeting, the following agreement was made regarding the consistency of a UE side AI/ML model:

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

The above agreement discusses a method to address the issue of additional conditions that may arise when the UE is trained under specific conditions, but inference is performed under different conditions. This discrepancy can lead to variations in the input sample distribution of the AI/ML model that the UE is trained on. In option 1, an associated ID is introduced.

As we understand it, when a UE-side AI/ML model is trained under specific NW conditions, the training session is assigned an associated ID. Once the model is fully trained and ready for inference, the NW signals the UE with the associated ID corresponding to the input samples it can provide given its current configuration. If this ID matches any trained AI/ML model at the UE, the UE uses this model for inference. Otherwise, the UE may request the gNB to revert to the legacy method.

### FFS on whether/how to define similar properties of a DL Tx beam or beam set/list

This FFS concerns what can be assumed by the UE with the same associated ID during training and inference. Since the primary purpose of the associated ID is to ensure the consistency of the AI/ML model between training and inference, it is logical to assume the following regarding the same associated ID during both phases:

- Coverage Assumptions:

  - Beam Properties (e.g., shape, width, side lobe level, etc.): For a given associated ID, the beams in Sets A and B should exhibit similar properties during both training and inference. During the training phase, the AI/ML model should be adapted to represent/model the distribution of the training dataset. The sample space represented by this distribution is influenced by beam measurements from either Sets A or B, which are highly dependent on the beam properties. To minimize generalization error for the associated ID, the mismatch between the distributions of the training and test datasets should be minimized. Therefore, for the same associated ID, the beam shape properties and the dataset measurements should be almost identical during both training and inference stages.

  - Sets A and B Size: The number of beams in Sets A and B should be consistent during both training and inference stages. If the AI/ML model is trained on a Set B containing N beams, then inference should also be performed on an input of no more than N beams. Similarly, if the AI/ML model is trained to infer beams from a Set A containing M beams, it is expected to predict beams from a Set A of up to M beams during the inference stage.

- Resource Assumptions: The arrangement/indexing of resource indices should remain consistent during both training and inference stages. For example, consider a UE-side AI/ML model whose input is the Set B defined by the resource set of resources n1 through nN. If the measurement from resource ni is used for pin i of the AI/ML model during training, then during inference, resource ni should also be applied to pin i of the AI/ML model input. Similarly, for Set A resources, if the UE-side AI/ML model output pin k is matched with the label from the resource of CRI k of the Set A resources, then during inference, output pin k should represent the result for CRI k.

Based on the above discussion, we have the following proposal:

- For ensuring consistency in network-side additional conditions across training and inference phases for UE-side models in BM Case 1 and BM Case 2, where network-side conditions may impact UE assumptions on beams of Set A/Set B, the following assumptions should be considered by a UE with the same associated ID during both training and inference stages:

- Coverage assumptions:

    - Beam properties: Beam shape, width, and side lobe levels should remain consistent.

    - Set Sizes: The sizes of Sets A and B should be similar.

- Resource assumptions: The arrangement/indexing of resource indices should be consistent during training and inference stages.

### How associated ID is introduced

To minimize specification effort, the associated ID is introduced within the existing CSI framework. Initially, the associated ID can be defined within the current RRC messaging of nzp-CSI-RS-ResourceSet and csi-SSB-ResourceSet. Our proposal is as follows:

- To ensure consistency of NW-side additional conditions across training and inference for UE-sided models in BM-Case 1 and BM-Case 2, where these conditions may impact UE assumptions on beams of Set A/Set B, the associated ID(s) is introduced within the current CSI framework. Specifically, introduce the associated ID(s) as a new IE within the RRC messaging of nzp-CSI-RS-ResourceSet and csi-SSB-ResourceSet.

### A globally defined associated ID

In RAN1#118 meeting the following proposal was made in agenda item 9.1.3.3 (other aspects of AI/ML model and data):

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

As previously explained in Section 5.1, the main purpose of the associated ID is to ensure consistency between training and inference by ensuring that the NW additional conditions are the same. However, in our view, there comes two issues that are related with the associated ID:

- Issue#1 (Intra-vendor): This issue occurs within the same vendor’s network. Assume that identical additional conditions are applied by different gNBs from the same vendor, covering a large geographical area. Given that these gNBs share the same additional conditions, the associated IDs for those cells are expected to be identical. During the inference stage, if a UE moves across these cells and consistently receives the same associated ID, it could indicate to the UE vendor that the same additional conditions are being utilized within the same network vendor, thereby revealing the network deployment to the UE.

- Issue#2 (Inter-vendor): Assume there are two gNBs of different network vendors, A and B, each connected to a UE (UE A to gNB A and UE B to gNB B) equipped with an AI/ML model. Both gNBs have different additional conditions that would impact the performance of the AI/ML model at the connected UE.

gNB A initiates a training session with its connected UE, assigning an associated ID#n to that training session based on gNB A's additional conditions. Similarly, gNB B initiates a training session with its connected UE, assigning an associated ID#m based on gNB B's additional conditions. After the models at both UEs are trained and ready for inference, UE A moves from gNB A to gNB B. Upon connection, gNB B sends its supported associated ID (ID#m) to UE A. In this case, UE A should inform gNB B that its AI/ML model cannot support the data type provided by gNB A (since it was trained on the additional conditions from gNB A and its associated ID is n≠m).

An interesting scenario arises if gNBs A and B operated independently and assigned the associated IDs to their training sessions without any agreement or communication. In other words, if the associated IDs collide, both gNBs could provide the same ID#n. If UE A moves from gNB A to gNB B, it would be assigned the same ID#n it was trained on and might assume that gNB B has the same additional conditions as gNB A. Hence, to avoid such collision, gNBs A and B need to share their mapping from the NW additional conditions to the associated ID, which in turns reveals the vendor’s proprietary information.

Based on this discussion, we have the following observation:

- For a UE-side AI/ML model in BM Case 1 and BM Case 2, to ensure consistency of the network-side additional conditions across training and inference, the associated ID is introduced. However, it has the following limitations:

- It should be consistent within the same cell or across cells from the same network vendor that span a small geographical area.

- It could reveal the network’s proprietary information if used within a large area (intra-vendor) or if the mapping is shared among different vendors (inter-vendor).

- Its applicability across multiple cells (intra or inter-vendor) depends on the assumptions made by the UE with the same associated ID.

And the following proposal:

- RAN1 should prioritize finalizing the discussion regarding what can be assumed by the UE with the same associated ID before addressing the applicability of the associated ID across different cells.

### Consistency based on performance monitoring

In this approach, the UE initially adopts one of its trained models for inference without prior knowledge of the NW additional conditions. The NW or the UE monitors the model's performance. If the AI/ML model performance is acceptable, the adopted model continues to be used. Otherwise, the UE switches between its trained models, monitoring performance until finding one with acceptable results. This method could provide a generalization capability for the UE without requiring the NW to characterize/group its additional conditions and assign them an ID. For the performance monitoring-based approach, ensuring consistency can be part of the AI/ML model performance monitoring discussion. Given the vast number of possible configurations of Set A/B and associated NW transmission parameters, the feasibility of using this method to address consistency is unclear.

## NW Sided AI/ML Models

### How to ensure consistency for NW side AI/ML model

Similar to UE-side AI/ML models, UEs can operate differently between training and inference stages. For a NW-side AI/ML model, the NW relies on the UE to collect the input measurements required for its model. Therefore, it is crucial that UEs behave consistently during training and inference to ensure that the input samples are collected uniformly for the ML model. As we understand, the receive beam at the UE side could potentially impact the collected measurements of Set B. During the Rel 18 evaluations, it was shown that if the received beam changes randomly, the prediction accuracy could degrade severely [1].

One way to mitigate this issue is to provide the NW with information about the receive beam, encapsulated in a beam index without revealing any proprietary information about the beam. To further reduce the overhead associated with reporting the beam index from the UE, the current receive beam can be assumed as the receive beam assumption. When the gNB configures the CSI-RS resource set for channel measurements, it can also define the QCL relationship of the resources included in the configured set. Consequently, the UE can use the most suitable receive beam it has identified for the QCL source for measuring all CSI-RS beams in the configured resource set.

- For a NW-side AI/ML model, to ensure consistency between training and inference, study the feasibility of the following mechanisms:

  - The UE reports the receive beam index to the NW.

  - To eliminate the UL reporting overhead, the best receive beam is assumed by the NW. The NW configures the QCL relationship, and the UE uses the receive beam identified for the QCL source for performing the measurements.

# Performance Monitoring

## UE Sided AI/ML Models

For a UE side AI/ML model, the following agreement was made regarding performance monitoring:

Agreement: For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 FFS Type 2 performance monitoring

### Type 1 option 1 monitoring

In Type 1 performance monitoring, the gNB receives input from the UE and takes actions based on that input, such as model switching or fallback to legacy systems. In Option 1, the UE blindly sends the inference output along with measurements from the resource set configured for performance monitoring. The gNB processes these inputs, calculates the KPI (i.e., accuracy/performance-based KPI), and performs the model action accordingly. To align with the agreement of the report for inference for a UE-side AI/ML model in Section 4, reporting the following could be considered as a starting point:

- Model related outputs: beam information of the top-K predicted beams, predicted RSRP, probability information of the predicted beams and the confidence information of the of the RSRP.

- Performance related measurements: CRI/SSBRI and the L1-RSRP of the top-K beams.

Regarding the FFS on the other content, we think that this discussion should come after finalizing the discussion on the report content for inference.

Based on this discussion, we have the following proposal:

- For the performance monitoring of a UE side AI/ML model, for the content of the report of type 1 option 1 monitoring, support the following as a starting point:

- Model related outputs:

  - Beam information of the top-K predicted beams.

  - Predicted RSRP if supported by the AI/ML model.

  - Probability information of the predicted beams.

  - Confidence information of the of the RSRP if supported by the AI/ML model.

- Performance related measurements from the resource set configured for monitoring:

  - CRI/SSBRI

  - L1-RSRP of the top-K beams

### Type 1 option 2 monitoring

In option 1, the UE frequently provides the gNB with inputs to assess the performance of the AI/ML model. While frequent updates of ML model outputs and resource measurements offer the NW flexibility to properly schedule and optimize transmission, this approach may lead to significant UL overhead. Hence, option 2 grants the UE more authority to utilize information from both the ML model and performance resource set measurements to calculate the performance metric/KPI. The UE then reports this metric to the gNB, which takes the monitoring decision, such as model switching or fallback to legacy. Option 2 reduces UL overhead, as the UE reports fewer parameters than in option 1. However, the NW/gNB receives less information about the performance of the UE-side model until the metrics are reported. In option 2, the calculated metric can be reported either statistically (resulting in less UL overhead but leaving the NW unaware of the performance condition for longer periods) or per sample (increasing UL overhead but providing the NW with more frequent updates). As a starting point, we support using the error between the predicted RSRP and the measured L1-RSRP, along with the beam prediction accuracy of the top-K beams. Based on this discussion, we have the following proposal:

- For type 1, option 2, performance monitoring of a UE side AI/ML model, regarding the FFS of:

- How to report- assess the impact of adopting the following methods:

  - Sample based performance metric reporting in order to provide the NW with frequent updates about the AI/ML model performance.

  - Statistical based performance metric reporting in order to reduce the UL overhead on the expense of less information being available at the gNB.

- What to report- Support using the following metrics as a starting point:

  - The error between the predicted RSRP and the measured L1-RSRP, where the predicted RSRP is based on AI/ML output.

  - The beam predication accuracy of the top-K beams, where the beam prediction accuracy is the percentage of the beam determined from RS measurements for performance monitoring to be one of the top-K predicted beams by the AI/ML model.

In RAN1#118 meeting, the following agreement was made regarding Type 1 option 2 performance monitoring:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

#### The monitoring KPIs

Alt#4 suggests that the probability information output by the UE-side AI/ML model can be utilized to assess the model’s performance when reported to the gNB. However, we believe that the performance monitoring stage must incorporate a ground truth reference, which in this case would be the RS measurements, to accurately evaluate the AI/ML model’s output. Alt#4, however, does not include any ground truth measurements in its performance monitoring, relying solely on the AI/ML model’s probability information.

Consider the following example illustrated in Figure 1: During the data collection and training phase, a UE moves across Cell A, collecting data and training its model. This UE-side AI/ML model consists of “M” output pins corresponding to “M” beams, with each pin value indicating the probability of the beam being the Top-1 beam. Due to geographical barriers, the UE can only collect data corresponding to beams “m” through “M”, leaving out beams 1 through m-1. Consequently, the UE-side AI/ML model is trained only on these beams.

Now, assume that after training, the UE moves from Cell A to Cell B, which has similar conditions. In Cell B, the UE’s location corresponds to the area where beams were barred in Cell A. In this scenario, the UE’s model, not having been trained on beams 1 through “m” will output low probabilities on the model output pins corresponding to these beams. Based on our understanding, one of the following scenarios could occur:

- The remaining model output pins, ranging from “m” to “M” could also exhibit low probability values. Consequently, when any pin probability value is reported to the gNB, it will determine that the UE-side model lacks confidence in its selection. In response, the gNB will initiate the appropriate procedure, such as model switching or falling to legacy.

- The AI/ML model output pins from “m” to “M” could exhibit high probability values. The AI/ML model will distribute the probabilities among the known labels (from “m” to “M”). Since it does not recognize labels from 1 to “m-1”, it might still assign higher probabilities to labels m, m+1, …, M even though none of these are correct. In this case, the gNB might interpret the high probabilities as an indication that the UE is confident in its model and that its beam selection is suitable for BM operation, when in fact it is not.

Figure 1 A UE is trained in Cell A (with a barrier) and performing inference in Cell B.

Based on the above discussion, we have the following proposal:

- For BM Case 1 and Case 2 in a UE side AI/ML model, deprioritize Alt 4 as a measure for Type 1 option 2 performance monitoring.

#### FFS on how to configure the resource set/resources for monitoring

During performance monitoring, the gNB allocates resources to the UE for measurement purposes. These measurements serve as ground truth for evaluating the UE’s AI/ML model. Ideally, the performance monitoring measurement set should match the size of Set A, with each resource in the monitoring set corresponding to a beam from Set A. This enables the UE to gather ground truth measurements for all beams in Set A and compare the top-K ground truth measurements with the top-K predicted beams. However, as the size of Set A increases, this method may impose a computational burden on the UE and increase the signalling overhead. Additionally, from the gNB’s perspective, it is inefficient in terms of DL resource utilization to transmit RS on resources equivalent to the size of a large Set A. Based on this analysis, we propose the following:

- Regarding the size of the performance monitoring set of a UE-side AI/ML model, support that the size of the performance monitoring set is less than or equal to the size of Set A and defined by the network.

When the performance monitoring measurement set size is smaller than Set A, the gNB should intelligently configure the resources of this set to provide measurements for the actual top-K beams, which the UE will use as ground truth. To achieve this, the gNB requires information to select the appropriate monitoring set for configuration. As an example, the UE could signal the gNB with information from set B along with that of the top-K beams (see Figure 2 below). Based on that information, both the UE and the gNB can identify specific resources for performance monitoring where the gNB transmits RS and the UE performs measurements.

Figure 2 Example demonstrating how beam information from Set B is mapped to the performance monitoring set, that is a subset of the total resource set configured for monitoring, where each resource corresponds to a beam in Set A

This approach requires that the gNB and the UE use the same mapping between the Set B beam information and the performance monitoring set. The mapping information needs to be regularly signalled to the UE before each monitoring instance. This ensures the UE performs the correct measurements, which may introduce increased downlink signalling overhead. To mitigate this overhead, the gNB can create a lookup table that defines the mapping between the beam information and the corresponding performance monitoring set. This mapping can be broadcast to the UE in the System Information Block (SIB) message, or any other gNB to UE dedicated message during initial access. Based on the above discussion, we have the following proposal:

- For Type 1 option 2 performance monitoring of a UE-side AI/ML model, support using a mapping from beam information to performance monitoring resources. This mapping information can be provided to the UE though SIB or any other gNB to UE dedicated message.

### Report triggering based on event(s) for Option 1 and/or Option 2

It is unclear how the UE can trigger a report based on an event in option 1. From our understanding, an event is defined as a scenario where the performance metric (whether sample-based or statistical) exceeds or falls below a specified threshold, prompting a report to be sent to the NW. In option 1, the UE does not calculate the performance metric needed to detect such an event; it simply reports the model output and measurement results to the NW without evaluating performance thresholds. Therefore, it is uncertain how a report can be triggered based on events in option 1. Conversely, this approach appears more applicable to option 2, where the UE calculates the performance metric and can thus determine when an event occurs. Based on this, we propose the following:

- For UE-side AI/ML performance monitoring, RAN1 should further study the following:

- An event is defined as the scenario when the performance metric falls above (or below) a certain threshold.

- The concept of triggering a report based on specific events is not applicable to option 1. It is, however, relevant to option 2, where the UE calculates performance metrics and can detect events.

### Type 2 performance monitoring

In Type 2 performance monitoring, the UE has full responsibility for calculating the performance metric and deciding on model switching, activation, or fallback to legacy. We believe that this approach has minimal specification impact and should be left to the discretion of the UE vendor for implementation.

- For UE side AI/ML performance monitoring, type 2 monitoring has no specification impact, and it should be left up to the UE vendor implementation.

## NW Sided AI/ML Models

For a NW side AI/ML model performance monitoring, we don’t see any specification impact needed. It is up to the gNB vendor implementation. Hence, we have the following proposal:

- For a NW side AI/ML model performance monitoring, there is no specification impact is needed, and it can be left up to the NW vendor implementation.

# Conclusion

In the previous sections we made the following observations:

- For BM Case 1 and Case 2, in a NW-sided or UE sided AI/ML models, for any LCM aspect (data collection, training, inference, and performance monitoring), the configuration of either Set A or Set B for the UE for measurement and reporting may impose a restriction on the gNB scheduling. However, it will not require any additional capability from the UE. This is an implementation issue and does not necessitate further specification effort.

- For a NW side AI/ML model, for BM Case1 and Case 2, regarding the reporting of Set B measurements for inference purpose, when (M) is equal to the size of the measurement resource set, and Set B is defined solely by the resources of this resource set, it may not be necessary to report the beam index of the largest L1-RSRP value. The measured absolute/differential L1-RSRPs alone may be sufficient for the AI/ML model to operate effectively.

- For a UE-side AI/ML model in BM Case 1 and BM Case 2, to ensure consistency of the network-side additional conditions across training and inference, the associated ID is introduced. However, it has the following limitations:

- It should be consistent within the same cell or across cells from the same network vendor that span a small geographical area.

- It could reveal the network’s proprietary information if used within a large area (intra-vendor) or if the mapping is shared among different vendors (inter-vendor).

- Its applicability across multiple cells (intra or inter-vendor) depends on the assumptions made by the UE with the same associated ID.

Based on the discussion in the previous sections we propose the following:

- For a NW-sided AI/ML model for BM-Case1 and BM-Case2, RAN1 should consider the following options:

- Set A: No configuration for Set A is required for the UE, as the inference process is conducted transparently from the UE.

- The gNB should configure reporting-related parameters, including the number of beams that the UE should report.

- Explicit configuration of Set B is necessary, with resources for Set B explicitly defined for the UE.

- For a UE-side AI/ML model, support configuring Sets A and B for inference results reporting, where:

  - Set A is virtually configured. It is not intended to provide measurements for the UE. Instead, it serves as a reference to map the UE-side AI/ML model output, in the case of classification models, to the corresponding CRIs that the gNB can interpret.

  - Set B is explicitly configured for measurements for the UE. It enables the UE to perform measurements that derive the UE’s AI/ML model.

- For UE-side AI/ML models, the association provides the UE with information about the beams that belong to Sets A and B. To facilitate this association, a new information element is required.

- For training data collection of a NW side AI/ML model, in BM Cases 1 and 2, RAN1 should further study the following:

- Support using the CSI framework for inputs and labels collection of data samples for the purpose of training the AI/ML model. New IEs may be introduced, e.g., “CSI-AIML-ResourceSet” and “AIML-info-RSRP” to provide the gNB the flexibility to define Set B.

- Support reporting all Set B measurements during the data collection and training phase.

- For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support:

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

- If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- For the maximum value of M, and in order to support the cases when Set B is equal to Set A, support reporting the size of Set A (256 beams) as a starting point.

- For the beam information, support reporting CRI/SSBRI as a starting point.

Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specification

- For the UE-side AI/ML model, regarding the content of the report inference results, support using options 1 through 4. Further study the benefits and gains of adopting options 3 and 4, considering the effects of quantization methods on system throughput and considering the additional overhead incurred.

- Regarding the definition of the confidence information, for option 4 of a UE side AI/ML model report content for inference, consider the following definition:

  - The confidence information represents a range within which the predicted RSRP of a beam in Set A is expected to fall a certain percentage of the time if the same input sample is reused to derive the AI/ML model.

FFS on the details of the calculation of the confidence interval.

- For a UE-side AI/ML model report content for inference, further study the applicability of adopting Option 4, including its pros and cons, in terms of the associated UL overhead and its potential to provide the gNB with information that could assist in configuring the performance monitoring set.

- For a UE side AI/ML model, for the FFS on beam information, support using CRI/SSBRI as a starting point.

- For a UE-side AI/ML model, the definition of the predicted top-K beams is related to the definition of the data set labels and the method of label collection. If the model output is specified (e.g., beam information such as CRI/SSBRI), it may be unnecessary to define the top-K predicted beams, and how they are generated during training, explicitly, and this could be left to implementation.

- For a UE side AI/ML model probability information reporting, support one decimal precision probabilities where only 4 bits (bit width) are required to represent the probability information per beam. The choice of rounding up/down the probability values is left up to the UE’s implementation.

- For a UE-side AI/ML model inference report on beam ranking, where the UE is required to report the top-K beams out of N beams in Set A, the following methods can be adopted:

  - CRI-based Reporting: The UE should report the beam information in a ranked manner using a CRI vector. This involves reporting a K-dimensional vector of CRIs, where the ith entry corresponds to the ith strongest beam.

  - Bitmap Reporting: The UE should report the bitmap K times, with each instance corresponding to a different beam. For example, the UE would report a matrix with K rows and N columns, where the ith row contains zeros except for the entry corresponding to the ith strongest beam.

FFS: The feasibility of the bitmap compared to the CRI vector signaling with respect to the associated UL overhead

- For ensuring consistency in network-side additional conditions across training and inference phases for UE-side models in BM Case 1 and BM Case 2, where network-side conditions may impact UE assumptions on beams of Set A/Set B, the following assumptions should be considered by a UE with the same associated ID during both training and inference stages:

- Coverage assumptions:

    - Beam properties: Beam shape, width, and side lobe levels should remain consistent.

    - Set Sizes: The sizes of Sets A and B should be similar.

  - Resource assumptions: The arrangement/indexing of resource indices should be consistent during training and inference stages.

- To ensure consistency of NW-side additional conditions across training and inference for UE-sided models in BM-Case 1 and BM-Case 2, where these conditions may impact UE assumptions on beams of Set A/Set B, the associated ID(s) is introduced within the current CSI framework. Specifically, introduce the associated ID(s) as a new IE within the RRC messaging of nzp-CSI-RS-ResourceSet and csi-SSB-ResourceSet.

- RAN1 should prioritize finalizing the discussion regarding what can be assumed by the UE with the same associated ID before addressing the applicability of the associated ID across different cells.

- For a NW-side AI/ML model, to ensure consistency between training and inference, study the feasibility of the following mechanisms:

  - The UE reports the receive beam index to the NW.

  - To eliminate the UL reporting overhead, the best receive beam is assumed by the NW. The NW configures the QCL relationship, and the UE uses the receive beam identified for the QCL source for performing the measurements.

- For the performance monitoring of a UE side AI/ML model, for the content of the report of type 1 option 1 monitoring, support the following as a starting point:

- Model related outputs:

  - Beam information of the top-K predicted beams.

  - Predicted RSRP if supported by the AI/ML model.

  - Probability information of the predicted beams.

  - Confidence information of the of the RSRP if supported by the AI/ML model.

- Performance related measurements from the resource set configured for monitoring:

  - CRI/SSBRI

  - L1-RSRP of the top-K beams

- For type 1, option 2, performance monitoring of a UE side AI/ML model, regarding the FFS of:

- How to report- assess the impact of adopting the following methods:

  - Sample based performance metric reporting in order to provide the NW with frequent updates about the AI/ML model performance.

  - Statistical based performance metric reporting in order to reduce the UL overhead on the expense of less information being available at the gNB.

- What to report- Support using the following metrics as a starting point:

  - The error between the predicted RSRP and the measured L1-RSRP, where the predicted RSRP is based on AI/ML output.

  - The beam predication accuracy of the top-K beams, where the beam prediction accuracy is the percentage of the beam determined from RS measurements for performance monitoring to be one of the top-K predicted beams by the AI/ML model.

- For BM Case 1 and Case 2 in a UE side AI/ML model, deprioritize Alt 4 as a measure for Type 1 option 2 performance monitoring.

- Regarding the size of the performance monitoring set of a UE-side AI/ML model, support that the the size of the performance monitoring set is less than or equal to the size of Set A and defined by the network.

- For Type 1 option 2 performance monitoring of a UE-side AI/ML model, support using a mapping from beam information to performance monitoring resources. This mapping information can be provided to the UE though SIB or any other gNB to UE dedicated message.

- For UE-side AI/ML performance monitoring, RAN1 should further study the following:

- An event is defined as the scenario when the performance metric falls above (or below) a certain threshold.

- The concept of triggering a report based on specific events is not applicable to option 1. It is, however, relevant to option 2, where the UE calculates performance metrics and can detect events.

- For UE side AI/ML performance monitoring, type 2 monitoring has no specification impact, and it should be left up to the UE vendor implementation.

- For a NW side AI/ML model performance monitoring, there is no specification impact is needed, and it can be left up to the NW vendor implementation.

# References

| [1] | TR 38.843 v2.0.1, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (Release 18),” Third Generation Partnership Porject (3GPP), 2023. |
| --- | --- |
| [2] | R1-2304749, “Evaluations of AI/ML for beam management, Ericsson,” 3GPP TSG-RAN WG1 Meeting 113, May 22-26, 2023. |